PUBLIC HEALTH ENGINEERING DIVISION THARPARKAR @ MITHI

FUND HEAD:	DISTRICT A.D.P

MAJOR HEAD:	WATER SUPPLY & DRAINAGE.

MINOR HEAD:	CONSTRUCTION OF SURFACE DRAINS AND C.C ROAD FOR DRAINAGE SYSTEM IN SANTORO FARM DISTRICT THARPARKAR

SERVICE HEAD/	DETAILED WORKING ESTIMATE FOR

DEPTT: HEAD 	CONSTRUCTION OF SURFACE DRAINS AND C.C ROAD FOR DRAINAGE SYSTEM IN SANTORO FARM DISTRICT THARPARKAR.

Estimate framed in the Office of the Executive Engineer Public Health Engg: Division Tharparkar @ Mithi for the probable expenditure that will be incurred on Construction of Surface Drains and C.C Road for Drainage System in Santoro Farm District Tharparkar.

ESTIMATED COST RS. 64,10,000/=

GENERAL DESCRIPTION

1. 	This Estimate has been prepared to cover the cost for Construction of Surface Drains and C.C Road for Drainage System in Santoro Farm District Tharparkar.

2.  	The expenditure will be charged to the Sub Head No: 1 of approved PC-I of the scheme for which provision of Rs. 19.668 Million exist in the approved PC-I.

3.	The work will be carried out as per specification and the instructions of the Engineer Incharge.

4.	Funds are available.

| ASSISTANT ENGINEER PUBLIC HEALTH ENGG: SUB-DIV:-I THARPARKAR @ MITHI |  | EXECUTIVE ENGINEER PUBLIC HEALTH ENGG: DIVISION THARPARKAR @ MITHI |
| --- | --- | --- |

DETAILED WORKING ESTIMATE FOR CONSTRUCTION OF SURFACE DRAINS AND C.C ROAD FOR DRAINAGE SYSTEM IN SANTORO FARM DISTRICT THARPARKAR

| 1. | Dismantling brick work in lime or cement mortar (G.S.I.No: 13 P-10). |
| --- | --- |

1 x 2 x 300 x 0.75 x 3.5+4 	= 1687.50 Cft

2	   Say	= 1688.00 Cft

1688.00 Sft		@ Rs. 1285/63	P% Cft			Rs. 21,701/=

| 2. | Dismantling and removing road metalling (G.S.I.No: 51 P-13. |
| --- | --- |

1 x 200 x 4.25 x 0.75	= 637.50 Cft

Say	= 638.00 Cft

638.00 Sft		@ Rs. 605/=		P% Sft				Rs. 3,860/=

| 3. | Dismantling brick or flagged flooring without concrete foundation (G.S.I.No: 30 P-12. |
| --- | --- |

1 x 200.0 x 3.0	= 600.00 Sft

600.00 Sft		@ Rs. 257/13	P%Sft				Rs. 1,543/=

| 4. | Excavation in foundation of buildings bridges and other structures including dag belling, dressing refilling around structures with excavated earth watering and ramming lead upto 5’ ft: (In sand, ashes, loose Soil) (G.I.S.No.18 (a) P/4). |
| --- | --- |

Type-A	1 x 4000 x 2.42 x 0.83+1.13   =   9486.00 Cft

2

Type-B	1 x 1500 x 3.0 x 1.5+2.0	     =   7875.00 Cft

2

Type-C	1 x 500 x 4.25 x 4.0+6.0	     = 10625.00 Cft

2

Total = 27986.00 Cft

27986.0		@ Rs. 2722/50	P‰ Cft			Rs. 76,192/=

| 5. | Cement concrete plain including placing, compacting, finishing and curing complete (Including screening and washing of stone aggregate without shuttering. (G.S.I. No. 5 P-15). |
| --- | --- |

(i) 	Ratio 1:4:8

Type-A	1 x 4000 x 2.42 x 0.25   	= 2450.00 Cft

Type-B	1 x 500 x 3.0 x 0.25	= 1125.00 Cft

Type-C	1 x 1500 x 4.25 x 0.33	=   701.00 Cft

For Road	1 x 1000 x 10+8 x 0.33	= 2970.00 Cft

2

Total 	= 7246.00 Cft

7246.00 		@ Rs. 11288/75	P%Cft				Rs. 8,17,983/=

(ii) 	Ratio 1:2:4

1 x 1000 x 10+8 x 0.25	= 2250.00 Cft

2

2250.00 		@ Rs. 14429/25	P%Cft				Rs. 3,24,658/=

| 6. | Pucca brick work n foundation and plinth in cement sand mortar (1:6) (G.S.I. No. 4 (i)/e P-20). |
| --- | --- |

Type-A	1 x 2 x 4000 x 0.75 x 0.83+1.13  	= 5880.00 Cft

2

Type-B	1 x 2 x 1500 x 0.75 x 1.50+2.50	= 3938.00 Cft

Type-C				        2

1st Step	1 x 2 x 500 x 1.13 x 5.0		= 5650.00 Cft

______________

Total 	= 15468.00 Cft

15468.00 Cft	 @ Rs. 11948/36	P% Cft			Rs.18,48,172/=

| 7. | Construction of standard open drain comet block of cement concrete (1:2:4) in situ design profile i.e. cost of mould as per drawing i/c applying floating coat of cement 1/32" thick to the exposed face finished smooth curing etc complete as per detailed drawing. (P.H.S.I No. D P-44). |
| --- | --- |

Type-A	1 x 4000 	= 4000.0 Rft

4000.0 Rft		@ Rs. 94/=		P.Rft				Rs. 3,76,000/=

Type-B	1 x 1500 	= 1500.0 Rft

1500.0 Rft		@ Rs. 174/=		P/Rft				Rs. 2,61,000/=

Type-C	1 x 500	= 500.00 Rft

500.00 Rft		@ Rs. 309/=		P/Rft				Rs. 1,54,500/=

| 8. | Cement Plaster (1:4) upto 20’ height 1/2” thick (G.S.I.No. 11 (b) P-51) |
| --- | --- |

Type-A	1 x 2 x 4000 x  1.25+1.55   = 11200.00 Sft

2

Type-B	1 x 2 x 1500 x 1.71+2.71	 =    6630.00 Sft

2

Type-C	1 x 2 x 500 x 4.21+6.21	 =   5210.00 Sft

2              _______________

Total = 23040.00 Sft

23040.00 Sft 	@ Rs. 2283/93	P% Sft				Rs. 5,26,217/=

| 9. | Erection and removal of centering for R.C.C or plain cement concrete works of partal wood (G.S.I.No. 19 (b) P-18). |
| --- | --- |

2 x 1000 x 0.25		= 500.00 Sft

1 x 130 x 10+8 x 0.25 	= 292.50 Sft

2		  Total	= 792.50 Sft

Say	793.00 Sft 		@ Rs. 3127/41	P% Sft				Rs. 24,800/=

| 10. | Reinforced cement concrete work including all labour and material except the cost of binding which will be paid separately. This rate also includes all kinds of forms moulds lifting shuttering curing rendering and fining washing of shingle (a) RCC work in roof slab beams columns rafts lintels and others structural members laid in situ or precast laid in position complete in all respects Ratio (1:2:4) 90 Lbs cement 2 Cft sand and 4 Cft shingle 1/8” to ¼" gauge (G.S.I. No.6) (a) Page No.16). |
| --- | --- |

Bracket 	40 x 3.50 x 0.75 x 0.50	=   52.50 Cft

Type-A 	10 x 12.0 x 2.42 x 0.50	= 145.20 Cft

50 x 4.0 x 2.42 x 0.25	= 121.00 Cft

Type-B	10 x 15.0 x 3.0 x 0.50	= 225.00 Cft

50 x 4.0 x 3.0 x 0.33	= 198.00 Cft

Type-C	2 x 15.0 x 3.50 x 0.75	=   78.75 Cft

5 x 12.0 x 3.50 x 0.50	= 105.00 Cft

Total	= 925.45 Cft

Say	925.0 Cft		@ Rs. 	337/=		P.Cft				Rs. 3,11,809/=

| 11. | Fabrication of mild steel reinforcement for cement concrete including cutting, bending laying in position making joints and fastening laying in position binding wire (also includes removal of rust from bars) (G.S.I. No.8 (b) P-17). |
| --- | --- |

925 x 4.50	  = 37.165 Cwt

112

37.165 Cwt		@ Rs. 5001/70	P.Cwt				Rs. 1,85,889/=

| 12. | Re-handling of earth work lead upto 50 ft. (G.S.I.No. 9 (b) P-2) |
| --- | --- |

Qty: same as item No. 4 i.e. 27986.00 Cft

27986.00 Cft		@ Rs. 1058/75	P‰ Cft			Rs. 29,630/=

| 13. | Laying earth in 6" layers leveling, dressing and watering for compaction etc complete (G.S.I.No. 13 (b) P-3). |
| --- | --- |

C.C Road 	1 x 2000 x 10+8 x 1.0+1.50	 = 22500.00 Cft

2	  2

22500.00 Cft		@ Rs. 354/=		P‰ Cft			Rs. 7,965/=

Total		Rs. 49,71,919/=

| 14. | Carriage of 100 Cft / 5 tons of all material like stone aggregate, spawl coal lime surkhi etc. B.G Rail fastening point and crossing bridges girders pipes sheets rails. M.S Bars etc or 1000 Nos bricks 10” x 5” x 3” or 1000 Nos tiles 12” x 6” x 2” or 150 Cft of timber or 100 mounds of fuel wood by trucks or any other means owned by the Contractor (Sch: of Carr: of Material (Item No. 1 P-1). |
| --- | --- |

| (i) | Bajri from Khanote to site of work @ Islamkot (Lead 131 miles). |
| --- | --- |

12255.0 Cft		@ Rs. 5952/96 	P% Cft        			Rs. 5,93,382/=

| (ii) | Hill Sand from Bholari site of work @ Islamkot (Lead 111 Miles). |
| --- | --- |

11690.0 Cft		@ Rs. 4190/76	P% Cft		         Rs. 4,89,900/=

| (iii) | Cement from Zeal Pak Cement Factory Hyderabad to site of work @ Islamkot (Lead 100 Miles). |
| --- | --- |

2602.0 Bags		@ Rs. 66/33		P.Bag				Rs. 1,72,590/=

| (iv) | Steel from Hyderabad to site of work @ Islamkot (Lead 97 Miles). |
| --- | --- |

37.165 Cwt 		@ Rs. 37/34		P.Cwt		          		Rs. 1,388/=

| v) | Bricks from Naukot to site of work @ Islamkot (Lead 05 miles). |
| --- | --- |

208818 Nos 		@ Rs. 865/89	P‰ Nos 			Rs. 1,80,813/=

Total	Rs. 64,09,992/=

Total	Rs. 64,10,000/=

| ASSISTANT ENGINEER PUBLIC HEALTH ENGG: SUB-DIV: NO:I THARPARKAR @ MITHI |  | EXECUTIVE ENGINEER PUBLIC HEALTH ENGG: DIVISION THARPARKAR @ MITHI |
| --- | --- | --- |

## MATERIAL STATEMENT

| Sr. No | Item | Qty | Bajri | Hill Sand | Cement | Steel | Bricks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 01 | C.C. Plain (1:4:8) | 7246.0 | 6521.0 | 3261.0 | 652.0 | --- | --- |
| 02 | Pucca brick work (1:6) In foundation | 15468.0 | --- | 4640.0 | 619.0 | --- | 208818 |
| 03 | Construction of connote block (1:2:4) T-A 4000 x 0.35 = 1400.0 T-B 1500 x 0.74 = 1110.0 T-C 500 x 1.37 = 685.0 Total = 3195.0 | 3195.0 | 2876.0 | 1438.0 | 575.0 | --- | --- |
| 04 | Cement plaster (1:4) 1/2" thick | 23040.0 | --- | 922.0 | 184.0 | --- | --- |
| 05 | R.C.C work (1:2:4) | 925.0 | 833.0 | 416.0 | 167.0 | 37.165 | --- |
| 06 | C.C Plain (1:2:4) | 2250.0 | 2025.0 | 1013.0 | 405.0 | --- | --- |
|  |  | # Total | 12255.0 | 11690.0 | 2602.0 | 37.165 | 208818 |
|  |  | # Say | 12255.0 Cft | 11690.0 Cft | 2602.0 Bags | 37.165Cwt | 208818 Nos: |

ASSISTANT ENGINEER

PUBLIC HEALTH ENGG: SUB-DIV: NO:I

THARPARKAR @ MITHI

SCHEDULE “B”

CONSTRUCTION OF SURFACE DRAINS AND C.C ROAD FOR DRAINAGE SYSTEM IN SANTORO FARM DISTRICT THARPARKAR

| 1. | Dismantling brick work in lime or cement mortar (G.S.I.No: 13 P-10). |
| --- | --- |

1688.0 Sft		@ Rs. 1285/63	P% Cft				Rs. 21,701/=

(Rupees One Thousand Two Hundred Eighty Five

and Paisas Sixty Three) only (Per Hundred Cft)

| 2. | Dismantling and removing road metalling (G.S.I.No: 51 P-13. |
| --- | --- |

638.0 Sft		@ Rs. 605/=		P% Sft				Rs. 3,860/=

(Rupees Six Hundred Five) only (Per Hundred Sft)

| 3. | Dismantling brick or flagged flooring without concrete foundation (G.S.I.No: 30 P-12. |
| --- | --- |

600.0 Sft		@ Rs. 257/13		P%Sft				Rs. 1,543/=

(Rupees Two Hundred Fifty Seven and Paisas Thirteen)

only (Per Hundred Sft)

| 4. | Excavation in foundation of buildings bridges and other structures including dag belling, dressing refilling around structures with excavated earth watering and ramming lead upto 5’ ft: (In sand, ashes, loose Soil) (G.I.S.No.18 (a) P/4). |
| --- | --- |

24779.0		@ Rs. 2722/50	P‰ Cft			Rs. 67,515/=

(Rupees Two Thousand Seven Hundred Twenty Two

and Paisas Fifty)  only (Per Thousand Cft)

| 5. | Cement concrete plain including placing, compacting, finishing and curing complete (Including screening and washing of stone aggregate without shuttering. (G.S.I. No. 5 P-16). |
| --- | --- |

(i) 	Ratio 1:4:8

4246.0 		@ Rs. 11288/75	P%Cft				Rs.4,79,320/=

(Rupees Eleven Thousand Two Hundred Eighty Eight

and Paisas Seventy Five)  only (Per Hundred Cft)

(ii) 	Ratio 1:2:4

2250.0 		@ Rs. 14429/25	P%Cft				Rs.3,24,658/=

(Rupees Fourteen Thousand Four Hundred Twenty

Nine and Paisas Twenty Five) only (Per Hundred Cft)

| 6. | C.C Brick or stone ballast ½" to 2" gauge (G.S.I. No. 4 P-15). |
| --- | --- |

2970.0 Cft	 	@ Rs. 9416/28	P% Cft				Rs.2,79,664/=

(Rupees Nine Thousand Four Hundred Sixteen

and Paisas Twenty Eight) only (Per Hundred Cft)

| 7. | Pucca brick work n foundation and plinth in cement sand mortar (1:6) (G.S.I. No. 4 (i)/e P-21). |
| --- | --- |

13013.0 Cft		 @ Rs. 11948/36	P% Cft				Rs.15,54,840/=

(Rupees Eleven Thousand Nine Hundred Forty Eight

and Paisas Thirty Six)  only (Per Hundred Cft)

| 8. | Construction of standard open drain comet block of cement concrete (1:2:4) in situ design profile i.e. cost of mould as per drawing i/c applying floating coat of cement 1/32" thick to the exposed face finished smooth curing etc complete as per detailed drawing. (P.H.S.I No. D P-58). |
| --- | --- |

Type-A

4000.0 Rft		@ Rs. 94/=		P.Rft				Rs. 3,76,000/=

(Rupees Ninety Four) only (Per Rft)

Type-B

1500.0 Rft		@ Rs. 174/=		P/Rft				Rs. 2,61,000/=

(Rupees One Hundred Seventy Four) only (Per Rft)

Type-C

500.00 Rft		@ Rs. 309/=		P/Rft				Rs. 1,54,500/=

(Rupees Three Hundred Nine) only (Per Rft)

| 9. | Cement Plaster (1:4) upto 12’ height 1/2” thick (G.S.I.No. 11 (b) P-52). |
| --- | --- |

20410.0 Sft 		@ Rs. 2283/93	P% Sft				Rs. 4,66,150/=

(Rupees Two Thousand Two Hundred Eighty Three

and Paisas Ninety Three) only (Per Hundred Sft)

| 10. | Erection and removal of centering for R.C.C or plain cement concrete works of partal wood (G.S.I.No. 19 (b) P-18). |
| --- | --- |

401.0 Sft 		@ Rs. 3127/41	P% Sft				Rs. 12,541/=

(Rupees Three Thousand One Hundred Twenty Seven

and Paisas Forty One) only (Per Hundred Sft)

| 11. | Reinforced cement concrete work including all labour and material except the cost of binding which will be paid separately. This rate also includes all kinds of forms moulds lifting shuttering curing rendering and fining washing of shingle (a) RCC work in roof slab beams columns rafts lintels and others structural members laid in situ or precast laid in position complete in all respects Ratio (1:2:4) 90 Lbs cement 2 Cft sand and 4 Cft shingle 1/8” to ¼" gauge (G.S.I. No.6) (a) Page No.16). |
| --- | --- |

901.0 Cft		@ Rs. 	337/=		P.Cft				Rs. 3,03,637/=

(Rupees Three Hundred Thirty Seven) only (Per Cft)

| 12. | Fabrication of mild steel reinforcement for cement concrete including cutting, bending laying in position making joints and fastening laying in position binding wire (also includes removal of rust from bars) (G.S.I. No.8 (b) P-17). |
| --- | --- |

32.18 Cwt		@ Rs. 5001/70	P.Cwt				Rs. 1,60,955/=

(Rupees Five Thousand One and Paisas Seventy)

only (Per Cwt)

| 13. | Re-handling of earth work lead upto 50 ft. (G.S.I.No. 9 (b) P-2) |
| --- | --- |

24799.0 Cft		@ Rs. 1058/75	P‰ Cft			Rs. 26,256/=

(Rupees One Thousand Fifty Eight and Paisas

Seventy Five) only (Per Thousand Cft)

| 14. | Earth work compacting (Soft ordinary or hard soil) Laying earth in 6" layers leveling, dressing and watering for compaction etc complete (G.S.I.No. 13 (b) P-3). |
| --- | --- |

11250.0 Cft		@ Rs. 354/=		P‰ Cft			Rs. 3,983/=

(Rupees Three Hundred Fifty Four) only

(Per Thousand Cft)

Total		Rs. 44,98,123/=

CONDITIONS

- The rates should be inclusive of all the Taxes i/c. Sales Tax, Octroi. Royalties etc, if any.

- No separate carriage will be allowed to the contractor.

- The Testing of the works/Pumping Machinery will be arranged by the contractor at his own cost, No separate payment will be made.

- No premium will be allowed on Non-Schedule items.

- Any error or omission in the specifications/items will be governed by the relevant specifications and schedule of rates.

- The decision of the Executive Engineer will be final and binding on all the parties in any dispute.

- The contractor will have to submit the certificate of manufacturer at the time of supply of Machinery.

| CONTRACTOR |  | EXECUTIVE ENGINEER PUBLIC HEALTH ENGG: DIVISION THARPARKAR @ MITHI |
| --- | --- | --- |