KARACHI METROPOLITAN CORPORATION

ENGINEERING DEPARATMENT

Tender Reference No.KMC/CM/ED/  114  /2015-16

BILL OF QUANTITIES

IMPROVEMENT / REPAIR OF FLAT NO.9 BAHADUR SHAH MARKET

UC-04, DISTRICT SOUTH.

NOTE: The Condition of contract will be available on KMC website (www.karachicity.gov.pk).

| Estimate Cost: - | Rs.8,78,000/= |
| --- | --- |
| Bid Security: - | Rs.17,600/= |
| Tender Cost: - | Rs.2,500/= |

BIDDING DATA

(This section should be filled in by the Engineer/Procuring Agency before issuance of the Bidding Documents).

| a) | Name of Procuring Agency: | Karachi Metropolitan Corporation(KMC). |
| --- | --- | --- |
| b) | Brief Description of Works: | Improvement / Repair of Flat No.9 Bahadur Shah Market UC-04, District South. |
| c) | Procuring Agency Address: | 4th Floor Civic Centre, Engineering Department KMC, Karachi. |
| d) | Estimated Cost: | Rs.8,78,000/= (Tender Cost Rs.2,500/=) |
| e) | Amount of Bid Security: | Rs.17,600/= (2%) |
| f) | Period of Bid validity(days): | 90 days. |
| g) | Security Deposit (Including bid security): | 10% of the total cost (2% of bid amount plus 8% from the bill). |
| h) | Percentage, if any, to be deducted from bills: | 8% |
| i) | Deadline for submission of bid along with time: | As per NIT |
| j) | Venue, Time and date of bid Opening: | Chief Engineer (South), KMC Room No. C-513, 5th Floor, Civic Centre, Gulshan-e-Iqbal, Karachi 16-05-2016 at 2.30 P.M. |
| k) | Liquidity damages: | Rs.2,500/= per day |
| l) m) | Method of Procurement: Deposit Receipt No Date & Amount: | Single Stage One Envelope Method. ___________________________ |

MINIMUM QUALIFICATION / ELIGIBILITY CRITERIA

The evidence / documents of the following minimum qualification / eligibility criteria will be checked during opening process of tender & if anyone is missing then the tender will be summarily rejected at the moment by the tender opening committee.

Eligibility / Minimum Qualification Criteria:

- NTN Certificate.

- Valid Professional TAX.

- Registered with Sindh Revenue Board.

- Statement of Turnover at least 3 years with minimum amount of Rs.1.0 Million.

- Having minimum Experience of 3 years in relevant field.

- Bid Security, as mentioned in the NIT & Bidding Documents, is furnished.

- Proof of previous experience of works of similar nature.

Signature of the Bidder with Stamp

KARACHI METROPOLITAN CORPORATIN

ENGINEERING DEPARTMENT

| Subject:- | IMPROVEMENT / REPAIR OF FLAT NO.9 BAHADUR SHAH MARKET UC-04, DISTRICT SOUTH. |
| --- | --- |

Estimated Cost:  Rs.8,78,000/=

Earnest Money:  Rs.17,600/=

Tender Cost:       Rs.2,500/=

| S.No. | DESCRIPTION | QTY | UNIT | Rate (in figure) | Rate (in figure) | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- |
| 1. | Dismantling glazed or encaustic tiles etc. | 388 | % Sft | 786.50 | Seven Hundred Eighty Six & Paisa Fifty Only | 3,052 |
| 2. | Dismantling cement concrete plain 1:2:4. | 78 | % Cft | 3327.50 | Thirty Three Hundred Twenty Seven & Paisa Fifty Only. | 2,595 |
| 3. | Scraping ordinary distemper oil bonded border and walls. | 3819 | % Sft | 226.88 | Two Hundred Twenty Six & Paisa Eighty Eight Only. | 8,665 |
| 4. | Providing and laying 2” thick topping CC 1:2:4 including surface finishing and dividing into panels. | 1216 | % Sft | 3275.50 | Thirty Two Hundred Seventy Five & Paisa Fifty Only. | 39,830 |
| 5. | S/F of Fiber Plastic Tank of approved quality and design and wall thickness as specified i/c cost of nuts, bolts and fixing in plate form of cement concrete 1:3:6 and making connections for in-let and out-let & over flow pipes etc complete (a) (250 Gallons) wall thickness 3.5 mm. | 2 | Each | 21989.61 | Twenty One Thousand Nine Hundred Eighty Nine & Paisa Sixty One Only. | 43,979 |
| 6. | P/F squinting type white glazed earthen ware W.C pan with i/c the cost of flushing cistern with internal fitting and flush pipe with bend and making requisite numbers of holes in wall plinth & floor for pipe connection & making good in cement concrete 1:2:4 W.C. pan of not less than 23” clear opening between flushing rims and 3 gallons flushing tank with 4” C.I trap. | 2 | Each | 5728.80 | Fifty seven Hundred Twenty Eight & Paisa Eighty Only. | 11,458 |
| 7. | P/F European white glazed earthen ware wash down W.C pan complete with and i/c the cost of white/black plastic seat (Best quality) and lid with C.P brass hinges and buffers, 3 gallons white glazed earthen ware low level flushing cistern with siphon fitting 1 ½” dia white porcelain enameled flush bend ¾” dia and making requisite number of holes in walls, plinth and floor for pipe connections and making god in cement concrete 1:2:4 (Foreign quality or equivalent). | 1 | Each | 5339.40 | Fifty Three Hundred Thirty Nine & Paisa Forty Only. | 5,339 |
| 8. | P/F 22" x 16" lavatory basin in white glazed earthen ware complete with & i/c the cost W.I. cantilever bracket 6" built in to walls, painted white in two coats after a primary coat of red load paint a pair of ½" Dia chrome platted pillar taps. 1-½" Dia rubber plug and chrome brass waste of approved pattern 1-½" dia malleable iron c.p brass union making and making traps, malleable iron and brass requisite number of holes in wall plinth & floor for pipe connection & making good in cement concrete 1:2:4 (standard pattern). | 2 | Each | 6237.00 | Sixty Two Hundred Thirty Seven Only. | 12,474 |
| 9. | Add extra labor for providing and fixing of earthen ware pedestal white or colored glazed (Foreign or equivalent). | 2 | Each | 938.47 | Nine Hundred Thirty Eight & paisa Forty Seven Only. | 1,877 |
| 10. | P/F steel stainless local make complete with cast iron or wrought iron brackets 6" built in wall 1-½" rubber plug chrome plated brass chain 1-½" C.P. brass waste with 1 ½” PVC waste pipe and making requisite number holes in walls & plinth and floor for pipe connection and making good in cement concrete 1:2:4 (a) steel sink stainless size 40" x 20” local make (Standard Pattern). | 1 | Each | 5052.30 | Five Thousand Fifty Two & Thirty Only. | 5,052 |
| 11. | Providing, Laying UPVC pipes of class ‘D’ fixing in trench i/c cutting, fitting and jointing with solvent cement i/c testing with water to a head of 122 meter or 400 ft ¾” thick. | 85 | P/Rft | 19.00 | Rupees Nineteen Only. | 1,615 |
| 12. | S/F long bib cock of crystal head with ½” dia. | 4 | Each | 1384.24 | Thirteen Hundred Eighty Four & Paisa Twenty Four Only. | 5,537 |
| 13. | Supplying fixing wash basin mixture of superior quality with C.P head ½” dia. | 2 | Each | 3179.00 | Thirty One Hundred & Seventy Nine Only. | 6,358 |
| 14. | Supplying & fixing C.P Muslim Shower with Crystal head etc complete. | 2 | Each | 3432.00 | Thirty Four Hundred Thirty Two Only. | 6,864 |
| 15. | Supplying & fixing jet shower with rod of superior quality single C.P head ½” dia. | 2 | Each | 1142.24 | Eleven Hundred Forty Two & Paisa Twenty Four Only. | 2,284 |
| 16. | First class teak wrought joinery in wire gauze doors windows with frames and 22 standard galvanized wire gauze 144 mesh per square inch including brass fittings complete (b) Paneled and glazed or fully glazed 1 ½” thick. | 33 | P/Sft | 1227.36 | Twelve Hundred Twenty Seven & Paisa Thirty Six Only. | 40,503 |
| 17. | S/F in position aluminum channels framing for sliding windows & ventilators of Alcop made with 5mm thick tinted glass glazing (Belgium ) & Aluminum fly screen i/c handles stoppers and locking arrangement etc. | 112 | P/Sft | 1647.69 | Sixteen Hundred Forty Seven & Paisa Sixty Nine Only. | 1,84,541 |
| 18. | Making & fixing steel grated door with 1/16” thick sheeting i/c angle iron frame 2”x2’ 3/8” and ¾” square bars 4” centre to centre with locking arrangement. | 28 | P/Sft | 726.72 | Seven Hundred Twenty Six & Paisa Seventy Two Only. | 20,348 |
| 19. | Supplying & fixing in position iron / steel grill of ¾” x ¼” size flat iron of approved design i/c painting 3 coats etc complete (weight not to be less than 3.7 Lbs / Square Foot of finished grill). | 152 | P/Sft | 180.50 | One Hundred Eighty & paisa Fifty Only. | 27,436 |
| 20. | Laying mosaic topping ½” thick in panels of approved any pattern of one part of white cement to two parts of marble chips of required shade and size and of approved. | 58 | % Sft | 11291.99 | Eleven Thousand Two Hundred Ninety One & Paisa Ninety Nine Only. | 6,549 |
| 21. | P/L tiles glazed 6”x6”x ¼” on floor or wall facing in required color and pattern of STILE specification jointed in white cement and pigment over a base of 1:2 grey cement mortar ¾” thick including washing and filling of joints with slaury of white cement and pigment in desired shape with finishing clearing and cost of wax polish etc complete including cutting tiles to proper profile. | 388 | % Sft | 30,509.77 | Thirty Thousand Five Hundred Nine & Seventy Seven Paisa Only. | 1,18,378 |
| 22. | Providing & laying Porcelain tiles on floor or wall facing in required color specification jointed in white cement and pigment over a base of 1:2 grey cement mortar 1” thick i/c washing and filling of joints with slaurry of white cement and pigment in desired shape with finishing, cleaning and cost of wax polish etc complete i/c cutting tiles to proper file (600 cm x 600 cm) Italian make or equivalent). | 547 | P/Sft | O/R |  |  |
| 23. | Providing and fixing deodar Almirah 9”-12” depth including boxing with back shelves shutters brass fittings complete. | 80 | P/Sft | 1778.50 | Seventeen Hundred Seventy Eight & paisa Fifty Only. | 1,42,280 |
| 24. | Distempering three coats. | 1144.50 | % Sft | 1079.65 | One Thousand Seventy Nine & Paisa Sixty Five Only. | 12,356 |
| 25. | Preparing the surface & painting with matt finish paint of approved make to old matt finish surface 2nd & subsequent coat. | 2655.00 | % Sft | 1989.62 | Nineteen Hundred Eighty Nine & Paisa Sixty Two Only. | 52,824 |
| 26. | Preparing surface and painting with emulsion paint. | 426.00 | % Sft | 1772.38 | Seventeen Hundred Seventy Two & Paisa Thirty Eight Only. | 7,550 |
| 27. | Providing & fixing 6”x2” or 6”x3” C.I floor trap of the approved site cleaning design with a C.I screwed down grating with or without a vent arm complete with & i/c making requisite number of holes in walls, plinth & floor for pipe connections & making good cement concrete 1:2:4. | 2 | Each | 2042.43 | Two Thousand Forty Two & Paisa Forty Three Only. | 4,085 |
| 28. | Providing & fixing 6”x4” C.C gully trap with 4” outlet complete with 4” thick 1:2:4 C.C for bed & ½” thick cement plaster (1:3) to the kerb, C.I grating 6”x6” & C.I cover and frame 12”x12” (inside) etc complete C.C Gully trap 6”x6”x4’ (i) with C.I cover & frame. | 2 | Each | 1358.17 | Thirteen Hundred Fifty Eight & Paisa Seventeen Only. | 2,716 |
| 29. | Providing / fixing approved quality Mortice lock. | 4 | Each | 1786.13 | Seventeen Hundred Eighty Six & Thirteen Paisa Only. | 7,145 |

KARACHI METROPOLITAN CORPORATION ENGINEERING DEPARTMENT

| Subject: | IMPROVEMENT / REPAIR OF FLAT NO.9 BAHADUR SHAH MARKET UC-04, DISTRICT SOUTH. |
| --- | --- |

| Estimated Cost:- | Rs. 8,78,000/= |
| --- | --- |
| Bid Security:- | Rs. 17,600/= |
| Tender Cost: - | Rs.2,500/= |

I / We hereby quoted as follows:

| 1. | PART – A: (Items Based on S/R) amounting to Rs.7,83,690/= ________% Above / Below / At Par the S/R. | Rs.____________________ |
| --- | --- | --- |
| 2. | PART – B: (Items Based on O/R) | Rs.____________________ |
| TOTAL A+B |  | Rs.____________________ |

The total amount is Rs.__________________ (Rupees_____________________________

(in Figure)						(In Words)

______________________________________________________ for complete job.

I / We have attached a pay order bearing No. _____________dated ____________issued from

____________________________________ amounting to Rs. ______________ as per NIT.		(Name of Bank)

Completion Time : 15 Days

Penalty per day: Rs. 2,500/=

Note:

- All rates quoted including the total amount of the bid shall be in figures & words (both).

- All corrections / overwriting shall be clearly re-written with initials & duly stamped by the bidder.

- The bid shall be properly signed, named & stamped by the 	authorized person of the firm and authorization letter for signatory shall be enclosed with the tender by the authorized Person, if other than the signatory of the firm.

- All prevailing rules regarding condition of contract will be applicable.

- 20% below is the maximum ceiling on schedule items 2012 by the authority (On item No. 17,18,22 & 24).

Signature of the Contractor with Stamp

Address: _________________________

_________________________________

Checked By:-