| NAME OF WORK: | M&R TO CITY COURT BUILDING REPAIRING/ MAINTAINANCE GENERAL LAVATORY FOR LAWYERS AT KARACHI BAR IN/C ADDITIONAL PARTITION FOR COURTS (DISTRICT WEST) FLOORING WORK, ALUMINUM DOORS & WINDOWS BLOCK MASONARY COLORING PAINTING & SANITARY WORK ETC . |
| --- | --- |

SCHEDULE”B”

| S/No | Description | Qty | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
|  | PART-A-I(SCHEDULE ITEM). |  |  |  |  |
| 1. | Excavation in foundation of building bridges and other structures including dagbelling dressing, refilling around structure with excavated earth watering and ramming lead upto 5 ft. (c) In hard soil or soft murum(18-c/4). | 110.25 Cft. | 3554/38 | %oCft. | 392/- |
| 2. | Cement concrete brick or stone ( 1:4:8) (4-b/15). | 20.79 Cft. | 9416/28 | %Cft. | 1958/- |
| 3. | Providing and laying 1:3:6 cement concrete solid block masonary wall 6” and below in thickness set in 1:6 cement mortar in ground floor super structure including raking out joints and curing etc complete (24/19). | 329.59 Cft. | 15771/01 | %Cft. | 51980/- |
| 4. | Cement plaster 1:4 upto 12” height (c) ¾” thick. ( 11-c/52). | 802.88 Cft. | 2590/50 | %Sft. | 20799/- |
| 5. | Providing and fixing G.I frames chowkats of size 7”x2” or 4 ½ x3” for door using 20 gauge G.I sheet i/c welded hings and fixing at site with necessary hold fasts filling with cement sand slurry of ration 1:6 and repairing the jambs the cost also i/c all carriage tools and plants used in making and fixing ( 29/93). | 34.75 r ft. | 228/90 | P.Rft. | 7954/- |
| 6. | Distempering (b) two coats. ( 24-b/54). | 348.66 Cft. | 1043/90 | %Sft. | 3640/- |
| 7. | Preparing the surface and painting with plastic emulsion of approved make (old surface ) (b) 2nd & subsequent coat. ( 41-a+b+b/56). | 2396.32 Sft. | 1980/73 | %Sft. | 47465/- |
| 8. | Painting with enimal paint on masonay wall (30A+b+b/73). | 581.10 Sft. | 2006/40 | %Sft. | 11659/- |
| 9. | Painting old surface © painting doors and windows any type(i) First coat (ii) Each Subsequent coat. ( 4-c(i+ii)/68.) | 613.0 sft. | 1160/06 | %Sft. | 7111/- |
| 10. | Preparing the surface and applying rock wall/ shield (Natural wall texture) coating to provide drable curse to wall, thickness B/W 2 mm to 32 mm (1/8” ) with acrylic copolymer emulsion, selecsted marble chipps adhesive and bectericdes , water resistance and fire and termiter resistence up to 20 “ hieht. (43/56). . | 849.59 Sft. | 4504/50 | %Sft. | 38270/- |
| 11. | Providing and laying 3” thick topping cement concrete 1:2:4 including surface finishing and dividing into panels(d) 3” thick . ( 16-d/42). | 105.0 Sft. | 4411/82 | %Sft. | 4632//- |
|  |  |  |  | Rs. | 195860/- |
|  |  |  |  | Total A-I Rs. |  |
|  | PART-A-II SHEDULE ITEM . |  |  |  |  |
| 1. | Providing and fixing in position doors, windows and ventilators of first class deodar wood frames, and 1 ¾” thick commercial ply veneer shutter of first class deodar skeleton (Hollow) and commercial ply wood (3 ply) on both sides including hold fasts hinges iron tower bolts handles and cleats with cord and one mortice lock and hooks(9/58(-) 25-b/61). | 44.87 Sft. | 856/53 | P.Sft. | 38432/- |
| 2. | Providing and laying tiles glazed 6”x6”x1/4” on floor or wall facing in required color and pattern of stile specification jointed in white cement and pigment over a base of 1:2 grey cement mortar ¾” thick including washing and filling of joints with slaurry of white cement and pigment in desired shape with finishing clearing and cost of wax polish etc complete including cutting tiles to proper profile. (47/60 ). | 1505.76 Sft. | 30509/77 | %Sft. | 459404/- |
| 3. | Supplying and fixing inposition aluminum channels framing for hinged doors or alcop made with 5mm thick tinted glass glazing (Belguim) and alpha (Japan) locks i/c handles , stoppers etc (b) Deluxe model (Bronze). (83-b/108). | 100.0 Sft. | 1507/66 | P.Sft. | 150766/- |
| 4. | Supplying and fixing inposition aluminum channels framing for slidding windows and ventilators of alcop made wth 5mm thic tinted glass glazing (Belguim) etc complete (b) Deluxe model ( Bronze) (85-b/108). | 13.50 Sft. | 1449/69 | P.Sft. | 19570/- |
|  |  | Rs. |  |  | 668172/- |
|  |  | Total A-ii Rs. |  |  |  |
|  | PART-A-III NON SCHEDULE ITEM. |  |  |  |  |
| 1. | Providing and laying matt/ glazed tiles make ( master / sonic) size (16”x16”x1/4”) on floor to required pattern and design i/c with cement sand mortar ¾” thick 1:6 ration finishing cleaning etc complete. | 176.75 Sft. |  | P.Sft. |  |
| 2. | Providing and fixing partition with fixed glass (frosted) 5mm thick using 4” lucky section in champion color as frame on floor or block masonary fixed with necessary fixtures rubber packing etc the cost of in/c tools and plants used in making and carriage from shop to site as directed by the Engineer incharge. | 141.30 Sft. |  | P.sft. |  |
| 3. | Providing and fixing precast slab 1 ½” thick with cement and mortar ratio 1:2: 4 finishing curing etc complete i/c the cost of carriage and labour charges upto 50’ height. | 108.0 Sft. |  | P.Sft. |  |
|  |  |  | Total A-III Rs. |  |  |
|  | PART-B-I SCHEDULE ITEM. |  |  |  |  |
| 1. | Providing and fixing orisa type white or color glazed earthen ware w.c pa with cost of low level plastic flush tank of 3 gallon capacity of approved quality i/c making requisite number of holes in wall plinth and floor and making good in cement concrete 1:2:4 (ii)with 4” dia earthen ware trap and plastic thumble. ( 3-a-ii/2). | 4-Nos . | 6166/60 | Each. | 24666/- |
| 2. | Providing and fixing European white glazed earthen ware wash down w.c pan complete with and i/c the cost of white/ black plastic seat (best quality) and lid with c.p brass hinges and buffers, 3 gallon white glazed earthen earthen ware low level flushing cistern with siphon fitting 1 ½” dia white porcelain enameled flush bend ¾” dia and making requisite number of holes in walls plinth and floor for pipe connections and making good in cement concrete 1:2:4 (Foreign quality). ( 5/2). | 2-Nos | 11477/40 | Each. | 22955/- |
| 3. | Providing and fixing 24”x18” lavatory basin in white glazed earthen ware complete wth and i/c the cost of W.I or C.I cantilever brackets 6 inches built into walls, painted white in two coats after a primary coat of red lead paint a pair of ½” dia rubber plug and chrome plated brass chain 1-1/4” dia malleable iron or brass unions and making requisite number of holes in walls plinth and floor for pipe connection and making good in cement concrete 1:2 :4 (foreign or equivalent) ( 10/3). | 4-Nos . | 4928/- | Each. | 19712/- |
| 4. | Add extra for labour for providing and fixing of earthen ware pedestal white or coloured glazed (Foreign or equivalen). ( 11/3). | 4-Nos . | 2533/47 | Each. | 10134/- |
| 5. | P/F flat back lipped from urinal basin Felt back urinal basin ( of not less tan 17” in height) of white glazed eathern ware complete with and i/c the cost of 1 gallon C.I automatic flushing cistern with fittings a pot cock C.I or W.I brackets standard flush pipe with fitting etc . (7/3). | 4-Nos | 4440/70 | Each. | 17763/- |
| 6. | S/Fixing cancealed tee stop cock of superior quality with c.p head ½” dia ock. ( 12-b/18). | 14-Nos | 889/46 | Each. | 12452/- |
| 7. | Supplying and fixing c.p muslim shower with double bib cock and ring pipe etc complete. ( 19-a/19). | 5-Nos . | 3432/- | Each. | 17160/- |
| 8. | Supplying and fixing lonf bib cock of superior quality with c.p head ½” dia. ( 13-a/19). | 4-Nos . | 1109/46 | Each. | 4438/- |
| 9. | Supplying and fixing bathroom accessories set 7-pieces i/c towel rod , brush holder soap tray shelf of approved quality and design etc complete. ( 22/19). | 01-No | 8122/40 | Each. | 8122/- |
| 10. | P/F 24”x18” beveled edge mirror of belguim glass complet with 1/8” thick hard board and c.p screws fixed to wooden pleat(3-a/7). . | 5-Nos . | 1711/60 | Each. | 8558/- |
| 11. | P/F manufacturing and supply 21” RCC mainhole cover (1-31). |  |  |  |  |
| a). | 21” dia . | 15-Nos | 913/63 | Each. | 13704/- |
| b). | 24” dia. | 10-Nos. | 1051/25 | Each. | 10513/- |
|  |  | Rs. |  |  | 170177/- |
|  |  | Total B-I Rs. |  |  |  |
|  | PART-B-II NON SCHEDULE ITEM. |  |  |  |  |
| 1. | Providing and fixing granite marble top ¾” thic as approved color make and design including making edges with round shape and chemical polishing including the cost of material labour wetage and cartage etc complete. | 19.0 Sft. |  | P.sft. |  |
| 2. | Providing and fixing Exhaust fan plastic body best quality complete in/c electric connection as pre required. | 2-Nos . |  | Each. |  |
| 3. | Supply and fixing UPVC pipe make (Pak Arab) (Sch- 40-E) jointed with best quality solution make (EZ) in/c all cost of material labour T & P etc complete. |  |  |  |  |
| i). | 4” dia . | 49.66 Rft. |  | P.Rft. |  |
| ii). | 1” dia. | 68.0 Rft. |  | P.Rft. |  |
| iii) | ½” dia . | 49.75 Rft. |  | P.R ft. |  |
| 4). | Providing and fixing UPVC fitting make (Pak Arab) sch 40-E) jointing with best quality solution make (EZ) in/c all cost of labour material and T & P . |  |  |  |  |
| i). | 4” dia tee. | 2-Nos . |  | Each. |  |
| ii). | 4” dia elbow. | 4-Nos . |  | Each. |  |
| iii). | 1” dia Tee | 6-Nos . |  | Each. |  |
| iv). | 1” dia elbow. | 8-Nos . |  | Each . |  |
| v). | ½” dia tee. | 8-Nos |  | Each. |  |
| vi). | ½” dia elbow. | 21-Nos |  | Each. |  |
|  |  |  | Total B-ii Rs. |  |  |
|  |  |  | G.Total (A+B) Rs. |  |  |

TERMS AND CONDITION

1. 	Any typographical  errors in the schedule ‘B’ are subjected to any correction with the 	reference to the schedule of Rates General item 2012 in force from `12-07-2012 as 	approved by the standing rates committee Sindh Karachi.

2.	100% well graded crushed bajri shall be used in item of RCC 1:2:4.

3.	No Premium shall be paid on Non schedule items.

4.	The work will be carried out as per PWD Specifications.

| Contractor. | --s/d-- ASSISTANT ENGINEER Provincial Building Sub Division-XI, Karachi | --s/d-- EXECUTIVE ENGINEER Provincial Building Division-No.III Karachi |
| --- | --- | --- |