BILL OF QUANTITIES

NAME OF WORK:-	 Providing Laying Jointing and testing PVC Pipe & Constt: of L.S.R for Water 			Supply Scheme Yameen Palari Taluka Thana Bula Khan District Jamshoro.

(A) 	Description and rate of item based on Composite schedule of Rates

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
|  |  | Group ‘A’ P/L/J Testing PVC Pipe 3”dia |  |  |  |
| 01 | 4080.00 Cft | Excavation for pipe lines into trenches and pits in soft soils i/c trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer incharge providing fence guards lights flags and temporary crossing for non vehicular traffic where ever required lift upto 5’ft(1.52m) and lead upto one chain (30.5mm) (P.H.S.I.NO; 4, P/61) | 3600/- | P%0cft | 14,688/- |
| 02 | 2040.00 Cft | Excavation for pipe lines into trenches and pits in Hard Rock i/c trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer incharge providing fence guards lights flags and temporary crossing for non vehicular traffic where ever required lift upto 5’ft(1.52m) and lead upto one chain (30.5mm) (P.H.S.I.NO; 6, P/62) | 3900/- | P%0cft | 7,956/- |
| 03 |  | Providing PVC pipe of class ‘B’ (equivalent make)fixing in trenches i/c cutting and jointing with “Z” joints with one rubber rings i/c testing with water to head of 61 meter or 200’ft.(P.H.S.I.NO;1, P/92) |  |  |  |
|  | 1020.0 Rft | 3” DIA | 90/- | P/Rft | 91,800/- |
| 04 |  | Supplying C.I. Sluice valve heavy pattern test pressure 300 lbs / sq; inch (P.H.S.I.NO;2, P/9) |  |  |  |
|  | 1 No; | 3” DIA | 4290/- | Each | 4,290/- |
| 05 |  | Air Valve double acting (P.H.SI.(M)6, P/12) |  |  |  |
|  | 1 No: | 3” DIA | 4000/- | Each | 4,000/- |
| 06 |  | P.V.C Z Jointing fittingI-18, (P.H.S.M.I-18, P/112) |  |  |  |
|  | 4 Nos: | i)Tee 3” dia | 900/- | Each | 3,600/- |
|  | 2 Nos: | ii)900 Bend 3” dia | 731/- | Each | 1,462/- |
|  | 20 Nos: | iii)Socket 3” dia | 405/- | Each | 8,100/- |

(cont: on page No:02)

(Page No:02)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 07 |  | Tail piece for PVC pipe (P.H.S(M)I-14, P/29) |  |  |  |
|  | 2 Nos: | 3” dia | 375/- | Each | 750/- |
| 08 |  | Making joints C.I. specials fitting (i/c laying of required diameter and testing the joint alongwith the asbestos cement pressure pipe line to the specified pressure and making good to all leaky joints etc: complete. (P.H.S.I.NO:1, P/56) |  |  |  |
|  | 20 Joints | 3” dia | 59/- | Each | 1,180/- |
| 09 |  | Jointing C.I/M.S flanged pipes and specials flanged and inside a trenches i/c supplying rubber packing of the required thickness nuts bolts with washers etc and other tools required for jointing and testing the joints to the specified pressure etc: complete.(P.H.S.I.NO;1, P/40)` |  |  |  |
|  | 4 Joints | 3” dia | 499/- | P/joint | 1,996/- |
| 10 | 2 Nos: | Providing chamber 3”x2” (915x615mm) inside dimension 4½”(1372 mm) deep as per approved design for sluice valve 3” to 12” dia with 18” (457mm) dia inside cast iron cover and frame (wt= 1 cwt 3 qr) fixed in RCC 1:2:4 (102 mm) thick with 5 lbs steel per cft)9” (299 mm) thick brick masonary wall set in 1:6 cement mortar 6” (1152mm ) thick cement concrete 1:3:6 in foundation 1” thick cement concrete 1:2:4 flooring ½” (12.5mm) thick cement plaster 1:3 to all inside wall surface and to top i/c providing and fixing M.S. foot beyond 2½” depth curring excavation back filling and disposal of surplus earth etc; complete (P.H.S.I.NO:1, P/49) | 18820/- | Each | 37,640/- |
| 11 | 5508.00 cft | Refilling the excavated stuff in trenches 6” thick layers i/c watering ramming to full compaction etc: complete (P.H.S.I.NO;24, P/53) | 2760/- | P%0cft | 15,202/- |
|  |  |  |  | Total | 192,664/- |
|  |  | GROUP ‘D’ CONSTT; OF L.S.R |  |  |  |
| 01 | 241.00 cft | Excavation in foundation of building bridges and other structures including dabbling dressing refilling around structures with excavated earth watering and ramming lead upto 5’ft (in ordinary soil)(GSINO;18. P/4) | 3172/25 | P%0cft | 765/- |
| 02 | 57.00 Cft | Cement concrete brick or stone ballast 1½” to 2” guage (Ratio 1:4:8) (G.S.I.NO:4, P/17) | 9416/28 | P%cft | 5,361/- |

(Cont: on page No:03)

(Page No:03)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 03 | 56.46 Cft | Cement concrete plain i/c placing compacting finishing and curring complete (i/c screening and washing of stone aggregate without shuttering Ratio 1:3:6 (G.S.I.NO:5, P/18) | 12595/- | P%cft | 7,111/- |
| 04 | 159.00 Cft | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds, lifting, shuttering currying rendering and finishing the exposed surface)\ (including screening and washing of shingle)(a) R.C.C. Work in roof slab, beams, columns, rafts, lintels and other structural members laid in suite or precuts laid in position complete in all respect (i) Ratio 1:2:4) 90 lbs cement 2 cft sand 4 cft shingle 1/8” to ¼” gauge. (G.S.I.NO;6(a) (i)P/19) | 337/- | P/Cft | 53,583/- |
| 05 | 6.38 Cwt | Fabrication of mild steel reinforcement for cement concrete including cutting bending laying in position making joints and fastenings including cost of binding wire (also includes removal of rust from bars (G.S.I.NO;7(ii)(a)P/20) | 5001/70 | P/Cwt | 31,911/- |
| 06 | 60 Kg: | Providing and fixing C.I. Main hole cover with frame i/c cost of material (P.H.S.I.NO:1, P/23) | 140/- | P/Kg: | 8,400/- |
| 07 | 197.41 Cft | Randum rubble masonary i/c hammer dressing in foundation and plinth in cement sand motor (1:6) (G.S.I.NO;1(iv) P/26) | 17723/- | P%cft | 34,987/- |
| 08 | 583.40 Sft | Cement plaster (1:4) ¾” thick upto 20’ height (G.S.I.NO;11, P/52) | 2283/93 | P%sft | 13,324/- |
| 09 | 26.71 Cft | Filling and watering and ramming earth under floor with new earth (excavated from out side) lead upto one chain and lift upto 5’ft (G.S.I.NO:21, P/04) | 1512/50 | P%0cft | 40/- |
| 10 |  | Providing G,I, Pipe of heavy quality (S.S.O.M.I.NO;1, P/12) |  |  |  |
|  | 57 Rft | 2½” dia | 310/47 | P/Rft | 17,697/- |
|  | 12 Rft | ¾” dia | 95/79 | P/Rft | 1,149/- |
| 11 |  | Supply of G.I. Bend (S.S. .NO;7, P/87) |  |  |  |
|  | 2 Nos: | Bend 2½” dia | 720/- | Each | 1,440/- |
| 12 |  | Providing & fixing full way gun metal valve with wheels threaded or flanged ends with rubber washing (light pattern) (S.S.I.NO8(ii) P/17) |  |  |  |
|  | 2 Nos: | 2½” dia | 1225/84 | Each | 2,452/- |
| 13 |  | Supplying fixing ¾” dia brasis or bib cosk slaid piston (S.S.I.NO;1(ii)P/16) |  |  |  |
|  | 6 Nos: | ¾” dia | 348/92 | Each | 2,094/- |
| 15 | 1 No: | Providing and fixing C.I. ventilator weight not less then 90lbs once in roof of tank approved Design rain with water right joints including painting etc complete (PHSI.No.4 P-55) | 4870/- | Each | 4,870/- |

(Cont: on page no:04)

(Page  No:04)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 16 | 3.00 Kg: | Supply of dampo Padlo (S.O.M.INO:39, P/04) | 125/- | P/Kg: | 375/- |
| 17 | 31.83 Rft | Providing “Construction joint” in concrete work of” 9 wide corrugated PVC water stop with bulb i.e soldering cost of material and labour Etc: complete.(P.H.S.I.NO:2, P/56) | 86/- | P/Rft | 2,737/- |
| 18 | 7.65 Cft | Cement concrete plain i/c placing compacting finishing and Currying complete (i/c screening and washing of stone aggregate without shuttering (Ratio 1:2:4) (G.S.I.NO; 5(a) p/18) | 14429/25 | P%cft | 1,104/- |
| 19 | 12 Nos: | Manufacturing & supplying M.S. Flange of different dia made of M.S. Sheet of various thickness including cutting drilling holes and welding of flanged with M.S. pipe etc: complete (P.H.S.I.NO;1, P/32) | 945/- | P/Kg: | 11,340/- |
| 20 |  | Jointing C.I/M.S flanged pipes and specials flanged and inside a trenches i/c supplying rubber packing of the required thickness nuts bolts with washers etc and other tools required for jointing and testing the joints to the specified pressure etc: complete. (P.H.S.I.NO;1, P/40) |  |  |  |
|  | 03 Joints | 3” dia | 499/- | P/Joint | 1,497/- |
|  |  |  |  | Total | 202,275/- |

A  B   S   T   R   A   C   T

| Group | Schedule Item | Non Schedule Item | Total |
| --- | --- | --- | --- |
| ‘A’ Providing Laying Jointing and Testing PVC Pipe | 192,664/- | ------ | 192,664/- |
| ‘B’ Construction of L.S.R | 202,275/- | ------ | 202,275/- |
| Total | 394,939/- |  | 394,939/- |

Amount Total (a) + (b) Rs:___________________

________% above / below on the rates of CSR, Amount to be added / deducted on the basis

Total (b) of premium quoted ______________

Total (A) = a + b in works & Figures______________________________________________

Contractor 						 	EXECUTIVE ENGINEER

PUBLIC HEALTH ENGG:DIVISION

JAMSHORO