BILL OF QUANTITIES

NAME OF WORK:-Repair of work at  Water Supply Scheme Thana Ahmed 				    Khan (O/M)  Taluka Thana Bula Khan  District Jamshoro.

Description and rate of item based on Composite schedule of Rates

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
|  |  | PART ‘A’ RISING MAIN 4” DIA |  |  |  |
| 01 | 1260.0 Cft | Excavation for pipe lines into trenches and pits in soft soil i/c trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chainas directed by Engineer incharge providing fence guards lights flags and temporary crossing for non vehicular traffic where ever required lift upto 5’ft(1.52m) and lead upto one chain (30.5mm) (P.H.S.I.NO; 1, P/60) | 3600/- | P%0cft | 4,536/- |
| 02 | 1260.0 Cft | Excavation for pipe lines into trenches and pits in Hard soil i/c trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer incharge providing fence guards lights flags and temporary crossing for non vehicular traffic where ever required lift upto 5’ft(1.52m) and lead upto one chain (30.5mm)(P.H.S.I.NO; 2, P/62) | 3900/- | P%0cft | 4,914/- |
| 03 |  | Providing PVC pipe of class ‘B’ (equivalent make)fixing in trenches i/c cutting and jointing with “Z” joints with one rubber rings i/c testing with water to head of 61 meter or 200’ft.(P.H.S.I.NO;1,P/22) |  |  |  |
|  | 160.00 Rft | 6” dia | 259/- | P/Rft | 41,440/- |
|  | 160.00 Rft | 3” dia | 90/- | P/Rft | 21,600/- |
| 04 |  | C.I.Sluice valve heavy (Test pressure 21.0 Kg/sq:cm or 300 lbs/sq: inch (Imported)(M.S.I.NO;2(d)P/11) |  |  |  |
|  | 6 Nos: | 6” dia | 9360/- | Each | 56,160/- |
|  | 2 Nos: | 3” dia | 4290/- | Each | 8,580/- |
|  | 2 Nos: | 4” dia | 5460/- | Each | 10,920/- |
| 05 |  | P.V.C. ‘Z’ Joints fitting (P.H.S.(M)I-18, P/27) |  |  |  |
|  | 4 No: | PVC Bend 6”dia 900 | 1913/- | Each | 7,652/- |
|  | 3 Nos; | PVC Bend 3”dia 900 | 731/- | Each | 2,193/- |
|  | 3 Nos: | Socket 6” dia | 1575/- | Each | 4,725/- |
|  | 4 Nos: | Socket 4” dia | 1125/- | Each | 4,500/- |

(Cont: on page No:02)

(Page No:02)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 06 |  | C.I.Tail Piece for P.V.C. Pipe (P.H.S.(M)I-1, P/101) |  |  |  |
|  | 12 Nos: | 6” dia | 618/75 | Each | 7,425/- |
|  | 04 Nos: | 3” dia | 375/- | Each | 1,500/- |
|  | 02 Nos: | 4” dia | 487/50 | Each | 975/- |
| 07 |  | C.I. Gibault joint(P.H.S.I(M)-14, P/17) |  |  |  |
|  | 04 Nos: | 6” dia | 1062/50 | Each | 4,250/- |
|  | 02 Nos: | 4” dia | 731/25 | Each | 1,463/- |
|  | 04 Nos: | 3” dia | 568/75 | Each | 2,275/- |
| 08 |  | G.I. Pipe of approved quality to be teted with pressure head of 200’ft (S.S.I.NO;1, P/125) |  |  |  |
|  | 20.0 Rft | 4” dia | 431/93 | Each | 8,639/- |
|  | 20.0 Rft | 6” dia | 695/04 | Each | 13,901/- |
| 09 | 140 Kg: | C.I. Flange with holes i/c trimming and facing for all size (Sch: of M.1, P/11) | 120/- | p/Kg: | 16,800/- |
| 10 |  | Jointing C.I/M.S flanged pipes and specials flanged and inside a trenches i/c supplying rubber packing of the required thickness nuts bolts with washers etc and other tools required for jointing and testing the joints to the specified pressure etc: complete. P.H.S.I.NO;1, P/35) |  |  |  |
|  | 4 Joints | 4” dia | 513/- | P/Joint | 2,052/- |
|  | 4 Joints | 6” dia | 938/- | P/Joint | 3,752/- |
| 11 | 1 Job | Repairing of panal board i/c supply of wire cutting fitting over haling etc: complete (M.R) | 15000/- | p/Job | 15,000/- |
| 12 | 25.00 Cft | Making foundation of cement concrete plain i/c placing compacting finishing and curring i/c washing of stone aggregate without shuttering (Ratio 1:2;4) (GSI.NO;5,P/16) | 14429/25 | P%Cft | 3,607/- |
| 13 | 7.50 Cft | Cement concrete brick work or stone ballast 1½’ to 2’ guage (Ratio 1:4:8) (GSI.NO; P/ ) | 9416/28 | P%Cft | 706/- |
| 14 | 40.0 Rft | PVC insulated and PVC Shated four core coper conductor 600/1000 volts 25mm2 (E.S.I.NO;102, P/12) | 396/34 | P/Rft | 15,854/- |
| 15 | 2 Jobs | Rewinding of electric motor 20 BHP i/c supplying installing etc: complete (GSI.NO;97,P/110) | 19825/- | P/job | 39,650/- |
| 16 | 2268.0 Cft | Refilling the excavated stuff in trenches 6” thick layers i/c watering ramming to full compaction etc: complete (P.H.S.I.NO;24, P/53) | 2760/- | P%0cft | 6,260/- |
| 17 | 1 No; | Providing & fixing automatic star delta starter model No:ASD-5) (N.S.I) | 48750/- | Each | 48,750/- |
|  |  |  |  | Total | 360,079/- |

(Cont: on page no:03)

(Page No:03)

Amount Total (a) + (b) Rs:___________________

________% above / below on the rates of CSR, Amount to be added / deducted on the basis

Total (b) of premium quoted ______________

Total (c ) offered for Non-schedule items_________

Total (A) = a + b in works & Figures__________________________________________

+

Contractor 						 	EXECUTIVE ENGINEER

PUBLIC HEALTH ENGG:DIVISION

JAMSHORO