biLL of quantity

Name of Work:	construction of  rural water supply scheme village  wahid bux khoso,  village ghulam qadir lashari, village bilawal khan noon, 7 village shahbaz khan kandrani  taluka thull,  districrt jacobabad

village wahid bux khan khoso taluka thull

construction of pump house for tube well

| Sr. No: | Item of Work | Qty: | Rate | Unit |  | Amount |
| --- | --- | --- | --- | --- | --- | --- |
| 01 | Excavation in foundation of building bridges and other structures i/c bailing, dressing, filling around the structure with excavated earth, watering, ramming Lead up to one chain & lift up to 5.0' (CSI No:18 P4) 285.00 | 140.00 | 3,176.05 |  | P %oCft | 445.00 |
| 02 | Cement Concrete bricks or stone ballast 1 1/2" to 2" gauge ratio 1:4:8 11(CSI No:4 P15) | 52.35 | 9,416.28 |  | P %Cft | 4929.00 |
| 03 | Pucca Brick work in foundation & plinth in cement sand mortar 1:6 (CSI No:4 P21) | 213.42 | 11948.36 |  | P %Cft | 25500.00 |
| 04 | Damp proof course with (Cement Sand & Shingle concrete 1:2:4) including two coats of asphaltic mixture 3" thick (CSI.No.28,P-19) | 39.55 | 4,982.18 |  | P %Sft | 1970.00 |
| 05 | Pucca Brick work in ground floor i/c striking of joints in cement sand mortar 1:6 (CSI No:5 P21) | 233.25 | 12,674.36 |  | P %Cft | 29563.00 |
| 06 | Maing and fixing steel grated door complete with locking arrangement angle iron frame 2’x2’x3/8’ 7 ¾’ sq bars 4’ TO CENTER9CSI NO.23 P92. | 22.75 | 726.72 |  | P.Sft | 16533.00 |
| 07 | Fabrication of mild steel reinforcement for cement concrete i/c cutting bending laying in position making joints and fastening i/c cost of binding wire Also includes removal of rust from bars. | 2.66 | 5001.70 |  | P. Cwt | 13304.00 |
| 08 | RCC work in roof slabs beams columns rafts lintels and other structural members laid in situ are precast laid in position etc complete in all respects ratio 1:2:4 {cement, sand, shingle 1/8" to 1/4"} (CSI 6 a P-16 & 17) | 66.21 | 337.00 |  | P.cft | 22313.00 |
| 09 | Filling watering ramming in floor surplus earth from foundation lead up to one chain and lift up to 5 ft: (CSI No.21 P-4) | 184.06 | 1512.50 |  | P%oCft | 278.00 |
| 10 | Providing Laying 3" thick CC Topping 1:2:4 i/c surface finishing and dividing into panels (CSI.NO.16 P-42) | 56.25 | 4411.82 |  | P% Sfr | 2482.00 |
| 11. | Cement plaster 1:4 upto 20' height 1/2" thick (CSI No:11 P-52) | 665.25 | 2,283.93 |  | P%Sft | 15194.00 |
| 12 | Fabrication of heavy steel reinforcement for cement concrete i/c cutting bending laying in position making joints and fastening i/c cost of binding wire also includes removal of rust from bars (CSI | 1.01 | 4928.49 |  | Cwt | 4978.00 |
| 13 | Preparing new surface and painting of doors & window any type i/c edge (CSI.NO.5 C P-70) Two coats | 69.50 | 1,489.68 |  | P %sft | 1035.00 |
| 14 | White washing three coat(CSI.NO.26 P-59) | 64.00 | 829.95 |  | P %Sft | 531.00 |
| 15 | White washing one coat(CSINO25P-59) | 665.25 | 416.63 |  | P%Sft | 2772.00 |
| 16 | Color Washing two coats (CSI.NO.25 P-5 | 665.25 | 859.90 |  | P %Sft | 7520.00 |
|  |  |  |  |  | Total | 1,47,548.00 |

TUBE  WELL 1 NO

| 01 | Boring for Tube Wells in all Water bearing soil from ground level up to 100' or 30.50 M Depth i/c sinking & withdrawing of casing pipe(PHSI No.1 P-No.36) 15”dia | 56.00 | 877.00 | P. Rft | 49112.00 |
| --- | --- | --- | --- | --- | --- |
| 02 | Providing Strong substantially built wooden locked box with compartments for preserving 2 Lbs (1 kg) samples strata’s (PHSI No.5 P-No.36) | 1 | 3530.00 | P No. | 3530.00 |
| 03 | Taking & preserving in Box Pipe B 2 Lbs (1kg) sample of strata’s from bore hole (PHSI No.6 P-No.36) | 2.00 | 101/00 | P sample | 202.00 |
| 04 | Taking sample of water from bore hole and placing in two separate bottles (PHSI No. 7 P No. 37) | 2.00 | 131/- | P.Sample | 262.00 |
| 05 | Supplying & Installation PVC Blind Pipe B Class of approved Design quality & make i/c necessary sockets etc Complete 12" Dia (PHSI No.12 P-44)) | 26.00 | 1329.40 | P.Rft | 34565.00 |
| 06 | Supplying & Installation PVC Strainer B Class of approved Design quality & make i/c necessary sockets etc complete 8" dia (RA Attached Based on Schedule) 12” dia | 30.00 | 1439.75 | P.Rft | 43193.00 |
| 07 | Supplying and fixing M.S bail plug (PHSI NO.8P-43) 12” dia | 1 | 2607.00 | Each | 2607.00 |
| 08 | Shrouding with fine graded bajri (3/8" to 1/8" or 9mm to 3mm in between bore and blind pipe of the following diameter of strainer (PHSI No.13 P no.38) 12” dia | 56.00 | 193.00 | P. Rft | 10808.00 |
| 09 | Supplying and installing PVC end plug 8” dai(PHSI NO. 8 P-43) 12” dia | 1.00 | 2200.00 | Each | 2200.00 |
| 10 | Development of tube well | 56.00 | 200/- | P.Rft | 11200.00 |
|  |  | TOTAL |  |  | 1,57,678.00 |

pupming machinary new 12 bhp deiasel  oil  engine  for tube well

| 01 | Supplying and Installing in position i/c transportation to site of work for Rural Water Supply Scheme Village Wahid Bux Khoso Tauka Thull District Jacobabad Diesel Engine set consisting of Diesel Oil Engine Golden made of 16.0 BHP RPM 2900 or equivalent type coupled with KSB Centrifugal Pump (ETA 40-200) Size 2 x 11/2" capable of Discharge 50 gallons per minute against a head of 100 ft. i/c frame & C.I pully with belt etc Complete i/c installation of pumping set on C.C foundation Ratio (1:2:4) and Ratio (1:4:8) with ballast with M.S nuts and bolts of 5/8 dia to be embedded in C.C foundation accommodate man or sub mains i/c providing and fixing Mattel board accommodation recessed on the wall with locking arrangement. (R.A Attached) | 1 | 235000.00 | P/set | 235000.00 |
| --- | --- | --- | --- | --- | --- |
|  | SUCTION AND DELIVERY |  |  |  |  |
| 02 | Providing laying and fixing in trenches i/c fitting jointing and testing etc: complete in all respect the high density polyethylene PE pipe (HDPE-100) for W/S confirming ISO confirming ISO 4427/DIN 8074/8075 B.S 3580 & PSI 3051. . | 48.00 | 178.00 | P.Rft | 8544.00 |
| 03 | C.I bend with flanged ends with holes i/c turning and facing flanged for all sizes. (P.H.S (M) I. No 8 P/ 11)3” D | 0.53 | 6096.00 | P. Cwt | 3200.00 |
| 04 | C.I tapered flat bottomed or central tapered flanged end with holes i/c turning facing flanged for all sizes (P.H.S. (M) I. No 7 P/ 11) | 0.21 | 6096.00 | P. Cwt | 1257.00 |
| 05 | Flange Adopter 3” dia (N.S.I) | 8.00` | 675.00 | Each | 5400.00 |
| 06 | Providing laying G.I pipe of approved quality to be tested pressure complete i/c the cost of bracking through walls & making good etc painting 3 coats after clearing the pipe with zink paint with pigment to match the pressure head of 200ft and handling (SSI NO.1-1 P-123) 3” dia | 8.00 | 304.02 | P.Rft | 2432.00 |
| 07 | BUTTFUSION JOINT 3” DIA | 8.00 | 600.00 | Each | 4800.00 |
| 08 | C.I Tappeer with one end flanged and socket at the other end for all size (P.H.S (M) I. No 4 P/ 12) | 0.26 | 6096.00 | P. Cwt | 1585.00 |
| 09 | C.I sluice valve heavy pattern test pressure 21 kg /sq, cm or 300 lbs /sq inch (P.H.S (M) I No 16 P/ 11)3”dia | 1.00 | 4290.00 | Eaxh | 4290.00 |
| 10 | Reflex valve heavy pattern test pressure 21 kg /sq, cm or 300 lbs /sq inch(P.H.S (M) I No 16 P/ 11) 3”dia | 1.00 | 1543.75 | Each | 1544.00 |
| 11 | C.I Foot valve heavy pattern with cone type gate (P.H.S (M) I No 4 P/ 10) 3”dia | 1.00 | 731.25 | Each | 731.00 |
| 12 | Jointing C.I/M.S flanged and specials pipes and inside a trenches i/c supplying of the rubber packing of the required thickness nuts bolts and washers etc and other tools required for jointing and testing the and washers etc and other tools required for jointing and testing the | 8.00 | 499.00 | P,Joint | 3992.00 |
| 13 | Providing and fixing M.S flange made out of M.S sheet 3/8” thick i/c drilling holes.3” | 8.00 | 500.89 | Each | 4007.00 |
|  |  |  |  | Total | 2,76,782.00 |

pipe line rising main & distribution system

| 01 | Excavation for pipelines in trenches and pits in soft soil i/c trimming dressing sides to true alignment and shape leveling of beds of trenches to correct level and grad cutting joint holes providing face guards lights, flags and temporary crossing for non vehicular traffic where-ever required lift upto 5' and lead upto one chain.)PHSI No:1,P-46.) | 7620.00 | 3600.00 | %0CFT | 27432.00 |
| --- | --- | --- | --- | --- | --- |
| 02 | Providing PVC Pipe of class "B"(equivalent make)fixing in trench i/c cutting fitting and jointion with"Z" joint with one rubber Rising i/c testing with water to head of 61 meter or 200 ft.(PHSI No:E1,P-22) |  |  | P.RFT |  |
|  | For 3" Dia | 1270.00 | 90.00 |  | 114300.00 |
| 03 | Cast Iron Specials cost iron fitting for PVC pipe(PHSI(M)1-1P- - Bend any degree (plain) 3” dia |  |  | Each |  |
|  | 3” dia | 4 | 570.00 |  | 2280.00 |
|  | TEE PLAIN |  |  |  |  |
|  | 3” dia | 3 | 771.00 |  | 2313.00 |
|  | PVC SOCKET |  |  |  |  |
|  | 3” Dia | 3 | 405.00 |  | 1215.00 |
|  | CI Gibault Joint |  |  |  |  |
|  | 3” Dia | 10 | 568.75 | Each | 5688.00 |
|  | End Plug |  |  |  |  |
|  | 3” Dia | 2 | 382.00 |  | 764.00 |
| 04 | Making joint to PVC special fitting i/c laying of special and cost of solvent cement of required diameter and esting the joints along with PVC pipe line B,C & D class to the specified pesure and making good to all leaky joints etc cpmplete (GSI-1 P-41) 3” dia | 10 | 62.00 | P.Joint | 620.00 |
| 05 | Refilling the excavated stuff in trenches 6" thick layer i/c watering ramming to full compaction etc.complete(PHSI.No.24,P-53 ) | 6858.00 | 2760.00 | %0CFT | 18928.00 |
| Total Rs. |  |  |  |  | 1,73,540.00 |

A B S T R A C T

| Sr. No: | Item of Work | SCHEDULE “B” AMOUNT | NON SCHEDULE “B” AMOUNT | TOTAL AMOUNT |
| --- | --- | --- | --- | --- |
| 1 | construction of new pump house 8x8 for tube well 1 No | 1,47,548.00 | - | 1,47,548.00 |
| 2 | tube wll 1 no | 1,57,678.00 | - | 1,57,678.00 |
| 3 | pumping machinary new 12 bhp diesel oil engine for tube well 1 No. | 36,382.00 | 2,40,400.00 | 2,76,782.00 |
| 4 | providing laying ( rising main & distribution system) | 1,73,540.00 | - | 1,73,540.00 |
|  | totla | 5,15,148.00 | 2,40,400.00 | 7,55,548.00 |

village ghulam qadir  lashari taluka thull

construction of new pump house 8x8 for tube well

| tube well 1 no. Sr. No: | Item of Work | Qty: | Rate | Unit |  | Amount |
| --- | --- | --- | --- | --- | --- | --- |
| 01 | Excavation in foundation of building bridges and other structures i/c bailing, dressing, filling around the structure with excavated earth, watering, ramming Lead up to one chain & lift up to 5.0' (CSI No:18 P4) 285.00 | 140.00 | 3,176.05 |  | P %oCft | 445.00 |
| 02 | Cement Concrete bricks or stone ballast 1 1/2" to 2" gauge ratio 1:4:8 11(CSI No:4 P15) | 52.35 | 9,416.28 |  | P %Cft | 4929.00 |
| 03 | Pucca Brick work in foundation & plinth in cement sand mortar 1:6 (CSI No:4 P21) | 213.42 | 11948.36 |  | P %Cft | 25500.00 |
| 04 | Damp proof course with (Cement Sand & Shingle concrete 1:2:4) including two coats of asphaltic mixture 3" thick (CSI.No.28,P-19) | 39.55 | 4,982.18 |  | P %Sft | 1970.00 |
| 05 | Pucca Brick work in ground floor i/c striking of joints in cement sand mortar 1:6 (CSI No:5 P21) | 233.25 | 12,674.36 |  | P %Cft | 29563.00 |
| 06 | Maing and fixing steel grated door complete with locking arrangement angle iron frame 2’x2’x3/8’ 7 ¾’ sq bars 4’ TO CENTER9CSI NO.23 P92. | 22.75 | 726.72 |  | P.Sft | 16533.00 |
| 07 | Fabrication of mild steel reinforcement for cement concrete i/c cutting bending laying in position making joints and fastening i/c cost of binding wire Also includes removal of rust from bars. | 2.66 | 5001.70 |  | P. Cwt | 13304.00 |
| 08 | RCC work in roof slabs beams columns rafts lintels and other structural members laid in situ are pre-cast laid in position etc complete in all respects ratio 1:2:4 {cement, sand, shingle 1/8" to 1/4"} (CSI 6 a P-16 & 17) | 66.21 | 337.00 |  | P.cft | 22313.00 |
| 09 | Filling watering ramming in floor surplus earth from foundation lead upto one chain and lift upto 5 ft: (CSI No.21 P-4) | 184.06 | 1512.50 |  | P%oCft | 278.00 |
| 10 | Providing Laying 3" thick CC Topping 1:2:4 i/c surface finishing and dividing into panels (CSI.NO.16 P-42) | 56.25 | 4411.82 |  | P% Sfr | 2482.00 |
| 11 | Cement plaster 1:4 upto 20' height 1/2" thick (CSI No:11 P-52) | 665.25 | 2,283.93 |  | P%Sft | 15194.00 |
| 12 | Fabrication of heavy steel reinforcement for cement concrete i/c cutting bending laying in position making joints and fastening i/c cost of binding wire also includes removal of rust from bars (CSI NO.8 (a) P-17) | 1.01 | 4928.49 |  | Cwt | 4978.00 |
| 13 | Preparing new surface and painting of doors & window any type i/c edge (CSI.NO.5 C P-70) Two coats | 69.50 | 1,489.68 |  | P %sft | 1035.00 |
| 14 | White washing three coat(CSI.NO.26 P-59) | 64.00 | 829.95 |  | P %Sft | 531.00 |
| 15 | White washing one coat(CSINO25P-59) | 665.25 | 416.63 |  | P%Sft | 2772.00 |
| 16 | Color Washing two coats (CSI.NO.25 P-5 | 665.25 | 859.90 |  | P %Sft | 7520.00 |
|  |  |  |  |  | Total | 1,47,548.00 |

| Sr. No: | Item of Work | Qty: | Rate | Unit |  | Amount |
| --- | --- | --- | --- | --- | --- | --- |
| 01 | Borring for Tube Wells in all Waterbearingsoil from ground level up to 100' or 30.50 M Depth i/c sinking & withdrawing of casing pipe(PHSI No.1 P-No.36) 15” dia | 46.00 | 877.00 |  | P. Rft | 40342.00 |
| 02 | Providing Strong substantailly built wooden locked box with compartments for preserving 2 Lbs (1 kg) samples stratas (PHSI No.5 P-No.36) | 1 | 3530.00 |  | P No. | 3530.00 |
| 03 | Taking & preserving in Box Pipe B 2 Lbs (1kg) sample of stratas from bore hole (PHSI No.6 P-No.36) | 2.00 | 101/00 |  | P sample | 202.00 |
| 04 | Taking sample of water from bore hole and placing in two separate bottles (PHSI No. 7 P No. 37) | 2.00 | 131/- |  | P.Sample | 262.00 |
| 05 | Supplying & Installation PVC Blind Pipe B Class of approved Design quality & make i/c necessary sockets etc Complete 12" Dia (pHSI NO.12P-44)) | 20.00 | 1329.40 |  | P.Rft | 26588.00 |
| 06 | Supplying & Installation PVC Strainer B Class of approved Design quality & make i/c necessary sockets etc complete 12" dia (PHSI NO>9 P-43)) | 26.00 | 1439.75 |  | P.Rft | 37434.00 |
| 07 | Supplying & Fixing M.S bail plug (PHSI NO.8 P-43) 12” dia | 1 | 2607.00 |  | Each | 2607.00 |
| 08 | Shrouding with fine graded bajri (3/8" to 1/8" or 9mm to 3mm in between bore and blind pipe of the following diameter of strainer (PHSI No.13 P no.38) 12”dia | 46.00 | 193.00 |  | P. Rft | 8878.00 |
| 09 | Supplying and installing PVC end plug (PHSI NO. 8 P-43) 12”dia | 1.00 | 2200.00 |  | Each | 2200.00 |
| 10 | Development of tube well | 46.00 | 200/- |  | P.Rft | 9200.00 |
|  |  |  |  |  | TOTAL | 1,31,243.00 |

Pumping machinery new 16 bhp diesel oil engine for tube well

| Sr. No: | Item of Work | Qty: | Rate | Unit |  | Amount |
| --- | --- | --- | --- | --- | --- | --- |
| 01 | Supplying and Installing in position i/c transportation to site of work for Rural Water Supply Scheme Village Ghulam Qadir Lashari Tauka Thull District Jacobabad Deisle Engine set consisting of Deisle Oil Enigne Golden made of 16.0 BHP RPM 2900 or equvalant type coupled with KSB Centrifugal Pump (ETA 40-200) Size 2 x 11/2" capable of Discharge 50 gallons per minute against a head of 100 ft. i/c frame & C.I Pully with belt etc Complete i/c installation of pumping set on C.C foundation Ratio (1:2:4) and Ratio (1:4:8) with ballast with M.S nuts and bolts of 5/8 dia to be embedded in C.C foundation accommodate man or sub mains i/c providing and fixing mattel board accomodation recessed on the wall withlockingarrangment.(R.AAttached) | 1 | 235000.0 |  | P/set | 2,35,000.00 |
| (B) | SUCTION AND DELIVERY |  |  |  |  |  |
| 02 | Providing laying and fixing in trenches i/c fitting jointing and testing etc: complete in all respect the high density polythylene PE pipe (HDPE-100) for W/S confirming ISO confirming ISO 4427/DIN 8074/8075 B.S 3580 & PSI 3051. . | 48.00 | 178.00 |  | P.Rft | 8544.00 |
| 03 | C.I bend with flanged ends with holes i/c turning and facing flanged for all sizes. (P.H.S (M) I. No 8 P/ 11) | 0.53 | 6096.00 |  | P. Cwt | 3200.00 |
| 04 | C.I tapered flat bottomed or central tapered flanged end with holes i/c turning facing flanged for all sizes (P.H.S. (M) I. No 7 P/ 11) | 0.21 | 6096.00 |  | P. Cwt | 1280.00 |
| 05 | Flange Adopter 3” dia (N.S.I) | 8.00` | 675.00 |  | Each | 5400.00 |
| 06 | Providing laying G.I pipe of approved quality to be tested pressure complete i/c the cost of bracking through walls & making good etc painting 3 coats after clearing the pipe with zink paint with pigment to match the pressure head of 200ft and handling (SSI NO.1-1 P-123) 3” dia | 8.00 | 304.02 |  | P.Rft | 2432.00 |
| 07 | BUTTFUSION JOINT 3” DIA | 8.00 | 600.00 |  | Each | 4800.00 |
| 08 | C.I Tapper with one end flanged and socket at the other end for all size (P.H.S (M) I. No 4 P/ 12)3” dia | 0.26 | 6096.00 |  | P. Cwt | 1585.00 |
| 09 | C.I sluice valve heavy pattern test pressure 21 kg /sq, cm or 300 lbs /sq inch (P.H.S (M) I No 16 P/ 11)3”dia | 1.00 | 4290.00 |  | Eaxh | 4290.00 |
| 10 | Reflex valve heavy pattern test pressure 21 kg /sq, cm or 300 lbs /sq inch(P.H.S (M) I No 16 P/ 11) 3”dia | 1.00 | 1543.75 |  | Each | 1544.00 |
| 11 | C.I Foot valve heavy pattern with cone type gate (P.H.S (M) I No 4 P/ 10) 3”d | 1.00 | 731.25 |  | Each | 731.00 |
| 12 | Jointing C.I/M.S flanged and specials pipes and inside a trenches i/c supplying of the rubber packing of the required thickness nuts bolts and washers etc and other tools required for jointing and testing the and washers etc and other tools required for jointing and testing the joints in the specified pressure etc complete (PHSI NO.1 P-40) | 8.00 | 499.00 |  | P,Joint | 3992.00 |
| 13 | Providing and fixing M.S flange made out of M.S sheet 3/8” thick i/c dirilling holes.Providing and fixing flang 3” dia | 8.00 | 500.89 |  | Each | 4007.00 |
|  |  |  |  |  | Total | 2,76,782.00 |

providing laying (rising main & distribution system)

| Sr. No: | Item of Work | Qty: | Rate | Unit |  | Amount |
| --- | --- | --- | --- | --- | --- | --- |
| 01 | Excavation for pipelines in trnches and pits in soft soil i/c trimming dressing sides to true alignment and shape leveling of beds of trenches to correct level and grad cutting joint holes providing face guards lights,flags and temperory crossing for non vehicular traffic where-ever required lift upto 5' and lead upto one chain.)PHSI No:1,P-46.) | 26880.00 | 3600.00 |  | %0CFT | 96768.00 |
| 02 | Providing PVC Pipe of class "B"(equivalent make)fixing in trench i/c cutting fitting and jointion with"Z" joint with one rubber Rising i/c testing with water to head of 61 meter or 200 ft.(PHSI No:E1,P-22) 3” dia | 4480.00 | 90.00 |  | P.RFT | 403200.00 |
| 03 | Cast Iron Specials cost iron fitting for PVC pipe(PHSI(M)1-1P - Bend any degree (plain) |  |  |  | Each |  |
|  | 3" Dia | 8 | 570.00 |  | Each | 4560.00 |
|  | TEE PLAIN |  |  |  |  |  |
|  | 3" Dia | 3 | 771.00 |  | Each | 2313.00 |
|  | PVC SOCKET |  |  |  |  |  |
|  | 3” Dia | 6 | 405.00 |  | Each | 2430.00 |
|  | CI Gibault Joint |  |  |  |  |  |
|  | 3” Dia | 10 | 568.75 |  | Each | 5688.00 |
|  | End Plug |  |  |  |  |  |
|  | 3” Dia | 2 | 382.00 |  | Each | 764.00 |
| 04 | Making joint to PVC special fitting i/c laying of special and cost of solvent cement of required diameter and esting the joints along with PVC pipe line B,C & D class to the specified pesure and making good to all leaky joints etc cpmplete (GSI-1 P-41) 3” dia | 10 | 62.00 |  | P.Joint | 620.00 |
| 05 | Refilling the excavated stuff in trenches 6" thick layer i/c watering ramming to full compaction etc.complete(PHSI.No.24,P-53 ) | 24192.00 | 2760.00 |  | %0CFT | 66770.00 |
|  |  |  |  |  | TOTAL | 5,83,113.00 |

`

A B S T R A C T

| Sr. No: | Item of Work | SCHEDULE “B” AMOUNT | NON SCHEDULE “B” AMOUNT | TOTAL AMOUNT |
| --- | --- | --- | --- | --- |
| 1 | construction of new pump house 8x8 for tube well 1 no. | 1,47,548.00 | - | 1,47,548.00 |
| 2 | tube well 1 no. | 1,31,243.00 | - | 1,31,243.00 |
| 3 | pumping machinary new 16 bhp diesel oil engine for tube well 1 no | 36,382.00 | 2,40,400.00 | 2,76,782.00 |
| 4 | Providing laying (rising main & distribution system) | 5,83,113.00 | - | 5,83,113.00 |
|  | TOTAL | 8,98,286.00 | 2,40,400.00 | 11,38,686.00 |

village bilawal khan nonari taulka  thull

construction of new pump house 8x8 for tube well 1 no

| Sr. No: | Item of Work | Qty: | Rate | Unit |  | Amount |
| --- | --- | --- | --- | --- | --- | --- |
| 01 | Excavation in foundation of building bridges and other structures i/c bailing, dressing, filling around the structure with excavated earth, watering, ramming Lead up to one chain & lift up to 5.0' (CSI No:18 P4) 285.00 | 140.00 | 3,176.05 |  | P %oCft | 445.00 |
| 02 | Cement Concrete bricks or stone ballast 1 1/2" to 2" gauge ratio 1:4:8 11(CSI No:4 P15) | 52.35 | 9,416.28 |  | P %Cft | 4929.00 |
| 03 | Pucca Brick work in foundation & plinth in cement sand mortar 1:6 (CSI No:4 P21) | 213.42 | 11948.36 |  | P %Cft | 25500.00 |
| 04 | Damp proof course with (Cement Sand & Shingle concrete 1:2:4) including two coats of asphaltic mixture 3" thick (CSI.No.28,P-19) | 39.55 | 4,982.18 |  | P %Sft | 1970.00 |
| 05 | Pucca Brick work in ground floor i/c striking of joints in cement sand mortar 1:6 (CSI No:5 P21) | 233.25 | 12,674.36 |  | P %Cft | 29563.00 |
| 06 | Maing and fixing steel grated door complete with locking arrangement angle iron frame 2’x2’x3/8’ 7 ¾’ sq bars 4’ TO CENTER9CSI NO.23 P92. | 22.75 | 726.72 |  | P.Sft | 16533.00 |
| 07 | Fabrication of mild steel reinforcement for cement concrete i/c cutting bending laying in position making joints and fastening i/c cost of binding wire Also includes removal of rust from bars. | 2.66 | 5001.70 |  | P. Cwt | 13304.00 |
| 08 | RCC work in roof slabs beams columns rafts lintels and other structural members laid in situ are precast laid in position etc complete in all respects ratio 1:2:4 {cement, sand, shingle 1/8" to 1/4"} (CSI 6 a P-16 & 17) | 66.21 | 337.00 |  | P.cft | 22313.00 |
| 09 | Filling watering ramming in floor surplus earth from foundation lead upto one chain and lift upto 5 ft: (CSI No.21 P-4) | 184.06 | 1512.50 |  | P%oCft | 278.00 |
| 10 | Providing Laying 3" thick CC Topping 1:2:4 i/c surface finishing and dividing into panels (CSI.NO.16 P-42) | 56.25 | 4411.82 |  | P% Sfr | 2482.00 |
| 11 | Cement plaster 1:4 upto 20' height 1/2" thick (CSI No:11 P-52) | 665.25 | 2,283.93 |  | P%Sft | 15194.00 |
| 12 | Fabrication of heavy steel reinforcement for cement concrete i/c cutting bending laying in position making joints and fastening i/c cost of binding wire also includes removal of rust from bars (CSI NO.8 (a) P-17) | 1.01 | 4928.49 |  | Cwt | 4978.00 |
| 13 | Preparing new surface and painting of doors & window any type i/c edge (CSI.NO.5 C P-70) Two coats | 69.50 | 1,489.68 |  | P %sft | 1035.00 |
| 14 | White washing three coat(CSI.NO.26 P-59) | 64.00 | 829.95 |  | P %Sft | 531.00 |
| 15 | White washing one coat(CSINO25P-59) | 665.25 | 416.63 |  | P%Sft | 2772.00 |
| 16 | Color Washing two coats (CSI.NO.25 P-5 | 665.25 | 859.90 |  | P %Sft | 7520.00 |
|  |  |  |  |  | Total | 1,47,548.00 |

tube  well  1 no.

| Sr. No: | Item of Work | Qty: | Rate | Unit |  | Amount |
| --- | --- | --- | --- | --- | --- | --- |
| 01 | Borring for Tube Wells in all Waterbearingsoil from ground level up to 100' or 30.50 M Depth i/c sinking & withdrawing of casing pipe(PHSI No.1 P-No.36) 15” dia | 46.00 | 877.00 |  | P. Rft | 40342.00 |
| 02 | Providing Strong substantailly built wooden locked box with compartments for preserving 2 Lbs (1 kg) samples stratas (PHSI No.5 P-No.36) | 1 | 3530.00 |  | P No. | 3530.00 |
| 03 | Taking & preserving in Box Pipe B 2 Lbs (1kg) sample of stratas from bore hole (PHSI No.6 P-No.36) | 2.00 | 101/00 |  | P sample | 202.00 |
| 04 | Taking sample of water from bore hole and placing in two separate bottles (PHSI No. 7 P No. 37) | 2.00 | 131/- |  | P.Sample | 262.00 |
| 05 | Supplying & Installation PVC Blind Pipe B Class of approved Design quality & make i/c necessary sockets etc Complete (PHSI NO.13 P-44) 12” dia | 20.00 | 1329.40 |  | P.Rft | 26588.00 |
| 06 | Supplying & Installation PVC Strainer B Class of approved Design quality & make i/c necessary sockets etc complete ( PHSI NO.12P-43)) 12” dia | 26.00 | 1439.75 |  | P.Rft | 37434.00 |
| 07 | Supplying and fixing M.S bail plug (PHSI NO.8P-423) 12” dia | 1.00 | 2607.00 |  | Each | 2607.00 |
| 08 | Shrouding with fine graded bajri (3/8" to 1/8" or 9mm to 3mm in between bore and blind pipe of the following diameter of strainer (PHSI No.13 P no.38)12” dia | 46.00 | 193.00 |  | P. Rft | 8878.00 |
| 09 | Supplying and installing PVC end plug 12” dai(PHSI NO. 8 P-43) | 1.00 | 2200.00 |  | Each | 2200.00 |
| 10 | Development of tube well 12” dia | 46.00 | 200/- |  | P.Rft | 9200.00 |
|  |  |  |  |  | Total | 1,31,243.00 |

pvc pipe line (rising main & distribution system)

| Sr. No: | Item of Work | Qty: | Rate | Unit |  | Amount |
| --- | --- | --- | --- | --- | --- | --- |
| 01 | Excavation for pipelines in trnches and pits in soft soil i/c trimming dressing sides to true alignment and shape leveling of beds of trenches to correct level and grad cutting joint holes providing face guards lights,flags and temperory crossing for non vehicular traffic where-ever required lift upto 5' and lead upto one chain.)PHSI No:1,P-46.) | 11700.00 | 3600.00 |  | %0CFT | 42120.00 |
| 02 | Providing PVC Pipe of class "B"(equivalent make)fixing in trench i/c cutting fitting and jointion with"Z" joint with one rubber Rising i/c testing with water to head of 61 meter or 200 ft.(PHSI No:E1,P-22)3” dia | 1950.00 | 90.00 |  | P.RFT | 175500.00 |
| 03 | Cast Iron Specials cost iron fitting for PVC pipe(PHSI(M)1-1P - Bend any degree (plain) |  |  |  |  |  |
|  | 3" Dia | 8 | 570.00 |  | Each | 4560.00 |
|  | TEE PLAIN |  |  |  |  |  |
|  | 3" Dia | 3 | 771.00 |  |  | 2313.00 |
|  | PVC SOCKET |  |  |  |  |  |
|  | 3” Dia | 6 | 405.00 |  | Each | 2430.00 |
|  | CI Gibault Joint |  |  |  |  |  |
|  | 3” Dia | 10 | 568.75 |  | Each | 5688.00 |
|  | End Plug |  |  |  |  |  |
|  | 3” Dia | 2 | 382.00 |  | Each | 764.00 |
| 04 | Making joint to PVC special fitting i/c laying of special and cost of solvent cement of required diameter and esting the joints along with PVC pipe line B,C & D class to the specified pesure and making good to all leaky joints etc cpmplete (GSI-1 P-41) 3” Dia | 10 | 62.00 |  | P.Jpoint | 620.00 |
| 05 | Refilling the excavated stuff in trenches 6" thick layer i/c watering ramming to full compaction etc.complete(PHSI.No.24,P-53 ) | 10530.00 | 2760.00 |  | %0CFT | 29063.00 |
|  |  |  |  |  | TOTAL | 2,63,058.00 |

PUMPING MACHINERY new 12 bhp diesel oil ENGINE TUBE well 01 no

| 01 | Supplying and Installing in position i/c transportation to site of work for Rural Water Supply Scheme Village Bilawal Khan Noonari Tauka Thull District Jacobabad Deisle Engine set consisting of Deisle Oil Enigne Made Golden of 16.0 BHP RPM 2900 or equvalant type coupled with KSB Centrifugal Pump (ETA 40-200) Size 2 x 11/2" capable of Discharge 70 gallons per minute against a head of 100 ft. i/c frame & C.I Pully with belt etc Complete i/c installation of pumping set on C.C foundation Ratio (1:2:4) and Ratio (1:4:8) with ballast with M.S nuts and bolts of 5/8 dia to be embedded in C.C foundation accommodate man or sub mains i/c providing and fixing mattel board accomodation recessed on the wall with locking arrangment. (R.A Attached) | 1 | 235000.00 | P/set | 2,35,000.00 |
| --- | --- | --- | --- | --- | --- |
|  | SUCTION AND DELIVERY |  |  |  |  |
| 02 | Providing laying and fixing in trenches i/c fitting jointing and testing etc: complete in all respect the high density polythylene PE pipe (HDPE-100) for W/S confirming ISO confirming ISO 4427/DIN 8074/8075 B.S 3580 & PSI 3051. 3”d. | 48.00 | 178.00 | P.Rft | 8544.00 |
| 03 | C.I bend with flanged ends with holes i/c turning and facing flanged for all sizes. (P.H.S (M) I. No 8 P/ 11) 3” Dia | 0.53 | 6096.00 | P. Cwt | 3200.00 |
| 04 | C.I tapered flat bottomed or central tapered flanged end with holes i/c turning facing flanged for all sizes (P.H.S. (M) I. No 7 P/ 11) | 0.21 | 6096.00 | P. Cwt | 1257.00 |
| 05 | Flange Adopter 3” dia (N.S.I) | 8.00` | 675.00 | Each | 5400.00 |
| 06 | Providing laying G.I pipe of approved quality to be tested pressure complete i/c the cost of bracking through walls & making good etc painting 3 coats after clearing the pipe with zink paint with pigment to match the pressure head of 200ft and handling (SSI NO.1-1 P-123) 3” dia | 8.00 | 304.02 | P.Rft | 2432.00 |
| 07 | C.I Tapper with one end flanged and socket at the other end for all size (P.H.S (M) I. No 4 P/ 12)3” dia | 0.26 | 6096.00 | P. Cwt | 1585.00 |
| 08 | BUTTFUSION JOINT 3” DIA | 8.00 | 600.00 | Each | 4800.00 |
| 09 | C.I sluice valve heavy pattern test pressure 21 kg /sq, cm or 300 lbs /sq inch (P.H.S (M) I No 16 P/ 11)3”dia | 1.00 | 4290.00 | Eaxh | 4290.00 |
| 10 | Reflex valve heavy pattern test pressure 21 kg /sq, cm or 300 lbs /sq inch(P.H.S (M) I No 16 P/ 11) 3”dia | 1.00 | 1543.75 | Each | 1544.00 |
| 11 | C.I Foot valve heavy pattern with cone type gate (P.H.S (M) I No 4 P/ 10) 3”d | 1.00 | 731.25 | Each | 731.00 |
| 12 | Jointing C.I/M.S flanged and specials pipes and inside a trenches i/c supplying of the rubber packing of the required thickness nuts bolts and washers etc and other tools required for jointing and testing the and washers etc and other tools required for jointing and testing the joints in the specified pressure etc complete(PHSI NO.1P-40)3” dia | 8.00 | 499.00 | P,Joint | 3992.00 |
| 13 | Providing and fixing M.S flange made out of M.S sheet 3/8” thick i/c drilling holes. Providing and fixing flange | 8.00 | 500.89 |  | 4007.00 |
|  |  |  |  | Total | 2,76,782.00 |

A B S T R A C T

| Sr. No: | Item of Work | SCHEDULE “B” AMOUNT | NON SCHEDULE “B” AMOUNT | TOTAL AMOUNT |
| --- | --- | --- | --- | --- |
| 1 | construction of new pump house 8x8 for tube well 1 No | 1,47,548.00 | - | 1,47,548.00 |
| 2 | tube wll 1 no | 1,31,243.00 | - | 1,31,243.00 |
| 3 | pumping machinary new 12 bhp diesel oil engine for tube well 1 No. | 36,382.00 | 2,40,400.00 | 2,76,782.00 |
| 4 | providing laying ( rising main & distribution system) | 2,63,058.00 | - | 2,63,058.00 |
|  | total | 5,78,231.00 | 2,40,400.00 | 8,18,631.00 |

village shahbaz khan kandrani taulka  thull

construction of new pump house 8x8 for tube well

| Sr. No: | Item of Work | Qty: | Rate | Unit |  | Amount |
| --- | --- | --- | --- | --- | --- | --- |
| 01 | Excavation in foundation of building bridges and other structures i/c bailing, dressing, filling around the structure with excavated earth, watering, ramming Lead up to one chain & lift up to 5.0' (CSI No:18 P4) 285.00 | 140.00 | 3,176.05 |  | P %oCft | 445.00 |
| 02 | Cement Concrete bricks or stone ballast 1 1/2" to 2" gauge ratio 1:4:8 11(CSI No:4 P15) | 52.35 | 9,416.28 |  | P %Cft | 4929.00 |
| 03 | Pucca Brick work in foundation & plinth in cement sand mortar 1:6 (CSI No:4 P21) | 213.42 | 11948.36 |  | P %Cft | 25500.00 |
| 04 | Damp proof course with (Cement Sand & Shi concrete 1:2:4) including two coats of asphaltic mixture 3" thick (CSI.No.28,P-19) | 39.55 | 4,982.18 |  | P %Sft | 1970.00 |
| 05 | Pucca Brick work in ground floor i/c striking of joints in cement sand mortar 1:6 (CSI No:5 P21) | 233.25 | 12,674.36 |  | P %Cft | 29563.00 |
| 06 | Maing and fixing steel grated door complete with locking arrangement angle iron frame 2’x2’x3/8’ 7 ¾’ sq bars 4’ TO CENTER9CSI NO.23 P92. | 22.75 | 726.72 |  | P.Sft | 16533.00 |
| 07 | Fabrication of mild steel reinforcement for cement concrete i/c cutting bending laying in position making joints and fastening i/c cost of binding wire Also includes removal of rust from bars. | 2.66 | 5001.70 |  | P. Cwt | 13304.00 |
| 08 | RCC work in roof slabs beams columns rafts lintels and other structural members laid in situ are precast laid in position etc complete in all respects ratio 1:2:4 {cement, sand, shingle 1/8" to 1/4"} (CSI 6 a P-16 & 17) | 66.21 | 337.00 |  | P.cft | 22313.00 |
| 09 | Filling watering ramming in floor surplus earth from foundation lead upto one chain and lift upto 5 ft: (CSI No.21 P-4) | 184.06 | 1512.50 |  | P%oCft | 278.00 |
| 10 | Providing Laying 3" thick CC Topping 1:2:4 i/c surface finishing and dividing into panels (CSI.NO.16 P-42) | 56.25 | 4411.82 |  | P% Sfr | 2482.00 |
| 11 | Cement plaster 1:4 upto 20' height 1/2" thick (CSI No:11 P-52) | 665.25 | 2,283.93 |  | P%Sft | 15194.00 |
| 12 | Fabrication of heavy steel reinforcement for cement concrete i/c cutting bending laying in position making joints and fastening i/c cost of binding wire also includes removal of rust from bars (CSI NO.8 (a) P-17) | 1.01 | 4928.49 |  | Cwt | 4978.00 |
| 13 | Preparing new surface and painting of doors & window any type i/c edge (CSI.NO.5 C P-70) Two coats | 69.50 | 1,489.68 |  | P %sft | 1035.00 |
| 14 | White washing three coat(CSI.NO.26 P-59) | 64.00 | 829.95 |  | P %Sft | 531.00 |
| 15 | White washing one coat(CSINO25P-59) | 665.25 | 416.63 |  | P%Sft | 2772.00 |
| 16 | Color Washing two coats (CSI.NO.25 P-5 | 665.25 | 859.90 |  | P %Sft | 7520.00 |
|  |  |  |  |  | Total | 1,47,548.00 |

TUBE   WELL  1 NO

| 01 | Borring for Tube Wells in all Waterbearingsoil from ground level up to 100' or 30.50 M Depth i/c sinking & withdrawing of casing pipe(PHSI No.1 P-No.36) 15”dia | 50.00 | 877.00 | P. Rft | 43580.00 |
| --- | --- | --- | --- | --- | --- |
| 02 | Providing Strong substantailly built wooden locked box with compartments for preserving 2 Lbs (1 kg) samples stratas (PHSI No.5 P-No.36) | 1 | 3530.00 | P No. | 3530.00 |
| 03 | Taking & preserving in Box Pipe B 2 Lbs (1kg) sample of stratas from bore hole (PHSI No.6 P-No.36) | 2 | 101/00 | P sample | 202.00 |
| 04 | Taking sample of water from bore hole and placing in two separate bottles (PHSI No. 7 P No. 37) | 2 | 131/- | P.Sample | 262.00 |
| 05 | Supplying & Installation PVC Blind Pipe B Class of approved Design quality & make i/c necessary sockets etc Complete 8" Dia (RA Attached Based on Schedule) | 24 | 1329.40 | P.Rft | 31906.00 |
| 06 | Supplying & Installation PVC Strainer B Class of approved Design quality & make i/c necessary sockets etc complete 8" dia (RA Attached Based on Schedule) | 26 | 1439.75 | P.Rft | 37434.00 |
| 07 | Supplying and fixing M.S bail plug(PHSI NO.8P-43) 12” dia | 1 | 2607.00 | Each | 2607.00 |
| 08 | Shrouding with fine graded bajri (3/8" to 1/8" or 9mm to 3mm in between bore and blind pipe of the following diameter of strainer (PHSI No.13 P no.38) | 50.00 | 193.00 | P. Rft | 9650.00 |
| 09 | Supplying and installing PVC end plug (PHSI NO. 8 P-43)12” dia | 1.00 | 2200.00 | Each | 2200.00 |
| 10 | Development of tube well | 50.00 | 200/- | P.Rft | 10000.00 |
|  |  |  |  |  | 1,41,641.00 |

pumping machinary new 12 bhp deiasel  oil  engine  for tube well 1 no.

| 01 | Supplying and Installing in position i/c transportation to site of work for Rural Water Supply Scheme Village Shahbaz Khan Kandrani Tauka Thull District Jacobabad Deisle Engine set consisting of Deisle Oil Enigne Golden made of 16.0 BHP RPM 2900 or equvalant type coupled with KSB Centrifugal Pump (ETA 40-200) Size 2 x 11/2" capable of Discharge 50 gallons per minute against a head of 100 ft. i/c frame & C.I pully with belt etc Complete i/c installation of pumping set on C.C foundation Ratio (1:2:4) and Ratio (1:4:8) with ballast with M.S nuts and bolts of 5/8 dia to be embedded in C.C foundation accommodate man or sub mains i/c providing and fixing mattel board accomodation recessed on the wall with locking arrangment. (R.A Attached) | 1 | 235000.00 | P/set | 235000.00 |
| --- | --- | --- | --- | --- | --- |
|  | SUCTION AND DELIVERY |  |  |  |  |
| 02 | Providing laying and fixing in trenches i/c fitting jointing and testing etc: complete in all respect the high density polythylene PE pipe (HDPE-100) for W/S confirming ISO confirming ISO 4427/DIN 8074/8075 B.S 3580 & PSI 3051. 3” dia. . | 48.00 | 178.00 | P.Rft | 8544. 00 |
| 03 | C.I bend with flanged ends with holes i/c turning and facing flanged for all sizes. (P.H.S (M) I. No 8 P/ 11) | 0.53 | 6096.00 | P. Cwt | 3200.00 |
| 04 | C.I tapered flat bottomed or central tapered flanged end with holes i/c turning facing flanged for all sizes (P.H.S. (M) I. No 7 P/ 11) | 0.21 | 6096.00 | P. Cwt | 1257.00 |
| 05 | Flange Adopter 3” dia (N.S.I) | 8.00` | 675.00 | Each | 5400.00 |
| 06 | Providing laying G.I pipe of approved quality to be tested pressure complete i/c the cost of bracking through walls & making good etc painting 3 coats after clearing the pipe with zink paint with pigment to match the pressure head of 200ft and handling (SSI NO.1-1 P-123) 3” dia | 8.00 | 304.02 | P.Rft | 2432.00 |
| 07 | BUTTFUSION JOINT 3” DIA | 8.00 | 600.00 | Each | 4800.00 |
| 08 | C.I Tappeer with one end flanged and socket at the other end for all size (P.H.S (M) I. No 4 P/ 12)3” dia | 0.26 | 6096.00 | P. Cwt | 1585.00 |
| 09 | C.I sluice valve heavy pattern test pressure 21 kg /sq, cm or 300 lbs /sq inch (P.H.S (M) I No 16 P/ 11)3”dia | 1.00 | 4290.00 | Eaxh | 4290.00 |
| 10 | Reflex valve heavy pattern test pressure 21 kg /sq, cm or 300 lbs /sq inch(P.H.S (M) I No 16 P/ 11) 3”dia | 1.00 | 1543.75 | Each | 1544.00 |
| 11 | C.I Foot valve heavy pattern with cone type gate (P.H.S (M) I No 4 P/ 10) 3”d | 1.00 | 731.25 | Each | 731.00 |
| 12 | Jointing C.I/M.S flanged and specials pipes and inside a trenches i/c supplying of the rubber packing of the required thickness nuts bolts and washers etc and other tools required for jointing and testing the and washers etc and other tools required for jointing and testing the | 8.00 | 499.00 | P,Joint | 3992.00 |
| 13 | Providing and fixing M.S flange made out of M.S sheet 3/8” thick i/c dirilling holes.Providing and fixing flang | 8.00 | 500.89 | Each | 4007.00 |
|  |  |  |  | Total | 2,76,782.00 |

pipe line rising main & distribution system

| 01 | Excavation for pipelines in trnches and pits in soft soil i/c trimming dressing sides to true alignment and shape leveling of beds of trenches to correct level and grad cutting joint holes providing face guards lights,flags and temperory crossing for non vehicular traffic where-ever required lift upto 5' and lead upto one chain.)PHSI No:1,P-46.) | 46800.00 | 3600.00 | %0CFT | 168480.00 |
| --- | --- | --- | --- | --- | --- |
| 02 | Providing PVC Pipe of class "B"(equivalent make)fixing in trench i/c cutting fitting and jointion with"Z" joint with one rubber Rising i/c testing with water to head of 61 meter or 200 ft.(PHSI No:E1,P-22) |  |  | P.RFT |  |
|  | For 3" Dia | 7800.00 | 90.00 |  | 702000.00 |
| 03 | Cast Iron Specials cost iron fitting for PVC pipe(PHSI(M)1-1P - Bend any degree (plain) 3” dia |  |  | Each |  |
|  | 3” dia | 10 | 570.00 |  | 5700.00 |
|  | TEE PLAIN |  |  |  |  |
|  | 3” dia | 5 | 771.00 |  | 3855.00 |
|  | PVC SOCKET |  |  |  |  |
|  | 3” Dia | 6 | 405.00 |  | 2430.00 |
|  | CI Gibault Joint |  |  |  |  |
|  | 3” Dia | 15 | 568.75 | Each | 8531.00 |
|  | End Plug |  |  |  |  |
|  | 3” Dia | 5 | 382.00 |  | 1910.00 |
| 04 | Making joint to PVC special fitting i/c laying of special and cost of solvent cement of required diameter and esting the joints along with PVC pipe line B,C & D class to the specified pesure and making good to all leaky joints etc cpmplete (GSI-1 P-41) 3” dia | 15 | 62.00 | P.Joint | 930.00 |
| 05 | Refilling the excavated stuff in trenches 6" thick layer i/c watering ramming to full compaction etc.complete(PHSI.No.24,P-53 ) | 42120.00 | 2760.00 | %0CFT | 116251.00 |
| Total Rs. |  |  |  |  | 10,10,087.00 |

A B S T R A C T

| Sr. No: | Item of Work | SCHEDULE “B” AMOUNT | NON SCHEDULE “B” AMOUNT | TOTAL AMOUNT |
| --- | --- | --- | --- | --- |
| 1 | construction of new pump house 8x8 for tube well 1 No | 1,47,548.00 | - | 1,47,548.00 |
| 2 | tube wll 1 no | 1,41,641.00 | - | 1,41,641.00 |
| 3 | pumping machinary new 12 bhp diesel oil engine for tube well 1 No. | 36,382.00 | 2,40,400.00 | 2,76,782.00 |
| 4 | providing laying ( rising main & distribution system) | 10,10,087.00 | - | 10,10,087.00 |
|  | totla | 13,35,658.00 | 2,40,400.00 | 15,76,058.00 |

GENERAL A B S T R A C T

| Sr. No: | Item of Work | SCHEDULE “B” AMOUNT | NON SCHEDULE “B” AMOUNT | TOTAL AMOUNT |
| --- | --- | --- | --- | --- |
| 1 | village wahid khoso | 5,15,148.00 | 2,40,400.00 | 7,55,548.00 |
| 2 | village ghulam qadir lashari | 8,98,286.00 | 2,40,400.00 | 11,38,686.00 |
| 3 | village bilawal kha noonari | 5,78,231.00 | 2,40,400.00 | 8,18,631.00 |
| 4 | village shahbaz khan kandrani | 13,35,658.00 | 2,40,400.00 | 15,76,058.00 |
|  | total | 33,27,323.00 | 9,61,600.00 | 42,88,923.00 |

Executive Engineer

Public Health Engg Division

Jacobabad.

Conditions:-

1.	The work shall have to be executed according to PHE/P.W.D specification.

2.	No premium on non-schedule item not be paid to the contractor.

3.	Any error and omission in the description of item or work and shall be governed with the relevant schedule of rates and estimates.

4.	The contractor shall have to bring the material to be used in the site of work from quarries mentioned in the estimate.

| Contractor | Executive Engineer Public Health Engineering Division Jacobabad. |
| --- | --- |