NAME OF WORK:	PROVIDING AND LAYING 3" Ø WATER SUPPLY LINE AT BILAL COLONY, MALIR, KARACHI

Estimated Cost: Rs:	329,400/-					            Time Limit: _______ Days

Earnest Money: Rs:	    6,588/-						Penalty     : Rs: 500/- P.Day

Tender Cost: Rs. 3000/-

| S.NO | # DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT (Rs) |
| --- | --- | --- | --- | --- | --- |
| 01 | Excavation for pipe line in trenches, and pits in all kind of soils of murum i/c trimming and dressing sides to true alignment and shape levelling of beds of trenches to correct level and grade, cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge . Providing fence guards, lights, flags and temporary crossings for non-vehicular traffic where ever required lift upto 5 ft. (1.52m) and lead upto one chain (30.5m). | 9000 Cft | 4650/- | %oCft | 41850/- |
| 02 | Providing,Laying & Fixing in trench i/c fitting, jointing & testing etc complete in all respect the high Density Polythylene PE pipes (HDPE-100) for W/S confirming ISO 4427/DIN8074/8075 B.S 3580 & PSI 3051. 3” Dia | 1500 Rft | 148/- | P.Rft | 222000/- |
| 03 | Providing and Fixing C.I Sluice Valve heavy pattern (Test Pressure 21.0 Kg / Sq. Com or 300 Lb / Sq. Inch) (Imported) 3” Dia | 1 Nos | 4290/- | Each | 4290/- |
| 04 | Providing and Fixing P.E Tee (Equal) 3” Dia | 2 Nos | 1341/- | Each | 2682/- |
| 05 | Providing and Fixing P.E Bend 45° (PN-10) 3” Dia | 3 Nos | 894/- | Each | 2682/- |
| 06 | Providing and Fixing P.E Bend 90° (PN-10) 3” Dia | 3 Nos | 1118/- | Each | 3354/- |
| 07 | Providing and Fixing P.E Stab End Place | 4 Nos | 1616/- | Each | 6464/- |
| 08 | Providing and Fixing Butt Fusion Joint | 4 Nos | 600/- | Each | 2400/- |
| 09 | Refilling the excavated stuff in trenches 6" thick layer i/c watering ramming to full compaction etc. complete. | 8100 Cft | 2760/- | %oCft | 22356/- |
| 10 | Providing Chamber 3'x2' (915x615 mm) inside diamension 4 1/2' (1372 mm) deep as per approved design for sluice value 3" to 12" dia with 18" (457 mm) dia inside cost iron cover and frame (wt=1 cwt 3 qr) fixed in RCC 1:2:4 (102 mm) thick (with 5 Lbs steel per cft) 9" (299 mm) thick brick masonary wall set in 1:6 cement mortar 6" (1152 mm) thick cement concrete 1:3:6 infoundation 1" (25 mm) thick cement concrete 1:3:6 in foundation 1" (25 mm) thick cement concrete 1:2:4 flooring 1/2" (12.5 mm) thick cement plaster 1:3 to all inside wall surface and to top i/c providing and fixing M.S foot rest at every one foot beyond 2 1/2 ft depth curing, excavation, back filling and disposal of surplus earth etc. complete. | 1 Nos | 18820/- | Each | 18,820/- |

| S.NO | # DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT (Rs) |
| --- | --- | --- | --- | --- | --- |
| 11 | Cartage of UPVC Pipe from ACIL Factory Karachi (Lead 10 miles) | 1500 Rft | 21.40 | % Rft | 321/- |
|  | Add: 10% Contractor Profit on Water Items (Rs. 21,872/-) |  |  |  | 2,187/- |
| TOTAL |  |  |  |  | 329,406/- |

Checked and Verified

I/We hereby quote _________ % above/below S.R

Rates in Words: ___________________________________

EXECUTIVE ENGINEER

SINDH KATCHI ABADIS AUTHORITY

GOVERNMENT OF SINDH

We the undersigned do hereby under take to strictly abide all the terms & condition of the SCHEDULE “B” documents which are currently in force for the works of Sindh Katchi Abadis Authority

Signature of Contractor: _______________

Address: 	______________________

______________________

Seal		________________________

SCHEDULE “B” Issued to M/s. ______________________________

Against Pay Order of SCHEDULE “B” Cost No: _____________ Dated ___________ Rs. ____________

And P.O of 2% Earnest Money No. _____________________ Dated ________________ Rs. ___________

Signature

Seal of Issuing Authority