FACE SHEET

____________

FUND HEAD		:		PROVINCIAL

MAJOR HEAD	:

MINOR HEAD           :		ESTIMATE FOR M/R TO CHIEF ENGINEER

CAMP OFFICE AT SUKKUR.

DEPTT: HEAD	:

THIS ESTIMATE HAS BEEN FRAMED IN THE OFFICE OF THE EXECUTIVE ENGINEER PROVINCIAL BUILDING DIVISION SUKKUR FOR THE PROBABLE EXPENDITURE THAT WILL BE INCURRED ON THE ABOVE SAID WORK.

AMOUNTING TO RS:  980300/-

ESTIMATE FOR M/R TO CHIEF ENGINEER CAMP OFFICE AT SUKKUR.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 1/- | Dismantling brick work in lime or cement mortar. (S.I.No.13 P-10). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1x7.0x0.75x9.0 1x11.58x0.75x12.0 1x6.25x0.75x12.0 1x19.0x1.13x12.0 1x13.25x1.13x12.0 1x57.0x1.13x0.25 2x3.50x0.75x7.0 1x12.0x0.75x12.0 | 47 104 56 258 180 16 37 108 | CFT “ |  |  |  |
|  |  |  | 806 | CFT | 1285/63 | %CFT | 10632/- |

| 2/- | Dismantling cement concrete reinforced separating reinforcement from concrete cleaning and straightening the same.(S.I.No:20 P-10). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1x13.50x13.50x0.50 1x21.25x21.25x0.50 1x14.0x16.0x0.50 | 91 226 112 | SFT |  |  |  |
|  |  |  | 338 | SFT | 5445/- | %SFT | 18404/- |

| 3/- | Pacca brick work in G.Floor in cement sand mortar 1:6. (S.I.No:5 P-21). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1x3.50x0.75x9.0 1x4.50x0.75x7.0 1x3.50x0.75x7.0 1x3.0x0.75x7.0 1x9.0x0.75x12.0 1x1.00x0.75x7.0 1x5.00x1.13x7.0 1x6.00x1.13x6.50 1x3.50x1.13x7.0 1x5.00x1.13x4.0 1x5.0x0.75x12.0 2x7.25x0.75x7.0 1x9.00x0.75x7.0 1x2.50x1.13x4.0 1x3.00x1.13x7.0 5x2.0x0.37x4.0 2x3.00x1.13x1.50 | 24 24 18 16 81 05 40 44 28 23 45 76 47 11 16 15 10 | CFT |  |  |  |
|  |  |  | 523 | CFT | 12674/36 | %CFT | 66287/- |

| 4/- | Excavation in foundation of Building Bridges and other structures including dagbelling dressing, refilling around structure with excavated earth Watering and ramming lead upto 5 ft. In ordinary soil. (S.I.No.18 (b) P-05). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Col: | 2x3.0x4.0x2.50 | 60 | CFT | 3176/25 | %0CFT | 191/- |

| 5/- | Cement concrete brick or stone ballast 1-1/2” to 2” gauge Ratio(1:4:8). (S.I.No:4 (b) P-15). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2x3.0x4.0x0.75 | 18 | CFT | 9416/28 | %CFT | 1695/- |

PAGE-02

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 6/- | Fabrication of mild steel reinforcement for cement concrete i/c cutting bending, bending, laying in position, making joints and fastenings including cost of binding wire (also includes removal of rust from bars). (S.I.No.7 (ii) P/20) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Col: footing 5/8” Col: 5/8” Ring 3/8” | 2x16x4.50x1.043 2x8x18.0x1.043 2x30x4.25x0.375 | 150 300 96 | Lbs |  |  |  |
|  | Beam ¾” ½” B/S wall ½” Ring 3/8” Vert. beam ½” Ring 3/8” Slab ½” “ Lintel ½” Ring 3/8” | 2x5x21.0x1.504 2x2x21.0x0.667 1x4x20.50x0.667 1x30x4.25x0.375 2x4x26.0x0.667 2x35.0x4.25x0.375 48x20.0x0.667 34x25.50x0.667 2x4x7.25x0.667 2x10x2.75x0.375 | 316 56 55 48 139 333 640 578 39 21 |  |  |  |  |
|  | Chajja beam ½” Ring 3/8” ½” Ring 3/8” ½” Ring 3/8” ½” Ring 3/8” ½” Ring 3/8” | 1x6x10.50x0.667 1x18x4.25x0.375 1x4x37.75x0.667 1x68x4.25x0.375 1x4x65.0x0.667 1x120x4.25x0.375 4x26.0x0.667 1x48x4.25x0.375 2x4x9.50x0.667 2x16x4.25x0.375 | 42 29 101 108 173 191 69 77 51 51 |  |  |  |  |
|  | Chajja main ½” Dist ½” | 286x8.0x0.667 1x3x150x0.667 | 1526 300 |  |  |  |  |
|  |  |  | 5489 | LBS |  |  |  |
|  |  | 5489 / 112 | 49.00 | Cwt | 5001/70 | P.CWT | 245083/- |

| 7/- | RCC work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds: lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle)a) RCC work in roof slab, beams columns rafts, lintels and other structural members laid in situ or pre cast laid in position complete in all respect. i) Ratio (1:2:4). (S.I.No:6(a)(i) P-19). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Footing Col: Beam Slab Side beam | 2x4.0x4.0x1.25 2x1.0x1.50x3.0 2x0.75x1.50x14.0 2x20.0x0.75x1.50 1x21.0x20.0x0.50 | 40 09 32 45 210 | CFT |  |  |  |
|  | Chajja 1x(8+25.50+64+37.5+9.50)x0.75x2.0 |  | 217 |  |  |  |  |
|  | Lintel | 2x7.0x1.13x1.0 2x1.25x0.75x12.0 | 16 23 |  |  |  |  |
|  |  |  | 734 | CFT | 337/- | P.CFT | 247358/- |

PAGE-03

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 8/- | Removing cement or lime plaster. (S.I.No.53 P-14). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Ent: | 1(5.66+6.92+9.66+.75)x12.50 1(20.75+16.50+57.0)x12.50 1x2(12.17+14.0)x12.0 1x2(22.50+12.0)x12.0 | 375 1178 628 828 | SFT |  |  |  |
|  | Bath O/S | 1x2(6.25+13.75)x12.0 1x(19.0+19.0+19.0)x12.0 | 480 684 |  |  |  |  |
|  |  |  | 4173 | SFT | 121/- | %SFT | 5049/- |

| 9/- | Applying floating coat of cement 1/32”thick. (S.I.No:14 P-52). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Same Qty item No. 08 | 4173 | SFT | 660/- | %SFT | 27541/- |

| 10/- | Cement plaster (1:6) upto 20’ height ½”thick. (S.I.No:13 (b) P-51). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Same Qty item No. 09 | 4173 | SFT | 2206/60 | %SFT | 92081/- |

| 11/- | Cement plaster (1:4) upto 20’ height 3/8”thick. (S.I.No:11 (a) P-51). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Same Qty item No. 10 | 4173 | SFT | 2197/52 | %SFT | 91703/- |

| 12/- | Supplying R.S Girder 8x4x3/8” size. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2x16.25x12.0 1x16.0x12.0 5x14.0x12.0 | 390 192 840 | Lbs |  |  |  |
|  |  |  | 1422 | Lbs |  |  |  |
|  | NET QTY: | 1422 / 112 | 12.69 | Cwt | 3850/- | P.CWT | 48557/- |

| 13/- | Supplying T.Iron 2x2x1/4” size. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 11x24.50x2.25 14x14.0x2.25 13x6.50x2.25 | 606 441 190 | Lbs |  |  |  |
|  |  |  | 1237 | Lbs |  |  |  |
|  | NET QTY: | 1237 / 112 | 11.04 | Cwt | 3575/- | P.CWT | 39468/- |

| 14/- | Erecting rolled steel beams or old rails in roofs etc, erection and fixing in position. (S.I.No.6 P-93). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Qty item No.12+13 | 12.69+11.04 | 23.73 | CFT | 186/34 | P.CWT | 4419/- |

|  |  | RS: | 898468/- |
| --- | --- | --- | --- |
|  | Add Cartage Amount | RS: | 81815/- |
|  | G. TOTAL | RS: | 980283/- |
|  | SAY | RS: | 980300/- |

| SUB ENGINEER | ASSISTANT ENGINEER PROVINCIAL BUILDING SUB-DIVISION SUKKUR |
| --- | --- |

ESTIMATE FOR M/R TO CHIEF ENGINEER CAMP OFFICE AT SUKKUR.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 1/- | First class tile roofing consisting of 4" earth and 1" mud plaster with Gobri leeping over 1/2" thick cement plaster 1:6 with34 Lbs. Of hot bitumen coating sand blinded provided over 2 layers of tiles 12" x 6" x 1 1/4" laid in 1:6 cement mortar with 1/2" thick sand wiched layer of 1:6 cement mortar including 1:2 cement pointing underside of tiles complete including curing etc. (S.I.No.1 P-32). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1x16.0x14.0 1x14.0x24.50 1x16.0x6.50 | 224 343 104 | SFT “ “ |  |  |  |
|  |  |  | 671 | SFT | 11443/10 | %SFT | 76783/- |

| 2/- | Removing door with chowkats (S.I.No:33 P-12). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 03 | Nos. | 142/18 | EACH | 427/- |

| 3/- | Removing window with sky light. (S.I.No:33 (b) P-12). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 03 | No. | 102/85 | EACH | 309/- |

| 4/- | First class deodar wood wrought, joinery in doors and windows etc, fixed in position including chowkats hold fasts hinges, iron tower bolts, chocks cleats, handles and cords with hooks, etc. 1 3/4" thick. (S.I.No:7 (b) P-57). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1x3.50x9.0 5x2.50x7.0 7x3.50x7.0 | 32 88 172 | SFT “ “ |  |  |  |
|  |  |  | 292 | SFT | 1273/76 | P.SFT | 371938/- |

| 5/- | Supplying & filling sand under floor & plugging into walls. (S.I.No.29 P-25). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1x19.0x13.25x1.50 1x5.00x19.0x1.50 1x12.0x12.0x1.50 | 378 143 216 | CFT |  |  |  |
|  |  |  | 737 | CFT | 1720/66 | %CFT | 12681/- |

| 6/- | Cement concrete brick or stone ballast 1-1/2” to 2” gauge Ratio(1:5:10). (S.I.No:4 (d) P-15). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Qty item No. | 737x0.33 1.50 | 162 | CFT | 8694/95 | %CFT | 14086/- |

| 7/- | Cement concrete plain including placing compacting, finishing and curing, complete (including screening and washing at stone aggregate without shuttering ratio 1:2:4. (S.I.No.05 P-15). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Qty item No. | 162x0.17 0.33 | 83 | CFT | 14429/25 | %CFT | 11976/- |

| 8/- | Single layer of tiles 9" x4 1/2" x 2" laid over 4" earth 1" mud plaster without Bhoosa grouted with cement sand 1:3 on top of R.C.C slab provide with 34 Lbs. Bitumen coating sand blinded. (S.I.No.05 P-33). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Ent: | 21.0x20.0 | 420 | SFT | 5310/35 | %SFT | 22303/- |

PAGE-02

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 9/- | Preparing the surface and painting with matt finish i/c rubbing the surface with bathy (Silicon carbide rubbing brick) filling the voids with zink/chalk plaster of Paris mixture applying first coat matt finish & painting 3 coats with matt finish of approved made etc. complete. (S.I.No.36 A P-55). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Qty same item No.11 | 4173 | SFT | 3444/38 | %SFT | 143734/- |

| 10/- | French polishing complete. (S.I.No.7 (b) P-72). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Qty double item No.18 (2x292) | 584 | SFT | 3841/75 | %SFT | 22436/- |

| 11/- | Laying white marble flooring fine dressed on the surface without winding set in lime mortar 1:2 including rubbing and polishing of the joints. 1" thick. (S.I.No:28 (b) P-43). |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Ent: Kit: Shelf |  | 1x19.0x19.0 1x21.0x2.0 1x57.0x1.13 | 361 42 64 | SFT |  |  |  |
|  |  |  |  | 467 | SFT | 628/93 | P.SFT | 293710/- |

| 12/- | Distempering three coats (S.I.No.24 P-60). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Ent. Chajja 1(5.66+6.92+9.66+7.75)x2.50 |  | 75 | SFT |  |  |  |
|  | Room ceiling Room Bath Ent: | 1x12.17x14.0 1x22.50x12.0 1x6.25x13.75 1x19.0x19.0 | 170 270 86 361 |  |  |  |  |
|  |  |  | 962 | SFT | 1079/65 | P.SFT | 10386/- |

|  |  | RS: | 968088/- |
| --- | --- | --- | --- |
|  | Add Cartage Amount | RS: | 15791/- |
|  | G. TOTAL | RS: | 983879/- |
|  | SAY | RS: | 983900/- |

| SUB ENGINEER | ASSISTANT ENGINEER PROVINCIAL BUILDING SUB-DIVISION SUKKUR |
| --- | --- |

MATERIAL STATEMENT

| S.# | ITEMS OF WORK | QTY | CEMENT | H/SAND | BAJRI | STONE METAL | BRICKS | STEEL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01. | P.Brick work 1:6 | 523 | 18 | 134 | - | - | 7061 | - |
| 02. | C.C brick 1:4:8 | 18 | 02 | 09 | - | 16 | - | - |
| 03. | Fabrication | 49.0 | - | - | - | - | - | 2.45 |
| 04. | RCC work 1:2:4 | 734 | 129 | 323 | 846 | - | - | - |
| 05. | App. Floating coat | 4173 | 17 | - | - | - | - | - |
| 06. | Cement Plaster 1:6 | 4173 | 22 | 167 | - | - | - | - |
| 07. | Cement Plaster 1:4 | 4173 | 24 | 125 | - | - | - | - |
|  |  | QTY | 212 | 757 | 645 | 16 | 7061 | 2.45 |
|  |  | RATE | 135.33 | 4548.92 | 1292.92 | 1162.66 | 773.84 | 2257.76 |
|  |  | UNIT | P.Bag | %CFT | %CFT | %CFT | %0NOS. | P.TON |
|  | AMOUNT |  | 28690/- | 34436/- | 8352/- | 186/- | 4619/- | 5532/- |
|  |  |  |  |  | TOTAL RS. |  | 81815/- |  |

| SUB ENGINEER | ASSISTANT ENGINEER PROVINCIAL BUILDING SUB-DIVISION SUKKUR |
| --- | --- |

MATERIAL STATEMENT

| S.# | ITEMS OF WORK | QTY | CEMENT | H/SAND | BAJRI | STONE METAL | BRICKS | STEEL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01. | 2nd Class tile roof | 671 | 07 | 27 | - | - | 2684 | - |
| 02. | C.C 1:5:10 | 162 | 11 | 79 | - | 159 | - | - |
| 03. | C.C Plain 1:2:4 | 83 | 15 | 37 | 73 | - | - | - |
|  |  | QTY | 35 | 143 | 73 | 159 | 2684 | - |
|  |  | RATE | 135.33 | 4548.92 | 1292.92 | 1162.66 | 773.84 | 2257.76 |
|  |  | UNIT | P.Bag | %CFT | %CFT | %CFT | %0NOS. | P.TON |
|  | AMOUNT |  | 4737/- | 6505/- | 944/- | 1849/- | 1756/- | - |
|  |  |  |  |  | TOTAL RS. |  | 15791/- |  |

| SUB ENGINEER | ASSISTANT ENGINEER PROVINCIAL BUILDING SUB-DIVISION SUKKUR |
| --- | --- |