BILL OF QUANTITIES

Description & Rate of items based on Schedule/ Market (offered Rates).

M&R TO STAFF QUARTERS A.D.C. COLONY SUKKUR (QUARTER NO:A-9, A-10 & C-6).

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

PART “A” BUILDINGS SCHEDULE ITEMS.

| 1/- | Dismantling cement concrete reinforced separating reinforcement from concrete cleaning and straightening the same.(S.I.No:20 P-10). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 120 | SFT | 5445/- | %SFT | 6534/- |

| 2/- | Dismantling cement concrete 1:2:4. (S.I.No.19 (c) P-10). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 409 | CFT | 3327/50 | %CFT | 13609/- |

| 3/- | Dismantling brick work in lime or cement mortar. (S.I.No:13 P-10). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 125 | CFT | 1285/63 | %CFT | 1607/- |

| 4/- | Removing cement or lime plaster. (S.I.No.53 P-14). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1015 | SFT | 121/- | %SFT | 1228/- |

| 5/- | Applying floating coat of cement 1/32”thick. (S.I.No:14 P-52). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 1615 | SFT | 660/- | %SFT | 6699/- |

| 6/- | Pacca brick work in G.Floor in cement sand mortar 1:6. (S.I.No:5 P-21). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 2025 | CFT | 12674/36 | %CFT | 256657/- |

| 7/- | Cement plaster (1:6) upto 20’ height ½”thick. (S.I.No:13 (b) P-51). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 2350 | SFT | 2206/60 | %SFT | 51855/- |

| 8/- | RCC work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds: lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle)a) R.C. work in roof slab, beams columns rafts, lintels and other structural members laid in situ or pre cast laid in position complete in all respect. i) Ratio (1:2:4). (S.I.No:6(a)(i) P-19). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 222 | CFT | 337/- | P.CFT | 74814/- |

| 9/- | Fabrication of mild steel reinforcement for cement concrete i/c cutting bending, bending, laying in position, making joints and fastenings including cost of binding wire (also includes removal of rust from bars). (S.I.No.7 (ii) P/20) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 12.432 | Cwt | 5001/70 | P.CWT | 6218/- |

| 10/- | Cement plaster (1:4) upto 20’ height 3/8”thick. (S.I.No:11 (b) P-51). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 2350 | SFT | 2197/52 | %SFT | 51642/- |

| 11/- | Providing and laying 1" thick topping cement concrete (1:2:4 ) including Surface finishing and dividing into panels 2” thick (S.I.No:4 P-16). 2”thick Flooring |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 2256 | SFT | 3275/50 | %SFT | 73896/- |

| 12/- | Two coats of bitumen laid hot using 34lbs for %sft over roof and blinded with sand at one cft per %sft. (S.I.No.13 P-34). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 225 | SFT | 1887/40 | %SFT | 42580/- |

Page No:2.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 13/- | Providing and laying tiles glazed 6" x 6" x 1/4" on floor or wall facing in required colour and pattern of STILE specification jointed in white cement and pigment over a base of 1:2 grey cement mortar 3/4" thick including washing and filling of joints with slaurry of white cement and pigment in desired shape with finishing, clearing and cost of wax polish etc. complete including cutting tiles to proper profile. (S.I.No.60 P-46). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 650 | SFT | 30509/77 | %SFT | 198314/- |

| 14/- | First class wrought, joinery in door & windows etc fixed to chowkats fixed in position i/c tower bolts cocks cleats, handles and cords with hooks etc. (S.I.No.07 (b) P-57). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 72 | SFT | 902/93 | P.SFT | 65011/- |

| 15/- | PNS, Painting doors and windows any type i/c edges two coats. (S.I.No.4 P-76). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 660 | SFT | 1160/06 | %SFT | 7656/- |

| 16/- | Laying white marble flooring fine dressed on the surface without winding set in lime mortar 1:2 including rubbing and polishing of the joints. 3/4" thick flooring. (S.I.No:28 (a) P-43). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 120 | SFT | 567/48 | P.SFT | 68098/- |

| 17/- | Providing and fixing 3/8" thick marble tiles of approved quality and colour and shade size 8" x 4" /6" x 4" in dado skirting anf facing removal /tucking of existing plaster surface etc. Over 1/2" thick base of cement mortar 1:3 setting of tiles in slurry of white cement over mortar base including filling the joints and washing the tiles with white cement slurry , current finishing, cleaning and polishing etc. complete. (S.I.No:68 P-49). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 85 | SFT | 186/04 | P.SFT | 15813/- |

| 18/- | Preparing the surface and painting with matt finish i/c rubbing the surface with Bathy (silicon carbide rubbing brick) filling the voids with zink /chalk / plaster of Paris mixture, applying first coat premix, making the surface smooth and then painting 3 coats with matt finish of approved make etc: complete .(new surface). (S.I.No.3 A P-54). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1600 | SFT | 2717/- | %SFT | 43472/- |

| 19/- | Distempering two coats (S.I.No.24 P-53). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 750 | SFT | 1079/65 | %SFT | 8097/- |

|  | PART “A” BUILDINGS SCHEDULER ITEMS TOTAL | RS: | 1049763/- |
| --- | --- | --- | --- |

| PART "A) | ________________%Above/Below Buildings Schedule Items | Total Rs:- |  |
| --- | --- | --- | --- |

|  |  |  | G. Total Rs:- |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | EXECUTIVE ENGINEER |  |  |
| CONTRACTOR |  |  |  | PROVINCIAL BUILDINGS DIVISION |  |  |
|  |  |  |  | SUKKUR |  |  |