BILL OF QUANTITIES

Discription & Rate of items based on Schedule/ Market (offered Rates).

NAME OF WORK: 	M/R to  T.H.Q Hospital Gambat District Khairpur.

(OPD Block)

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

1/-	Dismantling brick work in cement or lime mortar etc .(S.I No:     P.NO:    )

Qty: 08 cft					@RS: 1285.63		% Cft		RS: 103/-

2/-	Dismantling glazed or encaustic tiles etc (S.I No:55 P.NO:13).

Qty: 344 Sft					@RS: 786.50		% Sft		RS; 2706/-

3/-	Removing door with Chowakats (S.I No: 33 (a)P.NO:120.

Qty: 812 Sft					@RS: 121/-		% Sft		RS: 983/-

4/	Fabrication of mild steel reinforcement for cement concrete i/c cutting bending laying

in position making joints and fastenings i/c cost of binding wire (also includes removal

of rust from bars) (a) Using tor bars. (S.I.No.7 (a) P.No.20)

lintel

Qty: 0.370 cwt					@RS: 5001.70		P.cwt		RS: 1851/-

5/-	R.C.C. work in roof slab beams columns rafts lintels and other structural members laid

in situ or precast laid in position complete in all respects: (i) Ratio (1:2:4) 90 lbs

cement 2 cft sand 4 cft shingle 1/8” to 1/4” gauge. (S.I.No.6 (a-i) P.No. 19)

Qty: 05 cft					@RS: 337/-		P.Cft		RS: 1685/-

6/-	Pacca Brick work in ground floor in cement sand mortar Ratio (1:60 (S.I No:   P.    ).

Qty: 08 cft					@RS: 12674.36	% Cft		RS: 1014/-

7/-	Cement Concrete brick or Stone Ballast 1” to 2”-0 gauge Ratio (1:6:12) (S.I No: P.NO:    )

Qty: 35 cft					@RS: 8122.95		% Cft		RS: 2843/-

8/-	Cement Concrete plain including placing compacting finishing & curing, complete (including screening and washing of stone aggregate without shuttering. Ratio 1:2:4. (S.I. No. 5(f), P. No.16)

Qty: 18 cft					@RS: 14429.25	% Cft		RS: 2597/-

9/-	Applying Floating coat of cement  1/32” thick.

Qty: 812 Sft					@RS: 660/-		% Sft		RS: 5359/-

10/-	Cement Plaster 1:6 up to 20’-0’ height ½” thick (S.I No:    P.NO:      ).

Qty: 812 Sft					@RS: 2206.60		% Sft		RS: 17918/-

Page no:2.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

11/-	Cement Plaster 1:4 up to 20’-0’ height 3/8” thick (S.I No:    P.NO:      ).

Qty: 541 Sft					@RS: 2197.52		% Sft		RS: 11889/-

12/-	Providing and fixing G.I frames/chowkhats of size 7”x2” or 4½”x3” for door using 20 gauge G.I sheet i/c welded hinges and fixing at site with necessary hold fasts, filling with cement sand slurry of ratio 1:6 and repairing the jambs. The cost also i/c all carriage, tools and plants used in making and fixing.   (S.I.No. 29 Page No. 98

Qty: 50 Rft					@RS: 228.90		P.Sft		RS: 11445/-

13/-	Providing and laying tile glazed (6”x 6”1/4”) on floor of wall facing in required colour and pattern of STILE specification jointed in white cement and pigment over a base of 1:2 gray cement mortar ¾” thick i/c washing and filling joints with slurry of white cement and pigment in desired shape with finishing, clearing and cost of was polish etc. complete i/c cutting tiles to proper profile. Amended  (S.I. No. 60, P. No.47)

Qty: 522 Sft					@RS: 30509.77	% SFt		RS: 159261/-

14/-	First class deodar wood  wrought joinery  in doors & windows  etc fixed in position

including  hold fasts hinges  iron  tower bolts chock, cleats  handles  and  cords  with

hooks etc complete.	(S.I No: 54 P.NO:  65).

Qty: 56 Sft					@RS: 902.93		% Sft		RS:  50564/-

5/-	Distempering 02 coats

Qty: 105 Sft					@RS: 1043.90		% Sft		RS: 1096/-

16/-		Preparing the surface and painting with mat finish paint of approved make to old matt

finish 	surface paris and then painting with weather coat of approved make. (03 Coats)

(old surface) (S.I. No. 37  (a+b)  P. No.61)

Qty: 485 Sft					@RS: 2499.76		% sft		RS: 12124/-

17/-	Painting New Surface Painting Doors & Windows any type 03 coats.

Qty: 112 Sft						@RS: 2116.41		% Sft		RS: 2370/-

============================

PART “A” BUILDINGS SCH: ITEMS	Total RS: 285808/-

Page no:3.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

PART-   B  WATER SUPPLY & SANITRY FITTING.

1-		P/F squatting type white glazed of flushing cistern with internal fitting and flush pipe with

band and making requisite Nos. of holes in wall plinth & floor for pipe connections and making good in cement concrete 1:2:4. (A) W.C pan of not less than 23” clear opening between flashing rims and 3 gallons flushing tank. With 4” dia C.I. tape.

(S.I.NO.1(A) P.No.1)

Qty:02 Nos					@Rs:6014.80		each		RS:12030/-

2-	Add extra for labour for providing & fixing of earthenware pedestal white or coloured glazed (Standard pattern) .  (S.I.NO. 9 P. No.3)

Qty:06 Nos					@Rs:938/47		each		RS:5631/-

3-	P/F 6”x2” or 6”x3” C.I floor trap of the approved self cleaning design with a C.I screwed down gritting with or without a vent arm complete with and i/c making requisite number or holes in walls plinth and floor for pipe connections & making good in CC 1:2:4. (S.I. No. 20 P. No.5)

Qty:05 Nos					@Rs:2042/13		each		RS:10211/-

4-	P/F in position nylon connections complete with 1/2” dia brass stop cock with pair of

brass 	nuts and lining joints to nylon connection. (S.I. No. 23 P. No.6)

Qty:10 Nos					@Rs:447/15		each		RS:4472/-

5-		P/F 24” x 18” beveled edge mirror of Belgium glass complete with 1/8” thick hard board and C.P screw fixed to wooden plate (a) standard pattern.  (S.I.No. 03 (a) P.No7).

Qty:06 Nos					@Rs:1711/60		each		RS:10270/-

6-	S/F in position c.p stopcock (b)½” dia C.P stop cock standard pattern. (S.I. No. 4 (b) P.

No.13)

Qty:08 Nos					@Rs:843/92		each		RS:6751/-

7-		Supplying and fixing long bib-cock of superior quality with C.P. head ½” dia.

(S.I.NO.15(a) P. No.15)

Qty:10 Nos					@Rs:1109/46		each		RS:2219/-

8-		Supplying and fixing jet shower with rod of superior quality with c.p head ½” dia.

(S.I.NO.17 P. No.15)

Qty:10 Nos					@Rs:1142/24		each		RS:11422/-

Page no:4.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

9-		Supplying and fixing swan type pillar cock of superior quality single c.p head ½” dia.

(S.I.NO.18 (a) P. No.16)

Qty:06 Nos					@Rs:2882/-		each		RS:17292/-

10-	Supplying & fixing bath room accessories set (7 piece) i/c towel rod, brush holder soap tray. Shelf of approved quality and design etc. Complete (S.I. No. 24 P. No.16)

Qty:02 Nos					@Rs:8122/40		each		RS:162465/-

11-	providing & Fixing C.C gulley trap etc complete

Qty:12 Nos					@Rs:1429/67		each		RS:17156/-

12-	P/F steel stainless local make complete with cast iron or wrought iron brackets 6” built in wall 1-½” rubber plug chrome brass chain 1-½” C.P brass waste with 1-½” plate P.V.C waste pipe and making requisite numbers of holes in wall & plinth and floor for pipe connection and making good in cement concrete 1:2:4. (c) Steel sink stainless sized 33” x 18” local make (Standard Pattern). (S.I.NO. 09(c) P. No.05)

Qty:02 Nos					@Rs:5052/30		each		RS:10105/-

____________________________________________

PART “B” W/S & S/F SCHEDULE ITEM TOTAL 	RS:116435/-

PART-B- NON SHEDULE ITEM

1/-	P/F UPVC pipe ½” dia PAK-ARAB) SCH-40 surface by using clip /saddler/socket/reducer/bush/tee etc .or recessed in masonry C.C or R.C.C upto 20’ ft. height and making good wih C.c i/c curing finishing etc complete as per instruction of the Engineer In charge ,.Specification of the material should meet the requirement of class 12454-Bin accordance with the ASTM-D-1784 type 1 Grade 1. rate include all costs of labor ,material, cartage scaffolding ladders .etc complete (R.A Sanction).

For Hot /cold

Qty:60 rft								P.rft

2/-	P/F UPVC UNION solvent ) ½’ dia (PAK ARAB) SCH-40 make of approved quality and design of various size fixed to UPVC pipe /fitting paid separately using approved adhesive compound upto 20 ft. height as per instruction of the Engineer In charge .Specification of the material should meet the requirement of class 12454-B I Grade .Rate include all costs of labor material ,cartage scaffolding /ladders etc complete.

Qty:10 Nos								each

Page no:5.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

3/-	P/F UPVC pipe ¾” dia PAK-ARAB) SCH-40 surface by using clip /saddler/socket/reducer/bush/tee / elbow etc .or recessed in masonry C.C or R.C.C upto 20’ ft. height and making good wih C.c i/c curing rinishing etc complete as per instruction of the Engineer In charge ,.Specification of the material should meet the requirement of class 12454-Bin accordance with the ASTM-D-1784 type 1 Grade 1. rate include all costs of labor ,material, cartage scaffolding ladders .etc complete (R.A Sanction).

Qty:90 rft							P.rft

4/-	P/F UPVC UNION  ¾” dia PAK-ARAB) SCH-40  make approved quality and Design of various sizes Fixed to UPVC Pipe / Fitting paid Separately  upto 20’ ft. height as per instruction Sof the Engineer In charge Specification of the material should meet the requirement of class 12454-Bin accordance with the ASTM-D-1784 type 1 Grade 1. rate include all costs of labor ,material, cartage scaffolding ladders .etc complete (R.A Sanction).

Qty:10 Nos							each

5/-	P/F UPVC pipe 1” dia PAK-ARAB) SCH-40 surface by using clip /saddler/socket/reducer/bush/tee / elbow etc .or recessed in masonry C.C or R.C.C upto 20’ ft. height and making good wih C.c i/c curing finishing etc complete as per instruction of the Engineer In charge ,.Specification of the material should meet the requirement of class 12454-Bin accordance with the ASTM-D-1784 type 1 Grade 1. rate include all costs of labor ,material, cartage scaffolding ladders .etc complete (R.A Sanction).

Qty:150 rft							P.rft

6/-	P/F UPVC pipe 4” dia PAK-ARAB) SCH-40 surface by using clip /saddler/socket/reducer/bush/tee /  etc . or recessed in masonry C.C or R.C.C upto 20’ ft. height and making good wih C.C i/c curing finishing etc complete as per instruction of the Engineer In charge ,.Specification of the material should meet the requirement of class 12454-Bin accordance with the ASTM-D-1784 type 1 Grade 1. rate include all costs of labor ,material, cartage scaffolding ladders .etc complete (R.A Sanction).

Qty:20 rft							P.rft

7/-	P/F UPVC  PLUG TEE 4” dia  PAK-ARAB) SCH-40  make of approved quality and design of various sizes fixed to UPVC Pipe / fitting  paid separately using approved adhesive compound upto   20’ ft. height  as per instruction of the Engineer In charge ,.Specification of the material should meet the requirement of class 12454-Bin accordance with the ASTM-D-1784 type 1 Grade 1. rate include all costs of labor ,material, cartage scaffolding ladders .etc complete (R.A Sanction).

Qty:8 Nos							each

8/-	P/F UPVC BEND (LONG) 4” Of PAK ARAB make of approved quality include\ing / making joints by using pest/solution AGM make  as per instruction of the Engineer In charge ,.Rate includes all costs of labour , material, cartage, scaffolding etc complete. (R.A Sanctioned).

Qty:6 Nos							each

Page no:6.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

9/-	P/F 4” dia UPVC Socket of PAK ARAB make of approved quality including makings joints by using pest/ solution AGM make as per instruction of the Engineer In charge ,. ,.Rate includes all costs of labour , material, cartage, scaffolding etc complete. (R.A Sanctioned).

Qty:03 Nos							each

10/-	P/F   4” dia  UPVC Cowel / Terminal  Guard / End of  PAK ARAB make of approved quality & joints by using pest/solution AGM make  as per instruction of the Engineer In charge ,.Rate includes all costs of labour , material, cartage, scaffolding etc complete. (R.A Sanctioned).

Qty:2 Nos							each

11/-	P/F European type Commode set make PORTA USA in  matching Colour 7  apporved  shape & design couples with flush with set cover.

Qty:04 No’s							each

12/-	Providign & Fixing  white or coloured glazed earthenwere wash basin with Pedestal make PORTA USA standard size (24”x18”) etc complete

Qty:08 No’s							each

_____________________

PART “C” NON SCHEDULE ITEM TOTAL RS:-

| PART "A) |  | ________________%Above/Below Buildings Schedule Items |  |  | Total Rs:- |  |
| --- | --- | --- | --- | --- | --- | --- |
| PART "B" |  | ________________%Above/Below W/S & S/F Schedule Items |  |  | Total Rs:- |  |
| PART "E: |  | ___________________________________Non Schedule Items |  |  | Total Rs:- |  |
|  |  |  | G. Total Rs:- |  |  |  |
|  |  |  |  | EXECUTIVE ENGINEER |  |  |
| CONTRACTOR |  |  |  | PROVINCIAL BUILDINGS DIVISION |  |  |
|  |  |  |  | SUKKUR |  |  |

.

BILL OF QUANTITIES

Discription & Rate of items based on Schedule/ Market (offered Rates).

NAME  OF  WORK:-	M/R  TO  STAFF QTR’S  DATE PALM @ KOT DIJI

DISTRICT 	KHAIRPUR.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

1/-	Dismantling cement concrete plain (1:2:4) etc complete (S.I no:19(c)P.NO:10)/.

Qty:68 Cft					@Rs:3327.50		% Cft		RS:2263/-

2/-	Removing  cement or lime plaster (S.I No:53 P.NO:14).

Qty:787 Sft					@Rs:121/-		% Sft		RS:952/-

3/-	Dismantling Brick work in Lime mortar or Cement mortar (S.I No:13 P.NO:10).

Qty:221 Cft					@Rs:1285.63		% Cft		RS:2841/-

4/-	Damp Proof Course with Cement sand and Shingle concrete 1:2:4 including 02 two coat od asphaltic mixture.(S.I No:         P.NO:        ).

Qty:37 Sft					@Rs;4982.18		% Sft		RS:1843/-

5/-	Pacca  Brick  work in Other than Building  in cement sand mortar (1:6) (S.I No:5 P-25).

Qty:119 Cft					@Rs:12346.65		% Cft		RS:14693/-

6/-	Cement Pointing Struck joints on wall (a) Ratio 1:2 (S.I no: 19 P.NO:53).

Qty:294 Sft					@Rs:1287.44		% Sft		RS:3785/-

7/-	Applying Floating Coat of cement 1/32” thick (S.I No: 14 P.NO:59).

Qty:787 sft					@Rs:660/-		% Sft		RS:5194/-

8/-	Cement Plaster 1:6 up to 20’-0’ height ½” thick. (S.I No:     P.NO:       ).

Qty:787 sft					@Rs:2206.60		% Sft		RS:17366/-

9/-	Cement Plaster 1:4 up to 20’-0’ height 3/8” thick. (S.I No:     P.NO:       ).

Qty:787 sft					@Rs:2197.52		% Sft		RS:17294/-

10/-	Providing & Laying 1” thick Topping Cement Concrete (1:2:4) including surface finishing and dividing into panel 2” thick (S,I No:16P.No:42).

Qty:400 Sft					@Rs:3275.50		% SFt		RS:13102/-

Page no:2.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

11/-	First class deodar wood  wrought joinery  in doors & windows  etc fixed in position  including  hold fasts hinges  iron  tower bolts chock, cleats  handles  and  cords  with hooks etc complete.	(S.I No: 54 P.NO:  65).

Qty: 68 Sft					@Rs:1273.76		P.Sft		RS: 86783/-

12/-	Supplying and fixing in position iron/steel grill of ¾” x ¼” size flat iron of approved design including painting 3 coats etc complete (Weight or not less than 3.7 Lbs/sq feet of finished grill)(S.I.NO.27 Page No.97)

Qty:100 Sft					@Rs:180.50		P.Sft		RS:18050/-

13/-	Distempering 02 coats .

Qty:1343 Sft					@Rs:1043.90		% Sft		RS:14020/-

14/-	Providing and laying tile glazed (6”x 6”1/4”) on floor of wall facing in required colour and pattern of STILE specification jointed in white cement and pigment over a base of 1:2 gray cement mortar ¾” thick i/c washing and filling joints with slurry of white cement and pigment in desired shape with finishing, clearing and cost of was polish etc. complete i/c cutting tiles to proper profile. Amended  (S.I. No. 60, P. No.47)

Qty: 164	Sft				@Rs:30509.77		% Sft		RS: 50036/-

15/-	Providing & Fixing Galvanized wire gauze fixed to chowakats etc complete.

Qty: 36 Sft					@RS: 190.70		P.Sft		RS: 6866/-

16/-	Painting Old Surface Painting Doors & Windows any type 03 coats.

Qty: 282 Sft					@RS: 1160.06		% Sft		RS: 3271/-

17/-	Providing & Fixng RCC Spouts fixing in position etc complete.

Qty: 02 No’s					@RS: 524.98		each		RS: 1050/-

==================================

PART “A” BUILDINGS SCHEDULE ITEM TOTAL	RS: 254529/-

PART-B   Sch: Item  W/S  S/F)

1-		P/F squatting type white glazed of flushing cistern with internal fitting and flush pipe with band and making requisite Nos. of holes in wall plinth & floor for pipe connections and making good in cement concrete 1:2:4. (A) W.C pan of not less than 23” clear opening between flashing rims and 3 gallons flushing tank. With 4” dia C.I. tape. (S.I.NO.1(A) P.No.1)

Qty:01 Nos					@Rs:4802/60		each		RS:4803/-

2-	Providing G.I pipes special & clamps etc i/c fixing cutting and fitting complete with and i/c the cost of breaking through walls and roof making good etc painting two coats after cleaning the pipe etc with white zink paint with pigment to match the colour of the building and testing with water to a pressure head of 200 feet and handling. (Internal)(ii) ¾” dia G.I Pipe. (S.I.NO.1 (ii) ) P. No.11)	¾” dia

Qty40 rft					@Rs:95/79		P.Rft		RS:3832/-

Page no:3.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

3-	P/F 6”x2” or 6”x3” C.I floor trap of the approved self cleaning design with a C.I screwed down gritting with or without a vent arm complete with and i/c making requisite number or holes in walls plinth and floor for pipe connections & making good in CC 1:2:4. (S.I. No. 20 P. No.5)

Qty:02 Nos					@Rs:2042/43		each		RS:4085/-

4/-	Providing & Fixing Nylon connection  complete with  ½” dia Brass stop cock  with pair  of brass  nuts  and lining  joints  to nylon  connection (S.I No:23 P-6).

Qty:02 Nos					@Rs:447/15		each		RS:894/-

5/-	Supplying and fixing long bib-cock of superior quality with C.P. head ½” dia.

(S.I.NO.15(a) P. No.15)

Qty:03 Nos					@Rs:1109/46		each		RS:3328/-

6-		Supplying and fixing concealed stopcock of superior quality with c.p head ½” dia.

(S.I.NO.13 (a) P. No.15)

Qty:02 Nos					@Rs:843/92		each		RS:1688/-

7-		Supplying and fixing jet shower with rod of superior quality with c.p head ½” dia.

(S.I.NO.17 P. No.15).

Qty:01 Nos					@Rs:1142/24		each		RS:1142/-

8/-	P/F Asbestoses   pipe with collar dadex  or equivalent  i/c digging the finches required depth  & fixing  & jointing  with rubber ring  i/c  testing  to water pressure  Head of 200 ft:

4” dia

Qty:60 rft					@Rs:188/44		P,.rft		RS:11306/-

10/-	S/F in position c.p Tee cock ½” dia c.p bib cock standard pattern. (S.I. No. 3 (i-b) P. No.18)

Qty:03 Nos					@Rs:843/92		each		RS:2532/-

11/-	Providing & Fixing handles valves (china) etc complete

¾” dia	 02 nos					@Rs:271/92		each		RS:544/-

1” dia 02 nos					@Rs:365/42		each		RS:731/-

12/-	Providing G.I pipes special & clamps etc i/c fixing cutting and fitting complete with and i/c the cost of breaking through walls and roof making good etc painting two coats after cleaning the pipe etc with white zink paint with pigment to match the colour of the building and testing with water to a pressure head of 200 feet and handling. (Internal)(ii) ¾” dia G.I Pipe. (S.I.NO.1 (ii) ) P. No.11)

(iii) 1” dia    70 rft	@Rs:116/20			P.Rft			RS:8134/-

_____________________________________

PART “B” W/S & S/F SCHEDULE ITEM TOTAL 	RS:43019/-

Page no:4.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

(PART-B   Public Health  Schedule

1/-	Providing UPVC pressure  pipes of class “C” equivalent make fixing in trench i/c cutting, fitting and jointing with “Z” joint with one rubber ring i/c testing with water to head of 61 meter or 200ft. 4” dia. (P.H.E)  (S.I.No. 01 (b) P.No.22)

6” dia 	40 rft				@Rs:373/-		P.Rft			RS:14920/-

2/-	Boring for tube well in all water bearing soils from ground level up-to 100 ft or 30.5 meter depth i/c sinking and with drawing of casing pipe. (100 mm 4”) (S.I.No. 01 (b) Page No.36).

4” Dia

Qty450 rft				@Rs:242/-		P,rft			RS:9680/-

3/-	Construction of manhole  or inspection chamber  for the required dia  of circular sewer   and 3’-6’ (1067 mm ) depth with walls  of B.B  in cement mortar  1:3 cement plaster  1:3 ½”  thick in side 1”  (25 mm ) thick over  beching  and channel i/c  fixing C.I main hole cover with frame of clear opening  1 ½”x 1 ½”” 457x457 mm) of 1.75 cwt . (88.9 kg) embedded in plain c.c 1:2:4) & fixing (25 mm) dia M.S steps 6” 150 mm)   white projecting 4” (102 mm) from the face of walls at 12” (305 mm) c/c duly painted etc complete as per standard specification & drawing.(S.I No- P-46).

Qty:01 No				@Rs:14748/-		P.rft			RS:14748/-

___________________________________

PART “C” P.H.ENGR: SCHEDULE ITEM TOTAL	RS:39348/-

| PART "A) |  | ________________%Above/Below Buildings Schedule Items |  |  | Total Rs:- |  |
| --- | --- | --- | --- | --- | --- | --- |
| PART "B" |  | ________________%Above/Below W/S & S/F Schedule Items |  |  | Total Rs:- |  |
| PART "C" |  | ________________%Above/Below P.H.Engr: Schedule Items |  |  | Total Rs:- |  |
|  |  |  | G. Total Rs:- |  |  |  |
|  |  |  |  | EXECUTIVE ENGINEER |  |  |
| CONTRACTOR |  |  |  | PROVINCIAL BUILDINGS DIVISION |  |  |
|  |  |  |  | SUKKUR |  |  |

BILL OF QUANTITIES

Discription & Rate of items based on Schedule/ Market (offered Rates).

NAME OF WORK:-M/R TO   AGRICULTURE  DATE PALM RESEARCH WORK SHOP KOT DIJI @ KHAIRPUR

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

1/-	Dismantling cement concrete plain (1:2:4) etc complete (S.I no:19(c)P.NO:10).

Qty: 107  cft					@RS: 3327.50		% Cft		RS: 3560/-

2/-	Removing  cement or lime plaster (S.I No:53 P.NO:14).

Qty: 852 Sft					@RS: 121/-		% Sft		RS: 1241/-

3/-	Applying Floating Coat of cement 1/32” thick (S.I No: 14 P.NO:59).

Qty: 1286 Sft					@RS: 660/-		% Sft		RS: 8488/-

4/-	Cement Plaster 1:6 up to 20’-0’ height ½” thick. (S.I No:     P.NO:       ).

Qty: 1286 Sft					@RS: 2206.60  	% Sft		RS: 28377/-

5/-	Cement Plaster 1:4 up to 20’-0’ height 3/8” thick. (S.I No:     P.NO:       ).

Qty: 1286 Sft					@RS: 2197.52		% Sft		RS: 28260/-

6/-	Providing & Laying 1” thick Topping Cement Concrete (1:2:4) including surface finishing and dividing into panel 2” thick (S,I No:16P.No:42).

Qty: 626 Sft					@RS: 3275.50		% Sft		RS: 20505/-

7/-	First class deodar wood  wrought joinery  in doors & windows  etc fixed in position  including  hold fasts hinges  iron  tower bolts chock, cleats  handles  and  cords  with hooks etc complete.	(S.I No: 54 P.NO:  65).

Qty: 24 Sft					@RS: 1273.76		P.Sft		RS: 30570/-

8/-	Distempering 02 coats.

Qty: 968 Sft					@RS: 1043.90		% Sft		RS: 10105/-

9/-	Painting Old Surface Panting doors & window  any type 02 coats.

Qty: 192 Sft					@RS: 1160.06		% Sft		RS: 2227/-

10/-	Single Layer of tiles 9”x4 ½”x2” laid over 4” earth 1” mud plaster w/o bhoosa grouted with cement sand (1:3) on top of R.C.C slab etc complete.

Qty: 2100 Sft					@RS: 5310.35		% Sft		RS: 111517/-

11/-	Providing and laying tile glazed (6”x 6”1/4”) on floor of wall facing in required colour and pattern of STILE specification jointed in white cement and pigment over a base of 1:2 gray cement mortar ¾” thick i/c washing and filling joints with slurry of white cement and pigment in desired shape with finishing, clearing and cost of was polish etc. complete i/c cutting tiles to proper profile. Amended  (S.I. No. 60, P. No.47)

Qty: 422 Sft					@RS: 30509.77	% Sft		RS: 128751/-

Page no:2.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

12/-	Galvanized wire gauze fixed to choakats with ¾” strips (S.I no: 14 P.NO:60).

Qty: 120 Sft					@RS: 190.72		P.Sft		RS: 22886/-

13/-	Colour washing 02 coats etc.

Qty: 1430 Sft					@RS: 859.90		% Sft		RS: 12297/-

====================================

Total						RS: 408784/-

| PART "A) |  | ________________%Above/Below Buildings Schedule Items |  |  | Total Rs:- |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | G. Total Rs:- |  |  |  |
|  |  |  |  | EXECUTIVE ENGINEER |  |  |
| CONTRACTOR |  |  |  | PROVINCIAL BUILDINGS DIVISION |  |  |
|  |  |  |  | SUKKUR |  |  |

BILL OF QUANTITIES

Discription & Rate of items based on Schedule/ Market (offered Rates).

NAME OF WORK: 	Estimate for M/R to grain godown drib mehar ali shah district Khairpur.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

01/-	Dismantling C.C Plain 1:2:4 etc (S.I NO:      P-     )

Qty: 53 cft					@RS:3327.50		% Sft		RS:799/-

02/-	Cement Concrete plain including placing compacting finishing & curing, complete (including screening and washing of stone aggregate without shuttering. Ratio 1:2:4. (S.I. No. 5(f), P. No.16)

Qty: 24 cft					@RS: 14429.25	% Cft		RS: 3463/-

03/-	Removing  Cement or lime Plaster (S.I No:  53 P.No: 13  ).

Qty: 240 Sft					@RS: 121/-		% Sft		RS: 290/-

04/-	Cement Plaster 1:6 up to 20’-0 height ½” thick. ( S.I NO: 13(b)    P:NO:52   )

Qty: 240 sft					@RS:2206.60		% Sft		RS:5296/-

05/-	Cement Plaster 1:4 up to 20”-0 height 3/8” thick. . ( S.I NO: 11(a) P:NO:52   )

Qty: 240 sft					@RS: 2197.52		% Sft		RS:5274/-

06/-	Providing and laying tile glazed (6”x 6”1/4”) on floor of wall facing in required colour and pattern of STILE specification jointed in white cement and pigment over a base of 1:2 gray cement mortar ¾” thick i/c washing and filling joints with slurry of white cement and pigment in desired shape with finishing, clearing and cost of was polish etc. complete i/c cutting tiles to proper profile. Amended  (S.I. No. 60, P. No.47)

Qty: 144 Sft					@RS: 30509.77	% Sft		RS: 43934/-

07/-	Supplying & Fixing false ceiling  of plaster , in panels including making frame , work of deodar wood including painting with soligia paint.(S.NO:52 P.NO:64).

Qty: 216 Sft					@RS: 25293.42	% Sft		RS: 54634/-

08/-	Providing & Fixing plaster of pairs ceiling border of 8” -10” with of specified design & thickness i/c fixing besides ceiling with nails/screws with jetties.(S.I No:44 P.NO:39)

Qty: 84 Sft					@RS: 104.22		P.Rft		RS: 8754/-

09/-	Distempering 03 three coat (S.I No: 24(c) P.No:54 ).

Qty: 553 Sft					@RS: 1043.90		% Sft		RS: 5773/-

Page no:2.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

10/-	Painting Old Surface Painting  doors & windows  any type  03 coats. (S.I No: 4(c)P.No:68   ).

Qty: 39 Sft					@RS: 1106.70		% Sft		RS: 432/-

11/-	First class deodar wood  wrought joinery  in doors & windows  etc fixed in position  including  hold fasts hinges  iron  tower bolts chock, cleats  handles  and  cords  with hooks etc complete.	(S.I No: 54 P.NO:  65).

Qty: 29 Sft					@RS: 1273.76		P.Sft		RS: 36939/-

12/-	Providing and lying 1” thick C/C topping cement concrete (1:2:4) i/c surface finishing

and dividing into panels.  (S.I.No.16 (d) P.No.42) 3” thick.

Qty: 368 Sft					@RS: 4411.82		% Sft		RS: 16235/-

13/-	Galvanized wire gauze fixed to chowakats with ¾’  deodar strip  and screw. (S.I No: 4(c) P:58 ).

Qty: 32 Sft					@RS: 190.72		P.Sft		RS: 6103/-

========================

PART “A” BUILDINGS SCHEDULE ITEMS TOTAL 	RS:187926/-

(PART-B   Sch: Item  W/S  S/F)

1-	P/F squatting type white glazed of flushing cistern with internal fitting and flush pipe with band and making requisite Nos. of holes in wall plinth & floor for pipe connections and making good in cement concrete 1:2:4. (A) W.C pan of not less than 23” clear opening between flashing rims and 3 gallons flushing tank. With 4” dia C.I. tape. (S.I.NO.1(A) P.No.1)

Qty:01 Nos					@Rs:4802/60		each		RS:4803/-

2/-	P/F 6”x2” or 6”x3” C.I floor trap of the approved self cleaning design with a C.I screwed down gritting with or without a vent arm complete with and i/c making requisite number or holes in walls plinth and floor for pipe connections & making good in CC 1:2:4. (S.I. No. 20 P. No.5)

Qty:02 Nos					@Rs:2042/43		each		RS:4085/-

3/-	Providing & Fixing Nylon connection  complete with  ½” dia Brass stop cock  with pair  of brass  nuts  and lining  joints  to nylon  connection (S.I No:23 P-6).

Qty:02 Nos					@Rs:447/15		each		RS:894/-

4/-	Supplying and fixing long bib-cock of superior quality with C.P. head ½” dia. (S.I.NO.15(a) P. No.15)

Qty:03 Nos					@Rs:1109/46		each		RS:3328/-

5/-	Supplying and fixing concealed stopcock of superior quality with c.p head ½” dia.(S.I.NO.13 (a)

P. No.15)

Qty:02 Nos					@Rs:509.74		each		RS:1019/-

Page no:3.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

6/-	Supplying and fixing jet shower with rod of superior quality with c.p head ½” dia. (S.I.NO.17 P. No.15).

Qty:01 Nos					@Rs:1142/24		each		RS:1142/-

7/-	P/F 22”x16” lavatory basin in white glazed earthen ware complete with & i/c the cost of W.I or C.I cantilever brackets 6” built into wall painted white in two coats after a primary coat of red lead paint a pair of ½” dia chrome plate pillar traps 1-1/2” dia rubber plug & chrome plated brass chain 1-1/4” dia malleable iron or c.p brass traps malleable iron or brass union & making requisite number of holes in walls plinth & floor for pipe connection and making good in cement concrete 1:2:4 (standard pattern). Add extra for labour for providing & fixing of earthenware pedestal white or colored glazed (Standard pattern) .  (S.I.NO. 8 P. No.3).

Qty:01 No					@Rs:4694.80		each		RS:4695/-

8/-	Add extra for labour for providing & fixing of earthenware pedestal white or colored glazed (Standard pattern) .  (S.I.NO. 9 P. No.3)

Qty:01 nos					@Rs:938/47		each		RS:938/-

9/-	Providing & Fixing Swan type pillar cock etc complete

Qty:01 No					@Rs:795/-		each		RS:795/-

9/-	Providing & Fixing Handle valves  (china) etc complete

¾” Dia 		01 No				@Rs:271.92		each		RS:272/-

1” Dia		01 No				@Rs:365.42		each		RS:365/-

==================================

Total					RS:23005/-

| PART "A) |  | ________________%Above/Below Buildings Schedule Items |  |  | Total Rs:- |  |
| --- | --- | --- | --- | --- | --- | --- |
| PART "B" |  | ________________%Above/Below W/S & S/F Schedule Items |  |  | Total Rs:- |  |
|  |  |  | G. Total Rs:- |  |  |  |
|  |  |  |  | EXECUTIVE ENGINEER |  |  |
| CONTRACTOR |  |  |  | PROVINCIAL BUILDINGS DIVISION |  |  |
|  |  |  |  | SUKKUR |  |  |

BILL OF QUANTITIES

Discription & Rate of items based on Schedule/ Market (offered Rates).

NAME OF WORK: 	ESTIMATE FOR M/R TO FOOD RESIDENCE  AT SETHARJA  @ DISTRICT  KHAIRPUR.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

1/-	Dismantling Cement Concrete plain (1:2:4) (S.I No: 19 (c)P.NO:10).

Qty: 204 Cft					@RS: 3327.50		% Cft		RS: 6788/-

2/-	Removing door with Chowakats (S.I No: 33 (a)P.NO:12).

Qty: 03 NO’s					@RS: 142.18		each		RS: 427/-

3/-	Cement Plaster 1:6 up to 20’-0’ height ½” thick (S.I No:    P.NO:      ).

Qty: 1253 Sft					@RS: 2206.60		% Sft		RS: 27649/-

4/-	Cement Plaster 1:4 up to 20’-0’ height 3/8” thick (S.I No:    P.NO:      ).

Qty: 1253 Sft					@RS: 2197.52		% Sft		RS: 27535/-

5/-	First class deodar wood  wrought joinery  in doors & windows  etc fixed in position  including  hold fasts hinges  iron  tower bolts chock, cleats  handles  and  cords  with hooks etc complete.	(S.I No: 54 P.NO:  65).

Qty: 132 Sft					@RS: 1273.76		P.Sft		RS: 168136/-

6/-	Single Layer of tiles 9”x4 ½”x2” laid over 4” earth 1” mud plaster w/o bhoosa grouted with cement sand (1:3) on top of R.C.C slab etc complete.

Qty: 1077 Sft					@RS: 5310.35		% Sft		RS: 57192/-

7/-	Cement Concrete plain including placing compacting finishing & curing, complete (including screening and washing of stone aggregate without shuttering. Ratio 1:2:4. (S.I. No. 5(f), P. No.16)

Qty: 124 Cft					@RS: 14429.25	% Cft		RS: 17892/-

8/-	Providing and laying tile glazed (6”x 6”1/4”) on floor of wall facing in required colour and pattern of STILE specification jointed in white cement and pigment over a base of 1:2 gray cement mortar ¾” thick i/c washing and filling joints with slurry of white cement and pigment in desired shape with finishing, clearing and cost of was polish etc. complete i/c cutting tiles to proper profile. Amended  (S.I. No. 60, P. No.47)

Qty: 110 Sft					@RS: 30509.77	% Sft		RS: 33561/-

09/-		Providing and lying 1” thick C/C topping cement concrete (1:2:4) i/c surface finishing

and dividing into panels.  (S.I.No.16 (d) P.No.42)3” thick.

Qty: 1200 sft					@Rs:4411.82		% Sft		RS: 52942/-

10/-	Galvanized wire gauze fixed  to chowakats with ¾”   deodar strps.

Qty: 60 sft					@Rs:190.72		P.Sft		RS: 11443/-

Page no:2.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

11/-	Supplying & Fixing false ceiling  of plaster , in panels including making frame , work of deodar wood including painting with soligia paint.(S.NO:52 P.NO:64).

Qty: 108 sft					@Rs:25293.42		% Sft		RS: 27317/-

12/-	Providing & Fixing plaster of pairs ceiling border of 8” -10” with of specified design & thickness i/c fixing besides ceiling with nails/screws with jetties.(S.I No:44 P.NO:39)

Qty: 42 sft					@Rs:104.22		P.Rft		RS: 4377/-

13/-	Distempering 02 two coats (S.I NO:     P.NO     ).

Qty: 3975 Sft					@RS: 1043.90		% Sft		RS: 41495/-

14/-	Painting Doors & Windows any type 03 coats.

Qty: 268 Sft					@RS: 1270.83		% Sft		RS: 3406/-

15/-	Providing RCC Spout fixing in position etc complete.

Qty: 02 No’s					@RS: 524.98		each		RS: 1050/-

16/-	Painting guard bars gets iron bars grating , railing including standard braces (etc) and similar open work. (S.I NO:4(d)P.NO:69).

Qty: 904 Sft					@RS: 1270.83		% Sft		RS: 11488/-

=============================

PART”A” BUILDINGS SCHEDULE ITEM TOTAL RS: 492698/-

| PART "A) |  | ________________%Above/Below Buildings Schedule Items |  |  | Total Rs:- |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | G. Total Rs:- |  |  |  |
|  |  |  |  | EXECUTIVE ENGINEER |  |  |
| CONTRACTOR |  |  |  | PROVINCIAL BUILDINGS DIVISION |  |  |
|  |  |  |  | SUKKUR |  |  |

BILL OF QUANTITIES

Discription & Rate of items based on Schedule/ Market (offered Rates).

NAME OF WORK: 	ESTIMATE FOR M/R TO  CENTRAL JAIL  AT DISTRICT KHAIRPUR (Office Block)

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

1/-	Removing  cement or lime plaster etc complete

Qty:4314 sft					@Rs:121/-		% sft		RS:5220/-

2/-	Dismantling C.C Plain (1:2:4). (S.I No:    P.NO:   ).

Qty:  cft					@Rs:3327.50		% cft		RS:15273/-

3/-	Scraping Ordinary distemper oil bond distemper as paint on wall.

Qty: 3753 sft					@Rs:226.88		% Sft		RS: 8515/-

4/-	Applying Floating coat of cement 1/32” thick etc complete

Qty:4314 sft					@Rs:660/-		% sft		RS:28472/-

5/-	Cement Plaster 1:6 up to 20’-0 height ½” thick. ( S.I NO: 13(b)    P:NO:52   )

Qty:4314 sft					@Rs:2206.60		% sft		RS:95193/-

6/-	Cement Plaster 1:4 up to 20’-0’ height 3/8” thick.

Qty:4314 sft					@Rs:2197.52		% Sft		RS:94801/-

7/-	Pacca Brick work in Ground Floor in cement sand mortar Ratio (1:60 (S.I No:   P.    ).

Qty:54 cft					@Rs:12674.36		% cft		RS:6844/-

8/-	Cement Concrete plain including placing compacting finishing & curing, complete (including screening and washing of stone aggregate without shuttering. Ratio 1:2:4. (S.I. No. 5(f), P. No.16)

Qty: 459 cft					@Rs:14429.25		% cft		RS: 66230/-

9/-	Providing and laying tile glazed (6”x 6”1/4”) on floor of wall facing in required colour and pattern of STILE specification jointed in white cement and pigment over a base of 1:2 gray cement mortar ¾” thick i/c washing and filling joints with slurry of white cement and pigment in desired shape with finishing, clearing and cost of was polish etc. complete i/c cutting tiles to proper profile. Amended  (S.I. No. 60, P. No.47)

Qty:2639 sft					@Rs:30509.77		% Sft		RS:805152/-

10/-	Supplying & Fixing false ceiling  of plaster , in panels including making frame , work of deodar wood including painting with soligia paint.(S.NO:52 P.NO:64).

Qty:1328 sft					@Rs:25293.42		% sft		RS:335897/-

Page no:2.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

11/-	Providing & Fixing plaster of pairs ceiling border of 8” -10” with of specified design & thickness i/c fixing besides ceiling with nails/screws with jetties.(S.I No:44 P.NO:39)

Qty:266 rft					@Rs:104.22		P.Rft		RS:27723/-

12/-	Providing & Fixing Flase ceiling of Gypsem board in panel of required design & size including frame  work of aluminum tee section with nails to ceiling etc complete.

Qty:335 sft					@Rs:172/-		P.Sft		RS:57620/-

13/-	Supplying & Fixing in position Aluminum channel framing for hinged doors or Alcop made with 5 mm thick tinted glass glazing (Belgium) and Alpha (Japan) locks including handles stoppers etc (S.I NO:83 P.NO:108)

Qty:251 sft					@Rs:1507.66		P.sft		RS:378423/-

14/-	Laying white marble flooring fine dressed in the surface without winding set in lime mortar 1:2 i/c rubbing and polishing pf the joints (a) ¾” thick flooring. (S.I.NO.28 (a) Page No. 43)

Qty: 894 sft					@Rs:567.48		P.Sft		RS: 507327/-

15/-	Laying white marble dado fine dressed in the surface without winding set in lime

mortar 1:2 i/c rubbing and polishing pf the joints (a) ¾” thick flooring. (S.I.NO.28 (a)

Page No. 43

Qty:554 sft					@RS:186.04		P.sft		RS:103066/-

16/-	Distempering 03 coats.

Qty:2803 sft					@RS:1079.65		% sft		RS:30263/-

17/-	Preparing  the surface   painting  with matt finish  paint of approved  make  to old  matt finish  surface  02 coat

Qty:6255 sft					@Rs:2499.76		% sft		RS:156360/-

18/-	Painting Old Surface Painting Doors & Windows any type 02 coats.

Qty:140 sft					@Rs:1160.06		% sft		RS:1624/-

19/-	Single Layer of tiles 9”x4 ½”x2” laid over 4” earth 1” mud plaster w/o bhoosa grouted with cement sand (1:3) on top of R.C.C slab etc complete.

Qty:3456 sft					@Rs:5310.35		% sft		RS:183526/-

20/-	Providing & Fixing Ornamental C.C Jalli etc complete 2” thic.

Qty:288 sft					@Rs:226.02		P.sft		RS:65094/-

21/-	P/F cement paving blocks flooring having size of 197x97x80(mm)of cuty/ quddra / cobble shape with natural colours, having strength b/w 5000 psi to 8500 psi i/c filling the joints with hill sand and laying in specified manner / pattern and design etc. complete. (S.I. No.74 P. No.50).

Qty:  1080 Sft					@RS:223.97		P.Sft		RS: 241889/-

==================================

PART “A” BUILDINGS SCHEDULE ITEMS TOTAL	RS: 3214511/-

Page no:3.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

| PART "A) |  | ________________%Above/Below Buildings Schedule Items |  |  | Total Rs:- |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | G. Total Rs:- |  |  |  |
|  |  |  |  | EXECUTIVE ENGINEER |  |  |
| CONTRACTOR |  |  |  | PROVINCIAL BUILDINGS DIVISION |  |  |
|  |  |  |  | SUKKUR |  |  |

BILL OF QUANTITIES

Discription & Rate of items based on Schedule/ Market (offered Rates).

NAME OF WORK: 	ESTIMATE FOR M/R TO  CENTRAL JAIL  AT DISTRICT KHAIRPUR ( Repair of Supdtt:  B’low)

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

1/-	Excavation in foundation of building bridges and other structures i/c dag balling dressing refilling around structure with excavated earth watering and ramming lead up-to 5ft. (b) in ordinary soil. (S.I.No.18 (b) P.No.4)

Qty: 1100 cft					@RS: 3176.25		% Cft		RS: 3494/-

2/-	Cement concrete stone of brick ballast 1½” to 2” gauge. Ratio (1:5:10). (S.I.No.4 (c) P.No.15).

Qty: 275 cft					@RS: 8694.95		% Cft		RS: 23911/-

3/-	Random rubble masonry (un-coursed) (d) in cement sand mortar (iv) Ratio (S.I.No.1 (d)(iv) P.No.32).

Qty: 540 cft					@RS: 17723/-		% Cft		RS: 95704/-

4/-	Coursed rubble masonry including hammer dressing (d) in cement sand mortar (iv) Ratio 1:6.(S.I.No.2 (d)(iv) P.No.32)

Qty: 450 cft					@RS: 25321/-		% Cft		RS: 113945/-

5/-	Pacca Brick work in  foundation / Plinth  in cement sand  mortar Ratio 1:6 (S.I No:   P.NO:    ).

Qty: 59 cft					@RS: 11948.36	% Cft		RS: 7050/-

6/-	Fabrication of mild steel reinforcement for cement concrete i/c cutting bending laying in position making joints and fastenings i/c cost of binding wire (also includes removal of rust from bars) (a) Using tor bars. (S.I.No.7 (a) P.No.20)

P.Beam

Qty: 8.12 cwt					@RS: 5001.70		P.cwt		RS: 40614/-

7/-	R.C.C. work in roof slab beams columns rafts lintels and other structural members laid in situ or precast laid in position complete in all respects: (i) Ratio (1:2:4) 90 lbs cement 2 cft sand 4 cft shingle 1/8” to 1/4” gauge. (S.I.No.6 (a-i) P.No. 17)

Qty: 150 cft					@RS: 337/-		P.Cft		RS: 50550/-

8/-	Pacca Brick work in Other than building Ratio 1:6 etc complete

Qty: 1584 cft					@RS: 12346.85	% Cft		RS: 195571/-

9/-	Cement Plaster 1:6 up to 20’-0 height ½” thick. ( S.I NO: 13(b)    P:NO:52   )

Qty: 4225 sft					@RS: 2206.60		% Sft		RS: 93229/-

10/-	Cement Plaster 1:4 up to 20’-0’ height 3/8” thick.

Qty: 4225 sft					@RS: 2197.52		% Sft		RS: 92845/-

11/-	Cement Pointing on Stone work (Raised).

Qty: 720 sft					@RS: 1758.08		% Sft		RS: 12658/-

Page no:2.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

12/-	Supplying & Fixing Sand under floor plugging into wall.

Qty: 1872 cft					@RS: 1913.21		% Sft		RS: 35815/-

13/-	Cement Concrete plain including placing compacting finishing & curing, complete (including screening and washing of stone aggregate without shuttering. Ratio 1:2:4.(S.I.No.5(f),P. No.16)

Qty: 318 cft					@RS: 14429.25	% Cft		RS: 45885/-

14/-	Providing and laying tile glazed (6”x 6”1/4”) on floor of wall facing in required colour and pattern of STILE specification jointed in white cement and pigment over a base of 1:2 gray cement mortar ¾” thick i/c washing and filling joints with slurry of white cement and pigment in desired shape with finishing, clearing and cost of was polish etc. complete i/c cutting tiles to proper profile. Amended  (S.I. No. 60, P. No.47)

Qty: 248 sft					@Rs:30509.77		% Sft		RS: 75664/-

15/-	Distempering 03 coats.

Qty: 560 Sft					@RS: 1043.90		% Sft		RS: 13988/-

16/-	Painting Old Surface Painting Doors & Windows any type 02 coats.

Qty: 191 Sft					@RS: 1270.83		% Sft		RS: 2430/-

17/-	Glavanized wire gauze fixed to chowakats etc complete

Qty: 160 Sft					@RS: 190.72		P.Sft		RS: 30835/-

=================================

PART “A” BUILDINGS SCHEDULE ITEMS TOTAL 	RS: 934188/-

PART “B” W/S & S/F SCHEDULE ITEMS.

1/-	Laying PVC pipes of class “B” fixing in trench including cutting , fitting, & jointing with solvent  cement including  testing with water pressure  head of 61 mm or 200 ft:

Qty: 75 Rft					@RS: 136/-		P.Rft		RS: 10200/-

============

PART “B” W/S & S/F SCHEDULE ITEMS TOTAL	RS: 10200/-

| PART "A) |  | ________________%Above/Below Buildings Schedule Items |  |  | Total Rs:- |  |
| --- | --- | --- | --- | --- | --- | --- |
| PART "B" |  | ________________%Above/Below W/S & S/F Schedule Items |  |  | Total Rs:- |  |
|  |  |  |  |  | G. Total Rs:- |  |
|  |  |  |  | EXECUTIVE ENGINEER |  |  |
| CONTRACTOR |  |  |  | PROVINCIAL BUILDINGS DIVISION |  |  |
|  |  |  |  | SUKKUR |  |  |

BILL OF QUANTITIES

Discription & Rate of items based on Schedule/ Market (offered Rates).

NAME OF WORK: 	Estimate for M/R to  T.H.Q Hospital Gambat District Khairpur. ( Indor  Block)

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

1/-	Dismantling brick work in cement or lime mortar etc .(S.I No:     P.NO:    )

Qty: 08 cft					@RS: 1285.63		% Cft		RS: 103/-

2/-	Dismantling glazed or encaustic tiles etc (S.I No:55 P.NO:13).

Qty: 344 Sft					@RS: 786.50		% Sft		RS; 2706/-

3/-	Removing door with Chowakats (S.I No: 33 (a)P.NO:120.

Qty: 812 Sft					@RS: 121/-		% Sft		RS: 983/-

4/	Fabrication of mild steel reinforcement for cement concrete i/c cutting bending laying

in position making joints and fastenings i/c cost of binding wire (also includes removal

of rust from bars) (a) Using tor bars. (S.I.No.7 (a) P.No.20)

lintel

Qty: 0.370 cwt					@RS: 5001.70		P.cwt		RS: 1851/-

5/-	R.C.C. work in roof slab beams columns rafts lintels and other structural members laid

in situ or precast laid in position complete in all respects: (i) Ratio (1:2:4) 90 lbs

cement 2 cft sand 4 cft shingle 1/8” to 1/4” gauge. (S.I.No.6 (a-i) P.No. 19)

lintel

Qty: 05 cft					@RS: 337/-		P.Cft		RS: 1685/-

6/-	Pacca Brick work in ground floor in cement sand mortar Ratio (1:60 (S.I No:   P.    ).

Qty: 08 cft					@RS: 12674.36	% Cft		RS: 1014/-

7/-	Cement Concrete brick or Stone Ballast 1” to 2”-0 gauge Ratio (1:6:12) (S.I No: P.NO:    )

Qty: 35 cft					@RS: 8122.95		% Cft		RS: 2843/-

8/-	Cement Concrete plain including placing compacting finishing & curing, complete (including screening and washing of stone aggregate without shuttering. Ratio 1:2:4. (S.I. No. 5(f), P. No.16)

Qty: 18 cft					@RS: 14429.25	% Cft		RS: 2597/-

9/-	Applying Floating coat of cement  1/32” thick.

Qty: 812 Sft					@RS: 660/-		% Sft		RS: 5359/-

10/-	Cement Plaster 1:6 up to 20’-0’ height ½” thick (S.I No:    P.NO:      ).

Qty: 812 Sft					@RS: 2206.60		% Sft		RS: 17918/-

11/-	Cement Plaster 1:4 up to 20’-0’ height 3/8” thick (S.I No:    P.NO:      ).

Qty: 541 Sft					@RS: 2197.52		% Sft		RS: 11889/-

Page no:2.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

12/-	Providing and fixing G.I frames/chowkhats of size 7”x2” or 4½”x3” for door using 20 gauge G.I sheet i/c welded hinges and fixing at site with necessary hold fasts, filling with cement sand slurry of ratio 1:6 and repairing the jambs. The cost also i/c all carriage, tools and plants used in making and fixing.   (S.I.No. 29 Page No. 98

Qty: 50 Rft					@RS: 228.90		P.Sft		RS: 11445/-

13/-	Providing and laying tile glazed (6”x 6”1/4”) on floor of wall facing in required colour and pattern of STILE specification jointed in white cement and pigment over a base of 1:2 gray cement mortar ¾” thick i/c washing and filling joints with slurry of white cement and pigment in desired shape with finishing, clearing and cost of was polish etc. complete i/c cutting tiles to proper profile. Amended  (S.I. No. 60, P. No.47)

Qty:  1470 Sft					@RS: 30509.77	% Sft		RS: 448494/-

14/-	First class deodar wood  wrought joinery  in doors & windows  etc fixed in position

including  hold fasts hinges  iron  tower bolts chock, cleats  handles  and  cords  with

hooks etc complete.	(S.I No: 54 P.NO:  65).

Qty: 88 Sft					@RS: 902.93		% Sft		RS:  112091/-

15/-	Distempering 02 coats

Qty: 105 Sft					@RS: 1043.90		% Sft		RS: 1096/-

16/-		Preparing the surface and painting with mat finish paint of approved make to old matt

finish 	surface paris and then painting with weather coat of approved make. (03 Coats)

(old surface) (S.I. No. 37  (a+b)  P. No.61)

Qty: 485 Sft					@RS: 2499.76		% sft		RS: 12124/-

17/-	Painting New Surface Painting Doors & Windows any type 03 coats.

Qty: 112 Sft					@RS: 2116.41		% Sft		RS: 2370/-

============================

PART “A” BUILDINGS SCHEDULE ITEM TOTAL 	RS: 636568/-

PART-   B  WATER SUPPLY & SANITRY FITTING.

01/- 	P/F 24” x 18” beveled edge mirror of Belgium glass complete with 1/8” thick hard board and C.P screw fixed to wooden plate (a) standard pattern.  (S.I.No. 03 (a) P.No7).

Qty:01 Nos					@Rs:1711/60		each		RS:1712/-

02	S/F in position c.p stopcock (b)½” dia C.P stop cock standard pattern. (S.I. No. 4 (b) P.

No.13)

Qty:02 Nos					@Rs:843/92		each		RS: 2219/-

___________

PART “B” W/S & S/F SCHEDULE ITEM TOTAL	RS: 3931/-

Page no:3.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

PART “C”  NON SHEDULE ITEM.

1/-	P/F UPVC pipe ½” dia PAK-ARAB) SCH-40 surface by using clip /saddler/socket/reducer/bush/tee etc .or recessed in masonry C.C or R.C.C upto 20’ ft. height and making good wih C.c i/c curing finishing etc complete as per instruction of the Engineer In charge ,.Specification of the material should meet the requirement of class 12454-Bin accordance with the ASTM-D-1784 type 1 Grade 1. rate include all costs of labor ,material, cartage scaffolding ladders .etc complete (R.A Sanction).

Qty:20 rft							P.rft

02/-	P/F UPVC pipe ¾” dia PAK-ARAB) SCH-40 surface by using clip /saddler/socket/reducer/bush/tee / elbow etc .or recessed in masonry C.C or R.C.C upto 20’ ft. height and making good wih C.c i/c curing rinishing etc complete as per instruction of the Engineer In charge ,.Specification of the material should meet the requirement of class 12454-Bin accordance with the ASTM-D-1784 type 1 Grade 1. rate include all costs of labor ,material, cartage scaffolding ladders .etc complete (R.A Sanction).

Qty:30 rft							P.rft

03/-	P/F UPVC UNION  ¾” dia PAK-ARAB) SCH-40  make approved quality and Design of various sizes Fixed to UPVC Pipe / Fitting paid Separately  upto 20’ ft. height as per instruction Sof the Engineer In charge Specification of the material should meet the requirement of class 12454-Bin accordance with the ASTM-D-1784 type 1 Grade 1. rate include all costs of labor ,material, cartage scaffolding ladders .etc complete (R.A Sanction).

Qty:02 Nos							each

04/-	P/F UPVC pipe 4” dia PAK-ARAB) SCH-40 surface by using clip /saddler/socket/reducer/bush/tee /  etc . or recessed in masonry C.C or R.C.C upto 20’ ft. height and making good wih C.C i/c curing finishing etc complete as per instruction of the Engineer In charge ,.Specification of the material should meet the requirement of class 12454-Bin accordance with the ASTM-D-1784 type 1 Grade 1. rate include all costs of labor ,material, cartage scaffolding ladders .etc complete (R.A Sanction).

Qty:10 rft							P.rft

_____________________

PART “C” NON SCHEDULE ITEMS 	TOTAL RS:-

| PART "A) |  | ________________%Above/Below Buildings Schedule Items |  |  | Total Rs:- |  |
| --- | --- | --- | --- | --- | --- | --- |
| PART "B" |  | ________________%Above/Below W/S & S/F Schedule Items |  |  | Total Rs:- |  |
| PART "E: |  | ___________________________________Non Schedule Items |  |  | Total Rs:- |  |
|  |  |  | G. Total Rs:- |  |  |  |
|  |  |  |  | EXECUTIVE ENGINEER |  |  |
| CONTRACTOR |  |  |  | PROVINCIAL BUILDINGS DIVISION |  |  |
|  |  |  |  | SUKKUR |  |  |

BILL OF QUANTITIES

Discription & Rate of items based on Schedule/ Market (offered Rates).

NAME  OF  WORK:-	Estimate for M/R to  STAFF Qtr No: 1 Assistant Superintdent Jail Khairpur.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

1/-	Dismantling cement concrete plain (1:2:4) etc complete (S.I no:19(c)P.NO:10)/.

Qty:99 Cft					@Rs:3327.50		% Cft		RS:3294/-

2/-	Removing  cement or lime plaster (S.I No:53 P.NO:14).

Qty:386 Sft					@Rs:121/-		% Sft		RS:1120/-

3/-	Dismantling Glazed tile or encaustic tiles etc complete .

Qty:188 Sft					@Rs:786.50		% Sft		RS:1140/-

4/-	Rehandling  glazed or encaustic tiles  etc .

Qty:1050 Cft					@Rs:1085.75		% Cft		RS:1140/-

5/-	Applying Floating Coat of cement 1/32” thick (S.I No: 14 P.NO:59).

Qty:926 sft					@Rs:660/-		% Sft		RS:6112/-

6/-	Cement Plaster 1:6 up to 20’-0’ height ½” thick. (S.I No:     P.NO:       ).

Qty:926 sft					@Rs:2206.60		% Sft		RS:20433/-

7/-	Cement Plaster 1:4 up to 20’-0’ height 3/8” thick. (S.I No:     P.NO:       ).

Qty:926 sft					@Rs:2197.52		% Sft		RS:20349/-

8/-	Providing & Laying 1” thick Topping Cement Concrete (1:2:4) including surface finishing and dividing into panel 2” thick (S,I No:16P.No:42).

Qty:582 sft					@RS:3275.50		% sft		RS:19063/-

9/-	First class deodar wood  wrought joinery  in doors & windows  etc fixed in position  including  hold fasts hinges  iron  tower bolts chock, cleats  handles  and  cords  with hooks etc complete.

(S.I No: 54 P.NO:  65).

Qty: 109 sft					@RS:1273.76		P.Sft		RS: 138840/-

10/-	Providing and laying tile glazed (6”x 6”1/4”) on floor of wall facing in required colour and pattern of STILE specification jointed in white cement and pigment over a base of 1:2 gray cement mortar ¾” thick i/c washing and filling joints with slurry of white cement and pigment in desired shape with finishing, clearing and cost of was polish etc. complete i/c cutting tiles to proper profile. Amended  (S.I. No. 60, P. No.47)

Qty: 660 sft					@RS:30509.77		% Sft		RS: 201364/-

11/-	Single Layer of tiles 9”x4 ½”x2” laid over 4” earth 1” mud plaster w/o bhoosa grouted with cement sand (1:3) on top of R.C.C slab etc complete.

Qty:1000 sft					@RS:5310.35		% Sft		RS:53104/-

12/-	Distempering 02 coats .

Qty:1444 sft					@RS:1043.90		% Sft		RS:10574/-

Page no:2.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

13/-	Painting Old Surface Painting door & windows any type .

Qty:236 sft					@RS:1160.06		% sft		RS:2738/-

14/-	Supplying & Fixing false ceiling  of plaster , in panels including making frame , work of deodar wood including painting with soligia paint.(S.NO:52 P.NO:64).

Qty: 364 Sft					@Rs:25293.42		% Sft		RS: 92068/-

15/-	Providing & Fixing plaster of pairs ceiling border of 8” -10” with of specified design & thickness i/c fixing besides ceiling with nails/screws with jetties.(S.I No:44 P.NO:39)

Qty:108 Rft					@Rs:104.22		P.Rft		RS: 11256/-

16/-	Glavanized wire gauze fixed to chowakats including fixing in position.

Qty: 16 Sft					@RS: 190.72		P.Sft		RS: 3052/-

====================================

BUILDINGS SCHEDULE ITEMS 	TOTAL RS: 584947/-

| PART "A) |  | ________________%Above/Below Buildings Schedule Items |  |  | Total Rs:- |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | G. Total Rs:- |  |  |  |
|  |  |  |  | EXECUTIVE ENGINEER |  |  |
| CONTRACTOR |  |  |  | PROVINCIAL BUILDINGS DIVISION |  |  |
|  |  |  |  | SUKKUR |  |  |

BILL OF QUANTITIES

Discription & Rate of items based on Schedule/ Market (offered Rates).

NAME OF WORK:-	M/R TO   GOVERNMENT STAFF QUARTER COLONY @ KHAIRPUR BLOCK NO; 2,4,3  QTRS  NO: 1,3,7,8.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

1/-	Dismantling cement concrete plain (1:2:4) etc complete (S.I no:19(c)P.NO:10).

Qty: 107  cft					@RS: 3327.50		% Cft		RS: 3560/-

2/-	Removing  cement or lime plaster (S.I No:53 P.NO:14).

Qty: 852 Sft					@RS: 121/-		% Sft		RS: 1241/-

3/-	Applying Floating Coat of cement 1/32” thick (S.I No: 14 P.NO:59).

Qty: 1286 Sft					@RS: 660/-		% Sft		RS: 8488/-

4/-	Cement Plaster 1:6 up to 20’-0’ height ½” thick. (S.I No:     P.NO:       ).

Qty: 1286 Sft					@RS: 2206.60  	% Sft		RS: 28377/-

5/-	Cement Plaster 1:4 up to 20’-0’ height 3/8” thick. (S.I No:     P.NO:       ).

Qty: 1286 Sft					@RS: 2197.52		% Sft		RS: 28260/-

6/-	Providing & Laying 1” thick Topping Cement Concrete (1:2:4) including surface finishing and dividing into panel 2” thick (S,I No:16P.No:42).

Qty: 626 Sft					@RS: 3275.50		% Sft		RS: 20505/-

7/-	First class deodar wood  wrought joinery  in doors & windows  etc fixed in position  including  hold fasts hinges  iron  tower bolts chock, cleats  handles  and  cords  with hooks etc complete.	(S.I No: 54 P.NO:  65).

Qty: 72 Sft					@RS: 1273.76		P.Sft		RS: 109543/-

8/-	Distempering 02 coats.

Qty: 968 Sft					@RS: 1043.90		% Sft		RS: 10105/-

9/-	Painting Old Surface Panting doors & window  any type 02 coats.

Qty: 192 Sft					@RS: 1160.06		% Sft		RS: 2227/-

10/-	Single Layer of tiles 9”x4 ½”x2” laid over 4” earth 1” mud plaster w/o bhoosa grouted with cement sand (1:3) on top of R.C.C slab etc complete.

Qty: 2100 Sft					@RS: 5310.35		% Sft		RS: 111517/-

11/-	Providing and laying tile glazed (6”x 6”1/4”) on floor of wall facing in required colour and pattern of STILE specification jointed in white cement and pigment over a base of 1:2 gray cement mortar ¾” thick i/c washing and filling joints with slurry of white cement and pigment in desired shape with finishing, clearing and cost of was polish etc. complete i/c cutting tiles to proper profile. Amended  (S.I. No. 60, P. No.47)

Qty: 328 Sft					@RS: 30509.77	% Sft		RS: 100072/-

12/-	Galvanized wire gauze fixed to choakats with ¾” strips (S.I no: 14 P.NO:60).

Qty: 120 Sft					@RS: 190.72		P.Sft		RS: 22886/-

13/-	Colour washing 02 coats etc.

Qty: 1430 Sft					@RS: 859.90		% Sft		RS: 12297/-

Page no:2.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

14/-	Making & Fixing Steel Graed Door complete with locking arengement iron frame 2”x2”x3/8” and ¾” sq: bars 4” center to center (S.I NO: 23 P-97).

Qty: 35 Sft					@RS: 726.7		P.Sft		RS: 25435/-

====================================

PART “A” BUILDINGS SCHEDUL ITEMS TOTAL 	RS: 484513/-

| PART "A) |  | ________________%Above/Below Buildings Schedule Items |  |  | Total Rs:- |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | G. Total Rs:- |  |  |  |
|  |  |  |  | EXECUTIVE ENGINEER |  |  |
| CONTRACTOR |  |  |  | PROVINCIAL BUILDINGS DIVISION |  |  |
|  |  |  |  | SUKKUR |  |  |

BILL OF QUANTITIES

Discription & Rate of items based on Schedule/ Market (offered Rates).

NAME OF WORK:-	M/R TO   GOVERNMENT STAFF QUARTER COLONY @ KHAIRPUR BLOCK NO; 4  QTRS  NO: 4.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

1/-	Dismantling cement concrete plain (1:2:4) etc complete (S.I no:19(c)P.NO:10).

Qty: 107  cft					@RS: 3327.50		% Cft		RS: 3560/-

2/-	Removing  cement or lime plaster (S.I No:53 P.NO:14).

Qty: 852 Sft					@RS: 121/-		% Sft		RS: 1241/-

3/-	Applying Floating Coat of cement 1/32” thick (S.I No: 14 P.NO:59).

Qty: 1286 Sft					@RS: 660/-		% Sft		RS: 8488/-

4/-	Cement Plaster 1:6 up to 20’-0’ height ½” thick. (S.I No:     P.NO:       ).

Qty: 1286 Sft					@RS: 2206.60  	% Sft		RS: 28377/-

5/-	Cement Plaster 1:4 up to 20’-0’ height 3/8” thick. (S.I No:     P.NO:       ).

Qty: 1286 Sft					@RS: 2197.52		% Sft		RS: 28260/-

6/-	Providing & Laying 1” thick Topping Cement Concrete (1:2:4) including surface finishing and dividing into panel 2” thick (S,I No:16P.No:42).

Qty: 626 Sft					@RS: 3275.50		% Sft		RS: 20505/-

7/-	First class deodar wood  wrought joinery  in doors & windows  etc fixed in position  including  hold fasts hinges  iron  tower bolts chock, cleats  handles  and  cords  with hooks etc complete.	(S.I No: 54 P.NO:  65).

Qty: 72 Sft					@RS: 1273.76		P.Sft		RS: 109543/-

8/-	Distempering 02 coats.

Qty: 968 Sft					@RS: 1043.90		% Sft		RS: 10105/-

9/-	Painting Old Surface Panting doors & window  any type 02 coats.

Qty: 192 Sft					@RS: 1160.06		% Sft		RS: 2227/-

10/-	Single Layer of tiles 9”x4 ½”x2” laid over 4” earth 1” mud plaster w/o bhoosa grouted with cement sand (1:3) on top of R.C.C slab etc complete.

Qty: 2100 Sft					@RS: 5310.35		% Sft		RS: 111517/-

11/-	Providing and laying tile glazed (6”x 6”1/4”) on floor of wall facing in required colour and pattern of STILE specification jointed in white cement and pigment over a base of 1:2 gray cement mortar ¾” thick i/c washing and filling joints with slurry of white cement and pigment in desired shape with finishing, clearing and cost of was polish etc. complete i/c cutting tiles to proper profile. Amended  (S.I. No. 60, P. No.47)

Qty: 328 Sft					@RS: 30509.77	% Sft		RS: 100072/-

Page no:2.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

12/-	Galvanized wire gauze fixed to choakats with ¾” strips (S.I no: 14 P.NO:60).

Qty: 120 Sft					@RS: 190.72		P.Sft		RS: 22886/-

13/-	Colour washing 02 coats etc.

Qty: 1430 Sft					@RS: 859.90		% Sft		RS: 12297/-

14/-	Making & Fixing Steel Graed Door complete with locking arengement iron frame 2”x2”x3/8” and ¾” sq: bars 4” center to center (S.I NO: 23 P-97).

Qty: 35 Sft					@RS: 726.7		P.Sft		RS: 25435/-

====================================

PART “A” BUILDINGS SCHEDULE ITEMS TOTAL 	RS: 484513/-

| PART "A) |  | ________________%Above/Below Buildings Schedule Items |  |  | Total Rs:- |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | G. Total Rs:- |  |  |  |
|  |  |  |  | EXECUTIVE ENGINEER |  |  |
| CONTRACTOR |  |  |  | PROVINCIAL BUILDINGS DIVISION |  |  |
|  |  |  |  | SUKKUR |  |  |

BILL OF QUANTITIES

Discription & Rate of items based on Schedule/ Market (offered Rates).

NAME OF WORK:-	M/R TO   GOVERNMENT STAFF QUARTER COLONY @ KHAIRPUR BLOCK NO;   QTRS  NO: 2,5,7.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

1/-	Dismantling cement concrete plain (1:2:4) etc complete (S.I no:19(c)P.NO:10).

Qty: 107  cft					@RS: 3327.50		% Cft		RS: 3560/-

2/-	Removing  cement or lime plaster (S.I No:53 P.NO:14).

Qty: 852 Sft					@RS: 121/-		% Sft		RS: 1241/-

3/-	Applying Floating Coat of cement 1/32” thick (S.I No: 14 P.NO:59).

Qty: 1286 Sft					@RS: 660/-		% Sft		RS: 8488/-

4/-	Cement Plaster 1:6 up to 20’-0’ height ½” thick. (S.I No:     P.NO:       ).

Qty: 1286 Sft					@RS: 2206.60  	% Sft		RS: 28377/-

5/-	Cement Plaster 1:4 up to 20’-0’ height 3/8” thick. (S.I No:     P.NO:       ).

Qty: 1286 Sft					@RS: 2197.52		% Sft		RS: 28260/-

6/-	Providing & Laying 1” thick Topping Cement Concrete (1:2:4) including surface finishing and dividing into panel 2” thick (S,I No:16P.No:42).

Qty: 626 Sft					@RS: 3275.50		% Sft		RS: 20505/-

7/-	First class deodar wood  wrought joinery  in doors & windows  etc fixed in position  including  hold fasts hinges  iron  tower bolts chock, cleats  handles  and  cords  with hooks etc complete.	(S.I No: 54 P.NO:  65).

Qty: 72 Sft					@RS: 1273.76		P.Sft		RS: 109543/-

8/-	Distempering 02 coats.

Qty: 968 Sft					@RS: 1043.90		% Sft		RS: 10105/-

9/-	Painting Old Surface Panting doors & window  any type 02 coats.

Qty: 192 Sft					@RS: 1160.06		% Sft		RS: 222

10/-	Single Layer of tiles 9”x4 ½”x2” laid over 4” earth 1” mud plaster w/o bhoosa grouted with cement sand (1:3) on top of R.C.C slab etc complete.

Qty: 2100 Sft					@RS: 5310.35		% Sft		RS: 111517/-

11/-	Providing and laying tile glazed (6”x 6”1/4”) on floor of wall facing in required colour and pattern of STILE specification jointed in white cement and pigment over a base of 1:2 gray cement mortar ¾” thick i/c washing and filling joints with slurry of white cement and pigment in desired shape with finishing, clearing and cost of was polish etc. complete i/c cutting tiles to proper profile. Amended  (S.I. No. 60, P. No.47)

Qty: 328 Sft					@RS: 30509.77	% Sft		RS: 100072/-

12/-	Galvanized wire gauze fixed to choakats with ¾” strips (S.I no: 14 P.NO:60).

Qty: 120 Sft					@RS: 190.72		P.Sft		RS: 22886/-

13/-	Colour washing 02 coats etc.

Qty: 1430 Sft					@RS: 859.90		% Sft		RS: 12297/-

Page no:2.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

14/-	Making & Fixing Steel Graed Door complete with locking arengement iron frame 2”x2”x3/8” and ¾” sq: bars 4” center to center (S.I NO: 23 P-97).

Qty: 35 Sft			@RS: 726.7			P.Sft		RS: 25435/-

====================================

PAR T “A” BUILDINGS SCHEDULE ITEMS TOTAL 	RS: 484513/-

| PART "A) |  | ________________%Above/Below Buildings Schedule Items |  |  | Total Rs:- |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | G. Total Rs:- |  |  |  |
|  |  |  |  | EXECUTIVE ENGINEER |  |  |
| CONTRACTOR |  |  |  | PROVINCIAL BUILDINGS DIVISION |  |  |
|  |  |  |  | SUKKUR |  |  |

BILL OF QUANTITIES

Discription & Rate of items based on Schedule/ Market (offered Rates).

NAME OF WORK: 	Estimate for M/R to  ASSISTANT ENGINEER PROVINCIAL BUILDINGS SUB-DIVISION OFFICE KHAIRPUR.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

1/-	Dismantling brick work in cement or lime mortar etc .(S.I No:     P.NO:    )

Qty: 08 cft					@RS: 1285.63		% Cft		RS: 103/-

2/-	Dismantling glazed or encaustic tiles etc (S.I No:55 P.NO:13).

Qty: 344 Sft					@RS: 786.50		% Sft		RS; 2706/-

3/-	Removing door with Chowakats (S.I No: 33 (a)P.NO:120.

Qty: 812 Sft					@RS: 121/-		% Sft		RS: 983/-

4/	Fabrication of mild steel reinforcement for cement concrete i/c cutting bending laying

in position making joints and fastenings i/c cost of binding wire (also includes removal

of rust from bars) (a) Using tor bars. (S.I.No.7 (a) P.No.20)

lintel

Qty: 0.370 cwt					@RS: 5001.70		P.cwt		RS: 1851/-

5/-	R.C.C. work in roof slab beams columns rafts lintels and other structural members laid

in situ or precast laid in position complete in all respects: (i) Ratio (1:2:4) 90 lbs

cement 2 cft sand 4 cft shingle 1/8” to 1/4” gauge. (S.I.No.6 (a-i) P.No. 19)

lintel

Qty: 05 cft					@RS: 337/-		P.Cft		RS: 1685/-

6/-	Pacca Brick work in ground floor in cement sand mortar Ratio (1:60 (S.I No:   P.    ).

Qty: 08 cft					@RS: 12674.36	% Cft		RS: 1014/-

7/-	Cement Concrete brick or Stone Ballast 1” to 2”-0 gauge Ratio (1:6:12) (S.I No: P.NO:    )

Qty: 35 cft					@RS: 8122.95		% Cft		RS: 2843/-

8/-	Cement Concrete plain including placing compacting finishing & curing, complete (including screening and washing of stone aggregate without shuttering. Ratio 1:2:4. (S.I. No. 5(f), P. No.16)

Qty: 18 cft					@RS: 14429.25	% Cft		RS: 2597/-

9/-	Applying Floating coat of cement  1/32” thick.

Qty: 812 Sft					@RS: 660/-		% Sft		RS: 5359/-

10/-	Cement Plaster 1:6 up to 20’-0’ height ½” thick (S.I No:    P.NO:      ).

Qty: 812 Sft					@RS: 2206.60		% Sft		RS: 17918/-

11/-	Cement Plaster 1:4 up to 20’-0’ height 3/8” thick (S.I No:    P.NO:      ).

Qty: 541 Sft					@RS: 2197.52		% Sft		RS: 11889/-

Page no:2.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

12/-	Providing and fixing G.I frames/chowkhats of size 7”x2” or 4½”x3” for door using 20 gauge G.I sheet i/c welded hinges and fixing at site with necessary hold fasts, filling with cement sand slurry of ratio 1:6 and repairing the jambs. The cost also i/c all carriage, tools and plants used in making and fixing.   (S.I.No. 29 Page No. 98

Qty: 50 Rft					@RS: 228.90		P.Sft		RS: 11445/-

13/-	Providing and laying tile glazed (6”x 6”1/4”) on floor of wall facing in required colour and pattern of STILE specification jointed in white cement and pigment over a base of 1:2 gray cement mortar ¾” thick i/c washing and filling joints with slurry of white cement and pigment in desired shape with finishing, clearing and cost of was polish etc. complete i/c cutting tiles to proper profile. Amended  (S.I. No. 60, P. No.47)

Qty: 522 Sft					@RS: 30509.77	% SFt		RS: 159261/-

14/-	First class deodar wood  wrought joinery  in doors & windows  etc fixed in position

including  hold fasts hinges  iron  tower bolts chock, cleats  handles  and  cords  with

hooks etc complete.	(S.I No: 54 P.NO:  65).

Qty: 56 Sft					@RS: 902.93		% Sft		RS:  50564/-

15/-	Distempering 02 coats

Qty: 105 Sft					@RS: 1043.90		% Sft		RS: 1096/-

16/-		Preparing the surface and painting with mat finish paint of approved make to old matt

finish 	surface paris and then painting with weather coat of approved make. (03 Coats)

(old surface) (S.I. No. 37  (a+b)  P. No.61)

Qty: 485 Sft					@RS: 2499.76		% sft		RS: 12124/-

17/-	Painting New Surface Painting Doors & Windows any type 03 coats.

Qty: 112 Sft					@RS: 2116.41		% Sft		RS: 2370/-

============================

PART “A” BUILDINGS SCHEDULE ITEMS TOTAL 	RS: 285808/-

| PART "A) |  | ________________%Above/Below Buildings Schedule Items |  |  | Total Rs:- |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | G. Total Rs:- |  |  |  |
|  |  |  |  | EXECUTIVE ENGINEER |  |  |
| CONTRACTOR |  |  |  | PROVINCIAL BUILDINGS DIVISION |  |  |
|  |  |  |  | SUKKUR |  |  |

BILL OF QUANTITIES

Discription & Rate of items based on Schedule/ Market (offered Rates).

NAME OF WORK: 	Estimate for M/R to  GOVERNMENT STAFF QUARTER COLONY @ KHAIRPUR BLOCK NO:2 QTR NO: 1.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

1/-	Dismantling brick work in cement or lime mortar etc .(S.I No:     P.NO:    )

Qty: 08 cft					@RS: 1285.63		% Cft		RS: 103/-

2/-	Dismantling glazed or encaustic tiles etc (S.I No:55 P.NO:13).

Qty: 344 Sft					@RS: 786.50		% Sft		RS; 2706/-

3/-	Removing door with Chowakats (S.I No: 33 (a)P.NO:120.

Qty: 812 Sft					@RS: 121/-		% Sft		RS: 983/-

4/	Fabrication of mild steel reinforcement for cement concrete i/c cutting bending laying

in position making joints and fastenings i/c cost of binding wire (also includes removal

of rust from bars) (a) Using tor bars. (S.I.No.7 (a) P.No.20)

Qty: 0.370 cwt					@RS: 5001.70		P.cwt		RS: 1851/-

5/-	R.C.C. work in roof slab beams columns rafts lintels and other structural members laid

in situ or precast laid in position complete in all respects: (i) Ratio (1:2:4) 90 lbs

cement 2 cft sand 4 cft shingle 1/8” to 1/4” gauge. (S.I.No.6 (a-i) P.No. 19)

lintel

Qty: 05 cft					@RS: 337/-		P.Cft		RS: 1685/-

6/-	Pacca Brick work in ground floor in cement sand mortar Ratio (1:60 (S.I No:   P.    ).

Qty: 08 cft					@RS: 12674.36	% Cft		RS: 1014/-

7/-	Cement Concrete brick or Stone Ballast 1” to 2”-0 gauge Ratio (1:6:12) (S.I No: P.NO:    )

Qty: 35 cft					@RS: 8122.95		% Cft		RS: 2843/-

8/-	Cement Concrete plain including placing compacting finishing & curing, complete (including screening and washing of stone aggregate without shuttering. Ratio 1:2:4. (S.I. No. 5(f), P. No.16)

Qty: 18 cft					@RS: 14429.25	% Cft		RS: 2597/-

9/-	Applying Floating coat of cement  1/32” thick.

Qty: 812 Sft					@RS: 660/-		% Sft		RS: 5359/-

10/-	Cement Plaster 1:6 up to 20’-0’ height ½” thick (S.I No:    P.NO:      ).

Qty: 812 Sft					@RS: 2206.60		% Sft		RS: 17918/-

11/-	Cement Plaster 1:4 up to 20’-0’ height 3/8” thick (S.I No:    P.NO:      ).

Qty: 541 Sft					@RS: 2197.52		% Sft		RS: 11889/-

Page no:2.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

12/-	Providing and fixing G.I frames/chowkhats of size 7”x2” or 4½”x3” for door using 20 gauge G.I sheet i/c welded hinges and fixing at site with necessary hold fasts, filling with cement sand slurry of ratio 1:6 and repairing the jambs. The cost also i/c all carriage, tools and plants used in making and fixing.   (S.I.No. 29 Page No. 98

Qty: 50 Rft					@RS: 228.90		P.Sft		RS: 11445/-

13/-	Providing and laying tile glazed (6”x 6”1/4”) on floor of wall facing in required colour and pattern of STILE specification jointed in white cement and pigment over a base of 1:2 gray cement mortar ¾” thick i/c washing and filling joints with slurry of white cement and pigment in desired shape with finishing, clearing and cost of was polish etc. complete i/c cutting tiles to proper profile. Amended  (S.I. No. 60, P. No.47)

Qty: 522 Sft					@RS: 30509.77	% SFt		RS: 159261/-

14/-	First class deodar wood  wrought joinery  in doors & windows  etc fixed in position

including  hold fasts hinges  iron  tower bolts chock, cleats  handles  and  cords  with

hooks etc complete.	(S.I No: 54 P.NO:  65).

Qty: 60 Sft					@RS: 902.93		% Sft		RS:  54176/-

15/-	Distempering 02 coats

Qty: 105 Sft					@RS: 1043.90		% Sft		RS: 1096/-

16/-		Preparing the surface and painting with mat finish paint of approved make to old matt

finish 	surface paris and then painting with weather coat of approved make. (03 Coats)

(old surface) (S.I. No. 37  (a+b)  P. No.61)

Qty: 485 Sft					@RS: 2499.76		% sft		RS: 12124/-

17/-	Painting New Surface Painting Doors & Windows any type 03 coats.

Qty: 196 Sft					@RS: 2116.41		% Sft		RS: 2370/-

============================

PART “A” BUILDINGS SCHEDULE ITEMS TOTAL 	RS: 291198/-

| PART "A) |  | ________________%Above/Below Buildings Schedule Items |  |  | Total Rs:- |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | G. Total Rs:- |  |  |  |
|  |  |  |  | EXECUTIVE ENGINEER |  |  |
| CONTRACTOR |  |  |  | PROVINCIAL BUILDINGS DIVISION |  |  |
|  |  |  |  | SUKKUR |  |  |

BILL OF QUANTITIES

Discription & Rate of items based on Schedule/ Market (offered Rates).

NAME OF WORK:-	ESTIMATIMATE FOR M/R TO GOVERNMENT STAFF QUARTER COLONY @ KHAIRPUR  BLOCK NO: 4 QTR NO: 1,2.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

1/-	Dismantling cement concrete plain (1:2:4) etc complete (S.I no:19(c)P.NO:10).

Qty: 107  cft					@RS: 3327.50		% Cft		RS: 3560/-

2/-	Removing  cement or lime plaster (S.I No:53 P.NO:14).

Qty: 852 Sft					@RS: 121/-		% Sft		RS: 1241/-

3/-	Applying Floating Coat of cement 1/32” thick (S.I No: 14 P.NO:59).

Qty: 1286 Sft					@RS: 660/-		% Sft		RS: 8488/-

4/-	Cement Plaster 1:6 up to 20’-0’ height ½” thick. (S.I No:     P.NO:       ).

Qty: 1286 Sft					@RS: 2206.60  	% Sft		RS: 28377/-

5/-	Cement Plaster 1:4 up to 20’-0’ height 3/8” thick. (S.I No:     P.NO:       ).

Qty: 1286 Sft					@RS: 2197.52		% Sft		RS: 28260/-

6/-	Providing & Laying 1” thick Topping Cement Concrete (1:2:4) including surface finishing and dividing into panel 2” thick (S,I No:16P.No:42).

Qty: 626 Sft					@RS: 3275.50		% Sft		RS: 20505/-

7/-	First class deodar wood  wrought joinery  in doors & windows  etc fixed in position  including  hold fasts hinges  iron  tower bolts chock, cleats  handles  and  cords  with hooks etc complete.	(S.I No: 54 P.NO:  65).

Qty: 45 Sft					@RS: 902.93		P.Sft		RS: 55079/-

8/-	Distempering 02 coats.

Qty: 968 Sft					@RS: 1043.90		% Sft		RS: 10105/-

9/-	Painting Old Surface Panting doors & window  any type 02 coats.

Qty: 192 Sft					@RS: 1160.06		% Sft		RS: 2227/-

10/-	Single Layer of tiles 9”x4 ½”x2” laid over 4” earth 1” mud plaster w/o bhoosa grouted with cement sand (1:3) on top of R.C.C slab etc complete.

Qty: 2100 Sft					@RS: 5310.35		% Sft		RS: 111517/-

11/-	Providing and laying tile glazed (6”x 6”1/4”) on floor of wall facing in required colour and pattern of STILE specification jointed in white cement and pigment over a base of 1:2 gray cement mortar ¾” thick i/c washing and filling joints with slurry of white cement and pigment in desired shape with finishing, clearing and cost of was polish etc. complete i/c cutting tiles to proper profile. Amended  (S.I. No. 60, P. No.47)

Qty: 422 Sft					@RS: 30509.77	% Sft		RS: 128751/-

Page no:2.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

12/-	Galvanized wire gauze fixed to choakats with ¾” strips (S.I no: 14 P.NO:60).

Qty: 120 Sft					@RS: 190.72		P.Sft		RS: 22886/-

13/-	Colour washing 02 coats etc.

Qty: 1430 Sft					@RS: 859.90		% Sft		RS: 12297/-

====================================

PART”A” BUILDINGS SCHEDULE ITEMS TOTAL	RS: 433293/-

| PART "A) |  | ________________%Above/Below Buildings Schedule Items |  |  | Total Rs:- |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | G. Total Rs:- |  |  |  |
|  |  |  |  | EXECUTIVE ENGINEER |  |  |
| CONTRACTOR |  |  |  | PROVINCIAL BUILDINGS DIVISION |  |  |
|  |  |  |  | SUKKUR |  |  |

BILL OF QUANTITIES

Discription & Rate of items based on Schedule/ Market (offered Rates).

NAME OF WORK:-	ESTIMATIMATE FOR M/R TO GOVERNMENT STAFF QUARTER COLONY @ KHAIRPUR  BLOCK NO: A     QTR NO: 15.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

1/-	Dismantling cement concrete plain (1:2:4) etc complete (S.I no:19(c)P.NO:10).

Qty: 107  cft					@RS: 3327.50		% Cft		RS: 3560/-

2/-	Removing  cement or lime plaster (S.I No:53 P.NO:14).

Qty: 852 Sft					@RS: 121/-		% Sft		RS: 1241/-

3/-	Applying Floating Coat of cement 1/32” thick (S.I No: 14 P.NO:59).

Qty: 1286 Sft					@RS: 660/-		% Sft		RS: 8488/-

4/-	Cement Plaster 1:6 up to 20’-0’ height ½” thick. (S.I No:     P.NO:       ).

Qty: 1286 Sft					@RS: 2206.60  	% Sft		RS: 28377/-

5/-	Cement Plaster 1:4 up to 20’-0’ height 3/8” thick. (S.I No:     P.NO:       ).

Qty: 1286 Sft					@RS: 2197.52		% Sft		RS: 28260/-

6/-	Providing & Laying 1” thick Topping Cement Concrete (1:2:4) including surface finishing and dividing into panel 2” thick (S,I No:16P.No:42).

Qty: 626 Sft					@RS: 3275.50		% Sft		RS: 20505/-

7/-	First class deodar wood  wrought joinery  in doors & windows  etc fixed in position  including  hold fasts hinges  iron  tower bolts chock, cleats  handles  and  cords  with hooks etc complete.	(S.I No: 54 P.NO:  65).

Qty: 24 Sft					@RS: 902.93		P.Sft		RS: 21670/-

8/-	Distempering 02 coats.

Qty: 968 Sft					@RS: 1043.90		% Sft		RS: 10105/-

9/-	Painting Old Surface Panting doors & window  any type 02 coats.

Qty: 192 Sft					@RS: 1160.06		% Sft		RS:2227/-

10/-	Single Layer of tiles 9”x4 ½”x2” laid over 4” earth 1” mud plaster w/o bhoosa grouted with cement sand (1:3) on top of R.C.C slab etc complete.

Qty: 2100 Sft					@RS: 5310.35		% Sft		RS: 111517/-

11/-	Providing and laying tile glazed (6”x 6”1/4”) on floor of wall facing in required colour and pattern of STILE specification jointed in white cement and pigment over a base of 1:2 gray cement mortar ¾” thick i/c washing and filling joints with slurry of white cement and pigment in desired shape with finishing, clearing and cost of was polish etc. complete i/c cutting tiles to proper profile. Amended  (S.I. No. 60, P. No.47)

Qty: 422 Sft					@RS: 30509.77	% Sft		RS: 128751/-

Page no:2.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

12/-	Galvanized wire gauze fixed to choakats with ¾” strips (S.I no: 14 P.NO:60).

Qty: 48 Sft					@RS: 190.72		P.Sft		RS: 9155/-

13/-	Colour washing 02 coats etc.

Qty: 1296Sft					@RS: 859.90		% Sft		RS: 10756/-

====================================

PART “A” BUILDINGS SCHEDULE ITEMS TOTAL 	RS: 383665/-

| PART "A) |  | ________________%Above/Below Buildings Schedule Items |  |  | Total Rs:- |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | G. Total Rs:- |  |  |  |
|  |  |  |  | EXECUTIVE ENGINEER |  |  |
| CONTRACTOR |  |  |  | PROVINCIAL BUILDINGS DIVISION |  |  |
|  |  |  |  | SUKKUR |  |  |

BILL OF QUANTITIES

Discription & Rate of items based on Schedule/ Market (offered Rates).

NAME  OF  WORK:-	ESTIMATE FOR M/R STAFF QUARTER COLONY @

KHAIRPUR BLOCK NO:     QTR NO: 1, 4,12.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

1/- 	Dismantling Cement Concrete reinforcement Separating reinforcement from concrete cleaning and straightening the same (S.I No:20 P.NO:10).

Qty:410.0 cft					@Rs:5445/-		% Cft		RS:22325/-

2/-	Removing  cement or lime plaster (S.I No:53 P.NO:14).

Qty:3300 sft					@Rs:121/-		% Sft		RS:3993/-

3/-	Removing door with Chowakast (S.I No:33 (a)P.NO:13).

Qty: 04 Nos					@Rs:142.18		each		RS:569/-

4/-	Removing windows  and sky light with Chowakats 	(S.I No:33 (b) P.NO:13).

Qty: 04 Nos					@Rs:102.85		each		RS:411/-

5/-	Pacca Brick work in ground floor in (e) cement sand mortar ratio 1:6 (S.I No:5 (e) P.NO:25).

Qty:464 cft					@Rs:12674.36		% cft		RS:58809

6/-	Supplying & Fixing R.S Girder (4”x8”) (Material Schedule.

Qty:16.412 cwt					@Rs:3850/-		P.cwt		RS:63186/-

7/-	Supplying & Fixings R.S T-Iron (2”x2”x1/2”) Material Schedule.

Qty:10.385 cwt				@Rs:3575/-		P.cwt		RS:37126/-

8/-	Second class tile roofing consisting of 4” earth and 1” mud plaster with gobri leaping over ½” thick cement plaster 1:6 with 34 Lbs of hot bitumen coating sand blinded provided over one layer of 12”x6”x2” tiles laid in 1:6 cement mortar including 1:2 cement pointing under neath of tiles complete including curing etc.S.I.NO. 2 Page No. 33).

Qty:780 sft					@Rs:7607.25		% sft		RS:59337/-

9/-	Supplying & Filling sand under floor plugging into wall.

Qty:932 cft					@Rs:1141.25		% cft		RS:10636/-

10/-	Cement Concrete  Brick or Stone Ballast 1 ½” to 2” Gauge Ratio (1:6:120 (S.I No:      P.NO:       ).

Qty:154.0 cft					@Rs:8122.95		% Cft		RS:12509/-

11/-	Applying Floating Coat of cement 1/32” thick (S.I No: 14 P.NO:59).

Qty:330.0 sft					@Rs:660/-		% Sft		RS:21780/-

12/-	Cement Plaster 1:6 up to 20’-0’ height ½” thick. (S.I No:     P.NO:       ).

Qty:330.0 sft					@Rs:2206.60		% Sft		RS:72818/-

Page no:2.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

13/-	Cement Plaster 1:4 up to 20’-0’ height 3/8” thick. (S.I No:     P.NO:       ).

Qty:330.0 sft					@Rs:2197.52		% Sft		RS:72518/-

14/-	Making & Fixing Steel grated door  and window etc complete (S.I No:25 P.NO:61).

Qty:258 sft					@Rs:370.83		P.Sft		RS:95674/-

15/-	Providing & Laying 1” thick Topping Cement Concrete (1:2:4) including surface finishing and dividing into panel 2” thick (S,I No:16P.No:42).

Qty:736 sft					@Rs:3275.50		% Sft		RS:24108/-

16/-	First class deodar wood  wrought joinery  in doors & windows  etc fixed in position  including  hold fasts hinges  iron  tower bolts chock, cleats  handles  and  cords  with hooks etc complete.	(S.I No: 54 P.NO:  65).

Qty:163 sft					@Rs:902.93		P.Sft		RS: 147178/-

17/-	White washing 03 coats (S.I No:26(b)P.NO:54).

Qty:661 sft					@Rs:829.95		% Sft		RS:5486/-

18/-	Distempering 03 coats (S.I No:     P.NO:       ).

Qty:3718 sft					@Rs:1043.90		% Sft		RS:38812/-

19/-	Painting  OlD Surface (c) Painting door & window any type 02 coats.

Qty:516.0 sft					@Rs:1160.06		% Sft		RS:5986/-

20/-	R.C.C Spout  panel. fixing in position etc complete.

Qty:05 No’s					@Rs:524.98		each		RS:2625/-

21/-	Painting New Surface Painting  Guard Bars grating  iron standard braces etc  & similar open work 03 coats.

Qty:961 sft					@Rs:1270.83		% sft		RS:12213/-

===============================

PART “A” BUILDINGS SCHEDULE ITEMS TOTAL	RS:767688/-

| PART "A) |  | ________________%Above/Below Buildings Schedule Items |  |  | Total Rs:- |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | G. Total Rs:- |  |  |  |
|  |  |  |  | EXECUTIVE ENGINEER |  |  |
| CONTRACTOR |  |  |  | PROVINCIAL BUILDINGS DIVISION |  |  |
|  |  |  |  | SUKKUR |  |  |

BILL OF QUANTITIES

Discription & Rate of items based on Schedule/ Market (offered Rates).

NAME  OF  WORK:-	M/R TO OMBUDSMAN OFFICE @ KHAIRPUR.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

1/- 	Dismantling Cement Concrete reinforcement Separating reinforcement from concrete cleaning and straightening the same (S.I No:20 P.NO:10).

Qty:410.0 cft					@Rs:5445/-		% Cft		RS:22325/-

2/-	Removing  cement or lime plaster (S.I No:53 P.NO:14).

Qty:3300 sft					@Rs:121/-		% Sft		RS:3993/-

3/-	Removing door with Chowakast (S.I No:33 (a)P.NO:13).

Qty: 04 Nos					@Rs:142.18		each		RS:569/-

4/-	Removing windows  and sky light with Chowakats 	(S.I No:33 (b) P.NO:13).

Qty: 04 Nos					@Rs:102.85		each		RS:411/-

5/-	Pacca Brick work in ground floor in (e) cement sand mortar ratio 1:6 (S.I No:5 (e) P.NO:25).

Qty:464 cft					@Rs:12674.36		% cft		RS:58809/-

6/-	Supplying & Fixing R.S Girder (4”x8”) (Material Schedule.

Qty:16.412 cwt					@Rs:3850/-		P.cwt		RS:63186/-

7/-	Supplying & Fixings R.S T-Iron (2”x2”x1/2”) Material Schedule.

Qty:10.385 cwt				@Rs:3575/-		P.cwt		RS:37126/-

8/-	Second class tile roofing consisting of 4” earth and 1” mud plaster with gobri leaping over ½” thick cement plaster 1:6 with 34 Lbs of hot bitumen coating sand blinded provided over one layer of 12”x6”x2” tiles laid in 1:6 cement mortar including 1:2 cement pointing under neath of tiles complete including curing etc.S.I.NO. 2 Page No. 33).

Qty:780 sft					@Rs:7607.25		% sft		RS:59337/-

9/-	Supplying & Filling sand under floor plugging into wall.

Qty:932 cft					@Rs:1141.25		% cft		RS:10636/-

10/-	Cement Concrete  Brick or Stone Ballast 1 ½” to 2” Gauge Ratio (1:6:120 (S.I No:      P.NO:       ).

Qty:154.0 cft					@Rs:8122.95		% Cft		RS:12509/-

11/-	Applying Floating Coat of cement 1/32” thick (S.I No: 14 P.NO:59).

Qty:330.0 sft					@Rs:660/-		% Sft		RS:21780/-

12/-	Cement Plaster 1:6 up to 20’-0’ height ½” thick. (S.I No:     P.NO:       ).

Qty:330.0 sft					@Rs:2206.60		% Sft		RS:72818/-

13/-	Cement Plaster 1:4 up to 20’-0’ height 3/8” thick. (S.I No:     P.NO:       ).

Qty:330.0 sft			@Rs:2197.52			% Sft		RS:72518/-

Page no:2.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

14/-	Making & Fixing Steel grated door  and window etc complete (S.I No:25 P.NO:61).

Qty:258 sft					@Rs:370.83		P.Sft		RS:95674/-

15/-	Providing & Laying 1” thick Topping Cement Concrete (1:2:4) including surface finishing and dividing into panel 2” thick (S,I No:16P.No:42).

Qty:736 sft					@Rs:3275.50		% Sft		RS:24108/-

16/-	First class deodar wood  wrought joinery  in doors & windows  etc fixed in position  including  hold fasts hinges  iron  tower bolts chock, cleats  handles  and  cords  with hooks etc complete.	(S.I No: 54 P.NO:  65).

Qty:163 sft					@Rs:902.93		P.Sft		RS: 147178/-

17/-	White washing 03 coats (S.I No:26(b)P.NO:54).

Qty:661 sft					@Rs:829.95		% Sft		RS:5486/-

18/-	Distempering 03 coats (S.I No:     P.NO:       ).

Qty:3718 sft					@Rs:1043.90		% Sft		RS:38812/-

19/-	Painting  OlD Surface (c) Painting door & window any type 02 coats.

Qty:516.0 sft					@Rs:1160.06		% Sft		RS:5986/-

20/-	R.C.C Spout  panel. fixing in position etc complete.

Qty:05 No’s					@Rs:524.98		each		RS:2625/-

21/-	Painting New Surface Painting  Guard Bars grating  iron standard braces etc  & similar open work 03 coats.

Qty:961 sft					@Rs:1270.83		% sft		RS:12213/-

===============================

PART “A” BUILDINGS SCHEDULE ITEMS TOTAL 	RS:767688/-

| PART "A) |  | ________________%Above/Below Buildings Schedule Items |  |  | Total Rs:- |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | G. Total Rs:- |  |  |  |
|  |  |  |  | EXECUTIVE ENGINEER |  |  |
| CONTRACTOR |  |  |  | PROVINCIAL BUILDINGS DIVISION |  |  |
|  |  |  |  | SUKKUR |  |  |

BILL OF QUANTITIES

Discription & Rate of items based on Schedule/ Market (offered Rates).

NAME  OF  WORK:-	M/R to GOVERNMENT STAFF QUARTER COLONY @ KHAIRPUR BLOCK NO:  1  QTR NO: 1

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

1/-	Dismantling cement concrete plain (1:2:4) etc complete (S.I no:19(c)P.NO:10)/.

Qty:161 Cft					@Rs:3327.50		% Cft		RS:5357/-

2/-	Removing  cement or lime plaster (S.I No:53 P.NO:14).

Qty:386 Sft					@Rs:121/-		% Sft		RS:1120/-

3/-	Dismantling Glazed tile or encaustic tiles etc complete .

Qty:188 Sft					@Rs:786.50		% Sft		RS:1140/-

4/-	Rehandling  glazed or encaustic tiles  etc .

Qty:1050 Cft					@Rs:1085.75		% Cft		RS:1140/-

5/-	Applying Floating Coat of cement 1/32” thick (S.I No: 14 P.NO:59).

Qty:926 sft					@Rs:660/-		% Sft		RS:6112/-

6/-	Cement Plaster 1:6 up to 20’-0’ height ½” thick. (S.I No:     P.NO:       ).

Qty:926 sft					@Rs:2206.60		% Sft		RS:20433/-

7/-	Cement Plaster 1:4 up to 20’-0’ height 3/8” thick. (S.I No:     P.NO:       ).

Qty:926 sft					@Rs:2197.52		% Sft		RS:20349/-

8/-	Providing & Laying 1” thick Topping Cement Concrete (1:2:4) including surface finishing and dividing into panel 2” thick (S,I No:16P.No:42).

Qty:582 sft					@RS:3275.50		% sft		RS:19063/-

9/-	First class deodar wood  wrought joinery  in doors & windows  etc fixed in position  including  hold fasts hinges  iron  tower bolts chock, cleats  handles  and  cords  with hooks etc complete.

(S.I No: 54 P.NO:  65).

Qty:20 sft					@RS:1273.76		P.Sft		RS:25475/-

10/-	Providing and laying tile glazed (6”x 6”1/4”) on floor of wall facing in required colour and pattern of STILE specification jointed in white cement and pigment over a base of 1:2 gray cement mortar ¾” thick i/c washing and filling joints with slurry of white cement and pigment in desired shape with finishing, clearing and cost of was polish etc. complete i/c cutting tiles to proper profile. Amended  (S.I. No. 60, P. No.47)

Qty:132 sft					@RS:30509.77		% Sft		RS:40273/-

11/-	Single Layer of tiles 9”x4 ½”x2” laid over 4” earth 1” mud plaster w/o bhoosa grouted with cement sand (1:3) on top of R.C.C slab etc complete.

Qty:500 sft					@RS:5310.35		% Sft		RS:26552/-

Page no:2.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

12/-	Distempering 02 coats .

Qty:1444 sft					@RS:1043.90		% Sft		RS:10574/-

13/-	Painting Old Surface Painting door & windows any type .

Qty:236 sft					@RS:1160.06		% sft		RS:2738/-

====================================

PART “A” BUILDINGS SCHEDULE ITEMS TOTAL	RS:180326/-

| PART "A) |  | ________________%Above/Below Buildings Schedule Items |  |  | Total Rs:- |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | G. Total Rs:- |  |  |  |
|  |  |  |  | EXECUTIVE ENGINEER |  |  |
| CONTRACTOR |  |  |  | PROVINCIAL BUILDINGS DIVISION |  |  |
|  |  |  |  | SUKKUR |  |  |

BILL OF QUANTITIES

Discription & Rate of items based on Schedule/ Market (offered Rates).

NAME OF WORK: 	M/R TO ATTA MUHAMMAD HAMI LIBRARY  AT DISTRICT KHAIRPUR

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

PART “A(I)” BUILDINGS SCHEDULE ITEMS.

1/-	Dismantling Cement Concrete plain (1:2:4) (S.I No: 19 (c)P.NO:10).

qty:75 cft					@Rs:3327.50		% cft		RS:2496/-

2/-	Dismnatling Glazed or Encaustic tile  etc complete (S.I No: 55 P.NO:13)

Qty:30 sft					@Rs:786.50		% sft		RS:236/-

3/-	Removing door with Chowakats (S.I No: 33 (a)P.NO:1)

Qty:838 sft					@Rs:121/-		% Sft		RS:1014/-

4/-	Pacca Brick work in Ground Floor in cement sand mortar Ratio 1:6 etc complete.

Qty:220 cft					@Rs:12674.36		% Cft		RS:27884/-

5/-    	Fabrication of mild steel reinforcement for cement concrete i/c cutting bending laying in position making joints and fastenings i/c cost of binding wire (also includes removal of rust from bars) (a) Using tor bars. (S.I.No.7 (a) P.No.20)

Qty:3.375 cwt					@Rs:5001.70		P.cwt		RS;16881/-

6/-	R.C.C. work in roof slab beams columns rafts lintels and other structural members laid in situ or precast laid in position complete in all respects: (i) Ratio (1:2:4) 90 lbs cement 2 cft sand 4 cft shingle 1/8” to 1/4” gauge. (S.I.No.6 (a-i) P.No. 19)

Qty:76 cft					@Rs:337/-		P.Cft		RS:25612/-

7/-	Cement Plaster 1:6 up to 20” ht: ½” thick. (S.I NO:      P.NO:    )

Qty:2452 sft					@Rs:2206.60		% Sft		RS:54106/-

8/-	Cement Plaster 1:4up to 20” ht: 3/8hick. (S.I NO:      P.NO:    )

Qty:2452 sft					@RS.2197.52		%sft		RS.53883/-

9/-	Providing & Fixing Colour Create  to wall surface to provide etc complete

Qty:4804 sft					@RS.6319.50		%sft		RS.303589/-

10/-	Pointing flush on stone work Ratio 1:2 (S.I No:    P.NO:     ).

Qty:2450 sft					@Rs:1726.72		% sft		RS:42305/-

11/-	Scraping Ordinary oil Bond Distemper or paint wall (S.I no: 54 (b) P.NO:13).

Qty:4111 sft					@Rs:226.68		% sft		RS:9319/-

12/-	Galvanized wire gauze fixed  to chowakats with ¾” s  deodar strps.

Qty:255 sft					@Rs:190.72		P.Sft		RS:48634/-

13/-	Supplying & Fixing false ceiling  of plaster , in panels including making frame , work of deodar wood including painting with soligia paint.(S.NO:52 P.NO:64).

Qty:228 sft					@Rs:25293.42		% sft		RS:57669/-

14/-	Providing & Fixing plaster of pairs ceiling border of 8” -10” with of specified design & thickness i/c fixing besides ceiling with nails/screws with jetties.(S.I No:44 P.NO:39)

Qty:61 Rft					@Rs:104.22		P.Rft		RS:6357/-

Page no:2.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

15/-	Distempering 02 coats.

Qty:6714 sft					@Rs:1043.90		% sft		RS:70087/-

16/-	Colour washing 02 coats.

qty:4200 sft					@Rs:859.90		% sft		RS:36116/-

17/-	Preparing the surface and painting with weather coat of approved make to old

weather coat surface. (b) 2nd & subsequent coat. (S.I. No. 39  (b)  P. No.56)

qty:1499 sft					@Rs:1498.48		% sft		RS:22462/-

18/-	Providing and laying tile glazed (6”x 6”1/4”) on floor of wall facing in required colour and pattern of STILE specification jointed in white cement and pigment over a base of 1:2 gray cement mortar ¾” thick i/c washing and filling joints with slurry of white cement and pigment in desired shape with finishing, clearing and cost of was polish etc. complete i/c cutting tiles to proper profile. Amended  (S.I. No. 60, P. No.47)

Qty:102 sft					@Rs:30509.77		% Sft		RS:31120/-

19/-	R.C.C Spout  including  fixing in position.

Qty:06 No’s					@Rs:524.98		each		RS:3150/-

20/-	P/L HALA pattern tiles glazed 8”x8”x¼” on floor of wall facing in required pattern of STILE specification jointed in white cement and pigment over a base of 1:2 grey cement mortar ¾” thick i/c washing and filling joints with slurry of white cement and pigment in desired shape with finishing cleaning and cost of wax polish etc. complete i/c cutting

Qty:134 sft					@Rs:34520.31		% Sft		RS:46257/-

21/-	Painting Old Surface Painting Door & Windows any type 02 coats.

Qty:961 sft					@Rs:1160.06		% sft		RS:11148/-

22/-	P/F cement paving blocks flooring having size of 197x97x80(mm)of cuty/ quddra / cobble shape with natural colours, having strength b/w 5000 psi to 8500 psi i/c filling the joints with hill sand and laying in specified manner / pattern and design etc. complete. (S.I. No.74 P. No.50).

Qty: 4192 Sft					@RS:223.97		P.Sft		RS: 938882/-

==================================

PART “A(I)” BUILDINGS SCHEDULE ITEMS TOTAL	RS:1966871/-

PART-A(II)  Plinth Protection BUILDINGS SCHEDULE ITEMS.

1/- 	Excavation in foundation of building bridges and other structures i/c dag balling dressing refilling around structure with excavated earth watering and ramming lead up-to 5ft. (b) in ordinary soil. (S.I.No.18 (b) P.No.4)

Qty:164 cft					@Rs:3176.25		% Cft		RS:521/-

02/-	Cement concrete stone of brick ballast 1½” to 2” gauge. Ratio (1:5:10). (S.I.No.4 (c) P.No.15)

Qty:55 cft					@RS:8694.95		% cft		RS:4782/-

03/-	Pacca Brick work in foundation & Plinth in cement sand mortar 1:6 etc complete

Qty:232 cft					@Rs:11948.36		% cft		RS:27720/-

Page no:3.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

04/-	Supplying & Fixing Sand under floor plugging into wall.

Qty:639 cft					@Rs:1141.25		% cft		RS:7293/-

05/-	Cement concrete stone of brick ballast 1½” to 2” gauge. Ratio (1:6:12). (S.I.No.4 (c) P.No.15).

Qty:141 cft					@Rs:8122.95		% cft		RS;11453/-

06/-	Cement Plaster 1:6 up to 20’-0’ height ½” thick (S.I No:    P.NO:      ).

Qty:512 sft					@Rs:2206.60		% sft		RS:11298/-

07/-	Cement Plaster 1:4 up to 20’-0’ height 3/8” thick (S.I No:    P.NO:      ).

Qty:512 sft					@Rs:2197/52		% sft		RS:11251/-

08/-		Providing and lying 1” thick C/C topping cement concrete (1:2:4) i/c surface finishing

and dividing into panels.  (S.I.No.16 (d) P.No.42)2” thick

Qty:831 sft					@Rs:3275.50		% sft		RS:27219/-

====================

PART “A(II)” BUILDINGS SCHEDULE ITEMS TOTAL	RS:101537/-

| PART "A(I+II)” |  | ________________%Above/Below Buildings Schedule Items |  |  | Total Rs:- |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | G. Total Rs:- |  |  |  |
|  |  |  |  | EXECUTIVE ENGINEER |  |  |
| CONTRACTOR |  |  |  | PROVINCIAL BUILDINGS DIVISION |  |  |
|  |  |  |  | SUKKUR |  |  |

BILL OF QUANTITIES

Discription & Rate of items based on Schedule/ Market (offered Rates).

NAME  OF  WORK:-	M/R   TO  STAFF QUARTER @ CENTERAL JAIL KHAIRPUR .

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

1/-	Removing  cement or lime plaster (S.I No:53 P.NO:14).

Qty:1395 Sft					@Rs:121/-		% Sft		RS:1688/-

2/-	Dismantling cement concrete plain (1:2:4) etc complete (S.I no:19(c)P.NO:10)/.

Qty:29 Cft					@RS:3327.50		% Cft		RS:965/-

3/-	First class deodar wood  wrought joinery  in doors & windows  etc fixed in position  including  hold fasts hinges  iron  tower bolts chock, cleats  handles  and  cords  with hooks etc complete.(S.I No: 54 P.NO:  65).

Qty: 69 SFt					@Rs:1273.76		P.Sft		RS:87889/-

4/-	Cement Concrete plain including placing compacting finishing & curing, complete (including screening and washing of stone aggregate without shuttering. Ratio 1:2:4. (S.I. No. 5(f), P. No.16)

Qty:66 Cft					@Rs:14429.25		% Cft		RS:9523/-

5/-	Applying Floating Coat of cement 1/32” thick (S.I No: 14 P.NO:59).

Qty:1395 sft					@Rs:660/-		% Sft		RS:9207/-

6/-	Cement Plaster 1:6 up to 20’-0’ height ½” thick. (S.I No:     P.NO:       ).

Qty:1395 sft					@Rs:2206.60		% Sft		RS:30782/-

7/-	Cement Plaster 1:4 up to 20’-0’ height 3/8” thick. (S.I No:     P.NO:       ).

Qty:1395 sft					@Rs:2197.52		% Sft		RS:30655/-

8/-	Providing and laying tile glazed (6”x 6”1/4”) on floor of wall facing in required colour and pattern of STILE specification jointed in white cement and pigment over a base of 1:2 gray cement mortar ¾” thick i/c washing and filling joints with slurry of white cement and pigment in desired shape with finishing, clearing and cost of was polish etc. complete i/c cutting tiles to proper profile. Amended  (S.I. No. 60, P. No.47)

Qty:464 Sft					@Rs:30509.77		% Sft		RS:141565/-

9/-	Pacca Brick work  in Ground Floor in cement sand mortar Ratio (1:60 (S.I No    P.NO:    ).

Qty:57 Cft					@RS:12674.36		% Cft		RS:7224/-

10/-	Single Layer of tiles 9”x4 ½”x2” laid over 4” earth 1” mud plaster w/o bhoosa grouted with cement sand (1:3) on top of R.C.C slab etc complete.

Qty:354 Sft					@Rs:7607.25		% Sft		RS:15523/-

11/-	Providing and lying 1” thick C/C topping cement concrete (1:2:4) i/c surface finishing

and dividing into panels.  (S.I.No.16 (d) P.No.42) 3” thick.

Qty: 200 Sft					@RS: 4411.92		% Sft		RS: 8824/-

Page no:2.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

12/-	Galvanized wire gauze fixed to chowakats with ¾’  deodar strip  and screw. (S.I No: 4(c) P:58 ).

Qty: 36 Sft					@RS: 190.72		P.Sft		RS: 6866/-

13/-	Distempering 02 coats.

Qty:990 Sft					@Rs:1043.90		% Sft		RS:10335/-

14/-	Painting Old Surface Painting doors & windows any type 02 coats.

Qty:199 Sft					@Rs:1160.06		% Sft		RS:2309/-

15/-	Supplying & Fixing false ceiling  of plaster , in panels including making frame , work of deodar wood including painting with soligia paint.(S.NO:52 P.NO:64).

Qty: 482 Sft					@RS: 25293.42	% Sft		RS: 121914/-

16/-	Providing & Fixing plaster of pairs ceiling border of 8” -10” with of specified design & thickness i/c fixing besides ceiling with nails/screws with jetties.(S.I No:44 P.NO:39)

Qty: 147 rft					@RS: 104.22		P.Rft		RS: 15320/-

============================

PART “A” BUILDINGS SCHEDULE ITEMS TOTAL		RS: 500589/-

| PART "A” |  | ________________%Above/Below Buildings Schedule Items |  |  | Total Rs:- |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | G. Total Rs:- |  |  |  |
|  |  |  |  | EXECUTIVE ENGINEER |  |  |
| CONTRACTOR |  |  |  | PROVINCIAL BUILDINGS DIVISION |  |  |
|  |  |  |  | SUKKUR |  |  |

BILL OF QUANTITIES

Discription & Rate of items based on Schedule/ Market (offered Rates).

NAME OF WORK: 	M/R TO  SACHAL 	library  AT DISTRICT KHAIRPUR(Compound wall & Internal Path)

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

PART “A(I) BUILDINGS SCHEDULE ITEMS.

1/-	Excavation in foundation of building bridges and other structures i/c dag balling dressing refilling around structure with excavated earth watering and ramming lead up-to 5ft. (b) in ordinary soil. (S.I.No.18 (b) P.No.4)

Qty: 2875 cft					@RS: 3175.25		% Cft		RS: 9132/-

2/-	Cement concrete stone of brick ballast 1½” to 2” gauge. Ratio (1:5:10). (S.I.No.4 (c) P.No.15).

Qty: 575 Cft					@RS: 8694.95		% Cft		RS: 49996/-

3/-	Random rubble masonry (un-coursed) (d) in cement sand mortar (iv) Ratio (S.I.No.1 (d)(iv) P.No.32).

Qty: 897 cft					@RS: 17723/-		% Cft		RS: 158975/-

4/-	Coursed rubble masonry including hammer dressing (d) in cement sand mortar (iv) Ratio 1:6.(S.I.No.2 (d)(iv) P.No.32)

Qty: 1509 Cft					@RS: 25321/-		% Cft		RS: 382094/-

5/-	Fabrication of mild steel reinforcement for cement concrete i/c cutting bending laying in position making joints and fastenings i/c cost of binding wire (also includes removal of rust from bars) (a) Using tor bars. (S.I.No.7 (a) P.No.20)

Qty: 36.249 cwt				@RS: 5001.70		P.cwt		RS: 181307/-

6/-	R.C.C. work in roof slab beams columns rafts lintels and other structural members laid in situ or precast laid in position complete in all respects: (i) Ratio (1:2:4) 90 lbs cement 2 cft sand 4 cft shingle 1/8” to 1/4” gauge. (S.I.No.6 (a-i) P.No. 17)

Qty: 932 Cft					@RS: 337/-		P.Cft		RS; 314084/-

7/-	Cement Plaster 1:6 up to 20’-0 height ½” thick. ( S.I NO: 13(b)    P:NO:52   )

Qty: 6440 Sft					@RS: 2206.60		% Sft		RS: 142105/-

8/-	Cement Plaster 1:4 up to 20’-0’ height 3/8” thick. ( S.I NO: 13(b)    P:NO:52   )

Qty: 2590 Sft					@RS: 2197.52		% Sft		RS: 56916/-

9/-	Providing & Fixing Colour Create  to wall surface to provide etc complete

Qty: 3850 Sft					@RS: 6390.50		% Sft		RS: 246034/-

10/-	Supplying and fixing in position iron/steel grill of ¾” x ¼” size flat iron of approved design including painting 3 coats etc complete (Weight or not less than 3.7 Lbs/sq feet of finished grill)(S.I.NO.27 Page No.97)

Qty: 1380 Sft					@RS: 180.50		P.Sft		RS: 249090/-

11/-	Colour washing 03 coats etc

Qty: 2590 Sft					@RS: 1303.17		% Sft		RS: 33752/-

12/-	Pacca Brick work in Other than building Ratio 1:6  in cement sand mortar 1:6 etc

Qty: 2234 Sft					@RS: 12346.65	% Cft		RS: 275824/-

===================================

PART “A(I)” BUILDINGS SCHEDULE ITEMS TOTAL		RS: 2099309/-

Page no:2.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

PART “A(II) PATH BUILDINGS SCHEDULE ITEMS.

1/-	Cement concrete stone of brick ballast 1½” to 2” gauge. Ratio (1:6:12). (S.I.No.4 (c) P.No.15)

Qty: 2688 Cft					@RS: 8122.95		% Cft		RS: 218345/-

2/-	Cement Concrete plain including placing compacting finishing & curing, complete (including screening and washing of stone aggregate without shuttering. Ratio 1:2:4. (S.I. No. 5(f), P. No.16)

Qty: 914 Cft					@RS: 14429.25	% Cft		RS: 130055/-

3/-	Supplying & Fixing Sand Under floor pluging into wall.

Qty: 5375 Cft					@RS: 1913.21		% Cft		RS: 102835/-

4/-	P/F cement paving blocks flooring having size of 197x97x80(mm)of cuty/ quddra / cobble shape with natural colours, having strength b/w 5000 psi to 8500 psi i/c filling the joints with hill sand and laying in specified manner / pattern and design etc. complete. (S.I. No.74 P. No.50). 	Path

Qty: 5275 sft					@RS: 223.97		P.Sft	            RS: 1181442/-

================

PART “A(II)” BUILDINGS SCHEDULE ITEMS TOTAL		           RS: 1632677/-

| PART "A(I+II)” |  | ________________%Above/Below Buildings Schedule Items |  |  | Total Rs:- |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | G. Total Rs:- |  |  |  |
|  |  |  |  | EXECUTIVE ENGINEER |  |  |
| CONTRACTOR |  |  |  | PROVINCIAL BUILDINGS DIVISION |  |  |
|  |  |  |  | SUKKUR |  |  |

BILL OF QUANTITIES

Discription & Rate of items based on Schedule/ Market (offered Rates).

NAME OF WORK: 	M/R TO  ATTA MUHAMMAD HAMI LIBRARY @ KHAIRPUR (internal w/s & s/f & drainage)

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

PART “A” W/S & S/F SCHEDULE ITEMS.

1-		P/F squatting type white glazed of flushing cistern with internal fitting and flush pipe with band and making requisite Nos. of holes in wall plinth & floor for pipe connections and making good in cement concrete 1:2:4. (A) W.C pan of not less than 23” clear opening between flashing rims and 3 gallons flushing tank. With 4” dia C.I. tape. (S.I.NO.1(A) P.No.1)

Qty:06 Nos					@Rs:4802/60		each		RS:28816/-

2/-		P/F European type white glazed earthenware wash down w.c pan complete with and i/c the cost of white/black plastic seat (Best Quality) and lid with c.p brass hinges and buffers. 3 gallons white glazed earthen low level flushing cistern with siphon fitting 1-1/2” dia white porcelain enameled flush bend ¾” dia & cutting & making requisite number of holes in walls plinth and floor for pipe connection and making good in cement concrete 1:2:4. (Foreign Quality).

(S.I.NO.4 P. No2)

Qty:02 no					@Rs:11477/40	each		RS:22955/-

3/-	P/F 22”x16” lavatory basin in white glazed earthen ware complete with & i/c the cost of W.I or C.I cantilever brackets 6” built into wall painted white in two coats after a primary coat of red lead paint a pair of ½” dia chrome plate pillar traps 1-1/2” dia rubber plug & chrome plated brass chain 1-1/4” dia malleable iron or c.p brass traps malleable iron or brass union & making requisite number of holes in walls plinth & floor for pipe connection and making good in cement concrete 1:2:4 (standard pattern). Add extra for labour for providing & fixing of earthenware pedestal white or colored glazed (Standard pattern) .  (S.I.NO. 8 P. No.3).

Qty:02 No					@Rs:4694/80		each		RS:9390/-

4/-	Add extra for labour for providing & fixing of earthenware pedestal white or colored glazed (Standard pattern) .  (S.I.NO. 9 P. No.3)

Qty:02 nos					@Rs:938/47		each		RS:1877/-

5-	P/F 6”x2” or 6”x3” C.I floor trap of the approved self cleaning design with a C.I screwed down gritting with or without a vent arm complete with and i/c making requisite number or holes in walls plinth and floor for pipe connections & making good in CC 1:2:4. (S.I. No. 20 P. No.5)

Qty:10 Nos					@Rs:2042/43		each		RS:20424/-

6-	Providing & Fixing Nylon connection  complete with  ½” dia Brass stop cock  with pair  of brass  nuts  and lining  joints  to nylon  connection (S.I No:23 P-6).

Qty:10 Nos					@Rs:447/15		each		RS:4472/-

7-		Supplying and fixing long bib-cock of superior quality with C.P. head ½” dia.

(S.I.NO.15(a) P. No.15)

Qty:08 Nos					@Rs:1109/46		each		RS:8876/-

Page no:2.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

8-		Supplying and fixing concealed stopcock of superior quality with c.p head ½” dia.

(S.I.NO.13 (a) P. No.15)

Qty:10 Nos					@Rs:843/92		each		RS:8439/-

9/-		Supplying and fixing C.P Muslim Shower with double bib cock & ring pipe etc complete.(S.I.NO.19(a) P. No.19)

Qty:02 No					@Rs:3432/-		each		RS:6864/-

10/-		Supplying and fixing wash-basin mixture of superior quality with c.p head ½” dia.

(S.I.NO.14(a) P. No.19)

Qty:02 No					@Rs:2882/-		each		RS:5764/-

11/-	P/F Asbestoses   pipe with collar dadex  or equivalent  i/c digging the finches required depth  & fixing  & jointing  with rubber ring  i/c  testing  to water pressure  Head of 200 ft:

4” dia

Qty:150 rft					@Rs:188/44		P,.rft		RS:28266/-

12/-	S/F in position c.p Tee cock ½” dia c.p bib cock standard pattern. (S.I. No. 3 (i-b) P. No.18)

Qty:08 Nos					@Rs:843/92		each		RS:6751/-

13/-		Providing & Fixing handles valves (china) etc complete

¾” dia	 04 nos					@Rs|:271/92		each		RS:1088/-

1” dia 02 nos					@Rs:365/42		each		RS:731/-

14/-	P/F UPVC pipe ¾” dia PAK-ARAB) SCH-40 surface by using clip /saddler/socket/reducer/bush/tee / elbow etc .or recessed in masonry C.C or R.C.C upto 20’ ft. height and making good wih C.c i/c curing rinishing etc complete as per instruction of the Engineer In charge ,.Specification of the material should meet the requirement of class 12454-Bin accordance with the ASTM-D-1784 type 1 Grade 1. rate include all costs of labor ,material, cartage scaffolding ladders .etc complete (R.A Sanction).

Qty:100 rft					@Rs:83/-		P.rft		RS:8300/-

15/-	P/F UPVC UNION  ¾” dia PAK-ARAB) SCH-40  make approved quality and Design of various sizes Fixed to UPVC Pipe / Fitting paid Separately  upto 20’ ft. height as per instruction Sof the Engineer In charge Specification of the material should meet the requirement of class 12454-Bin accordance with the ASTM-D-1784 type 1 Grade 1. rate include all costs of labor ,material, cartage scaffolding ladders .etc complete (R.A Sanction).

Qty:10 Nos					@Rs:241/-		each		RS:2410/-

16/-	P/F UPVC pipe 1” dia PAK-ARAB) SCH-40 surface by using clip /saddler/socket/reducer/bush/tee / elbow etc .or recessed in masonry C.C or R.C.C upto 20’ ft. height and making good wih C.c i/c curing finishing etc complete as per instruction of the Engineer In charge ,.Specification of the material should meet the requirement of class 12454-Bin accordance with the ASTM-D-1784 type 1 Grade 1. rate include all costs of labor ,material, cartage scaffolding ladders .etc complete (R.A Sanction).

Qty:100 rft					@Rs:91/-		P.rft		RS:9100/-

17/-	P/F UPVC pipe 4” dia PAK-ARAB) SCH-40 surface by using clip /saddler/socket/reducer/bush/tee /  etc . or recessed in masonry C.C or R.C.C upto 20’ ft. height and making good wih C.C i/c curing finishing etc complete as per instruction of the Engineer In charge ,.Specification of the material should meet the requirement of class 12454-Bin accordance with the ASTM-D-1784 type 1 Grade 1. rate include all costs of labor ,material, cartage scaffolding ladders .etc complete (R.A Sanction).

Qty:100 rft		@Rs:290/-			P.rft			RS: 29000/-

==========================================

PART “A” W/S & S/F SCHEDULE ITEMS TOTAL 	RS: 202792/-

Page no:3.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

PART “B” P.H. ENGR: SCHEDULE ITEMS.

1/-	Providing UPVC pressure  pipes of class “C” equivalent make fixing in trench i/c cutting, fitting and jointing with “Z” joint with one rubber ring i/c testing with water to head of 61 meter or 200ft. 4” dia. (P.H.E)  (S.I.No. 01 (b) P.No.22)

6” dia 	100 rft					@Rs:373/-		P.Rft		RS: 37300/-

2/-	Boring for tube well in all water bearing soils from ground level up-to 100 ft or 30.5 meter depth i/c sinking and with drawing of casing pipe. (100 mm 4”) (S.I.No. 01 (b) Page No.36).

4” Dia

Qty:50 rft					@Rs:242/-		P,rft		RS:12100/-

3/-	Construction of manhole  or inspection chamber  for the required dia  of circular sewer   and 3’-6’ (1067 mm ) depth with walls  of B.B  in cement mortar  1:3 cement plaster  1:3 ½”  thick in side 1”  (25 mm ) thick over  beching  and channel i/c  fixing C.I main hole cover with frame of clear opening  1 ½”x 1 ½”” 457x457 mm) of 1.75 cwt . (88.9 kg) embedded in plain c.c 1:2:4) & fixing (25 mm) dia M.S steps 6” 150 mm)   white projecting 4” (102 mm) from the face of walls at 12” (305 mm) c/c duly painted etc complete as per standard specification & drawing.(S.I No- P-46).

Qty:06 No					@Rs:14748/-		P.rft		RS:88488/-

4/-	Construction of  wheel valve  chamber (S.I No:9 P-53)P.H.E).

Qty:02 Nos					@Rs:4103/-		each		RS:8206/-

_________________

PART “B” P.H.ENGR: SCHEDULE ITEMS TOTAL		RS:146094/-

| PART "A” |  | ________________%Above/Below W/S & S/F Schedule Items |  |  | Total Rs:- |  |
| --- | --- | --- | --- | --- | --- | --- |
| PART "B" |  | ________________%Above/Below P.H.Engr: Schedule Items |  |  | Total Rs:- |  |
|  |  |  | G. Total Rs:- |  |  |  |
|  |  |  |  | EXECUTIVE ENGINEER |  |  |
| CONTRACTOR |  |  |  | PROVINCIAL BUILDINGS DIVISION |  |  |
|  |  |  |  | SUKKUR |  |  |

BILL OF QUANTITIES

Discription & Rate of items based on Schedule/ Market (offered Rates).

NAME OF WORK: 	M/R TO  SACHAL ACADEMY AT DISTRICT KHAIRPUR

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

PART “A(I)” BUILDINGS SCHEDULE ITEMS.

1/-	Removing  cement or lime plaster etc complete

Qty: 4948 Sft					@RS: 121/-		% Sft		RS: 5987/-

2/-	Dismantling Pacca Brick work in lime or cement mortar .(S.I No:      P.NO:     ).

Qty: 366 cft					@RS: 1285.63		% Cft		RS: 4705/-

3/-	Dismantling Glazed tile or encaustic tile etc complete.

Qty: 681 sft					@RS: 786.50		% Sft		RS: 5356/-

4/-	Applying Floating coat  of cement etc complete

Qty: 4948 Sft					@RS: 660/-		% Sft		RS: 32657/-

5/-	Cement Plaster 1:6 up to 20’-0 height ½” thick. ( S.I NO: 13(b)    P:NO:52   )

Qty: 6138 Sft					@RS: 2206.60		% Sft		RS: 35441/-

6/-	Cement Plaster 1:4 up to 20’-0’ height 3/8” thick.

Qty: 1860 Sft					@RS: 2197.52		% Sft		RS: 40874/-

7/-	Providing & Fixing Colour Create  to wall surface to provide etc complete

Qty: 4948 Sft					@RS: 6319.50		% Sft		RS: 312689/-

8/-	Cement Concrete brick or stone ballast 1 ½” to 2” gauge Ratio (1:6:12) (S.I No; 4(d)P.14).

Qty: 3049 Cft					@RS: 8122.95		% Cft		RS: 247669/-

9/-	Cement Concrete plain including placing compacting finishing & curing, complete (including screening and washing of stone aggregate without shuttering. Ratio 1:2:4. (S.I. No. 5(f), P. No.16)

Qty: 3142 Cft					@RS: 14429.25	% Cft		RS:453367/-

10/-	Preparing  the surface   painting  with matt finish  paint of approved  make  to old  matt 	finish  surface  02 coat

Qty: 2077 Sft					@RS: 3444.38		% Sft		RS: 71540/-

11/-	Supply & Fixing  wall panel including supply and making deodar wooden second frame  work etc complete.

Qty: 1271 Sft					@RS: 31082.30	% Sft		RS: 395056/-

12/-	Painting Old Surface Painting Doors & Windows any type 02 coats.

Qty: 156 Sft					@RS: 1160.06		% Sft		RS: 1810/-

13/-	Galvanized wire gauze 144 mesh etc complete fixing in position.

Qty: 240 Sft					@RS: 190.72		P.Sft		RS: 45773/-

Page no:2.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

14/-	Providing & Fixing R.C.C Spout including in position etc complete

Qty: 12 No’s					@RS: 524.98		each		RS: 6300/-

15/-	French Polish complete (b) on Old work etc complete

Qty: 363 Sft					@RS: 1952.50		% Sft		RS: 7088/-

16/-	Supplying and fixing in position iron/steel grill of ¾” x ¼” size flat iron of approved design including painting 3 coats etc complete (Weight or not less than 3.7 Lbs/sq feet of finished grill)(S.I.NO.27 Page No.97)

Qty: 900 Sft					@RS: 180.50		P.Sft		RS: 81255/-

17/-	Distempering 02 coats etc.

Qty: 6257 Sft					@RS: 1043.90		% Sft		RS: 65317/-

18/-	Providing & Fixing approved quality  Mortic Lock etc complete.

Qty: 10 No’s					@RS: 1786.13		each		RS: 17861/-

19/-	Providing and laying tile glazed (6”x 6”1/4”) on floor of wall facing in required colour and pattern of STILE specification jointed in white cement and pigment over a base of 1:2 gray cement mortar ¾” thick i/c washing and filling joints with slurry of white cement and pigment in desired shape with finishing, clearing and cost of was polish etc. complete i/c cutting tiles to proper profile. Amended  (S.I. No. 60, P. No.47)

Qty: 1100 Sft					@RS: 30509.77	% Sft		RS: 335607/-

20/-	Supplying & Fixing false ceiling  of plaster , in panels including making frame , work of deodar wood including painting with soligia paint.(S.NO:52 P.NO:64).

Qty: 994 Sft					@RS: 25293.42	% Sft		RS: 251417/-

21/-	Providing & Fixing plaster of pairs ceiling border of 8” -10” with of specified design & thickness i/c fixing besides ceiling with nails/screws with jetties.(S.I No:44 P.NO:39)

Qty: 212 Rft					@RS: 104.22		P.Rft		RS: 22095/-

22/-	First class deodar wood  wrought joinery  in doors & windows  etc fixed in position  including  hold fasts hinges  iron  tower bolts chock, cleats  handles  and  cords  with hooks etc complete.(S.I No: 54 P.NO:  65).

Qty: 123 Sft					@RS: 902.93		P.Sft		RS: 111060/-

23/-	Two Coat of Bitumen Laid hot using 34 Lbs for % Sft Over roof and blinded with sand etc complete

Qty: 6257 Sft					@RS: 1887.40		% Sft		RS: 118095/-

24/-	P/F cement paving blocks flooring having size of 197x97x80(mm)of cuty/ quddra / cobble shape with natural colours, having strength b/w 5000 psi to 8500 psi i/c filling the joints with hill sand and laying in specified manner / pattern and design etc. complete. (S.I. No.74 P. No.50).

Qty: 11700 Sft					@RS: 223.97		P.Sft	            RS: 2620449/-

25/-	P/F Stair case iterrace  railing with ½” dia steel pipe 2’-0 c.c vertically Top railing with 4 No’s 5/8” dia pipe  horizontally between main pipe posts of 2 ½” dia as per required Design embedded properly in R.C.C work by means of iron  luges welded with M.S rods complete as per drawing.

Qty: 132 Sft					@RS: 1323.28		P.Sft		RS: 174673/-

Page no:3.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

26/-	P/L HALA pattern tiles glazed 8”x8”x¼” on floor of wall facing in required pattern of STILE specification jointed in white cement and pigment over a base of 1:2 grey cement mortar ¾” thick i/c washing and filling joints with slurry of white cement and pigment in desired shape with finishing cleaning and cost of wax polish etc. complete i/c cutting

Qty: 1155 Sft					@RS: 47651.56	% Sft		RS: 550376/-

27/-	Laying white marble fine dressed in the surface without winding set in lime mortar 1:2 i/c rubbing and polishing pf the joints (a) ¾” thick flooring. (S.I.NO.28 (a) Page No. 43)

Qty 1100 sft					@Rs:567.38		P.Sft		RS: 624118/-

========================================

PART “A(I)” BUILDINGS SCHEDULE ITEMS TOTAL    RS: 6638635/-

PART “A(II)   OVER HEAD TANK BUILDINGS SCHEDULE ITEMS.

1/-	Pacca Brick work in ground floor  in cement sand mortar  Ratio (1:6).(S.I No:    P.NO:    )

Qty: 103 Cft					@RS: 12674.36	% Cft		RS: 13055/-

2/-	Fabrication of mild steel reinforcement for cement concrete i/c cutting bending laying

in position making joints and fastenings i/c cost of binding wire (also includes removal

of rust from bars) (a) 	Using tor bars. (S.I.No.7 (a) P.No.20).

Qty: 3.962 cwt					@RS: 5001.70		P.cwt		RS: 19817/-

3/-	R.C.C. work in roof slab beams columns rafts lintels and other structural members laid in situ or precast laid in position complete in all respects: (i) Ratio (1:2:4) .

Qty: 87 Sft					@RS: 337/-		% Sft		RS: 29319/-

4/-	Cement Plaster 1:6 up to 20’-0’ height ½” thick,

Qty: 532 Sft					@RS: 2206.60		% Sft		RS: 11739/-

5/-	Cement Plaster 1:4 up to 20’-0’ height 3/8” thick,

Qty: 532 Sft					@RS: 2197.52		% Sft		RS: 11691/-

======================================

PART “A(II)” OVER HEAD TANK BUILDINGS SCHEDULE ITEMS TOTAL    	RS: 85621/-

| PART "A(I+II)” |  | ________________%Above/Below Buildings Schedule Items |  |  | Total Rs:- |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | G. Total Rs:- |  |  |  |
|  |  |  |  | EXECUTIVE ENGINEER |  |  |
| CONTRACTOR |  |  |  | PROVINCIAL BUILDINGS DIVISION |  |  |
|  |  |  |  | SUKKUR |  |  |

BILL OF QUANTITIES

Discription & Rate of items based on Schedule/ Market (offered Rates).

NAME  OF  WORK:-	M/R  AGRICULTURE WORK SHOP @ KHAIRPUR.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

PART “A” BUILDINGS SCHEDULE ITEMS.

1/- 	Dismantling Cement Concrete reinforcement Separating reinforcement from concrete cleaning and straightening the same (S.I No:20 P.NO:10).

Qty:410.0 cft					@Rs:5445/-		% Cft		RS:22325/-

2/-	Removing  cement or lime plaster (S.I No:53 P.NO:14).

Qty:3300 sft					@Rs:121/-		% Sft		RS:3993/-

3/-	Removing door with Chowakast (S.I No:33 (a)P.NO:13).

Qty: 04 Nos					@Rs:142.18		each		RS:569/-

4/-	Removing windows  and sky light with Chowakats 	(S.I No:33 (b) P.NO:13).

Qty: 04 Nos					@Rs:102.85		each		RS:411/-

5/-	Pacca Brick work in ground floor in (e) cement sand mortar ratio 1:6 (S.I No:5 (e) P.NO:25).

Qty:464 cft					@Rs:12674.36		% cft		RS:58809/-

6/-	Supplying & Fixing R.S Girder (4”x8”) (Material Schedule.

Qty:16.412 cwt					@Rs:3850/-		P.cwt		RS:63186/-

7/-	Supplying & Fixings R.S T-Iron (2”x2”x1/2”) Material Schedule.

Qty:10.385 cwt				@Rs:3575/-		P.cwt		RS:37126/-

8/-	Second class tile roofing consisting of 4” earth and 1” mud plaster with gobri leaping over ½” thick cement plaster 1:6 with 34 Lbs of hot bitumen coating sand blinded provided over one layer of 12”x6”x2” tiles laid in 1:6 cement mortar including 1:2 cement pointing under neath of tiles complete including curing etc.S.I.NO. 2 Page No. 33).

Qty:780 sft					@Rs:7607.25		% sft		RS:59337/-

9/-	Supplying & Filling sand under floor plugging into wall.

Qty:932 cft					@Rs:1141.25		% cft		RS:10636/-

10/-	Cement Concrete  Brick or Stone Ballast 1 ½” to 2” Gauge Ratio (1:6:120 (S.I No:      P.NO:       ).

Qty:154.0 cft					@Rs:8122.95		% Cft		RS:12509/-

11/-	Applying Floating Coat of cement 1/32” thick (S.I No: 14 P.NO:59).

Qty:330.0 sft					@Rs:660/-		% Sft		RS:21780/-

12/-	Cement Plaster 1:6 up to 20’-0’ height ½” thick. (S.I No:     P.NO:       ).

Qty:330.0 sft					@Rs:2206.60		% Sft		RS:72818/-

Page no:2.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

13/-	Cement Plaster 1:4 up to 20’-0’ height 3/8” thick. (S.I No:     P.NO:       ).

Qty:330.0 sft					@Rs:2197.52		% Sft		RS:72518/-

14/-	Making & Fixing Steel grated door  and window etc complete (S.I No:25 P.NO:61).

Qty:258 sft					@Rs:370.83		P.Sft		RS:95674/-

15/-	Providing & Laying 1” thick Topping Cement Concrete (1:2:4) including surface finishing and dividing into panel 2” thick (S,I No:16P.No:42).

Qty:736 sft					@Rs:3275.50		% Sft		RS:24108/-

16/-	First class deodar wood  wrought joinery  in doors & windows  etc fixed in position  including  hold fasts hinges  iron  tower bolts chock, cleats  handles  and  cords  with hooks etc complete.	(S.I No: 54 P.NO:  65).

Qty:163 sft					@Rs:902.93		P.Sft		RS: 147178/-

17/-	White washing 03 coats (S.I No:26(b)P.NO:54).

Qty:661 sft					@Rs:829.95		% Sft		RS:5486/-

18/-	Distempering 03 coats (S.I No:     P.NO:       ).

Qty:3718 sft					@Rs:1043.90		% Sft		RS:38812/-

19/-	Painting  OlD Surface (c) Painting door & window any type 02 coats.

Qty:516.0 sft					@Rs:1160.06		% Sft		RS:5986/-

20/-	R.C.C Spout  panel. fixing in position etc complete.

Qty:05 No’s					@Rs:524.98		each		RS:2625/-

21/-	Painting New Surface Painting  Guard Bars grating  iron standard braces etc  & similar open work 03 coats.

Qty:961 sft					@Rs:1270.83		% sft		RS:12213/-

22/-	Providing and laying tile glazed (6”x 6”1/4”) on floor of wall facing in required colour and pattern of STILE specification jointed in white cement and pigment over a base of 1:2 gray cement mortar ¾” thick i/c washing and filling joints with slurry of white cement and pigment in desired shape with finishing, clearing and cost of was polish etc. complete i/c cutting tiles to proper profile. Amended  (S.I. No. 60, P. No.47)

Qty: 804 sft				@Rs:30509.77			% Sft		RS: 245299/-

23/-	Supplying & Fixing false ceiling  of plaster , in panels including making frame , work of deodar wood including painting with soligia paint.(S.NO:52 P.NO:64).

Qty: 514 sft				@Rs:25293.42			% Sft		RS: 130008/-

Page no:3.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

24/-	Providing & Fixing plaster of pairs ceiling border of 8” -10” with of specified design & thickness i/c fixing besides ceiling with nails/screws with jetties.(S.I No:44 P.NO:39)

Qty: 178 sft				@Rs:104.22			P.Rft		RS: 18551/-

===============================

PART “A” BUILDINGS SCHEDULE ITEMS TOTAL    	RS:1161546/-

| PART "A” |  | ________________%Above/Below Buildings Schedule Items |  |  | Total Rs:- |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | G. Total Rs:- |  |  |  |
|  |  |  |  | EXECUTIVE ENGINEER |  |  |
| CONTRACTOR |  |  |  | PROVINCIAL BUILDINGS DIVISION |  |  |
|  |  |  |  | SUKKUR |  |  |

BILL OF QUANTITIES

Discription & Rate of items based on Schedule/ Market (offered Rates).

NAME  OF  WORK:-	M/R to  STAFF Qtr Agriculture @ Khairpur.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

PART “A” BUILDINGS SCHEDULE ITEMS.

1/-	Dismantling cement concrete plain (1:2:4) etc complete (S.I no:19(c)P.NO:10)/.

Qty:99 Cft					@Rs:3327.50		% Cft		RS:3294/-

2/-	Removing  cement or lime plaster (S.I No:53 P.NO:14).

Qty:386 Sft					@Rs:121/-		% Sft		RS:1120/-

3/-	Dismantling Glazed tile or encaustic tiles etc complete .

Qty:188 Sft					@Rs:786.50		% Sft		RS:1140/-

4/-	Rehandling  glazed or encaustic tiles  etc .

Qty:1050 Cft					@Rs:1085.75		% Cft		RS:1140/-

5/-	Applying Floating Coat of cement 1/32” thick (S.I No: 14 P.NO:59).

Qty:926 sft					@Rs:660/-		% Sft		RS:6112/-

6/-	Cement Plaster 1:6 up to 20’-0’ height ½” thick. (S.I No:     P.NO:       ).

Qty:926 sft					@Rs:2206.60		% Sft		RS:20433/-

7/-	Cement Plaster 1:4 up to 20’-0’ height 3/8” thick. (S.I No:     P.NO:       ).

Qty:926 sft					@Rs:2197.52		% Sft		RS:20349/-

8/-	Providing & Laying 1” thick Topping Cement Concrete (1:2:4) including surface finishing and dividing into panel 2” thick (S,I No:16P.No:42).

Qty:582 sft					@RS:3275.50		% sft		RS:19063/-

9/-	First class deodar wood  wrought joinery  in doors & windows  etc fixed in position  including  hold fasts hinges  iron  tower bolts chock, cleats  handles  and  cords  with hooks etc complete.

(S.I No: 54 P.NO:  65).

Qty: 109 sft					@RS:1273.76		P.Sft		RS: 138840/-

10/-	Providing and laying tile glazed (6”x 6”1/4”) on floor of wall facing in required colour and pattern of STILE specification jointed in white cement and pigment over a base of 1:2 gray cement mortar ¾” thick i/c washing and filling joints with slurry of white cement and pigment in desired shape with finishing, clearing and cost of was polish etc. complete i/c cutting tiles to proper profile. Amended  (S.I. No. 60, P. No.47)

Qty: 660 sft					@RS:30509.77		% Sft		RS: 201364/-

11/-	Single Layer of tiles 9”x4 ½”x2” laid over 4” earth 1” mud plaster w/o bhoosa grouted with cement sand (1:3) on top of R.C.C slab etc complete.

Qty:1000 sft					@RS:5310.35		% Sft		RS:53104/-

12/-	Distempering 02 coats .

Qty:1444 sft					@RS:1043.90		% Sft		RS:10574/-

13/-	Painting Old Surface Painting door & windows any type .

Qty:236 sft					@RS:1160.06		% sft		RS:2738/-

Page no:2.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

14/-	Supplying & Fixing false ceiling  of plaster , in panels including making frame , work of deodar wood including painting with soligia paint.(S.NO:52 P.NO:64).

Qty: 564 Sft					@Rs:25293.42		% Sft		RS: 142655/-

15/-	Providing & Fixing plaster of pairs ceiling border of 8” -10” with of specified design & thickness i/c fixing besides ceiling with nails/screws with jetties.(S.I No:44 P.NO:39)

Qty:108 Rft					@Rs:104.22		P.Rft		RS: 11256/-

16/-	Glavanized wire gauze fixed to chowakats including fixing in position.

Qty: 16 Sft					@RS: 190.72		P.Sft		RS: 3052/-

======================

PART “A” BUILDINGS SCHEDULE ITEMS TOTAL    	RS: 635534/-

| PART "A” |  | ________________%Above/Below Buildings Schedule Items |  |  | Total Rs:- |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | G. Total Rs:- |  |  |  |
|  |  |  |  | EXECUTIVE ENGINEER |  |  |
| CONTRACTOR |  |  |  | PROVINCIAL BUILDINGS DIVISION |  |  |
|  |  |  |  | SUKKUR |  |  |

BILL OF QUANTITIES

Discription & Rate of items based on Schedule/ Market (offered Rates).

NAME OF WORK: 	M/R to  T.H.Q Hospital Gambat District Khairpur. (Repair of M.S B’glow.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

PART “A” BUILDINGS SCHEDULE ITEMS.

1/-	Dismantling brick work in cement or lime mortar etc .(S.I No:     P.NO:    )

Qty: 08 cft					@RS: 1285.63		% Cft		RS: 103/-

2/-	Dismantling glazed or encaustic tiles etc (S.I No:55 P.NO:13).

Qty: 344 Sft					@RS: 786.50		% Sft		RS; 2706/-

3/-	Removing door with Chowakats (S.I No: 33 (a)P.NO:120.

Qty: 812 Sft					@RS: 121/-		% Sft		RS: 983/-

4/	Fabrication of mild steel reinforcement for cement concrete i/c cutting bending laying

in position making joints and fastenings i/c cost of binding wire (also includes removal

of rust from bars) (a) Using tor bars. (S.I.No.7 (a) P.No.20)

lintel

Qty: 0.370 cwt					@RS: 5001.70		P.cwt		RS: 1851/-

5/-	R.C.C. work in roof slab beams columns rafts lintels and other structural members laid

in situ or precast laid in position complete in all respects: (i) Ratio (1:2:4) 90 lbs

cement 2 cft sand 4 cft shingle 1/8” to 1/4” gauge. (S.I.No.6 (a-i) P.No. 19)

lintel

Qty: 05 cft					@RS: 337/-		P.Cft		RS: 1685/-

6/-	Pacca Brick work in ground floor in cement sand mortar Ratio (1:60 (S.I No:   P.    ).

Qty: 08 cft					@RS: 12674.36	% Cft		RS: 1014/-

7/-	Cement Concrete brick or Stone Ballast 1” to 2”-0 gauge Ratio (1:6:12) (S.I No: P.NO:    )

Qty: 35 cft					@RS: 8122.95		% Cft		RS: 2843/-

8/-	Cement Concrete plain including placing compacting finishing & curing, complete (including screening and washing of stone aggregate without shuttering. Ratio 1:2:4. (S.I. No. 5(f), P. No.16)

Qty: 18 cft					@RS: 14429.25	% Cft		RS: 2597/-

9/-	Applying Floating coat of cement  1/32” thick.

Qty: 812 Sft					@RS: 660/-		% Sft		RS: 5359/-

10/-	Cement Plaster 1:6 up to 20’-0’ height ½” thick (S.I No:    P.NO:      ).

Qty: 812 Sft					@RS: 2206.60		% Sft		RS: 17918/-

11/-	Cement Plaster 1:4 up to 20’-0’ height 3/8” thick (S.I No:    P.NO:      ).

Qty: 541 Sft					@RS: 2197.52		% Sft		RS: 11889/-

Page no:2.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

12/-	Providing and fixing G.I frames/chowkhats of size 7”x2” or 4½”x3” for door using 20 gauge G.I sheet i/c welded hinges and fixing at site with necessary hold fasts, filling with cement sand slurry of ratio 1:6 and repairing the jambs. The cost also i/c all carriage, tools and plants used in making and fixing.   (S.I.No. 29 Page No. 98

Qty: 50 Rft					@RS: 228.90		P.Sft		RS: 11445/-

13/-	Providing and laying tile glazed (6”x 6”1/4”) on floor of wall facing in required colour and pattern of STILE specification jointed in white cement and pigment over a base of 1:2 gray cement mortar ¾” thick i/c washing and filling joints with slurry of white cement and pigment in desired shape with finishing, clearing and cost of was polish etc. complete i/c cutting tiles to proper profile. Amended  (S.I. No. 60, P. No.47)

Qty: 486 Sft					@RS: 30509.77	% SFt		RS: 148277/-

14/-	First class deodar wood  wrought joinery  in doors & windows  etc fixed in position

including  hold fasts hinges  iron  tower bolts chock, cleats  handles  and  cords  with

hooks etc complete.	(S.I No: 54 P.NO:  65).

Qty: 21 Sft					@RS: 902.93		% Sft		RS:  18962/-

15/-	Distempering 02 coats

Qty: 105 Sft					@RS: 1043.90		% Sft		RS: 1096/-

16/-		Preparing the surface and painting with mat finish paint of approved make to old matt

finish 	surface paris and then painting with weather coat of approved make. (03 Coats)

(old surface) (S.I. No. 37  (a+b)  P. No.61)

Qty: 485 Sft					@RS: 2499.76		% sft		RS: 12124/-

17/-	Painting New Surface Painting Doors & Windows any type 03 coats.

Qty: 112 Sft					@RS: 2116.41		% Sft		RS: 1481/-

============================

PART “A” BUILDINGS SCHEDULE ITEMS TOTAL    RS: 242333/-

| PART "A” |  | ________________%Above/Below Buildings Schedule Items |  |  | Total Rs:- |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | G. Total Rs:- |  |  |  |
|  |  |  |  | EXECUTIVE ENGINEER |  |  |
| CONTRACTOR |  |  |  | PROVINCIAL BUILDINGS DIVISION |  |  |
|  |  |  |  | SUKKUR |  |  |

BILL OF QUANTITIES

Discription & Rate of items based on Schedule/ Market (offered Rates).

NAME OF WORK: 	M/R  T.H.Q Hospital Gambat @ District Khairpur ( Repair of Staff Qtr’s)

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

PART “A” BUILDINGS SCHEDULE ITEMS.

1/-	Dismantling brick work in cement or lime mortar etc .(S.I No:     P.NO:    )

Qty: 08 cft					@RS: 1285.63		% Cft		RS: 103/-

2/-	Dismantling glazed or encaustic tiles etc (S.I No:55 P.NO:13).

Qty: 344 Sft					@RS: 786.50		% Sft		RS; 2706/-

3/-	Removing door with Chowakats (S.I No: 33 (a)P.NO:120.

Qty: 812 Sft					@RS: 121/-		% Sft		RS: 983/-

4/	Fabrication of mild steel reinforcement for cement concrete i/c cutting bending laying

in position making joints and fastenings i/c cost of binding wire (also includes removal

of rust from bars) (a) Using tor bars. (S.I.No.7 (a) P.No.20)

lintel

Qty: 0.370 cwt					@RS: 5001.70		P.cwt		RS: 1851/-

5/-	R.C.C. work in roof slab beams columns rafts lintels and other structural members laid

in situ or precast laid in position complete in all respects: (i) Ratio (1:2:4) 90 lbs

cement 2 cft sand 4 cft shingle 1/8” to 1/4” gauge. (S.I.No.6 (a-i) P.No. 19)

lintel

Qty: 05 cft					@RS: 337/-		P.Cft		RS: 1685/-

6/-	Pacca Brick work in ground floor in cement sand mortar Ratio (1:60 (S.I No:   P.    ).

Qty: 08 cft					@RS: 12674.36	% Cft		RS: 1014/-

7/-	Cement Concrete brick or Stone Ballast 1” to 2”-0 gauge Ratio (1:6:12) (S.I No: P.NO:    )

Qty: 35 cft					@RS: 8122.95		% Cft		RS: 2843/-

8/-	Cement Concrete plain including placing compacting finishing & curing, complete (including screening and washing of stone aggregate without shuttering. Ratio 1:2:4. (S.I. No. 5(f), P. No.16)

Qty: 18 cft					@RS: 14429.25	% Cft		RS: 2597/-

9/-	Applying Floating coat of cement  1/32” thick.

Qty: 812 Sft					@RS: 660/-		% Sft		RS: 5359/-

10/-	Cement Plaster 1:6 up to 20’-0’ height ½” thick (S.I No:    P.NO:      ).

Qty: 812 Sft					@RS: 2206.60		% Sft		RS: 17918/-

11/-	Cement Plaster 1:4 up to 20’-0’ height 3/8” thick (S.I No:    P.NO:      ).

Qty: 541 Sft					@RS: 2197.52		% Sft		RS: 11889/-

12/-	Providing and fixing G.I frames/chowkhats of size 7”x2” or 4½”x3” for door using 20 gauge G.I sheet i/c welded hinges and fixing at site with necessary hold fasts, filling with cement sand slurry of ratio 1:6 and repairing the jambs. The cost also i/c all carriage, tools and plants used in making and fixing.   (S.I.No. 29 Page No. 98

Qty: 50 Rft					@RS: 228.90		P.Sft		RS: 11445/-

Page no:2.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

13/-	Providing and laying tile glazed (6”x 6”1/4”) on floor of wall facing in required colour and pattern of STILE specification jointed in white cement and pigment over a base of 1:2 gray cement mortar ¾” thick i/c washing and filling joints with slurry of white cement and pigment in desired shape with finishing, clearing and cost of was polish etc. complete i/c cutting tiles to proper profile. Amended  (S.I. No. 60, P. No.47)

Qty: 179Sft					@RS: 902.93		% Sft		RS:  161624/-

15/-	Distempering 02 coats

Qty: 105 Sft					@RS: 1043.90		% Sft		RS: 1096/-

16/-		Preparing the surface and painting with mat finish paint of approved make to old matt

finish 	surface paris and then painting with weather coat of approved make. (03 Coats)

(old surface) (S.I. No. 37  (a+b)  P. No.61)

Qty: 485 Sft					@RS: 2499.76		% sft		RS: 12124/-

17/-	Painting New Surface Painting Doors & Windows any type 03 coats.

Qty: 112 Sft					@RS: 2116.41		% Sft		RS: 2370/-

============================

PART “A” BUILDINGS SCHEDULE ITEMS TOTAL	RS: 686101/-

| PART "A” |  | ________________%Above/Below Buildings Schedule Items |  |  | Total Rs:- |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | G. Total Rs:- |  |  |  |
|  |  |  |  | EXECUTIVE ENGINEER |  |  |
| CONTRACTOR |  |  |  | PROVINCIAL BUILDINGS DIVISION |  |  |
|  |  |  |  | SUKKUR |  |  |

BILL OF QUANTITIES

Discription & Rate of items based on Schedule/ Market (offered Rates).

NAME OF WORK: 	M/R  T.H.Q Hospital Gambat @ District Khairpur ( Repair of Doctor’s B”low).

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

PART “A” BUILDINGS SCHEDULE ITEMS.

1/-	Galvanized wire gauze fixed  to chowakats with ¾” s  deodar strps.

Qty:   40sft					@Rs:190.72		P.Sft		RS: 7629/-

2/-	Laying white marble fine dressed in the surface without winding set in lime mortar 1:2 i/c rubbing and polishing pf the joints (a) ¾” thick flooring. (S.I.NO.28 (a) Page No. 43)

Qty: 240 sft					@Rs:567.38		P.Sft		RS:136195/-

3/-	Distempering 02 coats.

Qty: 576 sft		@Rs:1043.90				% Sft		RS: 6103/-

=================================

PART “A” BUILDINGS SCHEDULE ITEMS TOTAL		RS:149927/-

| PART "A” |  | ________________%Above/Below Buildings Schedule Items |  |  | Total Rs:- |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | G. Total Rs:- |  |  |  |
|  |  |  |  | EXECUTIVE ENGINEER |  |  |
| CONTRACTOR |  |  |  | PROVINCIAL BUILDINGS DIVISION |  |  |
|  |  |  |  | SUKKUR |  |  |

BILL OF QUANTITIES

Discription & Rate of items based on Schedule/ Market (offered Rates).

NAME OF WORK: 	M/R to grain godown Tando Masti Khan District Khairpur.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

01/-	Dismantling C.C Plain 1:2:4 etc (S.I NO:      P-     )

Qty: 53 cft					@RS:3327.50		% Sft		RS:799/-

02/-	Cement Concrete plain including placing compacting finishing & curing, complete (including screening and washing of stone aggregate without shuttering. Ratio 1:2:4. (S.I. No. 5(f), P. No.16)

Qty: 24 cft					@RS: 14429.25	% Cft		RS: 3463/-

03/-	Removing  Cement or lime Plaster (S.I No:  53 P.No: 13  ).

Qty: 240 Sft					@RS: 121/-		% Sft		RS: 290/-

04/-	Cement Plaster 1:6 up to 20’-0 height ½” thick. ( S.I NO: 13(b)    P:NO:52   )

Qty: 240 sft					@RS:2206.60		% Sft		RS:5296/-

05/-	Cement Plaster 1:4 up to 20”-0 height 3/8” thick. . ( S.I NO: 11(a) P:NO:52   )

Qty: 240 sft					@RS: 2197.52		% Sft		RS:5274/-

06/-	Providing and laying tile glazed (6”x 6”1/4”) on floor of wall facing in required colour and pattern of STILE specification jointed in white cement and pigment over a base of 1:2 gray cement mortar ¾” thick i/c washing and filling joints with slurry of white cement and pigment in desired shape with finishing, clearing and cost of was polish etc. complete i/c cutting tiles to proper profile. Amended  (S.I. No. 60, P. No.47)

Qty: 144 Sft					@RS: 30509.77	% Sft		RS: 43934/-

07/-	Supplying & Fixing false ceiling  of plaster , in panels including making frame , work of deodar wood including painting with soligia paint.(S.NO:52 P.NO:64).

Qty: 216 Sft					@RS: 25293.42	% Sft		RS: 54634/-

08/-	Providing & Fixing plaster of pairs ceiling border of 8” -10” with of specified design & thickness i/c fixing besides ceiling with nails/screws with jetties.(S.I No:44 P.NO:39)

Qty: 84 Sft					@RS: 104.22		P.Rft		RS: 8754/-

09/-	Distempering 03 three coat (S.I No: 24(c) P.No:54 ).

Qty: 553 Sft					@RS: 1043.90		% Sft		RS: 5773/-

10/-	Painting Old Surface Painting  doors & windows  any type  03 coats. (S.I No: 4(c)P.No:68   ).

Qty: 39 Sft					@RS: 1106.70		% Sft		RS: 432/-

11/-	First class deodar wood  wrought joinery  in doors & windows  etc fixed in position  including  hold fasts hinges  iron  tower bolts chock, cleats  handles  and  cords  with hooks etc complete.	(S.I No: 54 P.NO:  65).

Qty: 29 Sft					@RS: 1273.76		P.Sft		RS: 36939/-

Page no:2.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

12/-	Providing and lying 1” thick C/C topping cement concrete (1:2:4) i/c surface finishing

and dividing into panels.  (S.I.No.16 (d) P.No.42) 3” thick.

Qty: 368 Sft					@RS: 4411.82		% Sft		RS: 16235/-

13/-	Galvanized wire gauze fixed to chowakats with ¾’  deodar strip  and screw. (S.I No: 4(c) P:58 ).

Qty: 48 Sft					@RS: 190.72		P.Sft		RS:9155/-									==================					PART “A” BUILDINGS SCHEDULE ITEMS TOTAL	RS: 190978/-

| PART "A” |  | ________________%Above/Below Buildings Schedule Items |  |  | Total Rs:- |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | G. Total Rs:- |  |  |  |
|  |  |  |  | EXECUTIVE ENGINEER |  |  |
| CONTRACTOR |  |  |  | PROVINCIAL BUILDINGS DIVISION |  |  |
|  |  |  |  | SUKKUR |  |  |

BILL OF QUANTITIES

Discription & Rate of items based on Schedule/ Market (offered Rates).

NAME  OF  WORK:-	M/R to  STAFF Qtr  Assistant Superintdent 2  @ Centeral Jail Khairpur.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

BUILDINGS SCHEDULE ITEMS

1/-	Dismantling cement concrete plain (1:2:4) etc complete (S.I no:19(c)P.NO:10)/.

Qty:99 Cft					@Rs:3327.50		% Cft		RS:3294/-

2/-	Removing  cement or lime plaster (S.I No:53 P.NO:14).

Qty:386 Sft					@Rs:121/-		% Sft		RS:1120/-

3/-	Dismantling Glazed tile or encaustic tiles etc complete .

Qty:188 Sft					@Rs:786.50		% Sft		RS:1140/-

4/-	Rehandling  glazed or encaustic tiles  etc .

Qty:1050 Cft					@Rs:1085.75		% Cft		RS:1140/-

5/-	Applying Floating Coat of cement 1/32” thick (S.I No: 14 P.NO:59).

Qty:926 sft					@Rs:660/-		% Sft		RS:6112/-

6/-	Cement Plaster 1:6 up to 20’-0’ height ½” thick. (S.I No:     P.NO:       ).

Qty:926 sft					@Rs:2206.60		% Sft		RS:20433/-

7/-	Cement Plaster 1:4 up to 20’-0’ height 3/8” thick. (S.I No:     P.NO:       ).

Qty:926 sft					@Rs:2197.52		% Sft		RS:20349/-

8/-	Providing & Laying 1” thick Topping Cement Concrete (1:2:4) including surface finishing and dividing into panel 2” thick (S,I No:16P.No:42).

Qty:582 sft					@RS:3275.50		% sft		RS:19063/-

9/-	First class deodar wood  wrought joinery  in doors & windows  etc fixed in position  including  hold fasts hinges  iron  tower bolts chock, cleats  handles  and  cords  with hooks etc complete.

(S.I No: 54 P.NO:  65).

Qty: 109 sft					@RS:1273.76		P.Sft		RS: 138840/-

10/-	Providing and laying tile glazed (6”x 6”1/4”) on floor of wall facing in required colour and pattern of STILE specification jointed in white cement and pigment over a base of 1:2 gray cement mortar ¾” thick i/c washing and filling joints with slurry of white cement and pigment in desired shape with finishing, clearing and cost of was polish etc. complete i/c cutting tiles to proper profile. Amended  (S.I. No. 60, P. No.47)

Qty: 660 sft					@RS:30509.77		% Sft		RS: 201364/-

11/-	Single Layer of tiles 9”x4 ½”x2” laid over 4” earth 1” mud plaster w/o bhoosa grouted with cement sand (1:3) on top of R.C.C slab etc complete.

Qty:570 sft					@RS:5310.35		% Sft		RS:30269/-

12/-	Distempering 02 coats .

Qty:1444 sft					@RS:1043.90		% Sft		RS:10574/-

13/-	Painting Old Surface Painting door & windows any type .

Qty:164 sft					@RS:1160.06		% sft		RS:1902/-

Page no:2.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

14/-	Supplying & Fixing false ceiling  of plaster , in panels including making frame , work of deodar wood including painting with soligia paint.(S.NO:52 P.NO:64).

Qty: 168 Sft					@Rs:25293.42		% Sft		RS: 42493/-

15/-	Providing & Fixing plaster of pairs ceiling border of 8” -10” with of specified design & thickness i/c fixing besides ceiling with nails/screws with jetties.(S.I No:44 P.NO:39)

Qty:52 Rft					@Rs:104.22		P.Rft		RS: 5419/-

16/-	Glavanized wire gauze fixed to chowakats including fixing in position.

Qty: 16 Sft					@RS: 190.72		P.Sft		RS: 3052/-

====================================

PART “A” BUILDINGS SCHEDULE ITEMS TOTAL	RS: 505864/-

| PART "A” |  | ________________%Above/Below Buildings Schedule Items |  |  | Total Rs:- |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | G. Total Rs:- |  |  |  |
|  |  |  |  | EXECUTIVE ENGINEER |  |  |
| CONTRACTOR |  |  |  | PROVINCIAL BUILDINGS DIVISION |  |  |
|  |  |  |  | SUKKUR |  |  |

BILL OF QUANTITIES

Discription & Rate of items based on Schedule/ Market (offered Rates).

NAME OF WORK: 	M/R to Superintdent Office @ Centeral Jail Khairpur.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

BUILDINGS SCHEDULE ITEMS

1/-	Dismantling brick work in cement or lime mortar etc .(S.I No:     P.NO:    )

Qty: 08 cft					@RS: 1285.63		% Cft		RS: 103/-

2/-	Dismantling glazed or encaustic tiles etc (S.I No:55 P.NO:13).

Qty: 344 Sft					@RS: 786.50		% Sft		RS; 2706/-

3/-	Removing door with Chowakats (S.I No: 33 (a)P.NO:120.

Qty: 812 Sft					@RS: 121/-		% Sft		RS: 983/-

4/	Fabrication of mild steel reinforcement for cement concrete i/c cutting bending laying

in position making joints and fastenings i/c cost of binding wire (also includes removal

of rust from bars) (a) Using tor bars. (S.I.No.7 (a) P.No.20)

Qty: 0.370 cwt					@RS: 5001.70		P.cwt		RS: 1851/-

5/-	R.C.C. work in roof slab beams columns rafts lintels and other structural members laid

in situ or precast laid in position complete in all respects: (i) Ratio (1:2:4) 90 lbs

cement 2 cft sand 4 cft shingle 1/8” to 1/4” gauge. (S.I.No.6 (a-i) P.No. 19)

Qty: 05 cft					@RS: 337/-		P.Cft		RS: 1685/-

6/-	Pacca Brick work in ground floor in cement sand mortar Ratio (1:60 (S.I No:   P.    ).

Qty: 08 cft					@RS: 12674.36	% Cft		RS: 1014/-

7/-	Cement Concrete brick or Stone Ballast 1” to 2”-0 gauge Ratio (1:6:12) (S.I No: P.NO:    )

Qty: 35 cft					@RS: 8122.95		% Cft		RS: 2843/-

8/-	Cement Concrete plain including placing compacting finishing & curing, complete (including screening and washing of stone aggregate without shuttering. Ratio 1:2:4. (S.I. No. 5(f), P. No.16)

Qty: 18 cft					@RS: 14429.25	% Cft		RS: 2597/-

9/-	Applying Floating coat of cement  1/32” thick.

Qty: 812 Sft					@RS: 660/-		% Sft		RS: 5359/-

10/-	Cement Plaster 1:6 up to 20’-0’ height ½” thick (S.I No:    P.NO:      ).

Qty: 812 Sft					@RS: 2206.60		% Sft		RS: 17918/-

11/-	Cement Plaster 1:4 up to 20’-0’ height 3/8” thick (S.I No:    P.NO:      ).

Qty: 541 Sft					@RS: 2197.52		% Sft		RS: 11889/-

12/-	Providing and fixing G.I frames/chowkhats of size 7”x2” or 4½”x3” for door using 20 gauge G.I sheet i/c welded hinges and fixing at site with necessary hold fasts, filling with cement sand slurry of ratio 1:6 and repairing the jambs. The cost also i/c all carriage, tools and plants used in making and fixing.   (S.I.No. 29 Page No. 98

Qty: 50 Rft		@RS: 228.90				P.Sft		RS: 11445/-

Page no:2.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

13/-	Providing and laying tile glazed (6”x 6”1/4”) on floor of wall facing in required colour and pattern of STILE specification jointed in white cement and pigment over a base of 1:2 gray cement mortar ¾” thick i/c washing and filling joints with slurry of white cement and pigment in desired shape with finishing, clearing and cost of was polish etc. complete i/c cutting tiles to proper profile. Amended  (S.I. No. 60, P. No.47)

Qty: 486 Sft					@RS: 30509.77	% SFt		RS: 148277/-

14/-	First class deodar wood  wrought joinery  in doors & windows  etc fixed in position

including  hold fasts hinges  iron  tower bolts chock, cleats  handles  and  cords  with

hooks etc complete.	(S.I No: 54 P.NO:  65).

Qty: 21 Sft					@RS: 902.93		% Sft		RS:  18962/-

15/-	Distempering 02 coats

Qty: 105 Sft					@RS: 1043.90		% Sft		RS: 1096/-

16/-		Preparing the surface and painting with mat finish paint of approved make to old matt

finish 	surface paris and then painting with weather coat of approved make. (03 Coats)

(old surface) (S.I. No. 37  (a+b)  P. No.61)

Qty: 485 Sft					@RS: 2499.76		% sft		RS: 12124/-

17/-	Painting New Surface Painting Doors & Windows any type 03 coats.

Qty: 112 Sft					@RS: 2116.41		% Sft		RS: 1481/-

============================

PART “A” BUILDINGS SCHEDULE ITEMS TOTAL	RS: 242333/-

| PART "A” |  | ________________%Above/Below Buildings Schedule Items |  |  | Total Rs:- |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | G. Total Rs:- |  |  |  |
|  |  |  |  | EXECUTIVE ENGINEER |  |  |
| CONTRACTOR |  |  |  | PROVINCIAL BUILDINGS DIVISION |  |  |
|  |  |  |  | SUKKUR |  |  |

BILL OF QUANTITIES

Discription & Rate of items based on Schedule/ Market (offered Rates).

NAME OF WORK: 	M/R to GRAIN GODOWN TANDO MASTI KHAN @ DISTRICT KHAIRPUR.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

BUILDINGS SCHEDULE ITEMS

01/-	Excavation in foundation of building bridges and other structures i/c dag balling dressing refilling around structure with excavated earth watering and ramming lead up-to 5ft. (b) in ordinary soil. (S.I.No.18 (b) P.No.4)

Qty:2053 cft					@RS:3176/25		% Cft		RS:6521/-

02/-	Cement concrete stone of brick ballast 1½” to 2” gauge. Ratio (1:5:10). (S.I.No.4 (c) P.No.15)

Qty:551 Cft					@RS:8694/95		% Scft		RS: 47909/-

03/-	Cement Concrete plain including placing compacting finishing & curing, complete (including screening and washing of stone aggregate without shuttering. Ratio 1:3:6. (S.I. No. 5(f), P. No.16)

Qty:1473 Cft					@RS:12595/-		% Cft		RS:185524/-

04/-	Random rubble masonry (un-coursed) (d) in cement sand mortar (iv) Ratio 1:6. (S.I No:3

P.NO:28  ).

Qty: 558 Cft					@RS: 17723/-		% Cft		RS: 98894/-

05/-	Coursed rubble masonry including hammer dressing (d) in cement sand mortar (iv) Ratio 1:6.(S.I.No.4 (d)(iv) P.No.28)

Qty: 1717 Cft					@RS: 25321/-		% Cft		RS: 434762/-

06/-	R.C.C. work in roof slab beams columns rafts lintels and other structural members laid in situ or precast laid in position complete in all respects: (i) Ratio (1:2:4) 90 lbs cement 2 cft sand 4 cft shingle 1/8” to 1/4” gauge. (S.I.No.6 (a-i) P.No. 17)

Qty: 2255 Cft					@RS: 337/-		P.Cft		RS: 759935/-

07/-	Fabrication of mild steel reinforcement for cement concrete i/c cutting bending laying in position making joints and fastenings i/c cost of binding wire (also includes removal of rust from bars) (a) Using tor bars. (S.I.No.7 (a) P.No.20)

Qty: 100.66 Cwt				@RS: 5001.70		P.cwt		RS: 503471/-

08/-	Supplying & Fixing  Sand under floor plugging into wall.

Qty: 5731 Cft					@RS: 1141.25		% Cft		RS: 65405/-

09/-	Add: Extra for Lead  6-0 Miles

Qty: 5731 Cft					@RS: 771.96		% Cft		RS: 44241/-

10/-	Stone Pitching including Sub-base with hammer dressed stone on surface laid-in course including carriage of materials chain (S.I NO: 23 P-32).

Qty: 3352 Cft					@RS: 5377.63		% Cft		RS: 180258/-

Page no:2.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

11/-	Pacca  Brick  work in  foundation & plinth  work  in cement sand mortar (1:6) (S.I No:5 P-25).

Qty: 32 Cft					@RS: 11948.36	% Cft		RS: 3823/-

12/-	Cement Plaster 1:6 up to 20’-0 height ½” thick. ( S.I NO: 13(b)    P:NO:52   )

Qty: 590 Sft					@RS: 2206.60		% Sft	          RS: 13019/-

13/-	Cement Plaster 1:4 up to 20”-0 height 3/8” thick. . ( S.I NO: 11(a) P:NO:52

Qty: 590 Sft					@RS: 2197.52		% Sft	           RS:12965/-

14/-	Cement pointing raised on ston work 1:2 .

Qty: 380 Sft					@RS: 1726.73		% Sft	            RS: 6562/-

===================

PART “A” BUILDINGS SCHEDULE ITEMS TOTAL	RS: 2363337/-

| PART "A” |  | ________________%Above/Below Buildings Schedule Items |  |  | Total Rs:- |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | G. Total Rs:- |  |  |  |
|  |  |  |  | EXECUTIVE ENGINEER |  |  |
| CONTRACTOR |  |  |  | PROVINCIAL BUILDINGS DIVISION |  |  |
|  |  |  |  | SUKKUR |  |  |

BILL OF QUANTITIES

Discription & Rate of items based on Schedule/ Market (offered Rates).

NAME OF WORK: 	M/R TO  SACHAL  ACADEMY AT DISTRICT KHAIRPUR (Lavotry Block).

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

PART “A(I)” BUILDINGS SCHEDULE ITEMS.

1/-	Excavation in foundation of building bridges and other structures i/c dag balling dressing refilling around structure with excavated earth watering and ramming lead up-to 5ft. (b) in ordinary soil. (S.I.No.18 (b) P.No.4)

Qty: 1856 Cft					@RS: 3176.25		% Cft		RS: 5895/-

2/-	Cement concrete stone of brick ballast 1½” to 2” gauge. Ratio (1:5:10). (S.I.No.4 (c) P.No.15)

Qty: 744 Cft					@RS: 8694.95		% Cft		RS: 64690/-

3/-	Random rubble masonry (un-coursed) (d) in cement sand mortar (iv) Ratio (S.I.No.1 (d)(iv) P.No.32).

Qty: 720 cft					@RS: 17723/-		% Cft		RS: 127606/-

4/-	Coursed rubble masonry including hammer dressing (d) in cement sand mortar (iv) Ratio 1:6.(S.I.No.2 (d)(iv) P.No.32)

Qty: 524 cft					@RS: 25321/-		% Cft		RS: 132682/-

5/-	Fabrication of mild steel reinforcement for cement concrete i/c cutting bending laying in position making joints and fastenings i/c cost of binding wire (also includes removal of rust from bars) (a) Using tor bars. (S.I.No.7 (a) P.No.20)

Qty: 25.07 cwt					@RS: 5001.70		P.cwt		RS: 125128/-

6/-	R.C.C. work in roof slab beams columns rafts lintels and other structural members laid in situ or precast laid in position complete in all respects: (i) Ratio (1:2:4) 90 lbs cement 2 cft sand 4 cft shingle 1/8” to 1/4” gauge. (S.I.No.6 (a-i) P.No. 17)

Plinth Beam

Qty: 226 Cft					@RS: 337/-		P.Cft		RS: 76162/-

7/-	Pacca Brick work in Ground Floor in cement sand  mortar Ratio 1:6 (S.I No:   P.NO:    ).

Lav: Block

Qty: 1944 cft					@RS: 12674.36	% Cft		RS: 246390/-

8/-	First class deodar wood  wrought joinery  in doors & windows  etc fixed in position  including  hold fasts hinges  iron  tower bolts chock, cleats  handles  and  cords  with hooks etc complete.(S.I No: 54 P.NO:  65).

Qty: 328 sft					@RS: 902.93		P.Sft		RS: 296161/-

9/-	Supplying & Fixing R.S Girder (4”x8”) Material Schedule.

Qty: 24.285 cwt				@RS: 3850/-		P.cwt		RS: 93497/-

10/-	Supplying & Fixing R.S  T-Iron  Material Schedule.

Qty: 15.178 cwt				@RS: 3575/-		P.cwt		RS: 54261/-

Page no:2.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

11/-	Erecting rolled steel beam etc complete

Qty: 39.463 cwt				@RS: 186.34		P.cwt		RS: 7364/-

12/-	Second class tile roofing consisting of 4” earth and 1” mud plaster with gobri leaping over ½” thick cement plaster 1:6 with 34 Lbs of hot bitumen coating sand blinded provided over one layer of 12”x6”x2” tiles laid in 1:6 cement mortar including 1:2 cement pointing under neath of tiles complete including curing etc.S.I.NO. 2 Page No. 33).

Qty: 759 Sft					@RS: 7607.25		% Sft		RS: 57739/-

13/-	Providing and laying tile glazed (6”x 6”1/4”) on floor of wall facing in required colour and pattern of STILE specification jointed in white cement and pigment over a base of 1:2 gray cement mortar ¾” thick i/c washing and filling joints with slurry of white cement and pigment in desired shape with finishing, clearing and cost of was polish etc. complete i/c cutting tiles to proper profile. Amended  (S.I. No. 60, P. No.47)

Qty: 510 Sft					@RS: 30509.77	% Sft		RS: 155600/-

14/-	White Glazed tiles ¼” thick dado jointed in white cement and laid over 1:2 cement sand mortar ¾” thick including finishing (S.I No:37 P.No;45).

Qty: 1720 Sft					@RS: 28253.61	% Sft		RS: 485962/-

15/-	Cement Plaster 1:6 up to 20’-0 height ½” thick. ( S.I NO: 13(b)    P:NO:52   )

Qty: 4318 Sft					@RS: 2206.60		% Sft		RS: 95281/-

16/-	Cement Plaster 1:4 up to 20’-0’ height 3/8” thick.

Qty:4318 Sft					@RS: 2197.52		% Sft		RS:94889/-

17/-	Cement Pointing on Stone work (Raised).

Qty: 218 Sft					@RS: 1758.08		% Sft		RS: 3833/-

18/-	Distempering 03 coats.

Qty: 3416 Sft					@RS: 1565.85		% Sft		RS: 53489/-

19/-	Supplying & Fixing false ceiling  of plaster , in panels including making frame , work of deodar wood including painting with soligia paint.(S.NO:52 P.NO:64).

Qty: 510 Sft					@RS: 25293.42	% Sft		RS: 128996/-

20/-	Providing & Fixing plaster of pairs ceiling border of 8” -10” with of specified design & thickness i/c fixing besides ceiling with nails/screws with jetties.(S.I No:44 P.NO:39)

Qty: 384 rft					@RS: 104.22		P.rft		RS: 40020/-

21/-	Supplying & Fixing Sand Under floor plugging into wall

Qty: 510 cft					@RS: 1913.21		% Cft		RS: 9757-

================

PART “A(I)” BUILDINGS SCHEDULE ITEMS TOTAL	         RS: 2355402/-

Page no:3.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

PART “B(II)” Guard Room BUILDINGS SCHEDULE ITEMS.

1/-	Excavation in foundation of building bridges and other structures i/c dag balling dressing refilling around structure with excavated earth watering and ramming lead up-to 5ft. (b) in ordinary soil. (S.I.No.18 (b) P.No.5)

Qty:385.0 cft					@Rs:3176/25		% cft		RS:1223/-

2/-	Cement concrete stone of brick ballast 1½” to 2” gauge. Ratio (1:3:6). (S.I.No.4 (a) P.No.17)

Qty:143.0 cft					@Rs:9416/28		% cft		RS:13465/-

3/-	Random rubble masonry (un-coursed) (d) in cement sand mortar (iv) Ratio 1:6.

Qty:176.0 cft					@RS:17723/-		% cft		RS:31192/-

4/-	Coursed rubble masonry including hammer dressing (d) in cement sand mortar (iv) Ratio 1:6.(S.I.No.2 (d)(iv) P.No.32)

Qty:165.0 cft					@Rs:25321/-		% cft		RS:41780/-

5/-	Fabrication of mild steel reinforcement for cement concrete i/c cutting bending laying in position making joints and fastenings i/c cost of binding wire (also includes removal of

rust from bars) (a) Using tor bars. (S.I.No.7 (b) P.No.17).

Qty:2.24 cwt					@Rs:5001/70		P.cwt		RS:11204/=

6/-	R.C.C. work in roof slab beams columns rafts lintels and other structural members laid in situ or precast laid in position complete in all respects: (i) Ratio (1:2:4) .(S.I No:6(a)P-17).

Qty:50.0 cft					@Rs:337/-		P.Cft		RS:16850/-

7/-	Supplying & Fixing  R.S Girder (3”x6”x4”) size Material Schedule.

Qty:2.875 cwt					@Rs:3850/-		P.cwt		RS:11069/-

8/-	Supplying & Fixing T-Iron 2”x2”x1/4”  size  Material Schedule.

Qty:3.803 cwt					@Rs:3575/-		P.cwt		RS:9704/-

9/-	Erecting  rolled  steel beam  or old  rails  in roof  etc erection  and fixing   in position

(S.I NO:6 P-NO:96).

Qty:5.589 cwt					@Rs:186/34		P.cwt		RS:1041/-

10/-	Second class tile roofing consisting of 4” earth and 1” mud plaster with gobri leaping over ½” thick cement plaster 1:6 with 34 Lbs of hot bitumen coating sand blinded provided over one layer of 12”x6”x2” tiles laid in 1:6 cement mortar including 1:2 cement pointing under neath of tiles complete including curing etc.S.I.NO. 2 Page No38.)

Qty:155.0 sft					@Rs:7607/25		% sft		RS:11791/-

11/-	Supplying & Fixing sand under floor plugging into wall.

Qty:240.0 cft					@RS:1141/25		% Cft		RS:2739/-

12/-	Pacca brick  work in ground floor  20’ height  in (e)  cement   sand mortar ratio 1:6 (S.I No:7(i-e)P-26).

Qty:374.0 cft					@Rs:12674/36		% cft		RS:47402/-

13/-	Cement  Plaster 1:6 up to 20’-0’ height ½” thick (S.I No:          P NO:          ).

Qty:1355.0 sft					@Rs:2206/60		% sft		RS:29899/-

Page no:4.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

14/-	Cement  Plaster 1:4 up to 20’-0’ height 3/8” thick  (S.I No:          P No:        )

Qty:1355.0 sft					@Rs:2197/55		% Sft		RS:29776/-

15/-	Providing & Laying  2” thick  topping  cement concrete (1:2:4) including  surface  finishing  and dividing  into panels  (c)2”  thick (S.I No:16(c)P-47).

Qty:120.0 sft					@Rs:3275/50		% sft		RS:3931/-

16/-	First Class Deodar wood wrought, joinery in doors and windows etc, fixed in position including Chowkhats hold fats hinges, iron tower bolts, chocks cleats, handles and cords with hooks etc. deodar paneled or paneled and glazed or fully glazed, (b) 1 ¾’’ thick.(S.I. No. 7, (b) P. No. 65)

Qty:75.0 sft					@Rs:1273/76		P.Sft		RS:75532/-

17/-	Pointing  flush  stone work (Raised) (S.I No:20 P-59).

Qty:110.0 sft					@Rs:1726/73		% sft		RS:1899/-

18/-	Providing and laying tile glazed 6”x6”x ¼” on floor of wall facing in required colour and pattern of STILE specification jointed in white cement and pigment over a base of 1:2 gray cement mortar ¾” thick i/c washing and filling joints with slurry of white cement and pigment in desired shape with finishing, clearing and cost of was polish etc. complete i/c cutting tiles to proper profile.   (S.I. No. 60, P. No.47)

Qty:142.0 sft					@Rs:30509/77		% Sft		RS:43324/-

19/-	White washing  03 coats (S.I No:       P: NO       ).

Qty:120.0 sft					@Rs:829/95		% Sft		RS:996/-

20/-	Distempering 03 Coats. (S.I No:        P:   NO       )

Qty:1355 sft					@Rs:1079/65		% sft		RS:14629/-

21/-	Painting New Surface  Painting  guard bars gets  iron bars grating railing including standard  braces etc. and similar open work . 03 coats (S.I No:05(d-ii) P-NO:06).

Qty:177.0 sft					@Rs:1270/83		% sft		RS:2249/-

22/-	Painting New Surface  door & windows any type  03 coats.

Qty:148.0 sft					@Rs:2116/41		% sft		RS:3132/-

23/-	R.C.C  spout including Fixing  in position.

Qty:02 Nos					@Rs:524/98		each		RS:1050.0

=======================================

PART “A(II)” BUILDINGS SCHEDULE ITEMS TOTAL	RS:425877/-

Page no:5.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

| PART "A(I+II)” |  | ________________%Above/Below Buildings Schedule Items |  |  | Total Rs:- |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | G. Total Rs:- |  |  |  |
|  |  |  |  | EXECUTIVE ENGINEER |  |  |
| CONTRACTOR |  |  |  | PROVINCIAL BUILDINGS DIVISION |  |  |
|  |  |  |  | SUKKUR |  |  |

BILL OF QUANTITIES

Discription & Rate of items based on Schedule/ Market (offered Rates).

NAME OF WORK:- 	M/R TO  OLD  CIRCUIT HOUSE @

KHAIRPUR.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

PART “A” BUILDINGS SCHEDULE ITEMS.

1/-	Dismantling brick work in lime or cement mortar (S.I No:13 page-10).

Parapet room wall

Qty:939.0 cft					@Rs:1285/63		% Cft		RS:12072/-

2/-	Removing cement or lime plaster  (S.I No:53 page-13).

Qty:2955 sft					@Rs:121/-		% cft		RS:3576/-

3/-	Scraping ordinary  distemper oil bond distemper or paint on walls (S.I no: 54 P.NO:13.

Qty:14794 sft					@Rs:226.88		% sft		RS:33565/-

4/-		Dismantling glazed or  encaustic  tile etc (S.I No:55 P-13).

Qty:532.0 					@Rs:786/50		% sft		RS:4184/-

5/-		Removing door  with chowakats (S.I No:33 (a) P-12).

Qty:15 Nos					@Rs:142/18		each		RS:710/-

6/-		Pacca Brick work in ground floor in (e) cement sand mortar ratio 1:6. (S.I.No.5 (e) P.No.25)

Qty:939.0 cft					@Rs:12674/36		% cft	RS:119012/-

7/-		Providing & Fixing ornamental  cement jalli 2” thick (1:2:4) without steel (S.I No:11P-18).

Qty: 240.0 sft					@Rs:226/02		P.sft		RS:54245/-

8/-		Applying Floating coat of cement 1/32” thick. (S.I.No.14 P.No.59)

Qty:2955 sft					@Rs:660/-		% sft		RS:19503/-

9/		Cement Plaster 1:6 upto 20’ height (c) ½” Thick. (S.I. No.11 (a), P. No.58).

Qty:2955	sft					@Rs:2206/60/-		% sft		RS:65203/-

10/-	Cement Plaster 1:4 upto 20’ height (c) 3/8” Thick. (S.I. No.11 (a), P. No.58)

Qty:2955 sft					@Rs:2197/52/-		% sft		RS:64937/-

11/-	Providing and laying tile glazed (6”x 6”1/4”) on floor of wall facing in required colour and pattern of STILE specification jointed in white cement and pigment over a base of 1:2 gray cement mortar ¾” thick i/c washing and filling joints with slurry of white cement and pigment in desired shape with finishing, clearing and cost of was polish etc. complete i/c cutting tiles to proper profile. Amended  (S.I. No. 60, P. No.52)

Qty:532  sft					@Rs:30509/77		% sft		RS:162312/-

12/-	S/F false ceiling of plaster of  Paris, in panels i/c making frame, work of deodar wood i/c painting with soligia paint.(S.I.NO.52 Page No. 72).

Qty:268.0 sft					@Rs:25293/42		% sft	             RS:67786/-

13/-	P/F plaster of pairs ceiling border of 8”-10” with of specified design & thickness i/c fixing besides ceiling with nails screws with getties.(S.I.No.44 P.No.45)

Qty: 862 sft					@Rs:104/22		P.Rft		RS:89838/-

Page no:2.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

14/-	First class deodar wood wrought joinery  in door and windows  etc  fixed in i/c  chowakats holds fastes hinges , iron tower bolts chowks cleats  , handles, cord with hooks etc.

Deodar panelled or panelled &  glazed ,  or fully glazed 1¾” thixk e. (S.I.No.7 (b) P.No.58)

Qty: 49sft						@Rs: 902.93		P.sft		RS: 44244/-

15/- 	Preparing the surface and painting with mat finish paint of approved make to old matt finish surface paris and then painting with weather coat of approved make. (03 Coats) (old surface) (S.I. No. 37  (a+b)  P. No.61)

Qty:18523.0 sft					@Rs:3444/38		% sft		RS:638003/-

16/-	P/F   colour create to wall surface to provide ,durable ceust & aesthetics heaving thickness up to ¾” with specified colour heaving water . fire &  termite resistance up to 20’ ft : height (S.I NO:44 P-63).

Qty: 800 sft					@Rs:6319/50		% sft		RS: 44237/-

17/-	Galvanized wire gauze 144 mesh per square inch iron fitting complete. (d) Galvanized wire gauze fixed to chowkats with ¾” deodar strips and screw. (S.I.No.14 (d) P.No.67)

Qty:245.0 sft					@Rs:190/72		P.Sft		RS: 46726/-

18/-	Providing & Fixing  RCC spouts  i/c in position .

Qty:12.0 Nos					@Rs:524/98		each		RS:6300/-

19/-	Providing & Fixing approved Quality Mortic Lock etc complete

Qty:06 No’s					@Rs:1786.13		each		RS:10716/-

20/-	S/F wall panels i/c supply and making deodar wooden second frame work etc complete

Qty: 230 sft					@Rs:31082.30		% sft		RS: 71489/-

21/-	Single layer tile 9”x4 ½’x2” laid  over 4” earth 1” mud plaster with out Bhoosa  grouted with cement sand 1:3 on top of R.C.C slab provide with 34 Lbs bitumen coating  sand blunded. Etc complete

Qty:3369 sft					@Rs:5310.35		% sft		RS:178906/-

22/-	Providing & Fixing Single Layer Polythen sheet 0.13 mm thick for water proofing  as per specification and instruction of Engineer Incharge (S.I No: 38 P.NO: 380.

Qty:3369 sft					@ RS:10.70		P.Sft		RS:36048/-

23/-	French Polish complete (b) on old work.

Qty: 1092 sft					@Rs:1952.50		% sft		RS: 21321/-

24/-	Cement Pointing struct joints on walls 1:2 etc complete

Qty:1080 sft					@Rs:1287.44		% sft		RS:13904/-

27/-	Distempering 02 coats .

Qty:5784.0 sft					@Rs:1043.90		% Sft		RS:60379/-

28/-	P/L HALA pattern tiles glazed 8”x8”x¼” on floor of wall facing in required pattern of STILE specification jointed in white cement and pigment over a base of 1:2 grey cement mortar ¾” thick i/c washing and filling joints with slurry of white cement and pigment in desired shape with finishing cleaning and cost of wax polish etc. complete i/c cutting

Qty: 176 sft					@Rs:47651.56		% Sft		RS: 83867/-

==================

PART “A” BUILDINGS SCHEDULE ITEMS TOTAL RS: 1946783/-

Page no:3.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

| PART "A(I+II)” |  | ________________%Above/Below Buildings Schedule Items |  |  | Total Rs:- |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | G. Total Rs:- |  |  |  |
|  |  |  |  | EXECUTIVE ENGINEER |  |  |
| CONTRACTOR |  |  |  | PROVINCIAL BUILDINGS DIVISION |  |  |
|  |  |  |  | SUKKUR |  |  |

BILL OF QUANTITIES

Discription & Rate of items based on Schedule/ Market (offered Rates).

NAME OF WORK :-	SACHAL SAR MAST PUBLIC LIBRARY @ KHAIRPUR (Intt: W/S & S/F External

Drainage  & Extrernal Development)

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

PART-B    Internal W/S S/F (W/S & S/F SCHEDULE ITEMS.

1-		P/F squatting type white glazed of flushing cistern with internal fitting and flush pipe with band and making requisite Nos. of holes in wall plinth & floor for pipe connections and making good in cement concrete 1:2:4. (A) W.C pan of not less than 23” clear opening between flashing rims and 3 gallons flushing tank. With 4” dia C.I. tape.

(S.I.NO.1(A) P.No.1)

Qty:05						@Rs:5088/20		each		RS:25441/-

2-		P/F European type white glazed earthenware wash down w.c pan complete with and i/c the cost of white/black plastic seat (Best Quality) and lid with c.p brass hinges and buffers. 3 gallons white glazed earthen low level flushing cistern with siphon fitting 1-1/2” dia white porcelain enameled flush bend ¾” dia & cutting & making requisite number of holes in walls plinth and floor for pipe connection and making good in cement concrete 1:2:4. (Foreign Quality).(S.I.NO.5 P. No.2)

Qty:04 Nos					@Rs:11477/40	each		RS:45910/-

3-	P/F 24”x18” lavatory basin in white glazed earthen ware complete with & i/c the cost of W.I or C.I cantilever brackets 6” built into wall painted white in two coats after a primary coat of red lead paint a pair of ½” dia chrome plate pillar traps 1-1/2” dia rubber plug & chrome plated brass chain 1-1/4” dia malleable iron or c.p brass traps malleable iron or brass union & making requisite number of holes in walls plinth & floor for pipe connection and making good in cement concrete 1:2:4 (standard pattern). Add extra for labour for providing & fixing of earthenware pedestal white or colored glazed (Foreign equivalent ) .  (S.I.NO. 8 P.

Qty:12 Nos					@Rs:4253/70		each		RS:59136/-

4/-	Add extra for labour for providing & fixing of earthenware pedestal white or colored glazed (Standard pattern) .  (S.I.NO. 9 P. No.3)

Qty:12 Nos					@Rs:938/47		each		RS:11262/-

5-	P/F 6”x2” or 6”x3” C.I floor trap of the approved self cleaning design with a C.I screwed down gritting with or without a vent arm complete with and i/c making requisite number or holes in walls plinth and floor for pipe connections & making good in CC 1:2:4. (S.I. No. 20 P. No.5)

Qty:30   Nos					@Rs:2042/13		each		RS:61264/-

6-	P/F in position nylon connections complete with 1/2” dia brass stop cock with pair of brass 	nuts and lining joints to nylon connection. (S.I. No. 23 P. No.6)

Qty: 46   Nos					@Rs:447/15		each		RS:20569/-

7/-	P/F 24” x 18” beveled edge mirror of Belgium glass complete with 1/8” thick hard board and C.P screw fixed to wooden plate (a) standard pattern.  (S.I.No. 03 (a) P.No7).

Qty:12   Nos					@Rs:1711/60		each		RS:20539/-

8-	S/F in position concealed tee  stopcock (b)½” dia C.P stop cock standard pattern.

(S.I. No. 4 (b) P. No.13)

Qty:10   Nos					@Rs:889/46		each		RS:8895/-

Page no:2.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

9-		Supplying and fixing long bib-cock of superior quality with C.P. head ½” dia.

(S.I.NO.136P. No.19)

Qty:10 Nos					@Rs:1109/46		each		RS:11095/-

10-	Supplying and fixing jet shower with rod of superior quality with c.p head ½” dia.

(S.I.NO.05 P. No.19)

Qty:  04  Nos					@Rs:1142/24		each		RS:4569/-

11-	Supplying and fixing  wash basin  mixture of superior quality  with c.p head..

(S.I.NO.14 (b) P. No.19)

Qty: 05  Nos					@Rs:3179/-		each		RS:15895/-

12-	Supplying & fixing bath room accessories set (7 piece) i/c towel rod, brush holder soap tray. Shelf of approved quality and design etc. Complete (S.I. No. 24 P. No.16)

Qty:  04  Nos					@Rs:10322/40	each		RS:41290/-

13-	Providing & Fixing C.C gulley trap etc complete

Qty: 10   Nos					@Rs:1429/67		each		RS:14297/-

14/-	S/F chrome  plated brass towel rail  complete with brackets  fixing  on wooden  cleats 1”

laying C.P brass screw 24” dia.

Qty:04 Nos					@Rs:972/95		each		RS:3892/-

15-	P/F steel stainless local make complete with cast iron or wrought iron brackets 6” built in wall 1-½” rubber plug chrome brass chain 1-½” C.P brass waste with 1-½” plate P.V.C waste pipe and making requisite numbers of holes in wall & plinth and floor for pipe connection and making good in cement concrete 1:2:4. (c) Steel sink stainless sized 33” x 18” local make (Standard Pattern). (S.I.NO. 09(c) P. No.05)

Qty:  04   Nos					@Rs:5162/30		each		RS:20649/-

=============

PART “A” W/S & S/F SCHEDULE ITEMS TOTAL 	RS:364703/-

PART-     “ A”   (Plinth Protection) BUILDINGS SCHEDULE ITEMS.

1/- 	Excavation in foundation of building bridges and other structures i/c dag balling dressing refilling around structure with excavated earth watering and ramming lead up-to 5ft. (b) in ordinary soil. (S.I.No.18 (b) P.No.5)

Qty:1388.0 cft					@Rs:3176/25		%0cft		RS:4409/-

2/- 	Cement concrete stone of brick ballast 1½” to 2” gauge. Ratio (1:5:10). (S.I.No.4 (b) P.No.17)

Qty:463.0 cft					@Rs:8694/95		% cft		RS:4025/-

3/-	Pacca Brick work in foundation /Plinth  in cement sand  mortar Ratio (1:6)(S.I No: P-    ).

Qty:1215.0 cft					@Rs:11948/36	% Cft		RS:145173/-

Page no:3.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

4/-	Supplying & Fixing sand under floor plugging in to wall,

Qty:2760.0 cft					@Rs:1913/21		% cft		RS:52805/-

5/-	Cement concrete stone of brick ballast 1½” to 2” gauge. Ratio (1:6:12). (S.I.No.4 (b) P.No.17)

Qty:420.0 cft					@Rs:8122/95		% cft		RS:34117/-

06/-	Providing and lying 1” thick topping cement concrete (1:2:4) i/c surface finishing and dividing into panels. (c) 2” Thick  (S.I.No.16 (c) P.No.47)

Qty:1619.0 sft					@Rs:3275/50		% sft		RS:53030/-

07/-	Cement  Plaster 1:6 up to 20’-0’ height ½” thick.

Qty:1157.0 sft					@Rs:2206/60		% sft		RS:25530/-

08/-	Cement  Plaster 1:4 up to 20’-0’ height ” thick.3/8

Qty:1157.0 sft					@Rs:2197/52		% sft		RS:25425/-

===============================================

PART “A(I)” BULDINGS SCHEDULE ITEMS TOTAL 	RS:380747/-

PART-A(I)   (U. G. Tank ) BUILDINGS SCHEDULE ITEMS.

1/- 	Excavation in foundation of building bridges and other structures i/c dag balling dressing refilling around structure with excavated earth watering and ramming lead up-to 5ft. (b) in ordinary soil. (S.I.No.18 (b) P.No.5)

Qty:1056.0 cft					@Rs:3176/25		%0cft		RS:3354/-

2/- 	Cement concrete stone of brick ballast 1½” to 2” gauge. Ratio (1:5:10). (S.I.No.4 (b) P.No.17)

Qty:198.0 cft					@Rs:8694/95		% cft		RS:17216/-

3/-	Pacca Brick work in foundation /Plinth  in cement sand  mortar Ratio (1:6)(S.I No: P-    ).

Qty:220.0 cft					@Rs:11948/36	% cft		RS:26286/--

4/-	Perforated Pacca brick walling 4 ½” thick etc complete

Qty:95.00 sft					@Rs:3935/75		% sft		RS:3739/-

5/-	Fabrication of mild steel reinforcement for cement concrete i/c cutting bending laying in position making joints and fastenings i/c cost of binding wire (also includes removal of rust from bars) (a) Using tor bars. (S.I.No.7 (a) P.No.20)

Qty: 4.91cwt					@Rs:5001/70		P.cwt		RS:24558/-

6/- 	R.C.C. work in roof slab beams columns rafts lintels and other structural members laid in situ or precast laid in position complete in all respects: (i) Ratio (1:2:4) 90 lbs cement 2 cft sand 4 cft shingle 1/8” to 1/4” gauge.

Qty:132.0 cft					@Rs:337/-		P.cft		RS:44484/-

Page no:4.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

07/-	Cement  Plaster 1:6 up to 20’-0’ height ½” thick.

Qty:442.0 sft					@Rs:2206/60		% sft		RS:9753/-

08/-	Cement  Plaster 1:4 up to 20’-0’ height ” thick.3/8

Qty:442.0 sft					@Rs:2197/52		% sft		RS:9713/-

09/-	Cement Concrete plain including placing compacting finishing & curing, complete (including screening and washing of stone aggregate without shuttering. Ratio 1:2:4.

(S.I. No. 5(f), P. No.16)

Qty:54.0 cft					@Rs:14429/25	% cft		RS:7792/-

===============================================

PART “A(II)  U.G. TANK (BUILDINGS SCHEDULE ITEMS TOTAL 	RS:146895/-

PART-D  NON SHEDULE ITEM

1/-	P/F UPVC pipe ½” dia PAK-ARAB) SCH-40 surface by using clip /saddler/socket/reducer/bush/tee etc .or recessed in masonry C.C or R.C.C upto 20’ ft. height and making good wih C.c i/c curing finishing etc complete as per instruction of the Engineer In charge ,.Specification of the material should meet the requirement of class 12454-Bin accordance with the ASTM-D-1784 type 1 Grade 1. rate include all costs of labor ,material, cartage scaffolding ladders .etc complete (R.A Sanction).

Qty: 200.0 rft					@Rs:			P.rft		RS:-

2/-	P/F UPVC UNION solvent ) ½’ dia (PAK ARAB) SCH-40 make of approved quality and design of various size fixed to UPVC pipe /fitting paid separately using approved adhesive compound upto 20 ft. height as per instruction of the Engineer In charge .Specification of the material should meet the requirement of class 12454-B I Grade .Rate include all costs of labor material ,cartage scaffolding /ladders etc complete.

Qty:12 Nos					@Rs:			Each		RS:-

3/-	P/F UPVC pipe ¾” dia PAK-ARAB) SCH-40 surface by using clip /saddler/socket/reducer/bush/tee / elbow etc .or recessed in masonry C.C or R.C.C upto 20’ ft. height and making good wih C.c i/c curing rinishing etc complete as per instruction of the Engineer In charge ,.Specification of the material should meet the requirement of class 12454-Bin accordance with the ASTM-D-1784 type 1 Grade 1. rate include all costs of labor ,material, cartage scaffolding ladders .etc complete (R.A Sanction).

Qty: 200.0 rft					@Rs:			P.rft		RS:-

4/-	P/F UPVC UNION  ¾” dia PAK-ARAB) SCH-40  make approved quality and Design of various sizes Fixed to UPVC Pipe / Fitting paid Separately  upto 20’ ft. height as per instruction Sof the Engineer In charge Specification of the material should meet the requirement of class 12454-Bin accordance with the ASTM-D-1784 type 1 Grade 1. rate include all costs of labor ,material, cartage scaffolding ladders .etc complete (R.A Sanction).

[

Qty:10 nos					@Rs:			Each		RS:-

Page no:5.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

5/-	P/F UPVC pipe 1” dia PAK-ARAB) SCH-40 surface by using clip /saddler/socket/reducer/bush/tee / elbow etc .or recessed in masonry C.C or R.C.C upto 20’ ft. height and making good wih C.c i/c curing finishing etc complete as per instruction of the Engineer In charge ,.Specification of the material should meet the requirement of class 12454-Bin accordance with the ASTM-D-1784 type 1 Grade 1. rate include all costs of labor ,material, cartage scaffolding ladders .etc complete (R.A Sanction).

Qty:150.0 rft					@Rs:			P.rft		RS:-

6/-	P/F UPVC pipe 4” dia PAK-ARAB) SCH-40 surface by using clip /saddler/socket/reducer/bush/tee /  etc . or recessed in masonry C.C or R.C.C upto 20’ ft. height and making good wih C.C i/c curing finishing etc complete as per instruction of the Engineer In charge ,.Specification of the material should meet the requirement of class 12454-Bin accordance with the ASTM-D-1784 type 1 Grade 1. rate include all costs of labor ,material, cartage scaffolding ladders .etc complete (R.A Sanction).

Qty: 200 Rft					@Rs:			P.rft		RS:-

7/-	P/F UPVC  PLUG TEE 4” dia  PAK-ARAB) SCH-40  make of approved quality and design of various sizes fixed to UPVC Pipe / fitting  paid separately using approved adhesive compound upto   20’ ft. height  as per instruction of the Engineer In charge ,.Specification of the material should meet the requirement of class 12454-Bin accordance with the ASTM-D-1784 type 1 Grade 1. rate include all costs of labor ,material, cartage scaffolding ladders .etc complete (R.A Sanction)

Qty:10.0 Nos					@Rs:			Each		RS:-

8/-	P/F UPVC BEND (LONG) 4” Of PAK ARAB make of approved quality include\ing / making joints by using pest/solution AGM make  as per instruction of the Engineer In charge ,.Rate includes all costs of labour , material, cartage, scaffolding etc complete. (R.A Sanctioned).

Qty:11 Nos					@Rs:			Each		RS:-

9/-	P/F 4” dia UPVC Socket of PAK ARAB make of approved quality including makings joints by using pest/ solution AGM make as per instruction of the Engineer In charge ,. ,.Rate includes all costs of labour , material, cartage, scaffolding etc complete. (R.A Sanctioned).

Qty: 10.0 Nos					@Rs:			Each		RS:-

10/-	P/F   4” dia  UPVC Cowel / Terminal  Guard / End of  PAK ARAB make of approved quality & joints by using pest/solution AGM make  as per instruction of the Engineer In charge ,.Rate includes all costs of labour , material, cartage, scaffolding etc complete. (R.A Sanctioned).

Qty:06 Nos					@Rs:			Each		RS:-

11/-	P/F value full way best quality of local make gun metal with black or red painted sheet threaded i/c rubber washer and fixture with G.I pipe line etc complete. (R.A Sanctioned).

Qty:10 Nos					@Rs:			Each		RS:-

Page no:6.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

12/-	Providing & Fixing  cement concrete kurb stone having size 18”x12”x6” of city quddra / cobble  shape with pigment having strength between 5000 psi  to 8500 psi of 3” thick filling the joints with hill sand & laying in specified manner / pattern and design etc complete (R.A.A).

Qty: 1144 No’s				@Rs:			Each		RS:-

================

PART “D” NON SCHEDULE ITEMS TOTAL 	RS:-

(PART-C   Public Health  Schedule ITEMS.

1/-	Providing UPVC pressure  pipes of class “C” equivalent make fixing in trench i/c cutting, fitting and jointing with “Z” joint with one rubber ring i/c testing with water to head of 61 meter or 200ft. 4” dia. (P.H.E)  (S.I.No. 01 (b) P.No.22)

6” dia 	200 rft					@Rs:373/-		P.Rft		RS:74600/-

4” Dia  100 rft					@RS: 242/-		P.Rft		RS: 24200/-

2/-	Boring for tube well in all water bearing soils from ground level up-to 100 ft or 30.5 meter depth i/c sinking and with drawing of casing pipe. (100 mm 4”) (S.I.No. 01 (b) Page No.36).

4” Dia

Qty:50 rft					@Rs:242/-		P,rft		RS:12100/-

3/-	Construction of manhole  or inspection chamber  for the required dia  of circular sewer   and 3’-6’ (1067 mm ) depth with walls  of B.B  in cement mortar  1:3 cement plaster  1:3 ½”  thick in side 1”  (25 mm ) thick over  beching  and channel i/c  fixing C.I main hole cover with frame of clear opening  1 ½”x 1 ½”” 457x457 mm) of 1.75 cwt . (88.9 kg) embedded in plain c.c 1:2:4) & fixing (25 mm) dia M.S steps 6” 150 mm)   white projecting 4” (102 mm) from the face of walls at 12” (305 mm) c/c duly painted etc complete as per standard specification & drawing.(S.I No- P-46).

Qty:10 No					@Rs:14748/-		P.rft		RS:147480/-

4/-	Construction of  wheel valve  chamber (S.I No:9 P-53)P.H.E).

Qty:02 Nos					@Rs:4103/-		each		RS:8206/-

___________________________________

PART “C” P.H. ENGR: SCHEDULE ITEMS TOTAL 	RS: 266586/-

Page no:7.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

| PART "A(I+II)” |  | ________________%Above/Below Buildings Schedule Items |  |  | Total Rs:- |  |
| --- | --- | --- | --- | --- | --- | --- |
| PART "B" |  | ________________%Above/Below W/S & S/F Schedule Items |  |  | Total Rs:- |  |
| PART "C" |  | ________________%Above/Below P.H.Engr: Schedule Items |  |  | Total Rs:- |  |
| PART "D” |  | ___________________________________Non Schedule Items |  |  | Total Rs:- |  |
|  |  |  | G. Total Rs:- |  |  |  |
|  |  |  |  | EXECUTIVE ENGINEER |  |  |
| CONTRACTOR |  |  |  | PROVINCIAL BUILDINGS DIVISION |  |  |
|  |  |  |  | SUKKUR |  |  |

BILL OF QUANTITIES

Discription & Rate of items based on Schedule/ Market (offered Rates).

NAME OF WORK :-	M/R T SACHAL ACADEMY @ DISTRICT KHAIRPUR

(Int: W/S & S/F & External Devlopment).

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

PART-B   ( Internal W/S S/F).

1-		P/F squatting type white glazed of flushing cistern with internal fitting and flush pipe with band and making requisite Nos. of holes in wall plinth & floor for pipe connections and making good in cement concrete 1:2:4. (A) W.C pan of not less than 23” clear opening between flashing rims and 3 gallons flushing tank. With 4” dia C.I. tape.

(S.I.NO.1(A) P.No.1)

Qty:05						@Rs:5088/20		each		RS:25441/-

2-		P/F European type white glazed earthenware wash down w.c pan complete with and i/c the cost of white/black plastic seat (Best Quality) and lid with c.p brass hinges and buffers. 3 gallons white glazed earthen low level flushing cistern with siphon fitting 1-1/2” dia white porcelain enameled flush bend ¾” dia & cutting & making requisite number of holes in walls plinth and floor for pipe connection and making good in cement concrete 1:2:4. (Foreign Quality).(S.I.NO.5 P. No.2)

Qty:04 Nos					@Rs:11477/40	each		RS:45910/-

3-	P/F 24”x18” lavatory basin in white glazed earthen ware complete with & i/c the cost of W.I or C.I cantilever brackets 6” built into wall painted white in two coats after a primary coat of red lead paint a pair of ½” dia chrome plate pillar traps 1-1/2” dia rubber plug & chrome plated brass chain 1-1/4” dia malleable iron or c.p brass traps malleable iron or brass union & making requisite number of holes in walls plinth & floor for pipe connection and making good in cement concrete 1:2:4 (standard pattern). Add extra for labour for providing & fixing of earthenware pedestal white or colored glazed (Foreign equivalent ) .  (S.I.NO. 8 P.

Qty:12 Nos					@Rs:4253/70		each		RS:59136/-

4/-	Add extra for labour for providing & fixing of earthenware pedestal white or colored glazed (Standard pattern) .  (S.I.NO. 9 P. No.3)

Qty:12 Nos					@Rs:938/47		each		RS:11262/-

5-	P/F 6”x2” or 6”x3” C.I floor trap of the approved self cleaning design with a C.I screwed down gritting with or without a vent arm complete with and i/c making requisite number or holes in walls plinth and floor for pipe connections & making good in CC 1:2:4. (S.I. No. 20 P. No.5)

Qty:30   Nos					@Rs:2042/13		each		RS:61264/-

6-	P/F in position nylon connections complete with 1/2” dia brass stop cock with pair of brass 	nuts and lining joints to nylon connection. (S.I. No. 23 P. No.6)

Qty: 46   Nos					@Rs:447/15		each		RS:20569/-

Page no:2.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

7/-	P/F 24” x 18” beveled edge mirror of Belgium glass complete with 1/8” thick hard board and C.P screw fixed to wooden plate (a) standard pattern.  (S.I.No. 03 (a) P.No7).

Qty:12   Nos					@Rs:1711/60		each		RS:20539/-

8-	S/F in position concealed tee  stopcock (b)½” dia C.P stop cock standard pattern.

(S.I. No. 4 (b) P. No.13)

Qty:10   Nos					@Rs:889/46		each		RS:8895/-

9-		Supplying and fixing long bib-cock of superior quality with C.P. head ½” dia.

(S.I.NO.136P. No.19)

Qty:10 Nos					@Rs:1109/46		each		RS:11095/-

10-	Supplying and fixing jet shower with rod of superior quality with c.p head ½” dia.

(S.I.NO.05 P. No.19)

Qty:  04  Nos					@Rs:1142/24		each		RS:4569/-

11-	Supplying and fixing  wash basin  mixture of superior quality  with c.p head..

(S.I.NO.14 (b) P. No.19)

Qty: 05  Nos					@Rs:3179/-		each		RS:15895/-

12-	Supplying & fixing bath room accessories set (7 piece) i/c towel rod, brush holder soap tray. Shelf of approved quality and design etc. Complete (S.I. No. 24 P. No.16)

Qty:  04  Nos					@Rs:10322/40	each		RS:41290/-

13-	Providing & Fixing C.C gulley trap etc complete

Qty: 10   Nos					@Rs:1429/67		each		RS:14297/-

14/-	S/F chrome  plated brass towel rail  complete with brackets  fixing  on wooden  cleats 1”

laying C.P brass screw 24” dia.

Qty:04 Nos					@Rs:972/95		each		RS:3892/-

15-	P/F steel stainless local make complete with cast iron or wrought iron brackets 6” built in wall 1-½” rubber plug chrome brass chain 1-½” C.P brass waste with 1-½” plate P.V.C waste pipe and making requisite numbers of holes in wall & plinth and floor for pipe connection and making good in cement concrete 1:2:4. (c) Steel sink stainless sized 33” x 18” local make (Standard Pattern). (S.I.NO. 09(c) P. No.05)

Qty:  04   Nos					@Rs:5162/30		each		RS:20649/-

===========

PART “B” (INTERNAL W/S & S/F) W/S & S/F SCHEDULE ITEMS TOTAL	RS:364703/-

Page no:3.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

PART-   “A(I)”   (Plinth Protection) BUILDINGS SCHEDULE ITEMS.

1/- 	Excavation in foundation of building bridges and other structures i/c dag balling dressing refilling around structure with excavated earth watering and ramming lead up-to 5ft. (b) in ordinary soil. (S.I.No.18 (b) P.No.5)

Qty:1388.0 cft					@Rs:3176/25		%0cft		RS:4409/-

2/- 	Cement concrete stone of brick ballast 1½” to 2” gauge. Ratio (1:5:10). (S.I.No.4 (b) P.No.17)

Qty:463.0 cft					@Rs:8694/95		% cft		RS:4025/-

3/-	Pacca Brick work in foundation /Plinth  in cement sand  mortar Ratio (1:6)(S.I No: P-    ).

Qty:1215.0 cft					@Rs:11948/36	% Cft		RS:145173/-

4/-	Supplying & Fixing sand under floor plugging in to wall,

Qty:2760.0 cft					@Rs:1913/21		% cft		RS:52805/-

5/-	Cement concrete stone of brick ballast 1½” to 2” gauge. Ratio (1:6:12). (S.I.No.4 (b) P.No.17)

Qty:420.0 cft					@Rs:8122/95		% cft		RS:34117/-

06/-	Providing and lying 1” thick topping cement concrete (1:2:4) i/c surface finishing and dividing into panels. (c) 2” Thick  (S.I.No.16 (c) P.No.47)

Qty:1619.0 sft					@Rs:3275/50		% sft		RS:53030/-

07/-	Cement  Plaster 1:6 up to 20’-0’ height ½” thick.

Qty:1157.0 sft					@Rs:2206/60		% sft		RS:25530/-

08/-	Cement  Plaster 1:4 up to 20’-0’ height ” thick.3/8

Qty:1157.0 sft					@Rs:2197/52		% sft		RS:25425/-

===========

PART “A(I)” PLINTH PROTECTION (BUILDING SCH:ITAM TOTAL	RS:380747/-

PART-A(II)   (U. G. Tank )BUILDINGS SCHEDULE ITEMS.

1/- 	Excavation in foundation of building bridges and other structures i/c dag balling dressing refilling around structure with excavated earth watering and ramming lead up-to 5ft. (b) in ordinary soil. (S.I.No.18 (b) P.No.5)

Qty:1056.0 cft					@Rs:3176/25		%0cft		RS:3354/-

2/- 	Cement concrete stone of brick ballast 1½” to 2” gauge. Ratio (1:5:10). (S.I.No.4 (b) P.No.17)

Qty:198.0 cft					@Rs:8694/95		% cft		RS:17216/-

3/-	Pacca Brick work in foundation /Plinth  in cement sand  mortar Ratio (1:6)(S.I No: P-    ).

Qty:220.0 cft					@Rs:11948/36	% cft		RS:26286/--

4/-	Perforated Pacca brick walling 4 ½” thick etc complete

Qty:95.00 sft					@Rs:3935/75		% sft		RS:3739/-

5/-	Fabrication of mild steel reinforcement for cement concrete i/c cutting bending laying in position making joints and fastenings i/c cost of binding wire (also includes removal of rust from bars) (a) Using tor bars. (S.I.No.7 (a) P.No.20)

Qty: 4.91cwt					@Rs:5001/70		P.cwt		RS:24558/-

Page no:4.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

6/-	R.C.C. work in roof slab beams columns rafts lintels and other structural members laid in situ or precast laid in position complete in all respects: (i) Ratio (1:2:4) 90 lbs cement 2 cft sand 4 cft shingle 1/8” to 1/4” gauge.

Qty:132.0 cft					@Rs:337/-		P.cft		RS:44484/-

07/-	Cement  Plaster 1:6 up to 20’-0’ height ½” thick.

Qty:442.0 sft					@Rs:2206/60		% sft		RS:9753/-

08/-	Cement  Plaster 1:4 up to 20’-0’ height ” thick.3/8

Qty:442.0 sft					@Rs:2197/52		% sft		RS:9713/-

09/-	Cement Concrete plain including placing compacting finishing & curing, complete (including screening and washing of stone aggregate without shuttering. Ratio 1:2:4.

(S.I. No. 5(f), P. No.16)

Qty:54.0 cft					@Rs:14429/25	% cft		RS:7792/-

===============

PART “A(II) U.G. TANK BUILDINGS SCHEDULE ITEMS TOTAL 	RS:146895/-

PART-D  NON SHEDULE ITEM

1/-	P/F UPVC pipe ½” dia PAK-ARAB) SCH-40 surface by using clip /saddler/socket/reducer/bush/tee etc .or recessed in masonry C.C or R.C.C upto 20’ ft. height and making good wih C.c i/c curing finishing etc complete as per instruction of the Engineer In charge ,.Specification of the material should meet the requirement of class 12454-Bin accordance with the ASTM-D-1784 type 1 Grade 1. rate include all costs of labor ,material, cartage scaffolding ladders .etc complete (R.A Sanction).

Qty: 200.0 rft					@Rs:			P.rft		RS:-

2/-	P/F UPVC UNION solvent ) ½’ dia (PAK ARAB) SCH-40 make of approved quality and design of various size fixed to UPVC pipe /fitting paid separately using approved adhesive compound upto 20 ft. height as per instruction of the Engineer In charge .Specification of the material should meet the requirement of class 12454-B I Grade .Rate include all costs of labor material ,cartage scaffolding /ladders etc complete.

Qty:12 Nos					@Rs:			each		RS:-

3/-	P/F UPVC pipe ¾” dia PAK-ARAB) SCH-40 surface by using clip /saddler/socket/reducer/bush/tee / elbow etc .or recessed in masonry C.C or R.C.C upto 20’ ft. height and making good wih C.c i/c curing rinishing etc complete as per instruction of the Engineer In charge ,.Specification of the material should meet the requirement of class 12454-Bin accordance with the ASTM-D-1784 type 1 Grade 1. rate include all costs of labor ,material, cartage scaffolding ladders .etc complete (R.A Sanction).

Qty: 200.0 rft					@Rs:			P.rft		RS:-

4/-	P/F UPVC UNION  ¾” dia PAK-ARAB) SCH-40  make approved quality and Design of various sizes Fixed to UPVC Pipe / Fitting paid Separately  upto 20’ ft. height as per instruction Sof the Engineer In charge Specification of the material should meet the requirement of class 12454-Bin accordance with the ASTM-D-1784 type 1 Grade 1. rate include all costs of labor ,material, cartage scaffolding ladders .etc complete (R.A Sanction).

[

Qty:10 nos					@Rs:			each		RS:-

Page no:5.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

5/-	P/F UPVC pipe 1” dia PAK-ARAB) SCH-40 surface by using clip /saddler/socket/reducer/bush/tee / elbow etc .or recessed in masonry C.C or R.C.C upto 20’ ft. height and making good wih C.c i/c curing finishing etc complete as per instruction of the Engineer In charge ,.Specification of the material should meet the requirement of class 12454-Bin accordance with the ASTM-D-1784 type 1 Grade 1. rate include all costs of labor ,material, cartage scaffolding ladders .etc complete (R.A Sanction).

Qty:150.0 rft					@Rs:			P.rft		RS:-

6/-	P/F UPVC pipe 4” dia PAK-ARAB) SCH-40 surface by using clip /saddler/socket/reducer/bush/tee /  etc . or recessed in masonry C.C or R.C.C upto 20’ ft. height and making good wih C.C i/c curing finishing etc complete as per instruction of the Engineer In charge ,.Specification of the material should meet the requirement of class 12454-Bin accordance with the ASTM-D-1784 type 1 Grade 1. rate include all costs of labor ,material, cartage scaffolding ladders .etc complete (R.A Sanction).

Qty: 200 Rft					@Rs:			P.rft		RS:-

7/-	P/F UPVC  PLUG TEE 4” dia  PAK-ARAB) SCH-40  make of approved quality and design of various sizes fixed to UPVC Pipe / fitting  paid separately using approved adhesive compound upto   20’ ft. height  as per instruction of the Engineer In charge ,.Specification of the material should meet the requirement of class 12454-Bin accordance with the ASTM-D-1784 type 1 Grade 1. rate include all costs of labor ,material, cartage scaffolding ladders .etc complete (R.A Sanction).

Qty:10.0 Nos					@Rs:			Each		RS:-

8/-	P/F UPVC BEND (LONG) 4” Of PAK ARAB make of approved quality include\ing / making joints by using pest/solution AGM make  as per instruction of the Engineer In charge ,.Rate includes all costs of labour , material, cartage, scaffolding etc complete. (R.A Sanctioned).

Qty:11 Nos					@Rs:			Each		RS:-

9/-	P/F 4” dia UPVC Socket of PAK ARAB make of approved quality including makings joints by using pest/ solution AGM make as per instruction of the Engineer In charge ,. ,.Rate includes all costs of labour , material, cartage, scaffolding etc complete. (R.A Sanctioned

Qty: 10.0 Nos					@Rs:			Each		RS:-

10/-	P/F   4” dia  UPVC Cowel / Terminal  Guard / End of  PAK ARAB make of approved quality & joints by using pest/solution AGM make  as per instruction of the Engineer In charge ,.Rate includes all costs of labour , material, cartage, scaffolding etc complete. (R.A Sanctioned).

Qty:06 Nos					@@Rs:			Each		RS:-

11/- 	P/F value full way best quality of local make gun metal with black or red painted sheet threaded i/c rubber washer and fixture with G.I pipe line etc complete. (R.A Sanctioned).

Qty:10 Nos					@Rs:			Each		RS:-

12/-	Providing & Fixing  cement concrete kurb stone having size 18”x12”x6” of city quddra / cobble  shape with pigment having strength between 5000 psi  to 8500 psi of 3” thick filling the joints with hill sand & laying in specified manner / pattern and design etc complete (R.A.A).

Qty: 1144 No’s				@Rs:			Each		RS:-

============

PART “D” NON SCHEDULE ITEMS TOTAL	RS:

Page no:6.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

PART “C “  Public Health  Schedule

1/-	Providing UPVC pressure  pipes of class “C” equivalent make fixing in trench i/c cutting, fitting and jointing with “Z” joint with one rubber ring i/c testing with water to head of 61 meter or 200ft. 4” dia. (P.H.E)  (S.I.No. 01 (b) P.No.22)

6” dia 	200 rft					@Rs:373/-		P.Rft		RS:74600/-

4” Dia  100 rft					@RS: 242/-		P.Rft		RS: 24200/-

2/-	Boring for tube well in all water bearing soils from ground level up-to 100 ft or 30.5 meter depth i/c sinking and with drawing of casing pipe. (100 mm 4”) (S.I.No. 01 (b) Page No.36).

4” Dia

Qty:50 rft					@Rs:242/-		P,rft		RS:12100/-

3/-	Construction of manhole  or inspection chamber  for the required dia  of circular sewer   and 3’-6’ (1067 mm ) depth with walls  of B.B  in cement mortar  1:3 cement plaster  1:3 ½”  thick in side 1”  (25 mm ) thick over  beching  and channel i/c  fixing C.I main hole cover with frame of clear opening  1 ½”x 1 ½”” 457x457 mm) of 1.75 cwt . (88.9 kg) embedded in plain c.c 1:2:4) & fixing (25 mm) dia M.S steps 6” 150 mm)   white projecting 4” (102 mm) from the face of walls at 12” (305 mm) c/c duly painted etc complete as per standard specification & drawing.(S.I No- P-46).

Qty:10 No					@Rs:14748/-		P.rft		RS:147480/-

4/-	Construction of  wheel valve  chamber (S.I No:9 P-53)P.H.E).

Qty:02 Nos					@Rs:4103/-		each		RS:8206/-

___________________________________

PART “C” P.H. ENGR: SCHEDULE ITEMS TOTAL 	RS: 266586/-

| PART "A) |  | ________________%Above/Below Buildings Schedule Items |  |  | Total Rs:- |  |
| --- | --- | --- | --- | --- | --- | --- |
| PART "B" |  | ________________%Above/Below W/S & S/F Schedule Items |  |  | Total Rs:- |  |
| PART "C" |  | ________________%Above/Below P.H.Engr: Schedule Items |  |  | Total Rs:- |  |
| PART "D: |  | ___________________________________Non Schedule Items |  |  | Total Rs:- |  |
|  |  |  | G. Total Rs:- |  |  |  |
|  |  |  |  | EXECUTIVE ENGINEER |  |  |
| CONTRACTOR |  |  |  | PROVINCIAL BUILDINGS DIVISION |  |  |
|  |  |  |  | SUKKUR |  |  |

Page No.5.

| S.No. | QUNAITTY. | Name of item. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |

| PART "A) | ________________%Above/Below Buildings Schedule Items | Total Rs:- |  |
| --- | --- | --- | --- |
| PART "B" | ________________%Above/Below W/S & S/F Schedule Items | Total Rs:- |  |
| PART "C" | ________________%Above/Below P.H.Engr: Schedule Items | Total Rs:- |  |
| PART "D" | ________________%Above/Below Highways Schedule Items | Total Rs:- |  |
| PART "E” | ___________________________________Non Schedule Items | Total Rs:- |  |

|  |  |  | G. Total Rs:- |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | EXECUTIVE ENGINEER |  |  |
| CONTRACTOR |  |  |  | PROVINCIAL BUILDINGS DIVISION |  |  |
|  |  |  |  | SUKKUR |  |  |