BILL OF QUANTITIES

Discription & Rate of items based on Schedule/ Market (offered Rates).

M/R TO BAR ROOM AT HIGH COURT SUKKUR.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 1/- | Supplying & fixing in position Aluminum channels framing for sliding windows & ventilators of Alcop made with 5 mm thick tinted glass glazing (Belgium) & Aluminum fly screen I/c handles stoppers & locking arrangement etc. complete Deluxe model (Bronze). (S.I.No.84 (b) P-108). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 288 | SFT | 1647/69 | P.SFT | 474535/- |

| 2/- | Providing and laying HALA or pattern tiles glazed 6" x 6" x 1/4" on floor or wall facing in required colour and pattern of STILE specification jointed in white cement and pigment over a base of 1:2 grey cement mortar 3/4" thick including washing and filling of joints with slurry of white cement and pigment in desired shape with finishing, cleaning and cost of wax polish etc. complete including cutting tiles to proper profile. (S.I.No.61 P-48). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | NET QTY: | 316 (-)85 | 231 | SFT | 47651/56 | %SFT | 109867/- |

| 3/- |  | Preparing the surface and applying rock wall/shield (Natural work texture) coating to provide durable to wall thickness. (S.I. 43 P/56) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1182 | SFT | 4504/50 |  | %SFT |  | 53243/- |  |

| 4/- | Add extra labour for rock wall / colour Crete above 20, -0 height requiring scaffolding etc: for every 10' -0 additional height). (S.I.No.44 P-55). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 298 | SFT | 1/97 | P.SFT | 587/- |

| 5/- | S/F false ceiling of Plaster of Paris, in panels including making frame, work of deodar wood including painting with Soligia paint. (S.I.No.52 P-72). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1238 | SFT | 25293/42 | %SFT | 313133/- |

| 6/- | Providing and fabricating and fixing position first class Barona teak wood paneled glazed doors windows shutter i/c thickness with suitable course sections frame i/c best quality work filling etc complete (R/A sanctioned ) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 31 | SFT | 7285/- | P.Sft | 225835/- |

| 7/- | Laying white marble flooring fine dressed on the surface without winding set in lime mortar 1:2 including rubbing and polishing of the joints. 1" thick. (S.I.No:28 (b) P-43). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1765 | SFT | 628/93 | P.SFT | 1110061/- |

Page No:2.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 8/- | Cement concrete brick or stone ballast 1-1/2” to 2” gauge Ratio(1:5:10). (S.I.No:4 (d) P-15). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 408 | CFT | 8694/95 | %CFT | 35475/- |

| 9/- | Cement concrete plain including placing compacting, finishing and curing, complete (including screening and washing at stone aggregate without shuttering ratio 1:2:4. (S.I.No.05 P-15). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 277 | SFT | 14429/25 | %SFT | 39969/- |

| 10/- | Preparing the surface and painting with matt finish i/c rubbing the surface with bathy (Silicon carbide rubbing brick) filling the voids with zink/chalk plaster of Paris mixture applying first coat matt finish & painting 3 coats with matt finish of approved made etc. complete. (S.I.No.36 A P-55). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 2130 | SFT | 3444/38 | %SFT | 73365/- |

|  | PART “A” BUILDINGS SCHEDULE ITEMS TOTAL | RS: | 2436070/- |
| --- | --- | --- | --- |

| PART "A) | ________________%Above/Below Buildings Schedule Items | Total Rs:- |  |
| --- | --- | --- | --- |

|  |  |  | G. Total Rs:- |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | EXECUTIVE ENGINEER |  |  |
| CONTRACTOR |  |  |  | PROVINCIAL BUILDINGS DIVISION |  |  |
|  |  |  |  | SUKKUR |  |  |