BILL OF QUANTITIES

Description & Rate of items based on Schedule/ Market (offered Rates).

M/R TO CHIEF ENGINEER CAMP OFFICE AT SUKKUR.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 1/- | Dismantling brick work in lime or cement mortar. (S.I.No.13 P-10). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 806 | CFT | 1285/63 | %CFT | 10632/- |

| 2/- | Dismantling cement concrete reinforced separating reinforcement from concrete cleaning and straightening the same.(S.I.No:20 P-10). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 338 | SFT | 5445/- | %SFT | 18404/- |

| 3/- | Pacca brick work in G.Floor in cement sand mortar 1:6. (S.I.No:5 P-21). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 523 | CFT | 12674/36 | %CFT | 66287/- |

| 4/- | Excavation in foundation of Building Bridges and other structures including dagbelling dressing, refilling around structure with excavated earth Watering and ramming lead upto 5 ft. In ordinary soil. (S.I.No.18 (b) P-05). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 60 | CFT | 3176/25 | %0CFT | 191/- |

| 5/- | Cement concrete brick or stone ballast 1-1/2” to 2” gauge Ratio(1:4:8). (S.I.No:4 (b) P-15). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 18 | CFT | 9416/28 | %CFT | 1695/- |

| 6/- | Fabrication of mild steel reinforcement for cement concrete i/c cutting bending, bending, laying in position, making joints and fastenings including cost of binding wire (also includes removal of rust from bars). (S.I.No.7 (ii) P/20) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5489 / 112 | 49.00 | Cwt | 5001/70 | P.CWT | 245083/- |

| 7/- | RCC work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds: lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle)a) RCC work in roof slab, beams columns rafts, lintels and other structural members laid in situ or pre cast laid in position complete in all respect. i) Ratio (1:2:4). (S.I.No:6(a)(i) P-19). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 734 | CFT | 337/- | P.CFT | 247358/- |

| 8/- | Removing cement or lime plaster. (S.I.No.53 P-14). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 4173 | SFT | 121/- | %SFT | 5049/- |

| 9/- | Applying floating coat of cement 1/32”thick. (S.I.No:14 P-52). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 4173 | SFT | 660/- | %SFT | 27541/- |

| 10/- | Cement plaster (1:6) upto 20’ height ½”thick. (S.I.No:13 (b) P-51). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 4173 | SFT | 2206/60 | %SFT | 92081/- |

| 11/- | Cement plaster (1:4) upto 20’ height 3/8”thick. (S.I.No:11 (a) P-51). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 4173 | SFT | 2197/52 | %SFT | 91703/- |

| 12/- | Supplying R.S Girder 8x4x3/8” size. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | NET QTY: | 1422 / 112 | 12.69 | Cwt | 3850/- | P.CWT | 48557/- |
| 13/- | Supplying T.Iron 2x2x1/4” size. |  |  |  |  |  |  |
|  | NET QTY: | 1237 / 112 | 11.04 | Cwt | 3575/- | P.CWT | 39468/- |

Page No:2.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 14/- | Erecting rolled steel beams or old rails in roofs etc, erection and fixing in position. (S.I.No.6 P-93). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Qty item No.12+13 | 12.69+11.04 | 23.73 | CFT | 186/34 | P.CWT | 4419/- |

|  | BUILDINGS SCHEDULE ITEMS TOTAL | RS: | 898468/- |
| --- | --- | --- | --- |

| PART "A) | ________________%Above/Below Buildings Schedule Items | Total Rs:- |  |
| --- | --- | --- | --- |

|  |  |  | G. Total Rs:- |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | EXECUTIVE ENGINEER |  |  |
| CONTRACTOR |  |  |  | PROVINCIAL BUILDINGS DIVISION |  |  |
|  |  |  |  | SUKKUR |  |  |