CONST OF LINK ROAD FROM SUI LINE PURANI ROAD TO VILLAGE MUHAMMAD SHARIF KHOSO ROAD MILE 0/4-2650’

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A)	Description and rate of Items bases on composite schedule of Rates

Part “A” Const (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site |  | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 |  | 4 | 5 | 6 |
| 01/- | 32754 Cft: | Earth work preparing sub grade cutting of filling etc complete. |  | 1678/95 | P.0% Cft: | 204992/= |
| 02/- | 16377Cft: | Preparing sub base course by supplying & spreading stone metal 1 ½”-2” gauge of approved quarry in required 3” thickness in 1 layer each 3.0 each layers with 20 cft: pit canal sand and having plasticity index of not more than 6% watering & compacting to achieve 98/100% density as per modified AASHO specifications. Rate includes all costs of materials T&P and carriage upto site of work. |  | 10094/06 | P.% Cft: | 1653104/= |
| 03/- | 8189Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. |  | 11187/22 | P.% Cft: | 916121/= |
| 04/- | 5459Rft: | Laying brick on end edging including supply of 9”x 4 ½”x3” 1st: class burnt bricks excavation for laying edging with small side parallel to the road. Rate includes all costs of materials T&P and carriage upto site of work. |  | 3324/45 | P.% Rft: | 181482/= |
| 05/- | 32754 Sft: | Providing 1st: coat of surface dressing on or new existing surface with 30 lbs: bitumen and 4.0 cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. |  | 1715/95 | P.% Sft: | 562042/= |
| 06/- | 32754 Sft: | Providing 1” thick consolidated premixed carpet in proper camber and grade i/c supply of 10 Cft: bajri 4.0 Cft: hill sand of approve quality and guage and bitumen 67 lbs: of 80/100 penetration i/c mixing the mechanical mixture in required proportion i/c heating the material and cleaning the road surface. Rate i/c all cost of material T&P and carriage upto site of work. |  | 4630/81 | P.% Sft: | 1516776/= |
| 07/- | 54597Cft: | Earth Work embankment from barrow pits i/c laying in 6” layers, clod breaking remaining dressing complete, lead Upto 100’ft and lift upto 5’ft (In ordinary Soil) |  | 2208/37 | P.%0 Cft: | 250570/= |
|  |  |  | TOTAL |  |  | 5285088/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (B) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

RE-CONST/ RECONDITIONING OF ROAD FROM INDUS HIGHWAYS INTERNAL ROAD DARI TALUKA KANDHKOT ROAD MILE 0/0-0/1+330’

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A)		Description and rate of Items bases on composite schedule of Rates

Part “A”  (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site |  | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 |  | 4 | 5 | 6 |
| 01/- | 34333Cft: | Providing sand cushion i/c supplying & spreading pit/canal sand of approved quality from approved source of supply where ever available site of work i/c carriage site of work (R.A.A). |  | 1479/41 | P.% Cft: | 507930/= |
| 02/- | 8583Cft: | Preparing sub base course by supplying & spreading stone metal 1 ½”-2” gauge of approved quarry in required 3” thickness in 1 layer each 3.0 each layers with 20 cft: pit canal sand and having plasticity index of not more than 6% watering & compacting to achieve 98/100% density as per modified AASHO specifications. Rate includes all costs of materials T&P and carriage upto site of work. |  | 8084/81 | P.% Cft: | 693945/= |
| 03/- | 8583Cft: | Preparing sub base course by supplying & spreading stone metal 1 ½”-2” gauge of approved quarry in required 3” thickness in 1 layer each 3.0 each layers with 20 cft: pit canal sand and having plasticity index of not more than 6% watering & compacting to achieve 98/100% density as per modified AASHO specifications. Rate includes all costs of materials T&P and carriage upto site of work. |  | 8216/57 | P.% Cft: | 705255/= |
| 04/- | 8583Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. | 9244/07 |  | P.% Cft: | 793449/= |
| 05/- | 34333 Sft: | Providing 1st: coat of surface dressing on or new existing surface with 30 lbs: bitumen and 4.0 cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. | 1662/56 |  | P.% Sft: | 570812/= |
| 06/- | 34333 Sft: | Providing 1” thick consolidated premixed carpet in proper camber and grade i/c supply of 10 Cft: bajri 4.0 Cft: hill sand of approve quality and guage and bitumen 67 lbs: of 80/100 penetration i/c mixing the mechanical mixture in required proportion i/c heating the material and cleaning the road surface. Rate i/c all cost of material T&P and carriage upto site of work. | 4540/65 |  | P.% Sft: | 1558955/= |
|  |  |  | TOTAL |  |  | 4830345/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONST OF RETAINING WALL SODHO CHANNA U/C DARI TALUKA KANDHKOT

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

- Description and rate of Items bases on composite schedule of Rates.

Part  “A” 623 RFT(RETAINING WALL).

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 2000Cft: | Excavation in foundation of building bridge other structure i/cs dag belling dressing refilling around the structure with excavated earth watering and remaining lead upto one chain and left upto 5.0’ ordinary soil. | 3176/25 | P.%0 Cft: | 6352/= |
| 02/- | 500Cft: | Cement concrete with bricks are stone plaster 1½” to 2” guage ratio 1:2:4 | 11288/75 | P.% Cft: | 47081/= |
| 03/- | 3348Cft: | Pucca Brick work in foundation and plinth i/c cement sand mortor | 12501/41 | P.% Cft: | 418672/= |
| 04/- | 28Cft: | Cement concrete plain i/c placing compacting finishing curing complete i/c screening washing of stone aggregate with out suhttering ratio 1:2:4: | 14429/25 | P.% Cft: | 4040/= |
| 05/- | 50Sft: | Erection and removal of centering for R.C.C or plain cement work (2nd: class) for partal wood vertical (S.I.No:18)(b) P-20). | 7000/00 | P.% Sft: | 3500/= |
| 06/- | 700Sft: | Cement Pointing Struck jointing Wall ratio 1:3 | 2283/93 | P.% Sft: | 15988/= |
| 07/- | 15400Cft: | Filling watering and remaining earth under floor with new earth excavated from side lead upto 100’ft and lift upto 5’ft. | 3630/00 | P.0% Cft: | 20328/= |
|  |  |  | TOTAL |  | 508468/= |

Amount Total

__________________________% Above / Below on the rates of CSR. 	Total  =

Amount to be added /deducted on the

basis of premium quoted Total

Total in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF APPROACH ROAD FROM BRIDGE OVER MURAD WAH @ RD-49 ROAD MILE 0/0-0/3 TALUKA KASHMORE

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

- Description and rate of Items bases on composite schedule of Rates

Part “A” (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | With Lead 202252Cft: Without Lead 219404Cft: | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 85% density as per modified AASHO specifications. | 6726/23 3656/23 | P.0% Cft: P.0% Cft | 1360393/= 802191/= |
| 02/- | 24473Cft: | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 95-100% density as per modified AASHO specifications. | 2529/77 | P.% Cft: | 61911/= |
| 03/- | 12236Cft: | Providing sand cushion i/c supplying & spreading pit/canal sand of approved quality from approved source of supply where ever available site of work i/c carriage site of work (R.A.A). | 1479/41 | P.% Cft: | 181021/= |
| 04/- | 6118Cft: | Preparing sub base course by supplying & spreading stone metal 1 ½”-2” gauge of approved quarry in required 3” thickness in 1 layer each 3.0 each layers with 20 cft: pit canal sand and having plasticity index of not more than 6% watering & compacting to achieve 98/100% density as per modified AASHO specifications. Rate includes all costs of materials T&P and carriage upto site of work. | 9197/68 | P.% Cft: | 562714/= |
| 04/- | 6118Cft: | Providing Sub Base course by supplying and spreading stone metal 1 ½” gauge of approved quality from approved quarry in required thickness to proper camber and grade i/c hand packing mixing with 10’ screening & non plastic quarry of approved quality & gauge from approved source i/c watering and compacting to achieve 98-100% density as per modified AASHO specification ( rate i/cs all cost of material T&P and carriage upto site of wok.) | 9413/18 | P.% Cft: | 575898/= |
| 05/- | 4079Rft: | Laying brick on end edging including supply of 9”x 4 ½”x3” 1st: class burnt bricks excavation for laying edging with small side parallel to the road. Rate includes all costs of materials T&P and carriage upto site of work. | 2875/83 | P.% Rft: | 117305/= |

Contd: P/2

P-2

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 06/- | 6118Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. | 10482/52 | P.% Cft: | 641320/= |
| 07/- | 24473Sft: | Providing 1st: coat of surface dressing on or new existing surface with 30 lbs: bitumen and 4.0 cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. | 1699/79 | P.% Sft: | 415990/= |
| 08/- | 24473Sft: | Providing 1” thick consolidated premixed carpet in proper camber and grade i/c supply of 10 Cft: bajri 4.0 Cft: hill sand of approve quality and guage and bitumen 67 lbs: of 80/100 penetration i/c mixing the mechanical mixture in required proportion i/c heating the material and cleaning the road surface. Rate i/c all cost of material T&P and carriage upto site of work. | 4594/98 | P.% Sft: | 1124529/= |
|  |  |  | TOTAL |  | 5843272/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF APPROACH ROAD FROM BRIDGE OVER MURAD WAH @ RD-49 ROAD MILE 0/0-0/3 TALUKA KASHMORE

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-B  (3.0 FT: SPAN CULVERTS).

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 1106 Cft: | Excavation in foundation of building bridge other structure i/cs dag belling dressing refilling around the structure with excavated earth watering and remaining lead upto one chain and left upto 5.0’ ordinary soil. | 3176/25 | P.%0 Cft: | 3513/= |
| 02/- | 238 Cft: | Cement concrete with bricks are stone plaster 1½” to 2” guage ratio 1:2:4 | 9416/28 | P.% Cft: | 22411/= |
| 03/- | 502 Cft: | Pucca Brick work in foundation and plinth i/c cement sand mortor | 12501/41 | P.% Cft: | 62757/= |
| 04/- | 55 Cft: | Cement concrete plain i/c placing compacting finishing curing complete i/c screening washing of stone aggregate with out suhttering ratio 1:2:4: | 14429/25 | P.% Cft: | 7936/= |
| 05/- | 58 Sft: | Erection and removal of centering for R.C.C or plain cement work (2nd: class) for partal wood vertical | 7000/00 | P.% Sft: | 4060/= |
| 06/- | A. Tor Steel 5.90 Cwt B. M/ Steel 0.720 Cwt | Fabrication of m/s reinforcement cement concrete including cutting binding laying in position making joints & fastening of i/c binding wire also removal rust from bars. | 5001/70 4820/20 | P.Cwt: P.Cwt | 20807/= 4372/= |
| 07/- | 114 Rft: | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kind of forms moulds lifting shuttering curing rend. | 337/00 | P.Rft: | 38418/= |
| 08/- | 330 Sft: | Cement Plaster (1:3) upto 20’ height 1½” thick. | 2283/93 | P.% Sft: | 7537/= |
|  |  |  | TOTAL |  | 183828/= |

Amount Total (b)

__________________________% Above / Below on the rates of CSR. 	Total (b) =

Amount to be added /deducted on the

basis of premium quoted Total (b)

Total (b) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

CONSTRUCTION OF 30’RFT BRIDGE OVER KANDHKOT SHAKH NEAR VILLAGE GHULAM MUSTAFA BANGWAR U/C AKHERO

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-A  (BRIDGE WORK).

| Item No. | Qty’s |  | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 |  | 3 | 4 | 5 | 6 |
| 01/- | 9164 Cft: |  | Excavation in foundation of building bridge other structure i/cs dag belling dressing refilling around the structure with excavated earth watering and remaining lead upto one chain and left upto 5.0’ ordinary soil. | 3176/25 | P.%0 Cft: | 29107/= |
| 02/- | 2582Cft: |  | Cement concrete with bricks are stone plaster 1½” to 2” guage ratio 1:2:4 | 9416/28 | P.% Cft: | 243128/= |
| 03/- | 6190Cft: |  | Pucca Brick work in foundation and plinth i/c cement sand mortor | 12501/41 | P.% Cft: | 773837/= |
| 04/- | 714Cft: |  | Cement concrete plain i/c placing compacting finishing curing complete i/c screening washing of stone aggregate with out suhttering ratio 1:2:4: | 14429/25 | P.% Cft: | 103025/= |
| 05/- | 195Sft: |  | Erection and removal of centering for R.C.C or plain cement work (2nd: class) for partal wood vertical | 3127/41 | P.% Sft: | 6098/= |
| 06/- | 127.81Cwt |  | Fabrication of m/s reinforcement cement concrete including cutting binding laying in position making joints & fastening of i/c binding wire also removal rust from bars. | 4820/20 | P.Cwt | 616070/= |
| 07/- | 2105Rft: |  | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kind of forms moulds lifting shuttering curing rend. | 337/00 | P.Rft: | 709385/= |
| 08/- | 3220Sft: |  | Cement Plaster (1:3) upto 20’ height 1½” thick. | 2283/93 | P.% Sft: | 73543/= |
| 09/- |  | 25600 Sft | Earth Work embankment from barrow pits i/c layers 6” clod breaking remaining etc | 6278/37 | P.%0 Cft: | 160726/= |
|  |  |  |  | TOTAL |  | 2714919/= |

Amount Total (a)

__________________________% Above / Below on the rates of CSR. 	Total (a) =

Amount to be added /deducted on the

basis of premium quoted Total (a)

Total (a) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*