| M&R to Repair / Renovation of M& R To Facilitation Centre Mirpurkhas. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Schedule "B". |  |  |  |  |  |  |  |  |  |
| Sr. No: | Items of Work. |  | Qty: |  |  | Rate. |  | Unit. | Amount. |
|  | Part (A) Buildings Schedule Items. |  |  |  |  |  |  |  |  |
| 1 | Dismantling CC Plain 1:2:4 (S.I. No.19, P-10) |  | 513 |  | CFT | 3327.50 |  | % CFT | 17070 |
| 2 | Removing Cement or Lime Plaster (S.I. No.53/P-13) |  | 7492 |  | SFT | 121.00 |  | % SFT | 9065 |
| 3 | Scarping ordinary distemper oil bound distemper or paint on walls S.I.No.54 P-13) |  | 2475 |  | SFT | 226.88 |  | %SFT | 5615 |
| 4 | Pacca Brick work in Ground Floor in Cement sand mortar 1:6:S.I.5, P-21) |  | 1185 |  | CFT | 12674.36 |  | %CFT | 150191 |
| 5 | First class deodar wood wrought, Joinery in doors and windows etc. fixed in position including chowkhats hold fasts, hinges, iron tower bolts, chocks, chocks, cleats, handles and cords with hook etc, Deodar panelled or paneled and glazed or fully glazed (b) 1-3/4” think (S.I No.7 P-58) |  | 44.00 |  | SFT | 902.93 |  | P.Sft | 39729 |
| 6 | Cement Plaster 3/8” think Ratio 1:4 upto 20’ height S.I. No.11 P-52) |  | 7492 |  | SFT | 2197.52 |  | %SFT | 164638 |
| 7 | C.C. Plain 1:3:6 I/C placing compacting finishing and curing complete (I/C screening and washing of stone aggregate) w/o shuttering (S.I.#5 P-16 |  | 348 CFT |  |  | 12595.00 |  | %CFT | 43831 |
| 8 | P/L C.C topping (1:2:4) I/C surface finishing and dividing into penal |  | 1557 SFT |  |  | 4411.82 |  | %SFT | 69574 |
| 9 | 1/8” Gobri leeping on roofs or floors (S.I.12 P-35 |  | 2148.00 SFT |  |  | 91.74 |  | %SFT | 1971 |
| 10 | Replacing Keller eaten Bricks |  | 200 No. |  |  | 36.63 |  | Each | 7326 |
| 11 | Facing water spouts or parnala |  | 8 No. |  |  | 211.75 |  | Each | 1694 |
| 12 | Carriage of 100 Cft/5 tons of all material like stone aggregate spowl, coal lime surkhi etc BG Rail fastenings points and crossing bridges Girders, pipes sheets rail MS Bar etc. or 1000 No. Bricks 10”x5”x3” No. tiles 12”x6x2” or 150 CFT of timber or 100 mounds of fuel wood by trucks or any other means owners owned by the contractor (SI#1/P-1 Schedule of carriage) Lead4 Mile) |  | 513.00 CFT |  |  | 649.86 |  | %CFT | 3334 |
| 13 | Primary coats of chalks under distemper |  | 2047.00SFT |  |  | 442.75 |  | %SFT | 9063 |
| 14 | Distempering three coats on walls |  | 2047.00 SFT |  |  | 1079.65 |  | %SFT | 22100 |
| 15 | Preparing the surface and painting with finish i/c rubbing the surface with bathy (silicon carbide brick) filling the voids with Zink/chalk / plaster of Paris mixture, applying first coat premix making the surface smooth and then painting 3 coats with matt finish of approved make etc complete (new surface) 2nd & subsequent coat (S.I No.36 (b), p-55) New Surface |  | 5430 SFT |  |  | 3444.38 |  | %SFT | 187030 |
| 16 | Painting New Surface painting doors and windows any type 03-coats (S.I No.36 (b) (c-ii) P-68) New Surface |  | 96 SFT |  |  | 2116.41 |  | %SFT | 2032 |
| 18 | Painting old surface painting doors and windows any type 03-coats (S.I No.36 (b) (c-ii) P-68) Old Surface |  | 104 SFT |  |  | 1652.21 |  | %SFT | 1729 |
|  |  |  |  |  |  |  |  |  | 735992 |
| (B) | Part B Civil Work (Schedule Items) |  |  |  |  |  |  |  |  |
| 1 | P/Fixing G.I Framers/chowkhats of size 7”x2” or 4 ½” x 3” for Doors using 20 Gauge G/I Sheet I/C Welded hinges and fixing at site with necessary hold fasts, filling with cement sand slurry or ratio 1:6 repairing the jumps. The cost also i/c all carriage tools and plants used in making and fixing |  | 35 RFT |  |  | 228.90 |  | P/RFT | 8012 |
| 2 | P/Fixing G.I Framers/chowkhats of size 7”x2” or 4 ½” x 3” for Doors using 20 Gauge G/I Sheet I/C Welded hinges and fixing at site with necessary hold fasts, filling with cement sand slurry or ratio 1:6 repairing the jumps. The cost also i/c all carriage tools and plants used in making and fixing |  | 9 RFT |  |  | 240.50 |  | P/RFT | 2165 |
| 3 | P/Fixing in positiong iron/ steel grill of ¾” x 1x4” size flat iron approved design including painting three coats etc complete |  | 40 SFT |  |  | 180.50 |  | P.SFT | 7220 |
|  |  |  |  |  |  |  |  | TOTAL | 17397 |
| (C) Part –C (Civil Work) Schedule Items) |  |  |  |  |  |  |  |  |  |
| 1 | Lying floor of approved white glazed tiles ¼” thick in white cement 1.2 over ¾” Thick cement mortar 1.2 complete (S.I.No.24, P42) |  | 157SFT |  |  | 27678.86 |  | %SFT | 43456 |
| 2 | White glazed tiles ¼” thick dado joined in white cement and laid over 1:2 cement sand motar ¾” thick i/c finishing |  | 523 SFT |  |  | 28253.61 |  | %SFT | 147766 |
| 3 | Supplying and fixing in position aluminum channels farming for sliding windows and ventilators of alop mode with 5 mm think tined glass glazing (Belgium) & Aluminum fly screen i/c handles stoppers and locking arrangements etc. complete (b) Deluxe model (Bronze) |  | 40 SFT |  |  | 1647.69 |  | P/SFT | 65908 |
| 4 | Providing and fixing cerement paving blocks flooring having of 197 x 97 x90 mm of city/qudra/ cabble shape with natural colours having b/w 5000 psi to 8500 psi i/c filling the joints with hill sand and laying in specified manner/pattern and design etc complete |  | 3671 SFT |  |  | 199.70 |  | P/SFT | 733099 |
|  |  |  |  |  |  |  |  | TOTAL | 990229 |
| (D) Civil work non schedule work |  |  |  |  |  |  |  |  |  |
| 1 | Cement plaster ½” think in cement sand mortar ratio 1:4 i/c floating coat of cement with SBR Chemical upto 20’ height with smooth finish i/c curing etc complete as directed by the Engineer Incharge (Rates includes all cost of Material, Labour & Cartage (R.A) |  | 7492 SFT |  |  | 4768.72 |  | %SFT | 357273 |
| 2 | Providing and laying Master granite tiles fully glaze jointed in white cement and lid over 1:2 grey cement mortar ¾” thick in/c finishing and filling of joint with slurry of white cement or tile grout in desired shape in/c cutting tiles to proper profile. The rate includes all cost of labour, material and cartage as directed by Engineer incharge (on floor or facing ) size 24” x room floor |  | 2198 SFT |  |  | 462.00 |  | P.SFT | 1015476 |
| 3 | p/f gypsum board flase ceiling sheet size 2’x2 (DFB Brand or equivalent) in panels or required designed including mortal suspension system frame work of aluminum T/L Section hanged with nails wire to ceiling etc. complete including cartage at site wastage and necessary holes for lighting arrangements and inspection panel as directed by Engineer Room Ceiling |  | 1897 SFT |  |  | 167.60 |  | P/SFT | 317937 |
| 4 | Providing and laying Guttca brick facing 2-1/4” 9 2-1/4” size of approved designed set in cement sand mortar 1:3: 1” thick i/c filling the joint with white cement slurry using colour pigment for matching i/c leveling smooth finishing currying and scaffolding etc. complete as per specification and the rate includes all cost of labour material and cartage as directed by |  | 2475 SFT |  |  | 373.00 |  | P/SFT | 923175 |
| 5 | Making and fixing lamination sheet (Imported) wall paneling having wooden texture in framing of G.I. U.Channel Patti 1-1/2” c/c/ making with G.I. U.Clips etc. complete in all respects as directed by engineer incharge |  | 870 SFT |  |  | 457.00 |  | P.SFT | 397590 |
| 6 | Supplying /f glass door with 12 mm glass complete in all respect as directed by Engineer Incharge |  | 35 SFT |  |  | 675.00 |  | P.SFT | 23625 |
| 7 | P/f shade made of fiber glass sheet rest over of size 1-1/2” x 1-1-/2” M.S. angle iron erected over vertical pipe posts of M.S Pipe 2-1/2” dia (Clear Height 10’-2 & 2-0” buried in the ground @ 10’-0 c/c) this rate i/e all type of work such as excavating the trench up to required depth erecting M.W Pipe posts and making good in C.C. 1:2:4 cutting and fixing i/x painting one coat of red oxide and two coats of enamel paint over M.S Angle Iron and vertical pipe posts as directed by the Engineer incharge (R.A) |  | 288 SFT |  |  | 477.33 |  | P.SFT | 137471 |
|  |  |  |  |  |  |  |  | TOTAL | 3172547 |
| N O T E :- |  |  |  |  |  |  |  |  |  |
| 1) | Only hill sand will be used. |  |  |  |  |  |  |  |  |
| 2) | Premium on non schedule item will not be allowed. |  |  |  |  |  |  |  |  |
| 3) | Cartage of any item either supplied y the Department if the contractor will be not allowed. |  |  |  |  |  |  |  |  |
| 4) | Water arrangement will be made by the contractor at his own risk and cost. |  |  |  |  |  |  |  |  |
| 5) | Income tax at 7.50% will be deducted from each bill of the contractor as per out standing orders. |  |  |  |  |  |  |  |  |
| 6) | The Material used in the work or finished project can be got tested from any approved lab: as the direction by Engineer In charge or his representative and all the expenses in this connection with such testing shall be borne by the contractor executively without any reimbursement claim again his account. |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Deputy Inspector General of Police Mirpurkhas Range |  |  |  |
| Contractor |  |  |  |  |  |  |  |  |  |