FACE SHEET

FUNDS HEAD:-

MAJOR HEAD:-

MINOR HEAD:-	ESTIMATE FOR REPAIR / MAINTENANCE  OF GOVERNMENT HIGH SCHOOL SYED HADAL SHAH  TALUKA  KINGRI  DISTRICT KHAIRPUR.

This Estimate Has Been Framed In The Office Of The Executive Engineer (Education) Works & Services Department Khairpur For Probable Expenditure That Will Be Incurred On The Said Works.

Total Amount Rs.	  0.530   (million)

ESTIMATE FOR REPAIR / MAINTENANCE  OF GOVT: HIGH SCHOOL               S-HADAL SHAH  TAL:  KINGRI  DISTT: KHAIRPUR.

| Sr. No. | ITEM OF WORKS |  | No: L: B: D: | Qty: | Rate | Unit |  | Amount |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01. | Dismantling cement concrete plain 1:2:4 (S.I.No.19 b P.No.10). |  |  |  |  |  |  |  |  |
|  | Flooring: |  |  |  |  |  |  |  |  |
|  | Class Room (vi – X) |  | 1x5x 21.83 x 17.75 x 0.17= | 329.00 |  |  |  |  |  |
|  | Veranda |  | 1x 113.00 x 6.83 x 0.17= | 131.00 |  |  |  |  |  |
|  |  |  |  | 460.00 | 1306/80- | %Cft |  | Rs.6011/- |  |
| 02. | Removing cement or lime plaster from walls (S.I.No.53 P.No.13). |  |  |  |  |  |  |  |  |
|  | Main Building |  |  |  |  |  |  |  |  |
|  | Veranda I/S |  | 1x2(113.0+6.83)x5.0 = | 1198.0 |  |  |  |  |  |
|  | C/R (vi-X) |  | 5x2(21.83+17.75)x5.0 = | 1979.00 |  |  |  |  |  |
|  | F/S |  | 1x113.0x5.00 = | 565.00 |  |  |  |  |  |
|  |  |  |  | 3742.00 | 121/50 | %Sft |  | Rs.4547/- |  |
| 03. | Applying floating coat of cement (S.N 14 P.No 22) |  |  |  |  |  |  |  |  |
|  | Veranda I/S |  | 1x2(113.0+6.83)x5.0 = | 1198.0 |  |  |  |  |  |
|  | C/R (vi-X) |  | 5x2(21.83+17.75)x5.0 = | 1979.00 |  |  |  |  |  |
|  | F/S |  | 1x113.0x5.0 = | 565.00 |  |  |  |  |  |
|  |  |  |  | 3742.00 | 660/- | %Sft |  | Rs.24697/- |  |
| 04. | Cement plaster 1:6 upto 20 height 1/2 thick. (S.I.No. (a) P.No.52). |  |  |  |  |  |  |  |  |
|  | Veranda I/S |  | 1x2(113.0+6.83)x5.0 = | 1198.0 |  |  |  |  |  |
|  | C/R (vi-X) |  | 5x2(21.83+17.75)x5.0 = | 1979.00 |  |  |  |  |  |
|  | F/S |  | 1x113.0x5.0 = | 565.00 |  |  |  |  |  |
|  |  |  |  | 3742.00 |  |  |  |  |  |
|  | Deduction |  |  |  |  |  |  |  |  |
|  | C/R Doors |  | 5x(4.0x5.0) = | 100.00 |  |  |  |  |  |
|  | C/R Wind: |  | 5x(5.0x2.0) = | 50.00 |  |  |  |  |  |
|  | Verand OPP:Grills |  | 11x(5.5x2.0) = | 121.00 |  |  |  |  |  |
|  | Veranda Door |  | 1x(5.0x5.0) | 25.00 |  |  |  |  |  |
|  |  |  |  | 296.00 |  |  |  |  |  |
|  |  |  | NET 3742.0 – 296 =3446 | 3446.00 | 2206/60 | %Sft |  | Rs.76039/- |  |
| 05. | Cement plaster 1:4 upto 20 height 3/8 thick. (S.I.No.11 (a) P.No.52). |  |  |  |  |  |  |  |  |
|  |  |  | Same Qty: Item No.05= | 5342.00 |  |  |  |  |  |
|  |  |  |  | 3446.00 | 2197/52 | %Sft |  | Rs.75726/- |  |
| 06. |  | Providing and laying 2” thick topping cement concrete 1:2:4 including surface finishing and dividing into panels (S.I.NO.16 P-47) |  |  |  |  |  |  |  |
|  |  | Class Room (vi – X) | 1x5x 21.83 x 17.75 = | 1937.00 |  |  |  |  |  |
|  |  | Veranda | 1x 113.0 x 7.0 = | 791.00 |  |  |  |  |  |
|  |  |  |  | 2728.00 | 3275/50- |  | %Sft | Rs.89355/- |  |

| Sr. No. |  | ITEM OF WORKS |  | No: L: B: D: | Qty: |  | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 07. | White floor of approved white with glazed Tile 1:4thick in white cement 1:2 3/4thick cement mortar 1; 2.9S.I.NO. P-48) |  |  |  |  |  |  |  |  |
|  |  |  | 2x4.0x4.0= | 32.00 |  |  |  |  |  |
|  |  |  |  | 32.00 | 27747/04 | %Sft |  | Rs.8879/- |  |
| 08. |  | White glazed tile ¼ thick dado in white cement and laid over cement sand mortar ¾ thick including finishing. (S.I.NO. P-50). |  |  |  |  |  |  |  |
|  |  | Wash Room | 2x2(4.0+4.0)x4.5 = | 128.00 |  |  |  |  |  |
|  |  |  |  | 128.00 | 28299/30- |  | %Sft | Rs.36223/- |  |
| 09. |  | First class deodar wood wrought, joinery in doors & windows etc. fixed in position i/c chowkhats holds fasts hinges, iron tower blts. Chocks cleats, handles and cords with hooks, etc. Deodar paneled or paneled and glazed or fully glazed (with out chowkhats). (S.I.No.7 P.No.65). |  |  |  |  |  |  |  |
|  |  | Door | 1x3.5x6.0 | 21.00 |  |  |  |  |  |
|  |  |  |  | 21.00 | 902/93 |  | P.Sft | Rs.18961/- |  |
| 10. | White washing in three coats.(S.I.NO. P- ) |  |  |  |  |  |  |  |  |
|  |  |  | Same Qty Item No.06 | 2708.00 |  |  |  |  |  |
|  |  |  |  | 2708.00 | 425/84 | %Sft |  | Rs.11531/- |  |
| 11. | Distemper in two coats.,(S.I.NO. P- ) |  |  |  |  |  |  |  |  |
|  | Veranda I/S |  | 1x2(113.0+6.83)x12.5 = | 2996.0 |  |  |  |  |  |
|  | C/R (vi-X) |  | 5x2(21.83+17.75)x12.5 = | 4947.00 |  |  |  |  |  |
|  | F/S |  | 1x113.0x12.5 = | 1412.00 |  |  |  |  |  |
|  |  |  |  | 9355.00 |  |  |  |  |  |
|  | Deduction |  |  |  |  |  |  |  |  |
|  | C/R Doors |  | 5x(4.0x7.0) = | 140.00 |  |  |  |  |  |
|  | C/R Wind: |  | 5x(5.0x4.0) = | 100.00 |  |  |  |  |  |
|  | Verand OPP:Grills |  | 11x(5.5x4.0) = | 242.00 |  |  |  |  |  |
|  | Veranda Door |  | 1x(5.0x7.0) | 35.00 |  |  |  |  |  |
|  |  |  |  | 517.00 |  |  |  |  |  |
|  |  |  | NET 9355.0-517.0=8838.0 | 8838.00 | 1043/90 | %Sft |  | Rs.92260/- |  |
| 12. | Colour washing in two coats.(S.I.NO. P- ) |  |  |  |  |  |  |  |  |
|  | B/S of Main Building= 1x113.0x12.5 |  |  | 1412.0 |  |  |  |  |  |
|  | Short Sides of = = = 2x27.58x 12.5 |  |  | 689.0 |  |  |  |  |  |
|  |  |  |  | 2101.00 | 859/60 | %Sft |  | Rs.18066/- |  |

| Sr. No. |  | ITEM OF WORKS |  | No: L: B: D: | Qty: |  | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 13. | Painting OLD surface painting of sashes fan light glazed or guazed door & windows etc (2-Coats) (S.I.NO: 4/P-67) |  |  |  |  |  |  |  |  |
|  | Main Gate |  | 1x2x5.0x7.0 | 70.0 |  |  |  |  |  |
|  | Room Doors |  | 5x2x3.50x6.75 | 118.0 |  |  |  |  |  |
|  | Room Wind: |  | 10x2x5.0x4.0 | 400.0 |  |  |  |  |  |
|  | Veranda OPP Grills |  | 11x2x5.5x4.0 | 484.0 |  |  |  |  |  |
|  | Veranda Ventilators |  | 11x2x4.0x2.0 | 176.0 |  |  |  |  |  |
|  |  |  |  | 1248.00 | 674/19 | %Sft |  | Rs.8414/- |  |
|  |  |  |  | TOTAL |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Rs. 4,70,709/- |  |
|  |  |  |  | Add Cartridge |  |  |  | Rs. 60,392/- |  |
|  |  |  |  | G. TOTAL |  |  |  | Rs. 5,31,101/- |  |
|  |  |  |  | SAY |  |  |  | Rs. 5,30,000/- |  |

ESTIMATE FOR REPAIR / MAINTENANCE  OF GOVT: HIGH SCHOOL               S-HADAL SHAH  TAL:  KINGRI  DISTT: KHAIRPUR.

MATERIAL STATEMENT

| S# | NAME OF ITEM | QTY: | CEMENT | H/SAND | BAJRI | STONE | BRICKS | STEEL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Applying Floating | 3742 sft | 15 |  | - | - | - | - |
|  | Cement Plaster 1:6 | 3446 = | 19 | 138 | - | - | - | - |
|  | Cement Plaster 1:4 | 3446 = | 18 | 117 | - | - | - | - |
|  | C-C Topping 2” thick Ratio 1:2:4 | 2728 sft | 81 | 201 | 402 | - | - | - |
|  | Glazed Tile | 160 sft | 12 | 25 |  | - | - | - |
|  |  | Total | 145 | 481 | 402 | - | - | - |
|  |  | Rate | 144/33 | 7135/- | 1227/50 |  |  |  |
|  |  | Unit | P.beg | % cft | % Cft | % CFT | % cft | P.Ton |
|  |  | Amount | 20927/- | 34531/- | 4934/- | - | - | - |

Total	RS.60,392/-