NAME OF WORK:-	RENOVATION / REHABLITATION OF GOVERNMENT GIRLS HIGH SCHOOL NIZAMANI DISTRICT KHAIRPUR.

| Sr | Item of work | NO:L.B.D | QTY | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- | --- |

Main Building (Admin Block/Class Room)

01/-	Excavation in foundation of building bridges and other structures i/c dag balling dressing refilling around structure with excavated earth watering and ramming lead up-to 5ft. (b) in ordinary soil. (S.I.No.18 (b) P.No.4)

Bath			1x8.75x2.0x2.0		=	35	cft

I/S			2x6.75x2.0x2.0		=	54	cft

=	89	cft

Qty: 89 Cft		@RS: 3176.25		% 0Cft		RS: 283/-

02/-	Cement concrete stone of brick ballast 1½” to 2” gauge. Ratio (1:4:8). (S.I.No.4 (b) P.No.15)

Bath			1x8.75x2.0x0.50		=	09	cft

I/S			2x6.75x2.0x0.50		=	14	cft

Under floor		1x8.0x6.0x0.50		=	24	cft

=	47	cft

Qty: 47 Cft		@ RS: 9416.28		% Cft		RS: 4426/-

03/-	Pacca  Brick  work in  foundation & plinth  work  in cement sand mortar (1:6) (S.I No:5 P-25).

1st Step		1x8.75x1.50x1.0		=	13	cft

I/O			2x6.75x1.50x1.0		=	20	cft

2nd Step		1x8.75x1.13x2.0		=	20	cft

2x6.75x1.13x2.0		=	31	cft

=	84	cft

Qty: 84 Cft		@RS: 11948.36	% Cft		RS: 10037/-

04/-	R.C.C. work in roof slab beams columns rafts lintels and other structural members laid in situ or precast laid in position complete in all respects: (i) Ratio (1:2:4) 90 lbs cement 2 cft sand 4 cft shingle 1/8” to 1/4” gauge. (S.I.No.6 (a-i) P.No. 17)

Plinth Beam		1x8.75x0.75x0.75		=	05	cft

“			2x6.75x0.75x0.75		=	08	cft

Lintel Beam		1x5.0x0.75x0.50		=	19	cft

Roof slab		1x11.75x10.50x0.42		=	52	cft

=	84	cft

Qty: 84 Cft		@ RS: 337/-		P.Cft		RS: 28308/-

05/-	Fabrication of mild steel reinforcement for cement concrete i/c cutting bending laying in position making joints and fastenings i/c cost of binding wire (also includes removal of rust from bars) (a) Using tor bars. (S.I.No.7 (a) P.No.20)

Same Qty: of item NO: 5	84x5/112		=	3.75	cwt

=	3.75	cwt

Qty: 3.75 cwt		@RS: 5001.70		P.cwt		RS: 18756/-

| Sr | Item of work | NO:L.B.D | QTY | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- | --- |

06/-	Pacca Brick work in ground floor i:6 in cement sand mortar etc complete (S.I NO:    P.NO:     ).

Bath			1x8.75x0.75x9.50		=	62	cft

S/W			2x6.0x0.75x9.50		=	86	cft

=	148	cft

Deduction Door	1x2.50x0.75x7.0		=   (-)	13	cft

=	135	cft

Qty: 135 Cft		@RS: 12674.36	% Cft		RS: 17110/-

07/-	Supplying & Fixing sand under floor plugging in to wall. (S.I NO:     P.No:       ).

1x8.0x6.0x2.0			=	96	cft

=	96	cft

Qty: 96 Cft		@RS: 1141.25		% Cft		RS: 1096/-

08/-	Dismantling Cement Concrete plain (1:2:4) (S.I NO: 19 (c)P-10).

Class Room		2x22.0x18.x00.17		=	135	cft

=	135	cft

Qty: 135 Cft		@RS: 3327.50		% Cft		RS: 4492/-

09/-	Removing Cement or lime plaster (S.I NO: 53 P-13).

C/Room		1x2(22.0+18.0)x3.0		=	240	sft

Ver:			2x(100+19.0)x2.50		=	595	sft

O/S			1x(100+19.0)x4.0		=	476	sft

=	1311	sft

Qty: 1311 Sft		@RS: 121/-		% Sft		RS: 1586/-

10/-	Apllying floating coat of cement 1/32” thick. (S.I NO:   P.NO:   ).

Same Qty: of item NO: 9				=	1311	sft

=	1311	sft

Qty: 1311 Sft		@RS: 660/-		% Sft		RS: 8653/-

11/-	Cement Plaster 1:6 up to 20’-0 height ½” thick. ( S.I NO: 13(b)    P:NO:52   )

Same Qty: of item NO: 10				=	1311	sft

Bath			1x2(8.0+6.0)x9.50		=	266	sft

=	1577	sft

Qty: 1577 Sft		@RS: 2206.60		% Sft		RS: 34798/-

| Sr | Item of work | NO:L.B.D | QTY | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- | --- |

12/-	Cement Plaster 1:4 up to 20”-0 height 3/8” thick. . ( S.I NO: 11(a) P:NO:52   )

Same Qty: of item NO: 11				=	1577	sft

=	1577	sft

Qty: 1577Sft		@RS:2197.52		% Sft		RS: 34655/-

13/-	Cement concrete stone of brick ballast 1½” to 2” gauge. Ratio (1:6:12). (S.I.No.4 (d) P.No.15)

Play Ground 		1x62.0x58.0x0.50		=	1798	cft

1x22.0x10.0x0.50		=	110	cft

=	1908	cft

Qty: 1908 Cft		@RS: 8122.95		% Cft		RS: 154986/-

14/-	Cement Concrete plain including placing compacting finishing & curing, complete (including screening and washing of stone aggregate without shuttering. Ratio 1:2:4. (S.I. No. 5(f), P. No.16).

Class Room		3x22.0x18.x00.17		=	202	cft

Bath			1x8.0x6.0x0.17		=	08	cft

=	210	cft

Qty: 210 Cft		@RS: 14429.25	% Cft		RS: 30301/-

15/-	Providing and laying tile glazed (6”x 6”1/4”) on floor of wall facing in required colour and pattern of STILE specification jointed in white cement and pigment over a base of 1:2 gray cement mortar ¾” thick i/c washing and filling joints with slurry of white cement and pigment in desired shape with finishing, clearing and cost of was polish etc. complete i/c cutting tiles to proper profile. Amended  (S.I. No. 60, P. No.47)

Class Room		1x22.0x18.0			=	396	sft

Dado			2x(22.0+18.0)x0.50		=	40	sft

Bath			1x8.0x6.0			=	48	sft

1x2(8.0+6.0)x3.0		=	84	sft

=	520	sft

Qty: 520 Sft		@RS: 30509.77	% Sft		RS: 158651/-

16/-	First class deodar wood  wrought joinery  in doors & windows  etc fixed in position  including  hold fasts hinges  iron  tower bolts chock, cleats  handles  and  cords  with hooks etc complete.

(S.I No: 54 P.NO:  65).

Bath Door		1x2.50x6.83			=	17	sft

=	17	sft

Qty: 17 Sft		@RS: 1273.76		P.Sft		RS: 21654/-

| Sr | Item of work | NO:L.B.D | QTY | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- | --- |

17/-		Providing and lying 1” thick C/C topping cement concrete (1:2:4) i/c surface finishing

and dividing into panels.  (S.I.No.16 (d) P.No.42)3” thick.

Play Ground		1x62.0x58.0			=	3596	sft

=	3596	sft

Qty: 3596 Sft		@RS: 4411.82		% Sft		RS: 158649/-

==========================================

Total						RS: 688441/-

Add: Cost of Cartage						RS: 188514/-

===========================================

Total						RS: 876955/-

MATERIAL STATEMENT

Class Room

| S# | NAME OF ITEM | QTY: | CEMENT | H/SAND | CRUSH | Stone | Bricks | Steel |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | C.C 1:5:10 | 47 | 05 | 23 | - | 46 | - | - |
|  | Appplying floating | 1811 | 05 | - | - | - | - | - |
|  | Fabrication | 3.75/20 | - | - | - | - | - | 0.187 |
|  | R.C.C 1:2:4 | 84 | 15 | 37 | 74 | - | - | - |
|  | P.B Work G.Floor 1:6 | 219 | 08 | 56 | - | - | 2957 | - |
|  | Cement Plaster 1:6 | 1577 | 08 | 63 | - | - | - | - |
|  | Cement Plaster 1:4 | 1577 | 09 | 47 | - | - | - | - |
|  | C.C Plain 1:2:4 | 210 | 37 | 92 | 185 | - | - | - |
|  | C.C 1:6:12 | 1908 | 124 | 934 | - | 1870 | - | - |
|  | P/L C.C Topping 3” thick | 3596 | 158 | 396 | 791 | - | - | - |
|  |  | Total | 369 | 1648 | 1050 | 1916 | 2957 | 0.187 |
|  |  | Rate | 111.33 | 6697.88 | 1162.68 | 1195.24 | 643.55 | 213 |
|  |  | Unit | P.beg | % cft | % cft | % cft | % cft | % Cft |
|  |  | Amount | 41081 | 11038 | 12208 | 22901 | 1903 | 40 |

Total		RS: 188514/-

| Sr | Item of work | NO:L.B.D | QTY | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- | --- |

PART-B Apwa & Jannat Primary Section.

01/-	Dismantling Second Class tile roofing (S.I NO: 22 (b)P-11).

Class room		1x30.0x16.25			=	488	sft

=	488	sft

Qty: 488 Sft		@RS: 378.13		% Sft		RS: 1845/-

02/-	Removing Ceetn or lime plaster (S.I NO: 53 P-13).

Class room		1x2(28.50+16.25)x7.0		=	624	sft

“	“		3x2(20.0+20.0)x3.0		=	720	sft

Office			2x2(10.0+12.0)x3.0		=	264	sft

=	1611	sft

Qty: 1611 Sft		@RS: 121/-		% Sft		RS: 1949/-

03/-	Dismantling Cement Concrete plain 1:2:4 (S.I NO:    P.NO:    ).

Class room		2x20.0x20.0x0.17		=	136	cft

1x28.50x16.25x0.17		=	79	cft

=	215	cft

Qty: 215 Cft		@RS: 3327.50		% Cft		RS: 7042/-

04/-	Pacca Brick work in ground floor in cement sand mortar 1:6 (S.I NO:    P.NO:     ).

Under roof		2x28.0x0.75x1.0		=	42	cft

2x16.25x0.75x1.0		=	24	cft

=	66	cft

Qty: 66 cft		@RS: 12674.36	% Cft		RS: 8365/-

05/-	Supplying & Fixing R.S Girder  (4”x8”) Material Schedule .

3x18.0x14.0	=756/112	=	6.75	cwt

=	6.75	cwt

Qty: 6.75 cwt		@RS: 3850/-		P.cwt		RS: 25988/-

06/-	Supplying  & Fixing T-Iron (2’x2’x1/4”) Material Schedule.

6x28.0x2.25	=	378/112=	3.375	cwt

=	3.375	cwt

Qty: 3.375 cwt		@RS: 3575/-		P.cwt		RS: 12066/-

| Sr | Item of work | NO:L.B.D | QTY | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- | --- |

07/-	Second class tile roofing consisting of 4” earth and 1” mud plaster with gobri leaping over ½” thick cement plaster 1:6 with 34 Lbs of hot bitumen coating sand blinded provided over one layer of 12”x6”x2” tiles laid in 1:6 cement mortar including 1:2 cement pointing under neath of tiles complete including curing etc.S.I.NO. 2 Page No. 33).

Class Room		1x30.0x17.75			=	533	sft

=	533	sft

Qty: 533 Sft		@RS: 7607.05		% Sft		RS: 40547/-

08/-		Applying Floating coat of cement  1/32” thick. (S.I No: 14 P.NO: 53 ).

Same Qty: of item NO: 2				=	1611	sft

=	1611	sft

Qty: 1611 Sft		@RS: 660/-		% Sft		RS: 10633/-

09/-	Cement Plaster 1:6 up to 20’-0 height ½” thick. ( S.I NO: 13(b)    P:NO:52   )

Same Qty: of item NO: 8				=	1611	sft

=	1611	sft

Qty: 1611 Sft		@RS: 2206.60		% Sft		RS: 35548/-

10/-	Cement Plaster 1:4 up to 20”-0 height 3/8” thick. . ( S.I NO: 11(a) P:NO:52   )

Same Qty: of item NO: 9				=	1611	sft

=	1611	sft

Qty: 1611 Sft		@RS: 2197.52		% Sft		RS: 35402/-

11/-		Providing and lying 1” thick C/C topping cement concrete (1:2:4) i/c surface finishing

and dividing into panels.  (S.I.No.16 (d) P.No.42)2” thick.

Class Room		2x20.0x18.0			=	720	sft

Class room		1x28.50x16.25			=	463	sft

=	1183	sft

Qty: 1183 Sft		@RS: 3275.50		% Sft		RS: 38749/-

12/-	First class deodar wood  wrought joinery  in doors & windows  etc fixed in position  including  hold fasts hinges  iron  tower bolts chock, cleats  handles  and  cords  with hooks etc complete.

(S.I No: 54 P.NO:  65).

Door			2x4.0x6.83			=	55	sft

Window		2x4.0x5.0			=	40	sft

=	95	sft

Qty: 95 Sft		@RS: 1273.76		P.Sft		RS: 121007/-

| Sr | Item of work | NO:L.B.D | QTY | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- | --- |

13/-	Galvanized wire gauze fixed to chowakats with ¾’  deodar strip  and screw. (S.I No: 4(c) P:58 ).

Window		2x4.0x5.0			=	40	sft

Window		3x3.0x5.0			=	36	sft

h.w			4x3.0x2.0			=	24	sft

=	100	sft

Qty: 100 Sft		@RS: 190.72		P.Sft		RS: 19072/-

======================================

Total					RS: 358213/-

Add: Cost of Cartage 				RS: 25690/-

======================================

Total					RS: 383903/-

MATERIAL STATEMENT

Primary section

| S# | NAME OF ITEM | QTY: | CEMENT | H/SAND | CRUSH | Bricks |
| --- | --- | --- | --- | --- | --- | --- |
|  | P.B Work G.Floor 1:6 | 66 | 02 | 17 | - | 891 |
|  | Second class tile | 533 | 03 | 11 | - | 1066 |
|  | Appplying floating | 1611 | 06 | - | - | - |
|  | Cement Plaster 1:6 | 1611 | 09 | 64 | - | - |
|  | Cement Plaster 1:4 | 1611 | 09 | 48 | - | - |
|  | P/L C.C Topping 3” thick | 1183 | 35 | 88 | 175 | - |
|  |  | Total | 64 | 228 | 175 | 1957 |
|  |  | Rate | 111.33 | 6697.88 | 1162.68 | 643.55 |
|  |  | Unit | P.beg | % cft | % cft | % cft |
|  |  | Amount | 7125 | 15271 | 2035 | 1259 |

Total		RS: 25690/-

| Sr | Item of work | NO:L.B.D | QTY | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- | --- |

PART-C COMPOUND WALL

01/-	Pacca  Brick  work in Other than Building  in cement sand mortar (1:6) (S.I No:5 P-25).

S/S N/S		2x400.0x0.75x2.0		=	1200	cft

E/S S/W		2x150.0x0.75x2.0		=	450	cft

=	1650	cft

Qty: 1650 Cft		@RS: 12346.65	% Cft		RS: 203720/-

02/-	Providing & Fixing angle iron vertical posts for barbed wire fencing of sizw 2”x2”x1/4” embedded in R.C.C / Masnory including making cuts/holder @ 12” including fixing in pillar by chiseling and fitting the with cement sand moartar , separating & finishing the surface (S.I NO; 8 P-95).

C/wall			110x3.50			=	385	rft

=	385	rft

Qty: 385 rft		@RS: 169.18		P.Rft		RS: 65134/-

03/-	Providing & Fixing barbed wire fancing with 12 gauge 4 points @ 6” apart barbed wire including streaingthing & fixing in position in angle iron vertical posts (S.I NO: 9 P-96).

S/S N/S c/wall		2x4x400.0			=	3200	rft

E/S W/S		2x4x150.0			=	1200	rft

=	4400	rft

Qty: 4400 rft		@RS: 8.38		P.Rft		RS: 36872/-

04/-	Cement Plaster 1:6 up to 20’-0 height ½” thick. ( S.I NO: 13(b)    P:NO:52   )

S/S N/S c/wall		2x2x400.0x2.0			=	3200	sft

Top			2x1x400.0x0.83		=	664	sft

E/S W/S		2x2x150.x02.0			=	1200	sft

Top			2x1x150.0x0.83		=	249	sft

=	5313	sft

Qty: 5313 Sft		@RS: 2206.60		% Sft		RS; 17237/-

05/-	Cement Plaster 1:4 up to 20”-0 height 3/8” thick. . ( S.I NO: 11(a) P:NO:52   )

Same Qty: of item NO: 4				=	5313	sft

=	5313	sft

Qty: 5313 Sft		@RS: 2197.52		% Sft		RS: 116754/-

| Sr | Item of work | NO:L.B.D | QTY | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- | --- |

06/-	Cement Concrete plain including placing compacting finishing & curing, complete (including screening and washing of stone aggregate without shuttering. Ratio 1:2:4. (S.I. No. 5(f), P. No.16).

Holes of angle iron	1x110.0x0.50x0.50x1.0	=	28	cft

=	28	cft

Qty: 28 cft		@RS: 14429.25	% Cft		RS: 4040/-

=========================================

Total						RS: 543757/-

Add: Cost of Cartage					RS: 82105/-

=========================================

Total						RS: 625862/-

MATERIAL STATEMENT

c/wall

| S# | NAME OF ITEM | QTY: | CEMENT | H/SAND | CRUSH | Bricks |
| --- | --- | --- | --- | --- | --- | --- |
|  | P.B Work 1:6 | 1650 | 57 | 424 | - | 22275 |
|  | C.C Plain 1:2:4 | 28 | 05 | 12 | 25 | - |
|  | Cement Plaster 1:6 | 5313 | 28 | 213 | - | - |
|  | Cement Plaster 1:4 | 5313 | 30 | 159 | - | - |
|  |  | Total | 120 | 808 | 25 | 22275 |
|  |  | Rate | 111.33 | 6697.88 | 1162.68 | 643.55 |
|  |  | Unit | P.beg | % cft | % cft | % cft |
|  |  | Amount | 13360 | 54119 | 291 | 14335 |

Total		RS: 82105/-

| Sr | Item of work | NO:L.B.D | QTY | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- | --- |

PART-       W/S   & S/F

1-	P/F squatting type white glazed of flushing cistern with internal fitting and flush pipe with band and making requisite Nos. of holes in wall plinth & floor for pipe connections and making good in cement concrete 1:2:4. (A) W.C pan of not less than 23” clear opening between flashing rims and 3 gallons flushing tank. With 4” dia C.I. tape. (S.I.NO.1(A) P.No.1)

Qty:01 Nos		@Rs:4802/60			each				RS:4803/-

2/-	P/F 6”x2” or 6”x3” C.I floor trap of the approved self cleaning design with a C.I screwed down gritting with or without a vent arm complete with and i/c making requisite number or holes in walls plinth and floor for pipe connections & making good in CC 1:2:4. (S.I. No. 20 P. No.5)

Qty:02 Nos		@Rs:2042/43			each				RS:4085/-

3/-	Providing & Fixing Nylon connection  complete with  ½” dia Brass stop cock  with pair  of brass  nuts  and lining  joints  to nylon  connection (S.I No:23 P-6).

Qty:03 Nos		@Rs:447/15			each				RS: 1341/-

04/-	Providing G.I pipes special & clamps etc i/c fixing cutting and fitting complete with and i/c the cost of breaking through walls and roof making good etc painting two coats after cleaning the pipe etc with white zink paint with pigment to match the colour of the building and testing with water to a pressure head of 200 feet and handling. (Internal)(i) ½ dia G.I pipe. (S.I.NO.1 (i) P. No.12)

Qty: 30 Rft		@RS: 73.21			P.Rft				RS: 2196/-

04/-	Supplying and fixing long bib-cock of superior quality with C.P. head ½” dia. (S.I.NO.15(a) P. No.15)

Qty:03 Nos		@Rs:1109/46			each				RS:3328/-

05/-	Supplying and fixing concealed stopcock of superior quality with c.p head ½” dia.(S.I.NO.13 (a) . No.15)

Qty:02 Nos		@Rs:509.74			each				RS:1019/-

06/-	Supplying & fixing Fiber Glass Tank of approved quality and design and wall thickness as specified i/c cost of nuts bolts and fixing in plate form of cement concrete 1:3:6 and making connection for in-let outlet & over flow in-let outlet flow pipes etc. complete. (S.I.No.3 P.No.18) (Rate per gallon Rs: 33/60 for tanks upto capacity of 250 gallons) 100 Gallons.

Qty: 01 No		@RS: 6660/-			each				RS: 6660/-

===================================

Total					RS: 23432/-

NAME OF WORK:-	RENOVATION / REHABLITATION OF GOVERNMENT GIRLS HIGH SCHOOL NIZAMANI DISTRICT KHAIRPUR.

GENERAL ABSTRACT

PART-A		Admin /Class Room			RS: 876955/-

PART-B			Upwa/Jannat Primary section		RS: 383903/-

PART-C			C/wall i/c angle iron/barbed wire	RS: 625900/-

PART-D			W/S   &  S/F			RS: 23400/-

=============================================================

Total								RS: 1910000/-

NAME OF WORK:-	RENOVATION / REHABLITATION OF GOVERNMENT GIRLS HIGH SCHOOL NIZAMANI DISTRICT KHAIRPUR.

SCHEDULE-B

| Sr | Item of work | QTY | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |

Main Building (Admin Block/Class Room)

01/-	Excavation in foundation of building bridges and other structures i/c dag balling dressing refilling around structure with excavated earth watering and ramming lead up-to 5ft. (b) in ordinary soil. (S.I.No.18 (b) P.No.4)

Qty: 89 Cft		@RS: 3176.25		% 0Cft		RS: 283/-

Thirty one hundred seventy six rups:twenty five ps:

02/-	Cement concrete stone of brick ballast 1½” to 2” gauge. Ratio (1:4:8). (S.I.No.4 (b) P.No.15)

Qty: 47 Cft		@ RS: 9416.28		% Cft		RS: 4426/-

(Ninty four hudred, sixteen rupees, twenty eight ps:)

03/-	Pacca  Brick  work in  foundation & plinth  work  in cement sand mortar (1:6) (S.I No:5 P-25).

Qty: 84 Cft		@RS: 11948.36	% Cft		RS: 10037/-

(Eleven thousands nine hundered fourty eight rs: thirty six ps.)

04/-	R.C.C. work in roof slab beams columns rafts lintels and other structural members laid in situ or precast laid in position complete in all respects: (i) Ratio (1:2:4) 90 lbs cement 2 cft sand 4 cft shingle 1/8” to 1/4” gauge. (S.I.No.6 (a-i) P.No. 17)

Qty: 84 Cft		@ RS: 337/-		P.Cft		RS: 28308/-

(Three hudred thirt seven ps.)

05/-	Fabrication of mild steel reinforcement for cement concrete i/c cutting bending laying in position making joints and fastenings i/c cost of binding wire (also includes removal of rust from bars) (a) Using tor bars. (S.I.No.7 (a) P.No.20)

Qty: 3.75 cwt		@RS: 5001.70		P.cwt		RS: 18756/-

(Five thousands & one ripes, seventy ps:)

06/-	Pacca Brick work in ground floor i:6 in cement sand mortar etc complete (S.I NO:    P.NO:     ).

Qty: 135 Cft		@RS: 12674.36	% Cft		RS: 17110/-							(Tweleve thousands six hundred seventy four rupees, sixty five ps.)

07/-	Supplying & Fixing sand under floor plugging in to wall. (S.I NO:     P.No:       ).

Qty: 96 Cft		@RS: 1141.25		% Cft		RS: 1096/-

Eleven hundred fourty one rupees, twenty five ps.)

08/-	Dismantling Cement Concrete plain (1:2:4) (S.I NO: 19 (c)P-10).

Qty: 135 Cft		@RS: 3327.50		% Cft		RS: 4492/-

(thirty three hundreds twenty seven rups: fifty ps:

| Sr | Item of work | QTY | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |

09/-	Removing Cement or lime plaster (S.I NO: 53 P-13).

Qty: 1311 Sft		@RS: 121/-		% Sft		RS: 1586/-

(One hundred twenty one rupees.)

10/-	Apllying floating coat of cement 1/32” thick. (S.I NO:   P.NO:   ).

Qty: 1311 Sft		@RS: 660/-		% Sft		RS: 8653/-

(six hundreds sixty rupes)

11/-	Cement Plaster 1:6 up to 20’-0 height ½” thick. ( S.I NO: 13(b)    P:NO:52   )

Qty: 1577 Sft		@RS: 2206.60		% Sft		RS: 34798/-

(twenty two hundred six rupees, sixty ps.)

12/-	Cement Plaster 1:4 up to 20”-0 height 3/8” thick. . ( S.I NO: 11(a) P:NO:52   )

Qty: 1577Sft		@RS:2197.52		% Sft		RS: 34655/-

(twenty one hundred ninty seven rupes: fifty two ps.)

13/-	Cement concrete stone of brick ballast 1½” to 2” gauge. Ratio (1:6:12). (S.I.No.4 (d) P.No.15)

Qty: 1908 Cft		@RS: 8122.95		% Cft		RS: 154986/-

(eighty one hundreds twenty two rupes: ninty five ps)

14/-	Cement Concrete plain including placing compacting finishing & curing, complete (including screening and washing of stone aggregate without shuttering. Ratio 1:2:4. (S.I. No. 5(f), P. No.16).

Qty: 210 Cft		@RS: 14429.25	% Cft		RS: 30301/-

(fourteen thousands four hundred twenty nine rups. Twenty five ps.)

15/-	Providing and laying tile glazed (6”x 6”1/4”) on floor of wall facing in required colour and pattern of STILE specification jointed in white cement and pigment over a base of 1:2 gray cement mortar ¾” thick i/c washing and filling joints with slurry of white cement and pigment in desired shape with finishing, clearing and cost of was polish etc. complete i/c cutting tiles to proper profile. Amended  (S.I. No. 60, P. No.47)

Qty: 520 Sft		@RS: 30509.77	% Sft		RS: 158651/-

(thirty thousands five hundreds nine rupe & seventy seven ps:)

16/-	First class deodar wood  wrought joinery  in doors & windows  etc fixed in position  including  hold fasts hinges  iron  tower bolts chock, cleats  handles  and  cords  with hooks etc complete.	(S.I No: 54 P.NO:  65).

Qty: 17 Sft		@RS: 1273.76		P.Sft		RS: 21654/-

(tweleve hundreds seventy three rupes& seventy six)

17/-		Providing and lying 1” thick C/C topping cement concrete (1:2:4) i/c surface finishing and dividing into panels.  (S.I.No.16 (d) P.No.42)3” thick.

Qty: 3596 Sft		@RS: 4411.82		% Sft		RS: 158649/-

(fourty four hundreds eleven rupes & eighty two ps:)

==========================================

Total						RS: 688441/-

| Sr | Item of work | QTY | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |

PART-B Apwa & Jannat Primary Section.

01/-	Dismantling Second Class tile roofing (S.I NO: 22 (b)P-11).

Qty: 488 Sft		@RS: 378.13		% Sft		RS: 1845/-

(three hundreds seventy eight rupes & thirteen ps:)

02/-	Removing Ceetn or lime plaster (S.I NO: 53 P-13).

Qty: 1611 Sft		@RS: 121/-		% Sft		RS: 1949/-

(One hundred twenty one rupees.)

03/-	Dismantling Cement Concrete plain 1:2:4 (S.I NO:    P.NO:    ).

Qty: 215 Cft		@RS: 3327.50		% Cft		RS: 7042/-

(thirty three hundreds twenty seven rupes & fifty ps:)

04/-	Pacca Brick work in ground floor in cement sand mortar 1:6 (S.I NO:    P.NO:     ).

Qty: 66 cft		@RS: 12674.36	% Cft		RS: 8365/-

(Tweleve thousands six hundred seventy four rupees, sixty five ps.)

05/-	Supplying & Fixing R.S Girder  (4”x8”) Material Schedule .

Qty: 6.75 cwt		@RS: 3850/-		P.cwt		RS: 25988/-

(thirty eight hundreds fifty rupes:)

06/-	Supplying  & Fixing T-Iron (2’x2’x1/4”) Material Schedule.

Qty: 3.375 cwt		@RS: 3575/-		P.cwt		RS: 12066/-

(thirty five hundreds seventy fibe rupes)

07/-	Second class tile roofing consisting of 4” earth and 1” mud plaster with gobri leaping over ½” thick cement plaster 1:6 with 34 Lbs of hot bitumen coating sand blinded provided over one layer of 12”x6”x2” tiles laid in 1:6 cement mortar including 1:2 cement pointing under neath of tiles complete including curing etc.S.I.NO. 2 Page No. 33).

Qty: 533 Sft		@RS: 7607.05		% Sft		RS: 40547/-

(seventy six hundred seven rupees, twenty five ps.)

08/-		Applying Floating coat of cement  1/32” thick. (S.I No: 14 P.NO: 53 ).

Qty: 1611 Sft		@RS: 660/-		% Sft		RS: 10633/-

(six hundreds sixty rupes)

09/-	Cement Plaster 1:6 up to 20’-0 height ½” thick. ( S.I NO: 13(b)    P:NO:52   )

Qty: 1611 Sft		@RS: 2206.60		% Sft		RS: 35548/-

(twenty two hundred six rupees, sixty ps.)

10/-	Cement Plaster 1:4 up to 20”-0 height 3/8” thick. . ( S.I

NO: 11(a) P:NO:52   )

Qty: 1611 Sft		@RS: 2197.52		% Sft		RS: 35402/-

(twenty one hundred ninty seven rupes: fifty two ps.)

11/-		Providing and lying 1” thick C/C topping cement concrete (1:2:4) i/c surface finishing and dividing into panels.  (S.I.No.16 (d) P.No.42)2” thick.

Qty: 1183 Sft		@RS: 3275.50		% Sft		RS: 38749/-

(thirty two hundred seventy five rupes, fifty ps.)

| Sr | Item of work | QTY | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |

12/-	First class deodar wood  wrought joinery  in doors & windows  etc fixed in position  including  hold fasts hinges  iron  tower bolts chock, cleats  handles  and  cords  with hooks etc complete.

(S.I No: 54 P.NO:  65).

Qty: 95 Sft		@RS: 1273.76		P.Sft		RS: 121007/-

(tweleve hundreds seventy three rupes& seventy six)

13/-	Galvanized wire gauze fixed to chowakats with ¾’  deodar strip  and screw. (S.I No: 4(c) P:58 ).

Qty: 100 Sft		@RS: 190.72		P.Sft		RS: 19072/-

(One hundred ninty rupes & seventy two ps:)

======================================

Total					RS: 358213/-

| Sr | Item of work | QTY | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |

PART-C COMPOUND WALL

01/-	Pacca  Brick  work in Other than Building  in cement sand mortar (1:6) (S.I No:5 P-25).

Qty: 1650 Cft		@RS: 12346.65	% Cft		RS: 203720/-

(tweleve thousands three hundred fourty six rupes sixty five ps.)

02/-	Providing & Fixing angle iron vertical posts for barbed wire fencing of sizw 2”x2”x1/4” embedded in R.C.C / Masnory including making cuts/holder @ 12” including fixing in pillar by chiseling and fitting the with cement sand moartar , separating & finishing the surface (S.I NO; 8 P-95).

Qty: 385 rft		@RS: 169.18		P.Rft		RS: 65134/-

(One hundred sixtu nine rupes & eighteen ps:)

03/-	Providing & Fixing barbed wire fancing with 12 gauge 4 points @ 6” apart barbed wire including streaingthing & fixing in position in angle iron vertical posts (S.I NO: 9 P-96).

Qty: 4400 rft		@RS: 8.38		P.Rft		RS: 36872/-

(Eight rupees & thirty eight ps)

04/-	Cement Plaster 1:6 up to 20’-0 height ½” thick. ( S.I NO: 13(b)    P:NO:52   )

Qty: 5313 Sft		@RS: 2206.60		% Sft		RS; 17237/-

(twenty two hundred six rupees, sixty ps.)

05/-	Cement Plaster 1:4 up to 20”-0 height 3/8” thick. . ( S.I NO: 11(a) P:NO:52   )

Qty: 5313 Sft		@RS: 2197.52		% Sft		RS: 116754/-

(twenty one hundred ninty seven rupes: fifty two ps.)

06/-	Cement Concrete plain including placing compacting finishing & curing, complete (including screening and washing of stone aggregate without shuttering. Ratio 1:2:4. (S.I. No. 5(f), P. No.16).

Qty: 28 cft		@RS: 14429.25	% Cft		RS: 4040/-

(fourteen thousands four hundred twenty nine rups. Twenty five ps.)

=========================================

Total						RS: 543757/-

| Sr | Item of work | QTY | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |

PART-       W/S   & S/F

1-	P/F squatting type white glazed of flushing cistern with internal fitting and flush pipe with band and making requisite Nos. of holes in wall plinth & floor for pipe connections and making good in cement concrete 1:2:4. (A) W.C pan of not less than 23” clear opening between flashing rims and 3 gallons flushing tank. With 4” dia C.I. tape. (S.I.NO.1(A) P.No.1)

Qty:01 Nos		@Rs:4802/60			each				RS:4803/-

(fourty eight hundreds two rupes & sixty ps:)

2/-	P/F 6”x2” or 6”x3” C.I floor trap of the approved self cleaning design with a C.I screwed down gritting with or without a vent arm complete with and i/c making requisite number or holes in walls plinth and floor for pipe connections & making good in CC 1:2:4. (S.I. No. 20 P. No.5)

Qty:02 Nos		@Rs:2042/43			each				RS:4085/-

(twenty hundreds two rupes & fourty three ps:)

3/-	Providing & Fixing Nylon connection  complete with  ½” dia Brass stop cock  with pair  of brass  nuts  and lining  joints  to nylon  connection (S.I No:23 P-6).

Qty:03 Nos		@Rs:447/15			each				RS: 1341/-

(four hundreds fourty seven rupes & fifteen ps:)

04/-	Providing G.I pipes special & clamps etc i/c fixing cutting and fitting complete with and i/c the cost of breaking through walls and roof making good etc painting two coats after cleaning the pipe etc with white zink paint with pigment to match the colour of the building and testing with water to a pressure head of 200 feet and handling. (Internal)(i) ½ dia G.I pipe. (S.I.NO.1 (i) P. No.12)

Qty: 30 Rft		@RS: 73.21			P.Rft				RS: 2196/-

(seventy three rupees & twenty one ps:)

04/-	Supplying and fixing long bib-cock of superior quality with C.P. head ½” dia. (S.I.NO.15(a) P. No.15)

Qty:03 Nos		@Rs:1109/46			each				RS:3328/-

(eleven hundreds nine rupes & fourty six ps:)

05/-	Supplying and fixing concealed stopcock of superior quality with c.p head ½” dia.(S.I.NO.13 (a) . No.15)

Qty:02 Nos		@Rs:509.74			each				RS:1019/-

(five hundreds nine rupes & seventy four ps:)

06/-	Supplying & fixing Fiber Glass Tank of approved quality and design and wall thickness as specified i/c cost of nuts bolts and fixing in plate form of cement concrete 1:3:6 and making connection for in-let outlet & over flow in-let outlet flow pipes etc. complete. (S.I.No.3 P.No.18) (Rate per gallon Rs: 33/60 for tanks upto capacity of 250 gallons) 100 Gallons.

Qty: 01 No		@RS: 6660/-			each				RS: 6660/-

(sixty six hundreds sixty rupes)

===================================

Total					RS: 23432/-

NAME OF WORK:-	RENOVATION / REHABLITATION OF GOVERNMENT GIRLS HIGH SCHOOL NIZAMANI DISTRICT KHAIRPUR.

GENERAL ABSTRACT

PART-A		Admin /Class Room			RS: 876955/-

PART-B			Upwa/Jannat Primary section		RS: 383903/-

PART-C			C/wall i/c angle iron/barbed wire	RS: 625900/-

PART-D			W/S   &  S/F			RS: 23400/-

=============================================================

Total								RS: 1910000/-

FACE SHEET

FUNDS HEAD:.		__________________________________________

MAJOR HEAD:.		__________________________________________

MINOR HEAH:.		RENOVATION / REHABLITATION OF GOVERNMENT GIRLS

HIGH SCHOOL NIZAMANI DISTRICT KHAIRPUR.

This Estimate Has Been Framed in The Office Of The Executive Engineer (Buildings) Works & Service Department  Khairpur For Probable  Expenditure That Will Be  Incurred On the Said Works.

Amounting to Rs. 1.910 (M)