SCHEDULE OF QUANTITIES

NIT No: 01/2016-17, WORK No: 01

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:	      9,96,033/-		Earnest Money Rs:	         19,921/-

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | CONSTRUCTION OF NALLAH BANGASH PLAZA, BHUTTA VILLAGE UC-44, KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Excavation for foundation of building brigs and other structure i/c dressing refilling, around structure with excavate earth watering and ramming lead upto 05 feet. | 7,875 Cft | 3176.25 | 1000 Cft | 25,013.00 |
| 2. | Cement concrete brick or stone ballast 1½” to 2” gauge (1:4:8). | 788 Cft | 9416.28 | %Cft | 74,200.00 |
| 3. | Erection and removal of centering for R.C.C. or plain cement concrete works of Deodar Wood (2nd class). | 4,200 Sft | 3127.41 | %Sft | 1,31,351.00 |
| 4. | Cement concrete plain including placing compaction finishing and curing complete including screening and washing at stone aggregate without ratio 1:2:4. | 2,100 Cft | 14429.25 | %Cft | 3,03,014.00 |
| 5. | Fabrication of mild steel reinforcement for cement concrete i/c cutting, bending, layer in position making joints and fastening i/c cost of binding wire etc complete. | 21.10 Cwt | 5001.70 | P/Cwt | 1,05,536.00 |
| 6. | RCC work in roof slab, beams columns rafts lintels and other structural members laid in situ or precast laid in position complete in all respects (ratio 1:2:4). | 788 Cft | 337.00 | P/Cft | 2,65,556.00 |
| 7. | Manufacturing and supplying R.C.C. manhole covers casting c.c. 1:2:4 ratio 3 inch deep at center bar, reinforced with 3/8” dia tour steel 2.50” deep M.S. plate other curing, stacking and transportation within 10 miles. …………...………………….…21” Dia …..……………….……….... | 100 Nos | 913.63 | Each | 91,363.00 |
| Total Estimated Cost |  |  |  |  | 9,96,033.00 |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

*********************************

I/We hereby quoted Rates @                           %  above/below the S/R

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

NIT No: 01/2016-17, WORK No: 02

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:	      9,90,092/-		Earnest Money Rs:	         19,802/-

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | P/L R.C.C. PIPE DRAIN IN STREET No:41, SHER SHAH UC-01, KEAMARI ZONE, DMC(WEST0. |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Excavation for pipe line in trenches and pits in all kind of soil of murum i/c trimming and dressing side to true alignment and shape traveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guard light flags and temporary crossing for non vehicular traffic wherever required left upto 05’ and lead upto 01 chain. | 9,750 Cft | 4650.00 | 1000 Cft | 45,338.00 |
| 2. | A. PROVIDING, LAYING & JOINTING (Rubber Gasket, JOINTS). Providing, laying R.C.C. pipes of A.S.T.M. C-76-62 T/C: 76-70 of class II wall B and fixing in trench i/c cutting, fitting and jointing with rubber ring i/c testing with water to specified pressure. ……………………………..…12” Dia………………….…………….. | 1300 Rft | 412.00 | P/Rft | 5,35,600.00 |
| 3. | Providing manhole 4’ dia (inside) and 5’ clear depth complete with 21” dia ( inside ) C.I frame & 21” dia of 3” multiply 3 x 8, flat iron frame for R.C.C 1:2:4 pre-cast cover i/c the cost of M.S bars reinforcement and two handles of 5 clear depth C.C 1:2:4 cast in Situ walls with 50% screened graded bajri of 9” thick i/c benching and channel ½” thick cement plaster 1:3 on all inside walls surface complete with c.i Foot rest i/c cutting asphalt roads excavation in all kind of soil, back filling and disposal of surplus stuff etc. | 50 Nos | 9307.86 | Each | 4,65,393.00 |
| 4. | Add/Less for extra depth beyond 5’ depth, less than 5’ depth deduct at the same rate. | ( - ) 50 Rft | 1609.15 | P/Rft | ( - ) 80,458.00 |
| 5. | Refilling the excavated stuff in trenches 6” thick layer i/c watering ramming to full compaction etc complete. | 8,775 Cft | 2760.00 | 1000 Cft | 24,219.00 |
| Total Estimated Cost |  |  |  |  | 9,90,092.00 |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

*********************************

I/We hereby quoted Rates @                           %  above/below the S/R

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

NIT No: 01/2016-17, WORK No: 03

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:	      9,90,092/-		Earnest Money Rs:	         19,802/-

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | P/L R.C.C. PIPE DRAIN IN STREET No:41, SHER SHAH UC-01, KEAMARI ZONE, DMC(WEST0. |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Excavation for wet silt clearance etc complete. | 6,000 Cft | 5142.50 | ‰ Cft | 30,855.00 |
| 2. | Excavation for pipe line in trenches and pits in all kind of soil of murum i/c trimming and dressing side to true alignment and shape traveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guard light flags and temporary crossing for non vehicular traffic wherever required left upto 05’ and lead upto 01 chain. | 2,625 Cft | 4650.00 | 1000 Cft | 12,206.00 |
| 3. | A. PROVIDING, LAYING & JOINTING (Rubber Gasket, JOINTS). Providing, laying R.C.C. pipes of A.S.T.M. C-76-62 T/C: 76-70 of class II wall B and fixing in trench i/c cutting, fitting and jointing with rubber ring i/c testing with water to specified pressure. ………………………………..……..…12” Dia………………….………………... ………………………………..……..…18” Dia………………….………………... | 300 Rft 600 Rft | 412.00 618.00 | P/Rft P/Rft | 1,23,600.00 3,70,800.00 |
| 4. | Providing manhole 4’ dia (inside) and 5’ clear depth complete with 21” dia ( inside ) C.I frame & 21” dia of 3” multiply 3 x 8, flat iron frame for R.C.C 1:2:4 pre-cast cover i/c the cost of M.S bars reinforcement and two handles of 5 clear depth C.C 1:2:4 cast in Situ walls with 50% screened graded bajri of 9” thick i/c benching and channel ½” thick cement plaster 1:3 on all inside walls surface complete with c.i Foot rest i/c cutting asphalt roads excavation in all kind of soil, back filling and disposal of surplus stuff etc. | 38 Nos | 9307.86 | Each | 3,53,699.00.00 |
| 5. | Add/Less for extra depth beyond 5’ depth, less than 5’ depth deduct at the same rate. | ( - ) 38 Rft | 1609.15 | P/Rft | ( - ) 61,148.00 |
| 6. | Refilling the excavated stuff in trenches 6” thick layer i/c watering ramming to full compaction etc complete. | 2,363 Cft | 2760.00 | 1000 Cft | 6,522.00 |
| 7. | Cartage of 100 Cft / 05 Tons of all materials like stone aggregate, spawl, coal, lime, surkhi etc B.G. rail fastening point and crossing bridge girders, pipes sheet rails M.S. bar etc or 1000 number bricks 10”x5”x3” or 1000 manuds of fuel wood by trucks or any other means owned by the contractors (lead upto 15 miles etc). | 3,840 Cft | 1065.04 | %Cft | 40,898.00 |
| 8. | Filling earth from outside source under floor. | 4,500 Cft | 3630.00 | %Cft | 16,335.00 |
| 9. | Manufacturing and supplying R.C.C. manhole covers casting c.c. 1:2:4 ratio 3 inch deep at center bar, reinforced with 3/8” dia tour steel 2.50” deep M.S. plate other curing, stacking and transportation within 10 miles. ………………………………..……..…21” Dia………………….………………... | 40 Nos | 913.63 | Each | 36,545.00 |
| 10. | Manufacturing and supplying of R.C.C. ring slab of 21” inside 36” outside 7.5” width and 6” thick i/c 3/8” for steel bars two concentric ring with 3/8” 08 Nos. cross linked bars welded and two sunk type hooks casted etc. | 36 Nos | 1906.60 | Each | 68,638.00 |
| Total Estimated Cost |  |  |  |  | 9,98,950.00 |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

*********************************

I/We hereby quoted Rates @                           %  above/below the S/R

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

NIT No: 01/2016-17, WORK No: 04

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:	      9,98,569/-		Earnest Money Rs:	         19,971/-

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | CONSTRUCTION OF R.C.C. DUST BIN AT JINNAH ROAD SHER SHAH UC-01, KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Excavation for foundation of building brigs and other structure i/c dressing refilling, around structure with excavate earth watering and ramming lead upto 05 feet. | 900 Cft | 3176.25 | 1000 Cft | 2,859.00 |
| 2. | Cement concrete brick or stone ballast 1½” to 2” gauge (1:4:8). | 350 Cft | 9416.28 | %Cft | 32,957.00 |
| 3. | RCC work in roof slab, beams columns rafts lintels and other structural members laid in situ or precast laid in position complete in all respects (ratio 1:2:4). | 810 Cft | 337.00 | P/Cft | 2,72,970.00 |
| 4. | Fabrication of mild steel reinforcement for cement concrete i/c cutting, bending, layer in position making joints and fastening i/c cost of binding wire etc complete. | 34.75 Cwt | 5001.70 | P/Cwt | 1,73,809.00 |
| 5. | Providing and laying 2” thick topping and concrete (1:2:4) including surface finishing and dividing into panels. | 400 Sft | 3275.50 | %Sft | 13,102.00 |
| 6. | Excavation for pipe line in trenches and pits in all kind of soil of murum i/c trimming and dressing side to true alignment and shape traveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guard light flags and temporary crossing for non vehicular traffic wherever required left upto 05’ and lead upto 01 chain. | 4,813 Cft | 4650.00 | 1000 Cft | 22,380.00 |
| 7. | A. PROVIDING, LAYING & JOINTING (Rubber Gasket, JOINTS). Providing, laying R.C.C. pipes of A.S.T.M. C-76-62 T/C: 76-70 of class II wall B and fixing in trench i/c cutting, fitting and jointing with rubber ring i/c testing with water to specified pressure. ………………………………..……..…12” Dia…………………….………………... | 550 Rft | 412.00 | P/Rft | 2,26,600.00 |
| 8. | Providing manhole 4’ dia (inside) and 5’ clear depth complete with 21” dia ( inside ) C.I frame & 21” dia of 3” multiply 3 x 8, flat iron frame for R.C.C 1:2:4 pre-cast cover i/c the cost of M.S bars reinforcement and two handles of 5 clear depth C.C 1:2:4 cast in Situ walls with 50% screened graded bajri of 9” thick i/c benching and channel ½” thick cement plaster 1:3 on all inside walls surface complete with c.i Foot rest i/c cutting asphalt roads excavation in all kind of soil, back filling and disposal of surplus stuff etc. | 23 Nos | 9307.86 | Each | 2,14,081.00 |
| 9. | Add/Less for extra depth beyond 5’ depth, less than 5’ depth deduct at the same rate. | ( - ) 23 Rft | 1609.15 | P/Rft | ( - ) 37,010.00 |
| 10. | Refilling the excavated stuff in trenches 6” thick layer i/c watering ramming to full compaction etc complete. | 4,332 Cft | 2760.00 | 1000 Cft | 11,956.00 |
| 11. | Manufacturing and supplying R.C.C. manhole covers casting c.c. 1:2:4 ratio 3 inch deep at center bar, reinforced with 3/8” dia tour steel 2.50” deep M.S. plate other curing, stacking and transportation within 10 miles. ………………………………..……..…21” Dia…………………….………………... | 23 Nos | 913.63 | Each | 21,013.00 |
| 12. | Manufacturing and supplying of R.C.C. ring slab of 21” inside 36” outside 7.5” width and 6” thick i/c 3/8” for steel bars two concentric ring with 3/8” 08 Nos. cross linked bars welded and two sunk type hooks casted etc. | 23 Nos | 1906.60 | Each | 43,852.00 |
| Total Estimated Cost |  |  |  |  | 9,98,569.00 |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

*********************************

I/We hereby quoted Rates @                           %  above/below the S/R

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

NIT No: 01/2016-17, WORK No: 05

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:	      9,97,492/-		Earnest Money Rs:	         19,950/-

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | WINCHING OF PIPE DRAIN FROM STREET No:01 TO 17 BLOCK “B” AND STREET No:01 TO 09 BLOCK “A” I/C R.C.C. PIPE DRAIN IN SHER SHAH UC-02, KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Cleaning inside sewerage lines completely restoring original silt free space/diameter of pipe lines by labour and equipment. (Mechanical/electrically driven) including accessories like pulley’s steel rope, bucket and daggers sizing from 6” to18” dia. Their to and from pulling action (no of passes shall be as many as required) would be taken in two phase (First phase entire length from downstream to upstream) bringing down peack hours sewerage to flow inside pipe line ( as free flow) and in second phase from upstream to downstream of entire length ensuring no silt is observed in buckets (6” dia to 18” dia) except only sewage water and finally passing a steel ball of dia 12” lesser that the diameter of pipe but not exceeding 48” dia ball even for larger dia pipe to ensure perfect cleaning. The job including all the hire/cost of equipment and accessories of above winching drum set having, steel rope pully mounted over it steel bucket from 6” to 18” dia hooks and other protection likes safety barriers, traffic sings, traffic cones ensuring no damage to pipe along with ensuring safety to labour and other public property/lives and removal of silt/solid during desilting and clearance of site etc. complete as per full satisfaction of site engineer. ………………………….18” Dia……………………….. | 1400 Rft | 128.80 | P/Rft | 1,80,320.00 |
| 2. | Dismantling existing asphalt road etc. | 240 Cft | 605.00 | %Cft | 1,452.00 |
| 3. | Excavation for pipe line in trenches and pits in all kind of soil of murum i/c trimming and dressing side to true alignment and shape traveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guard light flags and temporary crossing for non vehicular traffic wherever required left upto 05’ and lead upto 01 chain. | 12,960 Cft | 4650.00 | 1000 Cft | 60,264.00 |
| 4. | A. PROVIDING, LAYING & JOINTING (Rubber Gasket, JOINTS). Providing, laying R.C.C. pipes of A.S.T.M. C-76-62 T/C: 76-70 of class II wall B and fixing in trench i/c cutting, fitting and jointing with rubber ring i/c testing with water to specified pressure. ………………………..…24” Dia……………………... ………………………..…12” Dia……………………... | 60 Rft 600 Rft | 869.00 412.00 | P/Rft P/Rft | 52,140.00 2,47,200.00 |
| 5. | Providing manhole 4’ dia (inside) and 5’ clear depth complete with 21” dia ( inside ) C.I frame & 21” dia of 3” multiply 3 x 8, flat iron frame for R.C.C 1:2:4 pre-cast cover i/c the cost of M.S bars reinforcement and two handles of 5 clear depth C.C 1:2:4 cast in Situ walls with 50% screened graded bajri of 9” thick i/c benching and channel ½” thick cement plaster 1:3 on all inside walls surface complete with c.i Foot rest i/c cutting asphalt roads excavation in all kind of soil, back filling and disposal of surplus stuff etc. | 26 Nos | 9307.86 | Each | 2,42,004.00 |

Cont’d Page No:02

Page No:02

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 6. | Add/Less for extra depth beyond 5’ depth, less than 5’ depth deduct at the same rate. | ( + ) 78 Rft | 1609.15 | P/Rft | ( + ) 1,25,514.00 |
| 7. | Refilling the excavated stuff in trenches 6” thick layer i/c watering ramming to full compaction etc complete. | 11,664 Cft | 2760.00 | 1000 Cft | 32,193.00 |
| 8. | Manufacturing and supplying R.C.C. manhole covers casting c.c. 1:2:4 ratio 3 inch deep at center bar, reinforced with 3/8” dia tour steel 2.50” deep M.S. plate other curing, stacking and transportation within 10 miles. ………………………..…21” Dia……………………... | 20 Nos | 913.63 | Each | 18,273.00 |
| 9. | Manufacturing and supplying of R.C.C. ring slab of 21” inside 36” outside 7.5” width and 6” thick i/c 3/8” for steel bars two concentric ring with 3/8” 08 Nos. cross linked bars welded and two sunk type hooks casted etc. | 20 Nos | 1906.60 | Each | 38,132.00 |
| Total Estimated Cost |  |  |  |  | 9,97,492.00 |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

*********************************

I/We hereby quoted Rates @                           %  above/below the S/R

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

NIT No: 01/2016-17, WORK No: 06

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:	      9,97,791/-		Earnest Money Rs:	         19,956/-

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | P/L R.C.C. PIPE DRAIN IN DIFFERENT AREA UC-44 & 45, KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Excavation for pipe line in trenches and pits in all kind of soil of murum i/c trimming and dressing side to true alignment and shape traveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guard light flags and temporary crossing for non vehicular traffic wherever required left upto 05’ and lead upto 01 chain. | 9,750 Cft | 4650.00 | 1000 Cft | 45,338.00 |
| 2. | A. PROVIDING, LAYING & JOINTING (Rubber Gasket, JOINTS). Providing, laying R.C.C. pipes of A.S.T.M. C-76-62 T/C: 76-70 of class II wall B and fixing in trench i/c cutting, fitting and jointing with rubber ring i/c testing with water to specified pressure. ……………………………..…12” Dia………………….…………….. | 1300 Rft | 412.00 | P/Rft | 5,35,600.00 |
| 3. | Providing manhole 4’ dia (inside) and 5’ clear depth complete with 21” dia ( inside ) C.I frame & 21” dia of 3” multiply 3 x 8, flat iron frame for R.C.C 1:2:4 pre-cast cover i/c the cost of M.S bars reinforcement and two handles of 5 clear depth C.C 1:2:4 cast in Situ walls with 50% screened graded bajri of 9” thick i/c benching and channel ½” thick cement plaster 1:3 on all inside walls surface complete with c.i Foot rest i/c cutting asphalt roads excavation in all kind of soil, back filling and disposal of surplus stuff etc. | 51 Nos | 9307.86 | Each | 4,74,70100 |
| 4. | Add/Less for extra depth beyond 5’ depth, less than 5’ depth deduct at the same rate. | ( - ) 51 Rft | 1609.15 | P/Rft | ( - ) 82,067.00 |
| 5. | Refilling the excavated stuff in trenches 6” thick layer i/c watering ramming to full compaction etc complete. | 8,775 Cft | 2760.00 | 1000 Cft | 24,219.00 |
| Total Estimated Cost |  |  |  |  | 9,97,791.00 |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

*********************************

I/We hereby quoted Rates @                           %  above/below the S/R

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

NIT No: 01/2016-17, WORK No: 07

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:	      9,96,118/-		Earnest Money Rs:	         19,922/-

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | WINCHING OF PIPE DRAIN FROM STREET No:19 TO 57 BLOCK “B” SHER SHAH UC-01, KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Cleaning inside sewerage lines completely restoring original silt free space/diameter of pipe lines by labour and equipment. (Mechanical/electrically driven) including accessories like pulley’s steel rope, bucket and daggers sizing from 6” to18” dia. Their to and from pulling action (no of passes shall be as many as required) would be taken in two phase (First phase entire length from downstream to upstream) bringing down peack hours sewerage to flow inside pipe line ( as free flow) and in second phase from upstream to downstream of entire length ensuring no silt is observed in buckets (6” dia to 18” dia) except only sewage water and finally passing a steel ball of dia 12” lesser that the diameter of pipe but not exceeding 48” dia ball even for larger dia pipe to ensure perfect cleaning. The job including all the hire/cost of equipment and accessories of above winching drum set having, steel rope pully mounted over it steel bucket from 6” to 18” dia hooks and other protection likes safety barriers, traffic sings, traffic cones ensuring no damage to pipe along with ensuring safety to labour and other public property/lives and removal of silt/solid during desilting and clearance of site etc. complete as per full satisfaction of site engineer. …………………………….…….36” Dia………………………………. | 2,250 Rft | 289.80 | P/Rft | 6,52,050.00 |
| 2. | Manufacturing and supplying R.C.C. manhole covers casting c.c. 1:2:4 ratio 3 inch deep at center bar, reinforced with 3/8” dia tour steel 2.50” deep M.S. plate other curing, stacking and transportation within 10 miles. …………………………….…….21” Dia………………………………. | 122 Nos | 913.63 | Each | 1,11,463.00 |
| 3. | Manufacturing and supplying of R.C.C. ring slab of 21” inside 36” outside 7.5” width and 6” thick i/c 3/8” for steel bars two concentric ring with 3/8” 08 Nos. cross linked bars welded and two sunk type hooks casted etc. | 122 Nos | 1906.60 | Each | 2,32,605.00 |
| Total Estimated Cost |  |  |  |  | 9,96,118.00 |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

*********************************

I/We hereby quoted Rates @                           %  above/below the S/R

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

NIT No: 01/2016-17, WORK No: 08

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:	      9,98,430/-		Earnest Money Rs:	         19,969/-

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | REPAIR OF R.C.C. SLAB AT CHARNAL ROAD UC-44, KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Excavation for wet silt clearance etc complete. | 18,920 Cft | 5142.50 | 1000 Cft | 97,296.00 |
| 2. | Cartage of 100 Cft / 05 Tons of all materials like stone aggregate, spawl, coal, lime, surkhi etc B.G. rail fastening point and crossing bridge girders, pipes sheet rails M.S. bar etc or 1000 number bricks 10”x5”x3” or 1000 manuds of fuel wood by trucks or any other means owned by the contractors (lead upto 15 miles etc). | 15,136 Cft | 1065.04 | %Cft | 1,61,204.00 |
| 3. | R.C.C. working roof slab beams, column, rafts, lintels and other structural members laid in situ or pre-cast laid in position complete in all respect 1:2:4. | 1,500 Cft | 337.00 | P/Cft | 5,05,500.00 |
| 4. | Fabrication of mild steel reinforcement for cement concrete including cutting, bending, laying in position making joints and fastenings including cost of binding wire (also includes removal of rust from bars) using tor bars. | 46.87 Cwt | 5001.70 | P/Cwt | 2,34,430.00 |
| Total Estimated Cost |  |  |  |  | 9,98,430.00 |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

*********************************

I/We hereby quoted Rates @                           %  above/below the S/R

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

NIT No: 01/2016-17, WORK No: 09

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:   8,50,080 (+) O/R		Earnest Money Rs:	17,002 (+) 2% of O/R

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | PROVIDING WINCHING AT MASROOR COLONY DILFULABAD UC-40, MAURIPUR KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Cleaning inside sewerage lines completely restoring original silt free space/diameter of pipe lines by labour and equipment. (Mechanical/electrically driven) including accessories like pulley’s steel rope, bucket and daggers sizing from 6” to18” dia. Their to and from pulling action (no of passes shall be as many as required) would be taken in two phase (First phase entire length from downstream to upstream) bringing down peack hours sewerage to flow inside pipe line ( as free flow) and in second phase from upstream to downstream of entire length ensuring no silt is observed in buckets (6” dia to 18” dia) except only sewage water and finally passing a steel ball of dia 12” lesser that the diameter of pipe but not exceeding 48” dia ball even for larger dia pipe to ensure perfect cleaning. The job including all the hire/cost of equipment and accessories of above winching drum set having, steel rope pully mounted over it steel bucket from 6” to 18” dia hooks and other protection likes safety barriers, traffic sings, traffic cones ensuring no damage to pipe along with ensuring safety to labour and other public property/lives and removal of silt/solid during desilting and clearance of site etc. complete as per full satisfaction of site engineer. …………………………….…….30” Dia………………………………. …………………………….…….24” Dia………………………………. …………………………….…….18” Dia………………………………. …………………………….…….15” Dia………………………………. …………………………….…….12” Dia………………………………. | 1,100 Rft 1,100 Rft 1,100 Rft 1,100 Rft 1,100 Rft | 257.60 209.30 128.80 96.60 80.50 | P/Rft P/Rft P/Rft P/Rft P/Rft | 2,83,360.00 2,30,230.00 1,41,680.00 1,06,260.00 88,550.00 |
| 2. | Supply of Labour on Hiring Basis for lifting the Garbage and cleaning of Nallah/Sewerage System (whenever, wherever, as much as required) and as per direction of Engineer Incharge. | 135 Nos |  | Each |  |
| Total Estimated Cost |  |  |  |  | 8,50,080 (+) OPEN RATE |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

*********************************

I/We hereby quoted Rates @                                    %  above/below the S/R

(+) Open Rate Rs:_________________ for complete work/job.

The total amount Rs:	 (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

NIT No: 01/2016-17, WORK No: 10

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:	      9,93,657/-		Earnest Money Rs:	         19,873/-

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | REHABILITATION OF C.C. DRAIN QASIM SHAH NALLAH IN UC-44, KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Dismantling cement concrete R.C.C. (1:2:4) disposal lead upto 10 miles etc. | 880 Cft | 5445.00 | %Cft | 47,916.00 |
| 2. | Excavation for wet silt clearance etc complete. | 39,000 Cft | 5142.50 | 1000 Cft | 2,04,458.00 |
| 3. | Cartage of 100 Cft / 05 Tons of all materials like stone aggregate, spawl, coal, lime, surkhi etc B.G. rail fastening point and crossing bridge girders, pipes sheet rails M.S. bar etc or 1000 number bricks 10”x5”x3” or 1000 manuds of fuel wood by trucks or any other means owned by the contractors (lead upto 15 miles etc). | 29,250 Cft | 1065.04 | %Cft | 3,11,524.00 |
| 4. | R.C.C. working roof slab beams, column, rafts, lintels and other structural members laid in situ or pre-cast laid in position complete in all respect 1:2:4. | 880 Cft | 309.78 | P/Cft | 2,72,606.00 |
| 5. | Fabrication of mild steel reinforcement for cement concrete including cutting, bending, laying in position making joints and fastenings including cost of binding wire (also includes removal of rust from bars) using tor bars. | 31.42 Cwt | 5001.70 | P/Cwt | 1,57,153.00 |
| Total Estimated Cost |  |  |  |  | 9,93,657.00 |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

*********************************

I/We hereby quoted Rates @                           %  above/below the S/R

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

NIT No: 01/2016-17, WORK No: 11

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:	      9,90,092/-		Earnest Money Rs:	         19,802/-

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | P/L R.C.C. PIPE DRAIN YAQOOB MANDA CHOWK UC-44, BHUTTA VILLAGE KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Excavation for pipe line in trenches and pits in all kind of soil of murum i/c trimming and dressing side to true alignment and shape traveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guard light flags and temporary crossing for non vehicular traffic wherever required left upto 05’ and lead upto 01 chain. | 9,750 Cft | 4650.00 | 1000 Cft | 45,338.00 |
| 2. | A. PROVIDING, LAYING & JOINTING (Rubber Gasket, JOINTS). Providing, laying R.C.C. pipes of A.S.T.M. C-76-62 T/C: 76-70 of class II wall B and fixing in trench i/c cutting, fitting and jointing with rubber ring i/c testing with water to specified pressure. ……………………………..…12” Dia………………….…………….. | 1300 Rft | 412.00 | P/Rft | 5,35,600.00 |
| 3. | Providing manhole 4’ dia (inside) and 5’ clear depth complete with 21” dia ( inside ) C.I frame & 21” dia of 3” multiply 3 x 8, flat iron frame for R.C.C 1:2:4 pre-cast cover i/c the cost of M.S bars reinforcement and two handles of 5 clear depth C.C 1:2:4 cast in Situ walls with 50% screened graded bajri of 9” thick i/c benching and channel ½” thick cement plaster 1:3 on all inside walls surface complete with c.i Foot rest i/c cutting asphalt roads excavation in all kind of soil, back filling and disposal of surplus stuff etc. | 50 Nos | 9307.86 | Each | 4,65,393.00 |
| 4. | Add/Less for extra depth beyond 5’ depth, less than 5’ depth deduct at the same rate. | ( - ) 50 Rft | 1609.15 | P/Rft | ( - ) 80,458.00 |
| 5. | Refilling the excavated stuff in trenches 6” thick layer i/c watering ramming to full compaction etc complete. | 8,775 Cft | 2760.00 | 1000 Cft | 24,219.00 |
| Total Estimated Cost |  |  |  |  | 9,90,092.00 |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

*********************************

I/We hereby quoted Rates @                           %  above/below the S/R

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

NIT No: 01/2016-17, WORK No: 12

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:	      9,99,080/-		Earnest Money Rs:	         19,982/-

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | REHABILITATION OF C.C. DRAIN SAEEDIA MASJID NALLAH IN UC-44, KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Dismantling cement concrete R.C.C. (1:2:4) disposal lead upto 10 miles etc. | 890 Cft | 5445.00 | %Cft | 48,461.00 |
| 2. | Excavation for wet silt clearance etc complete. | 39,000 Cft | 5142.50 | 1000 Cft | 2,04,458.00 |
| 3. | Cartage of 100 Cft / 05 Tons of all materials like stone aggregate, spawl, coal, lime, surkhi etc B.G. rail fastening point and crossing bridge girders, pipes sheet rails M.S. bar etc or 1000 number bricks 10”x5”x3” or 1000 manuds of fuel wood by trucks or any other means owned by the contractors (lead upto 15 miles etc). | 29,250 Cft | 1065.04 | %Cft | 3,11,524.00 |
| 4. | R.C.C. working roof slab beams, column, rafts, lintels and other structural members laid in situ or pre-cast laid in position complete in all respect 1:2:4. | 890 Cft | 309.78 | P/Cft | 2,75,704.00 |
| 5. | Fabrication of mild steel reinforcement for cement concrete including cutting, bending, laying in position making joints and fastenings including cost of binding wire (also includes removal of rust from bars) using tor bars. | 31.78 Cwt | 5001.70 | P/Cwt | 1,58,933.00 |
| Total Estimated Cost |  |  |  |  | 9,99,080.00 |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

*********************************

I/We hereby quoted Rates @                           %  above/below the S/R

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

NIT No: 01/2016-17, WORK No: 13

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:	      9,98,613/-		Earnest Money Rs:	         19,972/-

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | REPAIR OF FOOTPATH AT PAKISTAN MARINE ACADEMY IN MAURIPUR UC-40, KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Preparing sub-grade i/c earth excavate and filling to an average depth of 9” dressing to required cash and consolidation with power roll. | 10,000 Cft | 526.28 | %Cft | 52,628.00 |
| 2. | Cement concrete i/c placing compacting finishing and curing complete i/c screening and washing of stone screening and stone aggregate without shedding (1:4:8). | 3,300 Cft | 11288.75 | %Cft | 3,72,529.00 |
| 3. | Providing and laying 2” thick topping and concrete (1:2:4) i/c surface finishing and dividing into panels. | 10,000 Sft | 3275.50 | %Sft | 3,27,550/- |
| 4. | Manufacturing and supplying R.C.C. manhole covers casting c.c. 1:2:4 ratio 3 inch deep at center bar, reinforced with 3/8” dia tour steel 2.50” deep M.S. plate other curing, stacking and transportation within 10 miles. …………...……….…21” Dia …..……………….……. | 134 Nos | 913.63 | Each | 1,22,426.00 |
| 5. | Manufacturing and supplying of R.C.C. ring slab of 21” inside 36” outside 7.5” width and 6” thick i/c 3/8” for steel bars two concentric ring with 3/8” 08 Nos. cross linked bars welded and two sunk type hooks casted etc. | 50 Nos | 1906.60 | Each | 95,330.00 |
| 6. | Shifting and fixing R.C.C. precast ring slab in C.C. 1:2:4 ratio. | 50 Nos | 563.00 | Each | 28,150.00 |
| Total Estimated Cost |  |  |  |  | 9,98,613.00 |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

I/We hereby quoted Rates @                           %  above/below the S/R

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

NIT No: 01/2016-17, WORK No: 14

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:	      9,99,080/-		Earnest Money Rs:	         19,982/-

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | REPAIR/CLEANING AND DESILTING OF MAIN NALLAH OF UMER KHAN MOHALLAH IN UC-45, KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Dismantling cement concrete R.C.C. (1:2:4) disposal lead upto 10 miles etc. | 890 Cft | 5445.00 | %Cft | 48,461.00 |
| 2. | Excavation for wet silt clearance etc complete. | 39,000 Cft | 5142.50 | 1000 Cft | 2,04,458.00 |
| 3. | Cartage of 100 Cft / 05 Tons of all materials like stone aggregate, spawl, coal, lime, surkhi etc B.G. rail fastening point and crossing bridge girders, pipes sheet rails M.S. bar etc or 1000 number bricks 10”x5”x3” or 1000 manuds of fuel wood by trucks or any other means owned by the contractors (lead upto 15 miles etc). | 29,250 Cft | 1065.04 | %Cft | 3,11,524.00 |
| 4. | R.C.C. working roof slab beams, column, rafts, lintels and other structural members laid in situ or pre-cast laid in position complete in all respect 1:2:4. | 890 Cft | 309.78 | P/Cft | 2,75,704.00 |
| 5. | Fabrication of mild steel reinforcement for cement concrete including cutting, bending, laying in position making joints and fastenings including cost of binding wire (also includes removal of rust from bars) using tor bars. | 31.78 Cwt | 5001.70 | P/Cwt | 1,58,933.00 |
| Total Estimated Cost |  |  |  |  | 9,99,080.00 |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

I/We hereby quoted Rates @                           %  above/below the S/R

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

NIT No: 01/2016-17, WORK No: 15

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:	      9,99,080/-		Earnest Money Rs:	         19,982/-

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | REPAIR/CLEANING AND DESILTING OF NALLAH FROM NOOR ULLAH MASJID TO SEA IN UC-45, KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Dismantling cement concrete R.C.C. (1:2:4) disposal lead upto 10 miles etc. | 890 Cft | 5445.00 | %Cft | 48,461.00 |
| 2. | Excavation for wet silt clearance etc complete. | 39,000 Cft | 5142.50 | 1000 Cft | 2,04,458.00 |
| 3. | Cartage of 100 Cft / 05 Tons of all materials like stone aggregate, spawl, coal, lime, surkhi etc B.G. rail fastening point and crossing bridge girders, pipes sheet rails M.S. bar etc or 1000 number bricks 10”x5”x3” or 1000 manuds of fuel wood by trucks or any other means owned by the contractors (lead upto 15 miles etc). | 29,250 Cft | 1065.04 | %Cft | 3,11,524.00 |
| 4. | R.C.C. working roof slab beams, column, rafts, lintels and other structural members laid in situ or pre-cast laid in position complete in all respect 1:2:4. | 890 Cft | 309.78 | P/Cft | 2,75,704.00 |
| 5. | Fabrication of mild steel reinforcement for cement concrete including cutting, bending, laying in position making joints and fastenings including cost of binding wire (also includes removal of rust from bars) using tor bars. | 31.78 Cwt | 5001.70 | P/Cwt | 1,58,933.00 |
| Total Estimated Cost |  |  |  |  | 9,99,080.00 |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

I/We hereby quoted Rates @                           %  above/below the S/R

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

NIT No: 01/2016-17, WORK No: 16

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:	      9,95,381/-		Earnest Money Rs:	         19,908/-

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | PATCH REPAIR OF SPARCO ROAD IN MOACH GOTH KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Earth work excavation of road side nails i/c forming its banks all putting the same on embankment as directed cold breaking dressing etc. | 3282 Cft | 1437.00 | 1000 Cft | 4,716.00 |
| 2. | Providing and laying aggregate base course in proper grade and camber having CBR 80% as per AASHTO standard specifications i/c spreading and compacting by approved mechanical means (Motor grader, Vibratory roller and Smooth wheel roller etc) watering to maintain the moisture content the compaction of each layer shall 100 percent to the max dry density (rate i/c all cost of materials T&P and carriage upto 3 chains). | 4718 Cft | 5542.24 | %Cft | 2,61,483.00 |
| 3. | Providing and laying in single layer of 3 cm compacted thickness wearing course as per job mix formula Asphalt (60/70 grade) 4.5% of total mix (use of natural sand in original form is strictly prohibited formula prior by virtue of tack of prime coat to make un slippery bondage compacted by tandem roller at a density of 100% of the maximum dry density determined according to AASHTO-T-180 method D (Modified) Asphalt concrete wearing course shall be 1000 Kg (minimum) on stability and 20% maximum) on loss of stability at Marshal test criteria. All the material and methodology shall fully comply to the tests and specification of AASHTO and as directed by the Engineer Incharge (Rates include cost of material, labour and tests T&P all leads and lifts). | 9690 Sft | 6488.32 | %Sft | 6,28,718.00 |
| 4. | Kerb Stone. Providing and fixing precast edge block 3750 PSI Industrial Made size 6 inches thick x 12 inches long x 1½” high including the cost of cartage, excavation, form work for hunching 1450 PSI lean concrete 2250 PSI concrete for hunching 1:4 cement sand mortor. | 250 Rft | 297.01 | P/Rft | 74,253.00 |
| 5. | Cartage of 100 Cft / 05 Tons of all materials like stone aggregate, spawl, coal, lime, surkhi etc B.G. rail fastening point and crossing bridge girders, pipes sheet rails M.S. bar etc or 1000 number bricks 10”x5”x3” or 1000 manuds of fuel wood by trucks or any other means owned by the contractors (lead upto 15 miles etc). | 2461 Cft | 1065.04 | %Cft | 26,211.00 |
| Total Estimated Cost |  |  |  |  | 9,95,381.00 |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

I/We hereby quoted Rates @                           %  above/below the S/R

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

NIT No: 01/2016-17, WORK No: 17

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:	      9,97,791/-		Earnest Money Rs:	         19,956/-

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | P/L R.C.C. PIPE DRAIN AT TANGA STAND IN UC-02, SHER SHAH KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Excavation for pipe line in trenches and pits in all kind of soil of murum i/c trimming and dressing side to true alignment and shape traveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guard light flags and temporary crossing for non vehicular traffic wherever required left upto 05’ and lead upto 01 chain. | 9750 Cft | 4650.00 | 1000 Cft | 45,338.00 |
| 2. | A. PROVIDING, LAYING & JOINTING (Rubber Gasket, JOINTS). Providing, laying R.C.C. pipes of A.S.T.M. C-76-62 T/C: 76-70 of class II wall B and fixing in trench i/c cutting, fitting and jointing with rubber ring i/c testing with water to specified pressure. ……………………………..…15” Dia………………….…………….. | 1040 Rft | 515.00 | P/Rft | 5,35,600.00 |
| 3. | Providing manhole 4’ dia (inside) and 5’ clear depth complete with 21” dia ( inside ) C.I frame & 21” dia of 3” multiply 3 x 8, flat iron frame for R.C.C 1:2:4 pre-cast cover i/c the cost of M.S bars reinforcement and two handles of 5 clear depth C.C 1:2:4 cast in Situ walls with 50% screened graded bajri of 9” thick i/c benching and channel ½” thick cement plaster 1:3 on all inside walls surface complete with c.i Foot rest i/c cutting asphalt roads excavation in all kind of soil, back filling and disposal of surplus stuff etc. | 51 Nos | 9307.86 | Each | 4,74,701.00 |
| 4. | Add/Less for extra depth beyond 5’ depth, less than 5’ depth deduct at the same rate. | ( + ) 51 Rft | 1609.15 | P/Rft | ( + ) 82,067.00 |
| 5. | Refilling the excavated stuff in trenches 6” thick layer i/c watering ramming to full compaction etc complete. | 8775 Cft | 2760.00 | 1000 Cft | 24,219.00 |
| Total Estimated Cost |  |  |  |  | 9,97,791.00 |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

I/We hereby quoted Rates @                           %  above/below the S/R

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

NIT No: 01/2016-17, WORK No: 18

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:	      9,96,033/-		Earnest Money Rs:	         19,921/-

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | CONSTRUCTION OF NALLAH BADALA JAMAT KHANA BHUTTA VILLAGE UC-44, KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Excavation for foundation of building brigs and other structure i/c dressing refilling, around structure with excavate earth watering and ramming lead upto 05 feet. | 7,875 Cft | 3176.25 | 1000 Cft | 25,013.00 |
| 2. | Cement concrete brick or stone ballast 1½” to 2” gauge (1:4:8). | 788 Cft | 9416.28 | %Cft | 74,200.00 |
| 3. | Erection and removal of centering for R.C.C. or plain cement concrete works of Deodar Wood (2nd class). | 4,200 Sft | 3127.41 | %Sft | 1,31,351.00 |
| 4. | Cement concrete plain including placing compaction finishing and curing complete including screening and washing at stone aggregate without ratio 1:2:4. | 2,100 Cft | 14429.25 | %Cft | 3,03,014.00 |
| 5. | Fabrication of mild steel reinforcement for cement concrete i/c cutting, bending, layer in position making joints and fastening i/c cost of binding wire etc complete. | 21.10 Cwt | 5001.70 | P/Cwt | 1,05,536.00 |
| 6. | RCC work in roof slab, beams columns rafts lintels and other structural members laid in situ or precast laid in position complete in all respects (ratio 1:2:4). | 788 Cft | 337.00 | P/Cft | 2,65,556.00 |
| 7. | Manufacturing and supplying R.C.C. manhole covers casting c.c. 1:2:4 ratio 3 inch deep at center bar, reinforced with 3/8” dia tour steel 2.50” deep M.S. plate other curing, stacking and transportation within 10 miles. …………...………………….…21” Dia …..……………….……….... | 100 Nos | 913.63 | Each | 91,363.00 |
| Total Estimated Cost |  |  |  |  | 9,96,033.00 |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

I/We hereby quoted Rates @                           %  above/below the S/R

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

NIT No: 01/2016-17, WORK No: 19

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:	      9,97,138/-		Earnest Money Rs:	         19,943/-

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | IMPROVEMENT OF ROAD INFRONT OF BADALA JAMAT KHANA BHUTTA VILLAGE UC-44, KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Earth work excavation of road side nails i/c forming its banks all putting the same on embankment as directed cold breaking dressing etc. | 3282 Cft | 1437.00 | 1000 Cft | 4,716.00 |
| 2. | Providing and laying aggregate base course in proper grade and camber having CBR 80% as per AASHTO standard specifications i/c spreading and compacting by approved mechanical means (Motor grader, Vibratory roller and Smooth wheel roller etc) watering to maintain the moisture content the compaction of each layer shall 100 percent to the max dry density (rate i/c all cost of materials T&P and carriage upto 3 chains). | 4718 Cft | 5542.24 | %Cft | 2,61,483.00 |
| 3. | Providing and laying in single layer of 3 cm compacted thickness wearing course as per job mix formula Asphalt (60/70 grade) 4.5% of total mix (use of natural sand in original form is strictly prohibited formula prior by virtue of tack of prime coat to make un slippery bondage compacted by tandem roller at a density of 100% of the maximum dry density determined according to AASHTO-T-180 method D (Modified) Asphalt concrete wearing course shall be 1000 Kg (minimum) on stability and 20% maximum) on loss of stability at Marshal test criteria. All the material and methodology shall fully comply to the tests and specification of AASHTO and as directed by the Engineer Incharge (Rates include cost of material, labour and tests T&P all leads and lifts). | 9690 Sft | 6488.32 | %Sft | 6,28,718.00 |
| 4. | Kerb Stone. Providing and fixing precast edge block 3750 PSI Industrial Made size 6 inches thick x 12 inches long x 1½” high including the cost of cartage, excavation, form work for hunching 1450 PSI lean concrete 2250 PSI concrete for hunching 1:4 cement sand mortor. | 250 Rft | 297.01 | P/Rft | 74,253.00 |
| 5. | Cartage of 100 Cft / 05 Tons of all materials like stone aggregate, spawl, coal, lime, surkhi etc B.G. rail fastening point and crossing bridge girders, pipes sheet rails M.S. bar etc or 1000 number bricks 10”x5”x3” or 1000 manuds of fuel wood by trucks or any other means owned by the contractors (lead upto 15 miles etc). | 2626 Cft | 1065.04 | %Cft | 27,968.00 |
| Total Estimated Cost |  |  |  |  | 9,97,138.00 |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

I/We hereby quoted Rates @                           %  above/below the S/R

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

NIT No: 01/2016-17, WORK No: 20

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:	  8,30,285 (+) O/R		Earnest Money Rs:	16,606 (+) 2% of O/R

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | P/L AGGREGATE BASE COURSE I/C SEWERAGE SYSTEM IN MACHER COLONY, MAURIPUR AND SHER SHAH KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Excavation for pipe line in trenches and pits in all kind of soil of murum i/c trimming and dressing side to true alignment and shape traveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guard light flags and temporary crossing for non vehicular traffic wherever required left upto 05’ and lead upto 01 chain. | 750 Cft | 4650.00 | 1000 Cft | 3,488.00 |
| 2. | A. PROVIDING, LAYING & JOINTING (Rubber Gasket, JOINTS). Providing, laying R.C.C. pipes of A.S.T.M. C-76-62 T/C: 76-70 of class II wall B and fixing in trench i/c cutting, fitting and jointing with rubber ring i/c testing with water to specified pressure. ……………………………..…12” Dia………………….…………….. | 100 Rft | 412.00 | P/Rft | 41,200.00 |
| 3. | Providing manhole 4’ dia (inside) and 5’ clear depth complete with 21” dia ( inside ) C.I frame & 21” dia of 3” multiply 3 x 8, flat iron frame for R.C.C 1:2:4 pre-cast cover i/c the cost of M.S bars reinforcement and two handles of 5 clear depth C.C 1:2:4 cast in Situ walls with 50% screened graded bajri of 9” thick i/c benching and channel ½” thick cement plaster 1:3 on all inside walls surface complete with c.i Foot rest i/c cutting asphalt roads excavation in all kind of soil, back filling and disposal of surplus stuff etc. | 04 Nos | 9307.86 | Each | 37,231.00 |
| 4. | Add/Less for extra depth beyond 5’ depth, less than 5’ depth deduct at the same rate. | ( - ) 04 Rft | 1609.15 | P/Rft | ( - ) 6,437.00 |
| 5. | Refilling the excavated stuff in trenches 6” thick layer i/c watering ramming to full compaction etc complete. | 675 Cft | 2760.00 | 1000 Cft | 1,863.00 |
| 6. | Cleaning inside sewerage lines completely restoring original silt free space/diameter of pipe lines by labour and equipment. (Mechanical/electrically driven) including accessories like pulley’s steel rope, bucket and daggers sizing from 6” to18” dia. Their to and from pulling action (no of passes shall be as many as required) would be taken in two phase (First phase entire length from downstream to upstream) bringing down peack hours sewerage to flow inside pipe line ( as free flow) and in second phase from upstream to downstream of entire length ensuring no silt is observed in buckets (6” dia to 18” dia) except only sewage water and finally passing a steel ball of dia 12” lesser that the diameter of pipe but not exceeding 48” dia ball even for larger dia pipe to ensure perfect cleaning. The job including all the hire/cost of equipment and accessories of above winching drum set having, steel rope pully mounted over it steel bucket from 6” to 18” dia hooks and other protection likes safety barriers, traffic sings, traffic cones ensuring no damage to pipe along with ensuring safety to labour and other public property/lives and removal of silt/solid during desilting and clearance of site etc. complete as per full satisfaction of site engineer. …………………………….…….12” Dia………………………………. | 2,000 Rft | 80.50 | P/Rft | 1,61,000.00 |
| 7. | Providing and laying aggregate base course in proper grade and camber having CBR 80% as per AASHTO standard specifications i/c spreading and compacting by approved mechanical means (Motor grader, Vibratory roller and Smooth wheel roller etc) watering to maintain the moisture content the compaction of each layer shall 100 percent to the max dry density (rate i/c all cost of materials T&P and carriage upto 3 chains). | 8,000 Cft | 5542.24 | %Cft | 4,43,379.00 |

Cont’d Page No:02

Page No:02

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 8. | Supply of Labour on Hiring Basis for lifting the Garbage and cleaning of Nallah/Sewerage System (whenever, wherever, as much as required) and as per direction of Engineer Incharge. | 180 Nos |  | Each |  |
| 9. | Manufacturing and supplying R.C.C. manhole covers casting c.c. 1:2:4 ratio 3 inch deep at center bar, reinforced with 3/8” dia tour steel 2.50” deep M.S. plate other curing, stacking and transportation within 10 miles. ………………………………...…21” Dia………………….………... | 100 Nos | 913.63 | Each | 91,363.00 |
| 10. | Manufacturing and supplying of R.C.C. ring slab of 21” inside 36” outside 7.5” width and 6” thick i/c 3/8” for steel bars two concentric ring with 3/8” 08 Nos. cross linked bars welded and two sunk type hooks casted etc. | 30 Nos | 1906.60 | Each | 57,198.00 |
| Total Estimated Cost |  |  |  |  | 8,30,285 (+) OPEN RATE |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

I/We hereby quoted Rates @                                    %  above/below the S/R

(+) Open Rate Rs:_________________ for complete work/job.

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address: