PUBLIC HEALTH ENGINEERING DEPARTMENT

GOVERNMENT OF SINDH

PUBLIC HEALTH ENGINEERING DIVISION TANDO MUHAMMAD KHAN

BIDDING DOCUMENTS WITH FINANCIAL PROPOSAL

FOR

NAME OF WORK:-	Construction of Disposal Work i/c Collecting Tank 10’ dia, Screening	Chamber, Pump	House	10’x12’,

Compound Wall, Pumping Machinery, PVC Rising Main 4” dia & Oxidartion Pond for Rural Drainage Scheme Nabi Bux Leghari District Tando                         Muhammad Khan

| Name of Department: | Public Health Engineering Department. |
| --- | --- |
| Name of Procuring Agency: | Executive Engineer, Public Health Engg: Division Tando Muhammad Khan |

Document  issued to:-  ____________________________________________________________

TENDER FOR THE WORK

Issue to M/s_______________________Tender fee for Rs.___________________ Recieved Vide.___________________ D.R No.__________Dated:_____________ _______________________________ Dated of opening of Tender_____________

EXECUTIVE ENGINEER

PUBLIC HEALTH ENGG:DIVN:

TANDO MUHAMMAD KHAN

I/We hereby tender the execution for the Construction of Surface Drains Type A & B i/c C.C. Block for Rural Drainage Scheme Nabi Bux Leghari District Tando Muhammad Khan under written memorandum within the specified time in such memorandum at.

Percent ABOVE/BELOW the Estimate rated entered in the Schedule “B” Memorandum showing items of work to be carried out and in all respect with the specification designed and instructions writing referred to in Rule thereof.

a).	General Description: Drainage Schemes Nabi Bux Leghari

b).	Estimated Cost:				Rs: 4.839 Millions

c).	Earnest Money 2% Call Deposit:		Rs: 0.097

g).	Time of Completion				6 Months

Should be this tender be accepted I/We hereby agree to abide be the fulfill all the terms and provisions of the conditions of contract annexed hereto so as applicable and in default hereof to forfeit any pay the__________ the sum of money mentioned in the said conditions.

Receipt No.___________________ Dated:___________ from the Government Treasury / Sub-Treasury / ____________________ in respect of the sum of the Rs._____________________ is her with forwarded representing the earnest money

a).	The full value of which is to be absolutely forfeited to PHE Division Tando Muhammad Khan deposited the full amount of Security Deposit specified in the above memorandum in accordance with Clause of the said conditions otherwise the sum of Rs.____________ shall be remained by the PHE Division Tando Muhammad Khan on account of such Security Deposit specified in relevant Clause of the conditions/agreement.

N.B 	The Tender is required to undertake that all terms and conditions of the printed B-I Forms inclusive of the additional conditions etc approved by the Executive Engineer PHE Division Tando Muhammad Khan will be binding on him and these will be includes and signed by him in the even acceptance of his tender.

|  | EXECUTIVE ENGINEER PUBLIC HEALTH ENGG: DIVISION TANDO MUHAMMAD KHAN |
| --- | --- |

| Bill Of QUANTITY ( B )DESCRIPTION AND RATE OF ITEMS BASED ON COMPOSITE SCHEDULE OF RATES. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Na Name Of Work:- |  |  |  |  |  |  |  | CONSTRUCTION OF DISPOSAL WORKS I/C COLLECTING TANK 10' DIA,SCREENING CHAMBER,PUMP HOUSE 10'X12',COMPOUND WALL,PUMPING MACHINERY,PVC RISING MAIN 4" DIA & OXIDATION PONDS FOR RURAL DRAINAGE SCHEME NABI BUX LEGHARI DISTT: TANDO MUHAMMAD KHAN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| S: # |  |  | ITEM |  |  |  |  |  |  |  |  |  |  |  |  | QUANTITY |  |  |  | RATE |  |  |  | UNIT | AMOUNT |  |  |  |  |
|  |  |  | GROUP:R.C.C COLLECTING TANK 10' DIA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | Excavation of well in dry upto 20ft below ground level and disposal of soil with in one chain. (GSI No. 1, P/92). 0'to5' depth |  |  |  |  |  |  |  |  |  |  |  |  | 613.28 Cft |  |  |  | Rs:-2247/58 |  |  |  | P.%0 Cft | Rs:- 1378/- |  |  |  |  |
| 2 |  |  | R.C.C work I/c all labour & material except the cost of steel reinforcement & its labour of binding bending which will be paid separately. This rate also I/c all kind of forms mould lifting shuttering curing rendering and washing of shinmgle(a) R.C work in roof slab beams columns rafts lintel & other structure members laid in situ or prcast laid in position complete in all respect Ratio1:2:4 90 Lbs cement 2 Cft sand 4Cft shingle 1/8" to 1/4" gauge.(G.S.I.No:6 P/17) |  |  |  |  |  |  |  |  |  |  |  |  | 639.0 Cft |  |  |  | Rs:-337/- |  |  |  | P. Cft | Rs:- 215343/- |  |  |  |  |
| 3 |  |  | Fabrication of mild steel reinforcement for C.C I/c cutting bending laying position making & jointing I/c cost of bending laying in position making fastening I/c cost of binding wire also I/c removal of rust from bars.(G.S.I.No: 7 (ii) P/19). |  |  |  |  |  |  |  |  |  |  |  |  | 24.33 Cwt |  |  |  | Rs:-5001/70 |  |  |  | P. Cwt | Rs:- 121691/- |  |  |  |  |
| 4 |  |  | Wet stinkingof well for depth below spring level by means of devers including all chages for shoring loading and removing excavated material within one chain. (GSINo. 3, P-93) (a) 0'to5' depth |  |  |  |  |  |  |  |  |  |  |  |  | 613.28 Cft |  |  |  | Rs:-11763/80 |  |  |  | P.%0 Cft | Rs:- 7215/- |  |  |  |  |
| 5 |  |  | 5'to10' depth |  |  |  |  |  |  |  |  |  |  |  |  | 613.28 Cft |  |  |  | Rs:-24200/- |  |  |  | P.%0 Cft | Rs:- 14841/- |  |  |  |  |
| 6 |  |  | Bailing or pumping out sub soil water during excavation, concreting, cost insitu concrete or masonary work in foundation etc. (PHSINo, 1 8, P-52) . |  |  |  |  |  |  |  |  |  |  |  |  | 628.0 Cft |  |  |  | Rs:-543/- |  |  |  | P.% Cft | Rs:- 3410/- |  |  |  |  |
| 7 |  |  | Cement concrete brick or stone ballast 11/2" to 2" gauge ratio 1:4:8.(G.S.I.No:4 P/16). |  |  |  |  |  |  |  |  |  |  |  |  | 160.33 Cft |  |  |  | Rs:-9416/28 |  |  |  | P.% Cft | Rs:- 15097/- |  |  |  |  |
| 8 |  |  | Cement concrete plain I/c placing compacting, finishing and curing (I/c acreening washing of stone aggregate without shuttering .(G.S.I.No: 5 (f) P/17). Ratio 1:3:6 |  |  |  |  |  |  |  |  |  |  |  |  | 78.50Cft |  |  |  | Rs:-12595/- |  |  |  | P.%Cft | Rs:- 9887/- |  |  |  |  |
| 9 |  |  | Ratio 1:2:4 |  |  |  |  |  |  |  |  |  |  |  |  | 78.50Cft |  |  |  | Rs:-14429/25 |  |  |  | P.%Cft | Rs:- 11327/- |  |  |  |  |
| 10 |  |  | Cement plaster 1:4 upto 12' height 1/2" thick . (G.S.I.No: 11 (b) P/57). |  |  |  |  |  |  |  |  |  |  |  |  | 578.0 Sft |  |  |  | Rs:-2283/93 |  |  |  | P.% Sft | Rs:- 13201/- |  |  |  |  |
| 11 |  |  | Supply of padlo. (SM INO 45 b P/2) |  |  |  |  |  |  |  |  |  |  |  |  | 14.73 Lbs |  |  |  | Rs:-56/81 |  |  |  | P. Lbs | Rs:- 837/- |  |  |  |  |
| 12 |  |  | Providing and fixing in portion C.C. foundation 1:3:6 M.S ladder of M.S angle iron 2" x 2" 1' - 0" apart with M.S bars 3/2" dia approved 1' -0" apart painting three coats with anlterosive steel paint etc complete.(P.H.S.I. No: 3 P/42). |  |  |  |  |  |  |  |  |  |  |  |  | 15.0 Rft |  |  |  | Rs:-799/- |  |  |  | P. Rft | Rs:- 11985/- |  |  |  |  |
| 13 |  |  | Providing constrution joint sperate of 9"(225m) side corregated PVC waterstop(with bulb)I/c soldering cost of material and labour etc complete.(PHSI No:2 P/43) |  |  |  |  |  |  |  |  |  |  |  |  | 103.69 Rft |  |  |  | Rs:-84/10 |  |  |  | P. Rft | Rs:- 8917/- |  |  |  |  |
|  |  |  | GROUP:R.C.C SCREENING CHAMBER (5'X8') |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | Excavation in foundation of building bridges and other structure i/c dagbelling dressing, filling around the structure with excavated earth wateringand ramming lead upto 5' ft. (b) in ordinary soil) (GSI No. 18(b), P/4). 0' to 5' depth |  |  |  |  |  |  |  |  |  |  |  |  | 440.0 Cft |  |  |  | Rs:-3176/25 |  |  |  | P.%0 Cft | Rs:- 1398/- |  |  |  |  |
|  |  |  | 5' to 8' depth |  |  |  |  |  |  |  |  |  |  |  |  | 88.0 Cft |  |  |  | Rs:-3277/03 |  |  |  | P.%0 Cft | Rs:- 288/- |  |  |  |  |
| 2 |  |  | Extra for wet earth work (G.S.I No15 P/3 |  |  |  |  |  |  |  |  |  |  |  |  | 88.0 Cft |  |  |  | Rs:-1058/75 |  |  |  | P.% 0 Cft | Rs:- 93/- |  |  |  |  |
| 3 |  |  | Cement concrete brick or stone ballast 11/2" to 2" gauge ratio 1:4:8.(G.S.I.No:4 P/16). |  |  |  |  |  |  |  |  |  |  |  |  | 44.0 Cft |  |  |  | Rs:-9416/28 |  |  |  | P.% Cft | Rs:- 4143/- |  |  |  |  |
| 4 |  |  | Cement concrete plain I/c placing compacting, finishing and curing (I/c acreening washing of stone aggregate without shuttering .(G.S.I.No: 5 (f) P/17). Ratio 1:2:4 |  |  |  |  |  |  |  |  |  |  |  |  | 20.0 Cft |  |  |  | Rs:-14429/25 |  |  |  | P.%Cft | Rs:- 2886/- |  |  |  |  |
| 5 |  |  | R.C.C work I/c all labour & material except the cost of steel reinforcement & its labour of binding bending which will be paid separately. This rate also I/c all kind of forms mould lifting shuttering curing rendering and washing of shinmgle(a) R.C work in roof slab beams columns rafts lintel & other structure members laid in situ or prcast laid in position complete in all respect Ratio1:2:4 90 Lbs cement 2 Cft sand 4Cft shingle 1/8" to 1/4" gauge.(G.S.I.No:6 P/17) |  |  |  |  |  |  |  |  |  |  |  |  | 217.0 Cft |  |  |  | Rs:-337/- |  |  |  | P. Cft | Rs:- 73129/- |  |  |  |  |
| 6 |  |  | Fabrication of mild steel reinforcement for C.C I/c cutting bending laying position making & jointing I/c cost of bending laying in position making fastening I/c cost of binding wire also I/c removal of rust from bars.(G.S.I.No: 7 (ii) P/19). |  |  |  |  |  |  |  |  |  |  |  |  | 8.72 Cwt |  |  |  | Rs:-5001/70 |  |  |  | P. Cwt | Rs:- 43615/- |  |  |  |  |
| 7 |  |  | Pucca brik work in foundation and plinth in cement sand morter 1:6.(G.S.I.No: 4(I) P/3. |  |  |  |  |  |  |  |  |  |  |  |  | 18.05 Cft |  |  |  | Rs:-11948/36 |  |  |  | P.% Cft | Rs:- 2157/- |  |  |  |  |
| 8 |  |  | Cement plaster 1:4 upto 12' height 1/2" thick . (G.S.I.No: 11 (b) P/57). |  |  |  |  |  |  |  |  |  |  |  |  | 260.65 Sft |  |  |  | Rs:-2283/93 |  |  |  | P.% Sft | Rs:- 5953/- |  |  |  |  |
| 9 |  |  | Supply of padlo. (SM INO 45 b P/2) |  |  |  |  |  |  |  |  |  |  |  |  | 6.65 Lbs |  |  |  | Rs:-56/81 |  |  |  | P. Lbs | Rs:- 378/- |  |  |  |  |
| 10 |  |  | Providing constrution joint sperate of 9"(225m) side corregated PVC waterstop(with bulb)I/c soldering cost of material and labour etc complete.(PHSI No:2 P/43) |  |  |  |  |  |  |  |  |  |  |  |  | 31.0 Rft |  |  |  | Rs:-86/- |  |  |  | P. Rft | Rs:- 2666/- |  |  |  |  |
| 11 |  |  | Fabrication of heavy steel work with angles,tees,flat iron,round iron and sheet iron for making trusses, girder,tanks etc,I/C cutting rivetting,handling assembling and fixing but excluding erection in position. (G.S.I.No: 2 P/91).(for screening jali) |  |  |  |  |  |  |  |  |  |  |  |  | 2.01 Cwt |  |  |  | Rs:-4928/49 |  |  |  | P. Cwt | Rs:- 9901/- |  |  |  |  |
| 12 |  |  | Erection & Fitting in position of Iron trusses staging of water tanks etc (G.S.I.No: 3 P/91). |  |  |  |  |  |  |  |  |  |  |  |  | 2.01 Cwt |  |  |  | Rs:-271/04 |  |  |  | P. Cwt | Rs:- 545/- |  |  |  |  |
| 13 |  |  | Providing/fixing C.I manhole covers with frame i/c cost of material ( PHSI No 1P/35) |  |  |  |  |  |  |  |  |  |  |  |  | 1.75 Cwt |  |  |  | Rs:-6985/- |  |  |  | P. Cwt | Rs:- 12224/- |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | TOTAL |  |  | Rs:-159376/- |  |  |  |
|  |  |  | GROUP: INTER CONNECTION. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | Excavation for pipe lines in trenches and pits in soft soils I/c trimming and dressing sides to true alignment and shap levelling of beds of trenches in correct level and grade cutting joints holes and surplus earth within a oone chain as directed by Engineer incharge providing fence guards, light, flages and temporary crossing fpr non vehicular traffic where ever required upto 5ft (1.52m) and lead upto one chain 30.50m).( P.H.S.I.No: 1 P/60) |  |  |  |  |  |  |  |  |  |  |  |  | 320.0 Cft |  |  |  | Rs:-3600/- |  |  |  | P.%0 Cft | Rs:- 1152/- |  |  |  |  |
| 2 |  |  | Providing R.C.C. Pipe of ASTM C - 76 - 70 class II wall B and fixing in trenches I/c cutting fitting and jointing with Rubber rings I/c testing with water to specified pressure.(P.H.E.S.I.No: (B) 1 P/18). |  |  |  |  |  |  |  |  |  |  |  |  | 16.0 Rft |  |  |  | Rs:-412/- |  |  |  | P.Rft | Rs:- 6592/- |  |  |  |  |
| 3 |  |  | Refilling the excavated stuff in trenches 6" thick layer I/c watering ramming to full compacting etc complete. (P.H.S.I.No: 24 P/53). |  |  |  |  |  |  |  |  |  |  |  |  | 288.0 Cft |  |  |  | Rs:-2760/- |  |  |  | P.%0 Cft | Rs:- 795/- |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | TOTAL | Rs:-8539/- |  |  |  |  |
|  |  |  | GROUP: PUMP HOUSE (10'X12') |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | Excavation in foundation of building bridges and other structure i/c dagbelling dressing, filling around the structure with excavated earth wateringand ramming lead upto 5' ft. (b) in ordinary soil) (GSI No. 18(b), P/4). 0' to 5' depth |  |  |  |  |  |  |  |  |  |  |  |  | 141.0 Cft |  |  |  | Rs:-3176/25 |  |  |  | P.%0 Cft | Rs:- 448/- |  |  |  |  |
| 2 |  |  | Cement concrete brick or stone ballast 11/2" to 2" gauge ratio 1:4:8.(G.S.I.No:4 P/16). |  |  |  |  |  |  |  |  |  |  |  |  | 73.0 Cft |  |  |  | Rs:-9416/28 |  |  |  | P.% Cft | Rs:- 6874/- |  |  |  |  |
| 3 |  |  | Erection &Removal of centering for R.C.C or plain cement concrete work of Partal wood (Second class ) vertical. (Partal wood) (G.S.I.No: 19 P/18). |  |  |  |  |  |  |  |  |  |  |  |  | 376.0 Sft |  |  |  | Rs:-3127/41 |  |  |  | P.% Sft | Rs:- 11759/- |  |  |  |  |
| 4 |  |  | Cement concrete plain I/c placing compacting, finishing and curing (I/c acreening washing of stone aggregate without shuttering .(G.S.I.No: 5 (f) P/17). Ratio 1:3:6 |  |  |  |  |  |  |  |  |  |  |  |  | 188.0 Cft |  |  |  | Rs:-12595/- |  |  |  | P.%Cft | Rs:- 23679/- |  |  |  |  |
| 5 |  |  | Pucca brik work in foundation and plinth in cement sand morter 1:6.(G.S.I.No: 4(I) P/3. |  |  |  |  |  |  |  |  |  |  |  |  | 298.50 Cft |  |  |  | Rs:-12674/36 |  |  |  | P.% Cft | Rs:- 37833/- |  |  |  |  |
| 6 |  |  | R.C.C work I/c all labour & material except the cost of steel reinforcement & its labour of binding bending which will be paid separately. This rate also I/c all kind of forms mould lifting shuttering curing rendering and washing of shinmgle(a) R.C work in roof slab beams columns rafts lintel & other structure members laid in situ or prcast laid in position complete in all respect Ratio1:2:4 90 Lbs cement 2 Cft sand 4Cft shingle 1/8" to 1/4" gauge.(G.S.I.No:6 P/17) |  |  |  |  |  |  |  |  |  |  |  |  | 106.11 Cft |  |  |  | Rs:-337/- |  |  |  | P. Cft | Rs:- 35759/- |  |  |  |  |
| 7 |  |  | Fabrication of mild steel reinforcement for C.C I/c cutting bending laying position making & jointing I/c cost of bending laying in position making fastening I/c cost of binding wire also I/c removal of rust from bars.(G.S.I.No: 7 (ii) P/19). |  |  |  |  |  |  |  |  |  |  |  |  | 4.26 Cwt |  |  |  | Rs:-5001/70 |  |  |  | P. Cwt | Rs:- 21307/- |  |  |  |  |
| 8 |  |  | Filling watering & ramining earth in floor with surples earth from foundation lead upto one chain & lift upto 5'ft(GSI No.21 P/4). |  |  |  |  |  |  |  |  |  |  |  |  | 286.40 Cft |  |  |  | Rs:-1512/20 |  |  |  | P.%0 Cft | Rs:- 433/- |  |  |  |  |
| 9 |  |  | Making & fixing steel grated door with 1/2" thick shuttring i/c angle iron frame 2"x2"x1/8" and 3/4"sq bars.(G.S.I No 24 P/92). |  |  |  |  |  |  |  |  |  |  |  |  | 22.75 Sft |  |  |  | Rs:-726/72 |  |  |  | P. Sft | Rs:- 16533/- |  |  |  |  |
| 10 |  |  | Cement plaster 1:4 upto 12' height 1/2" thick .(G.S.I.No: 11 (b) P/57). |  |  |  |  |  |  |  |  |  |  |  |  | 972.25 Sft |  |  |  | Rs:-2283/93 |  |  |  | P.% Sft | Rs:- 22206/- |  |  |  |  |
| 11 |  |  | Providing 2' thick toping c.c 1:2:4 surface finishing and deviding into penals. (G.S.I No 16 P/42). |  |  |  |  |  |  |  |  |  |  |  |  | 120.0 Sft |  |  |  | Rs:-3275/50 |  |  |  | P.% Sft | Rs:- 3931/- |  |  |  |  |
| 12 |  |  | White wash 03 coat.(G.S.I.No26(a) P/59 ) |  |  |  |  |  |  |  |  |  |  |  |  | 204.0 Sft |  |  |  | Rs:-829/95 |  |  |  | P.% Sft | Rs:- 1693/- |  |  |  |  |
| 13 |  |  | Distempering Three Coats.(G.S.I.No:24 P/60 ). |  |  |  |  |  |  |  |  |  |  |  |  | 972.25 Sft |  |  |  | Rs:-1079/65 |  |  |  | P.% Sft | Rs:- 10497/- |  |  |  |  |
| 14 |  |  | Fabrication of heavy steel work with angles,tees,flat iron,round iron and sheet iron for making trusses, girder,tanks etc,I/C cutting rivetting,handling assembling and fixing but excluding erection in position. (G.S.I.No: 2 P/91).(for screening jali) |  |  |  |  |  |  |  |  |  |  |  |  | 2.54 Cwt |  |  |  | Rs:-4928/49 |  |  |  | P. Cwt | Rs:- 12518/- |  |  |  |  |
| 15 |  |  | Erection & Fitting in position of Iron trusses staging of water tanks etc (G.S.I.No: 3 P/91). |  |  |  |  |  |  |  |  |  |  |  |  | 2.54 Cwt |  |  |  | Rs:-271/04 |  |  |  | P. Cwt | Rs:- 688/- |  |  |  |  |
| 16 |  |  | PAINTING NEW SURFACE. Preparation face and painting of doors & window any type (I/c edges) two subsequent cost overprimary coat theree coats. (G.S.I.No: 5 P/70 ). |  |  |  |  |  |  |  |  |  |  |  |  | 106.0 Sft |  |  |  | Rs:-2116/41 |  |  |  | P. Sft | Rs:- 2243/- |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | TOTAL | Rs:-208401/- |  |  |  |  |
|  |  |  | Cost of 02 Nos Pump House (208401 x2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rs:-416802/- |  |  |  |  |
|  |  |  | GROUP: COMPOUND WALL (100'X75') |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | Excavation in foundation of building bridges and other structure i/c dagbelling dressing, filling around the structure with excavated earth wateringand ramming lead upto 5' ft. (b) in ordinary soil) (GSI No. 18(b), P/4). 0' to 5' depth |  |  |  |  |  |  |  |  |  |  |  |  | 1050.0 Cft |  |  |  | Rs:-3176/25 |  |  |  | P.%0 Cft | Rs:- 3335/- |  |  |  |  |
| 2 |  |  | Cement concrete brick or stone ballast 11/2" to 2" gauge ratio 1:4:8.(G.S.I.No:4 P/16). |  |  |  |  |  |  |  |  |  |  |  |  | 260.25 Cft |  |  |  | Rs:-9416/28 |  |  |  | P.% Cft | Rs:- 24482/- |  |  |  |  |
| 3 |  |  | Erection &Removal of centering for R.C.C or plain cement concrete work of Partal wood (Second class ) vertical. (Partal wood) (G.S.I.No: 19 P/18). |  |  |  |  |  |  |  |  |  |  |  |  | 2317.0 Sft |  |  |  | Rs:-3127/41 |  |  |  | P.% Sft | Rs:- 72462/- |  |  |  |  |
| 4 |  |  | Cement concrete plain I/c placing compacting, finishing and curing (I/c acreening washing of stone aggregate without shuttering .(G.S.I.No: 5 (f) P/17). Ratio 1:3:6 |  |  |  |  |  |  |  |  |  |  |  |  | 1159.0 Cft |  |  |  | Rs:-12595/- |  |  |  | P.%Cft | Rs:- 145976/- |  |  |  |  |
| 5 |  |  | Pucca brik work in foundation and plinth in cement sand morter 1:6.(G.S.I.No: 4(I) P/3. |  |  |  |  |  |  |  |  |  |  |  |  | 553.0 Cft |  |  |  | Rs:-12346/65 |  |  |  | P.% Cft | Rs:- 68277/- |  |  |  |  |
| 6 |  |  | R.C.C work I/c all labour & material except the cost of steel reinforcement & its labour of binding bending which will be paid separately. This rate also I/c all kind of forms mould lifting shuttering curing rendering and washing of shinmgle(a) R.C work in roof slab beams columns rafts lintel & other structure members laid in situ or prcast laid in position complete in all respect Ratio1:2:4 90 Lbs cement 2 Cft sand 4Cft shingle 1/8" to 1/4" gauge.(G.S.I.No:6 P/17) |  |  |  |  |  |  |  |  |  |  |  |  | 60.0 Cft |  |  |  | Rs:-337/- |  |  |  | P. Cft | Rs:- 20220/- |  |  |  |  |
| 7 |  |  | Fabrication of mild steel reinforcement for C.C I/c cutting bending laying position making & jointing I/c cost of bending laying in position making fastening I/c cost of binding wire also I/c removal of rust from bars.(G.S.I.No: 7 (ii) P/19). |  |  |  |  |  |  |  |  |  |  |  |  | 2.41 Cwt |  |  |  | Rs:-5001/70 |  |  |  | P. Cwt | Rs:- 12054/- |  |  |  |  |
| 8 |  |  | Cement plaster 1:6 upto 12' height 1/2" thick .(G.S.I.No: 11 (b) P/57). |  |  |  |  |  |  |  |  |  |  |  |  | 1786.0 Sft |  |  |  | Rs:-2206/60 |  |  |  | P.% Sft | Rs:- 39410/- |  |  |  |  |
| 9 |  |  | Cement plaster 1:4 upto 12' height 3/8" thick .(G.S.I.No: 11 (b) P/57). |  |  |  |  |  |  |  |  |  |  |  |  | 1786.0 Sft |  |  |  | Rs:-2197/52 |  |  |  | P.% Sft | Rs:- 39248/- |  |  |  |  |
| 10 |  |  | Cement pointing of struck on wall 1:3 .(G.S.I.No: 19 P/53). |  |  |  |  |  |  |  |  |  |  |  |  | 298.0 Sft |  |  |  | Rs:-1213/58 |  |  |  | P.% Sft | Rs:- 3616/- |  |  |  |  |
| 11 |  |  | Making & fixing steel grated door with 1/2" thick shuttring i/c angle iron frame 2"x2"x1/8" and 3/4"sq bars. (G.S.I No 24 P/92). |  |  |  |  |  |  |  |  |  |  |  |  | 72.0 Sft |  |  |  | Rs:-726/72 |  |  |  | P. Sft | Rs:- 52324/- |  |  |  |  |
| 12 |  |  | PAINTING NEW SURFACE. Preparation face and painting of doors & window any type (I/c edges) two subsequent cost overprimary coat theree coats. (G.S.I.No: 5 P/70 ). |  |  |  |  |  |  |  |  |  |  |  |  | 144.0 Sft |  |  |  | Rs:-1489/68 |  |  |  | P.% Sft | Rs:- 2145/- |  |  |  |  |
| 13 |  |  | White wash 01 coat.(G.S.I.No26(a) P/59) |  |  |  |  |  |  |  |  |  |  |  |  | 2084.0 Sft |  |  |  | Rs:-416/63 |  |  |  | P.% Sft | Rs:- 8683/- |  |  |  |  |
| 14 |  |  | Colour wash 02 coat.(G.S.I.No25 P/54 ) |  |  |  |  |  |  |  |  |  |  |  |  | 2084.0 Sft |  |  |  | Rs:-859/90 |  |  |  | P.% Sft | Rs:- 17920/- |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | TOTAL | Rs:-510152/- |  |  |  |  |
|  |  |  | GROUP: PUMPING MACHINERY 5.50 BHP(04 SETS) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | Supplying and installing in position i/c transportation to site of work at site of work electric pumping set consisting of A.C electric motor of 5.50 BHP 3 Phase 50 Cycles 400/420 volts of KWPZ/960 RPM ( 65 -315 ) size ( 3"x2.1/2") mounted on common steel frame or base plate capable of discharging 100 gallons per minute against a head of 50ft. i/c installation of pumping set on C.C foundation ratio L.2.4 with M.S and bolts of 5/8" dia to be ambaded in c.c foundation i/c providing and fixing on prepared board A.C 50 cycles "phase 12-25 amps trip reting push hotton type star delta starter complete with no volts and over load release and required i/c providing & fixing on prepared board iron (clad) triple pole and neutral switch fuse with rewire able type fuse link carriage 500 volts grade amps meter 0,50 amps installed on metailed board i/c providing and fixing metalled board to accommodate main or sub mains control switchs recessed in the wall size 3' x2.5" i/c providing phase indication bulbs with holder and providing internal electric wiring for mains with 3-7/0.83 (7/0.036) PVC insulated wire 660 volts grade in 25 mm (1") PVC conduit on surface as required i/c providing and fixing earthing set with 0.6 m 0.6 mm (2'x2'l/4") G.I plate buried in the ground at depth 3.7 m (12' ft) or less as per PAK PWD specification and testing of the pumping set against the required head for 72 hours etc: complete. |  |  |  |  |  |  |  |  |  |  |  |  | 2.0 Sets |  |  |  | Rs:-724320/- |  |  |  | P.Set | Rs:-1448640/- |  |  |  |  |
| 2 |  |  | Making / installing M.S pipe made out M.S sheet 3/16" thick cutting, bending,folding to correct curvature and shape i/c welding,etc complete (S.MI P/10) 3"dia |  |  |  |  |  |  |  |  |  |  |  |  | 36.0 Rft |  |  |  | Rs:-385/89 |  |  |  | P.Rft | Rs:-13892/- |  |  |  |  |
|  |  |  | 4" Dia |  |  |  |  |  |  |  |  |  |  |  |  | 21.0 Rft |  |  |  | Rs:-506/60 |  |  |  | P.Rft | Rs:-10639/- |  |  |  |  |
| 3 |  |  | Making / installing M.S flanges made out M.S sheet 3/16" thick cutting, bending,folding to correct curvature and shape i/cwelding,etc complete 3"dia |  |  |  |  |  |  |  |  |  |  |  |  | 8.0 Nos |  |  |  | Rs:-500/89 |  |  |  | Each | Rs:-4007/- |  |  |  |  |
|  |  |  | 4" Dia |  |  |  |  |  |  |  |  |  |  |  |  | 6.0 Nos |  |  |  | Rs:-596/75 |  |  |  | Each | Rs:-3581/- |  |  |  |  |
| 4 |  |  | Reflax valve heavy patterib test pressure 300 lbs/sq inch: (S.M.I.No: 6 P/11) 4"dia |  |  |  |  |  |  |  |  |  |  |  |  | 2.0 Nos |  |  |  | Rs:-1706/25 |  |  |  | Each | Rs:-3413/- |  |  |  |  |
| 5 |  |  | Foot valve heavy pattern w/cone type gate.(S.M.I.No: 4 p/93). 3"dia |  |  |  |  |  |  |  |  |  |  |  |  | 2.0 Nos |  |  |  | Rs:-731/25 |  |  |  | Each | Rs:-1463/- |  |  |  |  |
| 6 |  |  | C.I Sluice valve heavy pattern (Test pressure 300 lbs.(P.H.S.No: 2 p/10). 4"dia |  |  |  |  |  |  |  |  |  |  |  |  | 2.0 Nos |  |  |  | Rs:-5460/- |  |  |  | Each | Rs:-10920/- |  |  |  |  |
| 7 |  |  | Supplyin g C.I. Bend with flanged ends with holes I/c truning facing for all size. (S.R.M.I No: 8 P/12)6"dia |  |  |  |  |  |  |  |  |  |  |  |  | 42.01 Kg |  |  |  | Rs:-120/- |  |  |  | P.Kg | Rs:-5041/- |  |  |  |  |
| 8 |  |  | Supplying C.I. Tee flanged ends with holes I/c turning and facing of flanged for all size. S.M.I.No: 11 P/12)6"dia |  |  |  |  |  |  |  |  |  |  |  |  | 11.45 Kg |  |  |  | Rs:-120/- |  |  |  | P.Kg | Rs:-1374/- |  |  |  |  |
| 9 |  |  | Supplying C.I. Tappers 2 1/2 x 3 flate bottomed or central tappered flanged ends with holes, I/c turning and facing turning and of flange for all size.(S.M.I.No: 9 P/11). |  |  |  |  |  |  |  |  |  |  |  |  | 10.50 Kg |  |  |  | Rs:-120/- |  |  |  | P.Kg | Rs:-1260/- |  |  |  |  |
| 10 |  |  | Jointing C.I/M.S flanged piped & special flanged & inside trenches I/c supplying rubber packing of the required thickness nuts & bolts testing etc & bolts testing etc & other tools requirinng for jointing and testing the joints to the specified pressire etc complete (P.H.S.I.No: MP P/40). 3"dia 4"dia |  |  |  |  |  |  |  |  |  |  |  |  | 16.0 Joints 8.0 Joints |  |  |  | Rs:-499/- Rs:-513/- |  |  |  | P.Joint P.Joint | Rs:-7984/- Rs:-4104/- |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | TOTAL | Rs:-1516318/- |  |  |  |  |
|  |  |  | Cost of 04 Pumping Sets (1516318x2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rs:-3032636/- |  |  |  |  |
|  |  |  | GROUP: P.V.C RISING MAIN 4" DIA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | Excavation for pipe lines in trenches and pits in soft soils I/c trimming and dressing sides to true alignment and shap levelling of beds of trenches in correct level and grade cutting joints holes and surplus earth within a oone chain as directed by Engineer incharge providing fence guards, light, flages and temporary crossing fpr non vehicular traffic where ever required upto 5ft (1.52m) and lead upto one chain 30.50m).( P.H.S.I.No: 1 P/45 |  |  |  |  |  |  |  |  |  |  |  |  | 12950.0 Cft |  |  |  | Rs:-3600/- |  |  |  | P.%0 Cft | Rs:- 46620/- |  |  |  |  |
| 2 |  |  | Providing u P.V.C pipes of class "B" (equivalent make) fixing in trench I/c cutting fitting and jointing with Z joint with one rubber rings I/c testing with water to a head of 61 meter or 200 ft (P.H.S INO 1/E P/22) |  |  |  |  |  |  |  |  |  |  |  |  | 1850.0 Rft |  |  |  | Rs:-137/- |  |  |  | P.Rft | Rs:- 253450/- |  |  |  |  |
| 3 |  |  | Supply of CI spcials for P.V.C pressure pipe for "B" class C.I bend 90o.(S.M.I. No: 1 P/12). 4" DIA |  |  |  |  |  |  |  |  |  |  |  |  | 3.0 Nos |  |  |  | Rs:-893/75 |  |  |  | Each | Rs:-2681/- |  |  |  |  |
| 4 |  |  | C.I bend 45o.(S.M.I. No: 2 P/12). 4" DIA |  |  |  |  |  |  |  |  |  |  |  |  | 4.0 Nos |  |  |  | Rs:-731/25 |  |  |  | Each | Rs:-2885/- |  |  |  |  |
| 5 |  |  | C.I. short piece. (S.M.I.No: 9 p/15). 4" DIA |  |  |  |  |  |  |  |  |  |  |  |  | 2.0 Nos |  |  |  | Rs:-487.50 |  |  |  | Each | Rs:-975/- |  |  |  |  |
| 6 |  |  | C.I. Fire Hydrent Tee. (S.M.I.No: 10 p/16) 4" DIA |  |  |  |  |  |  |  |  |  |  |  |  | 2.0 Nos |  |  |  | Rs:-1112/50 |  |  |  | Each | Rs:-2225/- |  |  |  |  |
| 7 |  |  | C.I. Air valve double acting (3"x1"). (S.M.I.No: 8 p/12). |  |  |  |  |  |  |  |  |  |  |  |  | 2.0 Nos |  |  |  | Rs:-4000/- |  |  |  | Each | Rs:-8000/- |  |  |  |  |
| 8 |  |  | Supplyin g C.I. Bend with flanged ends with holes I/c truning and facing for all size.(S.R.M.I No: 8 P/12) |  |  |  |  |  |  |  |  |  |  |  |  | 19.10 Kg |  |  |  | Rs:-120/- |  |  |  | P.Kg | Rs:-2292/- |  |  |  |  |
| 9 |  |  | Making / installing M.S pipe made out M.S sheet 3/16" thick cutting, bending,folding to correct curvature and shape i/c welding,etc complete (as per RA approved by CE PHED Hyd) Based on sch rates 4" DIA |  |  |  |  |  |  |  |  |  |  |  |  | 30.0 Rft |  |  |  | Rs:-506/60 |  |  |  | P.Rft | Rs:-10132/- |  |  |  |  |
| 10 |  |  | Providing and fixing M.S flanges made out M.S plate having thikness and total weight as mfntioned against each item It includes the cost of making holes facing and shape welding nut bolts rubber pakings white lead fitter charges etc complete (Sch of MS Spetials I No 8 P/8 ) |  |  |  |  |  |  |  |  |  |  |  |  | 6.0 Nos |  |  |  | Rs:-596/75 |  |  |  | Each | Rs:-3581/- |  |  |  |  |
| 11 |  |  | Jointing C.I/M.S flanged pipe & special flanged & inside trenches I/c supplying rubber packing of the required thickness nuts & bolts testing etc & bolts testing etc & other tools requirinng for jointing and testing the joints to the specified pressire etc complete (P.H.S.I.No: MP P/40). |  |  |  |  |  |  |  |  |  |  |  |  | 2.0 Joints 6.0 Joints |  |  |  | Rs:-499/- Rs:-513/- |  |  |  | P.Joint P.Joint | Rs:-998/- Rs:-3078/- |  |  |  |  |
| 12 |  |  | Making Joints to P.V.C Special fitting (i/c laying specials and cost of solvent cement)of the required diameter and testing the joints along with the P.V.C pipe line "B" "C"and "D" class to the specified pressure and making good to all leaky joints etc complete. (P.H.S.I.No:N1 (a) P/41). |  |  |  |  |  |  |  |  |  |  |  |  | 19.0 Joints |  |  |  | Rs:-70/- |  |  |  | P.Joint | Rs:-1330/- |  |  |  |  |
| 13 |  |  | Providing chamber 3.1/2x2.1/2(11064x764 mm) inside dimension 7 ' (2.13m) deep as per approved design for sluice valve 15" to24" dia with 24" (610mm) dia inside cost iron cover and frame (wt=2 cwt 3qr) fixed in RCC 1:2:4 slab 6" thick(with 5Lbs steel per cft) 9" (299mm)thick brick mansonary wall set in1:6 cement motor 6" thick C.C 1:3:6 infoundation 1" (25mm) thick cement concrete 1:2:4 flooring 1/2" (12.5mm) thick cement plaster 1:3 to all inside wall surface and to top i/cproviding and fixing M.S foot rest at every one foot beyond 2 1/2 ft depth curing excavation back filling and disposal of surplus earth etc complete.(P.H.S.I.No: 3P/58). |  |  |  |  |  |  |  |  |  |  |  |  | 2.0 Nos |  |  |  | Rs:-28691/- |  |  |  | Each | Rs:-57382/- |  |  |  |  |
| 14 |  |  | Refilling the excavated stuff in trenches 6" thick layer I/c watering ramming to full compacting etc complete. (P.H.S.I.No: 24 P/53). |  |  |  |  |  |  |  |  |  |  |  |  | 11655.0 Cft |  |  |  | Rs:-2760/- |  |  |  | P.%0 Cft | Rs:- 32168/- |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | TOTAL | Rs:- 427797/- |  |  |  |  |
|  |  |  | GROUP: OXIDATION PONDS 03 NOS (75'X45') |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | Jungle clearance and removing trees within 100 feet (GSI No:4 P/95) |  |  |  |  |  |  |  |  |  |  |  |  | 32500.0 Sft |  |  |  | Rs:-75/63 |  |  |  | P.% Sft | Rs:- 24580/- |  |  |  |  |
| 2 |  |  | Excavation for tanks ana reservoirs in soft soils I/c trimming and dressing sides to true alignment design section/profile and shap levelling of beds of tanks to correct level and grade i/c laying of earth in 6"layers for construction of banks and dressing and disposal of surplus earth within a oone chain as directed by Engineer incharge providing fence guards, light, flages where ever required upto 5ft (1.52m) and lead upto one chain 30.50m). ( P.H.S.I.No: 1 P/67 |  |  |  |  |  |  |  |  |  |  |  |  | 31428.36.0 Cft |  |  |  | Rs:-3000/- |  |  |  | P.%0 Cft | Rs:- 94285/- |  |  |  |  |
| 3 |  |  | Excavation for tanks ana reservoirs in wet soils mud or clay I/c trimming and dressing sides to true alignment design section/profile and shap levelling of beds of tanks to correct level and grade i/c laying of earth in 6"layers for construction of banks and dressing and disposal of surplus earth within a oone chain as directed by Engineer incharge providing fence guards, light, flages where ever required upto 5ft (1.52m) and lead upto one chain 30.50m). ( P.H.S.I.No: 14 P/74 |  |  |  |  |  |  |  |  |  |  |  |  | 3128.24.0 Cft |  |  |  | Rs:-4800/- |  |  |  | P.%0 Cft | Rs:- 15016/- |  |  |  |  |
| 4 |  |  | Earth work compection (soft ordinary or hard soil) laying earth in 6" layer levelling & dressing etc complete (b) ordinary Soil.(G.S.I.No: 13 (a) P/3. |  |  |  |  |  |  |  |  |  |  |  |  | 34363.50 Cft |  |  |  | Rs:-354/- |  |  |  | P.%0 Cft | Rs:- 12165/- |  |  |  |  |
| 5 |  |  | Providing R.C.C. Pipe of ASTM C - 76 - 70 class II wall B and fixing in trenches I/c cutting fitting and jointing with Rubber rings I/c testing with water to specified pressure.(P.H.E.S.I.No: (B) 1 P/18). |  |  |  |  |  |  |  |  |  |  |  |  | 96.0 Rft |  |  |  | Rs:-618/- |  |  |  | P.Rft | Rs:-59328/- |  |  |  |  |
| 6 |  |  | Cement concrete brick or stone ballast 11/2" to 2" gauge ratio 1:4:8.(G.S.I.No:4 P/16). |  |  |  |  |  |  |  |  |  |  |  |  | 50.24 Cft |  |  |  | Rs:-9416/28 |  |  |  | P.% Cft | Rs:- 4731/- |  |  |  |  |
| 7 |  |  | R.C.C work I/c all labour & material except the cost of steel reinforcement & its labour of binding bending which will be paid separately. This rate also I/c all kind of forms mould lifting shuttering curing rendering and washing of shinmgle(a) R.C work in roof slab beams columns rafts lintel & other structure members laid in situ or prcast laid in position complete in all respect Ratio1:2:4 90 Lbs cement 2 Cft sand 4Cft shingle 1/8" to 1/4" gauge.(G.S.I.No:6 P/17) |  |  |  |  |  |  |  |  |  |  |  |  | 236.39 Cft |  |  |  | Rs:-337/- |  |  |  | P. Cft | Rs:- 79663/- |  |  |  |  |
| 8 |  |  | Fabrication of mild steel reinforcement for C.C I/c cutting bending laying position making & jointing I/c cost of bending laying in position making fastening I/c cost of binding wire also I/c removal of rust from bars.(G.S.I.No: 7 (ii) P/18). |  |  |  |  |  |  |  |  |  |  |  |  | 9.49 Cwt |  |  |  | Rs:-5001/70 |  |  |  | P. Cwt | Rs:- 47466/- |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | TOTAL | Rs:- 337234/- |  |  |  |  |
| ABSTRACT OF COST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | GROUP R.C.C COLLECTING 10' DIA |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rs: |  |  |  | 435,129.00 |  |  |  |  |  |  |  |  |  |
|  | GROUP R.C.C SCREENING CHAMBER (5'X8') |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rs: |  |  |  | 159,376.00 |  |  |  |  |  |  |  |  |  |
|  | GROUP INTER CONNECTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rs: |  |  |  | 8,539.00 |  |  |  |  |  |  |  |  |  |
|  | GROUP PUMP HOUSE (10'X12') 02 Nos |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rs: |  |  |  | 416,802.00 |  |  |  |  |  |  |  |  |  |
|  | GROUP COMPOUND WALL |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rs: |  |  |  | 510,152.00 |  |  |  |  |  |  |  |  |  |
|  | GROUP PUMPING MACHINERY (5.50 BHP 04 Sets) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rs: |  |  |  | 3,032,636.00 |  |  |  |  |  |  |  |  |  |
|  | GROUP P.V.C RISING MAIN 4"DIA |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rs: |  |  |  | 427,797.00 |  |  |  |  |  |  |  |  |  |
|  | GROUP OXIDATION PONDS |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rs: |  |  |  | 337,234.00 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | TOTAL |  |  |  | Rs: |  |  |  | 5,327,665.00 |  |  |  |  |  |  |  |  |  |
|  | Total Rs:- 5327665/- |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | ________%Above/Below on the rates of CSR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | Amount to be added/deducted on the basis Of premium quoted_______________Total |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Total ( A ) Rs:- _____________________________________________________ _____________________________________________________ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CONDITION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  |  |  | Any error provision in description of item,qty and unit will be governed by relevent schdule of rates and relevent rates analysis |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 |  |  |  | The decision of the Superintending Engineer will be install and biding and all the parties in any shape. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  |  |  | The rate should be inclusive of all taxes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 |  |  |  | No separate caring will be paid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 5 |  |  |  | No premium will be allowed on Non-schedule item |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CONTRACTOR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | EXECUTIVE ENGINEER PUBLIC HEALTH ENGG: DIVISION TANDO MUHAMMAD KHAN |  |  |  |  |  |  |  |  |  |  |  |  |  |