Construction of First Floor of Category 2nd Bungalow for District Officer Social Welfare Tharparkar converted into Annexi Building. (i)(Int: W/S, S/F & Ext W/S & Drainage).

ABSTRACT SHEET / SCHEDULE “B”

ITEMS BASED ON COMPOSED SCHEDULE OF RATES

| QUANTITY | S.# | ITEM OF WORK |  | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- |
|  | PART “A” W/S & S/F |  |  |  |  |  |
| 2-Nos | 01 | Providing and fixing squatting type white glazed earthen were W.C pan with including the cost of flushing cistern with internal fitting and flush pipe with bend & making requisite number of holes in walls plinth & floor for pipe connections & making good in cement concrete 1:2:4 (B) W.C of not less than 19” clear opening between flushing rims and 3 gallons flushing tank with 4” dia C.I trap & C.I thumble |  |  |  |  |
|  |  | (Rs. Four thousand eight hundred two Ps. Sixty only) |  | 4802/60 | Each | 9605/- |
| 2-Nos | 02 | Providing & fixing 22” x 16” lavatory basin in white glazed earthen ware complete with and i/c the cost of W.I or C.I cantilever brackets 6 inches built into walls. Painted white in two coats after a primary coats of red lead paint & pair of ½” dia rubber plug & chrome brass waste of approved pattern 1-1/4” dia malleable iron or c.p barass traps, malleable iron or brass unions and kaming requisite number of holes in walls plinth and floor for pipe connection & making good in cement concrete 1:2:4 |  |  |  |  |
|  |  | (Rs. Six thousand two hundred thirty seven only) |  | 6237/- | Each | 12474/- |
| 2-Nos | 03 | Add extra for labour for providing and fixing of earthen were pedestal white or coloured glazed standard pattern. |  |  |  |  |
|  |  | (Rs. Nine hundred thirty eight Ps. Forty seven only) |  | 938/47 | Each | 1877/- |
| 2-Nos | 04 | Providing & fixing Eureopean type white glazed earthen ware wash down w.c pan complete with & i/c cost of white / black plastic seat (Best Qty) & lid with c.p brass hinges best quality and buffers 3 gallons plastic flushing cistern with internal fitting with fitting and clamp ¾” dia and cutting & making requisite number of holes in walls, plinth & follr for Pipe connection & making good in cement concrete 1:2:4 |  |  |  |  |
|  |  | (Rs. Six thousand two hundred thirty seven only) |  | 5339/40 | Each | 10679/- |
| 10-Nos | 05 | P/F 6"x2" or 6"x3" C.I floor trap of the approved self cleaning design with screwed down gritting with or without a vent arm complete with and i/c making requisite number of holes in walls plinth in and floor for pipe condition & making good in C.C 1:2:4. |  |  |  |  |
|  |  | (Rs. Two thousand Forty two Ps. Forty three only) |  | 2042/43 | Each | 20244/- |
| 12-Nos | 06 | Providing & fixing in position nyloon connection complete with 1/2" dia dap brass stop cock with pair of brass nuts and lining joints ti nyloon connection. |  |  |  |  |
|  |  | (Rs. Four hundred forty seven Ps. Fifteen only) |  | 447/15 | Each | 5366/- |
| 5-Nos | 07 | S/Fixing cancealed stop cock of superior quality with c.p head ½” dia |  |  |  |  |
|  |  | (Rs. Four hundred seventy eight Ps. Twenty eight only) |  | 478/28 | Each | 2391/- |
| 16-Nos | 08 | S / Fixing cancealed tee – stop cock of superior quality with c.p head ½” dia |  |  |  |  |
|  |  | (Rs. Eight hundred forty three Ps. Ninety two only) |  | 843/92 | Each | 13503/- |
| 10-Nos | 09 | Supply and fixing in long bib cock of superior quality with C.P 1/2" dia . |  |  |  |  |
|  |  | (Rs. One thousand one hundred nine Ps. Forty six only) |  | 1109/46 | Each | 11095/- |
| 2-Nos | 10 | Supplying / Fixing wash basen mixture of superior quality with c.p head ½” dia |  |  |  |  |
|  |  | (Rs. Two thousand eight hundred eighty two only) |  | 2882/00 | Each | 5764/- |
| 2-No | 11 | S/F sink mixture cock of superior quality with C.P head etc complete |  |  |  |  |
|  |  | (Rs. Two thousand seven hundred forty five Ps. Sixty only) |  | 2745/60 | Each | 5491/- |
| 2-Nos | 12 | S/F Sawn type piller cock of superior quality with crystal head. |  |  |  |  |
|  |  | (Rs. Seven hundred ninety five Ps. Thirty only) |  | 795/30 | Each | 1590/- |
| 2-Nos | 13 | Supplying and fixing C.P Muslim Shower with crystal head etc complete |  |  |  |  |
|  |  | (Rs. Three thousand four hundred thirty two Only) |  | 3432/00 | Each | 6864/- |
| 2-Nos | 14 | Supp: & fixing bath room accessories (7 pieces) i/c towel rod, brush holder, soaptray shelf of approved design i/c cost of screws, nuts etc Cpmplete (Master Braqnd) |  |  |  |  |
|  |  | (Rs. Ten thousand three hundred twenty two Ps. Forty only) |  | 10322/40 | Each | 20645/- |
| 100-Rft | 15 | Providing, Laying uPVC pressure pipes of Class ‘D’ (equivalent make) fixing in trench i/c cutting, fitting and jointing with ‘Z’ joint with one rubber ring i/c testing with eater to a head 122 meter or 400 ft (f) 100 mm (4” dia) |  |  |  |  |
|  |  | (Rs. Ten thousand three hundred twenty two Ps. Forty only) |  | 226/- | P.Rft | 22600/- |
| 2-Nos | 16 | Supplying and fixing jet shower with rod of superior quality single c.p head ½” dia. |  |  |  |  |
|  |  | (Rs. One thousand one hundred forty two Ps. Twenty four only) |  | 1142/24 | Each | 2284/- |
|  |  |  |  | Total (a) |  | 152472/- |
| - (___________________% Above/Below) Amount added / Deducted |  |  |  |  |  |  |
| (Rupees:__________________________________________________________________ |  |  |  |  |  |  |
| __________________________________________________________________ Total (b) |  |  |  |  |  |  |
|  |  | (a + b) = Total “A” |  |  |  |  |

|  | ITEMS BASED ON MARKET / OFFERED RATES: |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | PART “B” NON-SCHEDULTE ITEMS |  |  |  |  |
|  | 01 | Providing & Fixing UPVC Pipe of Pak Arab |  |  |  |
| 200-Rft |  | a) 1” dia |  | P.Rft |  |
| 300-Rft |  | b) ¾” dia |  | P.Rft |  |
| 100-Rft |  | c) ½” dia |  | P.Rft |  |
| 6-Nos | 02 | Providing & Fixing UPVC Plug T of 4” dia |  |  |  |
|  |  |  |  | Each |  |
| 6-Nos | 03 | Providing & Fixing UPVC Plug bend 4” dia |  |  |  |
|  |  |  |  | Each |  |
| 2-Nos | 04 | Providing & Fixing UPVC Terminal guard 4” dia |  |  |  |
|  |  |  |  | Each |  |
| 161-Rft | 05 | Providing & Fixing Steel (S.S original) Railing for stair / Terrace with round pipe of 1 ½” dia and railing 2 ½” dia, pipes resisting of marble strips with appropriate design, embedded @ place i/c nuts bolts and polishing etc complete in all respects as per instruction of Engineer / incharge |  |  |  |
|  |  |  |  | P.Rft |  |
|  |  |  | Total “B” |  |  |

|  | ITEMS BASED ON COMPOSED SCHEDULE OF RATES |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PART “B” Buildings items: |  |  |  |  |  |
| 89-Cft | 01 | Pacca brick work in First Floor in cement mortar 1:6 |  |  |  |  |
|  |  | (Rs. Thirteen thousand one hundred twelve Ps. Ninety nine only) |  | 13112/99 | %Cft | 12572/- |
| 64-Cft | 02 | R.C.C work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rates also includes all kinds of forms moulds, lifting shuttering curing rendering & finishing the exposed surface i/c screening and washing of shingle (A) R.C work in roof slab beams columns rafts lintels and other structural members laid in situ or precast laid on position complete in all respect ration 1:2:4 90 lbs cement 2 cft, sand 4 cft, shingle 1/8" to 1/4" gauge. |  |  |  |  |
|  |  | (Rs. Three hundred thirty seven only) |  | 337/- | P.Cft | 21568/- |
| 2.88-Cwt | 03 | Fabrication of mid steel reinforcement for cement concrete including cutting bending laying in position making joints and fastening cost of binding wire also including removal of rust from bars. Tor Bar |  |  |  |  |
|  |  | (Rs. Five thousand one Ps. Seventy only) |  | 5001/70 | P.Cwt | 14405/- |
| 255-Sft | 04 | Cement plaster 1:6 upto 12’ height (b) ½” thick |  |  |  |  |
|  |  | (Rs. Two thousand two hundred six Ps. Sixty only) |  | 2206/60 | %Sft | 5627/- |
| 255-Sft | 05 | Cement plaster 1:4 upto 12’ height (a) 3/8” thick |  |  |  |  |
|  |  | (Rs. Two thousand one hundred ninety seven Ps. Fifty two only) |  | 2197/52 | %Sft | 5604/- |
|  |  |  |  | Total (a) |  | 59776/- |
| - (___________________% Above/Below) Amount added / Deducted |  |  |  |  |  |  |
| (Rupees:__________________________________________________________________ |  |  |  |  |  |  |
| __________________________________________________________________ Total (b) |  |  |  |  |  |  |
|  |  | (a + b) = Total “C” |  |  |  |  |

S U M M A R Y

COST OF BID:

| 1. | A – Cost based on composite Schedule of Rate | Rs. |
| --- | --- | --- |
| 2. | B – Cost bases on Market / Offered Rates | Rs. |
| 3. | C – Cost bases on composite Schedule of Rate | Rs. |
|  | Total Cost of Bid ( ) = | Rs. |

| Contractor |  | Executive Engineer Buildings Division Tharparkar |
| --- | --- | --- |