KARACHI WATER & SEWERAGE BOARD

OFFICE OF THE EXECUTIVE ENGINEER (SEW)

CLIFTON, DISTRICT SOUTH

SUBJECT:-	PROVIDING LAYING 30” DIA, 24” DIA, 18” DIA, 15” DIA & 12” DIA RCC WATER LINE FROM REGENT PLAZA TO JINNAH TURNING FROM IMPROVEMENT OF SEWERAGE SYSTEM JINNAH HOSPITAL AND ADJOINING AREAS OF CLIFTON / SADDAR TOWN.

| S.NO. | DESCRIPTION | QTY. | RATE |  | PER UNIT / ITEM | AMOUNT IN RUPEES |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Rupees in Figures. | Rupees in Words. |  |  |
| 1. | Scarifying the existing road surface. | 47900 Sft |  |  | % Sft |  |
| 2. | Dismantling and removing road metaling. | 47900 Cft |  |  | % Cft |  |
| 3. | Excavation for pipe line in trenches pits in all kinds of soils of murum i/c trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade, cutting joint holes and disposal of surplus earth within one chain as directed by Engineer Incharge. Providing fence guards, light, flags and temporary crossing for non vehicular traffic where required lift upto 5 ft (1.52 m) and lead upto one chain (30.5m) (i) Lift 0’ to 5’ ft | 191600 Cft |  |  | %0Cft |  |
| 4. | Add for additional lift of every three ft or part three of in item No.1 to 15 (A&B) for excavation for pipe lines and storage tanks trenches and pits. |  |  |  |  |  |
|  | Lift 5’ to 8’ Ft | 88000 Cft |  |  | %0Cft |  |
|  | Lift 8’ to 11’ Ft | 78000 Cft |  |  | %0Cft |  |
| 5. | Excavation for pipe line in trenches and pits in sandy or disintegrated rock by hammering practicable for prohibited i/c trimming and dressing sides true alignment and shape leveling of beds of trenches to correct level and grade, cutting joints holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guards, lights, flags and temporary crossing for non-vehicular a traffic where every required lift upto 5’ ft (1.52m) and lead upto one chain (30.5m). |  |  |  |  |  |
|  | Lift 5’ to 8’ Ft | 44000 Cft |  |  | %0Cft |  |
|  | Lift 8’ to 11’ Ft | 39000 Cft |  |  | %0Cft |  |

( Page NO.2 )

| S.NO. | DESCRIPTION | QTY. | RATE |  | PER UNIT / ITEM | AMOUNT IN RUPEES |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Rupees in Figures. | Rupees in Words. |  |  |
| 6. | Full hire charges of the pumping set per day inclusive of wages of driver and assistant fuel or electric energy platform required for placing pump etc at lower depth with suction and delivery pipe for pumping out water found at various depth from trenches i/c the cost of erection and dismantling after completion of the job and dismantling after completion of job of 10 HP from 10 feet depth trench from 1- feet depth. | 610 Days |  |  | P/Day |  |
| 7. | Providing / Laying RCC Pipes with (Rubber Ring Joint and Fitting and trench i/c cutting, fitting and joining with rubber ring i/c testing with water to specified pressure. |  |  |  |  |  |
|  | 30” Dia | 2000 Rft |  |  | P/Rft |  |
|  | 24” Dia | 2000 Rft |  |  | P/Rft |  |
|  | 18” Dia | 500 Rft |  |  | P/Rft |  |
|  | 15” Dia | 500 Rft |  |  | P/Rft |  |
|  | 12” Dia | 400 Rft |  |  | P/Rft |  |
| 8. | Providing manhole 4 feet internal diameter without RCC manhole covers 5 ft clean depth cast in situ 1:2:4 using 50% crush stone and 50% graded bajri 9” thick wall 1:4:8 cement concrete in 6 ft dia and 6” inch thick in foundation 1:2:4 C.C in benching ½” thick cement plastering with 1:3 cement mortar on inside wall surface of channel and benching to top including cost of excavation in all kind of soil backfilling and disposal of excavated stuff etc. complete as per design and instruction of the Engineer Incharge including C.I Frame 15 Kg. | 108 Nos. |  |  | Each |  |
| 9. | Add or deduct for extra / less depth than 5 ft of 4 ft dia manhole i/c footrest. | 570 Ft |  |  | P/Ft |  |
| 10. | Making connection with the existing manhole i/c the cost of cutting holes in wall making them good in cement concrete 1:2:4 and making to required channel etc complete. | 40 Nos. |  |  | Each |  |

(Continued on Next Page)

( Page NO.3 )

| S.NO. | DESCRIPTION | QTY. | RATE |  | PER UNIT / ITEM | AMOUNT IN RUPEES |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Rupees in Figures. | Rupees in Words. |  |  |
| 11. | Refilling the excavated stuff in trench 6” thick laying i/c watering ramming to full compaction etc complete. | 417698 Cft |  |  | %0Cft |  |
| 12. | Sand Haro or any other source of same modules of the fineness (Hill Sand). | 62654 Cft |  |  | % Cft |  |
| 13. | Carriage of 100 Cft / 5 Tons of all materials like stone, aggregate, Spawl, coal, lime, surkhi etc B.G Rail fastening points end crossing bridge, Girders, Pipes, sheets Rail, M.S Bars etc Nos. 100 Nos. titles 12” x 16” x 2’” or 150 Cft of timber 100 Maunds of fuel wood by trucks or any other means owned by the contractors. 7th Mile | 148210 Cft |  |  | % Cft |  |
| 14. | Manufacturing & Supplying of 21” dia RCC manhole cover cast in 1:2:4 concrete ratio 3” inch deep at center reinforced with ½” dia tor steel bars at 4” c/c welded to 3/16” thick 2” wide M.S plate two hook of 3/8” dia tor bars including compacting i/c curing and transportation in 10 Miles. From existing manholes. 21” Dia | 108 Nos. |  |  | Each |  |
|  | TOTAL |  |  |  |  |  |

EXECUTIVE ENGINEER (SEW) CLIFTON, DISTRICT SOUTH K.W.&.S.B.

I hereby quoted Amounting to Rs.________________ (In words) _____________________

_________________________________________________________________________.

Note :- All existing SPPRA Rule will be abide.

Signature & Stamp of the Contractor

Address ______________________

BIDDING CRITERIA

| 1. | Name of procuring Agency | K.W.&.S.B |
| --- | --- | --- |
| 2. | Name of work | PROVIDING LAYING 30” DIA, 24” DIA, 18” DIA, 15” DIA & 12” DIA RCC WATER LINE FROM REGENT PLAZA TO JINNAH TURNING FROM IMPROVEMENT OF SEWERAGE SYSTEM JINNAH HOSPITAL AND ADJOINING AREAS OF CLIFTON / SADDAR TOWN. |
| 3. | Brief Description of work | The PC-1 of above mentioned scheme for costing to Rs.21.516 Million has been framed by the XEN concerned regarding improvement of sewerage system from Jinnah Hospital and Adjoining Areas of Clifton / Saddar Town, the sewerage branch line is sunk down badly as the system is nearly 20 years old sewerage water is over flowing on the main road due to which there is a lot of difficulty for area peoples. |
| 4. | Procuring Agency Address: | OFFICE OF THE EXECUTIVE ENGINEER (SEW), Clifton, District South, KW&SB, at Boat Basin, Pumping Station Near Grammer School, Clifton, Karachi. |
| 5. | Estimate Cost. | Rs.21.516 Million. |
| 6. | Amount of Bid Security | 2% bid security of the quoted cost / price in shape of pay order / Bank Draft or Bank Guarantee from any schedule bank of Pakistan in favour of KW&SB. Bid Security must be accompanied with the Tender documents otherwise the tenders shall be treated as invalid / rejected. |
| 7. | Period of Bid Validity. | 90 days |
| 8. | Security Deposit including Bid Security. | @ 10% of Billed amount. |
| 9. | Venue, time and date of Bid Opening | The tender in sealed cover super scribed with the name of the work should be dropped in the Tender Box kept in the office of the Director (Design), KW&SB, near COD Filter Plant, Gulshan-e-Iqbal Karachi __________at 2.00 pm and will be opened on same day at 2.30 pm by Tender Opening Committee. |
| 10. | Deadline for submission of Bid with time. | 22-12-2016 at 2.00 pm |
| 11. | Time for completion. | 90 days |
| 12. | Liquidity damages. | Rs.0.50% of the Estimated Cost per day. |
| 13. | Bid issued to firm |  |
| 14. | Deposit Receipt No. and date. |  |
| 15. | Tender Cost. | Rs.3,000/= |
| 16. | Eligibility of Contractor | i) Having National Tax No. (Copy may be attached). ii) PEC valid registration E-6 & Field specialization in CE-09. iii). Three Years relevant work experience. iv). Three Years Turn Over Bank statement. v). Registration with SRB. |

EXECUTIVE ENGINEER (SEW)

CLIFTON TOWN, DIST. SOUTH

K.W.&.S.B.

AUTHORITY ISSUIGN BIDDING DOCUMENTS

-

TENDER DOCUMENTS

FOR THE WORK OF

PROVIDING LAYING 30” DIA, 24” DIA, 18” DIA, 15” DIA & 12” DIA RCC SEWER LINE FROM REGENT PLAZA TO JINNAH TURNING FROM IMPROVEMENT OF SEWERAGE SYSTEM JINNAH HOSPITAL AND ADJOINING AREAS OF CLIFTON / SADDAR TOWN.

EXECUTIVE ENGINEER (SEW)

CLIFTON, DISTRICT SOUTH

KARACHI WATER & SEWERAGE BOARD

CONTRACT DATA

Sub-Clauses of

Conditions of Contract

1.1.3 Procuring Agency’s Drawings, if any

1.14 The Procuring Agency means

Karachi Water & Sewerage Board

1.1.5 The Contractor means

To be Executed through firm.

1.1.7 Commencement Date means the date of issue of Engineer’s Notice to Commence which shall be issued within fourteen (14) days of the signing of the Contract Agreement.

1.1.9 Time for Completion 90 Days.

1.1.20 Engineer  Mr. ANWER MEMON EXECUTIVE ENGINEER (SEW), CLIFTON, DISTRICT SOUTH KW&SB.

1.3 Documents forming the contract listed in the order of priority:

- The Contract Agreement

- Letter of Acceptance.

- The completed form of bid.

- Contract Data.

- Conditions of Contract.

- The completed schedules to Bid including Schedule Prices.

- The Drawings, if any.

- The Specifications.

2.1 Provision of Site: On the Commencement Date

3.1 Authorized person : Mr. Anwer Memon, Executive Engineer (Sew), Clifton, District South, KW&SB.

3.2 Name and address of Engineer’s/Procuring Agency’s representative Mr. Anwer Memon, Executive Engineer (Sew), Boat Basin Pumping Station, Near Grammer School Boat Basin Karachi.

4.4	Performance Security:

Amount 2% of Bid Cost.

Validity 90 Days.

(Form : As provided under Standard Forms of these documents)

5.1 Requirements for Contractor’s design.

Specification Clause Nos N/A.

7.2 Programme:

Time for submission : Within fourteen (14) days of the Commencement Date.

Form of programme : __________________ (Bar chart/CPM/PERT or other).

7.4 Amount payable due to failure to complete shall be 0.05% per day upto a maximum of (10%) of sum stated in the Letter Acceptance (Usually the liquidated damages are set between 0.05 percent and 0.10 percent per day.

9.1 Period for remedying defects (90 days).

10.2 (e) Variation procedures:

Day work rates N/A (details).

11.1 Terms of Payments.

- Mobilization Advance.

- Mobilization Advance up to 10% of the Contract Price stated in the Letter of Acceptance shall be paid by the Procuring Agency to the Contractor on the works costing Rs.2.5 million or above on following conditions:

- On submission by the Contractor of a Mobilization Advance Guarantee for the full amount of the advance in the specified form from a Schedule Bank in Pakistan to the Procuring Agency.

(ii)  Contractor will pay interest on the mobilization advance at the rate of 10% per annum on the advance; and

(iii)  The Advance including the interest shall be recovered in 5 equal installments from the five (05) R.A Bills and in case the number of bills is less than five (05) then 1/5th of the advance inclusive of the interest thereon shall be recovered from each bill and the balance together with interest be recovered from the final bill. It may be insured that there is sufficient amount in the final bill to enable recovered of the Mobilization Advance.

OR

2) Secured Advance on Materials.

- The Contractor shall be entitled to receive from the Procuring Agency Secured Advance against an INDENTURE BOND in P.W Account Form No.31(Fin. R.Form No.2 acceptable to the Procuring Agency of such sum as the Engineer may consider proper in respect of non-perishable materials brought at the Site but not yet incorporated in the Permanent Works provided that:

(i)The materials are in accordance with the Specifications for the Premanent Works;

(ii)Such materials have been delivered to the Site and are properly stored and protected against loss or damage or deterioration to the satisfaction and verification of the Engineer but at the risk and cost of the Contractor;

(iii)The Contractor’s records of the requirements, orders, receipts and use of materials are kept in a form approved by the Engineer, and such records shall be available for inspection by the Engineer;

(iv)The Contractor shall submit with his monthly statement the estimated value of the materials on Site together with such documents as may be required by the Engineer for the purpose of valuation of materials and proving evidence of ownership and payment thereof;

(v)Ownership of such materials shall be deemed to vest in the Procuring Agency and these materials shall be not be removed from the Site or otherwise disposed of without written permission of the Procuring Agency;

(vi)The sum payable for such materials on Site shall not exceed 75% of the (i) landed cost of imported materials, or (ii) ex-factory / ex-warehouse price of locally manufactured or produced materials, or (iii) market price of stands other materials;

(vii) Secured Advance should not be allowed unless & until the previous advance, if any fully recovered;

(viii) Detailed account of advances must be kept in part-II of running account bill; and

(ix) Secured Advance may be permitted only against materials/quantities anticipated to be consumed / utilized on the work within a period of 3 months from the date of issue of secured advance and definitely not for full quantities of materials for the entire work/contract (b) Recovery of Secured Advance;(i) Secured Advance paid to the Contractor under the above provisions shall be effected from the monthly payments on actual consumption basis, but not later than period specified in the rules not more than three months (even if unutilized); other conditions.

(ii) As recoveries are made the outstanding accounts of the items concerned in Part-II should be reduced be making deduction entries in the column ; - deduct quantity utilized in work measured since previous bill, equivalent to the quantities of materials used by the contractor on items of work shown as executed in part-I of the bill.

(c ) Interim payments: The Contractor shall submit to the Engineer monthly statements of the estimated value of the work completed less the cumulative amount certified previously.

(i)The value of work completed comprises the value of the quantities of the items in the Bill of Quantities completed.

(ii) Value of secured advance on the materials and valuation of variations (if any).

(iii) Engineer may exclude any item certified in previous certificate or reduce the proportion of any item previously certified in any certificate in the light of later information.

(iv) Retention money and other advances are to be recovered from the bill submitted by contractor.

11.2 *(a) Valuation of the Works:

i)Lump sum price as prepaid (details) , or

ii) Lump sum price with schedules or rates ______(details), or

iii) Lump sum price with bill of quantities _______(details) , or

(iv) Re-measurement with estimated / bid quantities in the Schedule of Prices or on premium above or below quoted on the rates mentioned in CSR _______(details), or /and

v) Cost reimbursable _________(details)  (N/A).

11.3 Percentage of retention *: Five (5%).

11.6 Currency of payment: Pak. Rupees.

14.1 Insurances : (Procuring Agency may decide, keeping in view the nature and the scope of the work).

Type of cover

The Works.

Amount of cover

The sum stated in the Letter of Acceptance plus fifteen percent (15%)

Type of cover

Contractor’s Equipment:

Amount of cover

Full replacement cost

Type of cover

Third Party-injury to persons and damaged to property Through Insurance (CAR) The minimum amount of third party insurance should be assessed by the Procuring Agency and entered).

Workers:

______________________

______________________

Other cover*:

CAR.

(In each case name of insured is Contractor and Procuring Agency)

14.2 Amount to be recovered.

Premium plus 10% percent ( 10%)

15.3 Arbitration**

Place of Arbitration: Karachi.

*(Contractor having registration in PEC atleast C-6 Category)

** (1% Water charges will be recovered against the work done).

EXECUTIVE ENGINEER (SEW)

CLIFTON, DISTRICT SOUTH

K.W.&.S.B.

BIDDING DATA

(This section should be filled in by the Engineering / Procuring Agency before issuance of the Bidding Documents).

(a) Name of Procuring Agency:	KARACHI WATER & SEWERAGE BOARD.

(b) Brief Description of works:	The PC-1 of above mentioned scheme for costing to Rs.21.516 Million has been framed by the XEN concerned regarding improvement of sewerage system from Jinnah Hospital and Adjoining Areas of Clifton / Saddar Town, the sewerage branch line is sunk down badly as the system is nearly 20 years old sewerage water is over flowing on the main road due to which there is a lot of difficulty for area peoples.

(c) Procuring Agency’s address: OFFICE OF THE EXECUTIVE ENGINEER (SEW), Clifton, District  South, KW&SB, at Boat Basin, Pumping Station Near Grammer School, Clifton, Karachi.

(d) Estimated Cost:	RS.21.516 MILLION.

(E) Amount of Bid Security:	2%.

(f) Period of Bid Validity (days):	90 Days.

(g) Security Deposit: (including bid security): 10%

(h) Percentage, if any to be deducted from bills: 16.5%

(i) Deadline for submission of bids alongwith with time : 2.30 PM.

(j) Venue, Time and date of Bid Opening: The work should be dropped in the Tender Box kept in the office of the Director (Design), KW&SB, near COD Filter Plant, Gulshan-e-Iqbal Karachi 22-12-2016 at 2.00 pm and will be opened on same day at 2.30 pm by Tender Opening Committee.

(k) Time for Completion from written order of commence:-	90 Days.

(l) Liquidity damages:	0.05 % per day (0.05 of Estimated Cost or Bid cost per day of delay, but total not exceeding 10%.

(m) Deposit Receipt No: Date: Amount: ___________________________

AUTHORITY ISSUING BIDDING DOCUMENT)

KARACHI WATER & SEWERAGE BOARD

OFFICE OF THE EXECUTIVE ENGINEER (SEW)

CLIFTON, DISTRICT SOUTH

Boat Basin Pumping Station near Grammer School Boat Basin, Karachi.

Cell # 0300-2636342

NOTICE FOR INVITING TENDER THROUGH  PRESS

( On Offer Rate Basis ).

Estimated Cost Rs.21.516 Million

Sealed tenders are invited under SPP Rules-2010 for the work mentioned below :-

| 1. | Name of work :- | PROVIDING, LAYING 30” DIA, 24” DIA, 18” DIA, 15” DIA & 12” DIA RCC SEWER LINE FROM REGENT PLAZA TO JINNAH TURNING FROM IMPROVEMENT OF SEWERAGE SYSTEM JINNAH HOSPITAL AND ADJOINING AREAS OF CLIFTON / SADDAR TOWN. |
| --- | --- | --- |
| 2. | Eligibility of Contactor: | All eligible Contactor / Bidder should be enlisted with Pakistan Engineering Council in the relevant category for the year 2016, in C-6 or above of PEC alongwith Field Specialization of C.E-09. (i) Minimum 02 Nos Works completed during last 03 Years of relevant field. (ii) Turn over of Rs.21.516 Million at least last three years. (iii) Registration with (FBR) for Income Tax, Registration with Sindh Revenue Board (SRB). (iv) Bid Validity 90 days as per SPP Rules-2010 (Amended). |
| 3. | Address where Tender can be issued / purchased. | Tenders documents can be purchased from the Office of the Accounts Officer (Revenue), Finance Department, KW&SB at KW&SB Head Office, at 1st Floor Old KBCA Annexy Building, Behind Civic Centre, Gulshan-e-Iqbal Karachi, between 9.00 am to 1.00 pm in any working day except the date of opening of the tenders or can be down loaded from SPPRA’s Website. |
| 4. | Source of Funding. | Annual Development Programme 2016-2017. |
| 5. | Method of procurement | Single stage single envelope procedure. |
| 6. | Bid Security. | 2% bid security of the quoted cost / price in shape of pay order / demand draft or Bank Guarantee from any schedule bank of Pakistan in favour of KW&SB. Bid Security must be accompanied with the Tender documents otherwise the tenders shall be treated as invalid / rejected. |
| 7. | Tender Fee. | Rs.3,000/= Non-Refundable in shape of Pay Order in favour of KW&SB. |
| 8. | Issuance of Bidding Documents. | Documents will be issued / downloaded w.e.f. 1st issuance upto ____-____-2016, on payment of above mentioned tender fee (Non-refundable). |
| 9. | Place of submission of bidding / tender documents & Opening. | At the Office of Director Design, KW&SB / Convener Procurement Committee, KW&SB, near COD Filter Plant, Gulshan-e-Iqbal Karachi. |
| 10. | Receiving of tender / bidding documents. | ___-____-2016 at 2.00 pm. |
| 11. | Opening of tenders. | ___-___-2016, at 2.30 PM (Same Day). |
| 12. | Scope of work | Improvement of Sewerage System. |

(Continued on Next Page)

(Page No.2 )

Note :-

- The Procuring Agency may reject all or any bids subject to the relevant provision of SPP Rules-2010.

- Conditional electronic and telegraphic proposal will be rejected.

- Government notified blacklisted firm (s) shall not be entertained.

- Bid must be filled in figures and in words as well.

- Income tax returned for the last three years must be attached with bidding documents.

- In case of any unforeseen situation resulting in closure of office on the date of opening or if Government declares Holiday the tender shall be submitted / opened on the next working day at the same time & venue.

- No tender shall be issued on the date of opening of tender.

KARACHI WATER & SEWERAGE BOARD

KARACHI WATER & SEWERAGE BOARD

OFFICE OF THE EXECUTIVE ENGINEER (SEW)

CLIFTON, DISTRICT SOUTH

Boat Basin Pumping Station near Grammer School Boat Basin Karachi

Cell No.0300-2636342

NO.EE/SEW/CST/DS/KW&SB/2016/		                   	Dated: - ________/2016

The Director (C.B),

Sindh Public Procurement Regulatory Authority,

Karachi.

SUBJECT:-	REQUEST FOR HOISTING OF NOTICE INVITING TENDERS  FOR THE WORKS OF “PROVIDING LAYING 30” DIA, 24” DIA, 18” DIA, 15” DIA & 12” DIA RCC WATER LINE FROM REGENT PLAZA TO JINNAH TURNING FROM IMPROVEMENT OF SEWERAGE SYSTEM JINNAH HOSPITAL AND ADJOINING AREAS OF CLIFTON / SADDAR TOWN”.

Enclosed please find herewith (04) Four Nos Pressing Clippings published in Daily Newspapers, The Daily Soab,  dated: 03-12-2016, The Daily Pakistan Observer, dated: 03-12-2016, Daily Ausaf, dated: 03-12-2016 and Daily Express dated: 03-12-2016, as well as a pay order for amounting to Rs.2,000/=(Rupees Two Thousand Only) vide Pay Order No. __________  Dated:___________, ____________________________________Branch, Karachi, in favour of SPPRA for Notice Inviting Tender for uploading on website of SPPRA. The estimate cost is Above 1.00 Million.

Encl:   (1) Pay order.

(1) NIT published in daily newspapers.

(2) Bidding document 01 Set.

(3) Copy of CRC.

(4) Copy of Letter of Procurement Committee-I, KW&SB.

(5) Evaluation Criteria.

(6) Procurement Plan.

|  | EXECUTIVE ENGINEER (SEW) CLIFTON, DISTRICT SOUTH KW&SB. |
| --- | --- |

Copy to :

- The DMD(T/S), KW&SB.

- The Superintending Engineer, District South, KW&SB.

- The Superintending Engineer (Design), KW&SB.

- The Director (I.T), KW&SB, with the request to hoisting on website please.

- The Accounts Officer (Revenue), KW&SB.

- Office Copy.