SPECIFICATION –

SPECIAL PROVISION

SPECIFICATIONS - SPECIAL PROVISIONS

TABLE OF CONTENT

- DESCRIPTION OF PROJECT

  - Scope of Work

  - Design Parameters of Sea Water

  - Sea Water Intake System

  - Pre-Treatment System

  - Desalination Plant

  - Civil Work

  - Electrical System

  - Emergency DG Set

  - Operation and Maintenance

  - Scope of O&M

- The Site

2.1	Site of Works

- The Works under the Contract

- General Description

- GENERAL RULES OF SPECIFICATION

- Specification or as specified

- Standard and Codes

- Materials and Process

- Equivalent Materials, Process etc

- Approved, Directed, Instructed

- Alternatives

- Catalogues / Standards / Manufacturer’s Instructions, etc

- Applicabiity

- DRAWINGS

- Bid Drawings

- Construction Drawings, Supplementary Drawings

- Definition of Term Drawings

- Checking of Drawings

- Copies of Drawings

5.5.1    Bid Drawings

5.5.2    Construction Drawings, Supplementary Drawings

5.6       Drawings to be furnished by the Contractor / As Built Drawings

- SETTING OUT OF WORK AND SURVEY

- Reference Points, Lines

- Verification

- Survey Instruments

- No work without Joint Survey

- APPROVAL OF MATERIALS FOR WATER FILTRATION SYSTEM

- Quality of Materials

- Submission of Samples and Data

- Testing

- Testing Laboratory Certificates

- Inspection

- Approved Sample at Site

- Site Laboratory

- CONSTRUCTION SCHEDULE

- Submittal Data

- Requirements

- Monthly Reports

- SITE OFFICE AND TEMPORARY FACILITIES PROVIDED BY THE CONTRACTOR

9.1	Contractor’s Office, Facilities etc

9.2	Sign Board

- FACILITIES FOR ENGINEERS PERSONNEL PROVIDED BY THE CONTRACTOR

10.1	Engineer Facilities

10.2 	Furnishing and Maintaining Transportation Facilities for the Engineer / Engineers Supervision Team

- SAFETY

11.1 Accident Prevention, Protective Equipment

12/26	PAYMENT FOR WORK REQUIRED BY SPECIAL PROVISIONS

27.	ATTENDANCE OF MEETINGS

28.	DOCUMENTS NOT TO BE ALTERED OR MUTILATED

29.	PERSONAL LIABILITY OF PUBLIC OFFICIALS OR ENGINEER

30.	ACCESS AND EXISTING ROADS

31.	FIRST AID FACILITIES

32.	WITNESSING OF FACTORY ACCEPTANCE TESTS

33.	HANDING OVER OF THE PROJECT / START OF DEFECT LIABILITY PERIOD

34.	FINAL HAND-OVER

35.	EMPLOYER AND ENGINEER NOT PERSONALLY LIABLE

36.	PROGRESS PHOTOGRAPHS

37.	SITE ORDER BOOK

38.	REPORT ON PROGRESS OF WORK AND PHOTOGRAPHS

SPECIFICATIONS - SPECIAL PROVISIONS

- SCOPE OF WORK

1.1	Sea Water Desalination (RO) Plant

This project is aimed to provide safe drinking water to the inhabitants of Manora Island, Karachi. The below mentioned specification is general guidelines for contractor, therefore the successful bidder shall solely be responsible for the means, methods, techniques, sequences, and procedures of construction and shall keep on the work at all times during its progress and a resident qualified project engineer shall be deployed to manage the work efficiently and with best skills and attention  and to establish quality procedure to ensure compliance with the specified requirements, local codes and applicable codes for all construction operations required under the specifications of this Tender document.

SPECIFICATION OF SEA WATER DESALINATION PLANT

To ensure a reliable supply, the bidder shall be required to quote latest and reliable technology of Sea Water Desalination RO plant for effective and consistent operation for the designed period of the project, the scope shall include design engineering, supply, installation, testing and commissioning of RO plant and Construction of Plant room and civil works.

Where applicable, ISO, IEC and other relevant codes would be applied and maintained.

The guideline specifications of the Sea water RO Plant are given in following pages. However, bidder should propose their own design based on EPC Basis.

The tables presented followed in the documents are indicative and all technical specification of the proposed desalination Plant will depend on the design, type of the system and the technology to be offered by each plant manufacturer/supplier.

1.2	Design Parameters of Sea Water Desalination

### RAW WATER

Feed water should be collected either from Sea directly by constructing Sea Intake System or underground bore water.   The design condition of TDS shall be 40,000 mg/l.

### PRODUCT  WATER:

| S. No | Parameters | Unit | Value |
| --- | --- | --- | --- |
| 1 | Product Water Quality |  |  |
|  | TDS | mg/L | < 1000 |
|  | Free Chlorine | mg/L | < 0.1 |
|  | Total Hardness | mg/L as CaCO3 | Less than 80 |
|  | PH |  | 6 – 8 |
|  | Boron | mg/L | Nil |

1.3	SEA WATER INTAKE SYSTE

Feed water should be either directly from Sea by construct sea water intake or based on underground water by developing bore well depending upon the availability of underground aquifer.

1.4	PRE-TREATMENT SYSTEM

Pretreatment should be design in a way to remove any kind of water contamination and pollutant loads through efficient pretreatment process for removing total suspended solids (TSS), color, algae, it consisting of coarse screen, sea water conditioning through coagulation and flocculation, multimedia filtration or any other suitable system.

Technology should be European / American having a local or international references.

1.5	DESALINATION PLANT

In normal osmosis process pure solvent moves from low solute concentration (High Potential) to high solute concentration (Low potential) through a semi permeable membrane, the movement of pure solvent equalize the solute concentration and the difference in solvent level is known as Osmotic Pressure. By applying force in reverse direction to break the osmotic pressure and reverse the movement from high solute concentration to low solute concentration is known as Reverse Osmosis.

Sea water shall be fed to raw feed tank by means of pumping, and the water after pretreatment shall be fed to RO feed. A minimum retention time must be provided in order to keep minimum raw feed quantity and also settle down suspended impurities as well. Thereafter filter feed Pump shall be used to pump the water to the pre-filtration stage to remove residual suspended solids from raw feed water prior to feed RO to meet the SDI < 3.

SMBS shall be added to remove residual chlorine contents or reduce the oxidizing potential from raw water in order to protect membranes from damages. A specially developed antiscalant is then added to hinder the salt crystallization w.r.to calcium and magnesium salts. Furthermore enables the plant to operate with suitable range of LSI equal or less than zero in order to hinder the slat crystallization so as to prevent membranes from scaling.

The RO Plant shall be comprising  of state of the art equipment including Feed water pump, High Pressure Pump, Pressure Vessels, sea water membrane, Cartridge and Bag Filters, Control & SCADA System, material  should be SS 316 L / Duplex.

- Automatic CIP System

- Dosing system

- Post Chlorination system.

- Electrical system including Motor Control Center, Cable and cable trays.

Plumbing Job

Complete mechanical job including supply and installation Water Distribution piping System from Feed Water Tank to Desalination Plant, from RO Plant to Product Water Tank, to reject water.

- Disposal System

- Schedule 80 PVC Pipe.

- Schedule 40 PVC Pipe.

- High pressure Piping

- Low pressure

- Valve, regulator

- Low capacity controlling switch

- High capacity controlling switch

1.6	CIVIL WORKS:

The following civil works are included.

- Sea Intake system

- Plant room, switchgear room

- Construction of Lab and Office.

- Feed water storage tank  	- 	150,000 GPD

- Product water storage tanks 	-	150,000 GPD

- Disposal drain.

1.7	Electrical System:

The Contactor shall provide the electrical system, including Low voltage switchgear and internal electrification of the plant.

Client will provide the electrical connection with in the battery limits of the desalination plant.

1.8	Emergency DG Set:

Supply and installation of DG Set: 630 KVA 	    01 Nos

Supply and installation of DG Set: 300 KVA 	    01 Nos

1.9	OPERATION & MAINTENANCE

The O&M scope of work of the Contractor under this Agreement is to operate and maintain Desalination plant hereinafter called the “O&M Project”

1.9	OPERATION & MAINTENANCE  SCOPE OF WORK:

During the period of Contract, the O&M Contractor shall be responsible for complete operation of Sea Water Reverse Osmosis Plant and regular maintenance overhauling of the plant equipments as per the instructions of the manufacturer of the plant.

The O&M Contractor shall be responsible to procure, supply, transport chemicals, consumables and cartridges, filters and other parts for at least two months stock in advance to be used for water purification during the operation of the plants.

Periodical Cleaning and Conditioning of Feed and Product Water Storage:

The Contractor shall be responsible for periodical cleaning after every three months of raw water tanks at both sites, and shall be responsible to pay all custom duties, taxes including local taxes and levies related to import of such material.

Project Laboratory:

Establishment and setup of latest laboratory at site for water analysis to test chemical, physical and biological parameters.

Deployment of Skilled Technical Manpower:

During the period of contract, the O&M Contractor shall be responsible to appoint

- One full time Project Engineer.

- Two (2) Site Supervisor (2 supervisor for each Plant) to be based on sites.

- Two skilled operators at each Plant site shall be made available round the clock for the operation and maintenance of the plant(s)

- Security Staff

1.10	SCOPE OF O&M

- Establishment of centralized project office

- Availability of Chemicals , consumables, cartridge  for minimum period of two months

- Guarantee for availability of spares/ equipment during O&M period and in future.

- Periodical Cleaning and conditioning of Water storage tanks on quarterly basis

- Weekly Water sampling and testing from reputed laboratory

- Vehicle Running Network to supply consumables/ POL , staff transfer , water sampling and monitoring of projects

- Boarding and lodging expenditure of staff deployed by the manufacturer and their fringe benefits viz. salary, insurance, uniforms, food, conveyance /etc. to be borne by the manufacturer.

- Comprehensive Training to nominated staff of client

- Guarantee for uninterrupted supply of required purified water.

2. 	THE SITE

2.1. 	Site of Works

The Site of the Works is the area for construction lying within the right-of-way lines, boundaries and limits shown on the Drawings and any such additional areas adjacent thereto as may be designated by the Engineer subject to approval of Employer from time to time for the construction to be performed under the Contract, and all such areas and additional areas shall be comprised in the Site as defined in Clause 1 of the Conditions of Contract.

The Employer will give to the Contractor possession of the area designated and defined as the Site and shown on the drawing as may be required to implement as much of the Works, when the Engineer’s Notice to Commence the Works is given.

3. 	THE WORKS UNDER THE CONTRACT

3.1. 	General Description

The Contract comprises the execution and completion of the Works, remedying of any defects therein, maintenance of utility services, and the provisions of all labour, materials, equipment, and everything whether of a temporary or permanent nature required in and for such execution, completion, remedying and maintenance so far as the necessity for providing the same is specified or can reasonably be inferred from the Contract.

The following description of the Works to be performed under this Contract is general in nature and is not intended to describe all of the facilities to be provided under this Contract.

THE WORKS UNDER THIS CONTRACT AS DEFINED IN BIDDING DATA

4. 	GENERAL RULES OF SPECIFICATIONS

- Specification or as Specified

Specification” or “as specified" refers to the specifications outlined in these Documents and where no specifications are available for any work or where the same are found not applicable then the relevant applicable ASTM or BSS specifications or equivalent standards shall apply in the same order.

Any item for which no specifications are outlined but which are identified on drawings, shall be completed according to the standards as per ASTM / BSS, these include items that may be added in the future. The Employer / Employer’s Representative may supplement such specifications during the progress of work. All materials and processes used for these items shall be subjected to standard testing and, if found below the pertinent ASTM / BSS standards, shall be removed from the site immediately at Contractor's expense.

- Standards and Codes

Wherever reference is made in the specifications to the respective standards and codes in accordance to which goods and materials are to be furnished, and work is to be performed or tested, the provisions of the latest current edition or revision of the relevant standards and codes in effect shall apply, unless otherwise expressly set forth in the Contract.

- Materials and Processes

All goods and materials to be incorporated in the Works shall be new, unused, of the most recent or current models and incorporate all recent improvements in design and materials unless provided otherwise in the Contract.

- Equivalent Materials, Processes, etc.

Where specific materials, processes, etc. are specified and the same are not available other alternative materials and processes which ensure an equal or higher quality than those specified will be accepted subject to the Employer / Employer’s Representative's prior review and written approval. Differences between those specified and the proposed alternatives must be fully described in writing by the Contractor and submitted to the Employer / Employer’s Representative at least 30 days prior to the date when the Contractor desires the Employer / Employer’s Representative's approval who may give such approval after determining that the alternative proposed ensures equal or higher quality.

- Approved, Directed, Instructed

Approved, directed, instructed means the approval, etc. of the Employer / Employer’s Representative unless otherwise stated.

- Alternatives

Where alternative materials, processes etc., are specified the selection will depend on local conditions and discretion rests with the Employer / Employer’s Representative whose decision shall be final and binding.

- Catalogues / Standards / Manufacturer's Instructions, etc.

Wherever the manufacturer's/supplier's instructions, manuals, guarantees and ASTM/BSS Standards are referred to in the specifications and details of Bills of Quantities; all such literature shall be submitted by the Contractor to the Employer / Employer’s Representative for due checking, approval and record.

- Applicability

Unless stated or specified else-where to the contrary these General Rules shall apply to all sections of work irrespective of their sequence, location and description.

5. 	DRAWINGS

5.1. 	Bid Drawings

Bid Drawings issued with the Bid Documents, called the Bid Drawings, show scope of the work to be performed by the Contractor. The Drawings are generally in sufficient detail so as to be used as a basis for construction, fabrication and for placing orders for materials subject to corrections based on the future issue of supplementary Drawings as provided under  Sub-Clause 5.2 hereof.

5.2. 	Construction Drawings, Supplementary Drawings

After award of Contract, the Contractor shall carry out “Joint Survey” at Site of Works in pursuance to Sub-Clause 6.4, Specifications - Special Provisions. The Contractor shall submit to the Engineer “Joint Survey”, duly signed, dated and stamped by the representatives of the Employer, Consultant and Contractor.

Simultaneously, the Contractor shall submit to the Engineer detailed “Work Programme” in terms of Sub-Clause 14.1 Conditions of Contract.

After receipt of “Joint Survey” and “Work Programme” from the Contractor, the Engineer will start issuing Construction Drawings to the Contractor. The Engineer shall have authority to issue to the Contractor, from time to time, such Supplementary Drawings and instructions as shall be necessary for the purpose of the proper and adequate execution and completion of the Works and the remedying of any defects therein. The Contractor shall follow these Drawings.

The Contractor shall give notice to the Engineer regarding the part of the Drawings which in his opinion contain discrepancies or are not clear. The Engineer shall issue necessary clarifications or Supplementary Drawings in greater details as required to execute the Works. These Supplementary Drawings showing changes from the Bid Drawings, in the opinion of the Contractor, shall be reviewed by the Engineer for his determination of adjustment of the Contract Price under Clause 51 and 52 of the Conditions of Contract

5.3. 	Definition of Term Drawings

The term Drawings as used in the Specifications means the Drawings referred in Clauses 5.1 and 5.2 hereof.

5.4. 	Checking of Drawings

The Contractor shall check all Drawings carefully as soon as practicable after receipt thereof, and shall promptly notify the Engineer of any errors discovered.

5.5. 	Copies of Drawings

Drawings will be issued to the Contractor and the Employer as described below.

5.5.1. 	Bid Drawings

One (1) set each of the Bid Drawings will be issued to the Contractor and Employer along with Bid Documents. Additional sets will be provided at cost of reproduction upon written request of the Contractor.

5.5.2. 	Construction Drawings / Supplementary Drawings

One (1) print of each Construction Drawings / Supplementary Drawing will be issued to the Contractor and Employer free of charge. Additional sets will be provided at cost of reproduction upon written request of the Contractor.

5.6. 	Drawings to be furnished by the Contractor / As-Built Drawings

The Contractor shall submit to the Engineer for review of such drawings as required under the Contract, sufficiently in advance of the work intended to be executed.

The Contractor shall, at all times, keep on Site a separate set of prints on which all significant changes between the work shown on the Drawings and that which is actually constructed, shall be noted neatly, accurately and promptly as the work progresses. The Subcontractor(s) for plumbing, mechanical and electrical shall, at all times, keep on Site, a separate set of prints of the drawings (showing their parts of the Works) on which all significant changes between the work shown on the Drawings and that which is actually constructed, shall be noted neatly, accurately and promptly as the work progresses. Such drawings shall show the exact physical location and configuration of the works as actually installed.

The Contractor shall, within fourteen (14) days of issuance Taking-Over Certificate for whole of the Works, furnish to the Engineer for his approval two (2) copies of such marked up drawings. One (1) copy of each of the marked up drawings approved by the Engineer shall be returned to the Contractor by the Engineer and these shall be used for the preparation of the As - Built Drawings.

The Contractor shall furnish to the Engineer six (6) complete sets of all As -Built Drawings as well as AutoCAD soft copy within thirty (30) days of receipt of drawings stated above, from the Engineer.

6. 	SETTING OUT OF WORK AND SURVEY

6.1. 	Reference Points, Lines

The Contractor shall establish bench marks and / or reference line at the Site in accordance with the instructions of the Engineer. The Contractor shall set out its work from these bench marks and / or lines.

6.2. 	Verification

The Engineer may make checks as the work progress to verify lines and grades established by the Contractor and to determine the conformance of the work as it progresses with the requirements of the Drawings and Specifications. Such checking by the Engineer shall not relieve the Contractor of his responsibility to perform all work in accordance with the Drawings and Specifications and the lines and grades given therein.

6.3. 	Survey Instruments

The Contractor shall maintain at the Site the requisite surveying instruments in perfect working conditions for the use of the Engineer’s Representative to check levels and lines of the work at all times. These instruments shall include (but not limited to) One Total Station, Adequate nos. of Levels, theodolites, Tapes, etc.

6.4. 	No work without Joint Survey

The Contractor shall not start the excavation and / or embankment works until the Joint Survey has been done to establish the existing ground levels.

7. 	APPROVAL OF MATERIALS FOR WATER FILTRATION SYSTEM

7.1. 	Quality of Materials

All materials, fixtures, fittings, supplies and furnished under the Contract shall be new and unused, standard first grade quality and of the best workmanship and design. No inferior or low-grade materials, supplies or articles will be either approved or accepted, and all work of assembly and construction shall be done in a first-class and workmanlike manner. In asking for prices for materials intended for delivery to the Site and incorporation in the Works under any portion of these Specifications, the Contractor shall provide the manufacturer or supplier with complete information as may be necessary to secure compliance to this Clause and, in every case, he shall quote this Clause in full to each such manufacturer or supplier.

The Contractor shall submit at least three names of independent laboratories of international repute for testing of the equipment in nearest international location for the approval of Employer/ Engineer. All the equipment ordered by the Contractor shall be shipped only after the testing, by the manufacturer and the independent third party inspection agency, has been completed.

The Employer will select the manufacturer of his choice and approval will be conveyed to the Engineer and Contractor.

7.2. 	Submission of Samples and Data

As soon as practicable after award of Contract, the Contractor shall submit for the approval of the Engineer drawings, catalogues, diagrams and other descriptive data for all mechanical, electrical, architectural and such other materials, which the Contractor proposes for use under this Contract. For certain materials, data may be required to be submitted in accordance with a detail form furnished by the Engineer. Samples of materials (2 sets) shall be submitted by the Contractor to the Engineer at Contractor’s cost for approval sufficiently in advance of the materials intended to be incorporated in the Works.

7.3. 	Testing

The Contractor will ensure quality control testing as and when required.

The quality control testing shall be performed by the Contractor’s competent personnel in accordance with a site testing as approved by the Engineer. The Contractor shall keep a complete record of all quality tests programme performed on Site.

7.4. 	Testing Laboratory Certificates

The Engineer may accept a certificate from a commercial testing laboratory, satisfactory to him, certifying that the product has been tested within a period acceptable to the Engineer and that it conforms to the requirements of these Specifications.

7.5. 	Inspection

All material furnished and all work performed under this Contract will be subject to inspection by the Engineer at all times and in all states of completion both off-Site and on-Site. The Contractor shall furnish promptly without additional charge, all facilities, labour and materials reasonably needed for performing such inspection and testing as may be required by the Engineer.

7.6. 	Approved Sample at Site

The Contractor shall, at all times, keep on the Site approved samples. All such samples shall be made available to the Engineer/ Employer as and when required.

7.7. 	Site Laboratory

The Contractor shall establish a site laboratory for the purpose of necessary testing. The cost of equipment and salaries of the manpower required will be borne by the Contractor.

8. 	CONSTRUCTION SCHEDULE

8.1. 	Submittal Date

The program of Works submitted by the Contractor in accordance with Clause 12 ”Program to be Submitted”, of the Conditions of Contract shall be submitted in the form of a detailed schedule based on a computerized network analysis covering all construction activities indicating critical activities with critical path, resource scheduling for Contractor’s Equipment, material and labour, within the period stated in the Appendix A to Bid. All the milestone shall be clearly identified.

8.2. 	Requirements

The detailed submittal shall consist of schedules, network analysis tabulations and narrative descriptions of the proposed construction program.

Each summary or detailed schedule shall consist of a bar chart and a time-scaled network. The scheduled start and finish times for all activities on the bar chart shall agree with those on the network. All inter-relationships and inter-dependencies between structures shall be clearly indicated on the schedules.

The network shall show the order and interdependence of activities planned by the Contractor, and shall be time-scaled according to calendar dates.

8.3. 	Monthly Reports

Each month, the Contractor shall submit a report consisting of:

- Copies of the bar charts for the current phase with both actual progress and scheduled progress shown.

- Network analysis tabulations as in Sub-Clause 8.3 above, reflecting actual start and finish dates where applicable.

- A narrative report discussing any significant deviations from the schedule and, if necessary, explaining the steps proposed to be taken to maintain the approved schedule.

9. 	SITE OFFICE AND TEMPORARY FACILITIES PROVIDED BY THE CONTRACTOR

9.1. 	Contractor’s Office, Facilities etc.

The Contractor shall responsible to establish and maintain a Site office. The Contractor shall provide all facilities in connection with the execution, completion, of the Works, remedying defects therein and maintenance of the utilities services. The facilities shall, not be limited to, the Contractor’s Site Office, labour camps, workyard and storage areas, temporary water supply, waste water disposal, temporary electricity, medical unit, temporary roads, fire protection and firefighting equipment etc. The Contractor shall be solely responsible for arranging the facilities.

The Contractor shall arrange his labour camp, work yard, storage area, site office within the area available at the Site.

9.2. 	Sign Board

The Contractor shall erect and maintain at the Site in a location to be approved by the Engineer, 3 Sign Boards 4.45M height and 2 M wide for writing the name of Work, name of Employer, name of Consultants, name of Contractor and Project Cost. The notice board shall comprise of the following;

- Frame of 3” dia GI Pipe properly painted as per the direction of the Consultants/ Engineer and as per drawing.

- 2 Nos. Posts of 3” dia GI Pipe 4.45M above ground and 1M below ground embedded in 1:2:4 CC 2’x2’x4’ with proper arrangements of anchorage and brasses. Pipes painted with anti-rust as directed by the Engineer.

- 4 Nos. Steel Sheets 0.6M high and 2M wide fixed on both sides with 50mm gap between each. The background of plates is of white color whereas the writing would be black or red color (as approved by the Engineer)

- White imported 3M sheet used as background. The color of monogram would be, green, red or black etc. (as approved by the Engineer)

- Alphabets of appropriate size as approved by the Engineer in 3M reflective sheet in blue/ black color.

The Contractor shall maintain the display of the notice boards at his own cost throughout the length of the project.

10. 	FACILITIES FOR ENGINEER’S PERSONNEL PROVIDED BY THE CONTRACTOR

10.1	Engineer Facilities

- The Contractor shall arrange fully air-conditioned office accommodation of the Engineer for a total area of around 30 Sq.M. on monthly rental basis (within Project/ District Headquarter sites) including all costs of Electricity, telephone, water supply, sewerage, janitorial services, provision of stationery/consumable/supplies (as per the requirement of the Engineer), 01 No. messenger, 01 No. tea boy (including tea making items) and 02 Nos. Security Guards etc. All appointments shall be made by the Engineer and Contractor shall be responsible for all salaries, benefits etc. of the appointed people.

- 01 No. Desktop Dell or equivalent Computer each configured with latest generation intel Core i7 Processors, 8GB DDR-3 RAM, 1GB Dedicated Graphic Card, 32GB Solid Drive + 750GB SATA Hard Drive, 15.6" HD Screen, Camera, Biometric Reader, Bluetooth, Giga bit Ethernet LAN, Card Reader, WIFI 802.11b/g, touch & type alongwith genuine/ licensed Microsoft Windows 8 operating system and Microsoft Office Professional Edition latest version and 01 No. USB Flash Drive (16 GB each).

- 08 Nos. Digital Camera (16 Mega Pixel) built-in GPS along with 4 GB Card, Rechargeable Cell (04 Nos.), Battery charger, Pouch, Tripod etc. NIKON Coolpix S800c or equivalent.

- The Contractor shall provide an average cost of Rupees Twenty-Five thousand only (Rs. 25,000/-) per month pertaining to mobile telephone use in shape of Prepaid Mobile Cards.

In case of non-provision of above facilities, the same shall be arranged by the Employer / Engineer and be deducted from any monies due / becoming due to the Contractor by the Employer along with 10% overheads till these facilities are provided satisfactorily to the Engineer.

The Cost of all above facilities are deemed to be included in the contract price and no additional payment shall be made by the Engineer to the Contractor under any circumstances.

10.2	Furnishing and Maintaining Transportation Facilities for the Engineer / Engineer’s Supervision Team

- The Contractor shall cause to furnish, operate and maintain independent transportation facility to the Engineer by way One Toyoto Corolla and Two Nos. Suzuki Cultus VXRi Cars both comprehensively insured and with drivers for the exclusive use of the Engineer / Engineer’s supervision team to meet their transportation needs. The use of such transportation facilities shall be exclusively under the control of the Engineer and the contractor shall be wholly responsible for furnishing at all times above said facilities. The said facility shall be provided within one week of Engineer’s issue of commencement letter and shall continue un-interrupted till the completion of works. The said vehicles shall be handed-over back to the contractor on issue of completion certificate of the contract.

- The Contractor shall furnish, supply and provide, without specific direction of the Engineer all lubricants, tyres, other supplies, regular service and maintenance at all times for the above vehicles till the issue of the Completion Certificate of the contract.

- The Contractor shall provide an amount equivalent to 900 Litres (400 for Toyota corolla and 250 for each Suzuki Cultus) of Hi-Super Petrol / Diesel per month to the Engineer by 5th of each month in advance for complete duration of work including any extended period.

In case of non-provision of above facilities, the same shall be arranged by the Employer / Engineer and be deducted from any monies due / becoming due to the Contractor by the Employer along with 10% overheads till these facilities are provided satisfactorily to the Engineer.

Cost of all above facilities are deemed to be included in the Contract price and no additional / separate payment shall be made by the Employer to the Contractor under any circumstances.

11. 	SAFETY

11.1. 	Accident Prevention, Protective Equipment

The Contractor shall comply and enforce compliance by all his Subcontractors with the highest standards of safety and accident prevention in compliance with all applicable laws, ordinance and statutory provisions. Where overhead work is being carried out, warning signs shall be installed at ground level clearly warning of the overhead work.

All warning signs shall be in two languages, English and Urdu, and shall at all times be maintained in a clean and legible condition, to the satisfaction of the Engineer. Trash shall be removed at frequent intervals to the satisfaction of the Engineer.

If, safety precautions/ warning signs are not installed by the Contractor, Employer will charge an amount of Rs. 10,000/- for the site.

12. 	PAYMENT FOR WORK REQUIRED BY SPECIAL PROVISIONS

Unless otherwise specifically stated in the Contract, the price of all work required by the Special Provisions shall be considered to be included in the Contract Price.

13. 	The Bided Rates shall be inclusive of all lead and lift.

14. 	The Contractor's rates shall include all incidental charges in connection with the work such as the cost of removing trees, shrubs, grass, etc., which interfere with the execution of the work as well as the cost of Natural Ground Compaction (NGC) which will be carried out by the Contractor up to the satisfaction of the Engineer prior to the earthwork.

15. 	No alterations or additions shall be made by the Contractor in the Bill of Quantities and rates must be filled in ink or typed out both in figures and words clearly and legibly in the columns provided in the schedule of quantities. All corrections must be initialed by the contractors. Any Tender which does not comply with this condition will be liable to be summarily rejected and not taken into account when preparing comparative statement.

16. 	Materials obtained from excavations will be the property of the Employer. Serviceable materials are to be stacked in places pointed out by Engineer. The Contractor undertakes to have the site clean and free from rubbish to the satisfaction of the Engineer. All surplus materials, rubbish, etc., will be removed to places to be fixed by the Engineer and nothing extra will be paid for this.

17. 	On completion of the work or earlier as directed by the Engineer, the Contractor shall remove all temporary structure (Godowns, site offices, etc.), erected by him at the site of work. He shall fill tanks dug out by him at site, remove all debris and other materials like surplus sand, stone ballast, rubbish, etc.; and in short, shall leave the site in a neat and tidy condition.

18. 	The contractors in the course of their works should understand that all material (e.g., stone and other materials) obtained in the work or dismantling, excavation, etc., will be considered as Employer’s property and issued to the contractors (if they require the same for their own use) at rates approved by the Engineer. If the materials are not required by them they will be disposed off in the interest of Employer.

19. 	The contractor shall inspect the site of works and acquaint himself with the nature and requirements of the work, facilities of access for materials, removal of rubbish, cost of carriage, nature of strata, etc., before submitting his Bid.

20. 	The contractor shall have to make temporary approach roads, etc., at his own cost to facilitate movement of materials, such approach roads shall be aligned in a manner approved by the Engineer.

21. 	The contractor shall have to make proper arrangements for road crossing barriers during working hours in the day time as well as in the night when danger lights will have to be provided on either ends at his own cost and no extra cost will be paid. Sufficient barricades and red lights will be provided by the Contractor where required to avoid the chances of accidents. In case an accident occurs for failure on the part of the contractor, he shall be entirely responsible for the consequences.

22. 	The Contractor shall have to make arrangements for diversions for traffic wherever necessary and shall have to provide diversion and caution boards as per directions of the Engineer at his own cost for which no extra cost will be paid. The diversion shall be watered and consolidated as per directions of the Engineer.

23. 	No material shall be removed from the site without the written permission of the Engineer.

24. 	Dewatering including shoring wherever so required for any foundation area, pumping, bailing out water, drainage of water within plot areas if any shall be deemed to have been included in the rates quoted by the bidders and no extra payment will be made.

The rates shall be deemed inclusive of such incidental charges.

25. 	The Contractors shall execute all works at their own cost for diversion of water away from the plot as per site requirements to have full satisfaction of Engineer and no additional payment will be made on this account.

26. 	The Engineer, subject to approval of the Employer, reserves the right to select all materials and the type, grade, heating capacity and quantity of proportion of any or all materials as required for a particular work. The decision of Employer in this respect shall be final and binding on the Contractor. The rejects on materials must be carted at his own cost. If the rejected materials are not removed within one month of its rejection the materials will become the property of the Employer or will be removed at Contractors cost.

27. 	ATTENDANCE OF MEETINGS

The Contractor shall attend and shall cause his Sub-Contractors to attend any or all meetings when called by the Employer or the Engineer or his Representative to discuss progress of the work and other matters related to the work and the Contract, without any compensation from the Employer.

- The Contractor shall bear all expenses of the Employer and his agents and representatives and the Engineer, his agents and representatives if requested by the Contractor for any meetings, instructions and approvals away from the Site.

- The proceedings of the meetings shall be recorded by the Engineer which shall be circulated to all the participants including those of the Contractor. All decisions taken in the meetings shall be binding on the Contractor and shall form part of the Contract.

28.	DOCUMENTS NOT TO BE ALTERED OR MUTILATED

No alteration or mutilation (other than filling in all the blanks intended to be filled in) shall be made in the form of Bid or in any of the documents attached to it. Any comments which it is desired to make shall not be placed on any of the documents attached hereto, but shall take the form of a separate statement which shall be as brief as possible and referenced to items, clauses and pages of the annexed documents.

Such statements shall not qualify the acceptance of the Bid based upon a proposed change or changes in the annexed documents, nor shall be binding upon the Employer in any way in making the award. Alterations of already written prices must be signed in the place of alteration by the Bidder or his legally authorized representative.

29.	PERSONAL LIABILITY OF PUBLIC OFFICIALS OR ENGINEER

In carrying out any of the provisions of these specifications, or in exercising any power of authority granted to them by or within the scope of the Contract, there shall be no liability upon the Employer or his authorized representatives or the Engineer or his authorized representatives their personally or in their official capacity, it being understood that in all matters they act solely as agents and representatives of the Employer.

30.	ACCESS AND EXISTING ROADS

If the Contractor finds it necessary or elects to use existing roads, the Contractor shall make all necessary arrangements and obtain all permits from the relevant departments for travel over and use of such roads. The Contractor shall observe all rules regulations of the concerned department regarding the use of said roads. The cost of maintaining all necessary safety measures and temporary structures and making any necessary repairs, replacements or similar operations and all or any other costs required by reason of his use of such roads shall be borne by the Contractor and the Contractor shall save harmless and indemnify the Employer in respect of all claims, demands proceedings, damages, costs, charges and expenses whatsoever arising out of or in relation to any such operation or interference.

31.	FIRST AID FACILITIES

The Contractor shall provide and maintain adequate First Aid Facilities at all times, convenient to the Site to the approval of the Employer.

32.	WITNESSING OF FACTORY ACCEPTANCE TESTS

The Contractor shall arrange at-least Two (2) pre-shipment visit(s) of manufacturing factory by at-least but not limited to Two (02) officers of Employer and Two (02) officers of Engineer per trip. These visits shall be scheduled as per the work schedule of the Contractor and the Contractor shall notify the timing of these visits in advance. The pre-shipment visits shall be made after the independent inspection by the approved third party inspection agency is completed in a satisfactory manner. The Contractor shall arrange for the visit of Laboratory selected by the Representatives of Employer and Engineer prior to its approval.

The Contract price shall be deemed to include all the expenses of the above arrangements with and including the arrangement of the relevant visas for designated officers of the Employer and the Engineer, their visa fees, Air Tickets (Business Class), boarding, lodging, local transport costs in independent vehicles aboard etc., as well as cost of testing and third party inspection.

33.	HANDING OVER OF THE PROJECT/ START OF DEFECT LIABILITY PERIOD

On completion of the Warranty Period of 03 years, the Contractor shall hand-over the project in perfect working condition to the Employer or any agency designated by the Employer in writing.

Defect Liability Period shall start after the handing-over.

34.	FINAL HAND-OVER

At the end of the Defect Liability Period stipulated in the Contract, the Employer on application of the Contractor, shall decide the members of the Final Hand Over Committee and announce the same to the Contractor. The Committee, after inspection of Works, if satisfied that there are no deficiencies or defects due to Works of the Contractor shall certify the final hand-over, and then a final Certificate of Completion of Work within thirty (30) calendar days will be issued by the Employer.

35.	EMPLOYER AND ENGINEER NOT PERSONALLY LIABLE

No member or officer of the Government or the Employer or the Employer's Representative or the Engineer or his representatives or any one of their respective staffs or their employees shall be in anyway personally bound or liable for the acts or obligations of the contractor under the contract or answerable for any default or omission in the observance or performance of any of the acts, matters or things which are herein, contained.

36.	PROGRESS PHOTOGRAPHS

The contractor shall furnish to the Employer and to the Engineer every two weeks at least six photographs to clearly show the progress of construction. The photographs shall be submitted in glossy prints 20 cm x 20 cm. Each print shall be marked on the back with the date and serial number. There shall be no writing, lettering or marking on the face of the photographs. The set of photographs of the Engineer should accompany respective negatives.

37.	SITE ORDER BOOK

The Contractor shall maintain site order book {of triplicate leaves} at the Site, for taking down instructions of the Engineer and/ or the Employers, without any obligation and charges to the Employer / Engineer.

38.	REPORT ON PROGRESS OF WORK AND PHOTOGRAPHS

The Contractor shall, during the execution of the work, submit to the Employer (3 copies) and Engineer (2 copies) so as to reach them in the first week of every calendar month, a report on the actual progress of the works attained by him during the preceding month fully supported with colour photographs of (5” x 7”) size, at least 15, depicting the complete stages of the works. Each photograph should be properly pasted on A-4 size paper, indicating the location and other relevant information of the area photographed. The report will be submitted on the standard format to be supplied later on. In case the Supervision Engineer are different from the Design Engineer, one copy each of photographs should be sent to both the Engineer.

The set of photographs for the Design Engineer should be submitted with respective negatives.

SPECIFICATION –

TECHNICAL PROVISION

TECHNICAL SPECIFICATIONS

CIVIL WORKS

EXCAVATION, TRENCHING AND BACKFILLING

- SCOPE

The work covered by this section of the specifications consists of furnishing all plant, labour, equipment, appliances and materials and performing all operations in connection with excavation, trenching and back-filling for sewer and water supply lines and all other structures including all incidental works necessary for excavation to the required depth and dimensions in accordance with the applicable drawings, or as directed by the Engineer. The work shall be carried out in complete conformity with the specifications, set-forth hereunder.

- SETTING OUT

Lines and levels will be set out by the Contractor who shall be responsible for maintaining all stakes and witness points set-up by the Engineer for the execution of work in strict accordance with them.

- CLEARING AND GRUBBING

The sites of all excavations shall be cleared of all shrubs, plants, bushes, large roots, rubbish and other objectionable materials. All such materials shall be removed from site of work or otherwise disposed off at no extra cost in a manner satisfactory to the Engineer. All trees and shrubbery that are designated by the Engineer to remain shall be adequately protected and preserved in an approved manner.

- EXCAVATION

- General

The contractor shall remove the whole of the vegetation, top soil, concrete, flagging, paving, curbing, road metalling and other materials from the site of any excavation and shall keep separately and preserve the same for re-use where applicable. The ground shall be excavated for the permanent and temporary works to the required depths, width and levels so that the dimensions of the permanent work shall not be less than as shown on the Drawings, or as may be directed.

All rubbish, filth and matter of an offensive nature taken out of any excavation shall be disposed off at once and not left on the surface within the site.

- Earth Excavation for Sewers

Unless otherwise directed or permitted by the Engineer not more than 500 ft of any trench in advance of the end of the pipeline already laid shall be opened at any time. Trenches shall be excavated to the dimensions and depths shown on the drawings or ordered by the Engineer or in such a position or to such dimensions and depths as shall allow for the proper construction of the relevant structure or construction or proper excavation of the relevant operation. For excavation the width of trench allowable for payment shall be the external diameter of pipes plus 12 inches on both sides, for pipes up to 15 inches diameter. For diameters exceeding 15 inches, the width of trenches shall be external diameter plus 18 inches on both sides. For depth exceeding 5 feet slope allowance of 1.5 inch per foot (in depth for each side of the trenches) shall be made in addition to the width specified to the full depth of trenches. The Contractor shall make allowance for the additional excavation required for making joints and, where necessary, for concrete bedding or surround in the price tendered for trench excavation. These shall not be separately measured or paid.

The banks of the pipe trench shall be as nearly vertical as practicable. Bell holes and depressions for joints shall be dug after the trench bottom has been prepared. The pipe, except for joints, shall rest on the prepared bottom for its full length. Bell holes and depressions shall be only of such length, depth, and width as required for properly making the particular type of joints stones shall be removed to avoid point bearing. Whenever wet or otherwise unstable material that is incapable of properly supporting the pipe as determined by the Engineer is encountered in the bottom of the trench, such material shall be removed to the depth required and the trench backfilled to the proper grade with coarse sand, or other suitable approved granular material. Such replacement of unsuitable material will be paid for at the contract unit price for that item of work as shall be agreed upon, before execution of this work, with the Employer.

Where the Contractor has excavated to depths in excess of the requirements, from his neglect or from causes within his control, he shall refill and compact the excess excavation with suitable material approved by the Engineer, up to corrected level, at his own expense.

Excavation for appurtenances shall be sufficient to leave at least 12 inches but not more than 24 inches between the outer surface and the embankment or timber that may be used to hold and protect the banks. Any over-depth excavation below such appurtenances that has' not been directed by the Engineer, will be considered un-authorized and shall be refilled with compacted sand, gravel or concrete, as directed by the Engineer and at no additional cost to the Employer.

- Earth Excavation for Water Supply Lines

For excavation the width of trench allowable for payment shall be the external diameter of pipes plus 18” for pipes up to 12” diameter. For depth exceeding 5 feet slope allowance of 1.5 inch per foot (in depth for each side of the trenches) shall be made in addition to the width specified to the full depth of trenches. The Contractor shall make allowance for the additional excavation required for making joints and, where necessary, for concrete bedding or surround in the price tendered for trench excavation. These shall not be separately measured or paid.

- Excavations For Trenches / Drains

The Contractor shall excavate trenches / drains in straight lines between the tangent points of bends or changes of cross section and in smooth curves to the radius shown on the drawings or ordered by the Engineer. For depth exceeding 5 feet slope allowance of 1.5 inch per foot (in depth for each side of the trenches) shall be made in addition to the width specified to the full depth of trenches.

The side slopes and beds of trenches / drains shall be constructed and neatly trimmed equal to the best practicable finish. The area of the finished excavation at any cross-section shall not be less than that shown on the Drawings or ordered by the Engineer for that cross-section. For depth exceeding 5 feet slope allowance of 1.5 inch per foot (in depth for each side of the trenches) shall be made in addition to the width specified to the full depth of trenches.

When rock has to be excavated, the lower surface of the trenches shall be made as level and true as possible in accordance with the plans, drawings and instructions of the Engineer.

If the surface at the bottom of a trench be of rock which is sloping it shall be made level by chiseling. If the slope is considerable and is in the direction of the longitudinal axis of the trench it shall be cut and divided into horizontal terraces at different levels. Filling-in with concrete of any pockets or other trifling inequalities which it might be difficult or expensive to level shall be done only after the pockets have been measured and the measurements signed by the contractor in token of his acceptance. In the even of any dispute regarding the volume of the pockets the decision of the Engineer will be final and binding on the contractor.

  - Excavations for Reservoirs / Tanks and Foundations

The Contractor shall excavate tanks/ reservoirs and foundations to the lines and levels shown in the Drawings. As for as is practicable excavation shall be carried out in uniform layers over the full areas. The excavation shall be kept clear of water at all times. Bulk excavation may be carried out by machine or other approved methods to within 6 inches of the final surface. The final 6 inches of excavation shall be carefully carried out by hand. Embankment slopes shall be trimmed to the side slopes shown in the Drawings. For depth exceeding 5 feet slope allowance of 1.5 inch per foot (in depth for each side of the trenches) shall be made in addition to the width specified to the full depth of trenches.

  - For Pipelines

- The excavation shall be carried out to the required alignment, levels, slopes or gradients as per drawings or described in the specifications and bill of quantities taking into account bedding required below pipes or to such other dimensions and slopes as the Engineer may direct in writing to facilitate laying of pipes for sewerage network (both shallow and deep). The Contractor shall provide masonry pillars of suitable size and fix temporary benchmarks at intervals to be determined by the Engineer or his representative(s). No trench excavations shall be commenced without prior approval of the Engineer. Excavation shall proceed at the same rate as laying, jointing, testing and backfilling.

- The quantity of excavation shall be the volume of materials removed from below the original surface of the ground to the limits of excavation specified or shown on the drawings. For soft and unstable soils, the Contractor shall provide all necessary site supports including timbering or sheet piling to support the sides of trenches. The cost of supply of all material, plant and labour that may be necessary for site clearance, excavation, over break, timbering, sheet piling, shoring, strutting, refilling, watering and ramming, etc., shall be included in the Contract Rates for excavation. In all cases, the quantity of excavation measured shall be the in- site volume of the undisturbed material with in allowable limits mentioned in the specification. In case sides or ends of any excavation collapse under self-weight or due to any other reason, the contractor shall at his own cost remove all disturbed material. Should sides or ends of any excavation give way, the contractor shall at his own cost remove all disturbed material. No additional payment due to side slopes of pipe trenches if carried out by Contractor shall be allowable.

- Where the Contractor has excavated to depths in excess of the requirements, he shall refill and compact the excess excavation with 1:4:8 cement concrete upto the correct level at his own expense. Any excavation done in excess of specified width due to any reason, what so ever shall not be payable.

- For excavation above ground water table, the width of the trench shall be equal to the external diameter of the pipe plus 18’’dia. not exceeding 12’’ dia. For sewers of internal diameter exceeding 12’’ dia. The width of trench payable shall be equal to external diameter of pipe plus 30’’dia. The depth shall be as per longitudinal section of sewers and shall include for sewer bedding, to give minimum 30’’of the earth cushion over the pipe or to the depth of existing pipe where required to be connected.

- For excavation below ground water table up to a depth of 10ft, the width of trench allowable for payment shall be the external diameter of pipes plus 24’’ for pipes up to 12’’ diameter. For diameters exceeding 12’’, the width of trenches shall be external diameter plus 3ft.

- For excavation below ground water table and depths exceeding 10ft, the width of trench allowable for payment shall be the external diameter of pipe plus 3’-6’’.

- Additional excavation will be necessary at all manholes, valve chambers and pipe joints to facilitate the making of joint. Additional excavation for construction of manholes, valve chambers and joint holes shall be of such dimensions, so as to give clear working space. The Contractor shall make allowance for the additional excavation required for the manholes and valve chambers in the price tendered for trench excavation. These shall not be separately measured or paid.

- The length of the trench shall be measured along the centre line of the trench and the depth shall be measured vertically from original ground levels to the average bed level.

- The maximum length of trench to be left open shall be the length between manholes or not more than 160ft of jointed pipe line, whichever is the lesser, and shall remain visible for the purpose of inspection and testing. In exceptional circumstances where the nature of the ground or locality renders it necessary to reduce this distance, the contractor shall inform the Engineer, immediately. In the case of pressure pipelines, partial backfilling shall be carried out before testing.

- Where pipes are laid through rock or extra hard strata, the trench shall be excavated to depths below the barrel of the pipes specified in "Schedule for pipe bedding & surrounds". The space below the pipe barrel shall be refilled with specified granular bedding material.

  - Trial / Test Pits

The Contractor may be required to excavate trial pits and trial trenches upto about 10% of the total quantity of excavation specified in the contract at appropriate locations to determine the actual level of the existing water table, and position of existing conduits, water mains, gas mains, cable ducts and sewers etc. This excavation work shall be done carefully with due precaution, so as not to damage any existing services. The Contractor may be precluded from carrying out any permanent work until this information is obtained and may have to adopt his program in accordance with the information so obtained by the Contractor.

Trial test pits will be required to be dug before or during the execution of work at locations directed by the Engineer for determining the condition of soil, checking the location of utility services water levels etc. The size of individual trial pits may be kept 5ft x 5ft up to the required depth. The dimensions may be varied depending upon the site condition and as per instruction of the Engineer. The Contractor shall obtain prior permission from Engineer in writing before start of work on trial pits. No separate payment shall be made for trail pits required to be dug by the Contractor.

The cost incurred by the Contractor on the trial / test pits shall be deemed to be included by the Contractor in his rates for excavation.

  - Classification of Soils

Excavation shall include the removal of all materials in all kinds of soils or stratas of every name and nature. The sub- soil in the project area mostly comprises of clay with fine sand and silt and high sub-soil water level. A considerable amount of dewatering and supports for the sides of excavation will be essential including bore holes, well point system and side supports comprising of shuttering, bracing, strutting and sheet piling. However the Contractor shall make his own assessment after detailed study of the area and digging the required trial / test pits as required in this regard. No claim shall be allowed on account of any omission or error in such data trial / test pits.

If rock is encountered it shall be removed carefully and without excessive noise and vibration. Blasting shall not be allowable. The quantities of earthwork for each category of excavation i.e. soil, and rocks are provisional. The Engineer shall do the classification of soil during actual excavation. In case the Contractor meets rock during the excavation, the contractor shall request the Engineer in writing for a joint inspection for classification of soil. The Engineer shall visit the site during excavation and give his opinion in writing about classification of soil for the particular site or alignment.

The excavation payable shall be limited to the dimensions and elevations as indicated on the drawings. Foundations on made up ground shall be taken down to nascent soil as per direction and approval of the Engineer. Excavation shall extend to a sufficient distance away from walls and footings to allow for placing and removal of forms, installation of services and for inspection. No payment shall be made for this extra excavation. The Contractor's rate for excavation shall be deemed to include for such extra excavation.

In the event of any excavation being carried out deeper than specified, the same shall be filled in by the Contractor at his own cost to the required level with lean concrete if beneath footing or with proper compacted local river sand if beneath slab.

  - Mechanical Diggers and Other Appliances

The Contractor shall not use mechanical excavation in gardens or plantation areas unless approval in writing has been obtained from Employer and tenants.

In addition to the above, if the Engineer shall reasonable consider it unsuitable that any excavator, mechanical digger or other machine or appliances employed, or proposed to be employed by the contractor should not be used or that any such machine or appliance as aforesaid is unsuitable for use on the works or on any part of the works, the Engineer may order the Contractor not to use and / or to immediately remove from the works such machine or appliance.

- PRECAUTIONARY AND REMEDIAL MEASURES

- Protection of Existing Facilities and Structures

The Contractor shall take every necessary precaution not to endanger the safety, occupation or operation of any property, structures, installations or services in the vicinity of his operations and shall observe any restrictions imposed by authority concerned / Engineer to this end. Should any such property, structures, installations or services be endangered or damaged as a result of the Contractor's operations, he shall immediately report any such danger or damage to the Engineer's Representative and any authority concerned and shall forthwith undertake remedial measures to the satisfaction of the Engineer or the appropriate authority.

- Planking and Strutting

The Contractor shall provide, if required, at his own expense to the satisfaction of the Engineer all times support effectively the sides of the pipe trenches and other excavation by suitable timbering, sheet piling, sheeting, bracing, strutting etc. Where required the contractor shall use close timbering in all loose or sandy or unstable stratas both above or below ground level, if found necessary by the Engineer and accord approval. It is intend that all timbering and side supports for sewer trenches shall be removed as the work proceeds. The Contractor shall ensure that the removal of timbering and side supports is done gradually and carefully to avoid any damage to existing or new structures, roads, pavements or any other private or public property. All timbering, sheeting and their supports shall be of adequate strength and dimension and fully braced and strutted so that no collapse, subsidence or any damage to public or private property shall take place. The Contractor shall be solely responsible for the sufficiency of all timbering, sheet piling and their supports to be used and all damages to persons or property resulting from the improper quality, strength, placing, maintaining or removal of the same shall be payable by him under all circumstances.

In removing timbering, shoring and strutting and all other supports from excavation and trenches etc., special care shall be taken to avoid bringing pressure to beat on any concrete or other work until it has hardened sufficiently to resist such pressure.

- Removal of Water

The Contractor shall build all drains and do ditching, pumping and all other work necessary to keep the excavation clear of sewage, storm water and water from any source during the progress of the work and until the finished work is safe from injury. All water pumped or drained from the work shall be disposed of in a manner satisfactory to the Engineer and necessary precautions against flooding shall be taken. The contractor should submit the Methodology of dewatering for approval. It may also be noted that any approval of the methodology will not relieve the contractor from any of his responsibilities / obligations.

The Contractor shall be required to arrange well point equipment and / or adequate number of tube wells or both and pumping machinery for dewatering and lowering the existing water table for construction purposes  in the areas where sub-soil water or any sewage and water from any other sources are encountered. The system shall be capable of working non-stop 24 hours a day for the entire duration of the work without break during excavation, and for laying of sewer, pipes and bedding, construction of manhole, construction of structures, testing of sewers/ pipes and backfilling. The system of dewatering proposed to be adopted shall be submitted by the contractor with sufficient details along with the tender for approval of the Engineer. The Contractor is required to visit the site before submitting his tender and investigate the available mean of disposal of pumped water including laying of temporary pipeline for transmission of water during the period of excavation providing bedding, laying & jointing of sewer, pipes and construction of any structure up to ground level. The cost of all such works required for pumping and disposal of water from trenches/ pits shall be considered to be included in the BOQ rates for excavation.

- Maintenance of Excavation

All excavation shall be properly maintained with while they are opened land exposed. Sufficient suitable barricades, warning lights, flood lights, signs, and similar items shall be provided by the Contractor. The Contractor shall be responsible for any damage due to his negligence.

- Surplus Materials

All surplus materials shall be disposed off at locations approved by the Engineer. The disposal of surplus material shall not interfere with other works and shall not damage or spoil other material. When it is necessary to haul earth or rock material over street or pavement, the Contractor shall prevent such materials from falling on the street or pavement.

- Cutting Pavement

In cutting or breaking street surfacing, the Contractor shall not use equipment which will damage the adjacent pavement. Existing paved surface shall be cut back beyond the edge of trenches to form neat square cuts. The road ballast and other materials shall be placed on one side and shall be preserved for re-installment when the trench is filled. Wherever necessary or required for the convenience of the public or individual residents, at street crossings and at private driveways, the Contractor shall provide suitable temporary bridges over unfilled excavations. All such bridges shall be maintained in service until backfilling has been completed. The Contractor shall keep the road crossings manned 24 hours per day. During night time, enough red lights shall be provided to warn traffic. If detour is necessary, the Contractor shall make proper detour for the traffic and shall install- signs 3 ft x 4 ft in size indicating the detour.

- FILL, BACKFILLING AND RESTORING OF GROUND TO ORIGINAL CONDITION

- Fill, where required to raise the sub-grade for concrete slabs, shall be clean, unadulterated local river sand and shall be free from wood, stones and other debris. Excavated material shall only be used for fill if approved by the Engineer or his representative.

All fill backfilling or earthwork in embankment shall be compacted by mechanical rammer, or other approved equipment in layers not more than 150 mm thick. Each layer shall be uniformly spread and fully compacted and shall have proper moisture content for the required degree of compaction which shall be done by mechanical tampers as approved by Engineer.

After completion and final approval of the work of sewers and other construction as shown on drawings and prior to backfilling, forms shall be removed carefully and excavation shall be cleaned of stones and debris. Backfill shall be brought to a suitable elevation above ground to provide for anticipated settlement and shrinkage thereof.

Backfill shall not be placed against walls etc., prior to the water proofing treatment if provided and approved by the Engineer. Backfill shall be brought up evenly on each side of walls as far as practicable. Heavy equipment for spreading and compacting backfill shall not be operated closer to the wall than distance equal to the height of the backfill above the top of base slab footing. No back filling shall be done before the new structure has been cured for atleast two weeks.

- Backfilling and Restoring of Ground to Original Condition

The back filling of the trench shall be allowed after the sewer pipe has been laid and jointed over the specified bed, inspected, checked, tested and approved by the Engineer. Backfilling of the trenches shall be carried out by filling to depth up to half pipe level. The filling shall then be thoroughly rammed more filling shall be carried out and rammed again until the consolidated filling reaches pipe top level. Only selected, dry materials free from stones or debris shall be used for backfilling, which shall be spread and rammed evenly across the trench. Thereafter, the trench shall be filled in layers not exceeding 150 mm in depth, each layer being properly rammed before the next layer is placed so that 95-100% compaction is obtained as per AASHTO Standard.

On completion of backfilling, the Contractor shall level all grounds disturbed by him in the course of the work, spread topsoil where necessary as directed by the Engineer.

- Backfilling for Structures

Backfilling operations for structures shall be performed as part of the Contractor’s work under the payment items for earth excavation and at no cost to the Employer. It would comprise returning and filling the selected excavated material around foundations, and at back of walls etc., upto finished levels shown on the Drawings or as required in layers not exceeding 6 inches, carefully rammed and consolidated (With addition of water if required so as to achieve a minimum relative density of 85 or 90 as directed by the Engineer. No filling shall be made until the concrete foundations and footings etc., have been inspected and approved by the Engineer. Earth to be used for filling must be free of all the organic impurities, debris or any other foreign matter. Earth which contains more than 1% of salts particularly sulphates will not be used in filling.

- Backfilling of Trenches

The trenches shall not be completely backfilled until all required pressure tests are performed and until the lines as installed conform to the requirements of specifications. Where in the opinion of the Engineer, damage is likely to result from withdrawing sheeting, shoring the same shall be left in place and cut off at a level 1 ft. below ground surface. Sheeting left in place shall be paid for at the approved rate for the item of Trenches shall be backfilled to the ground surface with selected excavated material or other material that is suitable for proper compaction. Trenches improperly backfilled shall be reopened to the depth required for proper compaction, then refilled and compacted to the specified density. The surface shall be restored to its original or better condition. Pavement and base course disturbed by trenching operations shall be replaced.

- Lower Portion of Trench

Backfill material shall be deposited in 6 inch maximum thickness layers and compacted with suitable hand tampers to ninety five percent of maximum density until there is a cover of not less than 1 ft. over the pipe. The backfill material in this portion of trench shall consist of sandy clay or other approved materials free from stones and humps.

- Remainder of Trench

The remainder of the trench shall be backfilled with material that is free from stones larger than 6 inch in any dimension. Backfill material shall be compacted to 90 percent of maximum density for cohesive soils and 95 percent of maximum density for others.

- BORROW

In case of non-sufficiency of excavated material and un-suitability of earth for backfilling, conforming to the above specifications, such material shall be brought from the approved source, by the Contractor.

- GRADING

After the completion of all backfilling operations, the Contractor shall grade the work areas to the lines, grades and elevations shown on the drawings or as directed by the Engineer. Finished grading shall not be done until the installation of all utilities of appurtenances has been completed and tested. Prior to final acceptance, all damage due to settlement shall be repaired by an at the expense of the Contractor.

- TESTING OF SOIL IN PLACE

The Engineer will make tests using the calibrated sand cone method/core cutter method to determine the density of soil in place. If soil in place fails to meet the specified degree of compaction the areas represented by the failing tests shall be removed, replaced and compacted to the specified density in the manner directed by the Engineer and at no additional cost to the Employer.

- REMOVAL OF EXCESS AND UNDESIRABLE MATERIALS

- Excess and undesirable material from excavation not required for fill or backfill shall be disposed off, removed and / or deposited and leveled on the site where directed by the Engineer. Earth suitable and meant for backfill shall be stored at site in a manner not to interfere with the progress of construction works in progress.

- The Contractor shall keep all excavated soil sprinkled with water during the excavation work so as to prevent any dust nuisance.

- Surplus Excavation Debris etc.

All surplus soil arising out of the work shall be carried away to approved site, within a week, deposited and spread as directed by the Engineer.

The Contractor shall carry out the cutting of existing bituminous road as required for excavation for carrying out the work, to the full depth of hard crest of any existing thickness. The stone metal soling etc. shall be separately stacked along the side of excavation for possible reuse.

- PROTECTION OF UTILITY SERVICES

- Utility Lines

The Contractor shall take every necessary precaution not to endanger the safety, occupation or operation of any property, structures, installations or services in the vicinity of his operations and shall observe any restrictions imposed by authority concerned / Engineer to this end. Should any such property, structures, installations or services be endangered or damaged as a result of the Contractor's operations, he shall immediately report any such danger or damage to the Engineer's Representative and any authority concerned and shall forthwith undertake remedial measures to the satisfaction of the Engineer or the appropriate authority.

When any existing utility lines are encountered within the area of operations, the contractor shall take all necessary measures so that these are neither disturbed nor damaged. The Contractor shall be fully and solely responsible for any damage occurring due to non-providing of adequate measures for the protection of such services. The Contractor shall be required to obtain all necessary permissions from different departments / agencies in writing prior to start of work and maintain the affective liaison for trouble free progress of work(s). The contractor shall pay all fees, charges officially levied by such department / agencies while issuing required permission. The Contractor shall furnish originals of payment receipts alongwith his written request for allowing payments by the Engineer accordingly. In case of restoration to unavoidable damage to any utility service, line or by passing such line the procedure as detailed shall be followed in accordance with rules, regulation, specification or practice as preferred by the concerned department / agency.

- Damage to Surface

If carriage ways, verges or footways in roads, whether paved or unpaved, or gardens, plantations or other surfaces are damaged outside the limits of the excavations due to lack of proper traffic control or moving plant and equipment or other operations of the contractor then such surfaces shall be reinstated by the contractor at his own expenses. The surfaces shall be restored to their original condition using such materials as may be required whether obtained from the excavated materials or not.

- Maintenance of Traffic

The Contractor shall keep the road crossings manned 24 hours per day. During night time, enough red lights shall be provided to warn traffic. If detour is necessary, the Contractor shall make proper detour for the traffic and shall install signs 3 ft x 4 ft in size indicating the detour.

When the excavation is in roads, care shall be taken to cause the least inconvenience to traffic. When directed or necessary for the maintenance of traffic, the contractor shall remove from the site all materials as excavated from the trenches and return the same as necessary for refilling after the structures have been completed or the pipes tested and approved.

- Control of Traffic on Roads

The Contractor shall ensure that the flow of traffic over the existing roads and access to properties is maintained at all times during the contract. The flow of traffic is to take place at all time over a reasonable surface, which is to be segregated as far as possible from areas where work is in progress. The contractor shall provide flagmen and signaling equipment as may be necessary to control the traffic to the satisfaction of the Engineer and the appropriate controlling Authority. In the planning and execution of any temporary or permanent works, which may effect the traffic flow and / or access to properties, the contractor shall co-operate closely with the Engineer and the appropriate controlling Authority.

- MEASUREMENT AND PAYMENT

  - Excavation and Backfilling

Measurement and payment for excavation and filling shall be made in accordance with the following provision:

- Method of Measurement

The measurement shall be made of the earth acceptably excavated for trenches and structures within the lines and grades shown on the drawing or as directed by the Engineer.

- Basis of Payment

- Payment for earth excavations for trenches and structures will be made at unit price stated in Bid Schedule of this contract or in applicable Variation Orders.

- For the purpose of measurement, the depth of filling shall be taken as consolidated depth.

PLAIN & REINFORCED CONCRETE

- SCOPE OF WORK

The work to be done under this section of the specifications shall consist of furnishing all plant, labour, equipment, appliances and materials and in performing all operations in connection with providing and placing plain and reinforced cement concrete, in position as shown on the drawings including furnishing formwork, batching, mixing, finishing and curing as per drawings or as directed by the Engineer.

- CLASSES OF CONCRETE

The classes of concrete recognized in these specifications shall be designated: A, B, C, D1, D2, D3, Y and Lean concrete. The class of concrete to be used shall be as called for on the Drawings or as directed by the Engineer or specified in the special provisions. The following requirements shall govern unless otherwise shown on the Drawings.

Class A1 concrete shall be used everywhere, for non-reinforced and reinforced concrete structures, except as-noted below or directed by the Engineer. Concrete placed underwater shall be Class A2 with a minimum cement content of three hundred fifty (350) kg per cubic meter of concrete with a slump between ten (10) and fifteen (15) cm. Concrete placed for piles shall be Class A3 with a minimum content of four hundred (400) kg per cubic meter.

Class B Concretes shall be used only where specified.

Class C concretes shall be used for cribbing, or as otherwise directed by the Engineer or specified in the Special Provision or on the Drawing.

Class D1, D2 or D3, concrete shall be used for pre-stressed and post-tensioned elements, as indicated on drawings.

Class Y concrete shall be used as a filler in steel grid bridge floors, in thin reinforced sections or as otherwise specified in the Special Provisions.

Lean concrete shall be used in thin layers underneath footings and when called for on the Drawings or directed by the Engineer.

The concrete of the various classes shall satisfy the requirements shown in Table.

Table of Portland Cement Concrete Requirement

| Class of Concrete | Min. cement Kg/ Cubic Meter | Max. Size of Coarse Aggregate (mm) | 28 days compressive strength (Min) (Cylinder) (Kg/ Sq.cm.) | Consistency (Range in Slump) Vibrated (mm) | Maximum permissible water-Cement Ratio |
| --- | --- | --- | --- | --- | --- |
| A1 | 300 | 20 | 210 | 25-75 | 0.58 |
| A2 | 350 | 25 | 245 | 100-150 | 0.58 |
| A3 | 400 | 38 | 280 | 100-150 | 0.58 |
| B | 250 | 51 | 170 | 25-75 | 0.65 |
| C | 275 | 38 | 210 | 25-75 | 0.58 |
| D1 | 450 | 25 | 350 | 50-100 | 0.40 |
| D2 | 500 | 25 | 425 | 50-100 | 0.40 |
| D3 | 550 | 25 | 500 | 50-100 | 0.40 |
| Y | 400 | 13 | 210 | 25-75 | 0.58 |
| Lean Concrete | 175 | 51 | 100 | - | - |

- Types of Concrete Works

2.1.1	Underground Concrete

Concrete poured below Natural Surface Level with or without shuttering and shoring.

2.1.2	On-ground Concrete

Concrete poured by exiting formwork with necessary bracings on ground.

2.1.3	Elevated Concrete

Concrete poured by existing props, bracing and towers to support the formwork at higher levels.

- EFFECT OF MATERIALS STRENGTH

The cement and aggregate used have significant influence on strength properties. The use of unsound aggregates will produce large variations in the strength of concrete. In general, however, a good aggregate will develop the full strength of the cementing matrix and therefore should cause little variation in the product. Strength is increases slightly with larger size coarse aggregates.

- MATERIALS

- Cement

- The cement shall be fresh and of approved origin and manufacture, it shall be one of the following as may be specified by the Engineer.

- Ordinary or Rapid Hardening Portland Cement shall comply with the requirements of          PS 232 or BS 12.

- Sulphate-Resisting Portland Cement complying with the requirements of PS 612 or            BS 4027.

- Unless otherwise specified, ordinary Portland Cement shall conform the requirements of the standard specifications for Portland Cement, AASHTO Designation M55, ASTM Designation C150.

- Mill Certificates shall accompany delivery of the material to the work.

- There shall be sufficient cement on site to ensure that each section of work is completed without interruption. If the cement is supplied by the Employer, the contractors should inform the Employer of his requirements much before its use in construction.

- Cement reclaimed from cleaning of bags or from leaky containers shall not be used.

- The Contractor shall provide and erect (at his cost) in a suitable place dry, well ventilated, weather-proof and waterproof shed of sufficient capacity to store the cement.

- The cement shall be used as soon as possible after delivery and cement which has become stale or unsuitable through absorption of moisture from the atmosphere or otherwise shall be rejected and removed immediately from the site at the Contractor’s expense. Any cement in containers damaged so as to allow the contents to spill or access of the atmosphere to the cement prior to opening at the time of concrete mixing shall be rejected and removed immediately from the site at the Contractor’s expenses.

- Cement stored through a monsoon or for more than six months should not be used in reinforced concrete.

- Different brand or different types of cement from the same mill, or the same brand or type from differing mills shall not be mixed or used alternately in the same item of construction unless authorized by the Engineer, after preparing new mix designation.

- Water

The water for curing, for washing aggregates and for mixing shall be fresh, clean, clear, potable and free from impurities & deleterious matter. Water containing less than 2000 parts per million (ppm) of total dissolved solids with following limits on contaminants, recommended to be used for concrete.

| Contaminants | Conventional Reinforced Concrete | Pre-stressed Concrete | Non-reinforced Concrete |
| --- | --- | --- | --- |
| OIL | None | None | None |
| CHLORIDES, ppm | 1,000 | 650 | 2,000 |
| SULPHATES, ppm | 1,000 | 800 | 1,500 |

In no case shall the water contain an amount of imparities that will cause arrange in the setting time of Portland cement of more than twenty five (25) percent nor a reduction in the compressive strength of mortar at fourteen (14) days of more than five (5) percent when compared to the result obtained with distilled water.

In addition to the above requirements, water for curing concrete shall not be contain any impurities in a sufficient amount to cause discoloration of the concrete or produce etching of the surface.

When required by the Engineer, the quality of the mixing water shall be processed by the standard method of test for quality of water to be used in concrete. AASHTO Methods of Sampling and Testing, Designation T26 and BS 12.

- AGGREGATES

- The quality and sources of all aggregates for concrete shall be approved by the Engineer before the materials are delivered to the site. Aggregates shall be obtained from a source known to produce aggregates satisfactory for concrete and shall be chemically inert, strong, hard, durable of limited porosity and free from adhering coatings, clay lumps, coal and coal residues, and organic or other impurities that may cause corrosion of the reinforcement or may impair the strength or durability of the concrete.

The aggregates, if directed by the Engineer shall be tested in accordance with the requirements of BS 812.

- Wherever feasible the nominal maximum size of aggregate for cast-in-place reinforced concrete slabs and other thin members shall also be 20mm (¾”). If there are difficulties in placing such a concrete the maximum size may be restricted to 12mm (½“) provided the requirements for strength are satisfied.

- The nominal maximum size of the aggregate for precise concrete shall not be large than one-fifth of the narrowest dimension between sides of forms, one-third of the depth of slabs, nor three-fourths of the minimum clear distance between reinforcing bars or between bars and forms, whichever is least. In precast columns the nominal maximum size of the aggregate shall be limited as above but shall not be larger than two-thirds of the minimum clear distance between bars.

- Fine Aggregates

Fine aggregate shall be hard coarse sand, crushed stone or gravel screening and shall conform to the requirements of PS 243 and / or BS 882. Only fine aggregate of grading zones 1 to 3 (BS 882) mainly passing a 3/16 inch BS shall be used, aggregate of zones 4 may be used for special mixes.

Fine aggregates shall conform to BS 882 Grading Zone 1 and 2 and shall be graded as follows:

Percentage (by weight) passing

| B.S. Sieve | Grading Zone 1 | Grading Zone 2 |
| --- | --- | --- |
| 3/8” | 100 | 100 |
| 3/16” | 90 - 100 | 90 - 100 |
| No. 7 | 60 - 95 | 75 - 100 |
| No. 14 | 30 - 70 | 55 - 99 |
| No. 25 | 15 - 34 | 35 - 59 |
| No. 52 | 5 - 20 | 8 - 30 |
| No. 100 | 0 - 10 | 0 - 10 |

- Coarse Aggregate

Coarse Aggregates shall be crushed stones, comprising of angular or rounded in shape and shall have granular or crystalline or smooth (but not glassy) non powdery surface free from friable, flaky and laminated pieces, mica and shale and all such matters as may be injurious to the concrete. All coarse aggregate shall conform to BS.

- Storage

All aggregates shall be stored on properly constructed paving and bins or as directed by the Engineer. There shall be a physical partition between the stockpiles of coarse and fine aggregates. If required aggregates shall be washed and screened to the satisfaction of the Engineer.

- Sieve Analysis

Sieve analysis of all the aggregates to be used in the works shall be carried out as and when required by the Engineer. All aggregates shall be subject to the approval of the Engineer.

f)	Rejected Aggregates

Any aggregates not found to be of the specified standard shall be rejected by the Engineer and all such rejected material shall have to be removed from site without delay.

Concrete structures constructed with rejected aggregates shall be dismantled and rebuilt at the Contractor's expense.

- COMPOSITION OF CONCRETE

- Cement and Aggregates

The fine aggregate and the coarse aggregates shall be measured separately by weight. The proportions weight of cement to fine aggregate and coarse aggregate shall be determined by mix design approved by the Engineer. The Contractor shall propose mix designs duly supported by cube tests for each grade of concrete for the approval of the Engineer.

- Water Cement ratio

The quality of water used shall be just sufficient to produce a dense concrete of adequate strength and workability for its purpose. For all external if work and foundations the water / cement ratio shall not exceed 0.55.

- Workability

The workability shall be controlled by direct measurement of the water content, allowance being made for any water in the fine and coarse aggregates. The concrete shall be just sufficiently workable to be placed and compacted, without difficulty, by the means available.

The workability shall be determined by either the slump or compaction factor tests as directed by the Engineer and these shall be performed in accordance with the methods given in PS 422 and PS 717. The slump or compaction factor for each grade of concrete shall be determined during the Preliminary Test mixes and the value obtained shall not be modified without the written consent of the Engineer. Unless otherwise permitted or specified, the concrete shall be pro- portioned and produced to have a slump of 75 mm or less if consolidation is to be done by vibration, and 125mm or less if consolidation is to be by methods other than vibration. A tolerance of upto 25mm above the indicated maximum shall be allowed for individual batches provided the average for all batches or the most recent 10 batches tested, whichever is fewer, does not exceed the maximum limit, Concrete of lower than usual slump may be used provided it is properly placed and consolidated.

- PLASTICIZER / ADMIXTURE

Admixtures shall only be allowed to be used with written permission of the Engineer. If air-entraining agents, water reducing agents, set retarders or strength accelerators are permitted to be used, in exact dosage recommended by the manufacturer.

The Contractor shall submit concrete mix design for approval, prior to the commencement of work. No work shall be carried out without getting written approval of concrete Design Mix from the Engineer.

- FORMWORK

  - General

- The form work shall be inclusive of all labour, material, workmanship and alike. All form work and the Contractor thereto shall design supports and relevant drawings shall be submitted to the Engineer and his Representative for approval before the work is put in hand. Such an approval shall not relieve the contractor from all the obligations of the contract or give rise to any claim.

- Earth cuts shall not be used as forms for vertical surface of reinforced concrete work unless required or permitted.

- Mud centering shall not be permitted.

- Formwork shall be of wrought timber, steel, plywood, proprietary building boards which gives the required finish to the surface of concrete. Wooden formwork shall be free from loose knots and shall be well seasoned.

- The formwork shall conform to the shape, line and dimension as shown on the plans and be so constructed as to remain sufficiently rigid during the placing and compacting of the concrete and shall be sufficiently tight to prevent loss of liquid from the concrete.

The design and engineering of the formwork as well as its construction shall be the responsibility of the contractor. Where necessary to maintain the specified tolerances the formwork shall be cambered to compensate for anticipated deflections in the formwork due to the weight and pressure of the fresh concrete and due to construction loads. Design of formwork shall, in general, conform to ACI 347-68.

In normal circumstances (General where temperatures are above 20oC (68oF)) and where ordinary cement is used forms may be struck after expiry of the following periods:

| Concrete work | Period |
| --- | --- |
| Walls, columns and vertical sides of beams | 48 hours or as may be decided by the Engineer. |
| Slabs (shores or props left under) | 6 days |
| Beams soffits (shores or pros left under) | 12 days |
| Removal of shores or props to slabs: - Spanning upto 14 ft - Spanning above 14 ft. | 10 days 16 days |
| Removal of shores or props to beams: - Spanning upto 20 ft - Spanning above 20 ft. | 18 days 25 days |

For rapid hardening cement 3/7 of the above period will be sufficient in all cases except vertical sides of slabs beams and columns which should be retained for minimum 24 hours.

  - Making Forms

The form work for all concrete work to be cast in situ shall be made of sound and properly seasoned timber or other approved material for all works above ground water table. For all works below ground water table form work of steel shall be used. These forms work shall be properly jointed and erected with packing material to provide water tight forms. These forms shall be properly cleaned to give a smooth finished surface and shall be rigidly formed and designed by the Contractor to the shapes and forms as per drawings in accordance with the best existing practices so as to be able to withstand, without displacement, deflection or deformation movements of any kind, the pressure of the moist concrete and all other loads.

  - Rigid with Allowance for Camber and Bulges

It shall be fabricated and erected in position, perfect in alignment, levels and true to plumb and shape and securely braced so as to enable it to with stand all weights, live and vibrating, to be endured during placing of concrete and its subsequent hardening till the form work is struck. It shall be sufficiently rigid as not to loose its form or bulge, or deflect and to give the finished concrete the required lines, plumb, size and shape.

  - Materials and Labour

The Contractor shall supply all materials and labour, necessary for a good and speedily erection of form work such as shuttering, planks, struts, bolts, stays, gangways, boards, fillets etc. and shall do all that is essential in executing the job in a workman like manner to the satisfaction of the Engineer.

  - Form work not to interfere or injure work

The form work shall be so designed and arranged as not to unduly interfere with concrete, during its placing and easy to be removed without injuring the finished concrete.

Wedges, clamps, bolts and the rods shall be used, when permitted and where practicable, in making the form work rigid and in holding it to true position.

  - Joints in Formwork

All joints in the form work shall be sufficiently closed to prevent undue leakage of mortar from concrete or show any appearance of leaking mortar on concrete surface.

  - Treatment and Inspection of Forms

All rubbish particularly chipping, shavings and saw dust etc. shall be removed from the interior of the forms, immediately before placing concrete. Forms shall be coated with approved mould oil before reinforcement is placed. Surplus oil on forms and any oil on reinforcing steel shall be removed.

  - Removal of Shuttering

No struts or timbering which serve the purpose of supporting the shuttering or centering shall be struck and removed before the minimum periods for the main classes of work given as under:

| ## Removal of Shuttering | Cold Weather Days | Normal Weather Days |
| --- | --- | --- |
| Beams sides, walls and Columns (unloaded) Slabs soffits (Props left under) Removal of props to slabs Beams soffits (Props left under) | 5 10 18 13 | 3 7 14 10 |

Struts or other timbers or supports, the removal of which may cause the transference of load to the finished work, shall be kept in place for three weeks after the placing of the concrete.

  - Injury or damage

The Contractor shall be responsible for any injury to the work and any consequential damages caused by or arising from the removal and striking of forms, centering and supports, and any advice, permission or approval given by the Engineer or his Authorised Representative, related to the removal and striking of forms, centering and supports shall not relieve the Contractor from the responsibilities herein defined.

  - Treatment after Removal of Forms

Any minor surface honey combing or other irregularities are to be properly made good immediately upon the removal of the form work and the surface made good to the satisfaction of the Engineer and his Representative. Any small voids shall be neatly stopped with cement mortar consisting of one part of cement to two parts of sand and the whole surface rubbed over with carborundum stone and cement wash and bring the whole to a smooth and pleasing finish and uniform colour.

  - Form work shall not be measured or paid for separately and shall be deemed to be included in the unit price of concrete whether cast-in-situ or precast and subsequently fixed in position.

- BATCHING OF CONCRETE

The Contractor shall provide such means and equipment as are required to determine accurately control the amount of each separate ingredient entering the concrete. Such means, the equipment and its operation shall at all times be subject to approval by the Engineer. The amount of cement, water, sand and each size of coarse aggregate entering each batch of concrete shall be determined by weighing or by volumetric measurement if permitted by the Engineer.

- MIXING CONCRETE

  - Mixings General

The concrete shall be mixed only in the quantity required for immediate use. Concrete that has developed an initial set shall be rejected.

Concrete shall be thoroughly mixed in a mixer of an approved size and type that will ensure a uniform distribution of the materials throughout the mass.

All concrete shall be mixed in mechanically operated mixers. Mixing plant and equipment for transporting and placing concrete should be arranged with an ample auxiliary installation to provide a minimum supply of concrete in case of breakdown of machinery or in case the normal supply of concrete should be disrupted. The auxiliary supply of concrete shall be sufficient to complete the casting of a section up to a construction joint

Equipment having components made of aluminum or magnesium alloys, which would have contacted with plastic concrete during mixing, transporting or pumping of Portland cement concrete, shall not be used.

Concrete mixers shall be equipped with adequate water storage and a device for accurately measuring and automatically controlling the quantity of water used.

Materials shall be measured by weighing except as otherwise specified or where other methods are specifically authorized by the Engineer. The apparatus provided for weighing the aggregates and cement shall ensure accurate measurement of each ingredient.

The accuracy of all weighing devices except that for water shall be such that successive quantities can be measured to within one (1) percent of the desired value. Cement in standard packages (bags) approved by the Engineer need not be weighed. The water measuring device shall be accurate to plus or minus half percent ± 0.50%. All measuring devices shall be subject to the approval of the Engineer. Scales and measuring devices shall be tested at the expense of the Contractor as frequently as the Engineer may deem necessary to ensure their accuracy.

Weighing equipment shall be isolated so that vibration or movement of other operating equipment do not effect the accuracy of reading. When the entire plant is running, the scale reading at cut-off shall not vary from the weight designated by the Engineer more than one (1) percent for cement, one and half (1½) percent for any size of aggregate or one (1) percent for the total aggregates in any batch. Where volumetric measurements are authorized by the Engineer the weight proportion shall be converted to equivalent volumetric proportions. In such cases, suitable allowances shall be made for variations in the moisture condition of the aggregates, including the bulking effect in the fine aggregates. Boxes or similar containers of the exact volume required shall be filled and struck off. Measurement by wheel barrow volumes will not be permitted.

  - Mixing at Site

Concrete mixers may be of the revolving drum or the revolving blade type and the mixing drum or blades shall be operated uniformly at the mixing speed recommended by the manufacturer. The pick-up and throw-over blades of mixer shall be restored or replaced when any part or sections is worn two and half (2.5) cms. or below than the original height of the manufacturer's design. Mixers and agitators, which have an accumulation of hard concrete or rnortar, shall not be used.

When bulk cement is used and volume of the batch is one cubic meter or more, the scale and weigh hopper for Portland cement shall be separate and distinct from the aggregate hopper or hoppers. The discharge mechanism of bulk cement weigh hopper shall be interlocked against opening before the full amount of cement is in the hopper. The discharging mechanism shall also be interlocked against opening when the amount of cement in the hopper is underweight by more than one percent or overweight by more than three (3) percent of the amount specified

When the aggregates contain more water than the quantity necessary to produce a saturated surface-dry condition, representative samples shall be taken and the moisture content determined for each kind of aggregate.

The temperature of mixed concrete immediately before placing, shall be not more than thirty two (32) degree C. Aggregates and water shall be cooled as necessary to produce concrete within this temperatures limit. If ice is used to cool the concrete, discharge of the mixer will not be permitted until all ice is melted.

The batch shall be so charged into the mixer that some water will enter in advance of cement and aggregates. All water shall be in the drum by the end of the first quarter of the specified mixing time.

Cement shall be batched and charged into the mixer by means that will not result in loss due to the effect of wind, or in accumulation of cement on surfaces of conveyors or hoppers, or in other conditions which reduce or vary the required quantity of cement in the concrete mixture.

The entire contents of a batch mixer shall be removed from the drum before materials for a succeeding batch are placed therein. The materials composing a batch except water shall be deposited simultaneously into the mixer.

All concrete shall be mixed for a period of not less than one and half (1½) minutes after all materials, including water, are in the mixer. During the period of mixing, the mixer shall operate at the speed for which it has been designed.

Mixers shall be operated with an automatic timing device that can be locked by the Engineer. The time device and discharge mechanism shall be so interlocked that during normal operation no part of the batch will be discharged until the specified mixing time has elapsed. In case of failure of the timing device, the Contractor will be permitted to operate while it is being repaired, provided he furnishes an approved timepiece equipped with minute and second hands. lf the timing device is not repaired within twenty four (24) hours, further use of the mixer will be prohibited until repairs are made.

The first batch of concrete material placed in the mixer shall contain cement, sand and water in excess to the requirement of mix, to ensure that the drum does not extract mortar from the mix changing its design characteristics. When mixing is to stop for a period of one hour or more, the mixer shall be thoroughly cleaned.

  - Plant Mixing

At central mixing plant, batches shall be discharged from the weighing hopper into the mixer either directly by gravity or by an elevating container large enough to contain the batch. The plant shall be arranged to ensure that there is no loss of cement during transfer from weighing hopper to the mixer drum. The mixing time shall neither be less than fifty (50) second, nor more than ninety (90) seconds.

The plasticizer, accelerator or retarder or water reducing admixture, if required, shall be fed separately at the rate recommended by the manufacturer, as established by laboratory trials.

  - Transit Mixing

Truck mixers, unless otherwise authorized by the Engineer shall be of the revolving drum type, watertight and so constructed that the concrete can be mixed to ensure a uniform distribution of materials throughout the mass. All solid materials for the concrete shall be accurately measured and charged into the drum at the proportioning plant. The truck mixer shall be equipped with a device by which the quantity of water added can be readily verified. The mixing water may be added directly to the batch, in case the concrete batch is poured within twenty five(25) minutes of adding water.

The maximum size of batch in truck mixers shall not exceed the maximum rated capacity of the mixer as stated by the manufacturer, and stamped in metal on the mixer. Truck mixing shall be continued for not less than fifty (50) revolutions after all ingredients including water, are in the drum. The mixing speed shall not be less than six (6) rpm, nor more than ten (10) rpm.

Mixing shall begin within thirty (30) minutes after the cement has been added either to the water or aggregate but when cement is charged in to a mixer drum containing water or surface-wet aggregate and when the temperature is above thirty two (32) degree C, this limit shall be reduced to fifteen (15) minutes. The limitation in time between the introduction of the cement to the aggregate and the beginning of the mixing may be waived when, in the judgment of the Engineer the aggregate is sufficiently free from moisture, so that there will be no harmful effects on the cement.

  - Partial Mixing at the Central Plan

When a truck mixer, or an agitator provided with adequate mixing blades, is used for transportation the mixing time at the stationary plant mixer may be reduced to thirty (30) seconds and the mixing completed in a truck mixer/agitator. The mixing time in the truck mixer or agitator equipped with adequate mixing blades shall be as specified for truck mixing.

  - Stiff Concrete Mix

For mixing concrete of zero slump to be laid by pavers, gravity mixer shall not be used. Only force mixer of moving blades shall be allowed to ensure homogenous mix.

  - Hand Mixing

Hand mixing of materials shall not be allowed in any case.

- CONSOLIDATION

- All concrete shall be consolidated by vibration, spading, roding or forking so that the concrete is thoroughly worked around the reinforcement, around embedded items and into corners of forms, eliminating all air or stone packets which may cause honey-combing, pitting or planes of weakness. Internal vibrators shall have a minimum frequency of 8000 vibrations per min. and sufficient amplitude to consolidate the concrete effectively. They shall be operated by competent workmen, use of vibrators to transport concrete with forms shall not be allowed. Vibrators shall be inserted and withdrawn at points approximately 450mm (18”) apart. At each insertion, the duration shall be sufficient to consolidate the concrete but not sufficient to cause segregation, generally from 5 to 15 seconds. A spare vibrator shall be kept on the job site during all concrete placing operations.

- Over-vibration or vibration of very wet mixes is harmful and should be avoided.

- HAULING & DELIVERY OF MIXED CONCRETE

- Hauling

Mixed concrete may be transported to the delivery point in truck agitators or truck mixers operating at the speed designated by the manufacturer, provided the consistency and workability of the mixed concrete upon discharge at the delivery point is suitable for adequate placement and consolidation in place.

Truck agitators shall be loaded not to exceed the manufacturer’s rated capacity. They shall maintain the mixed concrete in a thoroughly mixed and uniform mass during hauling.

Bodies of non-agitating hauling equipments hall be so constructed that leakage of the concrete mix, or any part thereof, will not occur at any time, and they shall be self-cleaning during discharge.

For zero slump concrete to be laid be paver, concrete will be allowed to be hauled in open trucks. However concrete hauled in open-top vehicles shall be protected during hauling against rain or exposure to the sun for more than twenty (20) minutes when the ambient temperature exceeds twenty five (25) degree C.

No additional water shall be incorporated in to the concrete during hauling or after arrival at the delivery point.

The rate of discharge of mixed concrete from truck mixer agitators shall be controlled by the speed of rotation of the drum in the discharge direction with the discharge gate fully open.

When a truck mixer or agitator is used for transporting concrete to the delivery point, discharge shall be completed within one hour, or before two hundred fifty (250) revolutions of the drum or blades, whichever comes first, after the introduction of cement to the aggregates. Under conditions contributing to quick stiffening of the concrete, or when the temperature of the concrete is thirty (30) degree c or above, a time less than one hour will be required except when retarder is used in which case it shall be one (1) hour.

When non-agitating hauling equipment is used for transporting concrete to the delivery point, discharge shall be completed within one hour after the addition of the cement to the aggregates. Under conditions contributing to quick stiffening of the concrete, or when the temperature of the concrete is thirty (30) degree C or above, the time between the introduction of cement to the aggregates and discharge shall not exceed forty five (45) minutes.

- Delivery

The organization supplying concrete shall have sufficient plant capacity and Transportation vehicles to ensure continuous delivery at the rate required. The rate of the delivery of concrete during concreting operations shall be such as to provide for the proper handling, placing, and finishing of the concrete. The rate shall be such that the interval between batches shall not exceed twenty (20) minutes. The methods of delivering and handling the concrete shall be such as will facilitate placing with the minimum rehandling and without damage to the structure of the concrete.

- Retempering

The concrete shall be mixed only in such quantities as are required for immediate use and any concrete that has developed initial set shall not be used. Concrete that has partially hardened shall not be retempered or remixed.

- HANDLING AND PLACINE CONCRET

- General

In preparation for the placing of concrete all saw dust, chips and other construction debris and extraneous matter shall be removed from inside the formwork, and struts, stays and braces serving temporarily to hold the forms in correct shape and alignment, pending the placing of concrete at their locations, shall be removed when the concrete placing has reached an elevation rendering their services unnecessary. These temporary members shall be entirely removed from the forms and not buried in the concrete.

No concrete shall be used that does not reach its final position in the forms within the time stipulated above under item "Hauling and Delivery of Mixed Concrete".

Concrete shall be placed so as to avoid segregation of the materials and the displacement of the reinforcement. The use of long troughs, chutes and pipes for conveying concrete to the forms shall be permitted only on written authorization of the Engineer. In any case the Engineer will reject the use of equipment for concrete transportation that will allow segregation loss of fines, or in any other way will have a deteriorating effect on the concrete quality.

Open troughs and chutes shall be of metal or metal lined; where steep slopes are required the chutes shall be equipped with baffles or be in short lengths that reverse the direction of movement.

All chutes, troughs and pipes shall be kept clean and free from coatings of hardened concrete by thoroughly flushing with water after each run; water used for flushing shall be discharged clear off the structure.

When placing operations would involve dropping the concrete more than one and half (1½) meters, it shall be conveyed through sheet metal or other approved pipes. As far as practicable the pipe shall be kept buried in the newly placed concrete. After initial set of the concrete the forms shall not be jarred and no loading of any kind shall be placed on the ends of projecting reinforcement bars.

The concrete shall be placed as nearly as possible to its final position and the use of vibrators for extensive shifting of the mass of fresh concrete will not be permitted.

- Pneumatic Placing

Pneumatic placing of concrete will be permitted only if authorized by the Engineer. The equipments hall be so arranged that no vibration will occur that might damage freshly placed concrete.

Where concrete is conveyed and placed by pneumatic means, the equipment shall be suitable in kind and adequate in capacity for the work.

The machines hall be located as close as practicable to the work. The discharge lines shall be horizontal or inclined upwards from the machine. At the conclusion of placing the concrete, the entire equipment shall be thoroughly cleaned.

- Pumping

The placing of concrete by pumping will be permitted only if specified in the Special Provisions or if authorized by the Engineer. The equipment shall be so arranged that no vibration will occur that might damage freshly placed concrete.

Where concrete is conveyed and placed by mechanically applied pressure the equipment shall be suitable in kind and adequate in capacity for the work. The operation of the pump shall be such that a continuous stream of concrete without air pockets is obtained. When pumping is completed the concrete remaining in the pipeline if it is to be used, shall be ejected in such a manner that there will be no contamination of the concrete or separation of the ingredients. After this operation, the entire equipment shall be thoroughly cleaned.

- Placing Concrete Under Water

Concretes hall not be placed under water except where inevitable in which case approval must be sought from the Engineer and the work carried out under his immediate supervision. In this case the method of placing shall be as hereinafter specified.

Concrete deposited under water shall be Class A concrete with a minimum cement content of three hundred fifty (350) Kg per cubic meter of concrete.

The slump of concrete shall be maintained between ten (10) and fifteen (15) cm. To prevent segregation it shall be carefully placed in a compact mass, in its final position, means of a tremie, a bottom-dump bucket, or other approved means, and it shall not be disturbed after being placed. Water must not be allowed to flow past the fresh concrete surface.

A tremie shall consist of a tube having a diameter of not less than 25 cm constructed in sections having flanged couplings fitted with gaskets with a hopper at the top. The tremie shall be supported. So as to permit free movement of the discharge end over the entire top surface of the work and so as to permit rapid lowering when necessary to retard or stop the flow of concrete. The discharge end shall be closed at the start of work so as to prevent water entering the tube and shall be completely submerged in concrete at all times; the tremie tube shall be kept full to the bottom of the hopper. When a batch is dumped into the hopper, the flow of concrete shall be induced by slightly raising the discharge end, but always keeping it in the placed concrete. The flow shall be induced until the work is completed.

When the concrete is placed with a bottom-dump bucket, the top of the bucket shall be open. The bottom doors shall open freely downward and outward when tripped. The bucket shall be completely filled and slowly lowered to avoid backwash. It shall not be dumped until it rests on the surface upon which the concrete is to be deposited and when discharged shall be withdrawn slowly until well above the concrete.

Dewatering may proceed when the concrete seal is sufficiently hard and strong. All laitance or other unsatisfactory material shall be removed from the exposed surface by scraping, chipping or other means, which will not injure the surface of the concrete.

- Compaction

Concrete, during and immediately after placing shall be thoroughly compacted, except lean concrete under footings and concrete deposited under water. Concrete in walls, beams, columns, etc shall be placed in horizontal layers not more than thirty (30) centimeters thick except as hereinafter provided. When less than a complete layer is placed in one operation, it shall be terminated in a vertical bulkhead. Each layer shall be placed and compacted before the preceding layer has taken initial set to prevent injury to the green concrete and avoid surfaces of separation between the layers. Each layer shall be compacted so as to avoid the formation of a construction joint with a preceding layer, which has not taken an initial set.

The compaction shall be done by mechanical vibration. The concrete shall be vibrated internally unless special authorization of other methods is given by the Engineer or is provided herein. Vibrators shall be of a type, design, and frequency approved by the Engineer. The intensity of vibration shall be such as visibly to affect a mass of concrete with a 3 cm slump over a radius of at least half a meter. The Contractor shall provide a sufficient number of vibrators to properly compact each batch immediately after it is placed in the forms. Vibrators shall be manipulated so as to thoroughly work the concrete around the reinforcement and embedded fixtures and into the corners and angles of the forms and shall be applied at the point of placing and in the area of freshly placed concrete. The vibrators shall be inserted into and withdrawn from the concrete slowly. The vibration shall be of sufficient duration and intensity to compact the concrete thoroughly but shall not be continued at any one point to the extent that localized areas of grout are formed. Application of vibrators shall be at points uniformly spaced and not farther apart than twice the radius over which the vibration is visibly effective. Vibration shall not be applied directly to the reinforcement or to sections or layers of concrete that have hardened to the degree that the concrete ceases to be plastic under vibration. It shall not be used to make concrete flow in the forms over distances so great as to cause segregation and vibrators shall not be used to transport concrete neither in the forms nor in troughs or chutes.

Vibration shall be supplemented by such external vibrator as is necessary to ensure smooth surfaces and dense concrete along form surfaces and in corners and locations impossible to reach with the normal vibrators.

- Concrete in Hot & Cold Weather

- Concrete in hot weather

Concerting shall be avoided in timely hot weather and shall be done only in better part of the day. It should be particularly avoided in dry, hot and windy weather for member with large surface areas exposed to the weather. Aggregate shall be carefully stored under shelter and shall be sprinkled with cold water from time to time to check high temperature.

Water, to be used for concreting, shall be carefully used and, if necessary, crashed ice shall be added to bring the temperature as low as 70O F in order to at resulting temperature of concrete is not above 100O F. Execs of water shall not be added and the specified water of cement ratio shall be maintained.

- Concert In Cold Weather

When depositing concert is unavoidable at a temperature below 36O F, precaution shall be taken to ensure that the concert shall have a temperature of at least 40O F, at the time of placing of concert. The temperature of concert shall be maintained at not less than 40O F until it is thoroughly hardened. Sand or other material shall not be used in the prevention of freezing and no frozen materials or material containing ice or snow shall be used.

- Surface finishes/ rendering

- General

Concrete surface finishes shall be classified as follows:

Bridge Deck Surface Finish

Sidewalk Surface Finish

Ordinary Surface Form Finish

Class 1 Surface Form Finish

The bridge deck surface finish shall be given to the surface of the bottom Slabs of all box type underpass structures.

The requirements for sidewalk surface finish apply to the surface of the bottoms labs in box culverts, except that the acceptable variation from a three-meter straightedge shall be 10 mm, and brooming shall be omitted.

The ordinary surface form finish shall be the final finish applied to all Surfaces after removal of forms, unless otherwise specified or called for on the drawings.

The Class 1 surface form finish shall be applied only where specified or as required by the Engineer when the ordinary surface finish did not produce the required smooth, even surface of uniform texture and appearances.

- Bridge Deck Surface Finish

A smooth riding surface of uniform texture, true to the required grade and cross-section, shall be obtained on all bridge roadway decks. The Contractor may use hand tools, or finishing machines or a combination of both, conforming to the requirements specified herein for finishing bridge roadway deck concrete.

Finishing of concrete placed in bridge decks shall consist essentially of compacting and striking off the surface of the concrete as placed and floating with longitudinal floats the surface so struck off.

The placing of concrete in bridge roadway decks will not be permitted until the Engineer is satisfied that the rate of producing concrete will be sufficient to complete the proposed placing and finishing operations within the schedule time, that experienced finishing machine operators and concrete finishers are employed to finish the deck, that fogging equipment and all necessary finishing tools and equipment are on hand at the site of the work and in satisfactory condition for use. Finishing machines shall be set up sufficiently in advance of use to permit inspection by the Engineer during the daylight hours before each Pour.

The adjustment and operation of deck finishing machines shall be verified by moving the machine over the full length of the deck section to be placed and traversing the float completely across all end bulkheads before placement of concrete is begun.

Unless adequate lighting facilities are provided by the Contractor, the placing of concrete in bridge decks shall cease at such time that finishing operations can be completed during daylight hours.

Rails for the support and operation of finishing machines and headers for hand-operated strick-off devices shall be completely in place and firmly secured for the scheduled length for concrete placement before placing of concrete. Rails for finishing machines shall extend beyond both ends of the scheduled length for concrete placement to a sufficient distance that will permit the float of the finishing machine to fully clear the concrete to be placed. Rails or headers shall be adjustable for elevation and shall be set to elevations, with allowance for anticipated settlement, camber, and deflection of false work, as required to obtain a bridge roadway deck true to the required grade and cross-section. Rails or headers shall be of a type and shall be so installed that no springing or deflection will occur under the weight of the finishing equipment and shall be so located that finishing equipment may operate without interruption over the entire bridge roadway deck to be finished.

Rails or headers shall be adjusted as necessary to correct for unanticipated settlement or deflection which may occur during finishing operations.

Should settlement or other unanticipated events occur, which in the opinion of the Engineer would prevent pouring of bridge deck conforming to the requirements of these specifications, placing of deck concrete shall be discontinued until corrective measures satisfactory to the Engineer are provided. In the event satisfactory measures are not provided prior to initial set of the concrete in the affected area, the placing of concrete shall be discounted and a bulkhead installed at a location determined by the Engineer. All concrete in place beyond the bulkhead shall be removed.

Unless otherwise permitted by the Engineer, bridge deck concrete shall be placed in a uniform heading approximately parallel to the bridge pier or bent caps. The rate of placing concrete shall be limited to that which can be finished before the beginning of initial  set except that concrete for the deck surface shall not be placed more than (03) meters ahead of strick off.

After the concrete has been placed, compacted and consolidated, the surface of the concrete shall be carefully struck off by means of a hand operated strick board operating on headers, or by a finishing machine operating on rails. A uniform deck surface true to the required grade and cross-section shall be obtained.

Following strike off, the surface of the concrete shall be floated longitudinally. In the event strick-off is performed by means of a hand-operated strike board, two (2) separate hand-operated float boards for longitudinal floating shall be provided. The first float shall be placed in operation as soon as the condition of the concrete will permit and the second float shall be operated as far back of the first float as the workability of the concrete will permit.

In the event the strike off is performed with a finishing machine, longitudinal floating of the concrete shall be performed by means of a hand-operated float board or a finishing machine equipped with a longitudinal wooden float. The longitudinal wooden float on the finishing machine shall have a length of not less than two and half (2.5) meters nor more than three and half (3.5) meters. When both strike off and longitudinal floating are to be performed by finishing machines, one machine, with operator, shall be used for strike off and a second machine, with a second operator, shall be used for longitudinal floating. Longitudinal floating may be performed with the same finishing machine that is used for strike off provided that the length of deck unit being placed is not more than 10 meters and the strike off operation is completed for said deck unit before the condition of the concrete requires that longitudinal floating be started.

Finishing machines used for strike off having a wheel base 1.8 meters or less shall be followed by 2 separate hand-operated float boards for longitudinal floating. All the provisions in this item pertaining to hand-operated float boards shall apply to the 2 separate float boards for longitudinal floating.

Longitudinal floats, either hand-operated or machine-operated, shall be used with the long axis of the float parallel to the center line of the bridge roadway. The float shall be operated with a combined longitudinal and transverse motion planning off the high areas and floating the material removed into the low areas. Each pass of the float shall lap the pervious pass by one-half the length of the float. Floating shall be continued until a smooth riding surface is obtained.

In advance of curing operations, the surface of the concrete shall be textured by brooming with a stiff bristled broom or by other suitable devices, which will result in uniform scouring. The operation shall be performed at a time and in a manner to produce a hardened surface having a uniform texture.

Hand-operated float boards shall be from three and half (3.5) to five (5) meter long, ribbed and trussed as necessary to provide a rigid float and shall be equipped with an adjustable handle each end. The float shall be wood, not less than two and half (2.5) cms thick and from ten (10) cm to twenty (20) cm wide. Adjusting screws spaced as not to exceed 60 cms on centers shall be provided between the float and the rib. The float board shall be maintained free of twist and true at all time.

Hand-operated float boards shall be operated from transverse finishing bridges. The finishing bridges shall span completely the roadway area being floated & a sufficient number of finishing bridges shall be provided to permit operation of the floats without undue delay. Not less than two (2) transverse finishing bridges shall be provided when hand-operated float boards are used. When a finishing machine is used for longitudinal floating, one finishing bridge equivalent to the transverse finishing bridge specified herein shall be furnished for use by the Engineer.

All finishing bridges shall be of rigid construction and shall be free of excessive wobble and springing when used by the operators of longitudinal floats and shall be easily moved.

Immediate following completion of the deck finishing operations, the concrete in the deck shall be cured as specified in item 401.3.8 “Curing Concrete” hereinafter.

The finished surface of the concrete shall be tested by means of a straightedge three (3.0) meter long. The surface shall not vary more than three (3) mm from the lower edge of the straightedge. All high areas in the hardened surface in excess of three (3) mm as indicated by testing shall be removed by abrasive means. After grinding by abrasive mean has been performed, the surface of the concrete shall not be smooth or polished. Ground areas shall not be of uniform texture and shall present neat and approximately.

Where the concrete of the bridge deck is to be covered by bituminous surfacing, earth or other cover, two and half 2.5 cms or more in thickness, the surface of the concrete shall not vary more than nine (9) mm from the lower edge of the three (3) meter straightedge Bridge deck surface under the curbs, railings and sidewalk shall be struck off to the same plane as the roadway and left undisturbed when future widening is shown on the plans.

- Sidewalk Surface Finish

After the concrete has been placed it shall be compacted and the concrete shall be struck off by means of a strike board, floated with a wooden or cork floating and finish with a broom. An approved edging tool shall be used on all edges and at all expansion joints. Brooming shall be transverse to the line of traffic and if water is necessary, it shall be applied to the surface immediately in advance of brooming. The surface shall not vary more than six (6) mm under a three-meter straightedge, and the finished surface shall be free of blemishes.

- Ordinary Surface from Finish

Ordinary surface finish shall consist of filling holes or depressions in the surface of the concrete, repairing all rock pockets, removing stains and discoloration visible from traveled ways. Ordinary surface finish shall be applied to all concrete surfaces either as a final finish or preparatory to the Class 1 finish. On surfaces, which are to be buried underground or surface, which are enclosed, such as the cells of box girders; the removal of fins will not be required.

Except as provided herein, all from bolts and any metal placed for the convenience of the Contractor shall be removed to a depth of at-least two and half (2.5) cms below the surface of the concrete. All rock pockets and other unsound concrete shall be removed. The resulting holes or depression shall be cleaned and filled with mortar. From bolts projecting into the cells of box girders need not be removed unless permanent access is provided into the cells, in which case such bolts shall be removed flush with the surface of the concrete. Mortar used to fill bolt holes shall consist of one part cement and two parts sand. Other depressions and pockets shall be filled with either packed mortar or air blown mortar as directed by the Engineer. Mortar shall be cured in conformance with the requirements in item 401:3.8 (c) “Curing Structures”

If rock pockets or holes in the opinion of the Engineer, are of such an extent or character as to affect the strength of the structure materially or to endanger the life of the steel reinforcement, he may declare the concrete defective and require the removal and replacement of the portions of the structure affected.

- Class 1 Surface from Finish

Class 1 surface finish shall consist of finishing the surfaces of the structure as necessary to produce even surfaces of uniform texture and appearance, free of unsightly bulges, depression and other imperfections. The degree of care in building forms and character of materials used in form work will be a contributing factor in the amount of additional finishing required to produce even surfaces of uniform texture and appearance, free of unsightly bulges, depressions and other imperfections, and the Engineer shall be the sole judge in this respect.

After completion of the ordinary surface finish, areas which do not exhibit the required smooth, even surface of uniform texture and appearance shall be sanded with power sanders or other approved abrasive means until smooth, even surfaces of uniform texture and appearance are obtained. The use of power carborundum stones or disks will be required to remove bulges and other imperfections.

Class 1 surface finish shall not be applied until a uniform appearance can be obtained.

Class 1 surface finish may be required to be applied as the final finish for the following surfaces, unless otherwise directed by the Engineer.

- All form finish surfaces of bridge super-structures except the under surfaces between girders and the inside vertical surfaces of T girders.

- All surfaces of bridge piers, columns and abutments, and retaining walls above finished ground and to at least three tenth (0.3) meter below finished ground.

- All surfaces of open spandrel arch rings, spandrel columns and abutment walls.

- All surfaces of pedestrian undercrossing, except floors and surfaces to be covered with earth.

- Surface above finished ground of culvert headwalls, end walls and retaining walls.

- Surface inside of culvert barrels having a height of one and half (1.5) meters or more for a distance inside the barrel at least equal to the height of the culvert.

- All surface of railings.

- Surface Rendering

All faces of concrete which are to come in contact with backfill or pavement materials shall be applied two coats of hot bitumen of approved quality, before placing any material around concrete.

- FINISHING

    - Finishing of concrete surfaces shall be performed only by skilled workmen and as directed by the Engineer. Formed surfaces upon or against which backfill or concrete is to be placed will require no treatment after form removal except for the removal and repair of defective concrete and for the specified curing. Unformed surfaces that will be covered by backfill or by concrete shall be finished by sufficient leveling and screeding to produce an even uniform surface.

    - A hard steel trowel finish shall be applied to unformed surfaces that will be exposed or subjected to the action of flowing later. Floating and trowelling shall be started as soon as the screeded surface has stiffened sufficiently, and shall be the minimum necessary to produce a surface that is free from screed marks and is uniform in texture.

- CURING AND PROTECTION

- The concrete shall be kept continuously wet by the application of water for a minimum period of Seven (07) days after the concrete has been placed. Beginning immediately after placement, concrete shall be protected from premature drying, excessively hot or cold temperatures and mechanical injury and shall be maintained with minimal moisture loss at a relatively constant temperature for the period necessary for hydration of the cement and hardening of the concrete. Cotton mats, burlaps, rugs, carpets or earth or sand blanket, may be used as a curing medium to retain the moisture. The entire surface of the concrete shall be kept damp by applying water with a nozzle that so atomizes the flow that a mist and not a spray is formed.

At the expiration of the curing period the concrete surface shall be cleared of all curing mediums.

- Surface exposed to the air may be cured by the application of an imperious membrane (i.e. curing compound) with prior written approval from the Engineer.

- For concrete surfaces not in contact with forms, one of the following procedures shall be applied immediately after completion of placement and finishing:

- Ponding or continuous sprinkling.

- Application of absorptive mats or fabric kept continuously wet.

- Application of sand kept continuously wet.

- Curing shall be continued minimum Seven (07) days in the case of all concrete except concrete with Rapid-hardening Portland Cement for which the minimum period shall be three (03) days.

- TEST OF CONCRETE QUALITY

Samples of fine and coarse aggregate to be used shall be selected by the Engineer. It shall be the responsibility of the Contractor to designate the source or sources of aggregate and obtain the necessary samples and submit them for testing at least thirty (30) days before actual concreting operations are to being.

Samples of aggregates shall be obtained and tested in accordance with the following standard AASHTO methods.

- Sampling aggregates						T-2

- Sieve analysis 							T-27

- Amount of material passing the no. 200 sieve 		T-11

- Organic impurities						T-11

- Mortar Strength 						T-71

- Sodium sulphate soundness 				T-104

- Friable particles						T-112

- Abrasion loss							T-96

- Specific Gravity						T-84

- Absorption							T-85

- Production of Plastic Fines					T-210

- Fineness Modulus						T-27

- Sand Equivalent						T-17

- Potential Reactivity of Carbonate Rocks for

Concrete Aggregate (Rock Cylinder Method)		ASTM C 586

- Potential Alkali Reactivity of Cement

Aggregate Combinations

(Morta-Bar Method)						ASTM C 227

- Potential Reactivity of Aggregates

(Chemical Methods)						ASTM C 289

No aggregate for testing during the production of concrete shall be sampled at the discharge gates of the bins feeding the weight hopper. The Contractor, at his expense, shall provide safe and suitable facilities for obtaining the samples no concreting work on the project will be permitted until the Engineer signifies in writing his approval. Following the performance of the necessary tests, on all materials involved in making concrete.

- MEASUREMENT AND PAYMENT

  - General

Except otherwise specified herein or elsewhere in the Contract Documents no separate measurement and payment will be made for the under mentioned specified works related to the relevant items of the Bills of Quantities but shall not be limited to the following. The cost thereof shall be deemed to have been included in the quoted unit rate of the respective items of the Bills of Quantities.

  - Plain and Reinforced Concrete

- Measurement

Concrete shall be measured as executed but no deduction shall be made for the following:-

- Volume of any steel embedded in the concrete

- Volume occupied by water pipes, conduits etc. not exceeding 25 sq.cm. each in cross sectional area.

- Voids not exceeding 0.10 Sq.M. in work given in Sq.M. If any void exceeds 0.10 Sq.M. total void shall be deducted.

- Void, which are not to be deducted as specified above, refer only to opening or vents which are wholly within the boundary of measured areas. Openings or vents which are at the boundary, measured areas shall always be subject to deductions irrespective of size.

Concrete work shall be classified and measured separately as listed under items of Bills of Quantities.

Measurement of acceptably completed works of plain and reinforced cement concrete will be made on the basis of concrete placed and compacted in position within the neat lines of the structure as shown on the drawings or as directed by the Engineer.

- Payment

Payment will be made for the acceptable measured quantity of plain and reinforced cement concrete on the basis of unit rate quoted in the Bill of Quantities and shall constitute full compensation for all the works related to the items.

STEEL REINFORCEMENT

- SCOPE

The work under this section of specifications consists of furnishing, cutting, fabricating, bending and placing steel reinforcement and Welded wire, fabric of the type, size, shape and grade required in accordance with these specifications, in concrete structures or elsewhere as shown on the drawings and special provisions or as directed by the Engineer.

- APPLICABLE STANDARDS

Latest editions of the following Pakistan, British and ASTM Standards are relevant to these specifications wherever applicable.

Pakistan Standard

PS 241	Tensile Testing of Steel

PS 244	Bend test for Steel

PS 580	Rolled deformed Steel bars (intermediate grade) for concrete reinforcement.

PS 605	Rolled deformed steel bars (hard grade) for concrete reinforcement.

PS 606	Rolled formed Steel bars (structural grade) for concrete reinforcement.

PS 607	General technical delivery requirement for steel

British Standard

BS 693	General requirements for Oxy-acetylene welding of mild steel

BS 785	Hot rolled bars and hard drawn wire for the reinforcement of concrete

BS 1856	General requirement for the metal arc welding of mild steel

BS 4449	Hot rolled steel bars for reinforcement of concrete

BS 4461	Cold worked steel bars for reinforcement of concrete

BS 4466	Bending dimensions and scheduling of bars for the reinforcement of concrete

ASTM Standard

A 305	Minimum requirement for the deformations of deformed steel bars for concrete reinforcement

A 615	Deformed billet steel bars (Grades 40 and 60) for concrete reinforcement – AASHTO M-30.

In addition to the above, the latest editions of other Pakistan Standards, British standards, American Concrete Institute Standards, American Society for Testing and Materials Standards and other standard as may be specified by the Engineer for Special Material and construction are also relevant.

- MATERIAL AND SIZE OF BARS

    - Reinforcement for concrete shall conform to the respective Pakistan, British, ASTM, or other Standards as specified in the Drawings and in the Contract Documents or as may be specified by the Engineer.

    - Unless otherwise specified, all plain reinforcing bars shall comply with the requirements of BS 4449 for plain mild steel bars and shall have a minimum characteristic strength of 280 MPa (40000 psi)

    - Unless otherwise specified, all deformed reinforcing bars shall comply with the requirements of BS 4461 for deformed cold worked new stock billet steel bars and shall have minimum characteristic strength of 460 MPa (60000 psi).

    - If the reinforcement is supplied by the Employer, the Contractor should inform the Employer of his requirements much before its use in construction.

    - Reinforcement of all types is to be stored at on site in an approved manner so as to avoid damage.

    - If the reinforcement is supplied by the Employer, the Contractor should report immediately on receipt of any consignment, any deviation from the standard of three enforcement bars beyond those allowed in respective standards. If the Engineer directs, the Contractor shall test the samples of reinforcement at his cost and submit to him the test report.

    - Steel wire mesh reinforcement shall conform to requirements of ASTM Designation A 185 - 64 or BS 4483, 1969: Standard Specifications for Welded Steel Wire Fabric for concrete reinforcement. It shall be used where shown on the Drawings.

    - Reinforcement shall be free from all loose or flaky rust and mill scale, or coating, including ice, and any other substance that would reduce or destroy the bend. Reduced sections steel reinforcement shall not be used.

- DELIVERY & STORAGE

Steel reinforcement bars shall be kept in bundles firmly secured and tagged. Each bar or bundle of bars shall be identified by marks stamped on hot or cold or painted on or by any other means, The identifying marks shall contain the following information:

- Name of the producer or his trade.

- Standard to which the bars have been manufactured.

- The class type and strength

- The diameter

- The number of the test certificate

The method of storage shall be approved by the Engineer. Reinforcing bars shall be stored in racks or platforms above the surface of ground and shall be protected free from scaling, rusting, oiling, coatings, damage, contamination and structural defects prior to placement in works. Bars of different diameters and grades of steel reinforcement shall be kept separately.

- BAR BENDING SCHEDULES

The Contractor shall prepare bar bending schedules of all the reinforcing steel bars and these bar bending schedules shall be submitted to the Engineer for his approval. The Contractor shall obtain approval of the bar bending schedules atleast one month prior to the actual execution of the works at site.

- FABRICATING, BENDING & PLACING

    - All metal reinforcement shall be free from loose mill scale, loose rust, mud, oil, grease, or other harmful matter immediately before the concrete is placed.

    - Reinforcement is to be accurately placed as shown in the drawings, and secured against displacement by using 16 gauges GI wire ties or suitable slips at intersections and supported from the formwork by using concrete, metal or plastic chairs and spacers or hangers of an approved pattern. Where concrete blocks are used for ensuring the cover, they shall be made of mortar not leaner than 1 part of cement to 2 parts of sand.

    - Bars used for concrete reinforcement shall be fabricated in accordance with the dimensions shown in the bar bending schedule approved by the Engineer.

    - The cutting tolerance for all bars shall be ± 1 inch.

    - Where an overall or an internal dimension of a bent bar is specified in the schedule, the bending tolerance, unless otherwise stated, shall be as in Table - 1.

    - Bent bar reinforcement shall cold bent to the shapes shown on the drawings bars shall be bent around a pin having the following diameters (D) in relation to the diameter of the bar (d):

Strips & columns tie bars 	D = 4 x d

Other bars having		D = 5 x d	D = 10 x d

d < 3.5 cm (1-3/8”) (No. 11 bars)

d > 3.5 cm (1-3/8”)

Table

Bending Tolerances

| Dimensions of bent bars |  | Tolerance |  |
| --- | --- | --- | --- |
| Over | Up to & including | Plus | Minus |
| Inch | Inch | Inch | Inch |
| -- | 36 | 2 | 2 |
| 36 | 72 | 2 | 4 |
| 72 | -- | 2 | 10 |

    - Vertical bars in columns shall be offset at least one bar diameter at lapped splices. To ensure proper placement, templates shall be furnished for all column dowels.

    - Reinforcement shall not be bent or straightened in a manner that will injure the material.

    - No bars shall be bent twice in the same place, nor shall they be straightened after bending.

    - Unless permitted, by Engineer, reinforcement shall not be bent after being partially embedded in hardened concrete.

    - Bars which depend for their strength on cold working shall not be heated for any reason. Other kinds of reinforcement larger than 40 mm in dia: may be bent by the use of heat at cherry - red heat (not exceeding 840 Bars) bent shall not be cooled by quenching.

    - No splice of reinforcement shall be made except as shown on the working drawings.

    - Welding shall be permitted for bars only under suitable conditions and with suitable safeguards in accordance with BS 693, BS 1856, or AWS D 12.1, provided the type of reinforcement bar has the required welding properties. Tack welding may be used to fix in position bars that cross each other, only with prior approval of the Engineer.

    - Exposed reinforcement intended for bonding with future extensions is to be effectively protected from corrosion. Protection is also to be provided to reinforcement partly built into concrete where the exposed pat is to be built into later concrete.

    - No concreting is to be carried out until the reinforcement has been checked and approved by the Engineer.

    - All detailing shall be done as per ACI standards ACI - 315 and ACI - 318.

    - Minimum Concrete clear cover for reinforcing steel shall be as follows:

Structural Members	Minimum Cover, inch

a) 	Concrete cast against and

permanently exposed to earth	3 inch

b) 	Concrete exposed to earth or weather:-

Bar Dia > 20 mm	2 inch

Bar Dia > 16 mm	1.6 inch

c)	Concrete not exposed to weather or in contact with ground

Slabs, Walls	0.8 inch

Beams, Columns

(Primary Reinforcement)	1.6 inch

All reinforcing steel shall be held firmly in place before and during the placing of concrete by means of wires and supports adequate to prevent displacement during the course of construction.

- MEASUREMENT & PAYMENT

  - General

Except otherwise specified herein or elsewhere in the Contract Documents, no separate measurement and payment will be made for providing and installing chairs, supports, hooks, spacers, binding wires and laps not shown on Drawings including wastage and rolling margin, the cost of which shall be deemed to have been included in the quoted unit rate of the respective items of the Bill of quantities.

  - Measurement

All measurements of acceptably completed works of reinforcement shall be made in linear dimensions end to end according to the cut lengths shown in bar bending schedules approved by the Engineer and converted into theoretical weight as per schedules.

  - Payment

Payment will be made for acceptable measured quantity of reinforcement on the basis of unit rate per kg quoted in the Bills of Quantities and shall constitute full compensation for all the works related to the item.

BRICK AND CEMENT CONCRETE BLOCK WORK

- 3.1		SCOPE

This section consists of construction of brick/ cement concrete block work walls of any thickness with first class hand-mould and/or machine pressed bricks/cement concrete blocks with the specified ratio of cement mortar in foundation, plinth superstructure or for any other structure as directed by the Engineer, or shown in the Bid Schedule. The Contractor shall furnish all materials and all other requirements to produce finished brick/block work. Brick/block work and materials for brick/block work shall be in strict accordance with this section of the specifications and applicable drawings and subject to the terms and conditions of the Contract.

- 3.2		MATERIALS

- 3.2.1		Portland Cement

Portland cement shall conform to the stipulations and requirements set forth in Section "CONCRETE".

- 3.2.2		Mortar Sand

Sand for mortar used in construction of brickwork/blockwork required under these Specifications shall be furnished by the Contractor in accordance with the provisions and in conformity with the stipulations and requirements of ASTM Designation C144-70 or latest revision and shall have a fineness modulus between 1.6 to 2.5.

- 3.2.3		Water

The water used in the preparation of mortar shall be free from objectionable quantities of silt, organic matter, alkali salts and other impurities and it will be tested in accordance with BS-3148 and approved by the Engineer at the Contractor's cost.

- 3.2.4		Aggregate

Aggregates for mortar shall comply with the requirements of ASTM C144. Sand that has been in contact with seawater shall not be used unless it has been thoroughly washed to the satisfaction of the Engineer.

- 3.2.5		Additives

Additives where used, shall be proprietary products used in the proportions and manner recommended by the manufacturer. The additives shall in no way adversely affect the mortar strength or contain chemicals, which may e harmful to other building materials. To add gypsum to cement is strictly forbidden.

- 3.3		MORTAR AND GROUT

Materials for mortar, sand binding agent and water shall be mixed by volume for at least 3 minutes with the minimum amount of water to produce a correctly mixed mortar or grout of workable consistency in a mechanical batch mixer. For small jobs, hand mixing may be permitted, the ingredients being mixed with sufficient water to produce a correctly mixed workable mortar. Mortar used in masonry construction shall conform to ASTM C-270 standard.

Mortars shall be mixed in batches, which can be used within a period before the setting process commences. Once a mix begins drying off, it shall be rejected. No ingredients shall be added to it once the setting process has begun. Mortar shall not be retained for more than 30 minutes and shall be constantly worked over with hoe or shovel until used.

- 3.4		MORTAR BATCHING

Methods or equipment used for mixing mortar shall be such as will accurately determine and control the amount of each separate ingredient entering into the mortar and shall be subject to the approval of the Engineer. If a mixer is used it shall be of approved design and the mixing time after the ingredients are in the mixer, except for the full amount of water, shall not be less than two minutes.

Mortar shall be mixed only in sufficient quantities for immediate use and all mortar not used within 30 minutes after addition of water to the mix shall be wasted. Retampering of mortar shall not be allowed. Mixing pans and troughs shall be thoroughly cleaned and washed at the end of each day's work.

- 3.5		SCAFFOLDING

Contractor shall provide safe scaffolding of adequate strength for use of workmen at all levels and heights at his own expense. Scaffolding which is unsafe in the opinion of the Engineer shall not be used until it has strengthened and made safe for use of workmen. Cost of scaffolding etc., shall be included by the Contractor in the unit rate for masonry items.

Damage to masonry from scaffolding or from any other object shall be repaired by the Contractor at his own cost.

- 3.6.		JOINTING

Jointing is the forming of joints as work proceeds. Joints shall be as follows:

3.6.1		Exterior exposed joints shall be tightly formed to a weather joint with the point of the trowel.

3.6.2		Interior exposed joints shall be tightly formed to a concave joints.

3.6.3		Joints which are subsequently covered with plaster or other finish materials shall be struck flush.

- 3.7		BRICKS

The bricks used shall be of standard size (9"x4.5"x3") first class well burnt, uniform in shape, size, texture, colour and should produce a ringing sound when struck. The bricks shall be free from flaws, cracks, chips, stone nodules of lime or kan-kar or any other blemishes. The brick shall not absorb more than one sixth of its weight when soaked in water for one hour. Compressive strength shall not be less than of 1400 psi. Bricks over burnt, under burnt vitrified and irregular shall not be used. Bricks of uniform size shall be used throughout the work and source of supply shall not be diversified.

- 3.7.1		Soaking

Before use all bricks shall be soaked in clean water in tanks or pits for at-least two hours.

- 3.7.2		Laying of Bricks

All brickwork shall be skillfully laid with level courses, uniform joints, square corners, plumb verticals and true surfaces except when otherwise shown on the Drawings or directed by the Engineer. Brickwork will be of best standard of workmanship obtainable and objectionable offsets in the brickwork shall be avoided. Smoothest practicable finished surface of the brickwork shall be ensured. Unless otherwise specified bricks shall be laid in English Bond with frogs (Manufacturer's marks) upward.

All horizontal joints shall be parallel and truly level. Vertical joints in alternate coarses shall come directly over one another. Thickness of joints unless otherwise specified shall not be less than 1/4 of an inch and not more than 3/8 of an inch. The height of 4 coarses and 3 joints as laid shall not exceed by more than 1 inch the height of 4 bricks as piled one upon the other.

- 3.7.3		Curing

All brick work involving use of cement shall be cured by water curing or other acceptable methods. The Engineer shall approve all methods and operations of the Contractor in curing different portions of work.

When curing by water brickwork shall be kept wet for at least 14 days by covering with water saturated materials or by a system of perforated pipes, mechanical sprinklers, porous hose, ponding or by any other approved method which will keep all surfaces to be cured continuously wet. Water used for curing shall meet the requirements given in Clause 3.2 of these specifications.

- 3.8		BLOCKS

Cement, aggregates and water for concrete blocks shall conform to the requirements as specified in the section for plain and reinforced concrete or as approved by the Engineer.

- 3.8.1		Concrete Block Making

3.8.1.1 		The solid and hollow blocks as and where used by planning, shall be machine moulded. The block making machines shall be of the standard approved by the Engineer. They shall be operated according to the instructions laid down by the manufactures.

3.8.1.2		The blocks shall be continuously water cured by sprinkling water for a minimum of 10 days and covered between sprinkling operations with 4 mils thick polyethylene sheeting. After the 10 days water curing period the blocks shall be air dried. Under no circumstances will blocks be used in the work until they are completely dry. During curing period no surfaces of the block will be allowed to dry.

3.8.1.3		Cured concrete blocks shall be stored off the ground, stacked on level platforms, which allow air circulation under stacked units. Units shall be covered and protected against wetting.

3.8.1.4 		Care shall be exercised in the handling of all concrete blocks. No damaged blocks shall be used in the work.

3.8.1.5		The blocks cast on different dates shall be stacked separately and must be labeled showing the date on which they were cast.

- 3.8.2		Properties of Blocks

3.8.2.1		All blocks shall be of size and shape required to complete the work shown in the Drawings or as instructed by the Engineer.

3.8.2.2		The cement, sand and coarse aggregate shall be volume batched and their proportion may be adjusted so as to provide the concrete of the required strength when tested and shall be mixed in a concrete mixer.

3.8.2.3		All blocks shall conform to ASTM C 145 standard. The compressive strength based on gross area shall be minimum 8.30 MPa for an average of 3 blocks and minimum 7.0 MPa for lowest individual blocks with 28 days after casting Cement Concrete Solid Blocks.

3.8.2.4		The Contractor shall provide test certificates show in the average minimum crushing strength of the blocks prior to the commencement of the construction. Further test certificates shall be provided as required by the Engineer, to ensure that all batches of block strengths are to be determined in accordance with ASTM C- 140 Standard.

3.8.2.5		The test shall be carried out by a laboratory approved by the Engineer. Evidence shall be produced that the block manufacturer has an efficient method of quality control. The Engineer will require to test samples of blocks periodically and the Contractor shall make necessary arrangements accordingly. The method of sampling for all test shall be in accordance with ASTM C-140.

3.8.2.6		All properties or specifications of blocks, not explain in these Specifications or ASTM C 145 shall comply with the requirements of PS 419, as directed by the Engineer.

- 3.8.3		Soluble Salt Content

For exposed block work, the contents by weight percent of soluble sulphate, calcium, magnesium, potassium and sodium radicals, shall not exceed 0.30, 0.10, 0.30, 0.03 percent respectively when ascertained in accordance with BS 3921, at the cost of the Contractor.

- 3.8.4		Erection

3.8.4.1		Block shall be laid true to line, level and laid in accurately spaced courses in stretcher bond with vertical joints of each course located at centre of units in alternate courses below. Vertical joints shall be buttered in the entire height of blocks. Each course shall be bonded. Courses of block shall be kept plumb throughout and corner reveals shall be true and in plumb.

Standard with of mortar joints for both horizontal and vertical joints shall be 7/16 inch (maximum). Mortar joints in wall shall have full mortar coverage on vertical and horizontal faces between the blocks. Mortar joints on wall including struck joints, shall be thoroughly compacted and pressed tight against the edges of the blocks with proper tools. Blocks terminating against soffits of beam or slab construction shall be wedged tight with wedges and the joints shall be packed solidly with mortar between the top of the block and the bottom of slab or beam. Control expansion joints shall e kept free from mortar or other debris.

Unless otherwise shown on the drawings or specified by the Engineer, the spaces around door frames and other material or built in items shall be solidly filled with mortar. Spaces around the door and window hold fasts shall be filled in with Class C concrete. Work required to be built in with masonry including door frame anchors, wall plugs, dovetail anchors and accessories shall be built in as the erection progresses.

3.8.4.2		The block work shall be carried out in a uniform manner and no portion shall be carried more than one metre above the adjoining one at any times. All masonry shall be kept strictly true and square and the whole properly bonded together and levelled round each floor.

3.8.4.3		Sleeves, Chases, holes, sinking and mortices for other trades shall be correctly located and formed to the sizes as required by the relevant trades. Chiselling of completed walls or the formation of holes shall only be carried out with the approval of the Engineer.

3.8.4.4		Walls of blocks indicated as being non-load bearing shall be constructed on insitu concrete floor slab unit after the floor formwork is struck and the concrete has obtained sufficient strength to support their weight. Toothing into load-bearing walls shall not be permitted.

3.8.4.5		All bolts, anchors, ties, pipe sleeves, flushing metal attachments lintels and the like required to be built into the work shall be correctly inserted and executed as the work proceeds.

3.8.4.6		Walls or partitions abutting concrete columns or walls shall be securely anchored and tied with metal anchors or ties at not more than 18 inches vertical centres. Wall ties cast in with concrete shall be bent down after the removal of form work and shall be securely jointed into the mortar beds of walling.

- 3.8.5		Curing and Repairs

3.8.5.1		All block masonry shall be water cured and shall be kept wet for at least seven days, by an approved method, which will keep all surfaces to be cured continuously wet. Water used for curing shall meet the requirements of specifications for water used in the manufacture of blocks.

3.8.5.2		Tolerances

All block work shall be erected plumb and true to line and level with the maximum variation in any storey height or any length of wall being one mm in one metre. The maximum tolerance in the length, height or width of any single masonry wall shall be + 1/8 inch.

3.8.5.3		If, after the completion of any block masonry work, the block is not in alignment or level, or does not, conform to the lines and grades shown on the Drawings or shows a defective surface, it shall be removed and replaced by the Contractor at his expense unless the Engineer grants permission, in writing, to patch or replace the defective area.

- 3.9		MEASUREMENT AND PAYMENT

- 3.9.1		Material (Brick and cement concrete block work)

Measurement and payment for brick/cement concrete block work shall be made in accordance with the provisions given hereafter.

3.9.1.1		Method of Measurement

Measurement for brick/block work shall include number of cubic ft. of brick/block work provided within the limits as shown on the Drawings or as directed by the Engineer.

3.9.1.2   	Basis of Payment

Payment for brick/block work shall be made at the contract unit price per cubic feet. Payment shall constitute full compensation for furnishing all materials, equipment and labour including all incidentals necessary to complete the work:

--------------------------------------------------------------------------------------------------

Pay Item	       D e s c r i p t i o n				Unit

--------------------------------------------------------------------------------------------------

3.1		Provide and Lay Brick Masonry		 	Cft.

with cement sand mortar in

foundation and super structures.

3.2		Provide and Lay Block Masonry 			Cft.

with cement sand mortar in

foundation and super structures.

SURFACE RENDERING

- 4.1 		SCOPE

The work covered by this part of the Specifications consists of supplying all materials, labour, equipment, appliances in performing all operations required for doing the work of cement plastering, pointing, and white washing in accordance with the herein stated requirements except when specifically modified by the Engineer.

- 4.2		CEMENT PLASTER

- 4.2.1		General

The work to be carried out under this item shall consist of providing 1/2" thick plaster in grey cement as specified below. The work shall be carried out in accordance with applicable requirements of British Code of practice 211:1966 or latest revision.

- 4.2.2		Materials

- 4.2.2.1		Cement

All cement required for incorporation in this Section shall conform to the applicable requirements of Section "CONCRETE"

- 4.2.2.2		Sand

The sand shall be of medium to coarse grain and having a fineness modulus varying between 1.10 to 1.50 obtained from an approved quarry e.g. Lawrencepur/Local. The material shall be free from clay, vegetable matters and other impurities. Sand bearing clay shall be washed at the discretion of the Engineer.

- 4.2.2.3		Water

Water required for cement sand paste and curing purposes shall conform to applicable requirements of Section "CONCRETE"

- 4.2.3		Mortar Composition

Mortar for plastering shall consist of one part of Portland cement to 3 parts of sand by volume.

- 4.2.4		Material Batching

Material batching for preparation of mortar shall conform to stipulations and requirements set for in the Section "BRICK AND CEMENT CONCRETE BLOCK WORK".

- 4.2.5		Application of Plaster

The surface on which plaster is to be applied shall in case of brick work, be properly raked and wetted before application of plaster. Plaster shall be applied in a thickness of 1/2". If the specified thickness sis more than 1/2" then plaster shall be applied in two coats viz rendering coat and the final coat. Plaster shall be carried out to the full length of the wall or to the natural points. Vertical or horizontal joints which show themselves shall not be allowed. Rendering coat shall be roughened with waving lines drawn by wire brushes to provide bond for the final coat and it shall be properly moistened before application of subsequent coat. The final coat shall be finished with floats to provide smooth and uniform surface. All arises shall be straight and either truly horizontal or perpendicular and finished with 1/8" radius. Defective finishes if any shall be cut out and re-plastered at the expense of the Contractor. Plaster after finishes shall be kept moist for about 10 days to the satisfaction of Engineer.

- 4.3		POINTING

- 4.3.1		Surface Preparation

The joints of brickwork which is to be pointed shall be raked out with a hook to a depth of 1/2". The raking shall be done while the mortar is still green and not later than 48 hours of time of laying. After raking, the brick work is brushed to remove all loose dust from the joints and thoroughly washed with water, all putlog holes shall be filled up before pointing as the scaffolding for masonry has been taken down. The work shall be watered for 24 hours before pointing is done.

- 4.3.2		Materials

- 4.3.2.1		Cement

All cement required for incorporation in this section shall conform to the applicable requirements of Section "CONCRETE".

- 4.3.2.2		Sand

The sand required for incorporation in this Section shall conform to the applicable requirements of "CEMENT PLASTER" as per Clause 4.2.

- 4.3.2.3		Water

Water required for cement sand paste and curing purposes shall conform to applicable requirements of Section "CONCRETE".

- 4.3.3		Mortar Composition

Unless otherwise specified, the mortar shall be mixed by volume. The ratio of Cement Sand shall be as specified in the BOQ.

- 4.3.4		Material Batching

Material batching for preparation of mortar shall conform to stipulations and requirements set forth in Section "BRICK WORK".

    - Precautions

- Before starting work of pointing the following precautions shall be taken.

- Fine aggregate i.e. sand shall be washed before use.

- It shall be ensured that all joints are properly raked.

- The surface to be pointed shall be kept moist but excessive moisture shall be avoided.

- The scaffolding for pointing shall always be provided double.

- 4.3.6		Type of Pointing

Unless otherwise specified, the following types of pointing shall be done.

- 4.3.6.1		Deep or Struck Cement Pointing

This type of pointing shall be done to all un-plastered faces of brickwork where the brickwork is liable to be affected by dampness and saltpeter, such as in the plinths of buildings. The mortar shall be filled in the joints flush with masonry or brickwork with a pointing trowel and then pressed with proper pointing tools. Lining with a spike on a mass of mortar shall not be allowed.

- 4.3.6.2		Flush Cement Pointing

This type of pointing shall be done at all brickwork with exposed face, when `the finish of the face is not important or when a flush floor surface is required or when the floor or brickwork is subject to wear or to the effects of dampness and saltpeter. The mortar shall be filled and pressed into the joints with a jointing trowel, and finished off level with the edges of the bricks to give the smoothest possible appearance to the work.

- 4.3.7		Pointing Tools

The pointing tools for horizontal joint shall be such as to form weathered and struck joints; and for vertical joint, triangles, so as to make a (v) notch. Care shall be taken not to develop a cutting edge in the tools since the idea is to compress the green mortar into the joints and not to cut it away.

- 4.3.8		Edges of Bricks

The mortar shall not be spread irregularly over the edges and corners of the bricks which shall be left clearly visible. The practice of smearing mortar over defects in bricks, to hide them shall not be allowed and shall render the whole brickwork liable to be rejected.

- 4.3.9		Washing after Pointing

After pointing, the face of the work shall be cleared off all surplus mortar sticking to the face. No washing shall be done till the pointing has set.

- 4.3.10		Protection during curing

After completion, pointing shall be kept for 10 days and shall be protected during that period from extreme fluctuations of temperature and weather

All defects detected during curing or afterwards shall be treated at the Contractor's expenses according to directions of the Engineer.

- 4.4		PAINTING

The following codes and standards shall be followed wherever relevant and applicable and/or directed by the Engineer.

BS  242-66			Linseed Oil.

BS  245-76			Specification for mineral solvents (white spirit and related hydrocarbon solvents) for paints and other purposes.

BS 2523-83			Lead-based priming paint

BS 2569-64/45			Sprayed metal coatings.

BS 2992-70			Painters and decorators brushes.

BS CP. 3012-72			Cleaning and preparation of metal surfaces.

BS 4800-81			Paint colours for building purposes.

BS 5082-74			Water-thinned priming paints for wood.

BS 5358-76			Specifications for low-lead solvent-thinned priming paint for woodwork.

BS 6150-82			Code of practice for painting of buildings.

- 4.4.1		White or Colour Washing

The whitewash shall be made from pure fat lime brought to site of work in the form of un- slaked lime. Water shall be added to this lime in a container until the mixture is of  consistency cream and allowed to rest until cracks shall appear on its surface (48-72 hours). After screening through coarse cloth, gum at the rate of 4 oz. boiled with 10 oz. of rice shall be added to each cubic feet of white wash. The colour pigment if required shall be added and mixed with white wash and stirred to give the required shade. Enough quantity shall be prepared in one go so as to meet the requirement of one complete room.

- 4.4.2		Weather Resistant Paint

- 4.4.2.1		Selection of Paints

Concrete and Masonry

Cement based paints or one of the three common types of the exterior latex paints (polyvinyl acetate, styrene-butadiene and acrylic) of ICI/Burger make or equivalent shall be used whichever specified. Approved quality cement based or weather resistant emulsion paints shall be used as directed by the Engineer.

- 4.4.2.2		Primers

Concrete and Masonry

Boiled linseed oil or silicone water repellent primers ICI/Burger make or equivalent shall be used on concrete and masonry surfaces. Before application of paint, concrete and masonry surface should be allowed to dry for at least 3 weeks after cessation of curing.

- 4.4.2.3		Fillers

Concrete and Masonry

Paste of zinc oxide and varnish thinned with turpentine shall be used as filler on masonry and concrete.

- 4.4.2.4		Sealers

Concrete and Masonry

Water-insoluble and water-repellent substances dissolved in solvent such as petroleum naphtha or the special clear silicone compounds shall be used to seal masonry surfaces.

- 4.4.2.5		Thinners

Concrete and Masonry

Thinners such as turpentine, mineral spirit, water, xylene and linseed oil of approved quality shall only be used in accordance with the manufacturers’ instructions and with prior approval of the engineer.

- 4.4.2.6		Brushes

All brushes used for painting work shall conform to B.S 2992 or equivalent American Standards.

- 4.4.3   		Preparation of Surface

All loose material and dirt on the surface shall be removed with a brush. Holes and irregularities of surface shall be repaired with lime putty, and the surface shall be allowed to dry before applying whitewash or colour wash and weather resistant paint. All greasy spots shall be given a coat of rice, water and sand. Surfaces discolored by smoke shall be washed with a mixture of wood ashes and water or yellow earth before being white-washed or painted.

- 4.4.4		Application

Three coats of white or colour wash shall be applied on the prepared surface with a brush. Paint or finish to any surface shall be applied when ambient temperature is 10 degree centigrade or above and less than 43 degree centigrade unless other wise recommended by the manufacturer. No painting shall be done above 90% relative humidity. Drop cloths shall be placed to adequately protect all finished work.

All paint and coating materials shall be in thoroughly mixed condition at the time of application. All work shall be done in a workman-like manner, leaving the finished surface free from drips, ridges, waves, laps and brush marks. All paints shall be applied under dry and dust free conditions.

All primary paint shall be applied by brushing. The first coat of paint shall be applied immediately after cleaning.

- 4.5		MEASUREMENT AND PAYMENT

- 4.5.1		Cement Plaster

Measurement and payment for cement plaster shall be made in accordance with the provisions given hereafter.

- 4.5.1.1		Method of Measurement

Measurement shall be made of cement plastering for the actual area in square foot in accordance with this section of Specification or as directed by the Engineer.

- 4.5.1.2		Basis of Payment

Payment shall be made for the number of square foot of surface area cement plastered at the contract unit price per square foot and shall constitute full compensation for furnishing all materials, equipment and labour including all incidentals necessary to complete the work in strict accordance with this Section of Specification.

-------------------------------------------------------------------------------------------------

Pay Item                          Description				Unit

------------------------------------------------------------------------------------------------

4-1		Provide and apply ½” thick 1:3

Cement Sand Plaster for ceiling.			Sq.ft.

4-2		Provide and apply ½” thick 1:4

Cement Sand Plaster for walls.			Sq.ft.

-----------------------------------------------------------------------------------------------

- 4.5.2		Pointing

Measurement and payment for cement pointing shall be made in accordance with the provisions given hereafter.

- 4.5.2.1 		Method of Measurement

Measurement will be made of cement pointing for the actual area in sq.ft in accordance with this section of Specification or as directed by the Engineer.

- 4.5.2.2 		Basis of Payment

Payment shall be made  for the number of square feet of surface area cement pointed at the contract unit price per square feet and shall constitute full compensation for furnishing all materials, equipment and labour including all incidentals necessary to complete the work in strict accordance with this section of specification.

-------------------------------------------------------------------------------------------------------

Pay Item                          Description				Unit		       -------------------------------------------------------------------------------------------------------

4-3   		Provide and apply Cement Sand mortar

i)   Flush pointing				Sq.ft

ii)  Struck pointing				Sq.ft

4.5.3		Painting

Measurement and payment for white washing and weather resistant paint shall be made in accordance with the provisions given hereafter.

4.5.3.1		Method of Measurement

The measurement shall be made in sq.ft of the actual surfaces completed and approved.

4.5.3.2		Basis of Payment

Payment shall be made for number of square feet of the actual surface painted measured as provided above at the Contract unit price per square feet for the respective item and shall constitute full compensation for all materials, equipment, labour, including all incidentals necessary to complete the work.

--------------------------------------------------------------------------------------------------------

Pay Item                          Description				Unit

--------------------------------------------------------------------------------------------------------

4.3		Provide and apply white wash.			Sq.ft.

4.4		Provide and apply weather resistant paint.	Sq.ft.

4.3		Provide and apply vinyl emulsion paint.		Sq.ft.

4.4		Provide and apply enamel paint.			Sq.ft

DAMP PROOFING

- 5.1 		SCOPE

The work covered under this section shall consist of damp proofing concrete surface or brick/block masonry surface, in accordance with these specifications and at the locations shown on the Drawings or as directed by the Engineer.

- 5.2		GENERAL

The concrete surface shall be damp proofed with bitumen by the absorptive method where as brick/block masonry surfaces shall be first treated with a horizontal layer of Portland cement concrete and then covered with bitumen damp proof cover with Hessian base. Vertical faces of walls likely to come in contact with earth shall be first plastered with cement plaster and then treated with bitumen by the absorptive method. Damp proofing shall not be applied when the temperature is below 390 F and falling. The work shall be done by workmen experienced in the application of damp-proofing. Damp proofing shall be applied as shown on the Drawings.

- 5.3		DAMP-PROOFING MATERIALS

All asphaltic materials shall be delivered in sealed containers bearing the manufacturer's original labels. Bituminized kraft paper shall be delivered in rolls as per manufacturer's original packing. Other materials shall be delivered as provided under relevant clauses of these Specifications. All materials shall conform to the Specifications designated and be approved by the Engineer. Damp proofing materials shall conform to the following requirements along with the requirements shown on the Drawings.

- 5.4   		ASPHALT PRIMER

The asphalt primer shall consist of an asphaltic base thinned to a suitable brushing consistency, with a volatile solvent, and shall conform to the requirements set forth under ASTM 41-73 or latest revision.

- 5.5		ASPHALT

Asphalt for damp proofing shall meet the requirements of ASTM D 449-73, Type C or latest revision.

- 5.6		BITUMENISED KRAFT PAPER

3-ply brown kraft paper (0.00113 lbs/ft2 each ply) sandwiched with two layers of Bitumen (0.00083 lbs/ft2 each layer) as manufactured by Messrs Packages Ltd., Lahore or equivalent, as approved by the Engineer, shall be used.

- 5.7		PREPARATION OF SURFACES

Surfaces to receive damp proofing shall be smooth, clean and dry. Holes, joints and cracks shall be painted flush with mortar and high spots ground level with the surrounding surface. Before damp proofing, surfaces shall be swept clean of all foreign matter and shall be inspected and approved.

- 5.8		PLACEMENT PROCEDURES

The selection and combination of various waterproofing and damp proofing materials for different locations shall be as shown on the Drawings or as directed by the Engineer. Unless otherwise directed or approved by the Engineer, the following procedures shall be adopted.

- 5.9		DAMP PROOFING UNDER BRICK MASONRY WALLS

The damp proofing shall comprise of:

1 1/2" thick layer of plain cement concrete Class B shall be placed over the area to be damp proofed.

A priming coat of asphalt primer confirming to BS specification 1097 October 1973 before the application of asphalt coating.

An asphalt coat using not less than 30 lbs per 100 sq.ft. and 2 layers of Bituminized kraft paper or polythene sheet.

- 5.10		QUALITY CONTROL

Sampling of asphalt for Specifications compliance shall be done in accordance with ASTM D 140-70. Samples shall be taken from each consignment, as directed by the Engineer.

- 5.11		MEASUREMENT AND PAYMENT

- 5.11.1		Damp Proofing

Measurement and payment for Damp proofing work shall be made in accordance with the provisions of this clause specified hereinafter.

- 5.11.1.1		Method of Measurement

Measurement will be made per Sq.ft of superficial area or surface damp proofed as per this Section of Specification and accepted by the Engineer.

- 5.11.1.2		Basis of Payment

Payment will be made for the number of sq.ft of superficial area damp proofed at the Contract Unit Price and will include full compensation for furnishing and placing all materials and for all labour, equipment, tools and incidentals necessary to complete the work prescribed in the Bill of Quantities Items:

------------------------------------------------------------------------------------------------

5.1		Furnish and Apply 1 ½”  Damp Proofing	Sft.

Under masonry walls.

5.2		Furnish and Apply ¾” Vertical Damp 		Sft.

Proofing Under masonry walls.

-------------------------------------------------------------------------------------------------

ROOF INSULATION

- 6.1		SCOPE

The work consists of insulation with brick tiles of sizes 9"x4" x 1 1/2" or any other approved size laid in cement mortar (1:3) over rammed mud laid to grade as shown on drawings after applying two coats of bitumen on the R.C.C. roof slab surface at 30/25 lbs, respectively for first and second coats at specified heat and laying 20 lbs. polythene sheet complete in all respects.

- 6.2		MATERIALS

The brick tiles shall comply with the standards set in "Section Bricks" except for their thickness and strength. The cement, sand and water shall meet the requirements as given in Section "CONCRETE".

Bitumen shall be PB3 or PB4

The clay for making mud shall be clean, free of all organic and other injurious matters.

- 6.3		APPLICATION

- 6.3.1   		Bitumen Painting

Bitumen heated to the specified temperature and applied on R.C.C. roof slab cleaned and dried surface including sanding at 1 1/2 cu.ft per hundred sq.ft. of surface.

- 6.3.2		Laying Mud

The clay shall be mixed with reasonable quantity of water and thoroughly kneaded to form a thick paste to which copped straw at the rate of 10 lbs. per cu.ft of mud shall be added. It shall be laid and thumped with wooden trowels to form the slope as shown on the drawings.

- 6.3.3		Laying of Tiles

The brick tiles shall be laid in cement mortar (1:3) in fall/slope as shown on drawings.

- 6.3.4		Pointing

The brick tiles shall then be flush pointed in cement mortar (1:2)

- 6.3.5		Curing

The tiles laid shall be cured properly for ten days.

- 6.4		MEASUREMENT AND PAYMENT

- 6.4.1		Roof Insulation

Measurement and payment for roof insulation shall be made in accordance with the provisions given hereafter.

- 6.4.1.1		Method of Measurement

The measurement of the roof insulation shall be made in actual area acceptably laid in square feet complete in all respects as per relevant drawing or as directed by the Engineer.

- 6.4.1.2		Basis of Payment

Payment for roof insulation work shall be made for the number of the sq.ft measured of roof insulation provided above at the Contract Unit Price per sq.ft. It includes the cost of bitumen, mud laying, laying of tiles & pointing and shall constitute full compensation for providing and furnishing all materials, equipment, labour and all incidentals necessary to complete the work in accordance wit the specifications for B.O.Q. items.

--------------------------------------------------------------------------------------------------

Pay Item	     D e s c r i p t i o n				Unit

--------------------------------------------------------------------------------------------------

6.1		Provide and Lay Roof insulation			 Sft.

earth and tile roofing as per drawing

FLOORING

- 7.1		SCOPE

The work covered in this Section consists of furnishing all plant, labour and material etc., and of performing all operations in connection with making cement concrete floor in conformity with lines and dimensions shown on the Drawings and in strict accordance with these specifications.

- 7.2		MATERIALS

Cement, sand and aggregate shall conform to the requirement of relevant clauses in section "CONCRETE"

- 7.3 		BASE FOR FLOORING

The base for flooring shall be laid down when the earth filling has been done up to the specified level in a layer of 6 inches and has been properly watered and consolidated and correctly leveled.

A layer of sand about 4" thick shall be laid and rammed after having saturated so that a 4" layer is reduced to about 3" after compaction.

Portland cement concrete of Class C (2000 psi) shall be laid in one operation in a uniform layer of specified thickness, absolutely true and parallel to the required level of the finished surface. Concrete shall be cured for at least 7 days before any topping is laid. Before laying the surface shall be washed and scrubbed with wire brushes so that the concrete in the base and the topping are well bounded.

- 7.4		CEMENT CONCRETE FLOORING

Before laying the topping, the surface of the base shall be divided into symmetrical panels by glass strips. The size of panels, unless otherwise specified, shall not exceed 3 ft. square and concrete shall be placed in alternative panels. The top of the glass strips shall be adjusted to the specified level of the finished floor surface.

Cement concrete floor shall consist of laying a topping of cement concrete of Class B (3000 psi) of specified thickness over the prepared and finished base as or roughed surface of floor slabs.

Placing operation shall be specifically timed. No sooner the concrete has been evenly spread in a panel, then it shall be beaten for about 5 to 10 minutes with "wooden thapies" (about 5 lbs. weight).

Immediately after consolidation, the surface shall be leveled with a wooden trowel. Excessive trowelling in the early stages shall be avoided. The surface shall be tested with a straight edge to detect undulations, which, if found, shall be eliminated. The finer stuff in the concrete which has come to the surface with the stroking shall be quickly but carefully smoothen with the steel trowel. When the concrete has hardened sufficiently, trowelling shall be done with steel trowel. No dry cement or a mixture of dry cement shall be sprinkled on the surface for hardening the surface.

- 7.5		BRICK FLOORING

The work covered by this item consists of furnishing and laying 4 inch sand over prepared earth to required slope and grade. 3 inch thick layer of Class D (1000 psi) concrete is laid over it and 4.5 inch thick brick on edge are laid in 1:3 cement sand mortar. These joints of these bricks are struck at the top by flush pointing.

- 7.5.1		Method of Construction

The method consists of placing bricks on edge for flooring in 1:3 cement sand mortar over 4 inch sand and 3inch Class D (1000 psi) concrete and striking the joints of bricks with flush pointing and laid over thoroughly consolidated bottom by ramming and watering before laying this floor.

- 7.6		CURING

The concrete flooring properly laid shall be cured for 7 days.

- 7.7		MEASUREMENT AND PAYMENT

- 7.7.1   		Flooring Material

Measurement and payment for concrete flooring, brick flooring and compacted sand fill will be made in accordance with the provisions given hereafter.

- 7.7.1.1   	Method of Measurement

Measurement will be made for the number of square feet of flooring acceptably placed complete in all respects as per drawings and in strict accordance with this section of specification or as directed by the Engineer.

- 7.7.1.2		Basis of Payment

Payment will be made for the number of square feet of flooring measured as above at the Contract Unit Price per square feet and shall constitute full compensation for all work including earth and sand filling, glass strips, concrete, brick on edge and all other incidentals to complete the work.

--------------------------------------------------------------------------------------------------

Pay Item	     D e s c r i p t i o n					Unit

--------------------------------------------------------------------------------------------------

7.1		Provide and Lay compacted 3" sand fill

and Cement Concrete Floors using 1/4"

thick glass strips for panel.				Sft.

7.2		Provide and Lay compacted 4" sand fill

and brick on edge flooring laid over

3 inch thick glass class D concrete.			Sft.

  - Provide and Lay dry brick or stone

ballast 1-1/2” to 2” gauge under floor.			Cft.

7.4		Provide and Lay PCC class B (300

psi) floor 1-1/2” thick in ground floor

laid over dry brick.					Sft

  - Provide and Lay floors of 1 inch thick

floor of chip tile 12” x 12” x 1” in grey

cement over 1” cement mortar 1:4.			Sft

………………………………………………………………………………….

CONCEPTUAL BIDDING DRAWINGS