.SCHEDULE “B”.

Name of Work: -	Detailed Working Estimate for Construction of Disposal Work, Collecting Tank, Screening Chamber, Pump House, Compound Wall, PE Pipe rising Main and Pumping Machinery  for Drainage Scheme Khuhi Jalalji Taluka & District Naushahro Feroze.

| ###### S.No | Description of Item | Qty | # Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |

Part-I.

R.C.C. COLLECTING TANK 12’DIA.

1.		Excavation of well in dry upto 20’ below ground level and disposal 					of soil	within one chain in ordinary solid (CSI No.I> P.No.84).

From 0’ to 5’ Depth.

For Collecting tank 12’dia

Qty: 1004.80 cft 	@ Rs.2247/58		P%0Cft.			Rs.2258/=

2.		Reinforcement cement concrete work including all labour  and 						material except the cost of steel reinforcement and its labour for 						bending for and binding which will be paid separately.  This rate						also  includes all kinds of forms moulds lifting shuttering curing						rendering and finishing the exposed surface i/c Screening and 						washing  of Shingle, (a) RCC work in roof slab beams columns						raft lintels and other structural members laid in situe or pre cost						laid in positive complete  in ( Ratio 1:2:4) (C.S.I No.6 P.No.16)

Qty: 822.31 cft 	@ Rs.337/00		P-Cft				Rs.277118/=

3.		Fabrication of mild steel reinforcement for i/c cement concrete						i/c cutting bending laying in position making joint sand fastening						i/c cost of binding wire also i/c removal of rust from bars 							(C.S.I NO.8 P.No.16).

Qty: 34.21 cwt at Rs.5001/70 	P-Cwt 					Rs.171108/=

4.		Wet sinking of wells for depth below spring level by means							of divers i/c all charges for shoring leading and removing							excavated material with in one chain (C.I NO.4 P.No.94).

(i)	From 0’to 5’ depth

Qty: 825.23 cft @ Rs.11763/80   P%0 Cft.					Rs.9708/=

(ii)	From 5’ to 10’ depth

Qty: 825.23 cft @ Rs.24200/00    P%0 cft 					Rs.19971/=

(iii)	From  10’ to 15’ depth)

Qty: 330.09 cft at Rs.37230/77   P%0 cft 					Rs.12290/=

5.		Cement concrete Brick or stone Ballast 11/2 to 2” guage.

(CsI No. b Page No. 16).

i.	Ratio 1:4:8

Qty: 240.92 cft at Rs.9416/28      P%cft 					Rs.22686/=

6.		Cement concrete plain i./c placing compacting finshing of 							curing complete i/c dewatering of washing of stone aggregate						without shuttering (CSI No.5 Page No. 15).

Ratio 1:4:8.

Qty: 113.04 cft    @ Rs.11288/75 	P% cft 					Rs.12761/=

Ratio 1:3:6.

Qty: 113.04 cft    @ Rs.12595/= 	P% cft 					Rs.14237/=

Ratio 1:2:4.

Qty: 113.04 cft    @ Rs.14429/25 	P% cft 					Rs.16311/=

7.		Providing construction of joint in concrete work of 9” wide							corrugated P.V.C Water Stop (With Bulb) i/c soldering cost of 						material and labour etc complete (PHSI NO>2 P.NO>37).

Qty: 81.64 rft @ Rs.86/00 		P-Rft					Rs.7021/=

8.		Bailing or  pumping out sub soil water during excavation concreting					cost in situ concrete work in foundation etc (PHSI.NO.18 P.No.46)

Qty: 2034.72  cft 	@ Rs.543/00	P% cft					Rs.11049/=

Total			Rs.576518/=

PART-II.	SCREENING CHAMBER

1.		Excavation in foundation of building Bridges and other structure

including dug bailing dressing refilling around the structure with

excavated earth watering and ramming etc.complete.Lead upto

1 chain lift upto 5’ft (CSI No.18 No.4).

Qty; 630.00 cft 	@Rs.3176/25   P%0 cft.				Rs.2001/=

2.		Extra for wet earth works (CSI No.15 Page NO.4).

Qty; 236.25  cft 	@Rs.1058/75   P%0 cft.				Rs.250/=

3.		Cement concrete plain brick or stone ballast 1 1/2 to 2" guage						and curing i/c screening and washing of stone aggregate							without shuttering (CSI No. 5 Page No. 15) Ratio 1:4:8.

Qty; 39.38  cft 	@Rs.11288/75   P%0 cft.				Rs.4444/=

4.		Reinforcement cement concrete work including all labour  and 						material except the cost of steel reinforcement and its labour for 						bending for and binding which will be paid separately.  This rate						also  includes all kinds of forms moulds lifting shuttering curing						rendering and finishing the exposed surface i/c Screening and 						washing  of Shingle, (a) RCC work in roof slab beams columns						raft lintels and other structural members laid in situe or pre cost						laid in positive complete  in ( Ratio 1:2:4) (C.S.I No.6 P.No.16)

Qty: 270.00 cft 	@ Rs.337/00		P-Cft				Rs.90990/=

5.		Fabrication of mild steel reinforcement for i/c cement concrete						i/c cutting bending laying in position making joint sand fastening						i/c cost of binding wire also i/c removal of rust from bars 							(C.S.I NO.8 P.No.16).

Qty: 10.84 cwt at Rs.5001/70  	P-Cwt 					Rs.54218/=

6.		Excavation for pipe lines in trenches and its pits in soft							soils i/c trimming and dressing sides to tru alignment shape							leveling of beds of correct level surplus earth within a one

chain is directed by Engineer In charge providences guards

lifts flanges and temporary crossing for non	vehicular traffic

wherever required lift upto 5’ and lead upto	one chain

(PHSI NO.1 Page No.38).

For 15”dia RCC Pipe.

Qty: 240.00 cft at Rs.3600/=  P%0 Cft						Rs.864/=

7.		Providing RCC pipe of ASTM C-76-62 T/C-7-70 Class-B							wall and fixing in trenches i/c cutting fitting and joining  with						rubber ring i/c testing with water to specified pressure 							(PHSI No.1 Page No.18).

Qty: 16.00 Rft at  Rs. 412/0		P-Rft.						Rs.6592/=

8.		Fabrication of heavy steel work with angle tees flat iron round						iron sheets iron for making trenches handing girders tanks etc						i/c cutting drilling riveting assembling and fixing but including						errection in position (C.S.I No.16 Page No.89).

Qty 1.781  cwt at   Rs.4928/49 		P.Cwt					Rs.8778/=

9.		Erecting rolled steel beams or raft errection for iron etc 							(CSI No. 3 Page No.96).

Qty 1.781 cwt at   Rs.271/04  		P.Cwt						Rs.483/=

10.		Providing fixing and CI Main holes cover with frame i/c cost 						of material (PHSI No. 1 Page No.35).

Qty 1.00  cwt at   Rs.6985/00 		P.Cwt					           Rs.6985/=

11.		Pacca brick work in foundation and plinth in cement sand 							mortar ratio 1:6 (CSI No. 4 Page No. 23).

Qty: 10.95  cft at Rs.11948/36  	P% cft 						Rs.1308/=

12.		Cement plaster 1:4   1/2" thick upto 20' height (CSI No.11 P.No.58).

Qty: 44.65  sft at Rs.2283/93   	P% sft 						Rs.1020/=

13.		Providing construction of joint in concrete work of 9” wide corrugated 					PVC water stop (With Bulb) i/c soldering cost of material and labour 					etc complete (PHSI No. 2 Page No.37).

Qty: 29.00cft  at Rs.86/00   		P% cft 						Rs.2494/=

14.		Refilling the excavated staff in trenches in 6” layers i/c watering 						ramming to full compacting etc complete (PHSI NO.24) P.No.47.

Qty: 216.00cft  at Rs.2760/00   	P% cft 						Rs.596/=

Total	      Rs.181023/=

PART-III.	PUMP HOUSE.

1.		Excavation in foundation of building Bridges and other structure

including dug bailing dressing refilling around the structure with

excavated earth watering and ramming etc.complete.Lead upto

1 chain lift upto 5’ft (CSI No.18 No.4).

Qty; 198.00 cft 	@Rs.3176/25    P%0 cft.				Rs.629/=

2.		Cement concrete brick or stone ballast 11/2”to 2”gauage.

(CSI No.(b) P.No.16) Ratio 1:4:8.

Qty; 90.56 cft	@  Rs.9416/28   P% cft 					Rs.8527/=

3.		Random Rubble Masonary un coursed in cement sand mortar 1:6 (CSI no.3				Page No.27).

Qty; 105.74 cft	@  Rs.17723/=   P% cft 				Rs.18741/=

4.		Coursed rubble masonary i/c hammer dressing in cement sand						mortar 1:6 (CSI No.4 d Page No.27).

Qty; 220.31 cft	@  Rs.25321/=   	P% cft 				Rs.55785/=

5.		D.P.C with cement sand and shingle concet 1:2:4 i/c 2 coats of 						as Pluto Maxine 1:2:4 (CSI No. NO.28).

Qty; 58.75 sft	@  Rs.4982/18   		P% sft 				Rs.2927/=

6.		Pacca Brick work in ground floor i/c striking of joints in 							cement sand mortar 1:6 (CSI No. 5 Page No.22).

Qty; 313.50  cft	@  Rs.12674/36    P% cft 				Rs.39734/=

7.		Pacca Brick work in foundation and plinth in cement sand							Mortar Ratio 1:6 (CSI No.4 Page No.23).

Qty; 23.20  cft	@  Rs.11948/36    P% cft 					Rs.2772/=

8.		RCC work i/c all labour and material except the cost of steel						reinforcement and its labour for bending and binding of forms						mould lifting shuttering curing rendering and finishing the exposed						surface i/c serving and washing of shingle work in roof slab beams						rafts lintels column: and other structural members laid in situ for pre-cost					laid in position in all respects.Ratio 1:2:4 (CSI No.6 (a) P.No.16)

Qty; 111.16 cft  	@ 337/00   p/cft.				        	 Rs.37462/=

9.		Fabrication of mild steel reinforcement for cc i/c cutting bending						and laying in position making joints and fastening i/c the cost of  						binding wire also i/c removal of rust form bars.

(CSI No.8 P.No. 16).

Qty: 4.46  Cwt	@ Rs.5001/70   p/cwt						Rs.22308/=

10.		Filling watering and ramming earth in floor with surplus earth from					foundation load upto one chain and lift upto 5 feet.(CSI No.21 P.No.5)

Qty: 391.98  cft 	@ Rs.3630/00   P%0.cft 				Rs. 1423/=

11.		P/L 1”thick topping cement concrete 1:2:4 i/c surface finishing						and dividing into panels 2”thick (CSI No.16 © P.No.42).

Qty: 120.00 sft     	@ Rs.3275/50       P% Sft.				Rs.3931/=

12.		Fabrication of heavy steel work with angle tees flat iron round						iron sheets iron for making trenches handing  girders tanks etc						i/c cutting drilling riveting assembling and fixing but including						errection in position (C.S.I No.16 Page No.89).

Qty 4.41 cwt at   Rs.4928/49  	P.cwt						Rs.21735/=

13.	Cement Plaster 1:6   1/2" thick upto 12' height (CSI No. 11 Page No.58).

Qty 916.50  sft at   Rs.2206/60   	P% sft					Rs.20223/=

14.		Cement Plaster 1:4 upto 20" ft height 3/8"thick .								(CSI No. 1 Page No.58).

Qty 916.50 sft at   Rs.2197/52    	P% sft					Rs.20140/=

15.		Preparing Surface and Painting guards bars gates of iron

Bars Getting Railing i/c standard brasses etc and similar

Open works (CSI No.5 Page No.68).(Three coat).

Door & Window

Qty: 104.00 Sft	@ Rs.2116/41    P% Sft.				Rs.2201/=

16.		White washing 2 coats (CSI No.26 P.No.55).

Qty: 916.50 Sft	@ Rs.425/84         P% Sft				Rs.3903/=

17.		Color wash in 2 coats (CSI No.25 P.No.55).

Qty: 916.50 Sft	@ Rs.859/90   P% Sft.				Rs.7881/=

18.		Supplying and Fixing in position in iron steel grill of

¾” x ¼” Size flat Iron of approved design i/c painting

Three coats etc complete ( weight not less than 3.7 Lbs

p.Sq ft of finishing) ( CSI No. 31` Page No.99).

Qty 24.00 @ Rs.180/50  		P.Sft.				           Rs.4332/=

19.		Cement Pointing raised on stone 1:3 (CSI NO.20 Page No.52).

Qty 195.00 sft @ Rs.1758/=  		P%Sft.				           Rs.3428/=

Total	Rs.278082/=

PART-IV	COMPOUND WALL.

- Excavation in foundation of building Bridges and other structure

including dug bailing dressing refilling around the structure with

excavated earth watering and ramming etc. complete. Lead upto 1

chain lift upto 5’ft (CSI No.18 No.4).

C/Wall

Qty; 772.00 cft  @Rs.3176/25   P%0 cft					Rs.2452/=

- Cement concrete brick or stone ballast 11/2”to 2”gauage.

(CSI No.(b) P.No.16) Ratio 1:4:8.

Qty; 193.00 cft   @  Rs.9416/28  P% cft 					Rs.18173/=.

3.		Random rubble masonary un coursed in cement sand mortar						1:6 9CSI No.3 d Page No.27).

Qty; 414.00 cft   @  Rs.17723/=  	P% cft 					Rs.73373/=.

4.		Coursed rubble masonary i/c hammer dressing in cement sand mortar					1:6 (CSSI NO.4 Page No.27).

Qty; 632.50 cft   @  Rs.25321/=  	P% cft 					Rs.160155/=.

5.		D.P.C with cement sand  and shingle concet 1:2:4 i/c 2 coats of as 						Pluto Maxine 1:2:4 (CSI No.28 Page No.18).

Qty; 230.00 cft   @  Rs.4982/18  	P% cft 					Rs.11459/=

6.		Pacca brick work other than building i/c stricking of joints upto 20' height 					in cement sand mortar 1:6 (CSI No.7 Page No.26).

Qty: 311.05 cft at Rs.12346/65       P%cft 					Rs.38404/=

7.		RCC work i/c all labour and material except the cost of steel						reinforcement and its labour for bending and binding of forms						mould lifting shuttering curing rendering and finishing the exposed						surface i/c serving and washing of shingle work in roof slab beams						rafts lintels column: and other structural members laid in situ for pre-cost					laid in position in all respects.Ratio 1:2:4 (CSI No.6 P.No.16)

Qty; 51.50 cft  	@ 337/00   p/cft.				        	 Rs.17356/=

8.		Fabrication of mild steel reinforcement for cc i/c cutting bending						and laying in position making joints and fastening i/c the cost of  						binding wire also i/c removal of rust form bars.

(CSI No.8 (b) P.No. 16).

Qty: 2.06  Cwt 	@ Rs.5001/70  p/cwt					Rs.10304/=

9.	Cement Plaster 1:6   1/2" thick upto 12' height (CSI No. 11 Page No.58).

Qty : 1014.76  sft at   Rs.2206/60   	P% sft					Rs.22392/=

10.	Cement Plaster 1:4   3/8" thick upto 12' height (CSI No. 11 Page No.58).

Qty : 1014.76  sft at   Rs.2197/52   	P% sft					Rs.22300/=

11.		Pointing Flash based on stone work on stone (CSI NO.19 P.59).

Qty : 506.00  sft at   Rs.1758/08   	P% sft					Rs.8896/=

12.		Cement  Pointing joints on walls (CSI No.19 P/59).

Qty: 640.00 sft @ Rs.1213/58   P% Sft.					Rs.7767/=

13.	Making and fixing steel grated door 1/16” thick sheet i/c angle iron

frame 2x2”x 3/8” thick and ¾”Thick an box at  4” C/C with locking

arrangement etc (CSI NO.28 P.No.91).

Qty: 60.00 sft   @ Rs.726/72  P/Sft						Rs.43603/=

14.	Preparing Surface and Painting guards bars gates of iron bars getting

Railing i/c standard brasses etc and similar open works (CSI No.5 						Page No.68 (Two Coats).

Qty: 120.00 Sft	@ Rs.2116/41    P% Sft.				Rs. 2540/=

15.		white washing 1 coats (CSI No. 26 page No.55).

Qty: 1014.76  Sft	@ Rs.416/63    P% Sft.				Rs.4228/=

16.		Color Wash in 2 coats (CSI No. 25 Page No.55).

Qty: 1654.76  Sft	@ Rs.859/90    P% Sft.				Rs.14229/=

Total	Rs.457631/=

PART-V.	F.C. PIPE 6” DIA (RISING MAIN).

1.		Excavation for pipe lines in trenches and its pits in soft							soils i/c trimming and dressing sides to true alignment shape						leveling of beds of correct level surplus earth within a one

chain is directed by Engineer In charge providences guards

lifts flanges and temporary crossing for non	vehicular traffic

wherever required lift upto 5’ and lead upto	one chain

(PHSI NO.11 Page No.30).

F.C Pipe for 6”dia.

Qty: 21000.00 cft at Rs.3600/=  P%0 Cft					Rs.75600/=

2.		Providing laying and fixing in trenches i/c fitting jointing  etc 						complete in all respect the high density polyethylene PE pipes 						(HDPE-100) for w/s confirming ISO 4427/DIN8074/8075 BS						3580 & PSI 3051 (PHSI NO. F-1 Page No.25)(160 mm).

FC Pipe 6”Dia.

Qty: 3000.00 rft at Rs.440/=  P.Rft						Rs.1320000/=

3.		Specials for PE (PHSM NO.15 Page No.112).

Elbow 45* (6”dia) (160 mm).

Qty: 2.00 cft at Rs.2784/=  P.cft						Rs.5568/=

Elbow 90* (6”dia).

Qty: 2.00 cft at Rs.3480/=  P.cft						Rs.6960/=

4.		Butt fusion joints (6”Dia).

Qty: 4.00 cft at Rs.1000/=  P.cft						Rs.4000/=

5.		Refilling excavated stuff in trenches as 6” thick layers i/c watering						ramming to full compacting etc complete (PHSi No.24 P No.77).

Qty: 18900.00 cft at Rs.2760/=  P%0cft					Rs.52164/=

Total	Rs.1464292/=

PART-VI.	PUMPING MACHINERY.

1.	Supplying and installing in position @ Site of work Drainage Scheme Khuhi Jalalji, electric Motor 7.5 BHP (KSB made) 4 pole 1500 RPM 3 Phase 400 VAC 50 cycles coupled with KSB non clogging pump (type KWP-65-315) size 3” x 21/2 i/c steel frame sleeve etc complete capable of discharge 200 GPM against the head of 50 i/c installation Pumping set on cement concrete foundation ratio 1:2:4 with MS nuts Bolts in CC foundation i/c Providing and fixing automatic motor control unit ( MCU) consist oil starter (MEECO Pakistan) MCCB  breaker 10 A to 50 A amperes meter 500 V (SELCO China) and face indication light (China) i/c transformer oil (2 liters) (panel board 18” x 26” x 9” and internal wiring etc complete) providing internal electric wiring for main or sub main PVC  insulated with size 3-7/0.44 (6 mm 2) (7/.036) i/c providing and fixing earthling set with 0.6 M x 6.3 mm 2x21/2 CI Plate Buried in the ground at depth of 3.7 mm (12”) or less as per PAK PWD  Specificiation & testing the pumping machinery sets for 72 hours etc complete (M.R).

Qty: 1 sets   @ Rs.672000/=		P.Set					Rs.672000/=

2.	Providing and installing in position i/c transportation to site of work drainage scheme Khuhi Jalalji. Diesel Pumping set Diesel Oil Engine 12 BHP Golden made fixed of 1450 RPM i/c all required accessories coupled with Golden non clogging pump type (GSWG-103)                            including steel frame or base plate coupled i/c coupling i/c alignment charges capable of discharge 200 GPM against the head of 40 ft including installing pumping set on cement concrete foundation Ratio 1:2:4 and 1:4:8 stone ballast with MS Nuts bolts 5/8” thick to be imbedded capable in CC foundation including testing the pumping machinery sets for 72 hours etc complete (RA attached).

Qty: 1.00 No @ Rs.203000/= 		P.No.				Rs.203000/=

3.		Supplying CI Reflex valve heavy pattern test pressure							21 kg P/sq or 300 LBS /Sq inch (PHSI of No.8 Page No.6).

3”Dia

Qty: 02 Nos  @ Rs.1543.75		Each					Rs.3088/=

4.		Supply of CI Sluice Valve heavy patter test pressure 							21 kg sq or 300 lbs(PHSI No.1 Page No.35).

3"dia

Qty: 02 Nos  @ Rs.4290.00		Each					Rs.8580/=

5.		Providing and fixing MS flanage  4” dia made out of 3/8” thick						MS Sheet i/c cutting folding to curvature and shape i/c welding						etc complete(Based on Scheduled).

4"dia

Qty: 12.00 Rft.  @ Rs.596/75		P.Rft					Rs.7161/=

3"dia

Qty: 4.00 Nos.  @ Rs.500/89		P.Rft					Rs.2004/=

6.		Manufacturing and installing with MS pipe  4”dia made out of 						MS sheet 3/16” thick i/c cutting drilling holes etc complete (Based on schedule).

4"dia

Qty: 12.00 Nos.  @ Rs.506/60	P.No.					Rs.6079/=

3"dia

Qty: 06.00 Nos.  @ Rs.385/89	P.No.					Rs.3040/=

7.		Supplying CI Bend with flanged Ends with holes i/c turning and facing					of flanges for all size (SMI No.8 Page NO.11).

Bend 90o

4"dia 90o (4” &3” dia).

Qty: 1.25 Nos @ Rs.6096/=		P.No.					Rs.7620/=

8.		C.I.Foot valve heavy pattern with cone type gate (PHSI No.4 						Page No.97).

3”dia.

Qty: 01  No @ Rs.1543/75/=		P.No.					Rs.1543/=

4”dia.

Qty: 1.00  Nos @ Rs.1706/25		P.No.					Rs.1706/=

9.		Supplying CI Tapered flate bottomed or central tapered flanged ends 					with holes i/c turning and facing of flanged for all size.(PHSI No.						Page No.     )     4”x3”  "Dia.

Qty: 0.44 Nos   @ Rs.	6096/=		P.No.					Rs.2682/=

10.		Providing laying and fixing in trenches i/c fitting jointing and testing					etc complete in all respects the high density polyethylene PE Pipes for					Water supply confirming ISO 4427/ DIN8074/8075 BS-3580 & PSI 3051					(PHSI NO.F-1 Page No.25).

P.E Pipe 4”dia.

Qty: 22.00 Rft    @ Rs.268/=		P.Rft					Rs.5896/=

P.E Pipe 3”dia.

Qty: 12.00 Rft    @ Rs.178/=		P.Rft					Rs.2136/=

11.		Jointing CI/M flanged pipes and specials and flanged and in side trenches					i/c supplying rubber paking of the required thickness nuts bolts w.washers					etc complete and other tools required for jointing and testing the joints to the				specified pressure etc complete (PHSI NO.1 Page No.35 ).

3"dia.

Qty: 6.0 Nos    @ Rs.499/=		P. No.					Rs.2994/=

4"dia.

Qty: 12.0 Nos    @ Rs.513/=		P. No.					Rs.6156/=

12.		But fusion joint

3" Dia .

Qty: 06 Nos  @ Rs.600/=		Each					Rs.3600/=

4" Dia .

Qty: 12 Nos  @ Rs.1000/=		Each					Rs.12000/=

13.		Flange Adopter (M.R).

3”Dia.

Qty 6 Nos  @ Rs.675/=		Each					Rs.4050/=

4”Dia.

Qty 12 Nos  @ Rs.1215/=		Each					Rs.14580/=

14.		Supplying CI Tee with flanged ends with holes i/c turning and facing 					of flanges for all size (Smi No.10 Page No.11)

4”dia.

Qty:0.22 Nos @ Rs.6096/= 		P.No.					Rs.1341/=

Total	Rs.971256/=

PART-VII.		SURFACE DRAINS.

1.	Excavation in foundation of building Bridges and other structure

Include Dug bailing dressing refilling around the structure with

Excavated earth watering and ramming etc.complete.Lead up to

1 chain lift up to 5’ft (CSI No.18 P.No.4).

Qty: 8300.00 Cft Rs.3176/25 		P.%0 Cft				Rs.26363/=

2.	Cement concrete plain including placing compacting and curing

complete i/c  screening and washing of stone aggregate without

shuttering and washing of stone (CSI No.5 P.No.15)

i. Ratio  1:4:8.

Qty: 1887.50 cft at Rs.11288/75 P.% cft 			         		Rs.213075/=

3.	Pacca Brick Work in foundation upto Plinth in cement 							sand mortar.( Ratio 1:6) (CSI No:4 P.No:20).

Qty: 4875.00 cft  at  Rs.11948/36 P.% cft 	       				Rs.582482/=

4.	Construction of Standard Open Drain cunette block of Cement concrete (1:2:4) cost in situ to design profile i/c cost of mold as per drawing i/c applying floating coat of cement 1/32” thick be	 the exposed face finished along with curing etc complete as per detailed drawing (PHSI No.5 D  P.No.58).

Type I:      Qty: 2500.00 Rft @ Rs.94/00  	P.Rft.					Rs.235000/=

Type II:     Qty: 500.00 Rft @  Rs.174/00  	P.Rft.					Rs.87000/=

5.	Cement plaster (1:4) upto 12’ height ½” thick (CSI No.11 P.51).

Qty: 7755.00 sft @  Rs.2283/93  P.% sft.   	       			 	Rs.177005/=

6.	CC Plain i/c placing compacting finishing and curing complete i/c screening				and washing of stone aggregate without shuttering (CSI No. 5 Page NO.15).

Ratio :  1:2:4.

Qty: 131.60 cft @  Rs.14429/25  P.% cft.   	       			 	Rs.18989/=

7.	RCC work in all labour and material except the cost of steel reinforcement and its				labour for B/B which will be paid separetely. The rate also i/.c all kinds of forms				moulds lifting, shutting, curing rendering and finsihsing the exposed surface i/c  				S/W of shingle) (a) RCC work in Roof slab beams columns rafts lintels and other

structure member lad in situe or pre-cost laid in position Complete in all respects

(CSI  No.6 A Page No.17).(Ratio 1:2:4).

Qty: 143.08 Cft at Rs.337/=  		P.cft 		          			Rs.48218/=

7.	Fabrication of Mild Steel reinforcement concrete for CC i/c

Cutting, Bending Laying in position making joints and fastening

i/c Cost of binding wire also including removal of rust from bars

(CSI No.8 P.No.16)

Qty: 5.74 Cwt. at Rs.5001/70    	P/Cwt.	       				Rs.28709/=

Total 	Rs.1416841/=

PART-VIII.	CC TOPPING.

1.	Barrow Pit excavation un dressed lead upto 100 ft.

(CSI No.3  Page No.1).

Qty:6300.00  cft at RS.2117/50 P.%0 cft.					Rs.13340/=

2.	Earth Work compaction soft ordinary or hard soil (d) ramming earth

work all type of soil i.e laying earth 6"layers leveling and dressing etc

complete (CSI No.       P-       ).

Qty: 6300.00 cft @ RS.354/=              P%0 Cft.				Rs.2230/=

3.	Extra for every 50’ additional lead is part thereof ordinary soil 						(CSI No.8 Page NO.3) Lead upto 2 Miles.

Qty: 6300.00 cft @ RS.5039/=              P%0 Cft.				Rs.3175/=

4.	Cement concrete brick or stone ballast 11/2" to 2" gauge							(CSI No.b Page No.15) Ratio 1:4;8.

Qty:1188.00 cft @ RS.9416/28    	P% Cft.				Rs.111865/=

5.	Cement concrete plain including placing compacting and curing

complete i/c  screening and washing of stone aggregate without

shuttering and washing of stone (GSI No.5 P.No.15)

i. Ratio  1:2:4.

Qty: 900.00 cft at Rs.14429/30 	P.% cft 		         		Rs.129663/=

6.	Errection and removal of centering for R.C.C. or plain CC work of Deodar					wood 2nd class for partial wood (Vertical) (GSi No.19 b Page No.17).

Qty: 225.00 Sft at Rs.3127/41  	P.% Sft 		         		Rs.7037/=

Total	Rs.267511/=.

PART-IX.	BRICK PAVEMENT.

1.	Barrow Pit excavation un dressed lead upto 100 ft.

(CSI No.3  Page No.1).

Qty: 8925.00  cft at RS.2117/50  P.%0 cft.					Rs.18899/=

2.	Earth Work compaction soft ordinary or hard soil (d) ramming earth

work all type of soil i.e laying earth 6"layers levelling and dressing etc

complete (CSI No.13D P-03).

Qty:8925.00 cft @ RS.354/=              P%0 Cft.				Rs.3159/=

3.	Extra for every 50’ additional lead is part thereof ordinary soil							(CSI No.8 Page NO.3).

Qty:8925.00 cft @ RS.5039/=              P%0 Cft.				Rs.44973/=

4.	Dry brick on age paving sand grouted incuding preparation of Bed by watering				ramming and bringing same to proper chamber by ½” thick Mud Plaster.

Qty: 4250.00 sft @ Rs.3823/57              P% sft.				Rs.162502/=

Total	Rs. 229533/=.

ABSTRACT OF SCHEDULE B”.

| S. N O. | NAME OF WORK | #### SCHEDULE ITEMS | NON SCHEDULE ITEMS | ##### TOTAL SCH:B |
| --- | --- | --- | --- | --- |
| 1. | RCC Collecting Tank 15’dia | Rs.576518/= | - | Rs.576518/= |
| 2. | Screening Chamber. | Rs.181023/= | - | Rs.181023/= |
| 3. | Pump House | Rs.278082/= | - | Rs.278082/= |
| 4. | Compound Wall. | Rs.457631/= | - | Rs.457631/= |
| 5. | P.E Pipe 6”dia. | Rs.1464292/= | - | Rs.1464292/= |
| 6. | Pumping Machinery. | Rs.971256/= | - | Rs.971256/= |
| 7. | Surface Drains | Rs.1416841/= | - | Rs.1416841/= |
| 8. | CC topping | Rs.267510/= | - | Rs.267510/= |
| 9. | Brick Pavement. | Rs.229533/= | - | Rs.229533/= |
|  | Total | Rs. 5842686/= | - | Rs. 5842686/= |

CERTIFICATE.

- Any error or omission in the description of items or rates the original estimate schedule of rates will be referred to correct accordingly.

- No premium will be allowed on non-schedule items.

- The work will be carried out according to the drawing design and specifications

- shown in the estimate.

EXECUTIVE ENGINEER

PUBLIC HEALTH ENGG DIVISION

NAUSHAHRO FEROZE.