| ## NAME OF SCHEME:- | M&R: of road from Kumb – Narudhoro to Qabolani Gopang mile 0/0 – 1/0 (Selected portion) |
| --- | --- |

(SCHEDULE ‘B’)

| Qty | ITEM OF WORK |  | QTY: | RATE |  |  | UNIT |  | AMOUNT |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1/-Preparing Base course i/cs: supplying & spreading stone metal of approved quality properly graded to maximum size of 1 ½” in required thickness to proper camber & grade i/cs: supplying & spreading l5 cft: screening & Non plastic quarry fines filling depression with stone metal after initial rolling i/cs: watering & compacting the same so as to achieve 100% density as per modified AASHO Specification. (Rate i/cs: all costs of materials T&P & carriage upto site of work) (S.I No: 10 P/No: 4). |  |  |  |  |  |  |  |  |  |
| 436 Cft |  |  |  |  | @ Rs.6786/43 |  |  | P.% Cft: |  | Rs.29589/- |  |
|  |  | 2/-Providing Surface Dressing Ist: Coat on new or existing surface with 25lbs: bitumen & 2.75 Cft: Crush bajri of required size i/cs: cleaning the road surface rolling etc: complete. (Rate i/cs: all costs of materials T&P & carriage upto site of work) (S.I No: 13 P/No: 6). |  |  |  |  |  |  |  |  |  |
| 1742 Sft |  |  |  |  | @ Rs.1265/18 |  |  | P.% Sft: |  | Rs.22039/- |  |
|  |  | 3/-Providing 1”thick (consolidate) Premixed Carpet in proper camber & grade i/cs: supplying of 10 cft: Crush bajri 4 cft: Hillsand (of approved quality and grade) with 67lbs: bitumen of 80/100 penetration i/cs: fixing in machinical mixer in required proportion i/cs: heating materials and cleaning the road surface (Hillsand 2 cft: for mixing and 2 cft: for dusting) (S.I No: 17 P/No: 8). |  |  |  |  |  |  |  |  |  |
| 23657 Sft |  |  |  |  | @ Rs.4534/61 |  |  | P.% Sft: |  | Rs.1072753/- |  |
|  |  |  |  |  | Total |  |  |  |  | Rs:1124381/- |  |
|  |  | Decreased Cost of Bitumen = 7.26 ton @Rs:17595/- P.Ton |  |  |  |  |  |  |  | Rs:127740/- |  |
|  |  |  |  |  |  |  |  | Total |  | Rs: 996641/- |  |
| # CONTRACTOR |  |  |  |  |  | EXECUTIVE ENGINEER HIGHWAY DIVISION # KHAIRPUR |  |  |  |  |  |

NOTE: Subject to any Changes/

Variation in Rates/

Quantity as per T .Sanction.

| ## NAME OF SCHEME:- | M&R: of road from Haji Nawab Wassan Gate to Gul Hassan Ujjan mile 0/0 – 1/4 (selected portion) |
| --- | --- |

(SCHEDULE ‘B’)

| Qty | ITEM OF WORK | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- |

|  | 1/-Preparing Base course i/cs: supplying & spreading stone metal of approved quality properly graded to maximum size of 1 ½” in required thickness to proper camber & grade i/cs: supplying & spreading l5 cft: screening & Non plastic quarry fines filling depression with stone metal after initial rolling i/cs: watering & compacting the same so as to achieve 100% density as per modified AASHO Specification. (Rate i/cs: all costs of materials T&P & carriage upto site of work) (S.I No: 10 P/No: 4). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 436 Cft |  | @ Rs.6931/= |  | P.% Cft: | Rs.30219/- |  |
|  | 2/-Providing Surface Dressing Ist: Coat on new or existing surface with 25lbs: bitumen & 2.75 Cft: Crush bajri of required size i/cs: cleaning the road surface rolling etc: complete. (Rate i/cs: all costs of materials T&P & carriage upto site of work) (S.I No: 13 P/No: 6). |  |  |  |  |  |
| 1742 Sft |  | @ Rs.1267/94 |  | P.% Sft: | Rs.22088/- |  |
|  | 3/-Providing 1”thick (consolidate) Premixed Carpet in proper camber & grade i/cs: supplying of 10 cft: Crush bajri 4 cft: Hillsand (of approved quality and grade) with 67lbs: bitumen of 80/100 penetration i/cs: fixing in machinical mixer in required proportion i/cs: heating materials and cleaning the road surface (Hillsand 2 cft: for mixing and 2 cft: for dusting) (S.I No: 17 P/No: 8). |  |  |  |  |  |
| 23657 Sft |  | @ Rs.4450/29 |  | P.% Sft: | Rs.1052805/- |  |
|  |  | Total |  |  | Rs:1105112/- |  |
|  | Decreased Cost of Bitumen = 7.26 ton @Rs:17595/- P.Ton |  |  |  | Rs:127740/- |  |
|  |  |  |  | Total | Rs: 977372/- |  |
| # CONTRACTOR |  |  | EXECUTIVE ENGINEER HIGHWAY DIVISION # KHAIRPUR |  |  |  |

NOTE: Subject to any Changes/

Variation in Rates/

Quantity as per T. Sanction.

| ## NAME OF SCHEME:- | M&R: of road from old National Highway to Mumtaz Hussain Solangi (village Qaimuddin Solangi) mile 0/0 – 0/5 |
| --- | --- |

(SCHEDULE ‘B’)

| Qty | ITEM OF WORK | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- |

|  | 1/-Preparing Base course i/cs: supplying & spreading stone metal of approved quality properly graded to maximum size of 1 ½” in required thickness to proper camber & grade i/cs: supplying & spreading l5 cft: screening & Non plastic quarry fines filling depression with stone metal after initial rolling i/cs: watering & compacting the same so as to achieve 100% density as per modified AASHO Specification. (Rate i/cs: all costs of materials T&P & carriage upto site of work) (S.I No: 10 P/No: 4). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 412 Cft |  | @ Rs.6882/81 |  | P.% Cft: | Rs.28357/- |  |
|  | 2/-Providing Surface Dressing Ist: Coat on new or existing surface with 25lbs: bitumen & 2.75 Cft: Crush bajri of required size i/cs: cleaning the road surface rolling etc: complete. (Rate i/cs: all costs of materials T&P & carriage upto site of work) (S.I No: 13 P/No: 6). |  |  |  |  |  |
| 1647 Sft |  | @ Rs.1266/53 |  | P.% Sft: | Rs.20860/- |  |
|  | 3/-Providing 1”thick (consolidate) Premixed Carpet in proper camber & grade i/cs: supplying of 10 cft: Crush bajri 4 cft: Hillsand (of approved quality and grade) with 67lbs: bitumen of 80/100 penetration i/cs: fixing in machinical mixer in required proportion i/cs: heating materials and cleaning the road surface (Hillsand 2 cft: for mixing and 2 cft: for dusting) (S.I No: 17 P/No: 8). |  |  |  |  |  |
| 21210 Sft |  | @ Rs.4545/16 |  | P.% Sft: | Rs.964028/- |  |
|  |  | Total |  |  | Rs:1013245/- |  |
|  | Decreased Cost of Bitumen =6.50 ton @Rs:17595/- P.Ton |  |  |  | Rs:127740/- |  |
|  |  |  |  | Total | Rs:885505/- |  |
| # CONTRACTOR |  |  | EXECUTIVE ENGINEER HIGHWAY DIVISION # KHAIRPUR |  |  |  |

NOTE: Subject to any Changes/

Variation in Rates/

Quantity as per T. Sanction.

| ## NAME OF SCHEME:- | M&R: of road from Kumb-Babar road to Yar Muhammad Bhatti via Haji Jan Muhammad Tumrani mile 0/0 – 1/2 (In portion) |
| --- | --- |

(SCHEDULE ‘B’)

| Qty | ITEM OF WORK | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- |

|  | 1/-Preparing Base course i/cs: supplying & spreading stone metal of approved quality properly graded to maximum size of 1 ½” in required thickness to proper camber & grade i/cs: supplying & spreading l5 cft: screening & Non plastic quarry fines filling depression with stone metal after initial rolling i/cs: watering & compacting the same so as to achieve 100% density as per modified AASHO Specification. (Rate i/cs: all costs of materials T&P & carriage upto site of work) (S.I No: 10 P/No: 4). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 396 Cft |  | @ Rs.7027/38 |  | P.% Cft: | Rs.27828/- |  |
|  | 2/-Providing Surface Dressing Ist: Coat on new or existing surface with 25lbs: bitumen & 2.75 Cft: Crush bajri of required size i/cs: cleaning the road surface rolling etc: complete. (Rate i/cs: all costs of materials T&P & carriage upto site of work) (S.I No: 13 P/No: 6). |  |  |  |  |  |
| 1587 Sft |  | @ Rs.1269/10 |  | P.% Sft: | Rs.20103/- |  |
|  | 3/-Providing 1”thick (consolidate) Premixed Carpet in proper camber & grade i/cs: supplying of 10 cft: Crush bajri 4 cft: Hillsand (of approved quality and grade) with 67lbs: bitumen of 80/100 penetration i/cs: fixing in machinical mixer in required proportion i/cs: heating materials and cleaning the road surface (Hillsand 2 cft: for mixing and 2 cft: for dusting) (S.I No: 17 P/No: 8). |  |  |  |  |  |
| 21210 Sft |  | @ Rs.4557/74 |  | P.% Sft: | Rs.966591/- |  |
|  |  | Total |  |  | Rs:1014522/- |  |
|  | Decreased Cost of Bitumen =6.50 ton @Rs:17595/- P.Ton |  |  |  | Rs:127740/- |  |
|  |  |  |  | Total | Rs:886782/- |  |
| # CONTRACTOR |  |  | EXECUTIVE ENGINEER HIGHWAY DIVISION # KHAIRPUR |  |  |  |

NOTE: Subject to any Changes/

Variation in Rates/

Quantity as per T. Sanction.

| ## NAME OF SCHEME:- | M&R: of road for village Jhango Wassan mile 0/0 – 1/2 |
| --- | --- |

(SCHEDULE ‘B’)

| Qty | ITEM OF WORK | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- |

|  | 1/-Preparing Base course i/cs: supplying & spreading stone metal of approved quality properly graded to maximum size of 1 ½” in required thickness to proper camber & grade i/cs: supplying & spreading l5 cft: screening & Non plastic quarry fines filling depression with stone metal after initial rolling i/cs: watering & compacting the same so as to achieve 100% density as per modified AASHO Specification. (Rate i/cs: all costs of materials T&P & carriage upto site of work) (S.I No: 10 P/No: 4). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1346 Cft |  | @ Rs.7075/57 |  | P.% Cft: | Rs.95237/- |  |
|  | 2/-Providing Surface Dressing Ist: Coat on new or existing surface with 25lbs: bitumen & 2.75 Cft: Crush bajri of required size i/cs: cleaning the road surface rolling etc: complete. (Rate i/cs: all costs of materials T&P & carriage upto site of work) (S.I No: 13 P/No: 6). |  |  |  |  |  |
| 15386 Sft |  | @ Rs.1269/57 |  | P.% Sft: | Rs.68379/- |  |
|  | 3/-Providing 1”thick (consolidate) Premixed Carpet in proper camber & grade i/cs: supplying of 10 cft: Crush bajri 4 cft: Hillsand (of approved quality and grade) with 67lbs: bitumen of 80/100 penetration i/cs: fixing in machinical mixer in required proportion i/cs: heating materials and cleaning the road surface (Hillsand 2 cft: for mixing and 2 cft: for dusting) (S.I No: 17 P/No: 8). |  |  |  |  |  |
| 69440 Sft |  | @ Rs.4561/93 |  | P.% Sft: | Rs.3163242/- |  |
|  |  | Total |  |  | Rs:3326858/- |  |
|  | Decreased Cost of Bitumen =21.47 ton @Rs:17595/- P.Ton |  |  |  | Rs:377765/- |  |
|  |  |  |  | Total | Rs:2949093/- |  |
| # CONTRACTOR |  |  | EXECUTIVE ENGINEER HIGHWAY DIVISION # KHAIRPUR |  |  |  |

NOTE: Subject to any Changes/

Variation in Rates/

Quantity as per T. Sanction.