| Name of Work :- | M&R of road from Sathio Wah to Essan Dharejo mile 0/0-0/2+615’(0.586 K.m). |
| --- | --- |

SCHEDULE-B.

| Quantity. | Item of Work. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- |

01. Earth work embankment from barrow Pits including laying in 6”layers, clod breaking ramming dressing complete, lead upto 100’ft:lift upto 5ft: ( in ordinary soil.

| 56563 | Cft | @ | Rs: | 6228.37 | P.%0 Cft | Rs: | 352295.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |

02. Laying brick on end edging including supplying 9”x4-1/2”x3” first class bricks, excavation for having edging with small size parallel to the road (Rate includes lead upto site of work.

| 3986 | Rft | @ | Rs: | 2792.98 | P.% Rft | Rs: | 111308.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |

03. Preparing base course i/c supplying and spreading stone metal of approved quality properly graded to maximum size of 1-1/2”in required thickness to proper camber and grade including supplying and spreading 15 Cft screenings and non plastic quarry fines, filling depressions with stone metal after rolling including watering and compacting the same so as to achieve 100%density as per modified AASHO Specifications. (Rate includes providing and using templates, camber plates, screens, forms as directed (Rate includes all cost of materials T&P and carriage upto Site of work.

| 7892 | Cft | @ | Rs: | 7364.70 | P.% Cft | Rs: | 581222.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |

04. Providing surface dressing Ist coat on new or existing surface with 30 lbs of bitumen and 4.0 Cft of Crush bajri of required size including cleaning the road surface rolling etc, complete (Rate includes all cost of materials T&P and carriage upto site of work.

| 23917 | Sft | @ | Rs: | 1609.25 | P.% Sft | Rs: | 384884.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |

05. Providing 1”thick (Consolidated) premixed carpet in proper camber and grade including supplying 10 Cft Crush bajri 4 Cft hill sand (of approved quality and grade) 67 lbs bitumen of 80/100 penetration including mixing in mechanical mixer in required proportion, including heating materials and cleaning the road surface (Hill sand 2 Cft for mixing and 2 Cft for dusting (Rate includes all cost of materials T&P and carriage upto site of work.

| 23917 | Sft | @ | Rs: | 4487.59 | P.% Sft | Rs: | 1073297.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Note:- Subject to any Changes Variation in rates/

Quantities in Technical Sanction.

| CONTRACTOR. | Executive Engineer |
| --- | --- |
|  | Highways Division |
|  | Khairpur. |