| Name of Work :- | M&R of road from Chatti Chwok Muslim Commercial Bank Limited Khaki Shah Pull mile 0/0-0/3 (0.60 K.m). |
| --- | --- |

SCHEDULE-B.

| Quantity. | Item of Work. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- |

01. Preparing base course i/c supplying and spreading stone metal of approved quality properly graded to maximum size of 1-1/2”in required thickness to proper camber and grade including supplying and spreading 15 Cft screenings and non plastic quarry fines, filling depressions with stone metal after rolling including watering and compacting the same so as to achieve 100%density as per modified AASHO Specifications. (Rate includes providing and using templates, camber plates, screens, forms as directed (Rate includes all cost of materials T&P and carriage up to Site of work.

| 1215 | Cft | @ | Rs: | 7220.14 | P.% Cft | Rs: | 87725.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |

02. Providing surface dressing Ist coat on new or existing surface with 18 lbs of bitumen and 1.5 Cft of Crush bajri of required size including cleaning the road surface rolling etc, complete (Rate includes all cost of materials T&P and carriage upto site of work.

| 4861 | Sft | @ | Rs: | 898.77 | P.% Sft | Rs: | 43689.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |

03. Providing 1½” thick (Consolidated) premixed carpet in proper camber and grade including supplying 15 Cft bajri 5 Cft hill sand (of approved quality and grade) 93 lbs bitumen of 80/100 penetration including mixing in mechanical mixer in required proportion, including heating materials and cleaning the road surface (Hill sand 3 Cft for mixing and 2.0 Cft for dusting (Rate includes all cost of materials T&P and carriage upto site of work.

| 33378 | Sft | @ | Rs: | 6287.30 | P.% Sft | Rs: | 2098575.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Total | Rs: | 2229989.00 |

Note :- Subject to any changes variation in rates/

Quantities in Technical Sanction.

| CONTRACTOR. | Executive Engineer |
| --- | --- |
|  | Highways Division |
|  | Khairpur. |