| Name of Work :- | M&R of road from Old National Highway to Mohalla Ghareeb-abad Upto Jamali Disposal Jamli road mile 0/0-0/2 (0.40 K.m). |
| --- | --- |

SCHEDULE-B.

| Quantity. | Item of Work. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- |

01. Preparing base course i/c supplying and spreading stone metal of approved quality properly graded to maximum size of 1-1/2”in required thickness to proper camber and grade including supplying and spreading 15 Cft screenings and non plastic quarry fines, filling depressions with stone metal after rolling including watering and compacting the same so as to achieve 100%density as per modified AASHO Specifications. (Rate includes providing and using templates, camber plates, screens, forms as directed (Rate includes all cost of materials T&P and carriage up to Site of work.

| 2740 | Cft | @ | Rs: | 5252.50 | P.% Cft | Rs: | 197832.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |

02. Providing surface dressing Ist coat on new or existing surface with 18 lbs of bitumen and 1.5 Cft of Crush bajri of required size including cleaning the road surface rolling etc, complete (Rate includes all cost of materials T&P and carriage upto site of work.

| 1386 | Sft | @ | Rs: | 898.77 | P.% Sft | Rs: | 12457.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |

03. Providing surface dressing Ist coat on new or existing surface with 30 lbs of bitumen and 4.0 Cft of Crush bajri of required size including cleaning the road surface rolling etc, complete (Rate includes all cost of materials T&P and carriage upto site of work.

| 7251 | Sft | @ | Rs: | 1604.95 | P.% Sft | Rs: | 116375.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |

04. Providing 1” thick (Consolidated) premixed carpet in proper camber and grade including supplying 10 Cft Crush bajri 4 Cft hill sand (of approved quality and grade) 67 lbs bitumen of 80/100 penetration including mixing in mechanical mixer in required proportion, including heating materials and cleaning the road surface (Hill sand 2 Cft for mixing and 2 Cft for dusting (Rate includes all cost of materials T&P and carriage upto site of work.

| 16768 | Sft | @ | Rs: | 4474.33 | P.% Sft | Rs: | 750256.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |

05. Increased Cost of bitumen.

| 3.25 | Tons | @ | Rs: | 18748.00 | P.Ton | Rs: | 60931.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Total | Rs: | 1015989.00 |

Note:- Subject to any Changes Variation in rates/

Quantities in Technical Sanction.

| (CONTRACTOR). | Executive Engineer |
| --- | --- |
|  | Highways Division |
|  | Khairpur. |

| Name of Work :- | Construction of Toe Wall 6’height Old National Highway to Mohalla Ghareeb-abad Upto Jamali Disposal Jamali road |
| --- | --- |

SCHEDULE-B.

| Quantity | Item of work. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- |

01. Excavation in foundation of building, bridges and other structure i/c degbelling, dressing, refilling around structure with excavated earth watering and ramming lead upto one chain and lift upto 5’(S.I.No.18b, P-5).

| 2070 | Cft | @ | Rs: | 3176.25 | P. %0 Cft | Rs: | 6575.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |

02. C.C.brick or stone ballast 1-1/2” to 2” guage Ratio (1:4:8) (S.I.No.4, P-17).

| 517 | Cft | @ | Rs: | 9416.28 | P. % Cft | Rs: | 48682.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |

03. Pacca brick work in f/p in cement sand mortar (1:4). (S.I.No.4, P-25).

| 1509 | Cft | @ | Rs: | 12501.41 | P. % Cft | Rs: | 188646.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |

04. Erection and removal of centering for R.c.c.or Plain C.C. works of portal wood (S.I.No.18b, P-21).

| 115 | Sft | @ | Rs: | 3127.41 | P. % Sft | Rs: | 3597.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |

05. Cement Concrete Plain i/c placing compacting, finishing and curing complete (i/c screening and washing of stone aggregate without shuttering (1:2:4) (S.I.No.5h, P-18).

| 65 | Cft | @ | Rs: | 14429.25 | P. % Cft | Rs: | 9379.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |

06. Cement Plaster (1:3) upto 20’height ½”thick.

| 805 | Sft | @ | Rs: | 2344.59 | P. % Sft | Rs: | 18874.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Total | Rs: | 275753.00 |

Note:- Subject to any Changes Variation in rates/

Quantities in Technical Sanction.

| (CONTRACTOR). | Executive Engineer |
| --- | --- |
|  | Highways Division |
|  | Khairpur. |