(  SCHEDULE B )

NAME OF SCHEME: - M&R OF Kharal Abad road to village

Mithal Jageerani Mile 0/0-0/6 (Selected Portion)

| QTY. | ITEM OF WORK | QTY | RATE. | UNIT | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

1/-Laying Brick on end edging i/cs: supply of 9’’x4-1/2’’x3’’Ist: Class

Burnt bricks excavation for having edging with small size parallel

to the road (Rate i/cs: lead up to side of Work)

3399        Rft;             @Rs: 2917/51                P.%Rft:                   Rs: 99166/=

2/-Preparing Base course i/cs: supplying & spreading stone

metal of approved quality properly grade to maximum size of

1-1/2” in required thickness to proper camber & grade i/cs:

supplying & spreading 15 cft: screening & Non plastic quarry

fines filling depression with stone metal after initial rolling

i/cs: watering & compacting the same so as to achieve 100%

density as per modified AASHO Specification.(Rate i/cs: all cost

of materials T&P& carriage up to site of work.

8412       Cft:                  @Rs: 7991/15              P.% cft:                   Rs. 672257/=

3/-Providing Surface Dressing Ist: coat on new or existing surface

With 25  lbs: of bitumen & 2.75  cft: Crush bajri of required size i/cs:

cleaning the road surface rolling etc: complete.(Rate i/cs: all

cost of materials T&P& carriage up to site work).

22433        Sft:        @Rs:1289/41           P.% Sft:                                         Rs.289253 /=

4/-Providing I’’ thick (consolidate e) Premixed Carpet in proper

camber & grade i/cs: supplying of 10 Cft: Crush bajri 4 cft:

Hill Sand (of approved quality and grade) with 67 lbs: bitumen

of 80/100 penetration i/cs: mxing in mechanical mixer in required

proportion i/cs:heating materials and cleaning the road surface

(Hill sand 2cft:for mixing and 2cft: for dusting) Rate i/c all cost of

Materials T&P& carriage up to site of work

22433       Sft:      @Rs:4552/35            P.% Sft:                                     Rs. 1021359/=

5/Earth work embankment from barrow pits including laying in 6” layers

Breaking remming dressing complete lead up to 100’ ft up to 5 ft in ord: soil.

6118  Cft;               @Rs: 7233/57          P.%0Cft:                                 Rs: 44256/=

______________

Total Rs:2126291/=

4/  Decreased Cost of Bitumen.

9.21 Tons            @Rs: 17595/=            P.ton   (-)         Rs:162112/=

_______________

G.Total  Rs:1964189/=

Note:Subject any change/

variation quantities/Rates in

the Techincal Sanction

(CONTRACTOR)                                               EXECUTIVE ENGINEER

HIGHWAYS DIVISION

KHAIRPUR