SCHEDULE “B”

| NAME OF WORK | PROVIDING LAYING JOINTING & TESTING F.C RISING MAIN 10”DIA, PUMPING MACHINERY I/C INTERCONNECTION FOR DISPOSAL WORK TANDU MASSU KHAN ZONE FOR URBAN DRAINAGE SCHEME GHOTKI (MEGA PROJECT). |
| --- | --- |

| A | F.C RISING MAIN 10” DIA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 01 | Excavation for pipe line in trenches & pits in soft soil i/c trimming & dressing sides to true alignment & shape leveling of beds to correct level & grade, cutting joint, holes & disposal of surplus earth with in one chain as directed by Engineer incharge .providing fence guard, light flags & temporary crossing for non vehicular traffic where ever required lift up to 5 ft (1.52 m) & lead up to one chain (30.5 m) (P.H.S.I no-1 p-46) |  |  |  |  |
|  | 39375.00CFT | RS:3600/00 |  | P-%0 CFT | RS: 141750/00 |
| 02 | Providing laying jointing FC Pressure Pipe of Class B and fixing in trenches i/c cutting fitting jointing with Z joint with one rubber ring i/c testing with water to a head of 6im or 200 Ft. (PHSI No: 1 P No: 22) |  |  |  |  |
|  | 4500.00 RFT | RS: 619/00 |  | P-RFT | RS: 2785500/00 |
| 03 | CI Specials |  |  |  |  |
|  | CI Bends 10”dia 90 |  |  |  |  |
|  | 2 Nos. | RS: 6662/50 |  | EACH | RS: 13325/00 |
|  | CI Bends 10”dia 45 |  |  |  |  |
|  | 1 No. | RS: 5037/00 |  | EACH | RS: 5037/00 |
| 04 | Supplying FC Cement Joints B Class (Sch: of Mat: 4 P No: 90) |  |  |  |  |
|  | 3 Nos. | RS: 446/00 |  | EACH | RS: 1338/00 |
| 05 | One pair of Rubber Ring for FC Pressure Pipe B Class (Sch: of Mat: 7 P No: 90) |  |  |  |  |
|  | 06 Nos. | RS: 196/00 |  | EACH | RS: 1176/00 |
| 06 | Making joints to C.I specials fitting i/c laying of the required diameter & testing the joints along with the AC Pressure pipe line to the specified pressure & making good to all leaky joints etc complete (PHSI No: 24 P No: 81) |  |  |  |  |
|  | 05 Joints | RS: 72/00 |  | P.JOINT | RS: 360/00 |
| 07 | Refilling the excavated stuff in trenches 6’ thick layers i/c watering ramming to full compaction etc complete (P.H.S.I no-24 p-53) |  |  |  |  |
|  | 35437.50 CFT | RS: 2760/00 |  | P%0 CFT | RS: 97808/00 |
|  |  |  |  | TOTAL | RS: 3046294/00 |
| B | PUMPING MACHINERY |  |  |  |  |
| 01 | Supplying & installing in position i/c transportation to site of work at Zone Tandu Massu Zone for urban Drainage Scheme Ghotki City Mega Project, Electric pumping set 15BHP electric motor (Siemens Made) frame size 200-L-4 pole 1500 RPM totally enclosed fan cooled squirrel cage induction motors 3 phase 400 volts 5% Hz2S suitable for VFD operation ambient temperature 40C attitude 1000m above sea level construction 1M B3 (loot mounted) degree of protection IP 65 insulation class F service factor 1.1 coupled with KSB KWPZ non clogging sullage pup type (KWZP 100-250) 5”x4” mounted on KSB steel frame/ coupling with sleeve capable of giving discharge 500GPM against the required head 40ft i/c installed on C.C foundation Ratio (1:2:4) with M.S nuts and bolts of 5/8”dia to be embedded in CC foundation under / over voltage protection phase failure protection etc complete. Providing & laying main or sub mains with size 4-7/064 (16mm2) copper conductor in 1-1/2”dia PVC conduct on surface as required i/c providing and fixing earthing set with 2’x2x1/2” copper pate buried in ground at a depth of 12” or less if water common out from the ground level with out and charcoal etc i/c making the pipe 12”deep by execution of all type of soil earth plain to be connected with CC Bars compare with own in ½” GI pipe strength from the earth plate i/c earth work in ground at depth as per Pak PWD specification and testing the 72 hours. |  |  |  |  |
|  | 02SETS |  | RS: 752000/00 | P-SET | RS: 1504000/00 |

| C | DIESEL OIL ENGINE. |  |  |  |
| --- | --- | --- | --- | --- |
| 01 | Supplying and installing 20BHP (GOLDEN Made) model GD-1110L 2200 RPM coupled with KSB non clogging Horizontal Pump Type KSB-250 size 4”x5” capable discharging 500 against the head of 50.ft i/c frame pully for engine and pump with belt and installing of pump set CC foundation with nuts and bolts and testing of the pumping set against the required head at 48 hours R.A approved. |  |  |  |
|  | 01SET | RS: 569000/00 | P-SET | RS: 569000/00 |
| D | INTERCONNECTION |  |  |  |
| 01 | Providing laying & fixing in trenches i/c fitting pointing &testing in all respect the high density polyethylene PE pipe (HDPE-100) for W/S confirming ISO 4427/DIN8074/8075 B.S 3580 & PSI 3051 (PHSI No: F-I P No: 25) PN-10 160m & 200mm. |  |  |  |
|  | 48.00 RFT 06 “ Dia | RS: 530/00 | P.RFT | RS: 25440/00 |
|  | 16.00 RFT 08 “ Dia | RS: 833/00 | P.RFT | RS: 13328/00 |
| 02 | Supplying CI bend with flanged ends with holes including turning and facing of flanges of all size. |  |  |  |
|  | 03.00 06 “ Dia 90 degree |  |  |  |
|  | 03.00 08 “ Dia 90 degree |  |  |  |
|  | 06 Nos: | RS: 6096/00 | P.CWT | RS: 18105/00 |
| 03 | CI tee with flanges end with holes i/c turning and facing of flanges for all sizes (SMI No: P No: 12) |  |  |  |
|  | 1.55 CWT | RS: 6096/00 | P.CWT | RS: 9449/00 |
| 04 | Supplying & installing M.S pipe made out MS sheet 3/16” i/c folding to correct curvature shape i/c |  |  |  |
|  | 24.00 RFT | RS: 749/34 | P.CWT | RS: 17984/00 |
| 05 | Providing and fixing MS flanges mad of MS plate having a thickness and total weight as mentioned against each item i/c the cost of making. |  |  |  |
|  | 12.00 Nos: 06 “ Dia | RS: 1520/00 | P.RFT | RS: 18240/00 |
|  | 10.00 Nos: 08 “ Dia | RS: 1953/00 | P.RFT | RS: 19530/00 |
| 06 | CI tapered flat bottomed or central papered flanged with holes i/c turning and facing of flanges for all size (SMI No: 9 ) |  |  |  |
|  | 2.58 CWT | RS: 6090/00 | P.CWT | RS: 15728/00 |
| 07 | Supplying CI Sluice valve heavy pattern SM PHSI 2 P 10. |  |  |  |
|  | 03 Nos: 6” Dia | RS: 9360/00 | Each | RS: 28080/00 |
| 08 | CI supplying reflex valve heavy pattern (test pressure 21.00kg / sq cm or 300ibs inch) SM PHSI 6 P 11. |  |  |  |
|  | 03 Nos: 6” Dia | RS: 4062/50 | Each | RS: 12188/00 |
| 09 | Supplying of Foot valve heavy pattern with one cone end type gate SM PHSI 2 P 10. |  |  |  |
|  | 03 Nos: 6” Dia | RS: 1381/25 | Each | RS: 4144/00 |
| 10 | Butt fussion Joints 3”Dia |  |  |  |
|  | 12.00 Nos: 06 “ Dia | RS: 1000/00 | P.RFT | RS: 12000/00 |
|  | 10.00 Nos: 08 “ Dia | RS: 1800/00 | P.RFT | RS: 18000/00 |
| 11 | Flange Adopter 3”Dia |  |  |  |
|  | 12.00 Nos: 06 “ Dia | RS: 2025/00 | P.RFT | RS: 24300/00 |
|  | 10.00 Nos: 08 “ Dia | RS: 7425/00 | P.RFT | RS: 74250/00 |
| 12 | Jointing C.I/M.S flanges pipes & specials flanged & inside trenches i/c supplying rubber packing of the required thickness nuts bolts with washers etc & tools required for jointing & the joints to the specified pressure etc complete (PHSI No: 1 P No:40) 6’Dia |  |  |  |
|  | 12.00 Nos: 06 “ Dia | RS: 938/00 | P.RFT | RS: 11256/00 |
|  | 10.00 Nos: 08 “ Dia | RS: 940/00 | P.RFT | RS: 9400/00 |
|  |  |  | TOTAL | RS: 331414/00 |

ABSTRACT

| S# | NAME OF WORK | SCHEDULE | NON- SCHEDULE | TOTAL |
| --- | --- | --- | --- | --- |
| 1 | FC Rising Main 10” Dia | 3046294.00 | 00 | 3046294.00 |
| 2 | Pumping Machinery | 00 | 1504000.00 | 1504000.00 |
| 3 | Diesel Oil Engine | 00 | 569000.00 | 569000.00 |
| 4 | Interconnection | 331414.00 | 00 | 331414.00 |
|  | TOTAL | 3377708.00 | 2073000.00 | 5450708.00 |

| CONTRACTOR | EXECUTIVE ENGINEER PUBLIC HEALTH ENGG: DIVISION GHOTKI @ MIRPUR MATHELO |
| --- | --- |

CONDITION

| 01 | The work shall have to be executed according to PWD/PHE specification |
| --- | --- |
| 02 | No premium on non schedule item shall be allowed to the contractor |
| 03 | Any error omission in the description of item of work and the rate shall be governed with the relevant schedule of rate and estimate |
| 04 | The contractor shall have to bring the material to be used from the quarries mentioned in the estimate |

| CONTRACTOR | EXECUTIVE ENGINEER PUBLIC HEALTH ENGG: DIVISION GHOTKI @ MIRPUR MATHELO |
| --- | --- |