SCHEDULE “B”.

Name of Work: -	Providing Laying Jointing and Testing RCC Sewer 15” Dia Zone C  i/c Catch Pits for Rural Drainage Scheme Halani Taluka Mehrabpur District Naushahro Feroze.

| ###### S.No | Description of Item | Qty | # Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |

PART-I.        RCC SEWER 15”  DIA.

1.	Dismantling and removal road mantling (CSI No. 51 Page No.13).

Qty: 254.34 Rft @ Rs.605/=        	P.% cft. 				Rs.1539/=

2.	Excavation for pipe lines in trenches pits in soft soil i/c trimming dressing sides to

true alignment and shape leveling of beds of trenches to correct                                                       level and grade cutting joint holes and disposal of Surplus Earth                                                   within one chain as directed by Engineer Incharge fence guards,                                                       lights, flags and temporary crossing for non vehicular traffic wherever		                required lift up to 5’ft and lead upto one chain ( P.H.S.I No.1 P.No.60).

Depth 0-5'. 15"dia

Qty: 28420.00 Rft @ Rs.3600/=        	P.%0cft. 			Rs.102312/=

3.	Excavation for pipe lines in trenches pits in Wet Soil i/c trimming dressing sides to

true alignment and shape leveling of beds of trenches to correct                                                       level and grade cutting joint holes and disposal of Surplus Earth                                                   within one chain as directed by Engineer Incharge fence guards,                                                       lights, flags and temporary crossing for non vehicular traffic wherever		                required lift up to 5’ft and lead upto one chain ( P.H.S.I No.1 P.No.60).

Depth 0-5'.

Qty: 963.00 cft @ Rs.5400/=			P%0 cft	 		Rs.5200/=

4.	Excavation in foundation of building bridges and other structures i/c dag					belling dressing refilling around the structures with excavated Earth 					watering ramming lead upto one chain and lift upto to 5’ ft (CSI No.18 b 					Page No. 04).

4”dia.

Qty: 2317.32 cft @ Rs.3176/25	P%0 cft		 		Rs.19321/=

5.	Extra for wet earth work.

Qty: 2317.32 cft @ Rs.1058/80	P%0 cft		 		Rs.2453/=

6.	Providing RCC pipe of ASTM C-76-62 T/C Class—II wall B and fixing					in trenches  i/c cutting binding and jointing with rubber rings i/c testing 					with water to specified pressure (PHSI No. 1 Page no.17).

15” Dia.

Qty: 2030.00 Rft @ Rs.515/00	P.Rft			 		Rs.1045450/=

7.	Cement concrete plain i/c placing compacting finishing and curing						complete i/c screening and washing of stone aggregate without 						shuttering (CSI No.5 (i) Page No.16)

Ratio 1:4:8

Qty: 579.33 cft @ Rs.11288/75      	P.%cft. 				Rs.65399/=

Ratio 1:2:4

Qty: 388.00 cft @ Rs.14429/25      	P.%cft. 				Rs.55985/=

8.	Errection and removal of centering for RCC Pre Cost  						concrete work of deodar wood 2nd class(CSI No.19 Page No.17).

Qty: 410.00 cft @ Rs.3127/41  	P.%cft. 				Rs.12822/=

9.	RCC work in all labour and material except the cost of steel reinforcement and its

labour for B/B which will be paid separetely. The rate also i/.c all kinds of forms

moulds lifting, shutting, curing rendering and finsihsing the exposed surface i/c

S/W of shingle) (a) RCC work in Roof slab beams columns rafts lintels and other

structure member lad in situe or pre-cost laid in position Complete in all respects

(CSI  No.6  Page No.15).(Ratio 1:2:4).

Qty: 2333.40 Cft at Rs.337/= 		P.cft 		          			Rs.786356/=

10.	Fabrication of Mild Steel reinforcement concrete for CC i/c

Cutting, Bending Laying in position making joints and fastening

i/c Cost of binding wire also including removal of rust from bars

(CSI No.8 a  P.No.16).

Qty: 93.75 Cwt. at Rs.5001/70    P/Cwt.	       				Rs.468909/=

11.	Providing fixing  C.I Main Hole ramei/c cost of material							(PHSI No.1 Page No.23).

Qty: 143.50 Cwt. at Rs.6985/= 	P/Cwt.	       				Rs.1002348/=

12.	Refilling excavated stuff  6" thick layers i/c watering 						ramming to full compacting etc complete (PHSI No.24 Page No.77).

Qty: 26445.00 cft @ Rs.2760/=	P%0 cft.				Rs.72988/=

Total	Rs.3641082/=

PART-II.	CATCH PITS.

1.	Excavation in foundation of building bridges and other structure						include Dug bailing dressing refilling around the structure with 						excavated earth watering and ramming etc completed lead upto 						1 chain lift upto 5ft (CSI No. 18 Page No.4).

Qty: 80.00 cft @ Rs.3176/25		P.%0cft. 				Rs.254/=

2.	Cement concrete plain including placing compacting and curing

complete i/c  screening and washing of stone aggregate without

shuttering and washing of stone (CSI No.5 P.No.16)

Ratio  1:4:8.

Qty: 10.00 cft at Rs.11288/75		P.% cft 		         		Rs.1129/=

Ratio  1:2:4.

Qty: 03.00 cft at Rs.14429/25		P.% cft 		         		Rs.433/=

3.	Pacca Brick Work in foundation upto Plinth in cement 							sand mortar.( Ratio 1:6) (CSI No:4 E P.No:25).

Qty: 47.25  cft  at  Rs.11948/36 P.% cft 	       				Rs.5646/=

4.	Cement plaster 1:4 upto 20’ height ½” thick (CSI No.13 P.52).

Qty: 44.75  sft @  Rs.2283/93	P.% sft.       			 	Rs.1022/=

5.	Providing RCc pipe with collars and digging the trenches to required					depth and fixing to in position cutting and jointing with maxphalt 						composition and cement mortar 1:1: and testing with rater pressure etc					a head executed stuff (PHSI No. 1 Page No.15).

Qty: 12.00 Rft @  Rs.191/25 	P.Rft.       			 	Rs.2295/=

6.	fabrication of heavy steel work with angle tee flats iron round iron 						and sheet iron for making truses griders tanks tc i/c cutting bending 					drilling reviting handing assembling and fixing etc complete 						(CSI NO.2 Page NO.91).

Qty: 0.648  cwt @  Rs.4928/49 	P.cwt.       			 Rs.3195/=

7.	Small iron work such as gussel plate kness bends strips rings etc						i/c cutting drilling riverting handling assembling and fixing but 						excluding errection in position (CSI No. 1 Page No.91).

Qty: 0.589  Cwt. at Rs.6420/61 	 P/Cwt.			Rs.3784/=

Total	Rs.17757/=

ABSTRACT OF SCHEDULE B”.

| S.NO. | NAME OF WORK | SCHEDULE ITEMS | NON SCHEDULE ITEMS | TOTAL |
| --- | --- | --- | --- | --- |
| 1 | RCC SEWER 15" | Rs.3641082/= | - | Rs.3641082/= |
| 2. | CATCH PITS. | Rs.17757/= | - | Rs.17757/= |
|  | Total | Rs.3658839/= | - | Rs.3658839/= |

CERTIFICATE.

- Any error or omission in the description of items or rates the original estimate schedule of rates will be referred to correct accordingly.

- No premium will be allowed on non-schedule items.

- The work will be carried out according to the drawing design and specifications

shown in the estimate.

EXECUTIVE ENGINEER

PUBLIC HEALTH ENGG; DIVISION

NAUSHAHRO FEROZE