SCHEDULE OF QUANTITIES

WORK No: 01

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:   8,20,587 (+) O/R		Earnest Money Rs:	16,412 (+) 2% of O/R

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | P/L R.C.C. PIPE DRAIN 12” DIA FATHER COLONY UC-40, MAURIPUR KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Dismantling cement concrete plain 1:2:4. | 938 Cft | 3327.50 | %Cft | 31,212.00 |
| 2. | Excavation for pipe line in trenches and pits in all kind of soil of murum i/c trimming and dressing side to true alignment and shape traveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guard light flags and temporary crossing for non vehicular traffic wherever required left upto 05’ and lead upto 01 chain. | 5625 Cft | 4650.00 | 1000 Cft | 26,156.00 |
| 3. | A. PROVIDING, LAYING & JOINTING (Rubber Gasket, JOINTS). Providing, laying R.C.C. pipes of A.S.T.M. C-76-62 T/C: 76-70 of class II wall B and fixing in trench i/c cutting, fitting and jointing with rubber ring i/c testing with water to specified pressure. ………...………………………12” Dia ……...…..……………………. | 750 Rft | 412.00 | P/Rft | 3,09,000.00 |
| 4. | Providing manhole 4’ dia (inside) and 5’ clear depth complete with 21” dia ( inside ) C.I frame & 21” dia of 3” multiply 3 x 8, flat iron frame for R.C.C 1:2:4 pre-cast cover i/c the cost of M.S bars reinforcement and two handles of 5 clear depth C.C 1:2:4 cast in Situ walls with 50% screened graded bajri of 9” thick i/c benching & channel ½” thick cement plaster 1:3 on all inside walls surface complete with c.i Foot rest i/c cutting asphalt roads excavation in all kind of soil, back filling and disposal of surplus stuff etc. | 36 Nos | 9307.86 | Each | 3,35,083.00 |
| 5. | Add/Less for extra depth beyond 5’ depth, less than 5’ depth deduct at the same rate. | ( - ) 36 Rft | 1609.15 | P/Rft | ( - ) 57,929.00 |
| 6. | Refilling the excavated stuff in trenches 6” thick layer i/c watering ramming to full compaction etc complete. | 5063 Cft | 2760.00 | 1000 Cft | 13,974.00 |
| 7. | Cartage of 100 Cft / 05 Tons of all materials like stone aggregate, spawl, coal, lime, surkhi etc B.G. rail fastening point and crossing bridge girders, pipes sheet rails M.S. bar etc or 1000 number bricks 10’ x 5’ x 3’ or 1000 manuds of fuel wood by trucks or any other means owned by the contractors (lead upto 15 miles etc). | 938 Cft | 1065.04 | %Cft | 9,990.00 |
| 8. | Manufacturing and supplying of R.C.C. ring slab of 21” Dia inside 36”Dia outside 7.5” width and 6” thick i/c 3/8” Dia for steel bars two concentric ring with 3/8” Dia 08 Nos. cross linked bars welded and two sunk type hooks casted etc. | 25 Nos | 1906.60 | Each | 47,663.00 |
| 9. | Shifting and fixing R.C.C. pre-cast ring slab in c.c. 1:2:4 etc. | 25 Nos | 563.00 | Each | 14,075.00 |

Cont’d Page No: 02

Page No:02

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 10. | Manufacturing and supplying R.C.C. manhole covers casting c.c. 1:2:4 ratio 3 inch deep at center bar, reinforced with 3/8” dia tour steel 2.50” deep M.S. plate other curing, stacking and transportation within 10 miles. ………...………………………12” Dia ……...…..……………………. | 100 Nos | 913.63 | Each | 91,363.00 |
| 11. | Supply of Labour on Hiring Basis for lifting the Garbage (whenever, wherever, as much as required) and as per direction of Engineer Incharge. | 190 Nos |  | Each |  |
| Total Estimated Cost |  |  |  |  | 8,20,587 (+) Open Rate |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

*********************************

I/We hereby quoted Rates @                           %  above/below the S/R

(+) Open Rate Rs:_________________ for complete work/job.

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 02

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:   7,96,513 (+) O/R		Earnest Money Rs:	15,930 (+) 2% of O/R

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | P/L R.C.C. PIPE DRAIN 12” DIA MADNI COLONY UC-40, MAURIPUR KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Excavation for pipe line in trenches and pits in all kind of soil of murum i/c trimming and dressing side to true alignment and shape traveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guard light flags and temporary crossing for non vehicular traffic wherever required left upto 05’ and lead upto 01 chain. | 6000 Cft | 4650.00 | 1000 Cft | 27,900.00 |
| 2. | A. PROVIDING, LAYING & JOINTING (Rubber Gasket, JOINTS). Providing, laying R.C.C. pipes of A.S.T.M. C-76-62 T/C: 76-70 of class II wall B and fixing in trench i/c cutting, fitting and jointing with rubber ring i/c testing with water to specified pressure. ………...………………………12” Dia ……...…..……………………. | 800 Rft | 412.00 | P/Rft | 3,29,600.00 |
| 3. | Providing manhole 4’ dia (inside) and 5’ clear depth complete with 21” dia ( inside ) C.I frame & 21” dia of 3” multiply 3 x 8, flat iron frame for R.C.C 1:2:4 pre-cast cover i/c the cost of M.S bars reinforcement and two handles of 5 clear depth C.C 1:2:4 cast in Situ walls with 50% screened graded bajri of 9” thick i/c benching & channel ½” thick cement plaster 1:3 on all inside walls surface complete with c.i Foot rest i/c cutting asphalt roads excavation in all kind of soil, back filling and disposal of surplus stuff etc. | 40 Nos | 9307.86 | Each | 3,72,314.00 |
| 4. | Add/Less for extra depth beyond 5’ depth, less than 5’ depth deduct at the same rate. | ( - ) 40 Rft | 1609.15 | P/Rft | ( - ) 64,366.00 |
| 5. | Refilling the excavated stuff in trenches 6” thick layer i/c watering ramming to full compaction etc complete. | 5400 Cft | 2760.00 | 1000 Cft | 14,904.00 |
| 6. | Manufacturing and supplying of R.C.C. ring slab of 21” Dia inside 36”Dia outside 7.5” width and 6” thick i/c 3/8” Dia for steel bars two concentric ring with 3/8” Dia 08 Nos. cross linked bars welded and two sunk type hooks casted etc. | 20 Nos | 1906.60 | Each | 38,132.00 |
| 7. | Shifting and fixing R.C.C. pre-cast ring slab in c.c. 1:2:4 etc. | 20 Nos | 563.00 | Each | 14,075.00 |
| 8. | Manufacturing and supplying R.C.C. manhole covers casting c.c. 1:2:4 ratio 3 inch deep at center bar, reinforced with 3/8” dia tour steel 2.50” deep M.S. plate other curing, stacking and transportation within 10 miles. ………...………………………12” Dia ……...…..……………………. | 70 Nos | 913.63 | Each | 63,954.00 |

Cont’d Page No: 02

Page No:02

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 9. | Supply of Labour on Hiring Basis for lifting the Garbage (whenever, wherever, as much as required) and as per direction of Engineer Incharge. | 225 Nos |  | Each |  |
| Total Estimated Cost |  |  |  |  | 7,96,513 (+) Open Rate |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

*********************************

I/We hereby quoted Rates @                           %  above/below the S/R

(+) Open Rate Rs:_________________ for complete work/job.

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 03

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:   4,22,454 (+) O/R		Earnest Money Rs:	8,449 (+) 2% of O/R

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | IMPROVEMENT OF NALLAH HAJI RAFIQUE GALI UC-44, BHUTTA VILLAGE KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Dismantling cement concrete R.C.C. (1:2:4) disposal lead upto 10 miles etc. | 500 Cft | 5445.00 | %Cft | 27,225.00 |
| 2. | Cartage of 100 Cft / 05 Tons of all materials like stone aggregate, spawl, coal, lime, surkhi etc B.G. rail fastening point and crossing bridge girders, pipes sheet rails M.S. bar etc or 1000 number bricks 10”x5”x3” or 1000 manuds of fuel wood by trucks or any other means owned by the contractors (lead upto 15 miles etc). | 15000 Cft | 1065.04 | %Cft | 1,59,756.00 |
| 3. | R.C.C. working roof slab beams, column, rafts, lintels and other structural members laid in situ or pre-cast laid in position complete in all respect 1:2:4. | 500 Cft | 337.00 | P/Cft | 1,68,500.00 |
| 4. | Fabrication of mild steel reinforcement for cement concrete including cutting, bending, laying in position making joints and fastenings including cost of binding wire (also includes removal of rust from bars) using tor bars. | 13.39 Cwt | 5001.70 | P/Cwt | 66,973.00 |
| 5. | Supply of Labour on Hiring Basis for lifting the Garbage and cleaning of Nallah/Sewerage System (whenever, wherever, as much as required) and as per direction of Engineer Incharge. | 635 Nos |  | Each |  |
| Total Estimated Cost |  |  |  |  | 4,22,454 (+) Open Rate |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

*********************************

I/We hereby quoted Rates @                           %  above/below the S/R

(+) Open Rate Rs:_________________ for complete work/job.

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 04

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:   3,82,515 (+) O/R		Earnest Money Rs:	7,650 (+) 2% of O/R

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | IMPROVEMENT OF NALLAH BABA ISLAND IN UC-41, KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Dismantling cement concrete R.C.C. (1:2:4) disposal lead upto 10 miles etc. | 500 Cft | 5445.00 | %Cft | 27,225.00 |
| 2. | Cartage of 100 Cft / 05 Tons of all materials like stone aggregate, spawl, coal, lime, surkhi etc B.G. rail fastening point and crossing bridge girders, pipes sheet rails M.S. bar etc or 1000 number bricks 10”x5”x3” or 1000 manuds of fuel wood by trucks or any other means owned by the contractors (lead upto 15 miles etc). | 11250 Cft | 1065.04 | %Cft | 1,19,817.00 |
| 3. | R.C.C. working roof slab beams, column, rafts, lintels and other structural members laid in situ or pre-cast laid in position complete in all respect 1:2:4. | 500 Cft | 337.00 | P/Cft | 1,68,500.00 |
| 4. | Fabrication of mild steel reinforcement for cement concrete including cutting, bending, laying in position making joints and fastenings including cost of binding wire (also includes removal of rust from bars) using tor bars. | 13.39 Cwt | 5001.70 | P/Cwt | 66,973.00 |
| 5. | Supply of Labour on Hiring Basis for lifting the Garbage and cleaning of Nallah/Sewerage System (whenever, wherever, as much as required) and as per direction of Engineer Incharge. | 682 Nos |  | Each |  |
| Total Estimated Cost |  |  |  |  | 3,82,515 (+) Open Rate |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

*********************************

I/We hereby quoted Rates @                           %  above/below the S/R

(+) Open Rate Rs:_________________ for complete work/job.

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 05

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:   3,82,515 (+) O/R		Earnest Money Rs:	7,650 (+) 2% of O/R

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | IMPROVEMENT OF NALLAH BHIT ISLAND IN UC-41, KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Dismantling cement concrete R.C.C. (1:2:4) disposal lead upto 10 miles etc. | 500 Cft | 5445.00 | %Cft | 27,225.00 |
| 2. | Cartage of 100 Cft / 05 Tons of all materials like stone aggregate, spawl, coal, lime, surkhi etc B.G. rail fastening point and crossing bridge girders, pipes sheet rails M.S. bar etc or 1000 number bricks 10”x5”x3” or 1000 manuds of fuel wood by trucks or any other means owned by the contractors (lead upto 15 miles etc). | 11250 Cft | 1065.04 | %Cft | 1,19,817.00 |
| 3. | R.C.C. working roof slab beams, column, rafts, lintels and other structural members laid in situ or pre-cast laid in position complete in all respect 1:2:4. | 500 Cft | 337.00 | P/Cft | 1,68,500.00 |
| 4. | Fabrication of mild steel reinforcement for cement concrete including cutting, bending, laying in position making joints and fastenings including cost of binding wire (also includes removal of rust from bars) using tor bars. | 13.39 Cwt | 5001.70 | P/Cwt | 66,973.00 |
| 5. | Supply of Labour on Hiring Basis for lifting the Garbage and cleaning of Nallah/Sewerage System (whenever, wherever, as much as required) and as per direction of Engineer Incharge. | 682 Nos |  | Each |  |
| Total Estimated Cost |  |  |  |  | 3,82,515 (+) Open Rate |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

*********************************

I/We hereby quoted Rates @                           %  above/below the S/R

(+) Open Rate Rs:_________________ for complete work/job.

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 06

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:        9,43,841/-		Earnest Money Rs:	         18,877/-

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | P/L R.C.C. PIPE DRAIN 12” DIA QUBA MASJID IN UC-44, BHUTTA VILLAGE KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Dismantling cement concrete plain 1:2:4. | 1250 Cft | 3327.50 | %Cft | 41,594.00 |
| 2. | Excavation for pipe line in trenches and pits in all kind of soil of murum i/c trimming and dressing side to true alignment and shape traveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guard light flags and temporary crossing for non vehicular traffic wherever required left upto 05’ and lead upto 01 chain. | 7500 Cft | 4650.00 | 1000 Cft | 34,875.00 |
| 3. | A. PROVIDING, LAYING & JOINTING (Rubber Gasket, JOINTS). Providing, laying R.C.C. pipes of A.S.T.M. C-76-62 T/C: 76-70 of class II wall B and fixing in trench i/c cutting, fitting and jointing with rubber ring i/c testing with water to specified pressure. ………...………………………12” Dia ……...…..……………………. | 1000 Rft | 412.00 | P/Rft | 4,12,000.00 |
| 4. | Providing manhole 4’ dia (inside) and 5’ clear depth complete with 21” dia ( inside ) C.I frame & 21” dia of 3” multiply 3 x 8, flat iron frame for R.C.C 1:2:4 pre-cast cover i/c the cost of M.S bars reinforcement and two handles of 5 clear depth C.C 1:2:4 cast in Situ walls with 50% screened graded bajri of 9” thick i/c benching & channel ½” thick cement plaster 1:3 on all inside walls surface complete with c.i Foot rest i/c cutting asphalt roads excavation in all kind of soil, back filling and disposal of surplus stuff etc. | 55 Nos | 9307.86 | Each | 5,11,932.00 |
| 5. | Add/Less for extra depth beyond 5’ depth, less than 5’ depth deduct at the same rate. | ( - ) 55 Rft | 1609.15 | P/Rft | ( - ) 88,503.00 |
| 6. | Refilling the excavated stuff in trenches 6” thick layer i/c watering ramming to full compaction etc complete. | 6750 Cft | 2760.00 | 1000 Cft | 18,630.00 |
| 7. | Cartage of 100 Cft / 05 Tons of all materials like stone aggregate, spawl, coal, lime, surkhi etc B.G. rail fastening point and crossing bridge girders, pipes sheet rails M.S. bar etc or 1000 number bricks 10’ x 5’ x 3’ or 1000 manuds of fuel wood by trucks or any other means owned by the contractors (lead upto 15 miles etc). | 1250 Cft | 1065.04 | %Cft | 13,313.00 |
| Total Estimated Cost |  |  |  |  | 9,43,841.00 |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

*********************************

I/We hereby quoted Rates @                           %  above/below the S/R

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 07

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:        9,43,841/-		Earnest Money Rs:	         18,877/-

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | P/L R.C.C. PIPE DRAIN 12” DIA KATCHI MOHALLAH IN UC-44, BHUTTA VILLAGE KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Dismantling cement concrete plain 1:2:4. | 1250 Cft | 3327.50 | %Cft | 41,594.00 |
| 2. | Excavation for pipe line in trenches and pits in all kind of soil of murum i/c trimming and dressing side to true alignment and shape traveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guard light flags and temporary crossing for non vehicular traffic wherever required left upto 05’ and lead upto 01 chain. | 7500 Cft | 4650.00 | 1000 Cft | 34,875.00 |
| 3. | A. PROVIDING, LAYING & JOINTING (Rubber Gasket, JOINTS). Providing, laying R.C.C. pipes of A.S.T.M. C-76-62 T/C: 76-70 of class II wall B and fixing in trench i/c cutting, fitting and jointing with rubber ring i/c testing with water to specified pressure. ………...………………………12” Dia ……...…..……………………. | 1000 Rft | 412.00 | P/Rft | 4,12,000.00 |
| 4. | Providing manhole 4’ dia (inside) and 5’ clear depth complete with 21” dia ( inside ) C.I frame & 21” dia of 3” multiply 3 x 8, flat iron frame for R.C.C 1:2:4 pre-cast cover i/c the cost of M.S bars reinforcement and two handles of 5 clear depth C.C 1:2:4 cast in Situ walls with 50% screened graded bajri of 9” thick i/c benching & channel ½” thick cement plaster 1:3 on all inside walls surface complete with c.i Foot rest i/c cutting asphalt roads excavation in all kind of soil, back filling and disposal of surplus stuff etc. | 55 Nos | 9307.86 | Each | 5,11,932.00 |
| 5. | Add/Less for extra depth beyond 5’ depth, less than 5’ depth deduct at the same rate. | ( - ) 55 Rft | 1609.15 | P/Rft | ( - ) 88,503.00 |
| 6. | Refilling the excavated stuff in trenches 6” thick layer i/c watering ramming to full compaction etc complete. | 6750 Cft | 2760.00 | 1000 Cft | 18,630.00 |
| 7. | Cartage of 100 Cft / 05 Tons of all materials like stone aggregate, spawl, coal, lime, surkhi etc B.G. rail fastening point and crossing bridge girders, pipes sheet rails M.S. bar etc or 1000 number bricks 10’ x 5’ x 3’ or 1000 manuds of fuel wood by trucks or any other means owned by the contractors (lead upto 15 miles etc). | 1250 Cft | 1065.04 | %Cft | 13,313.00 |
| Total Estimated Cost |  |  |  |  | 9,43,841.00 |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

*********************************

I/We hereby quoted Rates @                           %  above/below the S/R

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 08

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:        9,43,841/-		Earnest Money Rs:	         18,877/-

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | P/L R.C.C. PIPE DRAIN 12” DIA YAQOOB MUNDA CHOWK UC-44, BHUTTA VILLAGE KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Dismantling cement concrete plain 1:2:4. | 1250 Cft | 3327.50 | %Cft | 41,594.00 |
| 2. | Excavation for pipe line in trenches and pits in all kind of soil of murum i/c trimming and dressing side to true alignment and shape traveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guard light flags and temporary crossing for non vehicular traffic wherever required left upto 05’ and lead upto 01 chain. | 7500 Cft | 4650.00 | 1000 Cft | 34,875.00 |
| 3. | A. PROVIDING, LAYING & JOINTING (Rubber Gasket, JOINTS). Providing, laying R.C.C. pipes of A.S.T.M. C-76-62 T/C: 76-70 of class II wall B and fixing in trench i/c cutting, fitting and jointing with rubber ring i/c testing with water to specified pressure. ………...………………………12” Dia ……...…..……………………. | 1000 Rft | 412.00 | P/Rft | 4,12,000.00 |
| 4. | Providing manhole 4’ dia (inside) and 5’ clear depth complete with 21” dia ( inside ) C.I frame & 21” dia of 3” multiply 3 x 8, flat iron frame for R.C.C 1:2:4 pre-cast cover i/c the cost of M.S bars reinforcement and two handles of 5 clear depth C.C 1:2:4 cast in Situ walls with 50% screened graded bajri of 9” thick i/c benching & channel ½” thick cement plaster 1:3 on all inside walls surface complete with c.i Foot rest i/c cutting asphalt roads excavation in all kind of soil, back filling and disposal of surplus stuff etc. | 55 Nos | 9307.86 | Each | 5,11,932.00 |
| 5. | Add/Less for extra depth beyond 5’ depth, less than 5’ depth deduct at the same rate. | ( - ) 55 Rft | 1609.15 | P/Rft | ( - ) 88,503.00 |
| 6. | Refilling the excavated stuff in trenches 6” thick layer i/c watering ramming to full compaction etc complete. | 6750 Cft | 2760.00 | 1000 Cft | 18,630.00 |
| 7. | Cartage of 100 Cft / 05 Tons of all materials like stone aggregate, spawl, coal, lime, surkhi etc B.G. rail fastening point and crossing bridge girders, pipes sheet rails M.S. bar etc or 1000 number bricks 10’ x 5’ x 3’ or 1000 manuds of fuel wood by trucks or any other means owned by the contractors (lead upto 15 miles etc). | 1250 Cft | 1065.04 | %Cft | 13,313.00 |
| Total Estimated Cost |  |  |  |  | 9,43,841.00 |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

*********************************

I/We hereby quoted Rates @                           %  above/below the S/R

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 09

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:        9,43,841/-		Earnest Money Rs:	         18,877/-

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | P/L R.C.C. PIPE DRAIN 12” DIA SULTANABAD IN UC-46, KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Dismantling cement concrete plain 1:2:4. | 1250 Cft | 3327.50 | %Cft | 41,594.00 |
| 2. | Excavation for pipe line in trenches and pits in all kind of soil of murum i/c trimming and dressing side to true alignment and shape traveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guard light flags and temporary crossing for non vehicular traffic wherever required left upto 05’ and lead upto 01 chain. | 7500 Cft | 4650.00 | 1000 Cft | 34,875.00 |
| 3. | A. PROVIDING, LAYING & JOINTING (Rubber Gasket, JOINTS). Providing, laying R.C.C. pipes of A.S.T.M. C-76-62 T/C: 76-70 of class II wall B and fixing in trench i/c cutting, fitting and jointing with rubber ring i/c testing with water to specified pressure. ………...………………………12” Dia ……...…..……………………. | 1000 Rft | 412.00 | P/Rft | 4,12,000.00 |
| 4. | Providing manhole 4’ dia (inside) and 5’ clear depth complete with 21” dia ( inside ) C.I frame & 21” dia of 3” multiply 3 x 8, flat iron frame for R.C.C 1:2:4 pre-cast cover i/c the cost of M.S bars reinforcement and two handles of 5 clear depth C.C 1:2:4 cast in Situ walls with 50% screened graded bajri of 9” thick i/c benching & channel ½” thick cement plaster 1:3 on all inside walls surface complete with c.i Foot rest i/c cutting asphalt roads excavation in all kind of soil, back filling and disposal of surplus stuff etc. | 55 Nos | 9307.86 | Each | 5,11,932.00 |
| 5. | Add/Less for extra depth beyond 5’ depth, less than 5’ depth deduct at the same rate. | ( - ) 55 Rft | 1609.15 | P/Rft | ( - ) 88,503.00 |
| 6. | Refilling the excavated stuff in trenches 6” thick layer i/c watering ramming to full compaction etc complete. | 6750 Cft | 2760.00 | 1000 Cft | 18,630.00 |
| 7. | Cartage of 100 Cft / 05 Tons of all materials like stone aggregate, spawl, coal, lime, surkhi etc B.G. rail fastening point and crossing bridge girders, pipes sheet rails M.S. bar etc or 1000 number bricks 10’ x 5’ x 3’ or 1000 manuds of fuel wood by trucks or any other means owned by the contractors (lead upto 15 miles etc). | 1250 Cft | 1065.04 | %Cft | 13,313.00 |
| Total Estimated Cost |  |  |  |  | 9,43,841.00 |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

*********************************

I/We hereby quoted Rates @                           %  above/below the S/R

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 10

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:   7,96,513 (+) O/R		Earnest Money Rs:	15,930 (+) 2% of O/R

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | P/L R.C.C. PIPE DRAIN 12” DIA GALI No:08, JINNAH ROAD UC-02, SHER SHAH KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Excavation for pipe line in trenches and pits in all kind of soil of murum i/c trimming and dressing side to true alignment and shape traveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guard light flags and temporary crossing for non vehicular traffic wherever required left upto 05’ and lead upto 01 chain. | 6000 Cft | 4650.00 | 1000 Cft | 27,900.00 |
| 2. | A. PROVIDING, LAYING & JOINTING (Rubber Gasket, JOINTS). Providing, laying R.C.C. pipes of A.S.T.M. C-76-62 T/C: 76-70 of class II wall B and fixing in trench i/c cutting, fitting and jointing with rubber ring i/c testing with water to specified pressure. ………...………………………12” Dia ……...…..……………………. | 800 Rft | 412.00 | P/Rft | 3,29,600.00 |
| 3. | Providing manhole 4’ dia (inside) and 5’ clear depth complete with 21” dia ( inside ) C.I frame & 21” dia of 3” multiply 3 x 8, flat iron frame for R.C.C 1:2:4 pre-cast cover i/c the cost of M.S bars reinforcement and two handles of 5 clear depth C.C 1:2:4 cast in Situ walls with 50% screened graded bajri of 9” thick i/c benching & channel ½” thick cement plaster 1:3 on all inside walls surface complete with c.i Foot rest i/c cutting asphalt roads excavation in all kind of soil, back filling and disposal of surplus stuff etc. | 40 Nos | 9307.86 | Each | 3,72,314.00 |
| 4. | Add/Less for extra depth beyond 5’ depth, less than 5’ depth deduct at the same rate. | ( - ) 40 Rft | 1609.15 | P/Rft | ( - ) 64,366.00 |
| 5. | Refilling the excavated stuff in trenches 6” thick layer i/c watering ramming to full compaction etc complete. | 5400 Cft | 2760.00 | 1000 Cft | 14,904.00 |
| 6. | Manufacturing and supplying of R.C.C. ring slab of 21” Dia inside 36”Dia outside 7.5” width and 6” thick i/c 3/8” Dia for steel bars two concentric ring with 3/8” Dia 08 Nos. cross linked bars welded and two sunk type hooks casted etc. | 20 Nos | 1906.60 | Each | 38,132.00 |
| 7. | Shifting and fixing R.C.C. pre-cast ring slab in c.c. 1:2:4 etc. | 20 Nos | 563.00 | Each | 14,075.00 |
| 8. | Manufacturing and supplying R.C.C. manhole covers casting c.c. 1:2:4 ratio 3 inch deep at center bar, reinforced with 3/8” dia tour steel 2.50” deep M.S. plate other curing, stacking and transportation within 10 miles. ………...………………………12” Dia ……...…..……………………. | 70 Nos | 913.63 | Each | 63,954.00 |

Cont’d Page No: 02

Page No:02

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 9. | Supply of Labour on Hiring Basis for lifting the Garbage (whenever, wherever, as much as required) and as per direction of Engineer Incharge. | 225 Nos |  | Each |  |
| Total Estimated Cost |  |  |  |  | 7,96,513 (+) Open Rate |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

*********************************

I/We hereby quoted Rates @                           %  above/below the S/R

(+) Open Rate Rs:_________________ for complete work/job.

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 11

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:   4,60,664 (+) O/R		Earnest Money Rs:	9,213 (+) 2% of O/R

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | IMPROVEMENT OF NALLAH USMAN GHANI MASJID ROAD UC-44, KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Dismantling cement concrete R.C.C. (1:2:4) disposal lead upto 10 miles etc. | 400 Cft | 5445.00 | %Cft | 21,780.00 |
| 2. | Cartage of 100 Cft / 05 Tons of all materials like stone aggregate, spawl, coal, lime, surkhi etc B.G. rail fastening point and crossing bridge girders, pipes sheet rails M.S. bar etc or 1000 number bricks 10”x5”x3” or 1000 manuds of fuel wood by trucks or any other means owned by the contractors (lead upto 15 miles etc). | 12750 Cft | 1065.04 | %Cft | 1,35,793.00 |
| 3. | R.C.C. working roof slab beams, column, rafts, lintels and other structural members laid in situ or pre-cast laid in position complete in all respect 1:2:4. | 400 Cft | 337.00 | P/Cft | 1,34,800.00 |
| 4. | Fabrication of mild steel reinforcement for cement concrete including cutting, bending, laying in position making joints and fastenings including cost of binding wire (also includes removal of rust from bars) using tor bars. | 10.71 Cwt | 5001.70 | P/Cwt | 53,568.00 |
| 5. | Erection and removal of centering for R.C.C. or plain cement concrete works of Deodar Wood (2nd class). | 900 Sft | 3127.41 | %Sft | 28,147.00 |
| 6. | Cement concrete plain including placing compaction finishing and curing complete including screening and washing at stone aggregate without ratio 1:2:4. | 600 Cft | 14429.25 | %Cft | 86,576.00 |
| 7. | Supply of Labour on Hiring Basis for lifting the Garbage and cleaning of Nallah/Sewerage System (whenever, wherever, as much as required) and as per direction of Engineer Incharge. | 580 Nos |  | Each |  |
| Total Estimated Cost |  |  |  |  | 4,60,664 (+) Open Rate |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

*********************************

I/We hereby quoted Rates @                           %  above/below the S/R

(+) Open Rate Rs:_________________ for complete work/job.

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 12

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:   3,69,904 (+) O/R		Earnest Money Rs:	7,398 (+) 2% of O/R

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | IMPROVEMENT OF NALLAH DERA SHER ALI TO DERA HAJI TARIQ IN UC-44, KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Dismantling cement concrete R.C.C. (1:2:4) disposal lead upto 10 miles etc. | 400 Cft | 5445.00 | %Cft | 21,780.00 |
| 2. | Cartage of 100 Cft / 05 Tons of all materials like stone aggregate, spawl, coal, lime, surkhi etc B.G. rail fastening point and crossing bridge girders, pipes sheet rails M.S. bar etc or 1000 number bricks 10”x5”x3” or 1000 manuds of fuel wood by trucks or any other means owned by the contractors (lead upto 15 miles etc). | 15000 Cft | 1065.04 | %Cft | 1,59,756.00 |
| 3. | R.C.C. working roof slab beams, column, rafts, lintels and other structural members laid in situ or pre-cast laid in position complete in all respect 1:2:4. | 400 Cft | 337.00 | P/Cft | 1,34,800.00 |
| 4. | Fabrication of mild steel reinforcement for cement concrete including cutting, bending, laying in position making joints and fastenings including cost of binding wire (also includes removal of rust from bars) using tor bars. | 10.71 Cwt | 5001.70 | P/Cwt | 53,568.00 |
| 5. | Supply of Labour on Hiring Basis for lifting the Garbage and cleaning of Nallah/Sewerage System (whenever, wherever, as much as required) and as per direction of Engineer Incharge. | 625 Nos |  | Each |  |
| Total Estimated Cost |  |  |  |  | 3,69,904 (+) Open Rate |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

*********************************

I/We hereby quoted Rates @                           %  above/below the S/R

(+) Open Rate Rs:_________________ for complete work/job.

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 13

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:        9,82,644/-		Earnest Money Rs:	          19,653/-

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | P/L R.C.C. PIPE DRAIN 12” DIA IN GALI No: 46, ALAMGIR ROAD UC-02, SHER SHAH KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Excavation for pipe line in trenches and pits in all kind of soil of murum i/c trimming and dressing side to true alignment and shape traveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guard light flags and temporary crossing for non vehicular traffic wherever required left upto 05’ and lead upto 01 chain. | 9750 Cft | 4650.00 | 1000 Cft | 45,338.00 |
| 2. | A. PROVIDING, LAYING & JOINTING (Rubber Gasket, JOINTS). Providing, laying R.C.C. pipes of A.S.T.M. C-76-62 T/C: 76-70 of class II wall B and fixing in trench i/c cutting, fitting and jointing with rubber ring i/c testing with water to specified pressure. ………...………………………12” Dia ……...…..……………………. | 1300 Rft | 412.00 | P/Rft | 5,35,600.00 |
| 3. | Providing manhole 4’ dia (inside) and 5’ clear depth complete with 21” dia ( inside ) C.I frame & 21” dia of 3” multiply 3 x 8, flat iron frame for R.C.C 1:2:4 pre-cast cover i/c the cost of M.S bars reinforcement and two handles of 5 clear depth C.C 1:2:4 cast in Situ walls with 50% screened graded bajri of 9” thick i/c benching & channel ½” thick cement plaster 1:3 on all inside walls surface complete with c.i Foot rest i/c cutting asphalt roads excavation in all kind of soil, back filling and disposal of surplus stuff etc. | 51 Nos | 9307.86 | Each | 4,74,701.00 |
| 4. | Add/Less for extra depth beyond 5’ depth, less than 5’ depth deduct at the same rate. | ( - ) 51 Rft | 1609.15 | P/Rft | ( - ) 97,214.00 |
| 5. | Refilling the excavated stuff in trenches 6” thick layer i/c watering ramming to full compaction etc complete. | 8775 Cft | 2760.00 | 1000 Cft | 24,219.00 |
| Total Estimated Cost |  |  |  |  | 9,82,644.00 |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

*********************************

I/We hereby quoted Rates @                           %  above/below the S/R

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 14

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:        9,82,644/-		Earnest Money Rs:	          19,653/-

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | P/L R.C.C. PIPE DRAIN 12” DIA MOHAMMADI COLONY UC-42, MACHER COLONY KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Excavation for pipe line in trenches and pits in all kind of soil of murum i/c trimming and dressing side to true alignment and shape traveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guard light flags and temporary crossing for non vehicular traffic wherever required left upto 05’ and lead upto 01 chain. | 9750 Cft | 4650.00 | 1000 Cft | 45,338.00 |
| 2. | A. PROVIDING, LAYING & JOINTING (Rubber Gasket, JOINTS). Providing, laying R.C.C. pipes of A.S.T.M. C-76-62 T/C: 76-70 of class II wall B and fixing in trench i/c cutting, fitting and jointing with rubber ring i/c testing with water to specified pressure. ………...………………………12” Dia ……...…..……………………. | 1300 Rft | 412.00 | P/Rft | 5,35,600.00 |
| 3. | Providing manhole 4’ dia (inside) and 5’ clear depth complete with 21” dia ( inside ) C.I frame & 21” dia of 3” multiply 3 x 8, flat iron frame for R.C.C 1:2:4 pre-cast cover i/c the cost of M.S bars reinforcement and two handles of 5 clear depth C.C 1:2:4 cast in Situ walls with 50% screened graded bajri of 9” thick i/c benching & channel ½” thick cement plaster 1:3 on all inside walls surface complete with c.i Foot rest i/c cutting asphalt roads excavation in all kind of soil, back filling and disposal of surplus stuff etc. | 51 Nos | 9307.86 | Each | 4,74,701.00 |
| 4. | Add/Less for extra depth beyond 5’ depth, less than 5’ depth deduct at the same rate. | ( - ) 51 Rft | 1609.15 | P/Rft | ( - ) 97,214.00 |
| 5. | Refilling the excavated stuff in trenches 6” thick layer i/c watering ramming to full compaction etc complete. | 8775 Cft | 2760.00 | 1000 Cft | 24,219.00 |
| Total Estimated Cost |  |  |  |  | 9,82,644.00 |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

*********************************

I/We hereby quoted Rates @                           %  above/below the S/R

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 15

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:        9,82,644/-		Earnest Money Rs:	          19,653/-

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | P/L R.C.C. PIEP DRAIN 12” DIA NAYA PARA UC-42, MACHER COLONY KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Excavation for pipe line in trenches and pits in all kind of soil of murum i/c trimming and dressing side to true alignment and shape traveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guard light flags and temporary crossing for non vehicular traffic wherever required left upto 05’ and lead upto 01 chain. | 9750 Cft | 4650.00 | 1000 Cft | 45,338.00 |
| 2. | A. PROVIDING, LAYING & JOINTING (Rubber Gasket, JOINTS). Providing, laying R.C.C. pipes of A.S.T.M. C-76-62 T/C: 76-70 of class II wall B and fixing in trench i/c cutting, fitting and jointing with rubber ring i/c testing with water to specified pressure. ………...………………………12” Dia ……...…..……………………. | 1300 Rft | 412.00 | P/Rft | 5,35,600.00 |
| 3. | Providing manhole 4’ dia (inside) and 5’ clear depth complete with 21” dia ( inside ) C.I frame & 21” dia of 3” multiply 3 x 8, flat iron frame for R.C.C 1:2:4 pre-cast cover i/c the cost of M.S bars reinforcement and two handles of 5 clear depth C.C 1:2:4 cast in Situ walls with 50% screened graded bajri of 9” thick i/c benching & channel ½” thick cement plaster 1:3 on all inside walls surface complete with c.i Foot rest i/c cutting asphalt roads excavation in all kind of soil, back filling and disposal of surplus stuff etc. | 51 Nos | 9307.86 | Each | 4,74,701.00 |
| 4. | Add/Less for extra depth beyond 5’ depth, less than 5’ depth deduct at the same rate. | ( - ) 51 Rft | 1609.15 | P/Rft | ( - ) 97,214.00 |
| 5. | Refilling the excavated stuff in trenches 6” thick layer i/c watering ramming to full compaction etc complete. | 8775 Cft | 2760.00 | 1000 Cft | 24,219.00 |
| Total Estimated Cost |  |  |  |  | 9,82,644.00 |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

*********************************

I/We hereby quoted Rates @                           %  above/below the S/R

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 16

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:          9,90,092/-		Earnest Money Rs:	         19,802/-

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | P/L R.C.C. PIPE DRAIN 12” DIA CUSTOM CLUB ROAD IN UC-43, KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Excavation for pipe line in trenches and pits in all kind of soil of murum i/c trimming and dressing side to true alignment and shape traveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guard light flags and temporary crossing for non vehicular traffic wherever required left upto 05’ and lead upto 01 chain. | 9750 Cft | 4650.00 | 1000 Cft | 45,338.00 |
| 2. | A. PROVIDING, LAYING & JOINTING (Rubber Gasket, JOINTS). Providing, laying R.C.C. pipes of A.S.T.M. C-76-62 T/C: 76-70 of class II wall B and fixing in trench i/c cutting, fitting and jointing with rubber ring i/c testing with water to specified pressure. ………...………………………12” Dia ……...…..……………………. | 1300 Rft | 412.00 | P/Rft | 5,35,600.00 |
| 3. | Providing manhole 4’ dia (inside) and 5’ clear depth complete with 21” dia ( inside ) C.I frame & 21” dia of 3” multiply 3 x 8, flat iron frame for R.C.C 1:2:4 pre-cast cover i/c the cost of M.S bars reinforcement and two handles of 5 clear depth C.C 1:2:4 cast in Situ walls with 50% screened graded bajri of 9” thick i/c benching & channel ½” thick cement plaster 1:3 on all inside walls surface complete with c.i Foot rest i/c cutting asphalt roads excavation in all kind of soil, back filling and disposal of surplus stuff etc. | 50 Nos | 9307.86 | Each | 4,65,393.00 |
| 4. | Add/Less for extra depth beyond 5’ depth, less than 5’ depth deduct at the same rate. | ( - ) 50 Rft | 1609.15 | P/Rft | ( - ) 80,458.00 |
| 5. | Refilling the excavated stuff in trenches 6” thick layer i/c watering ramming to full compaction etc complete. | 8775 Cft | 2760.00 | 1000 Cft | 24,219.00 |
| Total Estimated Cost |  |  |  |  | 9,90,092.00 |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

*********************************

I/We hereby quoted Rates @                           %  above/below the S/R

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 17

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:          9,90,092/-		Earnest Money Rs:	         19,802/-

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | P/L R.C.C. PIPE DRAIN 12” DIA BABUL ISLAM SOCIETY IN UC-43, KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Excavation for pipe line in trenches and pits in all kind of soil of murum i/c trimming and dressing side to true alignment and shape traveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guard light flags and temporary crossing for non vehicular traffic wherever required left upto 05’ and lead upto 01 chain. | 9750 Cft | 4650.00 | 1000 Cft | 45,338.00 |
| 2. | A. PROVIDING, LAYING & JOINTING (Rubber Gasket, JOINTS). Providing, laying R.C.C. pipes of A.S.T.M. C-76-62 T/C: 76-70 of class II wall B and fixing in trench i/c cutting, fitting and jointing with rubber ring i/c testing with water to specified pressure. ………...………………………12” Dia ……...…..……………………. | 1300 Rft | 412.00 | P/Rft | 5,35,600.00 |
| 3. | Providing manhole 4’ dia (inside) and 5’ clear depth complete with 21” dia ( inside ) C.I frame & 21” dia of 3” multiply 3 x 8, flat iron frame for R.C.C 1:2:4 pre-cast cover i/c the cost of M.S bars reinforcement and two handles of 5 clear depth C.C 1:2:4 cast in Situ walls with 50% screened graded bajri of 9” thick i/c benching & channel ½” thick cement plaster 1:3 on all inside walls surface complete with c.i Foot rest i/c cutting asphalt roads excavation in all kind of soil, back filling and disposal of surplus stuff etc. | 50 Nos | 9307.86 | Each | 4,65,393.00 |
| 4. | Add/Less for extra depth beyond 5’ depth, less than 5’ depth deduct at the same rate. | ( - ) 50 Rft | 1609.15 | P/Rft | ( - ) 80,458.00 |
| 5. | Refilling the excavated stuff in trenches 6” thick layer i/c watering ramming to full compaction etc complete. | 8775 Cft | 2760.00 | 1000 Cft | 24,219.00 |
| Total Estimated Cost |  |  |  |  | 9,90,092.00 |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

*********************************

I/We hereby quoted Rates @                           %  above/below the S/R

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 18

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:        9,82,644/-		Earnest Money Rs:	          19,653/-

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | P/L R.C.C. PIPE DRAIN 12” DIA DOCKS COLONY IN UC-43, KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Excavation for pipe line in trenches and pits in all kind of soil of murum i/c trimming and dressing side to true alignment and shape traveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guard light flags and temporary crossing for non vehicular traffic wherever required left upto 05’ and lead upto 01 chain. | 9750 Cft | 4650.00 | 1000 Cft | 45,338.00 |
| 2. | A. PROVIDING, LAYING & JOINTING (Rubber Gasket, JOINTS). Providing, laying R.C.C. pipes of A.S.T.M. C-76-62 T/C: 76-70 of class II wall B and fixing in trench i/c cutting, fitting and jointing with rubber ring i/c testing with water to specified pressure. ………...………………………12” Dia ……...…..……………………. | 1300 Rft | 412.00 | P/Rft | 5,35,600.00 |
| 3. | Providing manhole 4’ dia (inside) and 5’ clear depth complete with 21” dia ( inside ) C.I frame & 21” dia of 3” multiply 3 x 8, flat iron frame for R.C.C 1:2:4 pre-cast cover i/c the cost of M.S bars reinforcement and two handles of 5 clear depth C.C 1:2:4 cast in Situ walls with 50% screened graded bajri of 9” thick i/c benching & channel ½” thick cement plaster 1:3 on all inside walls surface complete with c.i Foot rest i/c cutting asphalt roads excavation in all kind of soil, back filling and disposal of surplus stuff etc. | 51 Nos | 9307.86 | Each | 4,74,701.00 |
| 4. | Add/Less for extra depth beyond 5’ depth, less than 5’ depth deduct at the same rate. | ( - ) 51 Rft | 1609.15 | P/Rft | ( - ) 97,214.00 |
| 5. | Refilling the excavated stuff in trenches 6” thick layer i/c watering ramming to full compaction etc complete. | 8775 Cft | 2760.00 | 1000 Cft | 24,219.00 |
| Total Estimated Cost |  |  |  |  | 9,82,644.00 |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

*********************************

I/We hereby quoted Rates @                           %  above/below the S/R

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 19

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:        9,82,644/-		Earnest Money Rs:	          19,653/-

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | P/L R.C.C. PIPE DRAIN 12” DIA MAJEED COLONY IN UC-43, KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Excavation for pipe line in trenches and pits in all kind of soil of murum i/c trimming and dressing side to true alignment and shape traveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guard light flags and temporary crossing for non vehicular traffic wherever required left upto 05’ and lead upto 01 chain. | 9750 Cft | 4650.00 | 1000 Cft | 45,338.00 |
| 2. | A. PROVIDING, LAYING & JOINTING (Rubber Gasket, JOINTS). Providing, laying R.C.C. pipes of A.S.T.M. C-76-62 T/C: 76-70 of class II wall B and fixing in trench i/c cutting, fitting and jointing with rubber ring i/c testing with water to specified pressure. ………...………………………12” Dia ……...…..……………………. | 1300 Rft | 412.00 | P/Rft | 5,35,600.00 |
| 3. | Providing manhole 4’ dia (inside) and 5’ clear depth complete with 21” dia ( inside ) C.I frame & 21” dia of 3” multiply 3 x 8, flat iron frame for R.C.C 1:2:4 pre-cast cover i/c the cost of M.S bars reinforcement and two handles of 5 clear depth C.C 1:2:4 cast in Situ walls with 50% screened graded bajri of 9” thick i/c benching & channel ½” thick cement plaster 1:3 on all inside walls surface complete with c.i Foot rest i/c cutting asphalt roads excavation in all kind of soil, back filling and disposal of surplus stuff etc. | 51 Nos | 9307.86 | Each | 4,74,701.00 |
| 4. | Add/Less for extra depth beyond 5’ depth, less than 5’ depth deduct at the same rate. | ( - ) 51 Rft | 1609.15 | P/Rft | ( - ) 97,214.00 |
| 5. | Refilling the excavated stuff in trenches 6” thick layer i/c watering ramming to full compaction etc complete. | 8775 Cft | 2760.00 | 1000 Cft | 24,219.00 |
| Total Estimated Cost |  |  |  |  | 9,82,644.00 |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

*********************************

I/We hereby quoted Rates @                           %  above/below the S/R

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 20

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:   7,83,962 (+) O/R		Earnest Money Rs:	15,679 (+) 2% of O/R

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | P/L R.C.C. PIPE DRAIN 15” DIA AT AKBER ROAD UC-02, SHER SHAH KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Dismantling cement concrete plain 1:2:4. | 900 Cft | 3327.50 | %Cft | 29,948.00 |
| 2. | Excavation for pipe line in trenches and pits in all kind of soil of murum i/c trimming and dressing side to true alignment and shape traveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guard light flags and temporary crossing for non vehicular traffic wherever required left upto 05’ and lead upto 01 chain. | 7200 Cft | 4650.00 | 1000 Cft | 33,480.00 |
| 3. | A. PROVIDING, LAYING & JOINTING (Rubber Gasket, JOINTS). Providing, laying R.C.C. pipes of A.S.T.M. C-76-62 T/C: 76-70 of class II wall B and fixing in trench i/c cutting, fitting and jointing with rubber ring i/c testing with water to specified pressure. ………...………………………12” Dia ……...…..……………………. | 600 Rft | 412.00 | P/Rft | 3,09,000.00 |
| 4. | Providing manhole 4’ dia (inside) and 5’ clear depth complete with 21” dia ( inside ) C.I frame & 21” dia of 3” multiply 3 x 8, flat iron frame for R.C.C 1:2:4 pre-cast cover i/c the cost of M.S bars reinforcement and two handles of 5 clear depth C.C 1:2:4 cast in Situ walls with 50% screened graded bajri of 9” thick i/c benching & channel ½” thick cement plaster 1:3 on all inside walls surface complete with c.i Foot rest i/c cutting asphalt roads excavation in all kind of soil, back filling and disposal of surplus stuff etc. | 30 Nos | 9307.86 | Each | 2,79,236.00 |
| 5. | Add/Less for extra depth beyond 5’ depth, less than 5’ depth deduct at the same rate. | ( - ) 30 Rft | 1609.15 | P/Rft | ( - ) 48,275.00 |
| 6. | Refilling the excavated stuff in trenches 6” thick layer i/c watering ramming to full compaction etc complete. | 6480 Cft | 2760.00 | 1000 Cft | 17,885.00 |
| 7. | Cartage of 100 Cft / 05 Tons of all materials like stone aggregate, spawl, coal, lime, surkhi etc B.G. rail fastening point and crossing bridge girders, pipes sheet rails M.S. bar etc or 1000 number bricks 10’ x 5’ x 3’ or 1000 manuds of fuel wood by trucks or any other means owned by the contractors (lead upto 15 miles etc). | 900 Cft | 1065.04 | %Cft | 9,585.00 |
| 8. | Manufacturing and supplying of R.C.C. ring slab of 21” Dia inside 36”Dia outside 7.5” width and 6” thick i/c 3/8” Dia for steel bars two concentric ring with 3/8” Dia 08 Nos. cross linked bars welded and two sunk type hooks casted etc. | 25 Nos | 1906.60 | Each | 47,665.00 |
| 9. | Shifting and fixing R.C.C. pre-cast ring slab in c.c. 1:2:4 etc. | 25 Nos | 563.00 | Each | 14,075.00 |

Cont’d Page No: 02

Page No:02

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 10. | Manufacturing and supplying R.C.C. manhole covers casting c.c. 1:2:4 ratio 3 inch deep at center bar, reinforced with 3/8” dia tour steel 2.50” deep M.S. plate other curing, stacking and transportation within 10 miles. ………...………………………12” Dia ……...…..……………………. | 100 Nos | 913.63 | Each | 91,363.00 |
| 11. | Supply of Labour on Hiring Basis for lifting the Garbage (whenever, wherever, as much as required) and as per direction of Engineer Incharge. | 235 Nos |  | Each |  |
| Total Estimated Cost |  |  |  |  | 7,83,962 (+) Open Rate |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

*********************************

I/We hereby quoted Rates @                           %  above/below the S/R

(+) Open Rate Rs:_________________ for complete work/job.

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 21

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:   4,22,454 (+) O/R		Earnest Money Rs:	8,449 (+) 2% of O/R

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | IMPROVEMENT OF NALLAH MUNDRA MOHALLAH IN UC-42, MACHER COLONY KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Dismantling cement concrete R.C.C. (1:2:4) disposal lead upto 10 miles etc. | 500 Cft | 5445.00 | %Cft | 27,225.00 |
| 2. | Cartage of 100 Cft / 05 Tons of all materials like stone aggregate, spawl, coal, lime, surkhi etc B.G. rail fastening point and crossing bridge girders, pipes sheet rails M.S. bar etc or 1000 number bricks 10”x5”x3” or 1000 manuds of fuel wood by trucks or any other means owned by the contractors (lead upto 15 miles etc). | 15000 Cft | 1065.04 | %Cft | 1,59,756.00 |
| 3. | R.C.C. working roof slab beams, column, rafts, lintels and other structural members laid in situ or pre-cast laid in position complete in all respect 1:2:4. | 500 Cft | 337.00 | P/Cft | 1,68,500.00 |
| 4. | Fabrication of mild steel reinforcement for cement concrete including cutting, bending, laying in position making joints and fastenings including cost of binding wire (also includes removal of rust from bars) using tor bars. | 13.39 Cwt | 5001.70 | P/Cwt | 66,973.00 |
| 5. | Supply of Labour on Hiring Basis for lifting the Garbage and cleaning of Nallah/Sewerage System (whenever, wherever, as much as required) and as per direction of Engineer Incharge. | 635 Nos |  | Each |  |
| Total Estimated Cost |  |  |  |  | 4,22,454 (+) Open Rate |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

*********************************

I/We hereby quoted Rates @                           %  above/below the S/R

(+) Open Rate Rs:_________________ for complete work/job.

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 22

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:       OPEN RATE		Earnest Money Rs:	        2% of O/R

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | CLEANING OF NALLAH BY LABOUR AND HIRING OF TRACTOR FOR BABA AND BHIT ISLAND UC-41, KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Hiring of Tractor Blade for leveling, dressing and cleaning of road and street i/c the wage of Driver, Assistant and Fuel etc complete. | 80 Hours |  | P/Hour |  |
| 2. | Supply of Labour on Hiring Basis for lifting the Garbage (whenever, wherever, as much as required) and as per direction of Engineer Incharge. | 570 Nos |  | Each |  |
| Total Estimated Cost |  |  |  |  | Open Rate |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

*********************************

I/We hereby quoted Rates Rs:                                   for complete work/job.

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 23

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:   4,22,454 (+) O/R		Earnest Money Rs:	8,449 (+) 2% of O/R

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | IMPROVEMENT AND CLEANNG/DESILTING OF NALLAH LODIA GALI BHIT ISLAND, UC-41, KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Dismantling cement concrete R.C.C. (1:2:4) disposal lead upto 10 miles etc. | 500 Cft | 5445.00 | %Cft | 27,225.00 |
| 2. | Cartage of 100 Cft / 05 Tons of all materials like stone aggregate, spawl, coal, lime, surkhi etc B.G. rail fastening point and crossing bridge girders, pipes sheet rails M.S. bar etc or 1000 number bricks 10”x5”x3” or 1000 manuds of fuel wood by trucks or any other means owned by the contractors (lead upto 15 miles etc). | 15000 Cft | 1065.04 | %Cft | 1,59,756.00 |
| 3. | R.C.C. working roof slab beams, column, rafts, lintels and other structural members laid in situ or pre-cast laid in position complete in all respect 1:2:4. | 500 Cft | 337.00 | P/Cft | 1,68,500.00 |
| 4. | Fabrication of mild steel reinforcement for cement concrete including cutting, bending, laying in position making joints and fastenings including cost of binding wire (also includes removal of rust from bars) using tor bars. | 13.39 Cwt | 5001.70 | P/Cwt | 66,973.00 |
| 5. | Supply of Labour on Hiring Basis for lifting the Garbage and cleaning of Nallah/Sewerage System (whenever, wherever, as much as required) and as per direction of Engineer Incharge. | 635 Nos |  | Each |  |
| Total Estimated Cost |  |  |  |  | 4,22,454 (+) Open Rate |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

*********************************

I/We hereby quoted Rates @                           %  above/below the S/R

(+) Open Rate Rs:_________________ for complete work/job.

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 24

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:   8,20,587 (+) O/R		Earnest Money Rs:	16,412 (+) 2% of O/R

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | P/L R.C.C. PIPE DRAIN 12” DIA AMEER MAVIA MASJID UC-46, GULSHAN-E-SIKENDERABAD KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Dismantling cement concrete plain 1:2:4. | 938 Cft | 3327.50 | %Cft | 31,212.00 |
| 2. | Excavation for pipe line in trenches and pits in all kind of soil of murum i/c trimming and dressing side to true alignment and shape traveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guard light flags and temporary crossing for non vehicular traffic wherever required left upto 05’ and lead upto 01 chain. | 5625 Cft | 4650.00 | 1000 Cft | 26,156.00 |
| 3. | A. PROVIDING, LAYING & JOINTING (Rubber Gasket, JOINTS). Providing, laying R.C.C. pipes of A.S.T.M. C-76-62 T/C: 76-70 of class II wall B and fixing in trench i/c cutting, fitting and jointing with rubber ring i/c testing with water to specified pressure. ………...………………………12” Dia ……...…..……………………. | 750 Rft | 412.00 | P/Rft | 3,09,000.00 |
| 4. | Providing manhole 4’ dia (inside) and 5’ clear depth complete with 21” dia ( inside ) C.I frame & 21” dia of 3” multiply 3 x 8, flat iron frame for R.C.C 1:2:4 pre-cast cover i/c the cost of M.S bars reinforcement and two handles of 5 clear depth C.C 1:2:4 cast in Situ walls with 50% screened graded bajri of 9” thick i/c benching & channel ½” thick cement plaster 1:3 on all inside walls surface complete with c.i Foot rest i/c cutting asphalt roads excavation in all kind of soil, back filling and disposal of surplus stuff etc. | 36 Nos | 9307.86 | Each | 3,35,083.00 |
| 5. | Add/Less for extra depth beyond 5’ depth, less than 5’ depth deduct at the same rate. | ( - ) 36 Rft | 1609.15 | P/Rft | ( - ) 57,929.00 |
| 6. | Refilling the excavated stuff in trenches 6” thick layer i/c watering ramming to full compaction etc complete. | 5063 Cft | 2760.00 | 1000 Cft | 13,974.00 |
| 7. | Cartage of 100 Cft / 05 Tons of all materials like stone aggregate, spawl, coal, lime, surkhi etc B.G. rail fastening point and crossing bridge girders, pipes sheet rails M.S. bar etc or 1000 number bricks 10’ x 5’ x 3’ or 1000 manuds of fuel wood by trucks or any other means owned by the contractors (lead upto 15 miles etc). | 938 Cft | 1065.04 | %Cft | 9,990.00 |
| 8. | Manufacturing and supplying of R.C.C. ring slab of 21” Dia inside 36”Dia outside 7.5” width and 6” thick i/c 3/8” Dia for steel bars two concentric ring with 3/8” Dia 08 Nos. cross linked bars welded and two sunk type hooks casted etc. | 25 Nos | 1906.60 | Each | 47,663.00 |
| 9. | Shifting and fixing R.C.C. pre-cast ring slab in c.c. 1:2:4 etc. | 25 Nos | 563.00 | Each | 14,075.00 |

Cont’d Page No: 02

Page No:02

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 10. | Manufacturing and supplying R.C.C. manhole covers casting c.c. 1:2:4 ratio 3 inch deep at center bar, reinforced with 3/8” dia tour steel 2.50” deep M.S. plate other curing, stacking and transportation within 10 miles. ………...………………………12” Dia ……...…..……………………. | 100 Nos | 913.63 | Each | 91,363.00 |
| 11. | Supply of Labour on Hiring Basis for lifting the Garbage (whenever, wherever, as much as required) and as per direction of Engineer Incharge. | 190 Nos |  | Each |  |
| Total Estimated Cost |  |  |  |  | 8,20,587 (+) Open Rate |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

*********************************

I/We hereby quoted Rates @                           %  above/below the S/R

(+) Open Rate Rs:_________________ for complete work/job.

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 25

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:   8,20,587 (+) O/R		Earnest Money Rs:	16,412 (+) 2% of O/R

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | P/L R.C.C. PIPE DRAIN 12” DIA MUBARAK MASJID UC-46, GULSHAN-E-SIKENDERABAD KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Dismantling cement concrete plain 1:2:4. | 938 Cft | 3327.50 | %Cft | 31,212.00 |
| 2. | Excavation for pipe line in trenches and pits in all kind of soil of murum i/c trimming and dressing side to true alignment and shape traveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guard light flags and temporary crossing for non vehicular traffic wherever required left upto 05’ and lead upto 01 chain. | 5625 Cft | 4650.00 | 1000 Cft | 26,156.00 |
| 3. | A. PROVIDING, LAYING & JOINTING (Rubber Gasket, JOINTS). Providing, laying R.C.C. pipes of A.S.T.M. C-76-62 T/C: 76-70 of class II wall B and fixing in trench i/c cutting, fitting and jointing with rubber ring i/c testing with water to specified pressure. ………...………………………12” Dia ……...…..……………………. | 750 Rft | 412.00 | P/Rft | 3,09,000.00 |
| 4. | Providing manhole 4’ dia (inside) and 5’ clear depth complete with 21” dia ( inside ) C.I frame & 21” dia of 3” multiply 3 x 8, flat iron frame for R.C.C 1:2:4 pre-cast cover i/c the cost of M.S bars reinforcement and two handles of 5 clear depth C.C 1:2:4 cast in Situ walls with 50% screened graded bajri of 9” thick i/c benching & channel ½” thick cement plaster 1:3 on all inside walls surface complete with c.i Foot rest i/c cutting asphalt roads excavation in all kind of soil, back filling and disposal of surplus stuff etc. | 36 Nos | 9307.86 | Each | 3,35,083.00 |
| 5. | Add/Less for extra depth beyond 5’ depth, less than 5’ depth deduct at the same rate. | ( - ) 36 Rft | 1609.15 | P/Rft | ( - ) 57,929.00 |
| 6. | Refilling the excavated stuff in trenches 6” thick layer i/c watering ramming to full compaction etc complete. | 5063 Cft | 2760.00 | 1000 Cft | 13,974.00 |
| 7. | Cartage of 100 Cft / 05 Tons of all materials like stone aggregate, spawl, coal, lime, surkhi etc B.G. rail fastening point and crossing bridge girders, pipes sheet rails M.S. bar etc or 1000 number bricks 10’ x 5’ x 3’ or 1000 manuds of fuel wood by trucks or any other means owned by the contractors (lead upto 15 miles etc). | 938 Cft | 1065.04 | %Cft | 9,990.00 |
| 8. | Manufacturing and supplying of R.C.C. ring slab of 21” Dia inside 36”Dia outside 7.5” width and 6” thick i/c 3/8” Dia for steel bars two concentric ring with 3/8” Dia 08 Nos. cross linked bars welded and two sunk type hooks casted etc. | 25 Nos | 1906.60 | Each | 47,663.00 |
| 9. | Shifting and fixing R.C.C. pre-cast ring slab in c.c. 1:2:4 etc. | 25 Nos | 563.00 | Each | 14,075.00 |

Cont’d Page No: 02

Page No:02

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 10. | Manufacturing and supplying R.C.C. manhole covers casting c.c. 1:2:4 ratio 3 inch deep at center bar, reinforced with 3/8” dia tour steel 2.50” deep M.S. plate other curing, stacking and transportation within 10 miles. ………...………………………12” Dia ……...…..……………………. | 100 Nos | 913.63 | Each | 91,363.00 |
| 11. | Supply of Labour on Hiring Basis for lifting the Garbage (whenever, wherever, as much as required) and as per direction of Engineer Incharge. | 190 Nos |  | Each |  |
| Total Estimated Cost |  |  |  |  | 8,20,587 (+) Open Rate |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

*********************************

I/We hereby quoted Rates @                           %  above/below the S/R

(+) Open Rate Rs:_________________ for complete work/job.

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 26

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:   8,20,587 (+) O/R		Earnest Money Rs:	16,412 (+) 2% of O/R

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | P/L R.C.C. PIPE DRAIN 12” DIA AMMA MASJID GALI UC-46, GULSHAN-E-SIKENDERABAD KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Dismantling cement concrete plain 1:2:4. | 938 Cft | 3327.50 | %Cft | 31,212.00 |
| 2. | Excavation for pipe line in trenches and pits in all kind of soil of murum i/c trimming and dressing side to true alignment and shape traveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guard light flags and temporary crossing for non vehicular traffic wherever required left upto 05’ and lead upto 01 chain. | 5625 Cft | 4650.00 | 1000 Cft | 26,156.00 |
| 3. | A. PROVIDING, LAYING & JOINTING (Rubber Gasket, JOINTS). Providing, laying R.C.C. pipes of A.S.T.M. C-76-62 T/C: 76-70 of class II wall B and fixing in trench i/c cutting, fitting and jointing with rubber ring i/c testing with water to specified pressure. ………...………………………12” Dia ……...…..……………………. | 750 Rft | 412.00 | P/Rft | 3,09,000.00 |
| 4. | Providing manhole 4’ dia (inside) and 5’ clear depth complete with 21” dia ( inside ) C.I frame & 21” dia of 3” multiply 3 x 8, flat iron frame for R.C.C 1:2:4 pre-cast cover i/c the cost of M.S bars reinforcement and two handles of 5 clear depth C.C 1:2:4 cast in Situ walls with 50% screened graded bajri of 9” thick i/c benching & channel ½” thick cement plaster 1:3 on all inside walls surface complete with c.i Foot rest i/c cutting asphalt roads excavation in all kind of soil, back filling and disposal of surplus stuff etc. | 36 Nos | 9307.86 | Each | 3,35,083.00 |
| 5. | Add/Less for extra depth beyond 5’ depth, less than 5’ depth deduct at the same rate. | ( - ) 36 Rft | 1609.15 | P/Rft | ( - ) 57,929.00 |
| 6. | Refilling the excavated stuff in trenches 6” thick layer i/c watering ramming to full compaction etc complete. | 5063 Cft | 2760.00 | 1000 Cft | 13,974.00 |
| 7. | Cartage of 100 Cft / 05 Tons of all materials like stone aggregate, spawl, coal, lime, surkhi etc B.G. rail fastening point and crossing bridge girders, pipes sheet rails M.S. bar etc or 1000 number bricks 10’ x 5’ x 3’ or 1000 manuds of fuel wood by trucks or any other means owned by the contractors (lead upto 15 miles etc). | 938 Cft | 1065.04 | %Cft | 9,990.00 |
| 8. | Manufacturing and supplying of R.C.C. ring slab of 21” Dia inside 36”Dia outside 7.5” width and 6” thick i/c 3/8” Dia for steel bars two concentric ring with 3/8” Dia 08 Nos. cross linked bars welded and two sunk type hooks casted etc. | 25 Nos | 1906.60 | Each | 47,663.00 |
| 9. | Shifting and fixing R.C.C. pre-cast ring slab in c.c. 1:2:4 etc. | 25 Nos | 563.00 | Each | 14,075.00 |

Cont’d Page No: 02

Page No:02

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 10. | Manufacturing and supplying R.C.C. manhole covers casting c.c. 1:2:4 ratio 3 inch deep at center bar, reinforced with 3/8” dia tour steel 2.50” deep M.S. plate other curing, stacking and transportation within 10 miles. ………...………………………12” Dia ……...…..……………………. | 100 Nos | 913.63 | Each | 91,363.00 |
| 11. | Supply of Labour on Hiring Basis for lifting the Garbage (whenever, wherever, as much as required) and as per direction of Engineer Incharge. | 190 Nos |  | Each |  |
| Total Estimated Cost |  |  |  |  | 8,20,587 (+) Open Rate |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

*********************************

I/We hereby quoted Rates @                           %  above/below the S/R

(+) Open Rate Rs:_________________ for complete work/job.

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 27

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:   8,20,587 (+) O/R		Earnest Money Rs:	16,412 (+) 2% of O/R

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | P/L R.C.C. PIPE DRAIN MADINA MASJID GULSHAN-E-SIKENDERABAD UC-46, KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Dismantling cement concrete plain 1:2:4. | 938 Cft | 3327.50 | %Cft | 31,212.00 |
| 2. | Excavation for pipe line in trenches and pits in all kind of soil of murum i/c trimming and dressing side to true alignment and shape traveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guard light flags and temporary crossing for non vehicular traffic wherever required left upto 05’ and lead upto 01 chain. | 5625 Cft | 4650.00 | 1000 Cft | 26,156.00 |
| 3. | A. PROVIDING, LAYING & JOINTING (Rubber Gasket, JOINTS). Providing, laying R.C.C. pipes of A.S.T.M. C-76-62 T/C: 76-70 of class II wall B and fixing in trench i/c cutting, fitting and jointing with rubber ring i/c testing with water to specified pressure. ………...………………………12” Dia ……...…..……………………. | 750 Rft | 412.00 | P/Rft | 3,09,000.00 |
| 4. | Providing manhole 4’ dia (inside) and 5’ clear depth complete with 21” dia ( inside ) C.I frame & 21” dia of 3” multiply 3 x 8, flat iron frame for R.C.C 1:2:4 pre-cast cover i/c the cost of M.S bars reinforcement and two handles of 5 clear depth C.C 1:2:4 cast in Situ walls with 50% screened graded bajri of 9” thick i/c benching & channel ½” thick cement plaster 1:3 on all inside walls surface complete with c.i Foot rest i/c cutting asphalt roads excavation in all kind of soil, back filling and disposal of surplus stuff etc. | 36 Nos | 9307.86 | Each | 3,35,083.00 |
| 5. | Add/Less for extra depth beyond 5’ depth, less than 5’ depth deduct at the same rate. | ( - ) 36 Rft | 1609.15 | P/Rft | ( - ) 57,929.00 |
| 6. | Refilling the excavated stuff in trenches 6” thick layer i/c watering ramming to full compaction etc complete. | 5063 Cft | 2760.00 | 1000 Cft | 13,974.00 |
| 7. | Cartage of 100 Cft / 05 Tons of all materials like stone aggregate, spawl, coal, lime, surkhi etc B.G. rail fastening point and crossing bridge girders, pipes sheet rails M.S. bar etc or 1000 number bricks 10’ x 5’ x 3’ or 1000 manuds of fuel wood by trucks or any other means owned by the contractors (lead upto 15 miles etc). | 938 Cft | 1065.04 | %Cft | 9,990.00 |
| 8. | Manufacturing and supplying of R.C.C. ring slab of 21” Dia inside 36”Dia outside 7.5” width and 6” thick i/c 3/8” Dia for steel bars two concentric ring with 3/8” Dia 08 Nos. cross linked bars welded and two sunk type hooks casted etc. | 25 Nos | 1906.60 | Each | 47,663.00 |
| 9. | Shifting and fixing R.C.C. pre-cast ring slab in c.c. 1:2:4 etc. | 25 Nos | 563.00 | Each | 14,075.00 |

Cont’d Page No: 02

Page No:02

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 10. | Manufacturing and supplying R.C.C. manhole covers casting c.c. 1:2:4 ratio 3 inch deep at center bar, reinforced with 3/8” dia tour steel 2.50” deep M.S. plate other curing, stacking and transportation within 10 miles. ………...………………………12” Dia ……...…..……………………. | 100 Nos | 913.63 | Each | 91,363.00 |
| 11. | Supply of Labour on Hiring Basis for lifting the Garbage (whenever, wherever, as much as required) and as per direction of Engineer Incharge. | 190 Nos |  | Each |  |
| Total Estimated Cost |  |  |  |  | 8,20,587 (+) Open Rate |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

*********************************

I/We hereby quoted Rates @                           %  above/below the S/R

(+) Open Rate Rs:_________________ for complete work/job.

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 28

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:       9,97,813/-		Earnest Money Rs:	           19,956/-

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | CLEANING/DESILTING/REPAIR OF NALLAH/R.C.C. RING SLAB AND MANHOLE COVERS AT ALAMGIR ROAD UC-01, SHER SHAH KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Dismantling cement concrete R.C.C. (1:2:4) disposal lead upto 10 miles etc. | 860 Cft | 5445.00 | %Cft | 46,827.00 |
| 2. | Excavation for wet silt clearance etc complete. | 26000 Cft | 5242.50 | 1000 Cft | 1,36,305.00 |
| 3. | Cartage of 100 Cft / 05 Tons of all materials like stone aggregate, spawl, coal, lime, surkhi etc B.G. rail fastening point and crossing bridge girders, pipes sheet rails M.S. bar etc or 1000 number bricks 10”x5”x3” or 1000 manuds of fuel wood by trucks or any other means owned by the contractors (lead upto 15 miles etc). | 20145 Cft | 1065.04 | %Cft | 2,14,552.00 |
| 4. | R.C.C. working roof slab beams, column, rafts, lintels and other structural members laid in situ or pre-cast laid in position complete in all respect 1:2:4. | 860 Cft | 337.00 | P/Cft | 2,89,820.00 |
| 5. | Fabrication of mild steel reinforcement for cement concrete including cutting, bending, laying in position making joints and fastenings including cost of binding wire (also includes removal of rust from bars) using tor bars. | 23.03 Cwt | 5001.70 | P/Cwt | 1,15,189.00 |
| 6. | Cleaning/desilting of manhole inspection chamber other cleaning and roding of connected sewer average 50’ per manhole removing sewerage and soil water throwing out side. | 50 Nos | 474.83 | Each | 23,742.00 |
| 7. | Manufacturing and supplying of R.C.C. ring slab of 21” Dia inside 36”Dia outside 7.5” width and 6” thick i/c 3/8” Dia for steel bars two concentric ring with 3/8” Dia 08 Nos. cross linked bars welded and two sunk type hooks casted etc. | 25 Nos | 1906.60 | Each | 47,665.00 |
| 8. | Shifting and fixing R.C.C. pre-cast ring slab in c.c. 1:2:4 etc. | 25 Nos | 563.00 | Each | 14,077.00 |
| 9. | Manufacturing and supplying R.C.C. manhole covers casting c.c. 1:2:4 ratio 3 inch deep at center bar, reinforced with 3/8” dia tour steel 2.50” deep M.S. plate other curing, stacking and transportation within 10 miles. ………...………………………12” Dia ……...…..……………………. | 120 Nos | 913.63 | Each | 1,09,635.00 |
| Total Estimated Cost |  |  |  |  | 9,97,813.00 |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

*********************************

I/We hereby quoted Rates @                           %  above/below the S/R

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 29

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:   3,80,434 (+) O/R		Earnest Money Rs:	7,609 (+) 2% of O/R

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | IMPROVEMENT OF NALLAH KAUSAR PUBLIC SCHOOL TO AL-QADOOS MASJID IN UC-45, KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Dismantling cement concrete R.C.C. (1:2:4) disposal lead upto 10 miles etc. | 420 Cft | 5445.00 | %Cft | 22,869.00 |
| 2. | Cartage of 100 Cft / 05 Tons of all materials like stone aggregate, spawl, coal, lime, surkhi etc B.G. rail fastening point and crossing bridge girders, pipes sheet rails M.S. bar etc or 1000 number bricks 10”x5”x3” or 1000 manuds of fuel wood by trucks or any other means owned by the contractors (lead upto 15 miles etc). | 15000 Cft | 1065.04 | %Cft | 1,59,756.00 |
| 3. | R.C.C. working roof slab beams, column, rafts, lintels and other structural members laid in situ or pre-cast laid in position complete in all respect 1:2:4. | 420 Cft | 337.00 | P/Cft | 1,41,540.00 |
| 4. | Fabrication of mild steel reinforcement for cement concrete including cutting, bending, laying in position making joints and fastenings including cost of binding wire (also includes removal of rust from bars) using tor bars. | 11.25 Cwt | 5001.70 | P/Cwt | 56,269.00 |
| 5. | Supply of Labour on Hiring Basis for lifting the Garbage and cleaning of Nallah/Sewerage System (whenever, wherever, as much as required) and as per direction of Engineer Incharge. | 615 Nos |  | Each |  |
| Total Estimated Cost |  |  |  |  | 3,80,434 (+) Open Rate |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

*********************************

I/We hereby quoted Rates @                           %  above/below the S/R

(+) Open Rate Rs:_________________ for complete work/job.

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 30

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:   3,75,170 (+) O/R		Earnest Money Rs:	7,503 (+) 2% of O/R

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | IMPROVEMENT OF NALLAH DERA HAJI BARKAT TO AL-ANAS SCHOOL IN UC-45, KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Dismantling cement concrete R.C.C. (1:2:4) disposal lead upto 10 miles etc. | 410 Cft | 5445.00 | %Cft | 22,325.00 |
| 2. | Cartage of 100 Cft / 05 Tons of all materials like stone aggregate, spawl, coal, lime, surkhi etc B.G. rail fastening point and crossing bridge girders, pipes sheet rails M.S. bar etc or 1000 number bricks 10”x5”x3” or 1000 manuds of fuel wood by trucks or any other means owned by the contractors (lead upto 15 miles etc). | 15000 Cft | 1065.04 | %Cft | 1,59,756.00 |
| 3. | R.C.C. working roof slab beams, column, rafts, lintels and other structural members laid in situ or pre-cast laid in position complete in all respect 1:2:4. | 410 Cft | 337.00 | P/Cft | 1,38,170.00 |
| 4. | Fabrication of mild steel reinforcement for cement concrete including cutting, bending, laying in position making joints and fastenings including cost of binding wire (also includes removal of rust from bars) using tor bars. | 10.98 Cwt | 5001.70 | P/Cwt | 54,919.00 |
| 5. | Supply of Labour on Hiring Basis for lifting the Garbage and cleaning of Nallah/Sewerage System (whenever, wherever, as much as required) and as per direction of Engineer Incharge. | 620 Nos |  | Each |  |
| Total Estimated Cost |  |  |  |  | 3,75,170 (+) Open Rate |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

*********************************

I/We hereby quoted Rates @                           %  above/below the S/R

(+) Open Rate Rs:_________________ for complete work/job.

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 31

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:   4,60,664 (+) O/R		Earnest Money Rs:	9,213 (+) 2% of O/R

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | IMPROVEMENT OF NALLAH TOUHEED MASJID UMER KHAN ROAD UC-44, KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Dismantling cement concrete R.C.C. (1:2:4) disposal lead upto 10 miles etc. | 400 Cft | 5445.00 | %Cft | 21,780.00 |
| 2. | Cartage of 100 Cft / 05 Tons of all materials like stone aggregate, spawl, coal, lime, surkhi etc B.G. rail fastening point and crossing bridge girders, pipes sheet rails M.S. bar etc or 1000 number bricks 10”x5”x3” or 1000 manuds of fuel wood by trucks or any other means owned by the contractors (lead upto 15 miles etc). | 12750 Cft | 1065.04 | %Cft | 1,35,793.00 |
| 3. | R.C.C. working roof slab beams, column, rafts, lintels and other structural members laid in situ or pre-cast laid in position complete in all respect 1:2:4. | 400 Cft | 337.00 | P/Cft | 1,34,800.00 |
| 4. | Fabrication of mild steel reinforcement for cement concrete including cutting, bending, laying in position making joints and fastenings including cost of binding wire (also includes removal of rust from bars) using tor bars. | 10.71 Cwt | 5001.70 | P/Cwt | 53,568.00 |
| 5. | Erection and removal of centering for R.C.C. or plain cement concrete works of Deodar Wood (2nd class). | 900 Sft | 3127.41 | %Sft | 28,147.00 |
| 6. | Cement concrete plain including placing compaction finishing and curing complete including screening and washing at stone aggregate without ratio 1:2:4. | 600 Cft | 14429.25 | %Cft | 86,576.00 |
| 7. | Supply of Labour on Hiring Basis for lifting the Garbage and cleaning of Nallah/Sewerage System (whenever, wherever, as much as required) and as per direction of Engineer Incharge. | 580 Nos |  | Each |  |
| Total Estimated Cost |  |  |  |  | 4,60,664 (+) Open Rate |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

*********************************

I/We hereby quoted Rates @                           %  above/below the S/R

(+) Open Rate Rs:_________________ for complete work/job.

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address: