BILL OF QUANTITIES

NAME OF WORK:-	Drainage Scheme for  Village Bajara  (Taluka Sehwan)  District Jamshoro.

(A) 	Description and rate of item based on Composite schedule of Rates

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
|  |  | Group ‘A’ Constt: Open surface drains Type I,II,III & Paving Block |  |  |  |
| 01 | 3630.0cft | Dismantling and removing road (G.S.INO;5,P/14) | 605/- | P%cft | 21,962/- |
| 02 |  | Dismantling Cement concrete plain (G.S.I.NO; ) |  |  |  |
|  | 375.0 Cft | Ratio (1:2:4). | 3327/50 | P%cft | 12,478/- |
| 03 | 100.80 Cft | Dismantling Cement Concrete reinforcement (G.S.I.NO; ) | 5445/- | P%cft | 5,489/- |
| 04 | 1500.0 Cft | Dismantling Brick work in lime (G.S.I.NO; ) | 1285/63 | P%cft | 19,284/- |
| 05 | 51636.0 Cft | Excavation in foundation of building bridges and other structures including dabbling dressing refilling around structures with excavated earth watering and ramming lead upto 5’ft (in sand soil ) (G.S.I.NO;18,. P/4) | 3176/25 | P%0cft | 164,009/- |
| 06 | 10781.00 Cft | Cement concrete plain i/c placing compacting finishing and curring complete(i/c screening and washing of stone aggregate without shuttering (Ratio 1:4:8) (G.S.I.NO;5(c)p/18) | 11288/75 | P%cft | 12,17,040/- |
| 07 | 28800.00 Cft | Pacca brick work in foundation and plinth in cement sand mortar (Ratio 1:6) (G.S.INO;4 ( c) P/20) | 11948/36 | P%cft | 34,41,128/- |
| 08 |  | Construction of standard open drains cunette block of cement concrete 1:2:4 in situ to the design profile i/c cost of mould as per drawing i/c applying floating coat of cement 1/32” thick to the exposed face finished smooth curring etc: complete as per detailed drawing (P.H.S.I.NO;1, P/58) |  |  |  |
|  | 10800.0 Rft | TYPE ‘A’ | 94/- | P/Rft | 10,15,200/- |
|  | 5200.0 Rft | TYPE ‘B’ | 174/- | P/Rft | 9,04,800/- |
|  | 300.00 Rft | TYPE ‘C’ | 309/- | P/Rft | 92,700/- |
| 09 | 49554.00 Sft | Cement plaster (1:4) upto 12’ft height thick ½” thick (G.S.I.NO;11( b) P/51) | 2283/93 | P%sft | 11,31,779/- |
| 10 | 1638.24 Cft | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour forbending and binding which will be paid separately. This rate also includes all kinds of forms moulds, lifting, shuttering curring rendering and finishing the exposed surface)\(including screening and washing of shingle)(a)R.C. Work in roofslab, beams, columns,rafts, lintels and other structural members laid in situe or precast laid in position complete in all respect(i)Ratio 1:2:4)90 lbs cement 2 cft sand 4 cft shingle 1/8” to ¼” guage. (G.S.I.NO;6(a)(i)P/19) | 337/- | P/Cft | 5,52,087/- |

(Cont: on page No;02)

(Page No:02)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 11 | 58.50 Cwt | Fabrication of mild steel reinforcement for cement concrete including cutting bending laying in position making joints and fastenings including cost of binding wire (also includes removal of rust from bars(G.S.I.NO;3(ii)(b)P/10) | 5001/70 | P/Cwt | 2,92,599/- |
| 12 | 20250.0 Cft | Barrow pit excavation undressed lead upto 100’ft(in ordinary soil)(G.S.I.NO;3(a)P/1) | 2117/50 | P%0cft | 42,879/- |
| 13 | 20250.0 Cft | Earth work compaction (in ordinary hard soil) Laying earth in 6” layers leveling dressing and watering for compaction etc: complete (G.S.I.NO;13( b) P/3) | 354/- | P%0cft | 7,169/- |
| 14 | 5605.80 Cft | Extra for every 50’ ft additional or part there off (a) for earth work (soft ordinary hard and very hard)(G.S.I.NO:P, P/3) | 5039/- | P%0cft | 28,247/- |
| 15 | 29700.00 Sft | Paving & Fixing cement paving blocks flooring having size of 197x97x80 (mm) of city / quddra/cobble shape with pigmented having strength b/w 5000 psi i/c fitting the joints with hill sand and layingin specified manner / patter and design etc: complete (G.S.I.NO;73,P/50) | 223/97 | P/sft | 66,51,909/- |
| 16 | 7425.00 Cft | Cement concrete plain i/c placing compacting finishing and curring complete(i/c screening and washing of stone aggregate without shuttering (Ratio 1:4:8) (G.S.I.NO;5(c)p/18) | 11288/75 | P%cft | 8,38,190/- |
|  |  |  |  | Total | 1,63,96,070/- |
|  |  | GROUP ‘G’ CONSTT: OF OXIDATION POND. |  |  |  |
| 01 | 52132.21 Cft | Excavation in foundation for tanks and reservoir in sofit soil trimming and dressing sides to true alignment/design section profiled and shape leveling of bed of tanks to correct level and grade i/c laying in 6” layers for constructionof banks and disposal of surplus earth with in one chain as directed by Engineer in charge i/c providing fence guard, lights, flags and wherever required lift upto5’ ft: and lead up to one chain etc: complete. (P.H.S.I.No: 1,P/67). | 3000/- | P%0cft | 1,56,397/- |
| 02 | 6048.00 Cft | Excavation in foundation for tanks and reservoir in wet soil / Clay or mud i/c trimming and dressing sides to true alignment/design section profiled and shape leveling of bed of tanks to correct level and grade i/c laying in 6” layers for construction of banks and disposal of surplus earth with in one chain as directed by Engineer in charge i/c providing fence guard, lights, flags and wherever required lift upto 5’ ft: and lead up to one chain etc: complete. (P.H.S.I.No: 4,P/74). | 4800/- | P%0cft | 29,030/- |

(Cont; on page No:03)

(Page No:03)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 03 | 57388.50 Cft | Earth work compaction (Sofit, ordinary or hard soil ) Laying earth in 6” layers levelling, dressing and watering for compaction etc: complete. (GSI.No:13(B)P/3). | 354/- | P%0cft | 20,314/- |
| 04 | 58180.0 ft | Extra for every 50; ft: additional lead or part there of (G.S.I.No:8,P/2) | 403/12 | P%0cft | 319/- |
|  |  | SUMP WELL. |  |  |  |
| 01 | 791.70 Cft | Excavation in foundation of building bridges and other structures including dabbling dressing refilling around structures with excavated earth watering and ramming lead upto 5’ft (in sand soil ) (G.S.I.NO;18,. P/4) | 3176/25 | P%0cft | 718/- |
| 02 | 28.26 Cft | Cement Concrete brick or stone ballast 1 ½” to 2” guage Ratio (1:4:8) (G.S.I.No:4,d,P/14). | 9416/28 | P%cft | 2,661/- |
| 03 | 187.00 Cft | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kinds of forms moulds, lifting, shuttering currying rendering and finishing the exposed surface)(including screening and washing of shingle)(a)R.C. Work in roof slab, beams, columns, rafts, lintels and other structural members laid in situe or precast laid in position complete in all respect(i)Ratio 1:2:4)90 lbs cement 2 cft sand 4 cft shingle 1/8” to ¼” gauge.(G.S.I.NO;6(a)(i)P/17) | 337/- | P/cft | 63,019/- |
| 04 | 7.51 Cwt | Fabrication of mild steel reinforcement for cement concrete including cutting bending laying in position making joints and fastenings including cost of binding wire (also includes removal of rust from bars (G.S.I.NO;7(ii)(a)P/20) | 5001/70 | P/Cwt | 37,563/- |
|  |  | INTER CONNECTION 18” DIA |  |  |  |
| 01 | 1960.0 Cft | Excavation for pipe lines into trenches and pits in Soft soil i/c trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer incharge providing fence guards lights flags and temporary crossing for non vehicular traffic where ever required lift upto 5’ft(1.52m) and lead upto one chain (30.5mm) (P.H.S.I.NO; 5, P/38) | 3600/- | P%0cft | 7,056/- |

(Cont: on page No; 04)

(Page No:04)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 02 |  | Providing Lying RCC ASTM Pipe C-76-62 T/C -76-70 of class II walls ‘B’ fixing in trenches i/c cutting and jointing with rubber rings i/c testing with water to specified pressure. (P.H.S.I.NO;1(b),P/17) |  |  |  |
|  | 112.0 Rft | 18” dia | 618/- | P/Rft | 69,216/- |
| 03 | 1764.0 Cft | Refilling the excavated stuff in trenches 6” thick layers i/c watering ramming to full compaction etc: complete (P.H.S.I.NO;24, P/53) | 2760/- | P%0cft | 4,869/- |
|  |  |  |  | Total | 3,91,164/- |

A B S T R A C T

| Group | Schedule Item | Non Schedule Item | Total |
| --- | --- | --- | --- |
| ‘A’ Constt: Open surface drains Type I,II,III & C.C Block | 1,63,96,070/- | ----- | 1,63,96,070/- |
| ‘B’ Constt: of Oxidation Pond | 3,91,164/- | ------ | 3,91,164/- |
| Total | 1,67,87,234/- | ----- | 1,67,87,234/- |

Amount Total (a) + (b) Rs:___________________

________% above / below on the rates of CSR, Amount to be added / deducted on the basis

Total (b) of premium quoted ______________

Total (c ) offered for Non-schedule items_________

Total (A) = a + b in works & Figures______________________________________________

Contractor 						 	EXECUTIVE ENGINEER

PUBLIC HEALTH ENGG:DIVISION

JAMSHORO