BILL OF QUANTITIES

NAME OF WORK:-	Water Supply scheme of Villages Hussain Leghari, Larh Ware Machi, 				Usman Solangi, and Bhamban in U.C. Shaikh 	Taluka Sehwan 					Sharif) ( District Jamshoro).

(A) 	Description and rate of item based on Composite schedule of Rates

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
|  |  | GROUP ‘A’ CONSTT: OF STORAGE TANK SIZE 80’x55’ |  |  |  |
| 01 | 30853.0 Cft | Excavation for tanks and reservoirs in soft soils i/c trimming and Dressing sides to true alignment / design section /profiles and shape leveling of beds of trenches to correct level and grade i/c laying of earth in 6’ layers construction of banks and dressing disposal of surplus earth . Excavated earth within one chain as directed by Engineer In charge. i /c providing fence guards lights, flags & temporary crossing for non vehicular traffic, where ever required lift up to 5’ ft (1.52m) and lead up to one chain. (30.5m) (P.H.S.I.No:1,P/67). | 3000/- | P%0cft | 92,559/- |
| 02 | 21892.0 Cft | Excavation for tanks and reservoirs in wet soils clay or mud i/c trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade, cutting joints holes earth and disposal of surplus within one chain as directed by Engineer In charge. Providing fence guards lights, flags & temporary crossing for non vehicular traffic, where ever required lift up to 5’ ft (1.52m) and lead up to one chain.(30.5m). (P.H.S.I.No:15,P/74). | 4800/- | P%0cft | 1,05,082/- |
| 03 | 42196.0 Cft | Extra for 50’ft additional lead or part there of for earth work (soft ordinary or hard very hard ) (P.H.S.I.NO;8 (a) P/2) | 1814/04 | P%0cft | 76,545/- |
| 04 | 5753.00 Cft | Cement concrete brick or stone ballast 1:1/2" to 2" guage (Ratio 1:4:8) (GSI.No.4/b P/14). | 9416/28 | P%cft | 5,41,719/- |
| 05 | 8971.0 Cft | Pacca brick work in foundation and plinth cement sand mortar (Ratio 1:6) (G.S.I.NO;4(e) P/20) | 11948/36 | P%cft | 10,71,887/- |
| 06 | 2207.06 Cft | Cement concrete plain including placing compacting finishing and curring complete including screening and washing of stone aggregate without shuttering (Ratio 1:2:4)(G.S.I.NO;5(c)P/15) | 14429/25 | P%cft | 3,18,462/- |
| 07 | 4551.0 Sft | Cement plaster (1:4) upto 12' ft: height ¾” thick (GSI.No.11/b P/52). | 2283/93 | P%sft | 1,37,247/- |
| 08 | 1194.00 Sft | Cement pointing struck joint on wall (1:3) upto 20’ft height (G.S.I.LNO;19(b) P/53) | 1213/58 | P%sft | 14,490/- |
| 09 | 111.07 Cft | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kinds of forms moulds, lifting, shuttering currying rendering and finishing the exposed surface)(including screening and washing of shingle)(a)R.C. Work in roof slab, beams, columns, rafts, lintels and other structural members laid in situe or precast laid in position complete in all respect(i)Ratio 1:2:4)90 lbs cement 2 cft sand 4 cft shingle 1/8” to ¼” gauge.(G.S.I.NO;6(a)(i)P/17)(C/P/No:02) | 337/- | P/Cft | 37,431/- |

(Page No:02)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 10 | 4.46 Cwt | Fabrication of mild steel reinforcement for cement concrete including cutting bending laying in position making joints and fastenings including cost of binding wire (also includes removal of rust from bars (G.S.I.NO;7(ii)(a)P/20) | 5001/70 | P/Cwt | 22,308/- |
|  |  | INTER CONNECTION 15” DIA |  |  |  |
| 11 | 560.0 cft | Excavation for pipe line in trenches and pits in all kind of soils of murm i/c trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade, cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guards, lights, flags and temporary crossings for non-vehicular traffic where ever required lift upto 5 ft: (1.52 m ) and lead upto one chain (30.50m) (PHS.I.No.5 P/62) | 3600/- | P%0cft | 2.016/- |
| 12 |  | Providing, laying RCC pipes of A.S.T.M C-76-62 T/C-76-70 of class II wall B and fixing in trench i/c cutting, fitting and jointing with rubber ring i/c testing with water to Specified pressure) (PHS.I.No.1 P/17) |  |  |  |
|  | 40.0 Rft | 12” dia | 412/- | P/Rft | 16,480/- |
| 13 | 504.00 cft | Refilling the excavated stuff in trenches 6” thick layers i/c watering ramming to full compaction etc: complete (P.H.S.I.NO;24,P/53) | 2760/- | P%0cft | 1,391/- |
|  |  |  |  | Total | 24,37,617/- |
|  |  | GROUP ‘B’ SUPPLY OF HYPOCHLORINATOR |  |  |  |
| 01 | 1 No; | Putton (USA) Series “A” Model LBS3SB –VTC –XX Hypo chlorinator (Electronic Metering pump) Flow-Rate 33 GPD Max, pressure 150 PSI/10-Bar Solenoid protection by themal over load with Auto Rest Manual control, Dosage adjustable 0-100% Highly reliable water resistant made for indoor and oudoor installation guided Ball check valve circuit protection against voltage & circuit upset material non corrosive made to inject sodium Hypochloriti solution in water supply delivery main line following standard accessories with each unit as per attached light Bulletin # EMPO25 BS. 1) clear flexible solution tubing 1) Stiff white discharge Tubing 1)Foot valve / strainer Assembly. 1)Back pressure injection volave. 1)Strainer weight. 1)Installation Operation Maintenance manual. | 75000/- | Each | 75,000/- |
|  |  |  |  | Total | 75,000/- |

(Cont: on page No;03)

(Page No:03)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
|  |  | Group‘D’ Construction of Pump House. |  |  |  |
| 01 | 1284.0 Cft | Excavation in foundation of building bridges and other structures including dabbling dressing refilling around structures with excavated earth watering and ramming lead upto 5’ft (in hard soil or soft murum) (G.S.I.NO;18,.(b) P/4) | 3176/25 | P%0cft | 4,078/- |
| 02 | 226.51 Cft | Cement concrete brick or stone ballast 1½” to 2” gauge Ratio 1:4:8) (G.S.I.NO;4(b)P/16) | 9416/28 | P%cft | 21,329/- |
| 03 | 778.14 Cft | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kinds of forms moulds, lifting, shuttering currying rendering and finishing the exposed surface)(including screening and washing of shingle)(a)R.C. Work in roof slab, beams, columns, rafts, lintels and other structural members laid in situe or precast laid in position complete in all respect(i)Ratio 1:2:4)90 lbs cement 2 cft sand 4 cft shingle 1/8” to ¼” gauge.(G.S.I.NO;6(a)(i)P/17) | 337/- | P/cft | 2,62,233/- |
| 04 | 31.26 Cwt | Fabrication of mild steel reinforcement for cement concrete including cutting bending laying in position making joints and fastenings including cost of binding wire (also includes removal of rust from bars (G.S.I.NO;7(ii)(a)P/20) | 5001/70 | P/Cwt | 1,56,353/- |
| 05 | 513.0 Cft | Pacca brick work in ground floor in cement sand mortor (1:6) (G.S.I.NO;2 (e) P/20) | 12674/36 | P%cft | 65,019/- |
| 06 | 1596.0 Sft | Cement plaster (1:4) 3/8” thick upto 12’ height (G.S.I.NO;11 (b) P/52) | 2197/52 | P%sft | 35,072/- |
| 07 | 1596.0 Sft | Cement plaster (1:4) 3/8” thick upto 12’ height (G.S.I.NO;11 (b) P/52) | 2206/60 | P%sft | 35,217/- |
| 08 | 240.0 sft | Laying white marble flooring fine dressed on the surface without winding set in lime mortar 1:2 including rubber and polishing of the joint (G.S.I.NO;28, P/43)3/4” thick | 567/48 | P/Sft | 1,36,195/- |
| 09 | 960.0 Cft | Filling watering & ramming earth in under floor with surplus earth from foundation lead upto one chain and lift upto 5’ft (G.S.I.NO;22, P/44) | 1512/50 | P%0Cft | 1,452/- |
| 10 | 56.00 Sft | Making and fixing steel grated door with 1/16” thick sheeting i/c angle iron frame2”x2”x3/8” and ¾” square bars 4” center to center with locking arrangement(G.S.I.NO;28, P/97) | 726/72 | P/sft | 40,696/- |
| 11 | 3.39 Cwt | Fabrication of heavy steel work with angles tees, flat iron , round iron and sheet iron for making trusses girdes tands etc including cutting tanks etc.i/c cutting drilling iverting handling assembling and fixing but excluding erection in position (G.S.I.No.2,P-91) | 4928/49 | P/Cwt | 16,708/- |

(Cont: on page No;04)

(Page No:04)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 12 | 40.00 Sft | Supplying and fixing in position iron/steel grill or ¾” x ¼” size flate iron of approved design i/c painting 3 coats etc: complete ((weight not be less than 3.7lbs/sq: feet of finished (GSI.NO;26, P/92) | 180/50 | P/Sft | 7,220/- |
| 13 | 3.39 Cwt | Errection and fitting in position of iron trusses staging of water tanks etc: (G.S.I.NO;3, P/96) | 271/04 | P/cwt | 919/- |
| 14 | 192.0 Sft | PAINTING NEW SURFACE. Preparing surface and painting of doors and windows in any type (3 coats) (G.S.I.NO;5P/69) | 2116/41 | P%sft | 4,064/- |
| 15 | 1596.00 Sft | Priming one coats of chalk. (G.S.I.NO; 23, P/54) | 442/75 | P%sft | 7,066/- |
| 167 | 2074.50 Sft | Distempering (2 coats) (G.S.I.NO;24, P/54) | 1043/90 | P%Sft | 21,656/- |
|  |  |  |  | Total | 8,15,277/- |
|  |  | Group ‘D’ Providing Installing Submersible Pump 7.50 BHP |  |  |  |
| 01 | 2 Sets | Supplying & installing in position i/c transportation to site of work at Hussain lighari 7.50 BHP A.C. Electric motor 3 Phase 50 cycles 400/420 volts of 2950 Centrifugal pump Type 40-200size 2” x1½” mounted on common steel frame or base plate capable of discharge130 gallons/ minute against a head of 100’ ft: i/c installing of pumping set on Cement Concrete foundation ratio 1:2;4 and C.C.(1:4:8) with ballast with M.S. nuts and bolts of 5/8” dia to be embedded in C.C. Foundation i/c providing and fixing on prepare board A.C 50 Cycles 3 phase 12/25 ampere trips rating push button type star delta starter complete with no volts and over load release and required i/c providing & fixing on prepared board iron (clad) triple pole and neutral switch fuse with rewire able type fuse link carried 500 volts grade 30 ampere providing and fixing volts meter 0-500 volts and ampere meter 0.50 Amps: installed on Metal led board i/c providing and fixing Metalled board to accommodate main or sub main control switch,s recessed in the wall size 3’x2 ½” i/c providing phase indication bulbs holder and providing internal Electric wiring for main with 3-7/1.04 7/0.44 P.V.C in (R.A Attached). (Cont: on page No:05) | 262250/- | P/Set | 5,24,500/- |

(Page No:05)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 02 | 2 set | Supplying & installing in position i/c transportation to site of work at Hussain Leghari & adjoining village A.C Electric pumping set consisting of A.C. Electric motor 10 BHP 3 Phase 50 cycles 400/420 volts of 2950 RPM couple with KSB pump Type 40-200 size 2”x1½” mounted on common steel frame or base plate capable of discharge 340 gallons/ minute against a head of 55’ ft: i/c installing of pumping set on Cement concrete foundation ratio 1:2;4 and C.C. (1:4:8) with ballast with M.S. nuts and bolts of 5/8” dia to be embedded in C.C. Foundation i/c providing and fixing on prepare board A.C 50 Cycles 3 phase 12/25 ampere trips rating push button type star delta starter complete with no volts and over load release and required i/c providing & fixing on prepared board iron (clad) triple pole and neutral switch fuse with rewire able type fuse link carried 500 volts grade 30 ampere providing and fixing volts meter 0-500 volts and ampere meter 0.50 Amps: installed on Metal led board i/c providing and fixing Metalled board to accommodate main or sub main control switch in the wall size 3’x2 ½” i/c providing phase indication bulbs holder and providing internal Electric wiring for main with 3-7/1.04 7/0.44 P.V.C(RA Attached) | 262250/- | P/Set | 5,24,500/- |
| 03 |  | Providing unuarmoured cable 4 core PVC/PVC Copper conductor 660/1000 volts (Elect: S.I.NO;1 (2& 3) P/10) |  |  |  |
| 04 | 20 Meter | 25 mm | 1386/- | P/Meter | 27,720/- |
| 05 |  | C.I. Sluice valve heavy pattern test pressure 21 kg/sq com or 300 lbs/sq inch.(imported) (P.H.S.I.NO: 2 P/ 11 ) |  |  |  |
|  | 4 Nos: | 3” dia | 4290/- | Each | 17,160/- |
| 06 |  | Reflex valve heavy pattern test pressure 300 lbs/sq; inch (P.H.S.(M)I-16, P/97) |  |  |  |
|  | 4 Nos: | 3” dia | 1543/75 | Each | 6,175/- |
| 07 | 55.36 Kg: | C.I. Tapper flate bottomed or central tapered flanged ends with holes Including turning and facing turning and facing of flanges for all size (P.H.S.I.M.I.NO;8, P/13) | 120/- | P/Kg: | 6,643/- |
| 08 |  | Foot valve heavy pattern with cone type gate (imported ) (M.S.I.NO;4 d P/11) |  |  |  |
|  | 4 Nos: | 3” dia | 731/25 | Each | 2,925/- |
| 09 |  | C.I short piece (S.M.I.NO;9, P/15) |  |  |  |
|  | 2 Nos: | 6” dia | 618/75 | Each | 1,238/- |

(cont: on page No:06)

(Page No:06)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 10 | 41.04 Kg: | C.I. Tee flanged end with holes, including turning and facing of flanges for all size (M.S.INO;12, P/12) | 120/- | Each | 4,925/- |
| 11 | 138.38 Kg: | C.I. Bend with flanged and work i/c jointing and fixing pf flanges for All size (M.S.I.No.8,P-12 | 120/- | Each | 16,606/- |
| 12 |  | Manufacturing and supplying M.S. Pipe 6” dia made out of M.S. Sheet 3/8” drilling holes and welding currect curvature etc: complete (R.A.Approved) |  |  |  |
|  | 88.0 Rft | 3” dia | 385/89 | P/Rft | 33,958/- |
|  | 42.00 Rft | 4” dia | 506/60 | P/Rft | 21,277/- |
| 13 |  | Manufacturing and supplying M.S. Flanged 6” dia made out of M.S. Sheet 3/8” drilling holes and welding etc: complete (P.SH.I.NO; P/ ) |  |  |  |
|  | 16 Nos: | 3” dia | 500/89 | P/Rft | 8,014/- |
|  | 12 Nos: | 4” dia | 506/60 | P/Rft | 7,161/- |
| 14 |  | Jointing C.I/M.S flanged pipes and specials flanged and inside a trenches i/c supplying rubber packing of the required thickness nuts bolts with washers etc and other tools required for jointing and testing the joints to the specified pressure etc: complete. (P.H.S.I.NO;1, P/40) |  |  | 8,014/- |
|  | 32 Joints | 3” dia | 499/- | P/joint | 15,968/- |
|  | 16 Joints | 6” dia | 938/- | P/Joint | 15,008/- |
|  |  |  |  | Total | 12,33,778/- |
|  |  | Group ‘E’ P/L/J and Testing Rising Main 6” dia |  |  |  |
| 01 | 66938.00 Cft | Excavation for pipe lines into trenches and pits in Soft soil i/c trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer incharge providing fence guards lights flags and temporary crossing for non vehicular traffic where ever required lift upto 5’ft(1.52m) and lead upto one chain (30.5mm) (P.H.S.I.NO; 1, P/60) | 3600/- | P%0cft | 2,40,977/- |
| 02 |  | Providing PVC pipe of class ‘B’ (equivalent make)fixing in trenches i/c cutting and jointing with “Z” joints with one rubber rings i/c testing with water to head of 61 meter or 200’ft. (P.H.S.I.NO;1,P/21) |  |  |  |
|  | 8500.0 Rft | 6” dia | 262/- | P/Rft | 22,27,000/- |
| 03 |  | P.V.C ‘Z’ Joints fitting (P.H.S(M),16 P/28) |  |  |  |
|  |  | i)P.V.C. Bend 90 |  |  |  |
|  | 4 Nos: | 6” dia | 1913/- | Each | 7,652/- |
| 04 |  | P.V.C. Socket (P.H.S.(M)O-14, P/29) |  |  |  |
|  | 10 Nos: | 6” dia | 1575/- | Each | 15,750/- |

(Cont: on page No:07)

(Page No:07)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 05 |  | Making joints C.I. specials fitting (i/c laying of specials and cost of solvent cement of required diameter and testing the joint along with PVC pipe line “B” Class and “D” Class of the specified pressure and making good to all leaky joints etc: complete.(P.H.S.I.NO:1, P/53) |  |  |  |
|  | 04 Nos: | 6” dia | 96/- | Each | 384/- |
| 06 |  | Fire hydrant tee (P.H.S.I.NO;10, P/18) |  |  |  |
|  | 6 Nos: | 6” dia | 1375/- | Each | 8,250/- |
| 07 |  | Manufacturing and supplying M.S. pipe 6” dia made out of M.S. Sheet 1/16” drilling holes and welding current curvature etc: complete (R.A.Approved) |  |  |  |
|  | 10 Nos: | 6” dia | 749/34 | Each | 44,960/- |
| 08 |  | Manufacturing and supplying M.S. flange 6” dia made out of M.S. Sheet 3/8” thick holes and welding currect curvature etc: complete (Based on sch: item) |  |  |  |
|  | 12 Nos: | 6” dia | 804/77 | Each | 9,657/- |
| 09 |  | Jointing C.I/M.S flanged pipes and specials flanged and inside a trenches i/c supplying rubber packing of the required thickness nuts bolts with washers etc and other tools required for jointing and testing the joints to the specified pressureetc: complete. (P.H.S.I.NO;1, P/40) |  |  |  |
|  | 3 Joints | 6” dia | 938/- | Each | 2,814/- |
| 10 |  | Supplying of Air valve double acting (3”x1”) imported (P.H.S.M.I.NO;8, P/12) |  |  |  |
|  | 6 Nos: | 6” dia | 4000/- | Each | 24,000/- |
| 11 | 6 Nos: | Providing Chamber 3'x2' (915x615 mm) in dimension 4-1/2' (1372mm) deep as per approved design for sluice valve to 12"dia with 18" (457mm) dia inside cost iron cover and frame (wt= 1 cwt 3 sq) fixed in RCC 1:2:4 (102mm) thick (with 5 lbs steel per cft) 9" (299 mm) thick brick masonary wall set in 1:6 cement mortor 6" (1152 mm) thick cement concrete 1:3:6 in foundation 1" (25mm) thick cement concrete 1:2:4 flooring 1/2" (12.50 mm) thick cement plaster 1:3 to all inside wall surface and to top i/c providing and fixing M.S. foot rest at every one foot beyond 2-1/2 ft depth curing, excavation, back filling and disposal of surplus earth etc complete (PHS. I.No.Q-1 P/ 49 ) | 18820/- | Each | 1,12,920/- |

(Cont: on page No:08)

(Page No:08)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 12 | 60244.0 Cft | Refilling the excavated stuff in trenches 6” thick layers i/c watering ramming to full compaction etc: complete (P.H.S.I.NO;24,P/53) | 2760/- | P%0cft | 1,66,273/- |
|  |  |  |  | Total | 28,60,637/- |
|  |  | Group ‘F’ P/L/J and Testing Delivery Main 6” dia |  |  |  |
| 01 | 70088.0 Cft | Excavation for pipe lines into trenches and pits in Soft soil i/c trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer incharge providing fence guards lights flags and temporary crossing for non vehicular traffic where ever required lift upto 5’ft(1.52m) and lead upto one chain (30.5mm) (P.H.S.I.NO; 1, P/60) | 3600/- | P%0cft | 2,52,317/- |
| 02 |  | Providing PVC pipe of class ‘B’ (equivalent make)fixing in trenches i/c cutting and jointing with “Z” joints with onerubber rings i/c testing with water to head of 61 meter or 200’ft. (P.H.S.I.NO;1,P/21) |  |  |  |
|  | 8900.0 Rft | 6” dia | 262/- | P/Rft | 23,31,800/- |
| 03 |  | P.V.C ‘Z’ Joints fitting (P.H.S(M),16 P/28) |  |  |  |
|  |  | i)P.V.C. Bend 90 |  |  |  |
|  | 8 Nos: | 6” dia | 1913/- | Each | 15,304/- |
| 04 |  | PVC. TEE (PHS(M) I-9, P/ |  |  |  |
|  | 6 Nos: | 6” dia | 1800/- | Each | 10,800/- |
| 05 |  | P.V.C. Socket (P.H.S.(M)O-14, P/29) |  |  |  |
|  | 10 Nos: | 6” dia | 1575/- | Each | 15,750/- |
| 06 | 2 Nos: | Reducer 6” x 4” | 893/75 | Each | 1,788/- |
|  | 2 Nos: | Reducer 4” x 3” | 568/75 | Each | 1,138/- |
| 07 |  | Making joints C.I. specials fitting (i/c laying of specials and cost of solvent cement of. required diameter and testing the joint along with PVC pipe line “B” Class and “D” Class of the specified pressure and making good to all leaky joints etc: complete.(P.H.S.I.NO:1, P/53) |  |  |  |
|  | 18 Nos: | 6” dia | 96/- | Each | 1,728/- |

(Cont; on page No:09)

(Page No:09)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 08 | 63079.0 Cft | Refilling the excavated stuff in trenches 6” thick layers i/c watering ramming to full compaction etc: complete (P.H.S.I.NO;24,P/53) | 2760/- | P%0cft | 1,74,098/- |
|  |  |  |  | Total | 28,04,723/- |
|  |  | Group ‘G’ P/L/J and Testing Distb: system 4” , 3” dia. |  |  |  |
| 01 | 2250.0 Cft | Dismantling and removing road (G.S.I.NO;51, P/14) | 605/- | P%Cft | 13,613/- |
| 02 | 60150.0 Cft | Excavation for pipe lines into trenches and pits in Soft soil i/c trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer incharge providing fence guards lights flags and temporary crossing for non vehicular traffic where ever required lift upto 5’ft(1.52m) and lead upto one chain (30.5mm) (P.H.S.I.NO; 1, P/60) | 3600/- | P%0cft | 2,16,540/- |
| 03 |  | Providing PVC pipe of class ‘B’ (equivalent make)fixing in trenches i/c cutting and jointing with “Z” joints with one rubber rings i/c testing with water to head of 61 meter or 200’ft. (P.H.S.I.NO;1,P/21) |  |  |  |
|  | 2200.0 Rft | 4” dia | 137/- | P/Rft | 3,01,400/- |
|  | 8200.0 Rft | 3” dia | 90/- | P/Rft | 7,38,000/- |
| 04 |  | P.V.C. ‘Z’ joints fitting (P.H.S(M)I, P/27) |  |  |  |
|  | 10 Nos: | 4” dia | 956/- | Each | 9,560/- |
|  | 20 Nos: | 3” dia | 731/- | Each | 14,620/- |
| 05 |  | PVC Tee |  |  |  |
|  | 5 Nos: | 4” dia | 1181/- | Each | 5,905/- |
|  | 10 Nos: | 3” dia | 900/- | Each | 9,000/- |
| 06 |  | P.V.C. Socket (P.H.S(M)I P/27) |  |  |  |
|  | 8 Nos; | 4” dia | 1125/- | Each | 9,000/- |
|  | 15 Nos: | 3” dia | 405/- | Each | 6,075/- |
| 07 |  | C.I. Sluice valve heavy pattern test pressure 21 kg/sq com or 300 lbs/sq inch.(imported) (P.H.S.I.NO: 2 P/ 11 ) |  |  |  |
|  | 2 Nos; | 4” dia | 5460/- | Each | 10,920/- |
|  | 5 Nos: | 3” dia | 4290/- | Each | 21,450/- |
| 08 |  | C.I. Tail Pieces for PVC (P.H.S(M)INO;9, P/15) |  |  |  |
|  | 4 Nos: | 4” dia | 487/50 | Each | 1,950/- |
|  | 10 Nos: | 3” dia | 375/- | Each | 3,750/- |

(Cont: on page No:10)

(Page No:10)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 09 |  | Making joints C.I. specials fitting (i/c laying of specials and cost of solvent cement of. required diameter and testing the joint along with PVC pipe line “B” Class and “D” Class of the specified pressure and making good to all leaky joints etc: complete. (P.H.S.I.NO:1, P/53) |  |  |  |
|  | 20 Nos: | 6” dia | 70/- | Each | 1,400/- |
|  | 40 Nos: | 3” dia | 59/- | Each | 2,360/- |
| 10 |  | Jointing C.I/M.S flanged pipes and specials flanged and inside a trenches i/c supplying rubber packing of the required thickness nuts bolts with washers etc and other tools required for jointing and testing the joints to the specified pressureetc: complete. (P.H.S.I.NO;1, P/40) |  |  |  |
|  | 4 Joints | 4” dia | 513/- | Each | 2,052/- |
|  | 10 Joint | 3” dia | 499/- | Each | 4,990/- |
| 11 | 7 Nos: | Providing Chamber 3'x2' (915x615 mm) in dimension 4-1/2' (1372mm) deep as per approved design for sluice valve to 12"dia with 18" (457mm) dia inside cost iron cover and frame (wt= 1 cwt 3 sq) fixed in RCC 1:2:4 (102mm) thick (with 5 lbs steel per cft) 9" (299 mm) thick brick masonary wall set in 1:6 cement mortor 6" (1152 mm) thick cement concrete 1:3:6 in foundation 1" (25mm) thick cement concrete 1:2:4 flooring 1/2" (12.50 mm) thick cement plaster 1:3 to all inside wall surface and to top i/c providing and fixing M.S. foot rest at every one foot beyond 2-1/2 ft depth curing, excavation, back filling and disposal of surplus earth etc complete (PHS. I.No.Q-1 P/ 49 ) | 18820/- | Each | 1,31,740/- |
| 12 | 56160.0 Cft | Refilling the excavated stuff in trenches 6” thick layers i/c watering ramming to full compaction etc: complete (P.H.S.I.NO;24,P/53) | 2760/- | P%0cft | 1,55,002/- |
|  |  |  |  | Total | 16,59,327/- |
|  |  | GROUP ‘G’ CONST: OF COMPOUND WALL |  |  |  |
| 01 | 2544.0 Cft | Excavation in foundation of building bridges and other structure including dag belling dressing, refilling arround structure with excavated earth watering ramming lift upto 5ft and lead upto one chain (G.S.I.NO: 18 P/4 ) | 3176/25 | P%0cft | 8,080/- |

(Cont: on page No;11)

(Page No:11)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 02 | 636.0 Cft | Cement concrete brick or stone ballast 1:1/2" to 2" guage (Ratio 1:4:8) (GSI.No.4/b P/14) | 9416/28 | P%cft | 59,888/- |
| 03 | 4389.0 Sft | Errection and removal of centering for R.C.C or plain cement concrete Works of Deodar wood(2nd Class)(Vertical) (G.S.I:No:19(ii),P/18). | 3127/41 | P%sft | 1,37,262/- |
| 04 | 935.0 Cft | Cement concrete plain i/c placing compacting, finishing & complete etc (Ratio 1:3;6) (G.S.I.No 5(h) P/ 15). | 12595/- | P%sft | 1,17,763/- |
| 05 | 1289.0 Cft | Pacce Brick work in other than building i/c stricking of joints in cement sand mortor (1;6) upto 20’ft height. (G.S.I.NO;7, P/22) | 12346/65 | P%Cft | 1,59,143/- |
| 06 | 78.00 cft | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kinds of forms moulds, lifting, shuttering currying rendering and finishing the exposed surface)(including screening and washing of shingle)(a)R.C. Work in roof slab, beams, columns, rafts, lintels and other structural members laid in situe or precast laid in position complete in all respect(i)Ratio 1:2:4)90 lbs cement 2 cft sand 4 cft shingle 1/8” to ¼” gauge.(G.S.I.NO;6(a)(i)P/17) | 337/- | P/cft | 26,286/- |
| 07 | 3.13 Cwt | Fabrication of mild steel reinforcement for cement concrete including cutting bending laying in position making joints and fastenings including cost of binding wire (also includes removal of rust from bars (G.S.I.NO;7(ii)(a)P/20) | 5001/70 | P/Cwt | 15,655/- |
| 08 | 7059.0 Sft | Cement Plaster (1:4) up to 12’ height ½” thick(G.S.I.No:11(b),P/51). | 2283/93 | P%sft | 1,61,223/- |
| 09 | 50.0 Sft | Making & fixing steel grated door with 1/16" thick sheeting including angle iron frame 2"x2"x3/8" and 3/4" square bars 4" centre to centre with locking arrangement (GSI.No.24 P/91) | 726/72 | P/Sft | 36,336/- |
| 10 | 9299.00 Sft | Priming coat of chalk distemper (G.S.I.NO;23, P/53) | 442/75 | P%Sft | 41,171/- |
| 11 | 9299.0 Sft | Distempering (Two coats) (G.S.I.NO;23, P/53) | 1043/90 | P%Sft | 97,072/- |
| 12 | 100.0 Sft | Painting new surface (c ) preparing surface and painting of doors and windows any type (including edges) (G.S.I.NO; (c ) P/69) | 2116/41 | P%sft | 2,116/- |
|  |  | (Cont: on page No;12) |  | Total | 8,61,995/- |

(Page No:12)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
|  |  | Group ‘H’ Providing and Installing Solar Panel System. |  |  |  |
|  |  | 10 BHP |  |  |  |
| 01 | 1 set | 1.Solar Panels 325 w high efficiency cell 20 years warranty cell poly crystal line, providing, installing, testing, connection and commissioning of solar PV modules, cell type polyerstalling, PV module efficiency 16-17% to design 10 KW solar (to run10 BHP Motor) PV arrat modules cells must be protected with IP65 compliance and must confirm to the latest edition of Q sele (made to Germany) and international standards with all accessories as required upto satisfaction of the Engineer incharge. 2.Solar mounting MS framing providing installing connecting testing and 10 BHP PV arrays. The structure design shall be designed to sustain worst weather connection. 3.Solar cable DC flexible 10 mm pure copper for whole system supply connection and termination of DC cable from PV modules of inverter to motor W.r.t system ratings volt’s / ampers copper gird cable to be laid in PVC conduit across the soute with the all accessories and material required from completion of job 4.Solar inverter VFD providing, installing testing and commissioning of (Sun indisen made in Germany) converter VFD for 12 K.W array, 3 Phase AC output voltage 440V, 50 HZ efficiency 97%& or higher completer with reverse current protection, DC over voltage protection, power control module, gird monitoring and AC/DC commectors for input output voltage complete with all accessories as required upto satisfaction of the Engineer incharge. 5.Breakers and breaker box (Terasaki made in Germany) 6 Junction Box 7 Lux, Ties Nut Bolt 8 PVC cable protection pipe bend for cable 9 Transportation of Goods. 10 Installation of the system | 1980000/- | P/Set | 19,80,000/- |
|  |  |  |  | Total | 19,80,000/- |

(Cont: on page No:13)

(Page No:13)

A B S T R A C T

| Group | Schedule Item | Non Schedule Item | Total |
| --- | --- | --- | --- |
| ‘A’ Construction of Storage Tank | 24,37,617/- |  | 24,37,617/- |
| ‘B’ Supply of Hypochlorinator | ----- | 75,000/- |  |
| ‘C’ Construction of Pump House. | 8,15,277/- | ---- | 8,15,277/- |
| ‘D’ Providing Installing Pumping Machinery | 1,84,778/- | 10,49,000/- | 12,33,778/- |
| ‘E’ Providing Laying Jointing and testing PVC Rising Main 6” dia | 28,60,637/- | ----- | 28,60,637/- |
| ‘F’ Providing Laying Jointing and testing PVC Delivery Main 6”dia | 28,04,723/- | ----- | 28,04,723/- |
| ‘G’ Providing, Laying Jointing and testing Distribution system 4”, 3” dia | 16,59,327/- | ----- | 16,59,327/- |
| ‘H’ Construction of Compound wall | 8,61,995/- | ---- | 8,61,995/- |
| ‘I’ Solar System 10 BHP | ---- | 19,80,000/- | 19,80,000/- |
| Total |  |  |  |

Amount Total (a) + (b) Rs:___________________

________% above / below on the rates of CSR, Amount to be added / deducted on the basis

Total (b) of premium quoted ______________

Total (c ) offered for Non-schedule items_________

Total (A) = a + b in works & Figures______________________________________________

Contractor 						 	EXECUTIVE ENGINEER

PUBLIC HEALTH ENGG:DIVISION

JAMSHORO