BILL OF QUANTITIES

NAME OF WORK:-	Drainage Scheme for Village Rakha and Jhandiyani (Taluka Sehwan) 				District Jamshoro.

(A) 	Description and rate of item based on Composite schedule of Rates

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
|  |  | Group ‘A’ Constt: Open surface drains Type I,II,III & C.C Block |  |  |  |
| 01 | 3630.0cft | Dismantling and removing road (G.S.INO;5,P/14) | 605/- | P%cft | 21,962/- |
| 02 |  | Dismantling Cement concrete plain (G.S.I.NO;19(a) P/16) |  |  |  |
|  | 1500.0Cft | Ratio (1:4:8). | 1663/75 | P%cft | 24,956/- |
|  | 375.0 Cft | Ratio (1:2:4). | 3327/50 | P%cft | 12,478/- |
| 03 | 100.80 Cft | Dismantling Cement Concrete reinforcement (G.S.I.NO; ) | 5445/- | P%cft | 5,489/- |
| 04 | 1500.0 Cft | Dismantling Brick work in lime (G.S.I.NO; ) | 1285/63 | P%cft | 19,284/- |
| 05 | 23541.0 Cft | Excavation in foundation of building bridges and other structures including dabbling dressing refilling around structures with excavated earth watering and ramming lead upto 5’ft (in sand soil ) (G.S.I.NO;18,. P/4) | 3176/25 | P%0cft | 74,772/- |
| 06 | 4985.00 Cft | Cement concrete plain i/c placing compacting finishing and curring complete(i/c screening and washing of stone aggregate without shuttering (Ratio 1:4:8) (G.S.I.NO;5(c)p/18) | 11288/75 | P%cft | 5,62,744/- |
| 07 | 13297.50 Cft | Pacca brick work in foundation and plinth in cement sand mortar (Ratio 1:6) (G.S.INO;4 ( c) P/20) | 11948/36 | P%cft | 15,88,833/- |
| 08 |  | Construction of standard open drains cunette block of cement concrete 1:2:4 in situ to the design profile i/c cost of mould as per drawing i/c applying floating coat of cement 1/32” thick to the exposed face finished smooth curring etc: complete as per detailed drawing (P.H.S.I.NO;1, P/58) |  |  |  |
|  | 2565.0 Rft | TYPE ‘A’ | 94/- | P/Rft | 4,94,910/- |
|  | 2400.0 Rft | TYPE ‘B’ | 174/- | P/Rft | 4,17,600/- |
| 09 | 23161.00 Sft | Cement plaster (1:4) upto 12’ft height thick ½” thick (G.S.I.NO;11( b) P/51) | 2283/93 | P%sft | 5,28,981/- |
| 10 | 1381.39 Cft | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour forbending and binding which will be paid separately. This rate also includes all kinds of forms moulds, lifting, shuttering curring rendering and finishing the exposed surface)\(including screening and washing of shingle)(a)R.C. Work in roofslab, beams, columns,rafts, lintels and other structural members laid in situe or precast laid in position complete in all respect(i)Ratio 1:2:4)90 lbs cement 2 cft sand 4 cft shingle 1/8” to ¼” guage. (G.S.I.NO;6(a)(i)P/19) | 337/- | P/Cft | 4,65,528/- |

(Cont: on page No;02)

(Page No:02)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 11 | 49.33 Cwt | Fabrication of mild steel reinforcement for cement concrete including cutting bending laying in position making joints and fastenings including cost of binding wire (also includes removal of rust from bars(G.S.I.NO;7(ii)(a)P/20) | 5001/70 | P/Cwt | 2,46,734/- |
| 12 | 959.00 Cft | Barrow pit excavation undressed lead upto 100’ft(in ordinary soil)(G.S.I.NO;3(a)P/1) | 2117/50 | P%0cft | 2,831/- |
| 13 | 24500.0 Cft | Earth work compaction (a)Laying earth in 6” layers leveling dressing and watering for compaction Etc: complete (G.S.I.NO;13( b) P/3) | 354/- | P%0cft | 8,673/- |
| 14 | 7964.00 Cft | Extra for every 50’ ft additional or part there off (a) for earth work (soft ordinary hard and very hard)(G.S.I.NO:P, P/3) | 5039/- | P%0cft | 40,139/- |
| 15 | 9240.0 Cft | Cement concrete brick or stone ballast 1½” to 2” guage(Ratio 1:4:8) (G.S.I.NO;4(b)P/17) | 9416/28 | P%cft | 8,70,064/- |
| 16 | 7000.0 Cft | Cement concrete plain i/c placing compacting finishing and curring complete(i/c screening and washing of stone aggregate without shuttering Ratio (1:2:4)(G.S.I.NO;5(a)p/18) | 14429/25 | P%cft | 10,10,048/- |
| 17 | 466.0 Sft | Errection and removal of centering for RCC or plain cement concrete work s of partal wood (G.S.I.NO;19(b) P/18) | 3127/41 | P%sft | 14,574/- |
|  |  |  |  | Total | 64,05,103/- |
|  |  | Group ‘B’ Construction of R.C.C. Collecting Tank 15’ dia |  |  |  |
|  |  | COLLECTING TANK 15’ DIA. |  |  |  |
| 01 |  | Excavation of wet in dry upto 20’ft below ground level and Disposal in hard soil within one chain (G.S.I.NO;1(b)(i) P/85) |  |  |  |
|  | 1271.70 cft | 0-5’ Depth | 2247/58 | P%0cft | 2,858/- |
| 02 |  | Wet sinking of wells for depth below spring level by means of divers i/c all charges of shoring removing excavated material with one chain in (Hard soil) (GS.I.NO;3, P/87) |  |  |  |
|  | 1202.0 Cft | 0-5’ Depth | 11763/80 | P%0cft | 14,140/- |
|  | 1202.0 Cft | 5.1’-10’ depth | 24200/- | P%0cft | 29,088/- |
| 03 | 360.0 Cft | Cement concrete brick or stone ballast 1½” to 2” guage (Ratio 1:4:8) (G.S.I.NO;4(b)P/17) | 9416/28 | P%cft | 33,993/- |
| 04 |  | Cement concrete plain i/c placing compacting finishing and curring complete(i/c screening and washing of stone aggregate without shuttering (G.S.I.NO;5(a)p/18) |  |  |  |
|  | 177.0 cft | Ratio 1:4:8 | 11288/75 | P%cft | 19,981/- |
|  | 177.0 cft | Ratio 1:3:6 | 12595/- | P%cft | 22,293/- |
|  | 177.0 cft | Ratio 1:2:4 | 14429/25 | P%cft | 25,540/- |

(Cont: on page No;03)

(Page No:03)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 05 | 925.0 Cft | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kinds of forms moulds, lifting, shuttering currying rendering and finishing the exposed surface)(including screening and washing of shingle)(a)R.C. Work in roof slab, beams, columns, rafts, lintels and other structural members laid in situe or precast laid in position complete in all respect(i)Ratio 1:2:4)90 lbs cement 2 cft sand 4 cft shingle 1/8” to ¼” gauge.(G.S.I.NO;6(a)(i)P/17) | 337/- | P/cft | 3,11,725/- |
| 06 | 35.23 Cwt | Fabrication of mild steel reinforcement for cement concrete including cutting bending laying in position making joints and fastenings including cost of binding wire (also includes removal of rust from bars (G.S.I.NO;7(ii)(a)P/20) | 5001/70 | P/Cwt | 1,76,210/- |
| 07 | 11 Nos: | Foot steps (R.A Approved). | 181/65 | Each | 1,998/- |
| 08 | 151.0 Rft | Providing Expansion construction joints in concrete work of 9’ wide Corrugated PVC water stopper i/c soldering cost of material and labour (P.H.S.I.NO;02, P/56) | 86/- | P/Rft | 12,986/- |
| 09 | 1648.49 Cft | Bailing or pumping sub soil water during excavation concrete cost in situe concrete or operation allowed during plugging (PHSI.NO;18, P/75) | 543/- | P%cft | 8,951/- |
| 10 | 48.0 Kg: | Supplying of Dampo padlo (G.S.I.M.NO;46, P/3) | 125/- | P/Kg: | 60,000/- |
|  |  | INTER CONNECTION 15” DIA |  |  |  |
| 11 | 24.0 Rft | Providing, laying RCC pipes of A.S.T.M C-76-62 T/C-76-70 of class II wall B and fixing in trench i/c cutting, fitting and jointing with rubber ring i/c testing with water to Specified pressure) (PHS.I.No.1 P/17) | 515/- | P/Rft | 12,360/- |
| 12 | 288.00 Cft | Excavation for pipe line in trenches and pits in all kind of soils of murm i/c trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade, cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guards, lights, flags and temporary crossings for non-vehicular traffic where ever required lift upto 5 ft: (1.52 m ) and lead upto one chain (30.50m) (PHS.I.No.5 P/62) | 4650/- | P%0cft | 1,339/- |
| 13 | 259.0 Cft | Refilling the excavated stuff in trenches 6" thick layer i/c watering ramming to full compaction etc complete (PHS.No.24 P/77) | 2760/- | P%0cft | 715/- |
|  |  |  |  | Total | 6,80,177/- |

(Cont: on page No;04)

(Page No:04)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
|  |  | Group ‘C’ Construction of Screening Chamber (Size 5’ x 8’) |  |  |  |
| 01 |  | Excavation of foundation of building and bridges and other structures of dag belling (in ordinary soil) (G.S.I.No. 18, P/04) |  |  |  |
|  | 440.0 cft | 0’ to 5’ depth | 3176/25 | P%0cft | 1,398/- |
| 02 |  | Wet sinking of wells for depth below spring level by means of devers including all charges for shoring loading and removing excavated material within (In ordinary soil) (G.S.I.No. 3 P/86) |  |  |  |
|  | 264.0 cft | 5' - 8' depth | 3277/03 | P%0cft | 865/- |
| 03 | 44.0 Cft | Cement concrete brick or stone ballast 1:1/2" to 2" guage (Ratio 1:4:8) (GSI.No.4/b P/14) | 9416/28 | P%cft | 4,143/- |
| 04 | 256.13 Cft | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kinds of forms moulds, lifting, shuttering currying rendering and finishing the exposed surface)(including screening and washing of shingle)(a)R.C. Work in roof slab, beams, columns, rafts, lintels and other structural members laid in situe or precast laid in position complete in all respect(i)Ratio 1:2:4)90 lbs cement 2 cft sand 4 cft shingle 1/8” to ¼” gauge.(G.S.I.NO;6(a)(i)P/17) | 337/- | P/cft | 86,316/- |
| 05 | 10.29 Cwt | Fabrication of mild steel reinforcement for cement concrete including cutting bending laying in position making joints and fastenings including cost of binding wire (also includes removal of rust from bars (G.S.I.NO;8(ii)(a)P/17) | 5001/70 | P/Cwt | 51,467/- |
| 06 | 7 Nos: | Foot steps (R.A approved). | 181/65 | Each | 1,272/- |
| 07 | 1.34 Cwt | Fabrication of heavy steel work with angle tees flat iron and sheet iron For making girders etc: including, cutting tranks etc: i/c assembling and Fixing but excluding erection in position(GSI.No:2,P/90). | 4928/49 | P/Cwt | 6,604/- |
| 08 | 58.0 Rft | Providing "Expansion joint" in concrete work 9" wide corrugated P.V.C water stop (with bulb) i/c soldering cost of material and labouretc complete (PHS.I.No. 2 P/56) | 86/- | P/Rft | 4,988/- |
| 09 | 1.00 Cwt | Providing and fixing C.I Main Hole cover with frame i/c cost of material Etc complete (GSI.No:1, P/35). | 6985/- | P/Cwt | 6,985/- |
|  |  |  |  | Total | 1,64,038/- |

(cont: on page No:05)

(Page No:05)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
|  |  | Group‘D’ Construction of Pump House (10’x12’) |  |  |  |
| 01 | 248.96 Cft | Excavation in foundation of building bridges and other structures including dabbling dressing refilling around structures with excavated earth watering and ramming lead upto 5’ft (in ordinary soil)(G.S.I.NO;18,. P/4) | 3176/25 | P%0cft | 791/- |
| 02 | 93.00 Cft | Cement concrete brick or stone ballast 1½” to 2” gauge Ratio 1:4:8) (G.S.I.NO;4(b)P/16) | 9416/28 | P%cft | 8,757/- |
| 03 | 239.0 Cft | Cement concrete plain including placing compacting finishing and curring complete including screening and washing of stone aggregate without shuttering (Ratio :3:6)(G.S.I.NO;5(c)P/15) | 12595/- | P%cft | 10,289/- |
| 04 | 179.0 Cft | Errection and removal of centering for R.C.C or plain cement concrete works of deodar wood)2nd class)(for partal wood) (Vertical)(G.S.I.NO;19,(b) P/17) | 3127/41 | P%sft | 22,545/- |
| 05 | 171.24 Cft | Filling watering and ramming earth under floor with surplus earth (Excavated from out side)Lead upto one chain and lift upto 5’ft(G.S.I.NO;22, P/44) | 1512/50 | P%0cft | 260/- |
| 06 | 304.0 Cft | Pacca brick work ground floor including striking of joints Up to 20’ ft height in cement sand mortar (1:6) (GSINO:I-e,P-21) | 12674/36 | P%cft | 38,530/- |
| 07 | 106.86 Cft | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kinds of forms moulds, lifting, shuttering currying rendering and finishing the exposed surface)(including screening and washing of shingle)(a)R.C. Work in roof slab, beams, columns, rafts, lintels and other structural members laid in situe or precast laid in position complete in all respect(i)Ratio 1:2:4)90 lbs cement 2 cft sand 4 cft shingle 1/8” to ¼” gauge.(G.S.I.NO;6(a)(i)P/17) | 337/- | P/cft | 36,012/- |
| 08 | 4.29 Cwt | Fabrication of mild steel reinforcement for cement concrete including cutting bending laying in position making joints and fastenings including cost of binding wire (also includes removal of rust from bars (G.S.I.NO;7(ii)(a)P/20) | 5001/70 | P/Cwt | 21,457/- |
| 09 | 120.0 sft | Laying white marble flooring fine dressed on the surface without winding set in lime mortar 1:2 including rubber and polishing of the joint (G.S.I.NO;28, P/43)3/4” thick | 567/48 | P/Sft | 68,098/- |
| 10 | 865.00 Sft | Cement plaster (1:6) ½” thick upto 20’ft height (G.S.I.NO;11(b) P/58) | 2206/60 | P%sft | 19,087/- |
| 11 | 865.00 sft | Cement plaster (1:4) 3/8” thick upto 12’ft height (G.S.I.NO;11(b) P/58) | 2197/52 | P%sft | 19,009/- |

(cont: on page No:06)

(Page No:06)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 12 | 28.00 Sft | Making and fixing steel grated door with 1/16” thick sheeting i/c angle iron frame2”x2”x3/8” and ¾” square bars 4” center to center with locking arrangement(G.S.I.NO;28, P/91) | 726/72 | P/sft | 20,348/- |
| 13 | 3.05 Cwt | Fabrication of heavy steel work with angles tees, flat iron , round iron and sheet iron for making trusses girdes tands etc including cutting tanks etc.i/c cutting drilling iverting handling assembling and fixing but excluding erection in position (G.S.I.No.2,P-91) | 4928/49 | P/Cwt | 15,032/- |
| 14 | 3.05 Cwt | Errection and fitting in position of iron trusses staging of water tanks etc: (G.S.I.NO;3, P/96) | 271/04 | P/cwt | 827/- |
| 15 | 128.0 Sft | PAINTING NEW SURFACE. Preparing surface and painting of doors and windows in any type (3 coats) (G.S.I.NO;5P/69) | 2116/41 | P%sft | 2,709/- |
| 16 | 429.0 Sft | Priming coats of chalk. (G.S.I.NO; 23, P/54) | 442/75 | P%sft | 1,899/- |
| 17 | 718.50 Sft | Distempering (2 coats) (G.S.I.NO;24, P/54) | 1043/90 | P%Sft | 7,500/- |
|  |  | Cost for 2 Nos: 293150x2=586,300/- |  | Total | 2,93,150/- |
|  |  | Group ‘E’ Supplying Installing Pumping Machinery 10 BHP |  |  |  |
| 01 | 1 set | Supplying & installing in position i/c transportation to site of work at Rakha & Jhandiyani A.C Electric pumping set consisting of A.C. Electric motor 10 BHP 3 Phase 50 cycles 400/420 volts of 1450 RPM 132A80 coupled with KSB pump Type KWPZ(65-315) size 3” x2 1/2” mounted on common steel frame or base plate capable of discharge 120 gallons/ minute against a head of 90’ ft: i/c installing of pumping set on Cement Concrete foundation ratio 1:2;4 and C.C. (1:4:8) with ballast with M.S. nuts and bolts of 5/8” dia to be embedded in C.C. Foundation A.C 50 Cycles 3 phase 12/25 ampere trips rating push button type star delta starter complete with no volts and over load release and required i/c providing & fixing on prepared board iron (clad) triple pole and neutral switch fuse with rewire able type fuse link carried 500 volts grade 30 ampere providing and fixing volts meter 0-500 volts and ampere meter 0.50 Amps: installed on Metal led board i/c providing and fixing Metal led providing phase indication bulbs holder and providing internal Electric wiring for main with 3-7/1.04 7/0.44 P.V.C in (R.A Attached). | 743960/- | P/Set | 7,43,960/- |

(Cont: on page No:07)

(Page No:07)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 02 | 1 Set | Providing & Installing in position i/c transportation to site of work at Bambha 16 BHP Diesel Pumping set(Golden Made) coupled with non-clogging KSB pump type KWPZ 65/315 size 3”x2 ½” mounted onCommon steel frame or base plate capable of discharge 120 gallons per Minute against a head of 90’ ft: i/c installing of pumping set on Cement concrete foundation ratio 1:2:4 and C.C. (1:4:8) with ballast with M.S. nuts and bolts of 5/8” dia to be embedded in C.C. foundation i/c testing of pumping set against the required head of 72 hours etc; complete. (M.R) | 591675/- | P/Set | 5,91,675/- |
| 03 |  | Providing Unuarmoured cable 4 core PVC /PVC Copper conductor 660/1000 Volts (Elec: S.I.No.1(2 & 3) P/10) |  |  |  |
|  | 15 Meter | 25 mm | 1386/- | P/meter | 20,790/- |
|  | 15 Meter | 50 mm | 2495/- | P/meter | 37,425/- |
| 04 |  | C.I. Sluice valve heavy pattern test pressure 21 kg /sq com or 300 lbs/sq inch (imported) (P.H.S.I.NO: 2 P/ 11 ) |  |  |  |
|  | 2 Nos: | 4” dia | 4290/- | Each | 8,580/- |
| 05 |  | Reflex valve heavy pattern (test pressure 21 kg/sq or 300 lbs/sq inch (imported) (P.H.S.I.NO: 6 P/ 11 ) |  |  |  |
|  | 2 Nos: | 4” dia | 1543/75 | Each | 3,088/- |
| 06 | 15.75 Kg: | C.I.Tappers flat bottomed or central tappered flanged end with holes i/c truning and facing turning and facing of flanges for all sizes (M.S.I.NO: 9 P/12) | 120/- | P/Kg: | 1,890/- |
| 07 |  | Foot valve heavy patter with cone type gate (imported) (M.S.I.NO: 4/d P/ 11) |  |  |  |
|  | 2 Nos: | 4” dia | 731/25 | Each | 1,463/- |
| 08 |  | C.I. Short piece (S.M.I.No. 9 P/15) |  |  |  |
|  | 1 No: | 4” dia | 487/50 | Each | 488/- |
| 09 | 11.45 Kg | C.I.Tee flanged end with holes, including turning and facing of flanges for all sizs (M.S.I.NO: 12 P/12) | 120/- | P/Kg: | 1,374/- |
| 10 | 42.01 Kg: | C.I. bend flanged end with holes, including turning and facing of flanges for all size (M.S.I.NO: 8 P/12) | 120/- | P/Kg: | 5,041/- |
| 11 |  | Manufacturing and supplying M.S. Pipe 6"dia made out of M.S. sheet 3/8" drilling holes and welding currect curvature etc complete ( R.A Approved) |  |  |  |
|  | 44.0 Rft | 3” dia Suction | 385/89 | P/Rft | 16,979/- |
|  | 21.00 Rft | 4” dia Delivery | 506/60 | P/Rft | 10,639/- |

(Cont: on page No:08)

(Page No:08)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 12 |  | Manufacturing and supplying M.S. flange made out of M.S. sheet 3/8" drilling holes and welding etc complete (R.A.Based on schedule) |  |  |  |
|  | 8 Nos: | 3” dia suction | 500/89 | Each | 4,007/- |
|  | 6 Nos: | 4” dia delivery | 596/75 | Each | 3,581/- |
| 13 |  | Jointing C.IM.S flanged pipes and specials fanged and inside a trench i/c supplying rubber packing of the required thickness nuts, bolts with washers etc ,and other tools required for jointing and testing the joints to the specified pressure etc complete. (P.H.S.I.NO: 1,P/40). |  |  |  |
|  | 6 Nos: | 3” dia | 499/- | P/joint | 7,984/- |
|  | 8 Nos: | 4” dia | 513/- | P/Joint | 4,104/- |
|  |  |  |  | Total | 14,63,068/- |
|  |  | Group ‘F’ Providing Laying Jointing and Testing PVC Rising Main 6” dia |  |  |  |
| 01 | 7800.0 Cft | Excavation for pipe lines into trenches and pits in Soft soil i/c trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer incharge providing fence guards lights flags and temporary crossing for non vehicular traffic where ever required lift upto 5’ft(1.52m) and lead upto one chain (30.5mm) (P.H.S.I.NO; 1, P/60) | 3600/- | P%0cft | 28,080/- |
| 02 |  | Providing PVC pipe of class ‘B’ (equivalent make)fixing in trenches i/c cutting and jointing with “Z” joints with one rubber rings i/c testing with water to head of 61 meter or 200’ft.(P.H.S.I.NO;1,P/21) |  |  |  |
|  | 1300.0 Rft | 4” dia | 137/- | P/Rft | 1,78,100/- |
| 03 |  | P.V.C ‘Z’ Joints fitting (P.H.S(M),16 P/27) |  |  |  |
|  | 2 Nos: | 4” dia | 956/- | Each | 1,912/- |
| 04 |  | P.V.C tee (P.H.S(M)I-P/112) |  |  |  |
|  | 1 No; | 4” dia | 1181/- | Each | 1,181/- |
| 05 | 2 Nos: | P.V.C. Socket 4” dia | 1125/- | Each | 2,250/- |

(Cont: on page No;09)

(Page No:09)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 06 |  | Making joints C.I. specials fitting (i/c laying of specials and cost of solvent cement of required diameter and testing the joint along with PVC pipe line “B” Class and “D” Class of the specified pressure and making good to all leaky joints etc: complete. (P.H.S.I.NO:1, P/41) |  |  |  |
|  | 6 Nos: | 4” dia | 70/- | Each | 420/- |
| 07 |  | Manufacturing & Supplying M.S Pipe 4” Dia made out of m.s sheet 3/16” Drilling holes and welding correct curvature etc:complete (R.A.Based on schedule item) |  |  |  |
|  | 70.0 Rft | 4” dia | 506/60 | P/Rft | 35,462/- |
| 08 |  | Manufacturing & Supplying M.S Flange 4” Dia made out of m.s sheet 3/8” Drilling holes and welding correct curvature etc: complete (R.A.Approved) |  |  |  |
|  | 7 Nos: | 4” dia | 596/75 | Each | 4,177/- |
| 09 | 38.20 Kg: | C.I Bend with flanged and with holes i/c turning facing for all size (P.H.S.I.NO;8, P/13) | 120/- | P/Kg: | 4,584/- |
| 10 | 7020.0 Cft | Refilling the excavated stuff in trenches 6” thick layers i/c watering ramming to full compaction etc: complete (P.H.S.I.NO;24, P/53) | 2760/- | P%0cft | 19,375/- |
|  |  |  |  | Total | 2,75,541/- |
|  |  | Group ‘G’ Construction of Compound Wall. |  |  |  |
| 01 | 1384.0 cft | Excavation in foundation of building bridges and other structure including dag belling dressing, refilling arround structure with excavated earth watering ramming lift upto 5ft and lead upto one chain (G.S.I.NO: 18 P/4 ) | 3176/25 | P%0cft | 4,396/- |
| 02 | 346.0 cft | Cement concrete brick or stone ballast 1:1/2" to 2" guage (Ratio 1:4:8) (GSI.No.4/b P/14) | 9416/28 | P%cft | 32,580/- |
| 03 | 2359.0 Sft | Errection and removal of centering for R.C.C or plain cement concrete Works of Deodar wood(2nd Class)(Vertical) (G.S.I:No:19(ii),P/18). | 3127/41 | P%sft | 73,776/- |
| 04 | 1179.50 Cft | Cement concrete plain i/c placing compacting, finishing & complete etc (Ratio 1:3;6) (G.S.I.No 5(h) P/ 14). | 12595/- | P%sft | 1,48,558/- |
| 05 | 554.0 Cft | Pacce Brick work in other than building i/c stricking of joints in cement sand mortor (1;6) upto 20’ft height. (G.S.I.NO;7, P/22) | 12346/65 | P%Cft | 68,400/- |

(Cont: on page No;10)

(Page No:10)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 06 | 78.00 Cft | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kinds of forms moulds, lifting, shuttering currying rendering and finishing the exposed surface)(including screening and washing of shingle)(a)R.C. Work in roof slab, beams, columns, rafts, lintels and other structural members laid in situe or precast laid in position complete in all respect(i)Ratio 1:2:4)90 lbs cement 2 cft sand 4 cft shingle 1/8” to ¼” gauge.(G.S.I.NO;6(a)(i)P/17) | 337/- | P/cft | 26,286/- |
| 07 | 3.13 Cwt | Fabrication of mild steel reinforcement for cement concrete including cutting bending laying in position making joints and fastenings including cost of binding wire (also includes removal of rust from bars (G.S.I.NO;7(ii)(a)P/20) | 5001/70 | P/Cwt | 15,655/- |
| 08 | 3055.0 Sft | Cement Plaster (1:4) up to 12’ height ½” thick(G.S.I.No:11(b),P/51). | 2283/93 | P%sft | 69,774/- |
| 09 | 50.00 Sft | Making & fixing steel grated door with 1/16" thick sheeting including angle iron frame 2"x2"x3/8" and 3/4" square bars 4" centre to centre with locking arrangement (GSI.No.24 P/91) | 726/72 | P/Sft | 36,336/- |
| 10 | 2920.00 Sft | Priming coat of chalk distemper(G.S.I.No:23,P53). | 442/75 | P%sft | 12,928/- |
| 11 | 2920.0 Sft | Distempering (Two coats) G.S.I.No:24(a),P54). | 1043/90 | P%sft | 30,482/- |
| 12 | 100.0 Sft | Painting new surface:- (c ) preparing surface and painting of doors and windows any type, (including edges ) (GSI.No.5/c( I & II) P/69) | 2116/41 | P%sft | 2,116/- |
|  |  |  |  | Total | 5,21,287/- |

(cont: on page No:11)

(Page No:11)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
|  |  | Group ‘H’ Providing and Installing Solar Panel System 7.50 BHP & 10 BHP |  |  |  |
|  |  | 7.50 BHP |  |  |  |
| 01 | 1 set | 1.Solar Panels 325 w high efficiency cell 20 years warranty cell poly crystal line, providing, installing, testing, connection and commissioning of solar PV modules, cell type polyerstalling, PV module efficiency 16-17% to design 7.50 KW solar (to run 7.50 BHP Motor) PV arrat modules cells must be protected with IP65 compliance and must confirm to the latest edition of Q sele (made to Germany) and international standards with all accessories as required upto satisfaction of the Engineer incharge. 2.Solar mounting MS framing providing installing connecting testing and 10 BHP PV arrays. The structure design shall be designed to sustain worst weather connection. 3.Solar cable DC flexible 10 mm pure copper for whole system supply connection and termination of DC cable from PV modules of inverter to motor W.r.t system ratings volt’s / ampers copper gird cable to be laid in PVC conduit across the soute with the all accessories and material required from completion of job 4.Solar inverter VFD providing, installing testing and commissioning of (Sun indisen made in Germany) converter VFD for 10 K.W array, 3 Phase AC output voltage 440V, 50 HZ efficiency 97& or higher completer with reverse current protection, DC over voltage protection, power control module, gird monitoring and AC/DC commectors for input output voltage complete with all accessories as required upto satisfaction of the Engineer incharge. 5.Breakers and breaker box (Terasaki made in Germany) 6 Junction Box 7 Lux, Ties Nut Bolt 8 PVC cable protection pipe bend for cable 9 Transportation of Goods. 10 Installation of the system | 1710000/- | P/Set | 17,10,000/- |
|  |  |  |  | Total | 17,100,00/- |

(Cont: on page No:12)

(Page No:12)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
|  |  | 10 BHP |  |  |  |
| 01 | 1 set | 1.Solar Panels 325 w high efficiency cell 20 years warranty cell poly crystal line, providing, installing, testing, connection and commissioning of solar PV modules, cell type polyerstalling, PV module efficiency 16-17% to design 10 KW solar (to run10 BHP Motor) PV arrat modules cells must be protected with IP65 compliance and must confirm to the latest edition of Q sele (made to Germany) and international standards with all accessories as required upto satisfaction of the Engineer incharge. 2.Solar mounting MS framing providing installing connecting testing and 10 BHP PV arrays. The structure design shall be designed to sustain worst weather connection. 3.Solar cable DC flexible 10 mm pure copper for whole system supply connection and termination of DC cable from PV modules of inverter to motor W.r.t system ratings volt’s / ampers copper gird cable to be laid in PVC conduit across the soute with the all accessories and material required from completion of job 4.Solar inverter VFD providing, installing testing and commissioning of (Sun indisen made in Germany) converter VFD for 12 K.W array, 3 Phase AC output voltage 440V, 50 HZ efficiency 97%& or higher completer with reverse current protection, DC over voltage protection, power control module, gird monitoring and AC/DC commectors for input output voltage complete with all accessories as required upto satisfaction of the Engineer incharge. 5.Breakers and breaker box (Terasaki made in Germany) 6 Junction Box 7 Lux, Ties Nut Bolt 8 PVC cable protection pipe bend for cable 9 Transportation of Goods. 10 Installation of the system | 1980000/- | P/Set | 19,80,000/- |
|  |  |  |  | Total | 19,80,000/- |

(Cont: on page No:13)

(Page No:13)

A B S T R A C T

| Group | Schedule Item | Non Schedule Item | Total |
| --- | --- | --- | --- |
| ‘A’ Constt: Open surface drains Type I,II,III & C.C Block | 64,05,103/- | ---- | 64,05,103/- |
| ‘B’ Construction of R.C.C. Collecting tank 15’ dia | 6,80,177/- | ----- | 6,80,177/- |
| ‘C’ Construction of Screening chamber Size 5’ x 8’ | 1,64,038/- | ----- | 1,64,038/- |
| ‘D’ Construction of Pump House | 5,86,300/- | ------ | 5,86,300/- |
| ‘E’ Supplying & Installing Pumping Machinery Electric motor 10BHP & Diesel Engine 16 BHP | 1,24,625/- | 13,35,830/- | 14,60,455/- |
| ‘F’ Providing Laying Jointing and testing PVC Pipe Rising Main 6” dia | 2,75,541/- | ------ | 2,75,541/- |
| ‘G’ Construction of Compound Wall. | 5,21,287/- | ------- | 5,21,287/- |
| ‘H’ Providing & Installing Solar Panel system | ----- | 33,00,000/- | 33,00,000/- |
| Total | 87,57,071/- | 46,35,830/- | 1,33,92,901/- |

Amount Total (a) + (b) Rs:___________________

________% above / below on the rates of CSR, Amount to be added / deducted on the basis

Total (b) of premium quoted ______________

Total (c ) offered for Non-schedule items_________

Total (A) = a + b in works & Figures______________________________________________

Contractor 						 	EXECUTIVE ENGINEER

PUBLIC HEALTH ENGG:DIVISION

JAMSHORO