5ILL OF QUANTITIES

NAME OF WORK:-	Water Supply and Drainage Scheme Pacca Wara Radhan	Taluka 			Sehwan Sharif) ( District Jamshoro).

(A) 	Description and rate of item based on Composite schedule of Rates

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
|  |  | GROUP ‘A’ PROVIDING INSTALLING TUBE WELL |  |  |  |
| 01 | 10 Tons | Carriage of steel G.I. and C.I. Pipes steel, G.I. F.C. and Aluminium sheets boring equipment and other heavy material i/c fitting by mechanical transport i/c loading un loading and stacking at site Hyderabad to site of work (Lead 109 miles) | 824/92 | P/Ton | 8,249/- |
| 02 |  | Boring tube well in water bearing soil from G.I. upto 100’ft depth i/c sinking and with drawing of cashing pipe (P.H.S.I.NO:1, P/36) |  |  |  |
|  | 65.0 Rft | 12” dia | 743/- | P/Rft | 48,295/- |
| 03 | 1 No: | Providing strong substantially build wooden locked box with compartments for preserving 2 lbs (1 kg) samples of starts (P.H.S.I.NO:5, P/43) | 3530/- | Each | 3,530/- |
| 04 | 5 No: | Taking & preserving in box 2 lbs (1 kg) sample of starts from bore holes (P.H.S.INO:6, P/43) | 101/- | Each | 505/- |
| 05 | 5 Samples | Taking samples of water from bore holes and placing in two separate bottles. (P.H.S.I.NO: 7, P/43) | 131/- | P/Sample | 655/- |
| 06 |  | Supply and installing PVC Blind pipe “B” Class of approved design quality and make i/c necessary socket etc: complete. (P.H.S.O.M.I.NO;12 P/44) |  |  |  |
|  | 39.0 Rft | 8” dia | 516/80 | P/Rft | 20,155/- |
| 07 |  | Supplying and installing PVC strainer ‘B’ Class of approved design quality and make i/c necessary socket etc: complete. (P.H.S.O.M.I.NO;9, P/ 43) |  |  |  |
|  | 26.0 Rft | 8” dia | 554/05 | P/Rft | 14,405/- |
| 08 | 65.0 Rft | Shrouding with graded bajri (3/8” to 1/8” or 9 to 3 mm) in between bore and blind pipe for the following diameter or Strainer (P.H.S.I.NO:13, P/45) | 105/- | P/Rft | 6,825/- |
| 09 | 65.0 Rft | Development of tube well. (PHSINO:8(a),P-43). | 200/- | P/Rft | 13,000/- |
| 10 | 1 No: | M.S. Bail of approved quality (8mm thick)2.6 long (P.H.S.I.Sch:-8 (b) P/43) | 2607/- | Each | 2,607/- |
| 11 | 1 No: | Construct concrete 1:2:4 around well hand casing complete (R.A. Approved) | 1285/- | Each | 1,285/- |
|  |  |  |  | Total | 1,19,511/- |
|  |  | Group‘B’ Construction of Pump House. |  |  |  |
| 01 | 209.0 Cft | Excavation in foundation of building bridges and other structures including dabbling dressing refilling around structures with excavated earth watering and ramming lead upto 5’ft (in ordinary soil) (G.S.I.NO;18,. P/4) | 3176/25 | P%0cft | 664/- |
| 02 | 66.00 Cft | Cement concrete brick or stone ballast 1½” to 2” gauge Ratio 1:4:8) (G.S.I.NO;4(b)P/16) | 9416/28 | P%cft | 6,215/- |

(Cont: on page No;02)

(Page No:02)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 03 | 245.00 Sft | Errection and removal of centering for R.C.C or plain cement concrete works of deodar wood)2nd class)(for partal wood) (Vertical)(G.S.I.NO;19,(b) P/17) | 3127/41 | P%sft | 7,662/- |
| 04 | 136.50 Cft | Cement concrete plain including placing compacting finishing and curring complete including screening and washing of stone aggregate without shuttering (Ratio 1:3:6)(G.S.I.NO;5(c)P/15) | 12595/- | P%cft | 17,192/- |
| 05 | 90.00 Cft | Filling watering and ramming earth under floor with surplus earth (Excavated from out side)Lead upto one chain and lift upto 5’ft(G.S.I.NO;22, P/44) | 1512/50 | P%0cft | 136/- |
| 06 | 217.0 Cft | Pacca brick work ground floor including striking of joints Up to 20’ ft height in cement sand mortar (1:6) (GSINO:I-e,P-21) | 12674/36 | P%cft | 27,503/- |
| 07 | 72.00 Cft | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kinds of forms moulds, lifting, shuttering currying rendering and finishing the exposed surface)(including screening and washing of shingle)(a)R.C. Work in roof slab, beams, columns, rafts, lintels and other structural members laid in situe or precast laid in position complete in all respect(i)Ratio 1:2:4)90 lbs cement 2 cft sand 4 cft shingle 1/8” to ¼” gauge.(G.S.I.NO;6(a)(i)P/17) | 337/- | P/cft | 24,264/- |
| 08 | 2.89 Cwt | Fabrication of mild steel reinforcement for cement concrete including cutting bending laying in position making joints and fastenings including cost of binding wire (also includes removal of rust from bars (G.S.I.NO;7(ii)(a)P/20) | 5001/70 | P/Cwt | 14,455/- |
| 09 | 64.00 Sft | Laying white marble flooring fine dressed on the surface without winding set in lime mortar 1:2 including rubber and polishing of the joint (G.S.I.NO;28, P/43)3/4” thick | 567/48 | P/Sft | 36,319/- |
| 10 | 554.25 sft | Cement plaster (1:6) ½” thick upto 20’ft height (G.S.I.NO;11(b) P/58) | 2206/60 | P%sft | 12,230/- |
| 11 | 554.25 Sft | Cement plaster (1:4) 3/8” thick upto 12’ft height (G.S.I.NO;11(b) P/58) | 2197/52 | P%sft | 12,180/- |
| 12 | 28.00 Sft | Making and fixing steel grated door with 1/16” thick sheeting i/c angle iron frame2”x2”x3/8” and ¾” square bars 4” center to center with locking arrangement(G.S.I.NO;28, P/97) | 726/72 | P/sft | 20,348/- |
| 13 | 3.05 Cwt | Fabrication of heavy steel work with angles tees, flat iron , round iron and sheet iron for making trusses girdes tands etc including cutting tanks etc.i/c cutting drilling iverting handling assembling and fixing but excluding erection in position (G.S.I.No.2,P-91) | 4928/49 | P/Cwt | 15,032/- |
| 14 | 3.05 Cwt | Errection and fitting in position of iron trusses staging of water tanks etc: (G.S.I.NO;3, P/96) | 271/04 | P/cwt | 827/- |

(Cont: on page No;03)

(Page No:03)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 15 | 128.0 Sft | PAINTING NEW SURFACE. Preparing surface and painting of doors and windows in any type (3 coats) (G.S.I.NO;5P/69) | 2116/41 | P%sft | 2,709/- |
| 16 | 312.0 Sft | On coats of chalk (G.S.I.NO;23, P/54) | 442/75 | P%sft | 1,381/- |
| 17 | 742.75 Sft | Distempering (2 coats) (G.S.I.NO;24, P/54) | 1043/90 | P%Sft | 7,754/- |
|  |  | Cost for 2 Nos: 206871x2=4,13,742/- |  | Total | 2,13,369/- |
|  |  | Group ‘C’ Providing Installing Submersible Pump 5.50 BHP |  |  |  |
| 01 | 1 Set | Providing & installing position i/c transportation to the site of work at village Pacca Wara radhan Taulka Sehwan Sharif Golden saer NS 956724 submersible pump 7.50HP Motor complete capable to discharge 60 G.P.M. against a head of 70 ft 5.50 HP (3x400Vx2 pole) i/c M.S. riser pipe of 3” dia 8 Nos: of 10’ft: each with flanged ends. The flanges are no touched to accommodate the cable i/c of 3” dia containing bore hole cover plate with 90 bend sluice valve jacket, nuts and bolts and 2 sets of installation clamps 7.50 HP motor control unit consist of metallic box circuit breaker. Am-Meter selector switch volt meter volt meter voltage relay (EVR)dry running protection relay indication lamps and off switch i/c Electric and cable 1x10mmxas per 3core meter cable clamps 45 Nos: cable termination kit etc: complete. | 453000/- | P/Set | 4,53,000/- |
| 02 |  | Providing G.I. Pipe of approved quality to be tested with pressure head of 200fT (S.S.I.No.P/5 ) |  |  |  |
|  | 30.0 Rft | 4” dia | 431/93 | P/Rft | 12,958/- |
| 03 |  | Manufacturing supplying M.S. flange of 4” dia made out Of M.S. Steel of 3/8” thick i/c cutting drilling holes and welding Of flange with M.S pipe etc R.A) base on sch;) |  |  |  |
|  | 6 Nos; | 4” dia | 596/75 | Each | 3,581/- |
| 04 | 5.25 Kg: | C.I. Tapper flate bottomed or central tapered flanged ends with holes Including turning and facing turning and facing of flanges for all size (P.H.S.I.M.I.NO;8, P/13) | 120/- | P/Kg: | 630/- |
| 05 |  | Reflex valve heavy pattern test pressure 300 lbs/sq; inch (P.H.S.(M)I-16, P/27) |  |  |  |
|  | 1 No: | 4” dia | 1706/25 | Each | 1,706/- |
| 06 |  | C.I Sluice valve heavy pattern test pressure 300 lbs/sq; inch (P.H.S.(M)I-16, P/12) |  |  |  |
|  | 1 No: | 4” dia | 5460/- | Each | 5,460/- |
| 07 | 28.65 Kg: | Bend with flanged and work i/c jointing and fixing pf flanges for all size (PHSM.I.No.8,P-12) | 120/- | P/Kg: | 3,438/- |
| 08 |  | Jointing C.I/M.S flanged pipes and specials flanged and inside a trenches i/c supplying rubber packing of the required thickness nuts bolts with washers etc and other tools required for jointing and testing the joints to the specified pressure etc: complete. (P.H.S.I.NO;1, P/40) |  |  |  |
|  | 9 Joints | 4” dia | 513/- | P/joint | 4,617/- |
|  |  | (Cont: on page no:04) |  | Total | 4,86,513/- |

(Page No:04)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
|  |  | GROUP ‘D’ SUPPLYING AND INSTALLING PUMPING MACHINERY 7.50 BHP |  |  |  |
| 01 | 2 sets | Supplying & installing in position i/c transportation to site of work at Pacca wara Radhan A.C Electric pumping set consisting of A.C. Electric motor 7.50 BHP 3 Phase 50 cycles 400/420 volts of 2950 RPM couple with KSB pump Type 40-200 size 2”x2½” mounted on common steel frame or base plate capable of discharge 70 gallons/ minute against a head of 150’ ft: i/c installing of pumping set on Cement concrete foundation ratio 1:2;4 and C.C. (1:4:8) with ballast with M.S. nuts and bolts of 5/8” dia to be embedded in C.C. Foundation i/c providing and fixing on prepare board A.C 50 Cycles 3 phase 12/25 ampere trips rating push button type star delta starter complete with no volts and over load release and required i/c providing & fixing on prepared board iron (clad) triple pole and neutral switch fuse with rewire able type fuse link carried 500 volts grade 30 ampere providing and fixing volts meter 0-500 volts and ampere meter 0.50 Amps: installed on Metal led board i/c providing and fixing Metalled board to accommodate main or sub main control switch in the wall size 3’x2 ½” i/c providing phase indication bulbs holder and providing internal Electric wiring for main with 3-7/1.04 7/0.44 P.V.C(RA Attached) | 245020/- | P/Set | 4,90,040/- |
| 02 | 15.0 Meter | Providing unuarmoured cable 4 core PVC/PVC Copper conductor 660/1000 volts (Elect: S.I.NO;1 (2& 3) P/10) | 2495/- | P/Meter | 37,425/- |
| 03 |  | C.I Sluice valve heavy pattern test pressure 300 lbs/sq; inch (P.H.S.(M)I-16, P/97) |  |  |  |
|  | 2 Nos: | 3” dia | 4290/- | Each | 8,580/- |
| 04 |  | Reflex valve heavy pattern test pressure 300 lbs/sq; inch (P.H.S.(M)I-02, P/12) |  |  |  |
|  | 2 Nos: | 3” dia | 1543/75 | Each | 3,088/- |
| 05 | 26.25 Kg: | C.I. Tapper flate bottomed or central tapered flanged ends with holes Including turning and facing turning and facing of flanges for all size (P.H.S.I.M.I.NO;8, P/13) | 120/- | P/Kg: | 3,150/- |
| 06 |  | Foot valve heavy pattern with cone type gate (imported ) (M.S.I.NO;4 d P/11) |  |  |  |
|  | 2 Nos: | 3” dia | 731/25 | Each | 1,463/- |
| 07 |  | C.I short piece (S.M.I.NO;9, P/15) |  |  |  |
|  | 1 No; | 4” dia | 487/50 | Each | 488/- |
| 08 | 11.45 Kg; | C.I. Tee flanged end with holes, including turning and facing of flanges for all size (M.S.INO;12, P/12) | 120/- | Each | 1,374/- |
| 09 | 42.01 Kg: | C.I. Bend with flanged and work i/c jointing and fixing pf flanges for All size (M.S.I.No.8,P-12) | 120/- | Each | 5,041/- |
| 10 |  | Manufacturing and supplying M.S. Pipe 6” dia made out of M.S. Sheet 3/8” drilling holes and welding currect curvature etc: complete (Based on schedule ) |  |  |  |
|  | 44.0 Rft | 4” dia | 385/89 | P/Rft | 16,979/- |
|  | 21.0 Rft | 6” dia | 506/60 | P/Rft | 10,639/- |

(cont: on page No:05)

(Page No:05)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 11 |  | Manufacturing and supplying M.S. Flanged 6” dia made out of M.S. Sheet 3/8” drilling holes and welding etc: complete (P.SH.I.NO; P/ ) |  |  |  |
|  | 08 Nos: | 3” dia | 500/89 | Each | 4,007/- |
|  | 6 Nos: | 4” dia | 596/75 | Each | 3,581/- |
| 12 |  | Jointing C.I/M.S flanged pipes and specials flanged and inside a trenches i/c supplying rubber packing of the required thickness nuts bolts with washers etc and other tools required for jointing and testing the joints to the specified pressure etc: complete. (P.H.S.I.NO;1, P/40) |  |  |  |
|  | 16 Joints | 3” dia | 499/- | P/joint | 7,984/- |
|  | 8 Nos: | 4” dia | 513/- | P/Joint | 4,104/- |
|  |  |  |  | Total | 5,97,944/- |
|  |  | GROUP ‘E’ CONSTT: OF CLEAR WATER TANK. |  |  |  |
|  |  | CLEAR WATER TANK 20’ DIA |  |  |  |
| 01 | 3077.20 Cft | Excavation for tanks and reservoirs in soft soils i/c trimming and Dressing sides to true alignment / design section /profiles and shape leveling of beds of trenches to correct level and grade i/c laying of earth in 6’ layers construction of banks and dressing disposal of surplus earth . Excavated earth within one chain as directed by Engineer In charge. i /c providing fence guards lights, flags & temporary crossing for non vehicular traffic, where ever required lift up to 5’ ft (1.52m) and lead up to one chain. (30.5m) (P.H.S.I.No:1,P/67). | 3000/- | P%0cft | 9,232/- |
| 02 | 3384.92 Cft | Excavation for tanks and reservoirs in wet soils clay or mud i/c trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade, cutting joints holes earth and disposal of surplus within one chain as directed by Engineer In charge. Providing fence guards lights, flags & temporary crossing for non vehicular traffic, where ever required lift up to 5’ ft (1.52m) and lead up to one chain.(30.5m). (P.H.S.I.No:15,P/74). | 5350/- | P%0cft | 18,109/- |
| 03 | 226.08 Cft | Cement concrete brick or stone ballast 1:1/2" to 2" guage (Ratio 1:4:8) (GSI.No.4/b P/15). | 9416/28 | P%cft | 21,288/- |
| 04 | 1402.00 Cft | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kinds of forms moulds, lifting, shuttering currying rendering and finishing the exposed surface)(including screening and washing of shingle)(a)R.C. Work in roof slab, beams, columns, rafts, lintels and other structural members laid in situe or precast laid in position complete in all respect(i)Ratio 1:2:4)90 lbs cement 2 cft sand 4 cft shingle 1/8” to ¼” gauge.(G.S.I.NO;6(a)(i)P/17) | 337/- | P/cft | 472,474/- |

(cont: on page No:06)

(Page No:06)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 05 | 62.58 Cwt | Fabrication of mild steel reinforcement for cement concrete including cutting bending laying in position making joints and fastenings including cost of binding wire (also includes removal of rust from bars (G.S.I.NO;7(ii)(a)P/20) | 5001/70 | P/Cwt | 3,13,006/- |
| 06 | 197.82 Rft | Providing “Construction joint” in concrete work 9” wide corrugated P.V.C. water stop with bulb i/c soldering cost of material and labour etc: complete (P.H.S.I.NO;2, P/56) | 86/- | p/Rft | 17,013/- |
| 07 | 812.00 Sft | Cement plaster (1:4) upto 12' ft: height 1/2" thick (GSI.No.11/b P/52). | 2283/93 | P%sft | 18,545/- |
| 08 | 28.00 Rft | Providing in position in C.C. foundation 1:3:6 M.S. ladder of M.S. angle iron 2”x2”x1/4” 1' apart with M.S. bar 3/4" dia spaced 1' apart painting 3 coats with anticorrosive steel paint etc complete. (P.H.S.I.NO: 3 P/55) | 799/- | P/Rft | 22,372/- |
| 09 | 3.50 Cwt | Providing and fixing C.I.Man hole with frame i/c cost of material etc complete (G.S.I.No. J/I P/35) | 6985/- | P/Cwt | 24,447/- |
| 10 | 53.38 Cft | Cement concrete plain including placing compacting finishing and curring complete including screening and washing of stone aggregate without shuttering (Ratio 1:2:4)(G.S.I.NO;5(c)P/15) | 14429/25 | P%cft | 7,702/- |
| 11 | 2 Nos; | Providing C.I Ventilator (weight not less than 80 lbs/each) in tank roof of approved design with water tight joint i/c painting etc: (P.H.SI.NO.4 P/55) | 4870/- | Each | 9,740/- |
|  |  |  |  | Total | 9,33,930/- |
|  |  | Group ‘F’ P/L/J and Testing Distb: system 4”, 3” dia. |  |  |  |
| 01 | 150.00 Cft | Dismantling and removing road (G.S.I.NO;5, P/14) | 605/- | P%cft | 908/- |
| 02 | 41850.0 Cft | Excavation for pipe lines into trenches and pits in Soft soil i/c trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer incharge providing fence guards lights flags and temporary crossing for non vehicular traffic where ever required lift upto 5’ft(1.52m) and lead upto one chain (30.5mm) (P.H.S.I.NO; 1, P/60) | 3600/- | P%0cft | 1,50,660/- |
| 03 |  | Providing PVC pipe of class ‘B’ (equivalent make)fixing in trenches i/c cutting and jointing with “Z” joints with one rubber rings i/c testing with water to head of 61 meter or 200’ft. (P.H.S.I.NO;1,P/21) |  |  |  |
|  | 7000.0 Rft | 4” dia | 137/- | P/Rft | 9,59,000/- |

(Cont: on page No:07)

(Page No:07)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 04 |  | P.V.C ‘Z’ Joints fitting (P.H.S(M),16 P/28) |  |  |  |
|  |  | i)P.V.C. Bend |  |  |  |
|  | 02 Nos: | 4” dia | 956/- | Each | 1,912/- |
| 05 |  | P.V.C. Socket (P.H.S.(M)O-14, P/29) |  |  |  |
|  | 4 Nos: | 4” dia | 1125/- | Each | 4,500/- |
| 06 |  | C.I Sluice valve heavy pattern test pressure 300 lbs/sq; inch (P.H.S.(M)I-16, P/97) |  |  |  |
|  | 02 Nos: | 4” dia | 5460/- | Each | 10,920/- |
| 07 |  | C.I. Tail piece for PVC (P.H.S.(M)I-09,P/15) |  |  |  |
|  | 04 Nos: | 4” dia | 487/50 | Each | 1,950/- |
| 08 |  | PVC Tee 4”x4”x3” (P.H.S.M.I.NO;18 , P/27) |  |  |  |
|  | 2 Nos: | 4” dia | 1181/- | Each | 2,362/- |
| 09 |  | Making joints C.I. specials fitting (i/c laying of specials and cost of solvent cement of required diameter and testing the joint along with PVC pipe line “B” Class and “D” Class of the specified pressure and making good to all leaky joints etc: complete.(P.H.S.I.NO:1, P/53) |  |  |  |
|  | 06 Nos: | 4” dia | 70/- | Each | 420/- |
| 10 |  | Jointing C.I/M.S flanged pipes and specials flanged and inside a trenches i/c supplying rubber packing of the required thickness nuts bolts with washers etc and other tools required for jointing and testing the joints to the specified pressureetc: complete. (P.H.S.I.NO;1, P/40) |  |  |  |
|  | 04 Joints | 4” dia | 513/- | Each | 2,052/- |
| 11 | 02 Nos: | Providing Chamber 3'x2' (915x615 mm) in dimension 4-1/2' (1372mm) deep as per approved design for sluice valve to 12"dia with 18" (457mm) dia inside cost iron cover and frame (wt= 1 cwt 3 sq) fixed in RCC 1:2:4 (102mm) thick (with 5 lbs steel per cft) 9" (299 mm) thick brick masonary wall set in 1:6 cement mortor 6" (1152 mm) thick cement concrete 1:3:6 in foundation 1" (25mm) thick cement concrete 1:2:4 flooring 1/2" (12.50 mm) thick cement plaster 1:3 to all inside wall surface and to top i/c providing and fixing M.S. foot rest at every one foot beyond 2-1/2 ft depth curing, excavation, back filling and disposal of surplus earth etc complete (PHS. I.No.Q-1 P/ 49 ) | 18820/- | Each | 37,640/- |

(Cont: on page No:08)

(Page No:08)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 12 | 37800.0 Cft | Refilling the excavated stuff in trenches 6” thick layers i/c watering ramming to full compaction etc: complete (P.H.S.I.NO;24,P/77 | 2760/- | P%0cft | 1,04,328/- |
|  |  |  |  | Total; | 12,76,652/- |
|  |  | Group ‘G’ P/L/J and Testing Distb: system 4”, 3” dia. |  |  |  |
| 01 | 1500.0 Cft | Dismantling and removing road (G.S.I.NO;5, P/14) | 605/- | P%cft | 9,075/- |
| 02 |  | Dismantling cement concrete plain (G.S.I.NO;19 ( c ) P/11) |  |  |  |
|  | 250.0 Cft | Ratio 1:2:4 | 3327/50 | P%cft | 8,319/- |
|  | 250.00 Cft | Ratio 1:4:8 | 1663/75 | P%cft | 4,159/- |
| 03 | 46750.0 Cft | Excavation for pipe lines into trenches and pits in Soft soil i/c trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer incharge providing fence guards lights flags and temporary crossing for non vehicular traffic where ever required lift upto 5’ft(1.52m) and lead upto one chain (30.5mm) (P.H.S.I.NO; 1, P/60) | 3600/- | P%0cft | 1,68,300/- |
| 04 |  | Providing PVC pipe of class ‘B’ (equivalent make)fixing in trenches i/c cutting and jointing with “Z” joints with one rubber rings i/c testing with water to head of 61 meter or 200’ft. (P.H.S.I.NO;1,P/21) |  |  |  |
|  | 2000.0Rft | 4” dia | 137/- | P/Rft | 2,74,000/- |
|  | 6125.0 Rft | 3” dia | 90/- | P/Rft | 5,51,250/- |
| 05 |  | P.V.C ‘Z’ Joints fitting (P.H.S(M),16 P/28) |  |  |  |
|  |  | i)P.V.C. Bend |  |  |  |
|  | 02 Nos: | 4” dia | 956/- | Each | 1,912/- |
|  | 12 Nos: | 3” dia | 731/- | Each | 8,772/- |
| 06 |  | PVC Tee |  |  |  |
|  | 04 Nos: | 4” dia | 1181/- | Each | 4,724/- |
|  | 16 Nos: | 3” dia | 900/- | Each | 14,400/- |
| 07 |  | P.V.C. Socket (P.H.S.(M)O-14, P/29) |  |  |  |
|  | 06Nos: | 4” dia | 1125/- | Each | 6,750/- |
|  | 25 Nos: | 3” dia | 405/- | Each | 10,125/- |
| 08 |  | C.I Sluice valve heavy pattern test pressure 300 lbs/sq; inch (P.H.S.(M)I-16, P/97) |  |  |  |
|  | 08 Nos: | 4” dia | 5460/- | Each | 43,680/- |
|  | 10 Nos: | 3” dia | 4290/- | Each | 42,900/- |
| 09 |  | C.I. Tail piece for PVC (P.H.S.(M)I-14,P/29) |  |  |  |
|  | 16 Nos: | 4” dia | 487/50 | Each | 7,800/- |
|  | 20 Nos: | 3” dia | 375/- | Each | 7,500/- |

(Cont; on page No:09)

(Page No:09)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 10 |  | Making joints C.I. specials fitting (i/c laying of specials and cost of solvent cement of required diameter and testing the joint along with PVC pipe line “B” Class and “D” Class of the specified pressure and making good to all leaky joints etc: complete.(P.H.S.I.NO:1, P/53) |  |  |  |
|  | 10 Nos: | 4” dia | 70/- | Each | 700/- |
|  | 44 Nos: | 3” dia | 59/- | Each | 2,596/- |
| 11 |  | Jointing C.I/M.S flanged pipes and specials flanged and inside a trenches i/c supplying rubber packing of the required thickness nuts bolts with washers etc and other tools required for jointing and testing the joints to the specified pressureetc: complete. (P.H.S.I.NO;1, P/40) |  |  |  |
|  | 16 Joints | 4” dia | 513/- | Each | 8,208/- |
|  | 20 Joints | 3” dia | 499/- | Each | 9,980/- |
| 12 | 18 Nos: | Providing Chamber 3'x2' (915x615 mm) in dimension 4-1/2' (1372mm) deep as per approved design for sluice valve to 12"dia with 18" (457mm) dia inside cost iron cover and frame (wt= 1 cwt 3 sq) fixed in RCC 1:2:4 (102mm) thick (with 5 lbs steel per cft) 9" (299 mm) thick brick masonary wall set in 1:6 cement mortor 6" (1152 mm) thick cement concrete 1:3:6 in foundation 1" (25mm) thick cement concrete 1:2:4 flooring 1/2" (12.50 mm) thick cement plaster 1:3 to all inside wall surface and to top i/c providing and fixing M.S. foot rest at every one foot beyond 2-1/2 ft depth curing, excavation, back filling and disposal of surplus earth etc complete (PHS. I.No.Q-1 P/ 49 ) | 18820/- | Each | 3,38,760/- |
| 13 | 43875.00 Cft | Refilling the excavated stuff in trenches 6” thick layers i/c watering ramming to full compaction etc: complete (P.H.S.I.NO;24,P/53) | 2760/- | P%0cft | 1,21,095/- |
|  |  |  |  | Total | 16,45,005/- |

(Cont: on page No:10)

(Page No:10)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
|  |  | Group ‘H’ Providing and Installing Solar Panel System 7.50 BHP |  |  |  |
|  |  | 7.50 BHP |  |  |  |
| 01 | 1 set | 1.Solar Panels 325 w high efficiency cell 20 years warranty cell poly crystal line, providing, installing, testing, connection and commissioning of solar PV modules, cell type polyerstalling, PV module efficiency 16-17% to design 7.50 KW solar (to run 7.50 BHP Motor) PV arrat modules cells must be protected with IP65 compliance and must confirm to the latest edition of Q sele (made to Germany) and international standards with all accessories as required upto satisfaction of the Engineer incharge. 2.Solar mounting MS framing providing installing connecting testing and 10 BHP PV arrays. The structure design shall be designed to sustain worst weather connection. 3.Solar cable DC flexible 10 mm pure copper for whole system supply connection and termination of DC cable from PV modules of inverter to motor W.r.t system ratings volt’s / ampers copper gird cable to be laid in PVC conduit across the soute with the all accessories and material required from completion of job 4.Solar inverter VFD providing, installing testing and commissioning of (Sun indisen made in Germany) converter VFD for 7.50 K.W array, 3 Phase AC output voltage 440V, 50 HZ efficiency 97& or higher completer with reverse current protection, DC over voltage protection, power control module, gird monitoring and AC/DC commectors for input output voltage complete with all accessories as required upto satisfaction of the Engineer incharge. 5.Breakers and breaker box (Terasaki made in Germany) 6 Junction Box 7 Lux, Ties Nut Bolt 8 PVC cable protection pipe bend for cable 9 Transportation of Goods. 10 Installation of the system | 1710000/- | P/Set | 17,10,000/- |
|  |  |  |  | Total | 17,100,00/- |

(Cont: on page NO:11)

(Page No:11)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
|  |  | Group ‘I’ Constt: Open surface drains Type I,II,III & C.C Block |  |  |  |
| 01 | 907.50 Cft | Dismantling and removing road (G.S.INO;5,P/14) | 605/- | P%cft | 5,490/- |
| 02 | 9875.00 Cft | Excavation in foundation of building bridges and other structures including dabbling dressing refilling around structures with excavated earth watering and ramming lead upto 5’ft (in sand soil ) (G.S.I.NO;18,. P/4) | 3176/25 | P%0cft | 31,365/- |
| 03 | 2150.0 Cft | Cement concrete plain i/c placing compacting finishing and curring complete(i/c screening and washing of stone aggregate without shuttering (Ratio 1:4:8) (G.S.I.NO;5(c)p/18) | 11288/75 | P%cft | 242,708/- |
| 04 | 5663.0 Cft | Pacca brick work in foundation and plinth in cement sand mortar (Ratio 1:6) (G.S.INO;4 ( c) P/20) | 11948/36 | P%cft | 6,76,636/- |
| 05 |  | Construction of standard open drains cunette block of cement concrete 1:2:4 in situ to the design profile i/c cost of mould as per drawing i/c applying floating coat of cement 1/32” thick to the exposed face finished smooth curring etc: complete as per detailed drawing (P.H.S.I.NO;1, P/58) |  |  |  |
|  | 2500.0 Rft | TYPE ‘A’ | 94/- | P/Rft | 2,35,000/- |
|  | 850.0 Rft | TYPE ‘B’ | 174/- | P/Rft | 1,47,900/- |
| 06 | 10007.0 Sft | Cement plaster (1:4) upto 12’ft height thick ½” thick (G.S.I.NO;11( b) P/51) | 2283/93 | P%sft | 2,28,553/- |
| 07 | 355.00 Cft | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour forbending and binding which will be paid separately. This rate also includes all kinds of forms moulds, lifting, shuttering curring rendering and finishing the exposed surface)\(including screening and washing of shingle)(a)R.C. Work in roofslab, beams, columns,rafts, lintels and other structural members laid in situe or precast laid in position complete in all respect(i)Ratio 1:2:4)90 lbs cement 2 cft sand 4 cft shingle 1/8” to ¼” guage. (G.S.I.NO;6(a)(i)P/19) | 337/- | P/Cft | 1,19,635/- |
| 08 | 12.67 Cwt | Fabrication of mild steel reinforcement for cement concrete including cutting bending laying in position making joints and fastenings including cost of binding wire (also includes removal of rust from bars(G.S.I.NO;7(ii)(a)P/20) | 5001/70 | P/Cwt | 63,372/- |
| 09 | 2125.0 Cft | Barrow pit excavation undressed lead upto 100’ft(in ordinary soil )(G.S.I.NO;3(a)P/1) | 2117/50 | P%0cft | 4,500/- |

(Cont: on page No;12)

(Page NO:12)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 10 | 12000.0 Cft | Earth work compaction (a)Laying earth in 6” layers leveling dressing and watering for compaction Etc: complete (G.S.I.NO;13( b) P/3) | 354/- | P%0cft | 4,248/- |
| 11 | 2125.00 Cft | Extra for every 50’ ft additional or part there off (a) for earth work (soft ordinary hard and very hard)(G.S.I.NO:P, P/3) | 5039/- | P%0cft | 10,708/- |
| 12 | 3960.00 Cft | Cement concrete brick or stone ballast 1½” to 2” guage(Ratio 1:4:8) (G.S.I.NO;4(b)P/17) | 9416/28 | P%cft | 3,72,885/- |
| 13 | 3000.0 Cft | Cement concrete plain i/c placing compacting finishing and curring complete(i/c screening and washing of stone aggregate without shuttering Ratio (1:2:4)(G.S.I.NO;5(a)p/18) | 14429/25 | P%cft | 4,32,878/- |
|  |  |  |  | Total | 25,75,878/- |

A B S T R A C T

| Group | Schedule Item | Non Schedule Item | Total |
| --- | --- | --- | --- |
| ‘A’ Providing installing tube well | 1,11,921/- | 8,249/- | 1,20,170/- |
| ‘B’ Construction of Pump House | 4,13,742/- | ---- | 4,13,742/- |
| ‘C’ . Providing Installing Electric Submersible pump 7.50 BHP | 33,513/- | 4,53,000/- | 4,86,513/- |
| ‘D’ Supplying Installing Pumping Machinery 7.50 BHP | 1,10,222/- | 4,90,160/- | 6,00,382/- |
| ‘E’ Constt: of Clear Water tank | 9,33,930/- | ----- | 9,33,930/- |
| ‘F’ P/L/J Testing Delivery Main PVC Pipe 4” dia | 12,76,652/- | ---- | 12,76,652/- |
| ‘G’ Providing Lying Jointing and testing Delivery main PVC Pipe Distb: system 3” dia | 16,45,005/- | -------- | 16,45,005/- |
| ‘H’ Solar system | ----- | 17,10,000/- | 17,10,000/- |
| ‘I’ Constt: of Surface drains Type ‘I’, ‘ii’ & C.C. Block | 25,75,878/- | ---- | 25,75,878/- |
| Total | 61,66,933/- | 37,99,409 | 99,66,342/- |

(Cont: on page No:13)

(Page No:13)

Amount Total (a) + (b) Rs:___________________

________% above / below on the rates of CSR, Amount to be added / deducted on the basis

Total (b) of premium quoted ______________

Total (c ) offered for Non-schedule items_________

Total (A) = a + b in works & Figures______________________________________________

Contractor 						 	EXECUTIVE ENGINEER

PUBLIC HEALTH ENGG:DIVISION

JAMSHORO