Schedule B

Name of Work: - Construction of park GT road outside Al-Noor cotton factory and Ptcl

exchange office town Ubauro.

01. Excavation in foundation of building bridges and structure i/c dag

Belling dressing refilling around the structure executes earth watering and

Ramming lead up to one chain lift up to 5ft in ordinary soil

(Sch: No.8 (b) P-4)

Qty: 2677.50 Cft @ Rs: 3176.25 P%0 Cft                                                  Rs:   8503/-

02. Cement concrete plan i/c compacting finishing and complete i/c screening

and washing stone aggregate without shuttering. Ratio 1:3:6

Qty: 535.0 Cft @ Rs: 12595.0 P% Cft                                                        Rs: 67383/-

03. Pacca brick work in foundation in cement sand mortar 1:4

Qty: 2011.00 Cft @ Rs: 12501.41 P% Cft                                                      Rs: 251403/-

04. Cement concrete plan i/c compacting finishing and complete i/c screening

and washing stone aggregate without shuttering. Ratio 1:2:4

(Sch: No.5 P-16)

Qty: 200.0 Cft @ Rs: 14429.25 P% Cft                                                           Rs:  28859/-

05. Erection of centering ….

Qty: 357.00 Sft @ Rs: 3588.48 P.Sft                                                             Rs:  12811/-

06. Pacca brick work in other than building (1:4) ratio…

Qty: 453.00 Cft @ Rs: 12501.41 P% Cft                                                      Rs: 58437/

07. Cement plaster 1:4 up to 20ft heights 1/2 “thick (G.S.I No 11(b) P-52)

Qty: 2840.0 Sft @ Rs: 2283.93 P% Sft.                                                                Rs: 64864/-

06. R.C.C Work in roof, slab beams, columns, rafts lintels and other Structural

members laid in situ or precost laid in position complete in all respect.

(I) Ratio (1:2:4) 90lbs. cement 2 cft. Sand 4 cft. Shingle 1/8x1/4 Gauge

(Sch: No.6 (A) P-17)

Qty: 82.80 Cft @ Rs: 337.0 P.Cft                                                                     Rs: 10110/-

07. Fabrication of Mild steel reinforcement for cement concrete i/c cutting,

Laying in position making joined and fastening i/c cost binding wire.

Qty: 3.69 Cwt @ Rs: 5001.70 P.Cwt                                                                      Rs:   18456/-

08. Providing & laying RCC pipe and collars of class “B” and fixing in trench i/c

Cutting , fitting  in trench jointing with .(P.H.E I no 2 © P-16)

06” dia

12.00 Rft @ Rs: 163.0 P.Rft                                                               Rs:    1956/-

09” dia

12.00 Rft @ Rs: 163.0 P.Rft                                                                Rs:    1956/-

09. Cement concrete plan i/c compacting finishing and complete i/c screening

and washing stone aggregate without shuttering. Ratio 1:2:4

(Sch: No.5 P-16)

Qty: 120.0 Cft @ Rs: 14429.25 P% Cft                                                      Rs:   17315/-

10.  Barrow pit excavation undressed lead up to 100. Ft. in ordinary soil extra 3 mile lead.

(Sch: No.3 (a) P-1)

Qty: 21600.0 Cft @ Rs: 7911.60 P%0 Cft                                                             Rs: 170891/-

11. Dressing leveling of Earth Work.

(Sch: No.11 (b)   P.03   )

Qty: 21600.0 @ Rs: 187.55 P%0 Cft                                                                       Rs:     4051/-

SUB-ENGINEER                                     ASSISTAN EXECUTIVE ENGINEER

TOWN COMMITTEE UBAURO                                TOWN COMMITTEE UBAURO

MATERIAL STATEMENT

| Sr. No | Name of Work | Qty: | Cement (Bags) | Hill Sand (Cft) | Bajri (Cft) | Bricks (Nos) | Steel (Cwt) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Part- A Surface Drain |  |  |  |  |  |  |  |
| 01. | C.C 1:4:8 | 847.0 | 80.46 | 406.56 | 813.12 | ----- | ----- |
| 02. | Pacca Brick work | 2100 | 63.00 | 539.70 | --- | 28350.0 | ---- |
| 03. | C.C 1:2:4 Golo | 1400 | 96.09 | 240.24 | 480.48 | ----- | ---- |
| 04 | Cement plaster | 2800 | 18.76 | 94.08 | ---- | ---- | ---- |
| 04. | R.CC 1:2:4 | 82.80 | 14.57 | 36.43 | 72.86 | ---- | ---- |
| 06. | Fabrication Steel | 3.69 Cwt | --- | ---- | ---- | ---- | 3.69 |
| 07. | CC plain 1:2:4 | 120.0 | 21.12 | 52.80 | 105.60 | ---- | ---- |
| Total |  |  | 294.0 | 1369.81 | 1472.06 | 28350.0 | 3.69 |
| Say |  |  | 294.0 | 1370.0 | 1472.0 | 28350.0 | 3.69 |

COST OF CARRIAGE

01. Cement Qty: 294.0 bags @ Rs: 182.73 P.Bag                    Rs:  53723/-

02. Hill Sand Qty: 1370.0 @ Rs: 3018.60 P% Cft                   Rs:   41355/-

03. Bajri Qty: 1472.0 Rs: 3980.28 P% Cft                               Rs:   58590/-

04. Bricks 28250.0 Nos @ Rs: 851.59 P%0 Nos	              Rs:    24154/-

05 Steel Qty: 3.69 Cwt @ Rs: 3316.28 P.Cwt                         Rs:        115/-

Rs: 177937/-

SUB-ENGINEER                                     ASSISTAN EXECUTIVE ENGINEER

TOWN COMMITTEE UBAURO                                TOWN COMMITTEE UBAURO

GENERAL ABSTRACT

Construction of surface drain CC and RCC slab in Jeelani colony

Town Ubauro

Part-A Surface Drain                                             Rs:  761717/-

Add 10% above Ceiling except (item no 10 &11)   + Rs:    58678/-

Rs:  820396/-

Add Carriage                                                             + Rs:  177937/-

Rs:  998333/-

Add 0.25% contingency                                               Rs:      2496/-

Total Rs: 1000829/-

Say Rs: 1000000/-

SUB-ENGINEER                                     ASSISTAN EXECUTIVE ENGINEER

TOWN COMMITTEE UBAURO                                  TOWN COMMITTEE UBAURO