Schedule B

Name of Work: - Construction of Latrine Bath rooms Rah-e-Rast near new Sabzi Market

Khanpur Mohalla Town Ubauro.     Part – A

01. Excavation in foundation of building bridges and structure i/c dag belling dressing

refilling around the structure executes earth watering and ramming lead up to one

chain lift up to 5ft in ordinary soil (Sch: No.8 (b) P-4)

Qty: 880.00 Cft @ Rs: 3176.25 P%0 Cft                                                         Rs:   2795/-

02. Cement concrete Brick or stone ballast 1½ to 2 gauge (Ratio 1:4:8)(Sch: No.4 (b) P.14)

Qty: 329.90 Cft @ Rs: 9416.28 P% Cft                                                          Rs: 31064/-

03. Pacca brick work in foundation in cement sand mortar 1:6 (Sch: No.4 (e) P=20)

Qty: 819.00 Cft @ Rs: 11948.36 P% Cft                                                          Rs: 97857/-

04. C.C Work in roof, slab beams, columns, rafts lintels and other Structural members

laid in situ or precost laid in position complete in all respect. (I) Ratio (1:2:4) 90lbs.

Qty: 208.00 Cft @ Rs: 337 P.Cft                                                                        Rs: 70096/-

05. Fabrication of Mild steel reinforcement for cement concrete i/c cutting, laying in

position making joined and fastening i/c cost binding wire.(CSI No 8(b) P-16)

Qty: 10.21 Cwt @ Rs: 5001.70 P.Cwt                                                                    Rs: 51089/-

06. Filling watering remaining earth floor out side lead upto 5 ft

Qty: 915.00 Cft @ Rs: 3660.0 P%0 Cft                                                                  Rs: 3321/-

07. Supplying & filling pit sand under floor and ploughing in well (Sch: No.29 P-26).

Qty: 183.00 Cft @ Rs: 1141.25 P% Cft                                                                  Rs: 2088/-

08. Add extra lead upto 3 mile…..

Qty: 1098.0 Cft @ Rs: 579.41 P% Cft                                                                    Rs: 6362/-

09. Pacca brick work in ground floor cement sand mortar 1:6 (Sch: No.4 (e) P=20)

Qty: 1108.00 Cft @ Rs: 12674.36 P% Cft                                                           Rs: 140432/-

10. G.I frame doors……. (CSI No 28 P- 98)

Qty: 136.00 Cft @ Rs: 228.90 P.Rft                                                                   Rs: 31130/-

11. S/F Guarder ……..

Qty: 3.51 Cwt @ Rs: 3850 P.Cwt                                                                        Rs: 13535/-

12.   S/F Tear……….

Qty: 18.98 Cwt @ Rs: 3575 P.Cwt                                                               Rs: 67869/-

13. Erection rolled steel beam ………

Qty: 22.49 Cwt @ Rs: 186.34 P.Cwt                                                              Rs:  4191/-

14. 1st Class roof Tile ………

Qty: 425.00 Sft @ Rs: 7607.25 P% Cft                                                         Rs: 32331/-

15. Cement plaster 1:6 up to 20ft heights 1.2 “thick (Sch: No. 11 (B) P.No. 51)

Qty: 3346.00 Sft @ Rs: 2206.60 P% Sft                                                             Rs: 73833/-

16. Cement plaster up to 20ft heights 3/8 “thick (Sch: No. 11 (C) P.No. 51)

Qty: 3346.00 Sft @ Rs: 2197.52 P% Sft                                                           Rs: 73529/-

17. 1st Class Wood ……..

Qty: 109.50 Sft @ Rs: 902.93 P.Sft                                                                    Rs: 98871/-

18. RCC Spout……….

Qty: 06 Nos. @ Rs: 261.25 P.Nos                                                                      Rs:  1568/-

19. Concrete Topping 3” Thick compacting finishing and Complete i/c screening and

washing stone aggregate without shuttering. Ratio 1:4:8 (G.S.I No.5 P-16)

Qty: 426.00 Sft @ Rs: 441.82 P% Sft                                                                Rs: 18794/-

20. White Wash……….

Qty: 366.00 Sft @ Rs: 829.95 P% Sft                                                                Rs:  3038/-

21. Primary coat of Chalk under Distempering………….

Qty: 3346.00 Sft @ Rs: 443.75 P% Sft                                                               Rs: 14848/-

22. Distemper 3 coat ………..

Qty: 3346.00 Sft @ Rs: 1079.65 P% Sft                                                             Rs: 36125/-

23. Painting door & windows …………

Qty: 241.00 Sft @ Rs: 2116.38 P% Sft                                                               Rs:   5079/-

24. Painting                 bars……….

Qty: 213.00 Sft @ Rs: 1270.80 P% Sft                                                                 Rs:  2707/-

Part – B

01. W.C………..                 02 Nos @ Rs: 5088.20 P.Nos                                                   Rs:   10176/-

02. Wash Basin …….         01 Nos @ Rs: 4253.50 P.Nos                                                    Rs:     4254/-

03. Add extra ……….        02 Nos @ Rs: 938.47 P.Nos                                                       Rs:     1877/-

04. C.I 4’’ Dia Ventilation Pipe 10.00 Rft @ Rs: 333.29 P.Rft                                             Rs:     3333/-

05. Providing and laying G.I Pip ½ ‘ dia 20 Nos @ Rs: 73.21 P.Nos                                   Rs:     1464/-

06. Bib Cock………..            05 Nos @ Rs: 1109.46 P.Nos                                                  Rs:    5545/-

SUB-ENGINEER                              ASSISTANT EXECUTIVE ENGINEER

TOWN COMMITTEE UBAURO                             TOWN COMMITTEE UBAURO