ESTIMATE

Name of Work: - Construction of Surface Drain B-type both side of Ranwti road Town

Ubauro Part-02.

01. Excavation in foundation of building bridges and structure i/c dag

Belling dressing refilling around the structure executes earth watering and

Ramming lead up to one chain lift up to 5ft in ordinary soil

(Sch: No.8 (b) P-4)

Qty: 4500.0Cft @ Rs: 3176.25 P%0 Cft                                                            Rs:  14293/-

02. Cement concrete Brick or stone ballast 1½ to 2 gauge (Ratio 1:4:8)

(Sch: No.4 (b) P.14)

Qty: 990.0 Cft @ Rs: 9416.28 P% Cft.                                                               Rs: 93221/-

03. Pacca brick work in foundation in cement sand mortar 1:6

(Sch: No.4 (e) P=20).

Qty: 2250.0 @ Rs: 11948.36 P% Cft                                                                   Rs: 268838/-

04. Construction of standard cunntte blocks open drains profile drawing

of cement concrete 1:2:4 situ to the design profile i-e applying flouting cost

of cement 1/32” to the exposed finishing complete as per deleted drawing.

(Sch: No.6 P-58).

Qty:  1000.0 Rft @ Rs: 174/- P /Rft.                                                              Rs: 174000/-

04. Cement plaster 1:4 up to 20ft heights 1/2 “

(Sch: No. 11 (B) P.No. 52)

Qty: 3000.0 Sft @ Rs: 2283.93 P% Sft.                                                          Rs: 68518/-

06. R.C.C Work in roof, slab beams, columns, rafts lintels and other Structural

members laid in situ or precost laid in position complete in all respect.

(I) Ratio (1:2:4) 90lbs. cement 2 cft. Sand 4 cft. Shingle 1/8x1/4 Gauge

(Sch: No.6 (A) P-17)

Qty: 120.0 Cft @ Rs: 337.0 P.Cft                                                               Rs: 40440/-

07. Fabrication of Mild steel reinforcement for cement concrete i/c cutting,

Laying in position making joined and fastening i/c cost binding wire.

Qty: 5.35 Cwt @ Rs: 5001.70 P.Cwt                                                                 Rs:  26759/

09. Providing & laying RCC pipe and collars of class “B” and fixing in trench i/c

Cutting , fitting  in trench jointing with .(P.H.E I no 2 © P-16)

06” dia

Qty: 36.00 Rft @ Rs: 163.0 P.Rft                                                                Rs: 5868/-

09” dia

Qty: 36.00 Rft @ Rs: 163.0 P.Rft                                                                 Rs:  5868/-

11. Providing & laying PVC pipes of class “B” “C” & “D” and fixing in trench i/c

Cutting, fixing & jointing with maxphalt composition and jointing with solvent

Cement i/c testing with water to head of 61 meter to 122 meter or 200 ft to 400 ft

(P.H.E I no E (4) P-23)

04” dia

Qty:  36.00 Rft @ Rs: 136.0 P.Rft                                                                 Rs:   4896/-

06” dia

Qty: 36.00 Rft @ Rs: 259.0 P.Rft                                                                 Rs:     9324/-

08” dia

Qty:   60.00 Rft @ Rs: 397.0 P.Rft                                                               Rs:   23820/-

SUB-ENGINEER                                                 ASSISTAN EXECUTIVE ENGINEER

TOWN COMMITTEE UBAURO                                             TOWN COMMITTEE UBAURO