Schedule B

Name of Work: - Construction of Water bound macadam road at Sardar khan dahar village

Town Ubauro.

01. Earth work for road embankment by bulldozers including ploughing mixing

clod breaking dressing and compacting with optimum moisture content. Lead

upto 100 ft, and lift upto 5 ft, in all types of soil except rock.

Qty: 73652.0 Cft @ Rs: 3656.23 P%0 Cft                                      Rs: 269289/-

02. Preparing sub base course by supplying and spreading stone metal 1 ½ -2 gauge

of approved quality from approved quarry in required thickness of 6” in 2 layers

to proper camber and grade including hand packing filling voids with 10 Cft:

screening and non plastic quarry fines of approved quality and gauge from

approved source, watering and compacting to archive 98-100% density as per

modified AASHO specification.(Rate includes all cost of material labour, T&P

and carriage upto site of work.

Qty: 10197.00 Cft @ Rs: 8821.72 P% Cft                                  Rs: 899550/-

03. Laying bricks on end edging including supplying of 9”x4” ½ “x 3” first class

Bricks, excavation for laying with small side parallel to the road, (Rate includes

All cost of material labour, T&P and carriage upto site work).

Qty: 3399.0 Rft @ Rs: 2729.94 P% Rft                                     Rs: 92791/-

04. Preparing sub base course i/c supplying and spreading stone metal of approved

quality from approved quarry properly graded to maximum size of 1 ½ and

required thickness to proper camber and grade i/c supplying and spreading 15 Cft

screening and non plastic quarry fines filling depression with stone metal after

initial rolling i/c watering and compacting the same so as to archive 100% density

as per modified AASHO specification. (This i/c providing and using templates

camber plates screen forms as directed. (Rate includes all cost of material labour, T&P

and carriage upto site of work.

Qty: 5099.00 Cft @ Rs: 9870.39 P% Cft                                Rs: 503291/-

05. Providing surface dressing 1st coat on new or existing surface with 30 lbs bitumen

And 4.0 Cft of required size i/c cleaning the road surface rolling etc, (Rate i/c all cost of

Materials T&P and carriage upto site of work).

Qty: 20394.00 Cft @ Rs: 1685.48 P% Cft                               Rs: 343737/-

06. Providing 1” thick (consolidated) premix carpet in proper camber and grade and 67

lbs of bitumen 80/100 penetration i/c mixing in mechanical mixer in required proportion

i/c heating the material and cleaning the road surface. (Rate includes all cost of

Materials T&P and carriage upto site of work).

Qty: 20394.0 Cft @ Rs: 4598.54 P% Cft                                Rs: 937826/-

SUB-ENGINEER                                           ASSISTANT EXECUTIVE ENGINEER

TOWN COMMITTEE UBAURO                                       TOWN COMMITTEE UBAURO