Schedule B

Name of Work: - Construction of Surface Drain and CC Block from Stadium to street of

Piyaro Dhandhu

01.  Barrow pit excavation undressed lead up to 100. Ft. in ordinary soil extra 6 mile lead.

(Sch: No.3(a) P-1)

Qty: 3500.00 Cft @ Rs: 7911.60 P%0 Cft                                            Rs: 27691/-

02. Dressing and leveling of earth work (Sch: no 11(B) P-3)

Qty: Same as item no 01 = 3500.00 Cft

Qty: 3500.00 Cft @ Rs: 187.550 P%0 Cft                                               Rs:    656/-

03. Excavation in foundation of building bridges and structure i/c dag

Belling dressing refilling around the structure executes earth watering and

Ramming lead up to one chain lift up to 5ft in ordinary soil

(Sch: No.8 (b) P-4)

Qty: 750.00 Cft @ Rs: 3176.25 P%0 Cft                                                       Rs:  2382/-

04 . Cement concrete Brick or stone ballast 1½ to 2 gauge (Ratio 1:4:8)

(Sch: No.4 (b) P.14)

Qty: 375.00 Cft @ Rs: 9416.28 P% Cft.                                                  Rs: 35311/-

05. Pacca brick work in foundation in cement sand mortar 1:6

(Sch: No.4 (e) P=20).

Qty: 1125.0 @ Rs: 11948.36 P% Cft                                                         Rs: 134419/-

06. Construction of standard cunntte blocks open drains profile drawing

of cement concrete 1:2:4 situ to the design profile i-e applying flouting cost

of cement 1/32” to the exposed finishing complete as per deleted drawing.

(Sch: No.6 P-58).

Qty:  500 Rft @ Rs: 94 P /Rft.                                                                     Rs: 47000/-

07. Cement plaster 1:4 up to 20ft heights 1/2 “

(Sch: No. 11 (B) P.No. 52)

Qty: 1250.0 Sft @ Rs: 2283.93 P% Sft.                                                                      Rs: 28549/-

08. R.C.C Work in roof, slab beams, columns, rafts lintels and other Structural

members laid in situ or precost laid in position complete in all respect.

(I) Ratio (1:2:4) 90lbs. cement 2 cft. Sand 4 cft. Shingle 1/8x1/4 Gauge

(Sch: No.6 (A) P-17)

Qty: 83.0 Cft @ Rs: 337.0 P% Cft                                                                           Rs: 27971/-

09. Fabrication of Mild steel reinforcement for cement concrete i/c cutting,

Laying in position making joined and fastening i/c cost binding wire.

Qty: 4.15 Cwt @ Rs: 5001.70 P.Cwt                                                                          Rs:  20757/-

Part-B Cement Concrete Block

01.  Barrow pit excavation undressed lead up to 100. Ft. in ordinary soil extra 3 mile lead.

(Sch: No.3 (a) P-1)

Qty: 3300.0 Cft @ Rs: 7911.60 P%0 Cft                                                                  Rs: 26108/-

02. Dressing leveling of Earth Work.

(Sch: No.11 (b)   P.03   )

Qty: Same as Item No: 01 = 3300.0 Cft

Qty: 3300.0 @ Rs: 187.55 P%0 Cft                                                                           Rs:   619/-

03. Cement concrete Brick or stone ballast 1½ to 2 gauge (Ratio 1:4:8)

(Sch: No.4 (b) P.14)

Qty: 1089.0 Cft @ Rs: 9416.28 P% Cft.                                                                     Rs: 102543/-

04. Laying CC Topping 3” thick Ratio: 1:2:4

Qty: 3300.0 Sft @ Rs: 4411.28 P% Sft                                                                      Rs: 145572/-

SUB-ENGINEER                              ASSISTANT EXECUTIVE ENGINEER

TOWN COMMITTEE UBAURO                             TOWN COMMITTEE UBAURO