SCHEDULE OF QUANTITIES

WORK No: 01

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, BALDIA ZONE

Estimated Cost Rs:	      9,92,314/-		Earnest Money Rs:	         19,846/-

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | P/L R.C.C. PIPE DRAIN AT MOHAMMADI MASJID ASHFAQIA ROAD IN UC-32, DMC(WEST) BALDIA ZONE. |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Excavation for pipe line in trenches and pits in all kind of soil of murum i/c trimming and dressing side to true alignment and shape traveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guard light flags and temporary crossing for non vehicular traffic wherever required left upto 05’ and lead upto 01 chain. | 8,100 Cft | 4650.00 | ‰ Cft | 37,665.00 |
| 2. | Excavation for pipe line in trenches and pits in hard rock by hammering and chiseling i/c trimming and dressing side to true alignment and shape traveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guard light flags and temporary crossing for non vehicular traffic wherever required left upto 05’ and lead upto 01 chain. | 2700 Cft | 24150.00 | ‰ Cft | 65,205.00 |
| 3. | A. PROVIDING, LAYING & JOINTING (Rubber Gasket, JOINTS). Providing, laying R.C.C. pipes of A.S.T.M. C-76-62 T/C: 76-70 of class II wall B and fixing in trench i/c cutting, fitting and jointing with rubber ring i/c testing with water to specified pressure. ……………………………….…..…12” Dia…………………….…………….. | 800 Rft | 412.00 | P/Rft | 3,29,600.00 |
| 4. | Construction of 4” dia manhole inside complete with 21” dia c.c. 1:2:4 pre cast C.I. frame 12 Kg i/c the cost of M.S. bars reinforcement and two handles 5” clear depth c.c. 1:2:4 cast in situ walls with 50% screened graded bajri of 9” thick i/c bending and channel i/c inside walls surface complete i/c cutting asphalt roads excavation in all kind of soil back filling and disposal of surplus stuff (KW&SB) approved rate. | 33 Nos | 14529.29 | Each | 4,79,467.00 |
| 5. | Extra depth beyond 5 feet or less than 5 feet deducted at the same rate (KW&SB) approved rate. | ( - ) 16.5 Rft | 2538.60 | P/Rft | ( - ) 41,887.00 |
| 6. | Refilling the excavated stuff in trenches 6” thick layer i/c watering ramming to full compaction etc complete. | 9,720 Cft | 2760.00 | ‰ Cft | 26,827.00 |
| 7. | Carriage of 100 Cft tons of all kinds material like cole bricks i/c loading unloading 10 miles. | 1080 Cft | 595.80 | %Cft | 6,435.00 |
| 8. | Manufacturing and supplying R.C.C. manhole covers casting c.c. 1:2:4 ratio 3 inch deep at center bar, reinforced with 3/8” dia tour steel 2.50” deep M.S. plate other curing, stacking and transportation within 10 miles. ……………………………………..…21” Dia……………………….………. | 36 Nos | 913.63 | Each | 32,891.00 |
| 9. | Manufacturing and supplying of R.C.C. ring slab of 21” inside 36” outside 7.5” width and 6” thick i/c 3/8” for steel bars two concentric ring with 3/8” 08 Nos. cross linked bars welded and two sunk type hooks casted etc. | 25 Nos | 1906.60 | Each | 47,665.00 |
| 10. | Shifting and fixing R.C.C. pre-cast ring slab in c.c. 1:2:4 etc. | 15 Nos | 563.09 | Each | 8,446.00 |
| Total Estimated Cost |  |  |  |  | 9,92,314.00 |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

BALDIA ZONE	BALDIA ZONE

I/We hereby quoted Rates @                           %  above/below the S/R

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 02

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, BALDIA ZONE

Estimated Cost Rs:	      9,92,877/-		Earnest Money Rs:	         19,858/-

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | REPLACEMENT OF R.C.C. PIPE IN DAMAGE PORTION DIFFERENT STREET IN UC-32, DMC(WEST) BALDIA ZONE. |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Excavation for pipe line in trenches and pits in all kind of soil of murum i/c trimming and dressing side to true alignment and shape traveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guard light flags and temporary crossing for non vehicular traffic wherever required left upto 05’ and lead upto 01 chain. | 8,100 Cft | 4650.00 | ‰ Cft | 37,665.00 |
| 2. | Excavation for pipe line in trenches and pits in hard rock by hammering and chiseling i/c trimming and dressing side to true alignment and shape traveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guard light flags and temporary crossing for non vehicular traffic wherever required left upto 05’ and lead upto 01 chain. | 2700 Cft | 24150.00 | ‰ Cft | 65,205.00 |
| 3. | A. PROVIDING, LAYING & JOINTING (Rubber Gasket, JOINTS). Providing, laying R.C.C. pipes of A.S.T.M. C-76-62 T/C: 76-70 of class II wall B and fixing in trench i/c cutting, fitting and jointing with rubber ring i/c testing with water to specified pressure. ……………………………….…..…12” Dia…………………….…………….. | 800 Rft | 412.00 | P/Rft | 3,29,600.00 |
| 4. | Construction of 4” dia manhole inside complete with 21” dia c.c. 1:2:4 pre cast C.I. frame 12 Kg i/c the cost of M.S. bars reinforcement and two handles 5” clear depth c.c. 1:2:4 cast in situ walls with 50% screened graded bajri of 9” thick i/c bending and channel i/c inside walls surface complete i/c cutting asphalt roads excavation in all kind of soil back filling and disposal of surplus stuff (KW&SB) approved rate. | 33 Nos | 14529.29 | Each | 4,79,467.00 |
| 5. | Extra depth beyond 5 feet or less than 5 feet deducted at the same rate (KW&SB) approved rate. | ( - ) 16.5 Rft | 2538.60 | P/Rft | ( - ) 41,887.00 |
| 6. | Refilling the excavated stuff in trenches 6” thick layer i/c watering ramming to full compaction etc complete. | 9,720 Cft | 2760.00 | ‰ Cft | 26,827.00 |
| 7. | Carriage of 100 Cft tons of all kinds material like cole bricks i/c loading unloading 10 miles. | 1080 Cft | 595.80 | %Cft | 6,435.00 |
| 8. | Manufacturing and supplying R.C.C. manhole covers casting c.c. 1:2:4 ratio 3 inch deep at center bar, reinforced with 3/8” dia tour steel 2.50” deep M.S. plate other curing, stacking and transportation within 10 miles. ……………………………………..…21” Dia……………………….………. | 36 Nos | 913.63 | Each | 32,891.00 |
| 9. | Manufacturing and supplying of R.C.C. ring slab of 21” inside 36” outside 7.5” width and 6” thick i/c 3/8” for steel bars two concentric ring with 3/8” 08 Nos. cross linked bars welded and two sunk type hooks casted etc. | 25 Nos | 1906.60 | Each | 47,665.00 |
| 10. | Shifting and fixing R.C.C. pre-cast ring slab in c.c. 1:2:4 etc. | 16 Nos | 563.09 | Each | 9,009.00 |
| Total Estimated Cost |  |  |  |  | 9,92,877.00 |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

BALDIA ZONE	BALDIA ZONE

I/We hereby quoted Rates @                           %  above/below the S/R

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 03

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, BALDIA ZONE

Estimated Cost Rs:	      4,45,248/-		Earnest Money Rs:	         8,905/-

Time Limit:	45 Days		Penalty Per Day Rs:	500/-

| SUBJECT: | P/L 12” DIA R.C.C. & C.C. FLOORING AT HOUSE No: 1725/2743 TO ISHFAQIA MASJID ROAD MAINWALI MUHALLAH IN UC-32, DMC(WEST) BALDIA ZONE. |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Excavation for pipe line in trenches and pits in all kind of soil of murum i/c trimming and dressing side to true alignment and shape traveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guard light flags and temporary crossing for non vehicular traffic wherever required left upto 05’ and lead upto 01 chain. | 2,400 Cft | 4650.00 | ‰ Cft | 11,160.00 |
| 2. | Excavation for pipe line in trenches and pits in hard rock by hammering and chiseling i/c trimming and dressing side to true alignment and shape traveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guard light flags and temporary crossing for non vehicular traffic wherever required left upto 05’ and lead upto 01 chain. | 450 Cft | 24150.00 | ‰ Cft | 10,867.00 |
| 3. | A. PROVIDING, LAYING & JOINTING (Rubber Gasket, JOINTS). Providing, laying R.C.C. pipes of A.S.T.M. C-76-62 T/C: 76-70 of class II wall B and fixing in trench i/c cutting, fitting and jointing with rubber ring i/c testing with water to specified pressure. ………………………………..…12” Dia…………………….……….. | 200 Rft | 412.00 | P/Rft | 82,400.00 |
| 4. | Construction of 4” dia manhole inside complete with 21” dia c.c. 1:2:4 pre cast C.I. frame 12 Kg i/c the cost of M.S. bars reinforcement and two handles 5” clear depth c.c. 1:2:4 cast in situ walls with 50% screened graded bajri of 9” thick i/c bending and channel i/c inside walls surface complete i/c cutting asphalt roads excavation in all kind of soil back filling and disposal of surplus stuff (KW&SB) approved rate. | 08 Nos | 14529.29 | Each | 1,16,234.00 |
| 5. | Extra depth beyond 5 feet or less than 5 feet deducted at the same rate (KW&SB) approved rate. | ( - ) 04 Rft | 2538.60 | P/Rft | ( - ) 10,154.00 |
| 6. | Refilling the excavated stuff in trenches 6” thick layer i/c watering ramming to full compaction etc complete. | 2,850 Cft | 2760.00 | ‰ Cft | 7,866.00 |
| 7. | Earth work excavation undressed lead upto a single through of Kassi Phawrah or Shovel. | 1575 Cft | 1361.25 | ‰ Cft | 2,144.00 |
| 8. | Cement concrete brick or stone ballast 1½” to 2” gauge (1:4:8). | 1,512 Cft | 9416.28 | %Cft | 97,835.00 |
| 9. | Providing and laying 2” thick topping and concrete (1:2:4) i/c surface finishing and dividing into panels. | 1,500 Sft | 3275.50 | %Sft | 1,03,178.00 |
| 10. | Cartage of 100 Cft Tons of all kind materials like cole brick i/c loading unloading 10 miles. | 84,840 Cft | 595.80 | %Cft | 11,082.00 |
| 11. | P/F C.I. manhole cover frame i/c cast of 1 Kg material etc. | 210 Kgs | 140.00 | P/Kg | 3,500.00 |

Cont’d Page No: 02

Page No:02

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 12. | Manufacturing and supplying R.C.C. manhole covers casting c.c. 1:2:4 ratio 3 inch deep at center bar, reinforced with 3/8” dia tour steel 2.50” deep M.S. plate other curing, stacking and transportation within 10 miles. ……………………………..…21” Dia……………………….………. | 36 Nos | 913.63 | Each | 9,136.00 |
| Total Estimated Cost |  |  |  |  | 4,45,248.00 |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

BALDIA ZONE	BALDIA ZONE

I/We hereby quoted Rates @                           %  above/below the S/R

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 04

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, BALDIA ZONE

Estimated Cost Rs:	      9,91,549/-		Earnest Money Rs:	         19,831/-

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | P/L 12” DIA R.C.C. PIPE DRAIN AT HAQANI MASJID TO SARHAD COLONY IN UC-31, DMC(WEST) BALDIA ZONE. |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Excavation for pipe line in trenches and pits in all kind of soil of murum i/c trimming and dressing side to true alignment and shape traveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guard light flags and temporary crossing for non vehicular traffic wherever required left upto 05’ and lead upto 01 chain. | 8,100 Cft | 4650.00 | ‰ Cft | 37,665.00 |
| 2. | Excavation for pipe line in trenches and pits in hard rock by hammering and chiseling i/c trimming and dressing side to true alignment and shape traveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guard light flags and temporary crossing for non vehicular traffic wherever required left upto 05’ and lead upto 01 chain. | 2700 Cft | 24150.00 | ‰ Cft | 65,205.00 |
| 3. | A. PROVIDING, LAYING & JOINTING (Rubber Gasket, JOINTS). Providing, laying R.C.C. pipes of A.S.T.M. C-76-62 T/C: 76-70 of class II wall B and fixing in trench i/c cutting, fitting and jointing with rubber ring i/c testing with water to specified pressure. ……………………………….…..…12” Dia…………………….…………….. | 800 Rft | 412.00 | P/Rft | 3,29,600.00 |
| 4. | Construction of 4” dia manhole inside complete with 21” dia c.c. 1:2:4 pre cast C.I. frame 12 Kg i/c the cost of M.S. bars reinforcement and two handles 5” clear depth c.c. 1:2:4 cast in situ walls with 50% screened graded bajri of 9” thick i/c bending and channel i/c inside walls surface complete i/c cutting asphalt roads excavation in all kind of soil back filling and disposal of surplus stuff (KW&SB) approved rate. | 33 Nos | 14529.29 | Each | 4,79,467.00 |
| 5. | Extra depth beyond 5 feet or less than 5 feet deducted at the same rate (KW&SB) approved rate. | ( - ) 16.5 Rft | 2538.60 | P/Rft | ( - ) 41,887.00 |
| 6. | Refilling the excavated stuff in trenches 6” thick layer i/c watering ramming to full compaction etc complete. | 9,720 Cft | 2760.00 | ‰ Cft | 26,827.00 |
| 7. | Carriage of 100 Cft tons of all kinds material like cole bricks i/c loading unloading 10 miles. | 1080 Cft | 595.80 | %Cft | 6,435.00 |
| 8. | Manufacturing and supplying R.C.C. manhole covers casting c.c. 1:2:4 ratio 3 inch deep at center bar, reinforced with 3/8” dia tour steel 2.50” deep M.S. plate other curing, stacking and transportation within 10 miles. ……………………………………..…21” Dia……………………….………. | 29 Nos | 913.63 | Each | 26,495.00 |
| 9. | Manufacturing and supplying of R.C.C. ring slab of 21” inside 36” outside 7.5” width and 6” thick i/c 3/8” for steel bars two concentric ring with 3/8” 08 Nos. cross linked bars welded and two sunk type hooks casted etc. | 25 Nos | 1906.60 | Each | 47,665.00 |
| 10. | Shifting and fixing R.C.C. pre-cast ring slab in c.c. 1:2:4 etc. | 25 Nos | 563.09 | Each | 14,077.00 |
| Total Estimated Cost |  |  |  |  | 9,91,549.00 |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

BALDIA ZONE	BALDIA ZONE

I/We hereby quoted Rates @                           %  above/below the S/R

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 05

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, BALDIA ZONE

Estimated Cost Rs:	      9,89,722/-		Earnest Money Rs:	         19,794/-

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | P/L 12” DIA R.C.C. PIPE DRAIN AT QADRI MUHALLAH IN UC-31, DMC(WEST) BALDIA ZONE. |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Excavation for pipe line in trenches and pits in all kind of soil of murum i/c trimming and dressing side to true alignment and shape traveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guard light flags and temporary crossing for non vehicular traffic wherever required left upto 05’ and lead upto 01 chain. | 8,100 Cft | 4650.00 | ‰ Cft | 37,665.00 |
| 2. | Excavation for pipe line in trenches and pits in hard rock by hammering and chiseling i/c trimming and dressing side to true alignment and shape traveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guard light flags and temporary crossing for non vehicular traffic wherever required left upto 05’ and lead upto 01 chain. | 2700 Cft | 24150.00 | ‰ Cft | 65,205.00 |
| 3. | A. PROVIDING, LAYING & JOINTING (Rubber Gasket, JOINTS). Providing, laying R.C.C. pipes of A.S.T.M. C-76-62 T/C: 76-70 of class II wall B and fixing in trench i/c cutting, fitting and jointing with rubber ring i/c testing with water to specified pressure. ……………………………….…..…12” Dia…………………….…………….. | 800 Rft | 412.00 | P/Rft | 3,29,600.00 |
| 4. | Construction of 4” dia manhole inside complete with 21” dia c.c. 1:2:4 pre cast C.I. frame 12 Kg i/c the cost of M.S. bars reinforcement and two handles 5” clear depth c.c. 1:2:4 cast in situ walls with 50% screened graded bajri of 9” thick i/c bending and channel i/c inside walls surface complete i/c cutting asphalt roads excavation in all kind of soil back filling and disposal of surplus stuff (KW&SB) approved rate. | 33 Nos | 14529.29 | Each | 4,79,467.00 |
| 5. | Extra depth beyond 5 feet or less than 5 feet deducted at the same rate (KW&SB) approved rate. | ( - ) 16.5 Rft | 2538.60 | P/Rft | ( - ) 41,887.00 |
| 6. | Refilling the excavated stuff in trenches 6” thick layer i/c watering ramming to full compaction etc complete. | 9,720 Cft | 2760.00 | ‰ Cft | 26,827.00 |
| 7. | Carriage of 100 Cft tons of all kinds material like cole bricks i/c loading unloading 10 miles. | 1080 Cft | 595.80 | %Cft | 6,435.00 |
| 8. | Manufacturing and supplying R.C.C. manhole covers casting c.c. 1:2:4 ratio 3 inch deep at center bar, reinforced with 3/8” dia tour steel 2.50” deep M.S. plate other curing, stacking and transportation within 10 miles. ……………………………………..…21” Dia……………………….………. | 27 Nos | 913.63 | Each | 24,668.00 |
| 9. | Manufacturing and supplying of R.C.C. ring slab of 21” inside 36” outside 7.5” width and 6” thick i/c 3/8” for steel bars two concentric ring with 3/8” 08 Nos. cross linked bars welded and two sunk type hooks casted etc. | 25 Nos | 1906.60 | Each | 47,665.00 |
| 10. | Shifting and fixing R.C.C. pre-cast ring slab in c.c. 1:2:4 etc. | 25 Nos | 563.09 | Each | 14,077.00 |
| Total Estimated Cost |  |  |  |  | 9,89,722.00 |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

BALDIA ZONE	BALDIA ZONE

I/We hereby quoted Rates @                           %  above/below the S/R

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 06

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, BALDIA ZONE

Estimated Cost Rs:	      9,90,636/-		Earnest Money Rs:	         19,813/-

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | P/L 12” DIA R.C.C. PIPE DRAIN AT NOORANI MUHALLAH IN UC-31, DMC(WEST) BALDIA ZONE. |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Excavation for pipe line in trenches and pits in all kind of soil of murum i/c trimming and dressing side to true alignment and shape traveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guard light flags and temporary crossing for non vehicular traffic wherever required left upto 05’ and lead upto 01 chain. | 8,100 Cft | 4650.00 | ‰ Cft | 37,665.00 |
| 2. | Excavation for pipe line in trenches and pits in hard rock by hammering and chiseling i/c trimming and dressing side to true alignment and shape traveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guard light flags and temporary crossing for non vehicular traffic wherever required left upto 05’ and lead upto 01 chain. | 2700 Cft | 24150.00 | ‰ Cft | 65,205.00 |
| 3. | A. PROVIDING, LAYING & JOINTING (Rubber Gasket, JOINTS). Providing, laying R.C.C. pipes of A.S.T.M. C-76-62 T/C: 76-70 of class II wall B and fixing in trench i/c cutting, fitting and jointing with rubber ring i/c testing with water to specified pressure. ……………………………….…..…12” Dia…………………….…………….. | 800 Rft | 412.00 | P/Rft | 3,29,600.00 |
| 4. | Construction of 4” dia manhole inside complete with 21” dia c.c. 1:2:4 pre cast C.I. frame 12 Kg i/c the cost of M.S. bars reinforcement and two handles 5” clear depth c.c. 1:2:4 cast in situ walls with 50% screened graded bajri of 9” thick i/c bending and channel i/c inside walls surface complete i/c cutting asphalt roads excavation in all kind of soil back filling and disposal of surplus stuff (KW&SB) approved rate. | 33 Nos | 14529.29 | Each | 4,79,467.00 |
| 5. | Extra depth beyond 5 feet or less than 5 feet deducted at the same rate (KW&SB) approved rate. | ( - ) 16.5 Rft | 2538.60 | P/Rft | ( - ) 41,887.00 |
| 6. | Refilling the excavated stuff in trenches 6” thick layer i/c watering ramming to full compaction etc complete. | 9,720 Cft | 2760.00 | ‰ Cft | 26,827.00 |
| 7. | Carriage of 100 Cft tons of all kinds material like cole bricks i/c loading unloading 10 miles. | 1080 Cft | 595.80 | %Cft | 6,435.00 |
| 8. | Manufacturing and supplying R.C.C. manhole covers casting c.c. 1:2:4 ratio 3 inch deep at center bar, reinforced with 3/8” dia tour steel 2.50” deep M.S. plate other curing, stacking and transportation within 10 miles. ……………………………………..…21” Dia……………………….………. | 28 Nos | 913.63 | Each | 25,582.00 |
| 9. | Manufacturing and supplying of R.C.C. ring slab of 21” inside 36” outside 7.5” width and 6” thick i/c 3/8” for steel bars two concentric ring with 3/8” 08 Nos. cross linked bars welded and two sunk type hooks casted etc. | 25 Nos | 1906.60 | Each | 47,665.00 |
| 10. | Shifting and fixing R.C.C. pre-cast ring slab in c.c. 1:2:4 etc. | 25 Nos | 563.09 | Each | 14,077.00 |
| Total Estimated Cost |  |  |  |  | 9,90,636.00 |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

BALDIA ZONE	BALDIA ZONE

I/We hereby quoted Rates @                           %  above/below the S/R

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 07

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, BALDIA ZONE

Estimated Cost Rs:	      9,92,062/-		Earnest Money Rs:	         19,841/-

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | CONSTRUCTION OF C.C. WORK FOR DUST BIN IN DIFFERENT POINTS JURISDICTION OF UC-31, DMC(WEST) BALDIA ZONE. |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Excavation in foundation of Building Bridges and other structures with excavated earth watering and ramming lead upto 5 feet in ordinary soil. | 4,158 Cft | 3176.25 | 1000 Cft | 13,207.00 |
| 2. | Stone pitching for top laying only (1.3” to 1.5”) thick in bed. | 2,079 Cft | 1996.00 | %Cft | 41,497.00 |
| 3. | Cement concrete brick or stone ballast 1½” to 2” gauge (1:4:8). | 1,386 Cft | 9416.28 | %Cft | 1,30,510.00 |
| 4. | Erection and removal of centering for R.C.C. or plain cement concrete works of Deodar Wood (2nd class). | 7,056 Sft | 3127.41 | %Sft | 2,20,670.00 |
| 5. | Cement concrete plain including placing compaction finishing and curing complete including screening and washing at stone aggregate without ratio 1:2:4. | 2,814 Cft | 14429.25 | %Cft | 4,06,039.00 |
| 6. | P/L ½” thick cement plaster 1:4 upto 12’ height. | 7,056 Sft | 2283.93 | %Sft | 1,61,154.00 |
| 7. | Painting with enamel paint on masonry wall. | 2,268 Sft | 837.10 | %Sft | 18,985.00 |
| Total Estimated Cost |  |  |  |  | 9,92,062.00 |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

BALDIA ZONE	BALDIA ZONE

I/We hereby quoted Rates @                           %  above/below the S/R

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 08

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, BALDIA ZONE

Estimated Cost Rs:	      9,96,383/-		Earnest Money Rs:	         19,928/-

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | IMPROVEMENT OF C.C. NALLAH AND R.C.C. SLAB AT JAWED BUILDER HOUSE NEAR EDHI CENTRE IN UC-32, DMC(WEST), BALDIA ZONE. |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Dismantling cement concrete R.C.C. (1:2:4) disposal lead upto 10 miles etc. | 2,400 Cft | 5445.00 | %Cft | 20,419.00 |
| 2. | Dismantling cement concrete plain 1:2:4. | 8,438 Cft | 3327.50 | %Cft | 4,991.00 |
| 3. | Carriage of 100 Cft tons of all kinds material like cole bricks i/c loading unloading 10 miles. | 1080 Cft | 595.80 | %Cft | 3,128.00 |
| 4. | Erection and removal of centering for R.C.C. or plain cement concrete works of Deodar Wood (2nd class). | 7,056 Sft | 3127.41 | %Sft | 9,382.00 |
| 5. | Cement concrete plain including placing compaction finishing and curing complete including screening and washing at stone aggregate without ratio 1:2:4. | 2,814 Cft | 14429.25 | %Cft | 43,288.00 |
| 6. | Fabrication of mild steel reinforcement for cement concrete including cutting, bending, laying in position making joints and fastenings including cost of binding wire (also includes removal of rust from bars) using tor bars. | 101.33 Cwt | 5001.70 | P/Cwt | 1,00,434.00 |
| 7. | RCC work in roof slab, beams columns rafts lintels and other structural members laid in situ or precast laid in position complete in all respects (ratio 1:2:4). | 3,050 Cft | 337.00 | P/Cft | 2,52,750.00 |
| 8. | Hiring of Skilled Labour for cleaning of Nallah (Afghani, Bajorri) for 08 Hours. | 475 Nos | 750.00 | Each | 3,56,250.00 |
| 9. | Hiring of Machinery with P.O.L. Excavator/Loader. | 16 Hours | 3088.13 | P/Hour | 49,410.00 |
| 10. | Hiring of Machinery with P.O.L. Dumper. | 48 Hours | 2006.90 | P/Hour | 96,331.00 |
| 11. | Hiring of Machinery with P.O.L. Tractor Trolly. | 40 Hours | 1500.00 | P/Hour | 60,000 |
| Total Estimated Cost |  |  |  |  | 9,96,383.00 |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

BALDIA ZONE	BALDIA ZONE

I/We hereby quoted Rates @                           %  above/below the S/R

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 09

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, BALDIA ZONE

Estimated Cost Rs:	      9,95,130/-		Earnest Money Rs:	         19,903/-

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | P/L 12” DIA R.C.C. PIPE DRAIN AT K.E.S.C. No: 766 STREET No:26 GOVT. SCHOOL STREET MUSLIM MUHALLAH BILAL MASJID ROAD IN UC-37, DMC(WEST) BALDIA ZONE. |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Excavation for pipe line in trenches and pits in all kind of soil of murum i/c trimming and dressing side to true alignment and shape traveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guard light flags and temporary crossing for non vehicular traffic wherever required left upto 05’ and lead upto 01 chain. | 8,100 Cft | 4650.00 | ‰ Cft | 37,665.00 |
| 2. | Excavation for pipe line in trenches and pits in hard rock by hammering and chiseling i/c trimming and dressing side to true alignment and shape traveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guard light flags and temporary crossing for non vehicular traffic wherever required left upto 05’ and lead upto 01 chain. | 2700 Cft | 24150.00 | ‰ Cft | 65,205.00 |
| 3. | A. PROVIDING, LAYING & JOINTING (Rubber Gasket, JOINTS). Providing, laying R.C.C. pipes of A.S.T.M. C-76-62 T/C: 76-70 of class II wall B and fixing in trench i/c cutting, fitting and jointing with rubber ring i/c testing with water to specified pressure. ……………………………….…..…12” Dia…………………….…………….. | 800 Rft | 412.00 | P/Rft | 3,29,600.00 |
| 4. | Construction of 4” dia manhole inside complete with 21” dia c.c. 1:2:4 pre cast C.I. frame 12 Kg i/c the cost of M.S. bars reinforcement and two handles 5” clear depth c.c. 1:2:4 cast in situ walls with 50% screened graded bajri of 9” thick i/c bending and channel i/c inside walls surface complete i/c cutting asphalt roads excavation in all kind of soil back filling and disposal of surplus stuff (KW&SB) approved rate. | 33 Nos | 14529.29 | Each | 4,79,467.00 |
| 5. | Extra depth beyond 5 feet or less than 5 feet deducted at the same rate (KW&SB) approved rate. | ( - ) 16.5 Rft | 2538.60 | P/Rft | ( - ) 41,887.00 |
| 6. | Refilling the excavated stuff in trenches 6” thick layer i/c watering ramming to full compaction etc complete. | 9,720 Cft | 2760.00 | ‰ Cft | 26,827.00 |
| 7. | Carriage of 100 Cft tons of all kinds material like cole bricks i/c loading unloading 10 miles. | 1080 Cft | 595.80 | %Cft | 6,435.00 |
| 8. | Manufacturing and supplying R.C.C. manhole covers casting c.c. 1:2:4 ratio 3 inch deep at center bar, reinforced with 3/8” dia tour steel 2.50” deep M.S. plate other curing, stacking and transportation within 10 miles. ……………………………………..…21” Dia……………………….………. | 36 Nos | 913.63 | Each | 32,891.00 |
| 9. | Manufacturing and supplying of R.C.C. ring slab of 21” inside 36” outside 7.5” width and 6” thick i/c 3/8” for steel bars two concentric ring with 3/8” 08 Nos. cross linked bars welded and two sunk type hooks casted etc. | 25 Nos | 1906.60 | Each | 47,665.00 |
| 10. | Shifting and fixing R.C.C. pre-cast ring slab in c.c. 1:2:4 etc. | 20 Nos | 563.09 | Each | 11,262.00 |
| Total Estimated Cost |  |  |  |  | 9,95,130.00 |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

BALDIA ZONE	BALDIA ZONE

I/We hereby quoted Rates @                           %  above/below the S/R

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address: