SCHEDULE -B

NAME OF WORK :-	 CONSTRUCTION OF  DRAINage SCHEME FOR VILLAGE

EIDAN  MAITLO U/C MEHAR ALI  TALUKA & District Khairpur.

.

PART –  I			Internal & External Drains

1.   Excavation in foundation of building bridges and other structure i/c dag belling refilling around the

structure with excavated earth watering ramming lead up to one chain and lift up to 5 ft. (PH Sch.

No.18 P.NO.4)

Qty.4941.00 cft.		@Rs.3176/25			P%0 cft.		Rs. 	15694/-

2.   Cement concrete plain i/c placing compacting finishing curing complete i/c screening and washing

stone aggregate w/o shuttering  (CSI No.5(i) P.NO.16)

- Ratio 1:4:8

Qty: 1026.00 cft.		@Rs. 11288/75		P% cft.			Rs. 	115,823/-

3.   Pacca Brick work in foundation and plinth in cement sand Ratio (1;6)  (CSI No.4 P.NO.20)

Qty: 3000.00 cft.		@Rs. 11948/36		P% cft.			Rs. 	358,451/-

4.   Construction of standard of open drains cunette block of cement concrete 1:2:4 in situ to design

profile i/c cost of mould as per drawing i/c applying floating coat of cement 1/32” thick to the

exposed face finished smooth curing etc complete as per detail drawing  (PHSI No.D P.NO.58)

For type-I

Qty: 1200.00 rft.		Rs. 94/-			P rft.			Rs. 	112,800/-

For type-II

Qty: 400.00 cft.		@Rs. 174/-			P rft.			Rs. 	69,600/-

5.   Cement plaster 1:4 20’ height ½” thick (CSI No.11 P.NO.52)

Qty: 4376.00 Sft.		@Rs. 2283/93			P% Sft.		Rs. 	99,945/-

6.	RCC work in roof slab, beams columns rafts, lintels, and other structure members laid in situ of 	pre 	cost laid in position complete in all respect 1:2:4  (CSI No.6 P.NO.16)

Qt. 185.83 cft.		Rs. 337/-			P cft.			Rs. 	62,625/-

7. 	Fabrication of mild steel reinforcement for cement concrete i/c cutting bending laying in

position making K=joints and fastering i/c cost of binding wire also i/c removal of rust

from bars (CSI No.8 (ii) P.NO.17).

Qt. 6.64 cwt.		@Rs. 5001/70			P cwt.			Rs. 	33,211/-

Total:	Rs.	868,149/-

.

PART - II		PAVER BLOCK

0.1      Cement concrete plain i/c placing compacting finishing curing complete i/c screening and washing stone aggregate w/o shuttering  (CSI No.5(i) P.NO.17)

(A) Ratio 1:4:8

Qt. 800.00 cft.		@Rs. 11288/75			P% cft.			Rs.	90310/-

(b) Ratio 1:2:4

Qt. 66.0 cft.		@Rs. 14429/25			P% cft.			Rs.	9523/-

02.	Hard Murrum or fine powdery murrum (SMI No. 71P-6)

Qt. 800 cft.		@Rs. 1800				P% cft.			Rs.	14400/-

03.	Providing & Fixing Cement Paving Blocks Flooring Having Size Of 197x97x60(Mm) Of City /Quddra/ Cobble Shape With natural color Having Strength B/W 5000 Psi To 8500 Psi I/C Filling The Joint With Hill Sand Laying In Specified Manner / Pattern And Design Etc Complete.

(CSI No-71 P-49)

Qt. 3200.00 sft.	@Rs. 199/77				P. sft.			Rs.	639040/-

Total:-			Rs:	662963/-

# general Abstract of cost

| S. No | Name of Work | Schedule Item | Non | G. Total |
| --- | --- | --- | --- | --- |

1.  Internal & External Drains	  Rs. 	868,149/=	Rs.	---       Rs. 	868,149/=

2. Paver  Block				  Rs. 	662963/=	 Rs.	---       Rs. 	662963/=

G.Total:        Rs. 1531112/= Rs.	---        Rs. 1531112/=

CertificateS

- Any error or omission in the description of items of works rates original schedule will be correct. Accordingly in the instruction of the Executive Engineer, Public Health Engineering Division-I Khairpur.

- No premium will be allowed on non-schedule items.

- The work to be carried out as per PWD/PHE specifications approval rates working estimates.

CONTRACTOR.				             	       EXECUTIVE ENGINEER

PUBLIC HEALTH ENGINEERING

DIVISION-I KHAIRPUR