Part –  		Chokidar hut

1.	Excavation in foundation building bridges and other structure i/c dag  bailing dressing refilling around the structure with excavated earth watering and ramming lead up to once chain and lift up to 5’ ft. (in ordinary soil) (CSI No.18 P.No.4)

Qty: 290.62	cft		@Rs: 3176/25		P%0 cft		Rs:	923/-

2.	Cement concrete brick or stone ballast ½ to 2 gauges Ratio 1:4:8 (CSI No.6 P.No.16).

Ratio: 1:4:8

Qty: 143.30	cft		@Rs: 9416/28		P% cft			Rs:	13494/-

3. 	Pacca brick work in foundation plinth in cement sand mortar 1:6 (CSI No – 5 Page  24)

Qty: 455.30	cft		@Rs: 11948/36	P% cft			Rs:	54401/-

4	Damp proof course of cement sand and shingle concrete 1:2:4 i/c two coats of asphalted mixture (CSI No.27 P.No.23)

Qty: 68.03	cft		@Rs: 4982/18		P% cft			Rs:	3389/-

5.	Pacca brick work in ground floor i/c stricking of jointing cement sand mortar 1:6 (CSI No: 5

P-25)

Qty: 367.80	cft		@Rs: 12674/36		P% cft		Rs:	46616/-

9.	RCC work in roof slab, beams columns rafts, lintels, and other structure members laid in situ of pre cost laid in position complete in all respect (PHSI No.6 P.NO.17)

Qty: 135.27	cft		@Rs: 337/-		P. cft			Rs:	45586/-

10.	Fabrication of mild steel reinforcement for cement concrete i/c cutting bending laying in position making joints and festering i/c cost of binding wire also i/c removal of rust from bars (CSI No.7 (a) P.NO.19

Qty: 5.43 	cwt		@Rs: 5001/70		P. cwt			Rs:	27159/-

11.	Fabrication of heavy steel work with angles tees flate iron round and sheet iron for making trusses girders tanks etc i/c cutting tanks etc i/c cutting drilling revetting handling assembling and firing but excluding errection in position (CSI No.2 (a) P.NO.90)

Qty: 4.41 cft			@Rs:4928/49		P%0 cft		Rs:	21735/-

12.	Errection & fitting  in position iron trusses fixing in position (C,S,I No.6 P-No.91)

Qty: 4.41	cwt		@Rs: 271/04		P. cwt			Rs:	1195/-

13	Supplying fixing in position iron steel gril using sold square bars of size ¾” x ¼” etc complete (CSI NO 26 – Page 97)

Qty: 24.00	sft		@Rs:180/50		P. sft			Rs:	4332/-

14.	Filling water and ramming earth in floor with surplus earth from foundation and lead upto one chain and lift upto 5’ ft (CSI No 7 page 21)

Qty: 414.73 cft		@Rs.1512/50		P%0 cft		Rs:	627/-

15.	Cement Plaster (1:6) up to 20 ft: Height ½” thick (CSI No 11.P.No. 58)

Qty: 581.24	sft		@Rs.2206/60		P% sft			Rs:	12826/-

16.	Cement Plaster (1:4) up to 12 ft: Height 3/8 thick (CSI No 11.P.No. 58)

Qty: 581.24	sft		@Rs.2197/52		P% sft			Rs:	12773/-

17.	Cement painting struck joint on walls 1:3 (CSI No – 19 Page 59) (Ratio 1.3)

Qty: 377.50	sft		@Rs.1213/58		P% sft			Rs:	4581/-

18.	Providing and laying 1” thick topping cement concrete (1:2:4) i/c surface fixing and deriding in to panels. (CSI No 16 P No 47)

2” thick

Qty: 175.03	sft		@Rs.3275/50		P% sft			Rs:	5733/-

19.	Preparing surface painting doors & windows any type i/c edge (CSI No – 5 page 76)

Qty: 104.00	sft		@Rs.2116/41		P% sft			Rs:	2201/-

20.	White washing (Three coats) CSI No – 26 Page 55)

Qty: 219.00	sft		@Rs.829/95		P% sft			Rs:	1818/-

21.	White washing (one Coat) (CSI No – 26 Page 55)

Qty: 581.24 sft		@Rs.416/63		P% sft			Rs:	2422/-

22.	Colour Washing Two Coats (CSI No – 25 Page 55)

Qty: 958.74 	sft		@Rs.859/90		P% sft			Rs:	8244/-

Total	Rs:	270055/-