# general Abstract of cost

| S. No | Name of Work | Schedule Item | Non | G. Total |
| --- | --- | --- | --- | --- |

1.  Internal & External Drains	  Rs. 	590445/=	Rs.	---       Rs. 	590445/=

2. C .C Block				  Rs. 902159/=	 Rs.	---       Rs. 	902159/=

3. Pumping Machinery 		  Rs. ----		 Rs.686000  Rs. 	686000/=

G.Total:        Rs. 1492602/= Rs.686000  Rs. 2178604/=

CertificateS

- Any error or omission in the description of items of works rates original schedule will be correct. Accordingly in the instruction of the Executive Engineer, Public Health Engineering Division-I Khairpur.

- No premium will be allowed on non-schedule items.

- The work to be carried out as per PWD/PHE specifications approval rates working estimates.

CONTRACTOR.				             	       EXECUTIVE ENGINEER

PUBLIC HEALTH ENGINEERING

DIVISION-I KHAIRPUR

SCHEDULE -B

NAME OF WORK :-	REHABILITATION OF DRAINAGE SCHEME FOR VILLAGE RAZI GOTH  TALUKA & DISTRICT KHAIRPUR.

| S. No | Qty: | Item of Work | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |

PART –  I			Internal & External Drains

1.   Excavation in foundation of building bridges and other structure i/c dag belling refilling around the

structure with excavated earth watering ramming lead up to one chain and lift up to 5 ft. (PH Sch.

No.18 P.NO.4)  In Ordinary Soil.

:

Qt 3317.50cft.		@Rs.3176/25			P%0 cft.		Rs. 	10537/-

2.   Cement concrete plain i/c placing compacting finishing curing complete i/c screening and washing

stone aggregate w/o shuttering  (CSI No.5(i) P.NO.17)

- Ratio 1:4:8

Qt. 755.00 cft.		@Rs. 11288/75		P% cft.			Rs. 	85230/-

3.   Pacca Brick work in foundation and plinth in cement sand Ratio (1;6)  (CSI No.4 P.NO.20)

Qt. 2100.00cft.		@Rs. 11948/36	P% cft.			Rs. 	250916/-

4.   	Construction of standard of open drains cunette block of cement concrete 1:2:4 in situ to design

profile i/c cost of mould as per drawing i/c applying floating coat of cement 1/32” thick to the

exposed face finished smooth curing etc complete as per detail drawing  (PHSI No.D P.NO.58)

Type-I

Qt. 1000.00 rft.		Rs. 94/-		P rft.			Rs. 	94000/-

Type-II

Qt. 200.00 rft.			@Rs. 174/00		P rft.			Rs. 	34800/-

5.   	Cement plaster 1:4 20’ height ½” thick (CSI No.11 P.NO.51)

Qt. 3124.00Sft.		@Rs. 2283/93		P% Sft.		Rs. 	74474/-

6.	RCC work in roof slab, beams columns rafts, lintels, and other structure members laid in situ of pre 	cost laid in position complete in all respect 1:2:4  (CSI No.5 P.NO.16)

Qt. 45.00cft.			Rs 337/-		P cft.			Rs. 	15165/-

7. 	Fabrication of mild steel reinforcement for cement concrete i/c cutting bending laying in

position making K=joints and fastering i/c cost of binding wire also i/c removal of rust

from bars (CSI No.7 (ii) P.NO.16).

Qt 1.607 cwt			@Rs. 5001/70		P cwt.			Rs. 	8038/-

8.   	Cement concrete plain i/c placing compacting finishing curing complete i/c screening and washing

stone aggregate w/o shuttering  (CSI No.5(i) P.NO.17)

- Ratio 1:2:4

Qt. 199.79 cft.		@Rs. 11288/75		P% cft.			Rs. 	17285/-

Total:	Rs:	590445/-

PART - II		C.C BLOCK

01.	Barrow pit excavation undressed led up to 100 rft in ordinary soil (CSI no.3 p.1)..

Qt. 12000.00 cft.		@Rs.2117/50			P%o cft.		Rs. 	25410/-

02.	Earth work compaction (soft ordinary hard soil Laying earth in 6” thick layer, leveling and dressing complete. (CSI.NO.13 (b) P/03)

Qty: 12000.00 Cft 		@Rs:-263/-			P%0 Cft	 	Rs. 	3156/-

03.	Extra for every 50 ft: additional lead or part therefore earth work soft or ordinary hard very hard.(CSI No: 8 P.No:3) Total land 2600 (-) 100 =  50 leads  @Rs. 100/78 Per Lead Rs. 5039/=

Qty: 12000 cft		@Rs: 5039/-			P%0 cfT		Rs. 	60468/-

04.	Cement concrete brick or stone ballast 1 ½” to 2”  guage Ratio 1:4:8 (CSI No.9 P.16).

Ratio: 1:4:8

Qt. 3960.00 cft.		@Rs. 9416/28			P% cft.			Rs. 	372885/-

0.5      Cement concrete plain i/c placing compacting finishing curing complete i/c screening and washing stone aggregate w/o shuttering  (CSI No.5(i) P.NO.17)

Ratio 1:2:4

Qt. 3000.00 cft.		@Rs.14429/25		P% cft.			Rs.	432878/-

06	Erection and removal of centering for RCC and Plain CC work of partial wood, (2 nd Class)

(CSI No.19 (b) P.No.17)

Qty. 300.00sft.		@Rs. 3127/41			P% sft.			Rs. 	9382/-

Total:-		Rs:	902159/-

Non-Schedule Item

PART V			(A)PUMPING MACHINERY

1	supplying installing in position at site of work for drainage scheme Danwro electric motor set 10 BHP (Siemens made)  four pole 1500RPM 3 phase 400 VAC 500 cycles coupled with KSB non clogging sullage pump (type KWPZ65-315) size 3”x1/2” i/c steel frame coupling with sleeve etc complete capable of discharge 150 gallons per minute against a head of 45 ft i/c installation of pumping set in cement concrete foundation ratio 1:2:4 with MS nuts an bolts embedded in cc foundation i/c providing and fixing automatic motor control unit (MCU) consist oil stator (MECO Pakistan) MCCB Breaker 10-A to 50-A (Shihlin Taiwan) cutout 60-A (China) ampere meter (60-A) (SELCO China) volt meter (50-V) (SELCO China) phase indication light (China) i/c transformer oil (2 Letter) (Panel Board 18’x26’x9’ & internal wiring etc complete) and providing internal electric wiring main or sub-main PVC insulated with size 3-7/0.044 (6mm2)i/c providing and fixing earthing set 0.6m in to 6.3mm 2x2x1/2 CI plate buried in the ground at depth of 3.7mm (12”)or less as per PAK PWD specification and testing of the pumping set required head of 72 hours etc complete (R.A)

Qty B/F MB No.704 P.No  11/2                =  1. set

Qty 1 Set				@Rs. 686000/-		P. set		Rs.686000/-