# general Abstract of cost

| S. No | Name of Work | Schedule Item | Non | G. Total |
| --- | --- | --- | --- | --- |

1.  Internal & External Drains	  Rs. 	912574/=	Rs.	---       Rs. 	912574/=

2. C .C Block				  Rs. 	515368/=	 Rs.	---       Rs. 	515368/=

G.Total:        Rs. 1427942/= Rs.	---        Rs. 1427942/=

CertificateS

- Any error or omission in the description of items of works rates original schedule will be correct. Accordingly in the instruction of the Executive Engineer, Public Health Engineering Division-I Khairpur.

- No premium will be allowed on non-schedule items.

- The work to be carried out as per PWD/PHE specifications approval rates working estimates.

CONTRACTOR.				             	       EXECUTIVE ENGINEER

PUBLIC HEALTH ENGINEERING

DIVISION-I KHAIRPUR

SCHEDULE -B

NAME OF WORK :-	 CONSTRUCTION OF  DRAINage SCHEME  VILLAGE JUMO PHULPOTO U/C KHAN PUR  TALUKA & District Khairpur.

.

PART –  I			Internal & External Drains

1.   Excavation in foundation of building bridges and other structure i/c dag belling refilling around the

structure with excavated earth watering ramming lead up to one chain and lift up to 5 ft. (PH Sch.

No.18 P.NO.4)

Qty.5428.05 cft.		@Rs.3176/25			P%0 cft.		Rs. 	17241/-

2.   Cement concrete plain i/c placing compacting finishing curing complete i/c screening and washing

stone aggregate w/o shuttering  (CSI No.5(i) P.NO.16)

- Ratio 1:4:8

Qty: 1107.30 cft.		@Rs. 11288/75		P% cft.			Rs. 	125,000/-

3.   Pacca Brick work in foundation and plinth in cement sand Ratio (1;6)  (CSI No.4 P.NO.20)

Qty: 32700.00 cft.		@Rs. 11948/36		P% cft.			Rs. 	390,711/-

4.   Construction of standard of open drains cunette block of cement concrete 1:2:4 in situ to design

profile i/c cost of mould as per drawing i/c applying floating coat of cement 1/32” thick to the

exposed face finished smooth curing etc complete as per detail drawing  (PHSI No.D P.NO.58)

For type-I

Qty: 1260.00 rft.		Rs. 94/-			P rft.			Rs. 	118,440/-

For type-II

Qty: 460.00 cft.		@Rs. 174/-			P rft.			Rs. 	80,040/-

5.   Cement plaster 1:4 20’ height ½” thick (CSI No.11 P.NO.52)

Qty: 4751.60 Sft.		@Rs. 2283/93			P% Sft.		Rs. 	108,523/-

6.	RCC work in roof slab, beams columns rafts, lintels, and other structure members laid in situ of 	pre 	cost laid in position complete in all respect 1:2:4  (CSI No.6 P.NO.16)

Qt. 140.83 cft.		Rs. 337/-			P cft.			Rs. 	47,460/-

7. 	Fabrication of mild steel reinforcement for cement concrete i/c cutting bending laying in

position making K=joints and fastering i/c cost of binding wire also i/c removal of rust

from bars (CSI No.8 (ii) P.NO.17).

Qt. 5.03 cwt.		@Rs. 5001/70			P cwt.			Rs. 	25,159/-

Total:	Rs.	912574/-

.

PART - II		C.C BLOCK

01.	Barrow pit excavation undressed led up to 100 rft in ordinary soil (CSI no.3 p.1)..

Qt.6580.00 cft.		@Rs. 2117/50			P%0 cft.		Rs. 	13933/-

02.	Earth work compaction (soft ordinary hard soil Laying earth in 6” thick layer, leveling and dressing complete. (CSI.NO.13 (b) P/03)

Qty: 6580.00	 Cft 		@Rs:-263/-			P%0 Cft	 	Rs. 	1730/-

03.	Extra for every 50 ft: additional lead or part therefore earth work soft or ordinary hard very hard.(CSI No: 8 P.No:3) Total land 2600 (-) 100 =  50 leads  @Rs. 100/78 Per Lead Rs. 5039/=

Qrt 6580.00 cft		@Rs: 5039/-			P%0 cf	t		Rs. 	33156/-

04.	Cement concrete brick or stone ballast 1 ½” to 2”  guage Ratio 1:4:8 (CSI No.4 P.17).

RATIO 1:4:8

Qt. 2171.40 cft.		@Rs. 9416/28			P% cft.			Rs. 	204465/-

05.	Cement concrete plain i/c placing compacting finishing curing complete i/c screening and washing stone aggregate w/o shuttering  (CSI No.5(i) P.NO.17)

Ratio 1:2:4

Qt. 1645.00 cft.		@Rs. 14429/25		P% cft.			Rs.	 237361/-

06.	Erection and removal of centering for RCC and Plain CC work of partial wood,

(CSI No.19 (b) P.No.18)

Qty. 470.0 sft.			@Rs. 3127/41			P% sft.			Rs. 	14699/-

07.	Providing R.C.C pipe with colors class-B and digging the trenches to required depth &  fixing 	position i/c cutting fixing & jointing with max phalt composition & cement mortar 1:1 and testing 	with water pressure to a head of 4” ft above the top of the highest pipe & refilling with excavated 	earth (WSSI No:2  P.No: 21)

9" dia pipe

Qt. 40.00 rft.			@Rs. 250/60			P rft.			Rs. 	10024/-

Total:-		Rs: 	515368/-