# general Abstract of cost

| S. No | Name of Work | Schedule Item | Non | G. Total |
| --- | --- | --- | --- | --- |

1.  Internal & External Drains	  Rs. 	977381/=	Rs.	---       Rs. 	977381/=

2. C .C Block				  Rs. 	682637/=	 Rs.	---       Rs. 	682637/=

G.Total:        Rs. 1660018/= Rs.	---        Rs. 1660018/=

CertificateS

- Any error or omission in the description of items of works rates original schedule will be correct. Accordingly in the instruction of the Executive Engineer, Public Health Engineering Division-I Khairpur.

- No premium will be allowed on non-schedule items.

- The work to be carried out as per PWD/PHE specifications approval rates working estimates.

CONTRACTOR.				             	       EXECUTIVE ENGINEER

PUBLIC HEALTH ENGINEERING

DIVISION-I KHAIRPUR

SCHEDULE -B

NAME OF WORK :-	DRAINAGE SCHEME vILLAGE ALAM JUNEJO U/C KHAN PUR  TALUKA & District Khairpur.

| S. No | Qty: | Item of Work | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |

PART –  I			Internal & External Drains

1.   Excavation in foundation of building bridges and other structure i/c dag belling refilling around the

structure with excavated earth watering ramming lead up to one chain and lift up to 5 ft. (PH Sch.

No.18 P.NO.4)  In Ordinary Soil.

:

Qt 6169.28cft.		@Rs.3176/25			P%0 cft.		Rs. 	19,595/-

2.   Cement concrete plain i/c placing compacting finishing curing complete i/c screening and washing

stone aggregate w/o shuttering  (CSI No.5(i) P.NO.17)

- Ratio 1:4:8

Qt. 1087.65 cft.	@Rs. 11288/75		P% cft.			Rs. 	122,782/-

3.   Pacca Brick work in foundation and plinth in cement sand Ratio (1;6)  (CSI No.4 P.NO.20)

Qt. 3495.00cft.		@Rs. 11948/36	P% cft.			Rs. 	417,595/-

4.   	Construction of standard of open drains cunette block of cement concrete 1:2:4 in situ to design

profile i/c cost of mould as per drawing i/c applying floating coat of cement 1/32” thick to the

exposed face finished smooth curing etc complete as per detail drawing  (PHSI No.D P.NO.58)

type-I

Qt. 930.00 rft.		Rs. 94/-			P rft.			Rs. 	87,420/-

type-II

Qt. 700.00 rft.		@Rs. 174/00			P rft.			Rs. 	121,800/-

5.   	Cement plaster 1:4 20’ height ½” thick (CSI No.11 P.NO.51)

Qt. 4920.20Sft.		@Rs. 2283/93		P% Sft.		Rs. 	112,374/-

6.	RCC work in roof slab, beams columns rafts, lintels, and other structure members laid in situ of pre 	cost laid in position complete in all respect 1:2:4  (CSI No.5 P.NO.16)

Qt. 185.83cft.		Rs 337/-			P cft.			Rs. 	62624/-

7. 	Fabrication of mild steel reinforcement for cement concrete i/c cutting bending laying in

position making K=joints and fastering i/c cost of binding wire also i/c removal of rust

from bars (CSI No.7 (ii) P.NO.16).

Qt1. 6.636 cwt		@Rs. 5001/70			P cwt.			Rs. 	33191/-

Total:	Rs:	977381/-

PART - II		C.C BLOCK

01.	Barrow pit excavation undressed led up to 100 rft in ordinary soil (CSI no.3 p.1)..

Qt. 8775.00 cft.		@Rs.2117/50			P%o cft.		Rs. 	18581/-

02.	Earth work compaction (soft ordinary hard soil Laying earth in 6” thick layer, leveling and dressing complete. (CSI.NO.13 (b) P/03)

Qty: 8775.00 Cft 		@Rs:-263/-			P%0 Cft	 	Rs. 	2307/-

03.	Extra for every 50 ft: additional lead or part therefore earth work soft or ordinary hard very hard.(CSI No: 8 P.No:3) Total land 2600 (-) 100 =  50 leads  @Rs. 100/78 Per Lead Rs. 5039/=

Qty: 8775.00 cft	@Rs: 5039/-		P%0 cfT			Rs. 	44217/-

04.	Cement concrete brick or stone ballast 1 ½” to 2”  guage Ratio 1:4:8 (CSI No.9 P.16).

Ratio: 1:4:8

Qt. 2895.75 cft.	@Rs. 9416/28			P% cft.			Rs. 	272672/-

0.4      Cement concrete plain i/c placing compacting finishing curing complete i/c screening and washing stone aggregate w/o shuttering  (CSI No.5(i) P.NO.17)

Ratio 1:2:4

Qt. 2193.75 cft.	@Rs.14429/25		P% cft.			Rs.	316541/-

06	Erection and removal of centering for RCC and Plain CC work of partial wood, (2 nd Class)

(CSI No.19 (b) P.No.17)

Qty. 585.00sft.	@Rs. 3127/41			P% sft.			Rs. 	18295/-

07	Providing RCC Pipe with collars class B and digging the trenches to required depth and 	Fixing to in position cutting and jointing with maxphalt composition and cement 	mortar 	1:1 and testing with rater pressure etc a head executed stuff. (PHSI No 1 P-	24)

a) 9” Dia Pipe

Qty: 40.00 Rft  		@Rs.250/60			P-Rft	 	Rs.	 10,024/=

Total:-		Rs:	682637/-