PART’A’ROAD WORK

| ## NAME OF SCHEME:- | Construction of road from N.H.Way – Molro road to village Haji Saeed Khan Sohu mile 0/0 – 0/6 ½” (1.30 Km) |
| --- | --- |

(SCHEDULE “B”)

| QTY: | ITEM OF WORK | RATE |  | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- |
|  | 1/-Earth work for road embankment By bulldozers i/cs: plugging mixing clod breaking dressing & compacting with optimum moisture content lead upto 100’ & lift upto 5ft: in all types of soil. (S.I No: 3 P/No: 1). A) COMPACTING UPTO 85%MODIFIED AASHO DENSITY. |  |  |  |  |
| 210000 | Cft: withextra lead | @Rs.8679/15 |  | P. %0Cft: | Rs.1822621/- |
|  | #### B) COMPACTING UPTO 95-100%MODIFIED DENSITY |  |  |  |  |
| 61861 | Cft: withextra lead | @Rs.11208/92 |  | P. %0 Cft: | Rs:693395/- |
|  | 2/-Preparing Sub-Base course by supplying & spreading stone metal 1 ½”-2”gauge of approved quality from approved quarry in required thickness to proper camber & grade i/cs: hand packing filling voids with 10 cft: screening & Non plastic quarry fines of approved source watering & compacting to achieve 98-100%density as per modified AASHO Specification (Rate i/cs: all costs of materials T&P & carriage upto site of work) (S.I No: 8© P/No: 3). |  |  |  |  |
| 26512Cft |  | @ Rs.5841/96 |  | P.% Cft: | Rs.1548820/- |
|  | 3/-Laying Brick on end edging i/cs: supply of 9”x4½”x3”Ist: Class Burnt bricks excavation for having edging with small size parallel to the road (Rate i/cs: lead upto side of work) |  |  |  |  |
| 8837 Rft |  | @ Rs.2844/57 |  | P.% Rft: | Rs.251374/- |
|  | 4/-Preparing Base course i/cs: supplying & spreading stone metal of approved quality properly graded to maximum size of 1 ½” in required thickness to proper camber & grade i/cs: supplying & spreading l5 cft: screening & Non plastic quarry fines filling depression with stone metal after initial rolling i/cs: watering & compacting the same so as to achieve 100% density as per modified AASHO Specification. (Rate i/cs: all costs of materials T&P & carriage upto site of work) |  |  |  |  |
| 13256 Cft |  | @ Rs.6786/43 | P.% Cft: |  | Rs.899609/- |
|  | 5/-Providing Surface Dressing Ist: Coat on new or existing surface with 30lbs: bitumen & 4.00 Cft: Crush bajri of required size i/cs: cleaning the road surface rolling etc: complete. (Rate i/cs: all costs |  |  |  |  |
| 53024 Sft |  | @ Rs.1592/56 |  | P.% Sft: | Rs.844439/- |
|  | 6/-Providing 1”thick (consolidate) Premixed Carpet in proper camber & grade i/cs: supplying of 10 cft: Crush bajri 4 cft: Hillsand (of approved quality and grade) with 67lbs: bitumen of 80/100 penetration i/cs: fixing in machinical mixer in required proportion i/cs: heating materials and cleaning the road surface (Hillsand 2 cft: for mixing and 2 cft: for dusting) (S.I No: 17 P/No: 8). |  |  |  |  |
| 53024 Sft |  | @ Rs.4465/70 |  | P.% Sft: | Rs.2367892/- |
|  | 7/-Earth work for embankment from barrow pit i/cs: laying in 6”layers clod breaking remaining dressing complete lead upto 100’ & lift upto 5.0’ (In ordinary Soil (S.I No: 2 P/No: 1). |  |  |  |  |
| 30100 cft: | Withextra lead | @Rs.7233/57 |  | P. %0Cft | Rs.217730/- |
|  |  | Total Rs: 8645880/- |  |  |  |
|  |  | Decreased Cost of Bitumen 22.96 ton @Rs:17595/P.Ton |  |  | Rs(-)403981/- |
|  |  | G.Total |  |  | Rs:8241899/- |

| NOTE: Subject to any Changes/ Variation in Rates/ Quantity as per T.Sanction. |  |
| --- | --- |
| # CONTRACTOR | EXECUTIVE ENGINEER HIGHWAY DIVISION # KHAIRPUR |

PART’A’I M/STRUCTURE

| ## NAME OF SCHEME:- | Construction of 3.0’ Span Culvert along road from N.H.Way – Molro road to village Haji Saeed Khan Sohu mile 0/0 – 0/6 ½” (1.30 Km) |
| --- | --- |

(Schedule “B”)

| QTY: | ITEM OF WORK | RATE | UNIT |  | AMOUNT |
| --- | --- | --- | --- | --- | --- |
|  | 1/-Excavation in foundation of building bridges & other structure i/cs: degbelling dressing refilling around the structure lead upto 100’& lift upto 5.0’in ordinary soil. |  |  |  |  |
| 1250Cft: |  | @Rs:3716/25 |  | P.% 0 Cft: | Rs:3970/= |
|  | 2/-Cement concrete brick or stone ballast of 1 ½”-2”guage ratio (1:4:8) |  |  |  |  |
| 245 Cft: |  | @Rs:9416/28 |  | P. % Cft: | Rs.23070/- |
|  | 3/-Pacca Brick work in foundation & plinth in cement send mortar ratio (1:4) |  |  |  |  |
| 357 Cft: |  | @Rs:12501/41 |  | P. % Cft: | Rs.44630/= |
|  | 4/-Cement concrete plain i/cs: placing compacting finishing & curing complete i/cs: screening & washing of stone aggregate without shuttering ratio (1:2:4) |  |  |  |  |
| 21 Cft: |  | @Rs:14429/25 |  | P. % Cft: | Rs.3030/= |
|  | 5/-Fabrication of mild steel reinforcement for cement concrete i/cs: cutting bending lying in position making joints & fastening i/cs: cost of mild steel & binding wire also rust from bars. |  |  |  |  |
| 6.82 Cwt |  | @Rs.4820/20 |  | P. Cwt: | Rs.32922/= |
|  | 6/-Rienforced cement concrete work i/cs: all labor & materials except the cost of steel reinforcement & its labor for bending & binding which will be paid separately. This rate also i/cs: all kinds of forms moulds lifting shuttering curing rending & finishing surface (I/cs: screening & washing of shingle) RCC work in roof slabs beams columns rafts & other structure member laid in situ or precuts in position complete in all respect ratio (1:2:4) 90 lbs: cement 2 cft: sand 4 cft: shingle 4/8”-1/4”gauge. |  |  |  |  |
| 157 Cft: |  | @ Rs.337/= |  | P. Cft: | Rs.52909/= |
|  | 7/-Cement plaster (1:3) upto 20’height ½”thick. |  |  |  |  |
| 198 Sft: |  | @ Rs.2344/59 |  | P. % Sft: | Rs.4642/= |
|  |  | Total |  |  | Rs.165173/ |
|  |  | Therefore will be 05 Nos |  |  | Rs.825865/ |

| NOTE: Subject to any Changes/ Variation in Rates/ Quantity as per T.Sanction. |  |
| --- | --- |
| # CONTRACTOR | EXECUTIVE ENGINEER HIGHWAY DIVISION # KHAIRPUR |

PART’C’ SCHEME BOARD

| ## NAME OF SCHEME:- | Construction of road from N.H.Way – Molro road to village Haji Saeed Khan Sohu mile 0/0 – 0/6 ½” (1.30 Km) |
| --- | --- |

(Schedule “B”)

| QTY: | ITEM OF WORK | RATE | UNIT |  | AMOUNT |
| --- | --- | --- | --- | --- | --- |
|  | 1/-Providing of Scheme board of M.S Sheet of 3 ½”x3’x3’ of 1/8”guage and welding with 2”dia pipe around the sheet & for stand 2 Nos: i/cs: cutting painting writing & fixing at site in situ (1:3:6) plate from (2’x2’x2) etc: complete. |  |  |  |  |
| 01 No: |  | @Rs:16956/- |  | P.Each | Rs:16956/- |
|  |  | Total |  |  | Rs.16956/- |

| NOTE: Subject to any Changes/ Variation in Rates/ Quantity as per T.Sanction. |  |
| --- | --- |
| # CONTRACTOR | EXECUTIVE ENGINEER HIGHWAY DIVISION # KHAIRPUR |