(SCHEDULE B )

(PART “A” ROAD WORK)

NAME OF SCHEME: - CONST: OF ROAD AND  PAVING BLOCK

OF MAIN ROAD OF SAGYOON CITY U/C SAGYOON TALUKA

SOBHODERO DISTT: KHAIRPUR

| QTY | ITEM OF WORK | RATE. | UNIT | AMOUNT |
| --- | --- | --- | --- | --- |

2/-Prepariong Sub –Base course by supplying & spreading stone

metal 1-1/2’’ gauge of approved quality from approved quarry in

required thickness to proper camber & grade i/cs: hand packing

mixed with shingle pit run gravel fines having P.I of not more than

then 6% in the ratio 2:3 including watering and compacting to achieve

98-100% density as per modified AASHO Specification (Rate i/cs:

all cost of materials T&P& carriage up to 3 chains  )

(S.I No:8 P/No:3)

21413          Cft:             @Rs: 5557/57              P.% cft:                  Rs.1190042/=

2/-Preparing Base course i/cs: supplying & spreading stone

metal of approved quality properly grade to maximum size of

1-1/2 in required thickness to proper camber & grade i/cs:

supplying & spreading 15 cft: screening & Non plastic quarry

fines filling depression with stone metal after initial rolling

i/cs: watering & compacting the same so as to achieve 100%

density as per modified AASHO Specification.(Rate i/cs: all cost

of materials T&P& carriage up to site of work.

6100           Cft:             @Rs: 7750/21                P.% cft:                 Rs. 472762/=

3./Providing Surface dressing  Two Coat on new or existing surface

With30+18 = 48  lbs: of bitumen of 80/100 penetration and 4.0+1.5 = 5.50  cft:

Crusher bajri with i/cs: cleaning the road surface rolling etc complete. Rate i/c

all cost of materials T&P carriage up to site of work.

12200     Sft:              @Rs:2899/84                    P.% Sft:               Rs.353780/=

5/-Providing I ½ “  thick (consolidate e) Premixed Carpet in proper/

camber & grade i/cs: supplying of 15 Cft: Crush bajri 5 cft:

Hill Sand (of approved quality and grade) with 93  lbs: bitumen

of 80/100 penetration i/cs: fixing in mechanical mixer in required

i/c mixing the material T&P& carriage up to site of work.

12200      Sft:              @Rs:6399/74                    P.%  Sft:                      Rs.780768/=

_____________

Total Rs: 2797352/=

8)       Decreased Cost of Bitumen

8.06     Tons             @Rs: 17595/=              P.ton          (-)        Rs:141815/=

______________

G.Total  Rs:2655537/=

Note:- Subject to any changes/

Variation quantities/rates in

the technical Sanction

(CONTRACTOR)                                        EXECUTIVE ENGINEER

HIGHWAYS DIVISION

KHAIRPUR

(SCHEDULE B)

PART”C”SCHEME BOARD

NAME OF SCHEME: - CONST: OF ROAD AND  PAVING BLOCK

OF MAIN ROAD OF SAGYOON CITY U/C SAGYOON TALUKA

SOBHODERO DISTT: KHAIRPUR

| Quality | Item of Work | Rate | Unit | Amount. |
| --- | --- | --- | --- | --- |

1.  Supply of Scheme board of M.S Sheet of 3 ¼ ‘’-2’x3’ of 1/8’’ guage and

wielding with 2’’ dia pipe around the sheet and for Stand 2 Nos: including

cutting painting writing and fixing at site in situ (1:3:6) plate from (2’x2’x2’) etc

complete .

01    No:                           @Rs: 16956/=                       Rs: 16956/=

__________

Total Rs: 16956/=

Note:- Subject to any changes/

Variation quantities/rates in

the technical Sanction

(CONTRACTOR)                                         EXECUTIVE ENGINEER

HIGHWAYS DIVISION

KHAIRPUR

PART”B”MASSONARY STRUCTURE

DRAIN NALI

NAME OF SCHEME: - CONST: OF ROAD AND  PAVING BLOCK

OF MAIN ROAD OF SAGYOON CITY U/C SAGYOON TALUKA

SOBHODERO DISTT: KHAIRPUR

| QTY | ITEM OF WORK | RATE. | UNIT | AMOUNT |
| --- | --- | --- | --- | --- |

1. Excavation in foundation of building bridges and other structure including

Dig belling dressing refilling around structure with excavated earth watering and

Ramming lead up to one chain lift up to 5.o’ in ordinary soil.

48945       Cft:                                     @Rs: 3176/25            P.%0 Cft:                              Rs. 1654571/=

2. Cement concrete brick in stone ballast 1-1/2 to 1:4:8. (Hill / sand to be used)

677          Cft:                                    @Rs: 9416/28                P.%cft:                                Rs. 63748/=

3. Pacca brick work in foundation in plinth i1:4.

2250           Cft:                                 @Rs: 12501/41               P.%cft:                               Rs.  281281/=

4. Const: of standard of open drain countee Block.

Type I Drain  500  Cft:                  @Rs: 94/=    P.cft:                 Rs: 47000/=

Type  II  Drain  500  Cft:                @Rs: 174/=  P.cft:                 Rs: 87000/=

4.Cement Concrete plain i/c plashing compacting finishing and curing complete (i/c

screening and washing of stone aggregate without shuttering Ratio (1:2:4).

50             Cft:                                  @Rs: 14429/25     P.%cft:                                       Rs. 7214/=

7. Cement plaster (1:3) up to 20’hieght ½ “ thick.

3000             Sft:                                 @Rs: 2344/59      P.% Sft:                                 Rs:  70337/=

6. R.C.C  Work including all labor and material   except the Cost of steel reinforcement

and its labor for bending and binding which will paid Separately. This rate also includes

all kinds of forms mould lifting shuttering curing rendering and finishing the exposed

surface (including screening and washing of shingle)(a)RCC works in roof slab, beams

columns rafts lintels and other structural member laid  in situ recast laid in position

complete in all respect ratio (1:2:4)  90 lbs: cement 2cft:sand 4cft:shingle 4/8’’guage.

90    Cft:               @Rs: 337/=           P.Cft:                                  Rs: 30330/=

5. Fabrication of mild steel reinforcement for cement concrete including cutting bending,

laying in position making joints and fastenings including cost of binding wire also

includes removal of rust from bars.

3.61   Cwt:                               @Rs: 5001/70          P.% Cwt:                       Rs:18056/=

_____________

Total Rs: 608741/=

Note:- Subject to any changes/

Variation quantities/rates in

the technical Sanction

(CONTRACTOR)                                                              EXECUTIVE ENGINEER

HIGHWAYS DIVISION

KHAIRPUR

(SCHEDULE B )

(PART “A” ROAD WORK)

NAME OF SCHEME: - CONST: OF ROAD AND  PAVING BLOCK

OF MAIN ROAD OF SAGYOON CITY U/C SAGYOON TALUKA

SOBHODERO DISTT: KHAIRPUR

| QTY | ITEM OF WORK | RATE. | UNIT | AMOUNT |
| --- | --- | --- | --- | --- |

1/-Prepariong Sub –Base course by supplying & spreading stone

metal 1-1/2’’ gauge of approved quality from approved quarry in

required thickness to proper camber & grade i/cs: hand packing

mixed with shingle pit run gravel fines having P.I of not more than

then 6% in the ratio 2:3 including watering and compacting to achieve

98-100% density as per modified AASHO Specification (Rate i/cs:

all cost of materials T&P& carriage up to 3 chains  )

48945       Cft:                   @Rs: 3380/43           P.% Cft:                     Rs.1654571/=

02. Cement concrete brick in stone ballast 1-1/2 to 2” gauge Ratio  1:4:8.

24472          Cft:                   @Rs: 9416.28          P.%cft:                        Rs. 2304352/=

03.Crushed fire bricks grated as sand (S.I.No. 51 P.No.04)

8320    Cft:                           @Rs: 2500/=             P.%cft:                        Rs. 208000/=

04. Supplying and filling Sand under floor and plugging in wells.(S.I.No29,P-31)

8320       Cft:                       @Rs: 151/25              P. %cft:                       Rs. 12584/=

07. Providing & fixing cement paving blocks flooring having size of 197x97x

60 (mm) of city/ quddra /cobble shape with natural colours, having strength

B/w 5000 psi to 8500 psi i/c filling the joints with hill sand and laying in specified

Manner/pattern and design etc complete.

48945               Sft:           @Rs: 199.77                 P.sft:                Rs.9777742/=

____________

Total Rs: 13957249/=

Note:- Subject to any changes/

Variation quantities/rates in

the technical Sanction

(CONTRACTOR)                                         EXECUTIVE ENGINEER

HIGHWAYS DIVISION

KHAIRPUR