(SCHEDULE B)

CONSTRUCTION OF WAITING SHADE AT BUS STOP VILLAGE USTAD NASEER MUHAMMAD NAREEJO TALUKA KINGRI DISTRICT KHAIRPUR

(QUANTITY OF BOQ)

| SR# | ITEM OF WORKS |  | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- |
| 01. | Excavation in foundation of buildings bridges and other structures including dag belling dressing refilling around structure with excavated earth watering and ramming lead upto 5ft. (b) in ordinary soil. (S.I.No.18 P.No.5). |  |  |  |  |  |
|  |  |  | 574.00 | 3176/25 | %0Cft | Rs.1823/- |
|  |  |  | (Rs. Three Thousand One Hundred Seventy Six Ps. Twenty Five Only). |  |  |  |
| 02. | Cement concrete brick or stone ballast 1-1/2 to 2 gauge. (A) ratio 1:5:10. (S.I.No.4 P.No.17). |  |  |  |  |  |
|  |  |  | 179.00 | 8694/95 | %Cft | Rs.15564/- |
|  |  |  | (Rs. Eight Thousand Six Hundred Ninety Four Ps. Ninety Five Only). |  |  |  |
| 03. | Pacca Brick work in foundation & plinth in cement & sand mortar Ratio 1:6 (S.I.No.4 (c) P.No.25). |  |  |  |  |  |
|  |  |  | 350.00 | 11948/36 | %Cft | Rs.41819/- |
|  |  |  | Rupees Eleven Thousand Nine Hundred Forty Eight Ps. Thirty Six Only. |  |  |  |
| 04. | Coursed Rubble masonry i/c hammer cement sand mortar ratio 1:6 (S.I.No.3 P.No.31). |  |  |  |  |  |
|  |  |  | 346.00 | 25321/- | %Cft | Rs.87610/- |
|  |  |  | (Rs. Twenty Five Thousand Three Hundred Twenty One Only). |  |  |  |
| 05. | R.C.C works in roof slab, beams columns rafts lintels and other structural members laid in situ recast laid in position complete in all respects ratio 1:2:4 90lbs cement 2cft sand 4cft shingle 1/8 to ¼ gauge (S.I.No.06 P.No.19). |  |  |  |  |  |
| (a) | Ground Floor |  | 426.00 | 337/- | %Cft | Rs.143562/- |
|  |  |  | Rupees Three Hundred Thirty Seven Only. |  |  |  |
| 06. | Fabrication of mild steel reinforcement for cement concrete including cutting bending, laying in position making joints and fastenings including cost of binding wire also including cost of binding wire also includes removal of rust from bars.(S.I.No.07 P.No.20). |  |  |  |  |  |
|  |  |  | 22.821 | 5001/70 | P.Cwt | Rs.114143/- |
|  |  |  | Rupees Five Thousand One Ps. Seventy Only. |  |  |  |
| 07. | Pacca brick work in Ground Floor and plinth in Ratio 1:6. (S.I.No.04 P.No.24). |  |  |  |  |  |
| (a) | Ground Floor |  | 595.00 | 12674/36 | %Cft | Rs.75412/- |
|  |  |  | Rupees Twelve Thousand Six Hundred Seventy Four Ps. Thirty Six Only. |  |  |  |

| 08. | Filling watering and ramming earth in floor with surplus earth from foundation lead upto one chain and lift upto 5ft.(S.I.NO:11 P-5) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 383.00 | 1512/50 | %0Cft | Rs.579/- |
|  |  |  | Rupees One Thousand Five Hundred Twelve Ps. Fifty Only. |  |  |  |
| 09. | Filling watering and ramming earth under floor with new excavated from outside lead upto one chain and lift upto 5feet. (S.I.No.12 P.No.5). |  |  |  |  |  |
|  |  |  | 661.00 | 3630/- | %0Cft | Rs.2399/- |
|  |  |  | Rupees Three Thousand Six Hundred Thirty Only. |  |  |  |

| SR# | ITEM OF WORKS |  | QTY: | RATE |  | UNIT |  | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10. | Extra lead for 3”mile. |  |  |  |  |  |  |  |
|  |  |  | 661.00 | 579/41 | %Cft |  | Rs.3830/- |  |
|  |  |  | Rupees Five Hundred Seventy Nine Ps. Forty One Only. |  |  |  |  |  |
| 11. | Cement concrete brick or stone ballast 1-1/2 to 2 guage ratio 1:6:12.(S.I.NO.4 P-14) |  |  |  |  |  |  |  |
|  |  |  | 87.00 | 8122/95 | %Cft |  | Rs.7067/- |  |
|  |  |  | Rupees.Eight thousand One Hundred Twenty Two Ps.Ninety Five |  |  |  |  |  |
| 12. | Cement plaster 1:6 upto 20 height 1/2”thick. (S.I.No. 13 P.No. 51 ). |  |  |  |  |  |  |  |
|  |  |  | 922.00 | 2260/06 |  | %Sft | Rs.20345/- |  |
|  |  |  | (Rs. Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only). |  |  |  |  |  |
| 13. | Cement plaster 1:4 upto 20 height 3/8”thick. (S.I.No.11 P.No.58). |  |  |  |  |  |  |  |
|  |  |  | 922.00 | 2197/52 |  | %Sft | Rs.20261/- |  |
|  |  |  | (Rs. Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only). |  |  |  |  |  |

| 14. | Cement concrete plain including placing compacting finishing and curing, complete (including screening and washing of stone aggregate without shuttering Ratio 1:2:4. (S.I.No.05 P.No.18). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 49.00 | 14429/- | %Cft | Rs.7070/- |
|  |  |  | Rupees Fourteen Thousand Four Hundred Twenty Nine Only. |  |  |  |

| 15. | Laying floor of approved coloured glazed tiles ¼” thick laid in white cement & 1:2 pigment on a bed of ¾” thick cement mortar 1:2 (S.I No. 25/P-42) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 387.00 | 27747/06 | %Sft | Rs.107381/- |
|  |  |  | (Rs. Two Thousand Seven Hundred Forty Seven Ps. Zero Six Only). |  |  |  |
| 16. | White glazed tile ¼ thick dado in white cement and laid over cement sand mortar ¾ thick including finishing. (S.I.NO. P-50). |  |  |  |  |  |
|  |  |  | 288.00 | 28253/61 | %Sft | Rs.81502/- |
|  |  |  | Rupees Twenty Eight Thousand Two Hundred Fifty Three Ps. Sixty One Only. |  |  |  |

| 17. | Primary coat of chalk under distemper. (S.I.No.23 P.No.59). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 922.00 | 442/72 | %Sft | Rs.4082/- |
|  |  |  | Rupees Four Hundred Forty Two Ps. Seventy Two Only. |  |  |  |
| 18. | Distemper in Three coats. (S.I.No.24 P.No.59). |  |  |  |  |  |
|  |  |  | 922.00 | 1079/65 | %Sft | Rs.9954/- |
|  |  |  | Rupees One Thousand Seventy Nine Ps. Sixty Five Only. |  |  |  |
| 19. | P/L tiles HALA or pattern tiles glazed 6"x6"x1/2" on floor or wall facing in required floor and pattern of STILE specification jointed in white cement and pigment over a base of 1:2 (SINO.61 P-48) |  |  |  |  |  |
|  |  |  | 219.00 | 47651/56 | %Sft | Rs.104356/- |
|  |  |  | Rupees Forty Seven Thousand Six Hundred Fifty One Ps. Fifty Six Only. |  |  |  |

| SR# | ITEM OF WORKS | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- |

| 20. | Cement Pointing i/c streaking of joints on wall ratio 1:3 (S.I.NO.16 P.No.21) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 522.00 | 1213/58 | %Sft | Rs.6335/- |
|  |  |  | Rupees One Thousand Two Hundred Thirteen Ps. Fifty Eight Only. |  |  |  |
| 21. | Colour wash two coats. |  |  |  |  |  |
|  |  |  | 522.00 | 859/90 | %Sft | Rs.4489/- |
|  |  |  | Rupees Eighty Hundred Fifty Nine Ps. Ninety Only. |  |  |  |
| 22. | Providing and fixing iron steel grill using solid square bars of size ½”x1/2” placed at 4” i/c and frame of flat iron patti of ¾”x3/4” i/c circle shape at 1-0 a part equivalent fitted with screws are pins i/c painting 3 coats with 1st coat of red oxide paint etc. (S.I.No.26 P.No.97). |  |  |  |  |  |
|  |  |  | 64.00 | 180/50 | P.Sft | Rs.11552/- |
|  |  |  | Rupees One Hundred Eighty Ps. Fifty Only. |  |  |  |

|  |  |  | TOTAL | Rs.871135/- |
| --- | --- | --- | --- | --- |

% Above / Below (Amount to be added / deducted).		Rs.

Grand Total		Rs.

Total Amount in word:

Note:-	 Quantities and rates are provisional and may be changed as per /according to the Technical Sanction by the competent authority.

| CONTRACTOR | EXECUTIVE ENGINEER (BUILDINGS) WORKS & SERVICES DEPARTMENT KHAIRPUR |
| --- | --- |

(SCHEDULE B)

CONSTRUCTION OF WAITING SHADE AT KHURA ROAD NEAR WHEAT GODOWN TALUKA KINGRI DISTRICT KHAIRPUR

(QUANTITY OF BOQ)

| SR# | ITEM OF WORKS |  | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- |
| 01. | Excavation in foundation of buildings bridges and other structures including dag belling dressing refilling around structure with excavated earth watering and ramming lead upto 5ft. (b) in ordinary soil. (S.I.No.18 P.No.5). |  |  |  |  |  |
|  |  |  | 574.00 | 3176/25 | %0Cft | Rs.1823/- |
|  |  |  | (Rs. Three Thousand One Hundred Seventy Six Ps. Twenty Five Only). |  |  |  |
| 02. | Cement concrete brick or stone ballast 1-1/2 to 2 gauge. (A) ratio 1:5:10. (S.I.No.4 P.No.17). |  |  |  |  |  |
|  |  |  | 179.00 | 8694/95 | %Cft | Rs.15564/- |
|  |  |  | (Rs. Eight Thousand Six Hundred Ninety Four Ps. Ninety Five Only). |  |  |  |
| 03. | Pacca Brick work in foundation & plinth in cement & sand mortar Ratio 1:6 (S.I.No.4 (c) P.No.25). |  |  |  |  |  |
|  |  |  | 350.00 | 11948/36 | %Cft | Rs.41819/- |
|  |  |  | Rupees Eleven Thousand Nine Hundred Forty Eight Ps. Thirty Six Only. |  |  |  |
| 04. | Coursed Rubble masonry i/c hammer cement sand mortar ratio 1:6 (S.I.No.3 P.No.31). |  |  |  |  |  |
|  |  |  | 346.00 | 25321/- | %Cft | Rs.87610/- |
|  |  |  | (Rs. Twenty Five Thousand Three Hundred Twenty One Only). |  |  |  |
| 05. | R.C.C works in roof slab, beams columns rafts lintels and other structural members laid in situ recast laid in position complete in all respects ratio 1:2:4 90lbs cement 2cft sand 4cft shingle 1/8 to ¼ gauge (S.I.No.06 P.No.19). |  |  |  |  |  |
| (a) | Ground Floor |  | 426.00 | 337/- | %Cft | Rs.143562/- |
|  |  |  | Rupees Three Hundred Thirty Seven Only. |  |  |  |
| 06. | Fabrication of mild steel reinforcement for cement concrete including cutting bending, laying in position making joints and fastenings including cost of binding wire also including cost of binding wire also includes removal of rust from bars.(S.I.No.07 P.No.20). |  |  |  |  |  |
|  |  |  | 22.821 | 5001/70 | P.Cwt | Rs.114143/- |
|  |  |  | Rupees Five Thousand One Ps. Seventy Only. |  |  |  |
| 07. | Pacca brick work in Ground Floor and plinth in Ratio 1:6. (S.I.No.04 P.No.24). |  |  |  |  |  |
| (a) | Ground Floor |  | 595.00 | 12674/36 | %Cft | Rs.75412/- |
|  |  |  | Rupees Twelve Thousand Six Hundred Seventy Four Ps. Thirty Six Only. |  |  |  |

| 08. | Filling watering and ramming earth in floor with surplus earth from foundation lead upto one chain and lift upto 5ft.(S.I.NO:11 P-5) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 383.00 | 1512/50 | %0Cft | Rs.579/- |
|  |  |  | Rupees One Thousand Five Hundred Twelve Ps. Fifty Only. |  |  |  |
| 09. | Filling watering and ramming earth under floor with new excavated from outside lead upto one chain and lift upto 5feet. (S.I.No.12 P.No.5). |  |  |  |  |  |
|  |  |  | 661.00 | 3630/- | %0Cft | Rs.2399/- |
|  |  |  | Rupees Three Thousand Six Hundred Thirty Only. |  |  |  |

| SR# | ITEM OF WORKS |  | QTY: | RATE |  | UNIT |  | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10. | Extra lead for 3”mile. |  |  |  |  |  |  |  |
|  |  |  | 661.00 | 579/41 | %Cft |  | Rs.3830/- |  |
|  |  |  | Rupees Five Hundred Seventy Nine Ps. Forty One Only. |  |  |  |  |  |
| 11. | Cement concrete brick or stone ballast 1-1/2 to 2 guage ratio 1:6:12.(S.I.NO.4 P-14) |  |  |  |  |  |  |  |
|  |  |  | 87.00 | 8122/95 | %Cft |  | Rs.7067/- |  |
|  |  |  | Rupees.Eight thousand One Hundred Twenty Two Ps.Ninety Five |  |  |  |  |  |
| 12. | Cement plaster 1:6 upto 20 height 1/2”thick. (S.I.No. 13 P.No. 51 ). |  |  |  |  |  |  |  |
|  |  |  | 922.00 | 2260/06 |  | %Sft | Rs.20345/- |  |
|  |  |  | (Rs. Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only). |  |  |  |  |  |
| 13. | Cement plaster 1:4 upto 20 height 3/8”thick. (S.I.No.11 P.No.58). |  |  |  |  |  |  |  |
|  |  |  | 922.00 | 2197/52 |  | %Sft | Rs.20261/- |  |
|  |  |  | (Rs. Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only). |  |  |  |  |  |

| 14. | Cement concrete plain including placing compacting finishing and curing, complete (including screening and washing of stone aggregate without shuttering Ratio 1:2:4. (S.I.No.05 P.No.18). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 49.00 | 14429/- | %Cft | Rs.7070/- |
|  |  |  | Rupees Fourteen Thousand Four Hundred Twenty Nine Only. |  |  |  |

| 15. | Laying floor of approved coloured glazed tiles ¼” thick laid in white cement & 1:2 pigment on a bed of ¾” thick cement mortar 1:2 (S.I No. 25/P-42) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 387.00 | 27747/06 | %Sft | Rs.107381/- |
|  |  |  | (Rs. Two Thousand Seven Hundred Forty Seven Ps. Zero Six Only). |  |  |  |
| 16. | White glazed tile ¼ thick dado in white cement and laid over cement sand mortar ¾ thick including finishing. (S.I.NO. P-50). |  |  |  |  |  |
|  |  |  | 288.00 | 28253/61 | %Sft | Rs.81502/- |
|  |  |  | Rupees Twenty Eight Thousand Two Hundred Fifty Three Ps. Sixty One Only. |  |  |  |

| 17. | Primary coat of chalk under distemper. (S.I.No.23 P.No.59). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 922.00 | 442/72 | %Sft | Rs.4082/- |
|  |  |  | Rupees Four Hundred Forty Two Ps. Seventy Two Only. |  |  |  |
| 18. | Distemper in Three coats. (S.I.No.24 P.No.59). |  |  |  |  |  |
|  |  |  | 922.00 | 1079/65 | %Sft | Rs.9954/- |
|  |  |  | Rupees One Thousand Seventy Nine Ps. Sixty Five Only. |  |  |  |
| 19. | P/L tiles HALA or pattern tiles glazed 6"x6"x1/2" on floor or wall facing in required floor and pattern of STILE specification jointed in white cement and pigment over a base of 1:2 (SINO.61 P-48) |  |  |  |  |  |
|  |  |  | 219.00 | 47651/56 | %Sft | Rs.104356/- |
|  |  |  | Rupees Forty Seven Thousand Six Hundred Fifty One Ps. Fifty Six Only. |  |  |  |

| 20. | Cement Pointing i/c streaking of joints on wall ratio 1:3 (S.I.NO.16 P.No.21) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 522.00 | 1213/58 | %Sft | Rs.6335/- |
|  |  |  | Rupees One Thousand Two Hundred Thirteen Ps. Fifty Eight Only. |  |  |  |

| SR# | ITEM OF WORKS |  | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- |
| 21. | Colour wash two coats. |  |  |  |  |  |
|  |  |  | 522.00 | 859/90 | %Sft | Rs.4489/- |
|  |  |  | Rupees Eighty Hundred Fifty Nine Ps. Ninety Only. |  |  |  |
| 22. | Providing and fixing iron steel grill using solid square bars of size ½”x1/2” placed at 4” i/c and frame of flat iron patti of ¾”x3/4” i/c circle shape at 1-0 a part equivalent fitted with screws are pins i/c painting 3 coats with 1st coat of red oxide paint etc. (S.I.No.26 P.No.97). |  |  |  |  |  |
|  |  |  | 64.00 | 180/50 | P.Sft | Rs.11552/- |
|  |  |  | Rupees One Hundred Eighty Ps. Fifty Only. |  |  |  |

|  |  |  | TOTAL | Rs.871135/- |
| --- | --- | --- | --- | --- |

% Above / Below (Amount to be added / deducted).		Rs.

Grand Total		Rs.

Total Amount in word:

Note:-	 Quantities and rates are provisional and may be changed as per /according to the Technical Sanction by the competent authority.

| CONTRACTOR | EXECUTIVE ENGINEER (BUILDINGS) WORKS & SERVICES DEPARTMENT KHAIRPUR |
| --- | --- |

(SCHEDULE B)

CONSTRUCTION OF COMPOUND WALL @ DARGAH MIAN AHMED SHAH TOWN AHMEDPUR TALUKA KINGRI DISTRICT KHAIRPUR

(QUANTITY OF BOQ)

| SR# | ITEM OF WORKS |  | QTY: | RATE | UNIT |  | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 01. | Excavation in foundation of buildings bridges and other structures including dag belling dressing refilling around structure with excavated earth watering and ramming lead upto 5ft. (b) in ordinary soil. (S.I.No.18 P.No.5). |  |  |  |  |  |  |
|  |  |  | 2008.00 | 3176/25 | %0Cft |  | Rs.6377/94 |
|  |  |  | (Rs. Three Thousand One Hundred Seventy Six Ps. Twenty Five Only). |  |  |  |  |
| 02. | Cement concrete brick or stone ballast 1-1/2 to 2 gauge. (A) ratio 1:5:10. (S.I.No.4 P.No.17). |  |  |  |  |  |  |
|  |  |  | 502.00 | 8694/95 | %Cft |  | Rs.43648/64 |
|  |  |  | (Rs. Eight Thousand Six Hundred Ninety Four Ps. Ninety Five Only). |  |  |  |  |
| 03. | Pacca Brick work in foundation & plinth in cement & sand mortar Ratio 1:6 (S.I.No.4 (c) P.No.25). |  |  |  |  |  |  |
|  |  |  | 1129.00 | 11948/36 | %Cft |  | Rs.134897/- |
|  |  |  | Rupees Eleven Thousand Nine Hundred Forty Eight Ps. Thirty Six Only. |  |  |  |  |
| 04. | Coursed Rubble masonry i/c hammer cement sand mortar ratio 1:6 (S.I.No.3 P.No.31). |  |  |  |  |  |  |
|  |  |  | 567.00 | 25321/- | %Cft |  | Rs.143570/07 |
|  |  |  | (Rs. Twenty Five Thousand Three Hundred Twenty One Only). |  |  |  |  |
| 05. | R.C.C works in roof slab, beams columns rafts lintels and other structural members laid in situ recast laid in position complete in all respects ratio 1:2:4 90lbs cement 2cft sand 4cft shingle 1/8 to ¼ gauge (S.I.No.06 P.No.19). |  |  |  |  |  |  |
| (a) | Ground Floor |  | 284.00 | 337/- | %Cft |  | Rs.95708/- |
|  |  |  | Rupees Three Hundred Thirty Seven Only. |  |  |  |  |
| 06. | Fabrication of mild steel reinforcement for cement concrete including cutting bending, laying in position making joints and fastenings including cost of binding wire also including cost of binding wire also includes removal of rust from bars.(S.I.No.07 P.No.20). |  |  |  |  |  |  |
|  |  |  | 12.678 | 5001/70 | P.Cwt |  | Rs.63411/55 |
|  |  |  | Rupees Five Thousand One Ps. Seventy Only. |  |  |  |  |
| 07. | Pacca brick work other than building in Ratio 1:6. (S.I.No.04 P.No.24). |  |  |  |  |  |  |
|  | C/wall |  | 1383.00 | 12346/65 | %Cft |  | Rs.170754/16 |
|  |  |  | Rupees Twelve Thousand Three Hundred Sixty Six Ps. Sixty Five Only. |  |  |  |  |
| 08. | Cement plaster 1:6 upto 20 height 1/2”thick. (S.I.No. 13 P.No. 51 ). |  |  |  |  |  |  |
|  |  |  | 1886.00 | 2260/06 | %Sft | Rs.41616/47 |  |
|  |  |  | (Rs. Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only). |  |  |  |  |
| 09. | Cement plaster 1:4 upto 20 height 3/8”thick. (S.I.No.11 P.No.58). |  |  |  |  |  |  |
|  |  |  | 1886.00 | 2197/52 | %Sft | Rs.41445/22 |  |
|  |  |  | (Rs. Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only). |  |  |  |  |

| SR# | ITEM OF WORKS | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- |

| 10. | White washing Three coats. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1757.00 | 829/50 | %Sft | Rs.14574/- |
|  |  |  | Rupees Eighty Hundred Fifty Nine Ps. Ninety Only. |  |  |  |
| 11. | Cement Pointing i/c streaking of joints on wall ratio 1:3 (S.I.NO.16 P.No.21) |  |  |  |  |  |
|  |  |  | 1757.00 | 1213/58 | %Sft | Rs.21322/60 |
|  |  |  | Rupees One Thousand Two Hundred Thirteen Ps. Fifty Eight Only. |  |  |  |
| 12. | Colour wash two coats. |  |  |  |  |  |
|  |  |  | 1757.00 | 859/90 | %Sft | Rs.15108/44 |
|  |  |  | Rupees Eighty Hundred Fifty Nine Ps. Ninety Only. |  |  |  |
| 13. | M/F steel grated doors complete with locking arrangement angloe iron frame 2"x2"x3.8" and 3/4" sq. bars 4" centre to centre (SINO.23 P-92) |  |  |  |  |  |
|  |  |  | 30.00 | 594/57 | P.Sft | Rs.17837/- |
|  |  |  | Rupees Five Hundred Ninety Four Ps. Fifty Seven Only. |  |  |  |
| 14. | Painting New surface ( d ) Preparing surface and painting guard bars gates of iron bars grating railing (including standards braces)And similar open work cats) (S.I.No.5(d) P-70) for Two Coats |  |  |  |  |  |
|  |  |  | 60.00 | 896/39 | P.Sft | Rs.537/83 |
|  |  |  | Rupees Eight Hundred Ninety Six Ps. Thirty Nine Only. |  |  |  |

|  |  |  | TOTAL | Rs.810809/- |
| --- | --- | --- | --- | --- |

% Above / Below (Amount to be added / deducted).		Rs.

Grand Total		Rs.

Total Amount in word:

Note:-	 Quantities and rates are provisional and may be changed as per /according to the Technical Sanction by the competent authority.

| CONTRACTOR | EXECUTIVE ENGINEER (BUILDINGS) WORKS & SERVICES DEPARTMENT KHAIRPUR |
| --- | --- |

(SCHEDULE B)

CONSTRUCTION OF COMPOUND WALL FOR DARGAH PIR SYED QASIM SHAH

TALUKA KINGRI DISTRICT KHAIRPUR

(QUANTITY OF BOQ)

| SR# | ITEM OF WORKS |  | QTY: | RATE | UNIT |  | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 01. | Excavation in foundation of buildings bridges and other structures including dag belling dressing refilling around structure with excavated earth watering and ramming lead upto 5ft. (b) in ordinary soil. (S.I.No.18 P.No.5). |  |  |  |  |  |  |
|  |  |  | 3960.00 | 3176/25 | %0Cft |  | Rs.12578/- |
|  |  |  | (Rs. Three Thousand One Hundred Seventy Six Ps. Twenty Five Only). |  |  |  |  |
| 02. | Cement concrete brick or stone ballast 1-1/2 to 2 gauge. (A) ratio 1:5:10. (S.I.No.4 P.No.17). |  |  |  |  |  |  |
|  |  |  | 990.00 | 8694/95 | %Cft |  | Rs.86080/- |
|  |  |  | (Rs. Eight Thousand Six Hundred Ninety Four Ps. Ninety Five Only). |  |  |  |  |
| 03. | Pacca Brick work in foundation & plinth in cement & sand mortar Ratio 1:6 (S.I.No.4 (c) P.No.25). |  |  |  |  |  |  |
|  |  |  | 2227.00 | 11948/36 | %Cft |  | Rs.266090/- |
|  |  |  | Rupees Eleven Thousand Nine Hundred Forty Eight Ps. Thirty Six Only. |  |  |  |  |
| 04. | Coursed Rubble masonry i/c hammer cement sand mortar ratio 1:6 (S.I.No.3 P.No.31). |  |  |  |  |  |  |
|  |  |  | 1119.00 | 25321/- | %Cft |  | Rs.283342/ |
|  |  |  | (Rs. Twenty Five Thousand Three Hundred Twenty One Only). |  |  |  |  |
| 05. | R.C.C works in roof slab, beams columns rafts lintels and other structural members laid in situ recast laid in position complete in all respects ratio 1:2:4 90lbs cement 2cft sand 4cft shingle 1/8 to ¼ gauge (S.I.No.06 P.No.19). |  |  |  |  |  |  |
| (a) | Ground Floor |  | 559.00 | 337/- | %Cft |  | Rs.188383/- |
|  |  |  | Rupees Three Hundred Thirty Seven Only. |  |  |  |  |
| 06. | Fabrication of mild steel reinforcement for cement concrete including cutting bending, laying in position making joints and fastenings including cost of binding wire also including cost of binding wire also includes removal of rust from bars.(S.I.No.07 P.No.20). |  |  |  |  |  |  |
|  |  |  | 24.955 | 5001/70 | P.Cwt |  | Rs.124817/- |
|  |  |  | Rupees Five Thousand One Ps. Seventy Only. |  |  |  |  |
| 07. | Pacca brick work other than building in Ratio 1:6. (S.I.No.04 P.No.24). |  |  |  |  |  |  |
|  | C/wall |  | 2902.00 | 12346/65 | %Cft |  | Rs.358300 |
|  |  |  | Rupees Twelve Thousand Three Hundred Sixty Six Ps. Sixty Five Only. |  |  |  |  |
| 08. | Cement plaster 1:6 upto 20 height 1/2”thick. (S.I.No. 13 P.No. 51 ). |  |  |  |  |  |  |
|  |  |  | 4270.00 | 2260/06 | %Sft | Rs.94222/- |  |
|  |  |  | (Rs. Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only). |  |  |  |  |
| 09. | Cement plaster 1:4 upto 20 height 3/8”thick. (S.I.No.11 P.No.58). |  |  |  |  |  |  |
|  |  |  | 4270.00 | 2197/52 | %Sft | Rs.93834/- |  |
|  |  |  | (Rs. Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only). |  |  |  |  |

| SR# | ITEM OF WORKS | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- |

| 11. | Cement Pointing i/c streaking of joints on wall ratio 1:3 (S.I.NO.16 P.No.21) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 3712.00 | 1213/58 | %Sft | Rs.45064/- |
|  |  |  | Rupees One Thousand Two Hundred Thirteen Ps. Fifty Eight Only. |  |  |  |
| 12. | Colour wash two coats. |  |  |  |  |  |
|  |  |  | 3712.00 | 859/90 | %Sft | Rs.31919/- |
|  |  |  | Rupees Eight Hundred Fifty Nine Ps. Ninety Only. |  |  |  |
| 13. | M/F steel grated doors complete with locking arrangement angloe iron frame 2"x2"x3.8" and 3/4" sq. bars 4" centre to centre (SINO.23 P-92) |  |  |  |  |  |
|  |  |  | 40.00 | 726/72 | P.Sft | Rs.29069/- |
|  |  |  | Rupees Seven Hundred Twenty Six Ps. Seventy Five Only. |  |  |  |
| 14. | Painting New surface(a) Preparing surface painting corrugated surface, patent roofing etc. (SINO.5(a+b) P-69) for Two Coats |  |  |  |  |  |
|  |  |  | 80.00 | 1489/68 | P.Sft | Rs.1192/- |
|  |  |  | Rupees One Thousand Four Hundred Eighty Nine Ps. Sixty Eight Only. |  |  |  |

|  |  |  | TOTAL | Rs.1614890/- |
| --- | --- | --- | --- | --- |

% Above / Below (Amount to be added / deducted).		Rs.

Grand Total		Rs.

Total Amount in word:

Note:-	 Quantities and rates are provisional and may be changed as per /according to the Technical Sanction by the competent authority.

| CONTRACTOR | EXECUTIVE ENGINEER (BUILDINGS) WORKS & SERVICES DEPARTMENT KHAIRPUR |
| --- | --- |

(SCHEDULE B)

CONSTRUCTION OF COMPOUND WALL FOR DARGAH DINAL SHAH MASOOM

AT KOLAB JEAL TALUKA KINGRI DISTRICT KHAIRPUR

(QUANTITY OF BOQ)

PART A COMPOUND WALL

| SR# | ITEM OF WORKS |  | QTY: | RATE | UNIT |  | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 01. | Excavation in foundation of buildings bridges and other structures including dag belling dressing refilling around structure with excavated earth watering and ramming lead upto 5ft. (b) in ordinary soil. (S.I.No.18 P.No.5). |  |  |  |  |  |  |
|  |  |  | 4400.00 | 3176/25 | %0Cft |  | Rs.13976/- |
|  |  |  | (Rs. Three Thousand One Hundred Seventy Six Ps. Twenty Five Only). |  |  |  |  |
| 02. | Cement concrete brick or stone ballast 1-1/2 to 2 gauge. (A) ratio 1:5:10. (S.I.No.4 P.No.17). |  |  |  |  |  |  |
|  |  |  | 1100.00 | 8694/95 | %Cft |  | Rs.95644/- |
|  |  |  | (Rs. Eight Thousand Six Hundred Ninety Four Ps. Ninety Five Only). |  |  |  |  |
| 03. | Pacca Brick work in foundation & plinth in cement & sand mortar Ratio 1:6 (S.I.No.4 (c) P.No.25). |  |  |  |  |  |  |
|  |  |  | 2476.00 | 11948/36 | %Cft |  | Rs.295841/- |
|  |  |  | Rupees Eleven Thousand Nine Hundred Forty Eight Ps. Thirty Six Only. |  |  |  |  |
| 04. | Coursed Rubble masonry i/c hammer cement sand mortar ratio 1:6 (S.I.No.3 P.No.31). |  |  |  |  |  |  |
|  |  |  | 1243.00 | 25321/- | %Cft |  | Rs.314740/ |
|  |  |  | (Rs. Twenty Five Thousand Three Hundred Twenty One Only). |  |  |  |  |
| 05. | R.C.C works in roof slab, beams columns rafts lintels and other structural members laid in situ recast laid in position complete in all respects ratio 1:2:4 90lbs cement 2cft sand 4cft shingle 1/8 to ¼ gauge (S.I.No.06 P.No.19). |  |  |  |  |  |  |
| (a) | Ground Floor |  | 662.00 | 337/- | %Cft |  | Rs.223094/- |
|  |  |  | Rupees Three Hundred Thirty Seven Only. |  |  |  |  |
| 06. | Fabrication of mild steel reinforcement for cement concrete including cutting bending, laying in position making joints and fastenings including cost of binding wire also including cost of binding wire also includes removal of rust from bars.(S.I.No.07 P.No.20). |  |  |  |  |  |  |
|  |  |  | 29.553 | 5001/70 | P.Cwt |  | Rs.147815/- |
|  |  |  | Rupees Five Thousand One Ps. Seventy Only. |  |  |  |  |
| 07. | Pacca brick work other than building in Ratio 1:6. (S.I.No.04 P.No.24). |  |  |  |  |  |  |
|  | C/wall |  | 3424.00 | 12346/65 | %Cft |  | Rs.422749 |
|  |  |  | Rupees Twelve Thousand Three Hundred Sixty Six Ps. Sixty Five Only. |  |  |  |  |
| 08. | Cement plaster 1:6 upto 20 height 1/2”thick. (S.I.No. 13 P.No. 51 ). |  |  |  |  |  |  |
|  |  |  | 4646.00 | 2260/06 | %Sft | Rs.102519/- |  |
|  |  |  | (Rs. Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only). |  |  |  |  |
| 09. | Cement plaster 1:4 upto 20 height 3/8”thick. (S.I.No.11 P.No.58). |  |  |  |  |  |  |
|  |  |  | 4646.00 | 2197/52 | %Sft | Rs.102097/- |  |
|  |  |  | (Rs. Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only). |  |  |  |  |

| SR# | ITEM OF WORKS |  | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- |
| 10. | Cement Pointing i/c streaking of joints on wall ratio 1:3 (S.I.NO.16 P.No.21) |  |  |  |  |  |
|  |  |  | 4320.00 | 1213/58 | %Sft | Rs.52427/- |
|  |  |  | Rupees One Thousand Two Hundred Thirteen Ps. Fifty Eight Only. |  |  |  |
| 11. | Colour wash two coats. |  |  |  |  |  |
|  |  |  | 4320.00 | 859/90 | %Sft | Rs.37148/- |
|  |  |  | Rupees Eight Hundred Fifty Nine Ps. Ninety Only. |  |  |  |
| 12. | M/F steel grated doors complete with locking arrangement angloe iron frame 2"x2"x3.8" and 3/4" sq. bars 4" centre to centre (SINO.23 P-92) |  |  |  |  |  |
|  |  |  | 42.00 | 726/72 | P.Sft | Rs.30522/- |
|  |  |  | Rupees Seven Hundred Twenty Six Ps. Seventy Five Only. |  |  |  |
| 13. | Painting New surface(a) Preparing surface painting corrugated surface, patent roofing etc. (SINO.5(a+b) P-69) for Two Coats |  |  |  |  |  |
|  |  |  | 84.00 | 1489/68 | P.Sft | Rs.1251/- |
|  |  |  | Rupees One Thousand Four Hundred Eighty Nine Ps. Sixty Eight Only. |  |  |  |

|  |  |  | TOTAL | Rs.1839823/- |
| --- | --- | --- | --- | --- |

% Above / Below (Amount to be added / deducted).		Rs.

Grand Total		Rs.

Total Amount in word:

Note:-	 Quantities and rates are provisional and may be changed as per /according to the Technical Sanction by the competent authority.

| CONTRACTOR | EXECUTIVE ENGINEER (BUILDINGS) WORKS & SERVICES DEPARTMENT KHAIRPUR |
| --- | --- |

(SCHEDULE B)

CONSTRUCTION OF COMPOUND WALL FOR DARGAH DINAL SHAH MASOOM

AT KOLAB JEAL TALUKA KINGRI DISTRICT KHAIRPUR

PART B Repair work of Dargah

| SR# | ITEM OF WORKS |  | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- |
| 01. | Removing cement or lime plaster from walls (S.I.No.53 P.No.13). |  |  |  |  |  |
|  |  |  | 3375.00 | 121/- | %Sft | Rs.4062/- |
|  |  |  | (Rupees . One Hundred Twenty One Only) . |  |  |  |
| 02. | Applying floating coat of cement (S.N 14 P.No 22) |  |  |  |  |  |
|  |  |  | 3375.00 | 660/- | %Sft | Rs.22275/- |
|  |  |  | (Rupees. Six Hundred Sixty Only). |  |  |  |
| 03. | Cement plaster 1:6 upto 20 height 1/2”thick. (S.I.No. 13 P.No. 51 ). |  |  |  |  |  |
|  |  |  | 3375.00 | 2260/06 | %Sft | Rs.74472/- |
|  |  |  | (Rs. Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only). |  |  |  |
| 04. | Cement plaster 1:4 upto 20 height 3/8”thick. (S.I.No.11 P.No.58). |  |  |  |  |  |
|  |  |  | 3375.00 | 2197/52 | %Sft | Rs.74166/- |
|  |  |  | (Rs. Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only). |  |  |  |

| 05. | Primary coat of chalk under distemper.(S.NO.23 P-54) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 3375.00 | 442/75 | %Sft | Rs.14942/- |
|  |  |  | Rupees Four Hundred Forty Two Ps. Seventy Five Only. |  |  |  |
| 06. | Distemper in three coats.,(S.I.NO. P- ) |  |  |  |  |  |
|  |  |  | 3375.00 | 1079/65 | %Sft | Rs.36438/- |
|  |  |  | Rupees One Thousand Seventy Nine Ps. Sixty Five Only. |  |  |  |
| 07. | Marble Polishing old floor (S.NO P ) |  |  |  |  |  |
|  |  |  | 256.00 | 36/60 | P.Sft | Rs.9370/- |
|  |  |  | Rupees Thirty Six Ps. Sixty Only. |  |  |  |
|  |  |  | TOTAL |  |  | Rs.235725/- |

% Above / Below (Amount to be added / deducted).		Rs.

Grand Total		Rs.

Total Amount in word:

Note:-	 Quantities and rates are provisional and may be changed as per /according to the Technical Sanction by the competent authority.

| CONTRACTOR | EXECUTIVE ENGINEER (BUILDINGS) WORKS & SERVICES DEPARTMENT KHAIRPUR |
| --- | --- |

(SCHEDULE B)

CONSTRUCTOIN OR COMPOUND WALL EIDGAH AT KHABAR FAKIR U/C KOLAB JEAL TALUKA KINGRI DISTRICT KHAIRPUR

PART A COMPOUND WALL

| SR# | ITEM OF WORKS |  | QTY: | RATE | UNIT |  | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 01. | Excavation in foundation of buildings bridges and other structures including dag belling dressing refilling around structure with excavated earth watering and ramming lead upto 5ft. (b) in ordinary soil. (S.I.No.18 P.No.5). |  |  |  |  |  |  |
|  |  |  | 1520.00 | 3176/25 | %0Cft |  | Rs.4828/- |
|  |  |  | (Rs. Three Thousand One Hundred Seventy Six Ps. Twenty Five Only). |  |  |  |  |
| 02. | Cement concrete brick or stone ballast 1-1/2 to 2 gauge. (A) ratio 1:5:10. (S.I.No.4 P.No.17). |  |  |  |  |  |  |
|  |  |  | 380.00 | 8694/95 | %Cft |  | Rs.33041/- |
|  |  |  | (Rs. Eight Thousand Six Hundred Ninety Four Ps. Ninety Five Only). |  |  |  |  |
| 03. | Pacca Brick work in foundation & plinth in cement & sand mortar Ratio 1:6 (S.I.No.4 (c) P.No.25). |  |  |  |  |  |  |
|  |  |  | 1286.00 | 11948/36 | %Cft |  | Rs.153657/- |
|  |  |  | Rupees Eleven Thousand Nine Hundred Forty Eight Ps. Thirty Six Only. |  |  |  |  |
| 05. | R.C.C works in roof slab, beams columns rafts lintels and other structural members laid in situ recast laid in position complete in all respects ratio 1:2:4 90lbs cement 2cft sand 4cft shingle 1/8 to ¼ gauge (S.I.No.06 P.No.19). |  |  |  |  |  |  |
| (a) | Ground Floor |  | 216.00 | 337/- | %Cft |  | Rs.72792/- |
|  |  |  | Rupees Three Hundred Thirty Seven Only. |  |  |  |  |
| 06. | Fabrication of mild steel reinforcement for cement concrete including cutting bending, laying in position making joints and fastenings including cost of binding wire also including cost of binding wire also includes removal of rust from bars.(S.I.No.07 P.No.20). |  |  |  |  |  |  |
|  |  |  | 9.642 | 5001/70 | P.Cwt |  | Rs.48226/- |
|  |  |  | Rupees Five Thousand One Ps. Seventy Only. |  |  |  |  |
| 07. | Pacca brick work other than building in Ratio 1:6. (S.I.No.04 P.No.24). |  |  |  |  |  |  |
|  | C/wall |  | 1028.00 | 12346/65 | %Cft |  | Rs.126924 |
|  |  |  | Rupees Twelve Thousand Three Hundred Sixty Six Ps. Sixty Five Only. |  |  |  |  |
| 08. | Cement plaster 1:6 upto 20 height 1/2”thick. (S.I.No. 13 P.No. 51 ). |  |  |  |  |  |  |
|  |  |  | 1393.00 | 2260/06 | %Sft | Rs.30738/- |  |
|  |  |  | (Rs. Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only). |  |  |  |  |
| 09. | Cement plaster 1:4 upto 20 height 3/8”thick. (S.I.No.11 P.No.58). |  |  |  |  |  |  |
|  |  |  | 1393.00 | 2197/52 | %Sft | Rs.30611/- |  |
|  |  |  | (Rs. Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only). |  |  |  |  |

| SR# | ITEM OF WORKS | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- |

| 10. | Cement Pointing i/c streaking of joints on wall ratio 1:3 (S.I.NO.16 P.No.21) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1295.00 | 1213/58 | %Sft | Rs.15716/- |
|  |  |  | Rupees One Thousand Two Hundred Thirteen Ps. Fifty Eight Only. |  |  |  |
| 11. | Colour wash two coats. |  |  |  |  |  |
|  |  |  | 1295.00 | 859/90 | %Sft | Rs.11136/- |
|  |  |  | Rupees Eight Hundred Fifty Nine Ps. Ninety Only. |  |  |  |
| 12. | M/F steel grated doors complete with locking arrangement angloe iron frame 2"x2"x3.8" and 3/4" sq. bars 4" centre to centre (SINO.23 P-92) |  |  |  |  |  |
|  |  |  | 35.00 | 726/72 | P.Sft | Rs.25435/- |
|  |  |  | Rupees Seven Hundred Twenty Six Ps. Seventy Five Only. |  |  |  |
| 13. | Painting New surface(a) Preparing surface painting corrugated surface, patent roofing etc. (SINO.5(a+b) P-69) for Two Coats |  |  |  |  |  |
|  |  |  | 35.00 | 1489/68 | P.Sft | Rs.1043/- |
|  |  |  | Rupees One Thousand Four Hundred Eighty Nine Ps. Sixty Eight Only. |  |  |  |

|  |  |  | TOTAL | Rs.554147/- |
| --- | --- | --- | --- | --- |

% Above / Below (Amount to be added / deducted).		Rs.

Grand Total		Rs.

Total Amount in word:

Note:-	 Quantities and rates are provisional and may be changed as per /according to the Technical Sanction by the competent authority.

| CONTRACTOR | EXECUTIVE ENGINEER (BUILDINGS) WORKS & SERVICES DEPARTMENT KHAIRPUR |
| --- | --- |

(SCHEDULE B)

CONSTRUCTOIN OR COMPOUND WALL EIDGAH AT KHABAR FAKIR U/C KOLAB JEAL TALUKA KINGRI DISTRICT KHAIRPUR

PART B EID GAH FLOORING

| SR# | ITEM OF WORKS |  | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- |
| 01. | Cement concrete brick or stone ballast 1-1/2 to 2 gauge. (A) ratio 1:5:10. (S.I.No.4 P.No.17). |  |  |  |  |  |
|  |  |  | 1044.00 | 8694/95 | %Cft | Rs.90775/- |
|  |  |  | (Rs. Eight Thousand Six Hundred Ninety Four Ps. Ninety Five Only). |  |  |  |
| 02. | Cement concrete plain including placing compacting finishing and curing, complete (including screening and washing of stone aggregate without shuttering Ratio 1:3:6. (S.I.No.05 P.No.18). |  |  |  |  |  |
|  |  |  | 538.00 | 14429/25- | %Cft | Rs.77629/- |
|  |  |  | Rupees Fourteen Thousand Four Hundred Twenty Nine Only. |  |  |  |
| 03. | P/F cement paving blocks flooring having size of 197x97x60(mm) of city/Quddra / cobble shape with natural colours, having strength b/w 5000 psi to 8500 psi i/c filling the joints with hill sand and laying specified manner / pattern and design etc complete (SN:72 P-49) |  |  |  |  |  |
|  |  |  | 3163.00 | 223/97- | 1Sft | Rs.708417/- |
|  |  |  | Rupees Three Hundred Thirty Seven Only. |  |  |  |
|  |  |  |  | TOTAL |  | Rs.876821/- |

% Above / Below (Amount to be added / deducted).		Rs.

Grand Total		Rs.

Total Amount in word:

Note:-	 Quantities and rates are provisional and may be changed as per /according to the Technical Sanction by the competent authority.

| CONTRACTOR | EXECUTIVE ENGINEER (BUILDINGS) WORKS & SERVICES DEPARTMENT KHAIRPUR |
| --- | --- |

(SCHEDULE B)

CONSTRUCTOIN OR COMPOUND WALL EIDGAH AT KHABAR FAKIR U/C KOLAB JEAL TALUKA KINGRI DISTRICT KHAIRPUR

PART C EARTH FILLING

| SR# | ITEM OF WORKS | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- |

| 01. | Filling watering and ramming earth under floor with new excavated from outside lead upto one chain and lift upto 5ft (S.I.No.12 P.No.5). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1668.00 | 3630/- | %0Cft | Rs.58806/- |
|  |  |  | Rupees Three Thousand Six Hundred Thirty Only. |  |  |  |
| 02. | Extra lead for 3”mile. |  |  |  |  |  |
|  |  |  | 16200.00 | 579/41 | %Cft | Rs.93865/- |
|  |  |  | Rupees Five Hundred Seventy Nine Ps. Forty One Only. |  |  |  |

| 03. | Dressing and leveling of earth work to designed Section etc completed in ordinary soil. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 16200.00 | 187/55- | %Cft | Rs.30383/- |
|  |  |  | Rupees One Hundred Eighty Seven Ps. Fifty Five Only. |  |  |  |
|  |  |  |  | TOTAL |  | Rs.183054/- |

% Above / Below (Amount to be added / deducted).		Rs.

Grand Total		Rs.

Total Amount in word:

Note:-	 Quantities and rates are provisional and may be changed as per /according to the Technical Sanction by the competent authority.

| CONTRACTOR | EXECUTIVE ENGINEER (BUILDINGS) WORKS & SERVICES DEPARTMENT KHAIRPUR |
| --- | --- |

(SCHEDULE B)

CONSTRUCTION OF COMPOUND WALL OF EID GAH AT VILLAGE KHOOH WALI MUHAMMAD U/C WADA MAHEESAR  TALUKA KINGRI DISTRICT KHAIRPUR

PART A COMPOUND WALL

| SR# | ITEM OF WORKS |  | QTY: | RATE | UNIT |  | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 01. | Excavation in foundation of buildings bridges and other structures including dag belling dressing refilling around structure with excavated earth watering and ramming lead upto 5ft. (b) in ordinary soil. (S.I.No.18 P.No.5). |  |  |  |  |  |  |
|  |  |  | 1840.00 | 3176/25 | %0Cft |  | Rs.8544/- |
|  |  |  | (Rs. Three Thousand One Hundred Seventy Six Ps. Twenty Five Only). |  |  |  |  |
| 02. | Cement concrete brick or stone ballast 1-1/2 to 2 gauge. (A) ratio 1:5:10. (S.I.No.4 P.No.17). |  |  |  |  |  |  |
|  |  |  | 460.00 | 8694/95 | %Cft |  | Rs.39997/- |
|  |  |  | (Rs. Eight Thousand Six Hundred Ninety Four Ps. Ninety Five Only). |  |  |  |  |
| 03. | Pacca Brick work in foundation & plinth in cement & sand mortar Ratio 1:6 (S.I.No.4 (c) P.No.25). |  |  |  |  |  |  |
|  |  |  | 1036.00 | 11948/36 | %Cft |  | Rs.123785/- |
|  |  |  | Rupees Eleven Thousand Nine Hundred Forty Eight Ps. Thirty Six Only. |  |  |  |  |
| 04. | Coursed Rubble masonry i/c hammer cement sand mortar ratio 1:6 (S.I.No.3 P.No.31). |  |  |  |  |  |  |
|  |  |  | 520.00 | 25321/- | %Cft |  | Rs.131669/ |
|  |  |  | (Rs. Twenty Five Thousand Three Hundred Twenty One Only). |  |  |  |  |
| 05. | R.C.C works in roof slab, beams columns rafts lintels and other structural members laid in situ recast laid in position complete in all respects ratio 1:2:4 90lbs cement 2cft sand 4cft shingle 1/8 to ¼ gauge (S.I.No.06 P.No.19). |  |  |  |  |  |  |
| (a) | Ground Floor |  | 998.00 | 337/- | %Cft |  | Rs.100426/- |
|  |  |  | Rupees Three Hundred Thirty Seven Only. |  |  |  |  |
| 06. | Fabrication of mild steel reinforcement for cement concrete including cutting bending, laying in position making joints and fastenings including cost of binding wire also including cost of binding wire also includes removal of rust from bars.(S.I.No.07 P.No.20). |  |  |  |  |  |  |
|  |  |  | 13.303 | 5001/70 | P.Cwt |  | Rs.66538/- |
|  |  |  | Rupees Five Thousand One Ps. Seventy Only. |  |  |  |  |
| 07. | Pacca brick work other than building in Ratio 1:6. (S.I.No.04 P.No.24). |  |  |  |  |  |  |
|  | C/wall |  | 1397.00 | 12346/65 | %Cft |  | Rs.172483/- |
|  |  |  | Rupees Twelve Thousand Three Hundred Sixty Six Ps. Sixty Five Only. |  |  |  |  |
| 08. | Cement plaster 1:6 upto 20 height 1/2”thick. (S.I.No. 13 P.No. 51 ). |  |  |  |  |  |  |
|  |  |  | 1896.00 | 2260/06 | %Sft | Rs.41837/- |  |
|  |  |  | (Rs. Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only). |  |  |  |  |
| 09. | Cement plaster 1:4 upto 20 height 3/8”thick. (S.I.No.11 P.No.58). |  |  |  |  |  |  |
|  |  |  | 1896.00 | 2197/52 | %Sft | Rs.41665/- |  |
|  |  |  | (Rs. Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only). |  |  |  |  |

| SR# | ITEM OF WORKS | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- |

| 10. | Cement Pointing i/c streaking of joints on wall ratio 1:3 (S.I.NO.16 P.No.21) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1760.00 | 1213/58 | %Sft | Rs.21359/- |
|  |  |  | Rupees One Thousand Two Hundred Thirteen Ps. Fifty Eight Only. |  |  |  |
| 11. | Colour wash two coats. |  |  |  |  |  |
|  |  |  | 1760.00 | 859/90 | %Sft | Rs.31438/- |
|  |  |  | Rupees Eight Hundred Fifty Nine Ps. Ninety Only. |  |  |  |
| 12. | M/F steel grated doors complete with locking arrangement angloe iron frame 2"x2"x3.8" and 3/4" sq. bars 4" centre to centre (SINO.23 P-92) |  |  |  |  |  |
|  |  |  | 60.00 | 726/72 | P.Sft | Rs.43603/- |
|  |  |  | Rupees Seven Hundred Twenty Six Ps. Seventy Five Only. |  |  |  |
| 13. | Painting New surface(a) Preparing surface painting corrugated surface, patent roofing etc. (SINO.5(a+b) P-69) for Two Coats |  |  |  |  |  |
|  |  |  | 120.00 | 674/60 | %Sft | Rs.810/- |
|  |  |  | Rupees Six Hundred Seventy Four Ps. Sixty Only. |  |  |  |

|  |  |  | TOTAL | Rs.1839823/- |
| --- | --- | --- | --- | --- |

% Above / Below (Amount to be added / deducted).		Rs.

Grand Total		Rs.

Total Amount in word:

Note:-	 Quantities and rates are provisional and may be changed as per /according to the Technical Sanction by the competent authority.

| CONTRACTOR | EXECUTIVE ENGINEER (BUILDINGS) WORKS & SERVICES DEPARTMENT KHAIRPUR |
| --- | --- |

(SCHEDULE B)

CONSTRUCTION OF COMPOUND WALL OF EID GAH AT VILLAGE KHOOH WALI MUHAMMAD U/C WADA MAHEESAR  TALUKA KINGRI DISTRICT KHAIRPUR

PART B PAVED COURTYARD

| SR# | ITEM OF WORKS |  | QTY: | RATE | UNIT |  | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 01. | Excavation in foundation of buildings bridges and other structures including dag belling dressing refilling around structure with excavated earth watering and ramming lead upto 5ft. (b) in ordinary soil. (S.I.No.18 P.No.5). |  |  |  |  |  |  |
|  |  |  | 525.00 | 3176/25 | %0Cft |  | Rs.1668/- |
|  |  |  | (Rs. Three Thousand One Hundred Seventy Six Ps. Twenty Five Only). |  |  |  |  |
| 02. | Cement concrete brick or stone ballast 1-1/2 to 2 gauge. (A) ratio 1:5:10. (S.I.No.4 P.No.17). |  |  |  |  |  |  |
|  |  |  | 2788.00 | 8694/95 | %Cft |  | Rs.242415/- |
|  |  |  | (Rs. Eight Thousand Six Hundred Ninety Four Ps. Ninety Five Only). |  |  |  |  |
| 03. | Pacca Brick work in foundation & plinth in cement & sand mortar Ratio 1:6 (S.I.No.4 (c) P.No.25). |  |  |  |  |  |  |
|  |  |  | 461.00 | 11948/36 | %Cft |  | Rs.55082/- |
|  |  |  | Rupees Eleven Thousand Nine Hundred Forty Eight Ps. Thirty Six Only. |  |  |  |  |
| 04. | Cement plaster 1:6 upto 20 height 1/2”thick. (S.I.No. 13 P.No. 51 ). |  |  |  |  |  |  |
|  |  |  | 1225.00 | 2260/06 | %Sft | Rs.27031/- |  |
|  |  |  | (Rs. Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only). |  |  |  |  |
| 05. | Cement plaster 1:4 upto 20 height 3/8”thick. (S.I.No.11 P.No.58). |  |  |  |  |  |  |
|  |  |  | 1225.00 | 2197/52 | %Sft | Rs26920/- |  |
|  |  |  | (Rs. Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only). |  |  |  |  |

| 06. | Supplying and filling sand under floor and plugging in walls.(S.I.No.29 P.No.31). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1440.00 | 1141/25 | %Cft | Rs.16434/- |
|  |  |  | Rupees One Thousand One Hundred Forty One Ps. Seventy Five Only. |  |  |  |

| 07. | P/L 3" thick topping c.c. 1:2:4 including surface finishing and dividing into panels.(SINO.16(c) P-41) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 7650.00 | 4411/82 | %Sft | Rs.337504/- |
|  |  |  | Rupees One Thousand Two Hundred Thirteen Ps. Fifty Eight Only. |  |  |  |

|  |  |  | TOTAL | Rs.707054/- |
| --- | --- | --- | --- | --- |

% Above / Below (Amount to be added / deducted).		Rs.

Grand Total		Rs.

Total Amount in word:

Note:-	 Quantities and rates are provisional and may be changed as per /according to the Technical Sanction by the competent authority.

| CONTRACTOR | EXECUTIVE ENGINEER (BUILDINGS) WORKS & SERVICES DEPARTMENT KHAIRPUR |
| --- | --- |

(SCHEDULE B)

CONSTRUCTION OF COMPOUND WALL DARGAH NAGHAY SHAH & MUSAFIR KHANA AT U/C MANGHANWARI  TALUKA KINGRI DISTRICT KHAIRPUR.

PART A MUSAFIR KHANA

| SR# | ITEM OF WORKS |  | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- |
| 01. | Excavation in foundation of buildings bridges and other structures including dag belling dressing refilling around structure with excavated earth watering and ramming lead upto 5ft. (b) in ordinary soil. (S.I.No.18 P.No.5). |  |  |  |  |  |
|  |  |  | 596.00 | 3176/25 | %0Cft | Rs.1893/- |
|  |  |  | (Rs. Three Thousand One Hundred Seventy Six Ps. Twenty Five Only). |  |  |  |
| 02. | Cement concrete brick or stone ballast 1-1/2 to 2 gauge. (A) ratio 1:5:10. (S.I.No.4 P.No.17). |  |  |  |  |  |
|  |  |  | 343.00 | 8694/95 | %Cft | Rs.29832/- |
|  |  |  | (Rs. Eight Thousand Six Hundred Ninety Four Ps. Ninety Five Only). |  |  |  |

| 03. | Un Coursed Rubble masonry in cement sand mortar ratio 1:6 (S.I.No.2 P.No.27). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 290.00 | 17723/- | %Cft | Rs.51397/- |
|  |  |  | Rupees Seventeen Thousand Seven Hundred Twenty Three Only. |  |  |  |

| 04. | Coursed Rubble masonry i/c hammer cement sand mortar ratio 1:6 (S.I.No.3 P.No.31). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 262.00 | 25321/- | %Cft | Rs.66341/- |
|  |  |  | (Rs. Twenty Five Thousand Three Hundred Twenty One Only). |  |  |  |
| 05. | Pacca Brick work in foundation & plinth in cement & sand mortar Ratio 1:6 (S.I.No.4 (c) P.No.25). |  |  |  |  |  |
|  |  |  | 23.00 | 11948/36 | %Cft | Rs.2748/- |
|  |  |  | Rupees Eleven Thousand Nine Hundred Forty Eight Ps. Thirty Six Only. |  |  |  |
| 06. | R.C.C works in roof slab, beams columns rafts lintels and other structural members laid in situ recast laid in position complete in all respects ratio 1:2:4 90lbs cement 2cft sand 4cft shingle 1/8 to ¼ gauge (S.I.No.06 P.No.19). |  |  |  |  |  |
| (a) | Ground Floor |  | 415.00 | 337/- | %Cft | Rs.139855/- |
|  |  |  | Rupees Three Hundred Thirty Seven Only. |  |  |  |
| 07. | Fabrication of mild steel reinforcement for cement concrete including cutting bending, laying in position making joints and fastenings including cost of binding wire also including cost of binding wire also includes removal of rust from bars.(S.I.No.07 P.No.20). |  |  |  |  |  |
|  |  |  | 18.526 | 5001/70 | P.Cwt | Rs.92661/- |
|  |  |  | Rupees Five Thousand One Ps. Seventy Only. |  |  |  |
| 08. | Filling watering and ramming earth in floor with surplus earth from foundation lead upto one chain and lift upto 5ft.(S.I.NO:11 P-5) |  |  |  |  |  |
|  |  |  | 397.00 | 1512/50 | %Cft | Rs.600/- |
|  |  |  | (Rs. One Thousand Five Hundred Eleven Ps. Fifty Only). |  |  |  |

| 09. | Filling watering and ramming earth under floor with new excavated from outside lead upto one chain and lift upto 5ft (S.I.No.12 P.No.5). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 597.00 | 3630/- | %0Cft | Rs.2167/- |
|  |  |  | Rupees Three Thousand Six Hundred Thirty Only. |  |  |  |
| 10. | Extra lead for 3”mile. |  |  |  |  |  |
|  |  |  | 597.00 | 579/41 | %Cft | Rs.3459/- |
|  |  |  | Rupees Five Hundred Seventy Nine Ps. Forty One Only. |  |  |  |

| SR# | ITEM OF WORKS | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- |

| 11. | Supplying and filling sand under floor and plugging in walls.(S.I.No.29 P.No.31). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 239.00 | 1141/75 | %Cft | Rs.2728/- |
|  |  |  | Rupees One Thousand One Hundred Forty One Ps. Seventy Five Only. |  |  |  |
| 12. | Pacca brick work in Ground Floor and plinth in Ratio 1:6. (S.I.No.04 P.No.24). |  |  |  |  |  |
| (a) | Ground Floor |  | 706.00 | 12674/36 | %Cft | Rs.89481/- |
|  |  |  | Rupees Twelve Thousand Six Hundred Seventy Four Ps. Thirty Six Only. |  |  |  |
| 13. | Providing and fixing G.I. frames chowkats of size 7x2xor 4-1/2x3 for door using 20 gauge G.I. sheet i/c welded hinge and fixing at with cement sand slurry of ration 1.6 despairing the jambs. The cost also i/c all carriage tools and plants used in making and fixing.(S.I.NO.18 P-98). |  |  |  |  |  |
|  |  |  | 35.00 | 228/90 | P.Rft | Rs.8012/- |
|  |  |  | Rupees Two Hundred Twenty Eight Ps. Ninety Only. |  |  |  |
| 14. | Providing and fixing G.I. Frames chowkats of size 7x2xir 4-1/2x3 for windows using 20guage G.I. sheet i/c welded hinges and fixing at site with necessary hold fasts, filling with cement sand slurry of ration 1;6 and repainting the jamb. The cost also i/c all carriage tools and plants used in making and fixing. (S.I.No.29 P.No.98). |  |  |  |  |  |
|  |  |  | 57.00 | 240/50 | P.Rft | Rs.13709/- |
|  |  |  | Rupees Two Hundred Forty Ps. Fifty Only. |  |  |  |
| 15. | Cement plaster 1:6 upto 20 height ½ thick. (S.I.No.13 P.No.58). |  |  |  |  |  |
| (a) | Ground Floor |  | 977.00 | 2206/60 | %Sft | Rs.21558/- |
|  |  |  | Rupees Two Thousand Two Hundred Six Ps. Sixty Five Only. |  |  |  |
| 16. | Cement plaster 1:4 upto 20 height 3/8 thick. (S.I.No.11 P.No.58). |  |  |  |  |  |
| (a) | Ground Floor |  | 977.00 | 2197/52 | %Sft | Rs.21470/- |
|  |  |  | Rupees Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only. |  |  |  |
| 17. | Providing and fixing iron steel grill using solid square bars of size ½”x1/2” placed at 4” i/c and frame of flat iron patti of ¾”x3/4” i/c circle shape at 1-0 a part equivalent fitted with screws are pins i/c painting 3 coats with 1st coat of red oxide paint etc. (S.I.No.26 P.No.97). |  |  |  |  |  |
|  |  |  | 53.00 | 180/50 | P.Sft | Rs.9567/- |
|  |  |  | Rupees One Hundred Eighty Ps. Fifty Only. |  |  |  |
| 18. | Laying floor of approved colour glazed tiles 1/4" thick laid in white cement and pigement on a bned of 3/4" thick cement mortor 1:2 ( S.I.No. 25 P- 43 ) |  |  |  |  |  |
| (a) | Ground Floor |  | 480.00 | 27747/06 | %Sft | Rs.133186/- |
|  |  |  | Rupees Twenty Seven Thousand Seven Hundred Forty Seven Ps. Zero Six Only. |  |  |  |
| 19. | White glazed tile 1/4" thick dado jointed in white cement and laid over 1:2 cement sand mortar 3/4" thick including finishing. (SINO.37 P-45) |  |  |  |  |  |
|  |  |  | 88.00 | 28253/61 | %Sft | Rs.24863/- |
|  |  |  | Rupees Twenty Eight Thousand Two Hundred Fifty Three Ps. Sixty One Only. |  |  |  |

| SR# | ITEM OF WORKS | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- |

| 20. | First class deodar wood wrought, joinery in doors & windows etc. fixed in position i/c chowkhats holds fasts hinges, iron tower blts. Chocks cleats, handles and cords with hooks, etc. Deodar paneled or paneled and glazed or fully glazed (without chowkhats). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 102.00 | 902/93 | P.Sft | Rs.92099/- |
|  |  |  | Rupees Nine Hundred Two Ps. Ninety Three Only. |  |  |  |

| 21. | Cement Pointing i/c streaking of joints on wall ratio 1:3 (S.I.NO.16 P.No.21) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1048.00 | 1213/58 | %Sft | Rs.12718/- |
|  |  |  | Rupees One Thousand Two Hundred Thirteen Ps. Fifty Eight Only. |  |  |  |
| 22. | Colour wash two coats. |  |  |  |  |  |
|  |  |  | 1048.00 | 859/90 | %Sft | Rs.9012/- |
|  |  |  | Rupees Eight Hundred Fifty Nine Ps. Ninety Only. |  |  |  |

| 23. | Primary coat of chalk under distemper. (S.I.No.23 P.No.59). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 977.00 | 442/75 | %Sft | Rs.4326/- |
|  |  |  | Rupees Four Hundred Forty Two Ps. Seventy Five Only. |  |  |  |
| 24. | Distemper in two coats. (S.I.No.24 P.No.60). |  |  |  |  |  |
|  |  |  | 977.00 | 1079/65 | %Sft | Rs.10548/- |
|  |  |  | Rupees One Thousand Seventy Nine Ps. Sixty Five Only. |  |  |  |

| 25. | Painting New surface(a) Preparing surface painting corrugated surface, patent roofing etc. (SINO.5(a+b) P-69) for Two Coats |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 203.00 | 1489/68 | %Sft | Rs.3024/- |
|  |  |  | Rupees One Thousand Four Hundred Eighty Nine Ps. Sixty Eight Only. |  |  |  |

|  |  |  | TOTAL | Rs.707054/- |
| --- | --- | --- | --- | --- |

% Above / Below (Amount to be added / deducted).		Rs.

Grand Total		Rs.

Total Amount in word:

Note:-	 Quantities and rates are provisional and may be changed as per /according to the Technical Sanction by the competent authority.

| CONTRACTOR | EXECUTIVE ENGINEER (BUILDINGS) WORKS & SERVICES DEPARTMENT KHAIRPUR |
| --- | --- |

(SCHEDULE B)

CONSTRUCTION OF COMPOUND WALL DARGAH NAGHAY SHAH & MUSAFIR KHANA AT U/C MANGHANWARI  TALUKA KINGRI DISTRICT KHAIRPUR.

PART B COMPOUND WALL

| SR# | ITEM OF WORKS |  | QTY: | RATE | UNIT |  | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 01. | Excavation in foundation of buildings bridges and other structures including dag belling dressing refilling around structure with excavated earth watering and ramming lead upto 5ft. (b) in ordinary soil. (S.I.No.18 P.No.5). |  |  |  |  |  |  |
|  |  |  | 2840.00 | 3176/25 | %0Cft |  | Rs.9020/- |
|  |  |  | (Rs. Three Thousand One Hundred Seventy Six Ps. Twenty Five Only). |  |  |  |  |
| 02. | Cement concrete brick or stone ballast 1-1/2 to 2 gauge. (A) ratio 1:5:10. (S.I.No.4 P.No.17). |  |  |  |  |  |  |
|  |  |  | 710.00 | 8694/95 | %Cft |  | Rs.61734/- |
|  |  |  | (Rs. Eight Thousand Six Hundred Ninety Four Ps. Ninety Five Only). |  |  |  |  |
| 03. | Pacca Brick work in foundation & plinth in cement & sand mortar Ratio 1:6 (S.I.No.4 (c) P.No.25). |  |  |  |  |  |  |
|  |  |  | 1597.00 | 11948/36 | %Cft |  | Rs.190815/- |
|  |  |  | Rupees Eleven Thousand Nine Hundred Forty Eight Ps. Thirty Six Only. |  |  |  |  |
| 04. | Coursed Rubble masonry i/c hammer cement sand mortar ratio 1:6 (S.I.No.3 P.No.31). |  |  |  |  |  |  |
|  |  |  | 887.00 | 25321/- | %Cft |  | Rs.179779/ |
|  |  |  | (Rs. Twenty Five Thousand Three Hundred Twenty One Only). |  |  |  |  |
| 05. | R.C.C works in roof slab, beams columns rafts lintels and other structural members laid in situ recast laid in position complete in all respects ratio 1:2:4 90lbs cement 2cft sand 4cft shingle 1/8 to ¼ gauge (S.I.No.06 P.No.19). |  |  |  |  |  |  |
| (a) | Ground Floor |  | 480.00 | 337/- | %Cft |  | Rs.161760/- |
|  |  |  | Rupees Three Hundred Thirty Seven Only. |  |  |  |  |
| 06. | Fabrication of mild steel reinforcement for cement concrete including cutting bending, laying in position making joints and fastenings including cost of binding wire also including cost of binding wire also includes removal of rust from bars.(S.I.No.07 P.No.20). |  |  |  |  |  |  |
|  |  |  | 21.428 | 5001/70 | P.Cwt |  | Rs.107176/- |
|  |  |  | Rupees Five Thousand One Ps. Seventy Only. |  |  |  |  |
| 07. | Pacca brick work other than building in Ratio 1:6. (S.I.No.04 P.No.24). |  |  |  |  |  |  |
|  | C/wall |  | 2159.00 | 12346/65 | %Cft |  | Rs.258415/- |
|  |  |  | Rupees Twelve Thousand Three Hundred Sixty Six Ps. Sixty Five Only. |  |  |  |  |
| 08. | Cement plaster 1:6 upto 20 height 1/2”thick. (S.I.No. 13 P.No. 51 ). |  |  |  |  |  |  |
|  |  |  | 3003.00 | 2260/06 | %Sft | Rs.66264/- |  |
|  |  |  | (Rs. Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only). |  |  |  |  |
| 09. | Cement plaster 1:4 upto 20 height 3/8”thick. (S.I.No.11 P.No.58). |  |  |  |  |  |  |
|  |  |  | 3003.00 | 2197/52 | %Sft | Rs.65991/- |  |
|  |  |  | (Rs. Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only). |  |  |  |  |

| SR# | ITEM OF WORKS | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- |

| 10. | Cement Pointing i/c streaking of joints on wall ratio 1:3 (S.I.NO.16 P.No.21) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 2796.00 | 1213/58 | %Sft | Rs.35997/- |
|  |  |  | Rupees One Thousand Two Hundred Thirteen Ps. Fifty Eight Only. |  |  |  |
| 11. | Colour wash two coats. |  |  |  |  |  |
|  |  |  | 2796.00 | 859/90 | %Sft | Rs.24042/- |
|  |  |  | Rupees Eight Hundred Fifty Nine Ps. Ninety Only. |  |  |  |
| 12. | M/F steel grated doors complete with locking arrangement angloe iron frame 2"x2"x3.8" and 3/4" sq. bars 4" centre to centre (SINO.23 P-92) |  |  |  |  |  |
|  |  |  | 75.00 | 726/72 | P.Sft | Rs.54504/- |
|  |  |  | Rupees Seven Hundred Twenty Six Ps. Seventy Five Only. |  |  |  |
| 13. | Painting New surface(a) Preparing surface painting corrugated surface, patent roofing etc. (SINO.5(a+b) P-69) for Two Coats |  |  |  |  |  |
|  |  |  | 150.00 | 1489/68 | %Sft | Rs.2234/- |
|  |  |  | Rupees One Thousand Four Hundred Eighty Nine Ps. Sixty Eight Only. |  |  |  |

|  |  |  | TOTAL | Rs.1217731/- |
| --- | --- | --- | --- | --- |

% Above / Below (Amount to be added / deducted).		Rs.

Grand Total		Rs.

Total Amount in word:

Note:-	 Quantities and rates are provisional and may be changed as per /according to the Technical Sanction by the competent authority.

| CONTRACTOR | EXECUTIVE ENGINEER (BUILDINGS) WORKS & SERVICES DEPARTMENT KHAIRPUR |
| --- | --- |

(SCHEDULE B)

CONSTRUCTION OF IMAM BARGAH @ VILLAGE BALOCH ABAD U/C MAHEESAR WADA

TALUKA KINGRI DISTRICT KHAIRPUR.

PART A IMAM BARGAH

| SR# | ITEM OF WORKS |  | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- |
| 01. | Excavation in foundation of buildings bridges and other structures including dag belling dressing refilling around structure with excavated earth watering and ramming lead upto 5ft. (b) in ordinary soil. (S.I.No.18 P.No.5). |  |  |  |  |  |
|  |  |  | 744.00 | 3176/25 | %0Cft | Rs.2363/- |
|  |  |  | (Rs. Three Thousand One Hundred Seventy Six Ps. Twenty Five Only). |  |  |  |
| 02. | Cement concrete brick or stone ballast 1-1/2 to 2 gauge. (A) ratio 1:5:10. (S.I.No.4 P.No.17). |  |  |  |  |  |
|  |  |  | 451.00 | 8694/95 | %Cft | Rs.39214/- |
|  |  |  | (Rs. Eight Thousand Six Hundred Ninety Four Ps. Ninety Five Only). |  |  |  |

| 03. | Un Coursed Rubble masonry in cement sand mortar ratio 1:6 (S.I.No.2 P.No.27). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 372.00 | 17723/- | %Cft | Rs.65930/- |
|  |  |  | Rupees Seventeen Thousand Seven Hundred Twenty Three Only. |  |  |  |

| 04. | Coursed Rubble masonry i/c hammer cement sand mortar ratio 1:6 (S.I.No.3 P.No.31). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 334.00 | 25321/- | %Cft | Rs.84572/- |
|  |  |  | (Rs. Twenty Five Thousand Three Hundred Twenty One Only). |  |  |  |
| 05. | R.C.C works in roof slab, beams columns rafts lintels and other structural members laid in situ recast laid in position complete in all respects ratio 1:2:4 90lbs cement 2cft sand 4cft shingle 1/8 to ¼ gauge (S.I.No.06 P.No.19). |  |  |  |  |  |
| (a) | Ground Floor |  | 235.00 | 337/- | %Cft | Rs.78858/- |
|  |  |  | Rupees Three Hundred Thirty Seven Only. |  |  |  |
| 06. | Fabrication of mild steel reinforcement for cement concrete including cutting bending, laying in position making joints and fastenings including cost of binding wire also including cost of binding wire also includes removal of rust from bars.(S.I.No.07 P.No.20). |  |  |  |  |  |
|  |  |  | 10.491 | 5001/70 | P.Cwt | Rs.52473/- |
|  |  |  | Rupees Five Thousand One Ps. Seventy Only. |  |  |  |
| 07. | Filling watering and ramming earth in floor with surplus earth from foundation lead upto one chain and lift upto 5ft.(S.I.NO:11 P-5) |  |  |  |  |  |
|  |  |  | 496.00 | 1512/50 | %Cft | Rs.750/- |
|  |  |  | (Rs. One Thousand Five Hundred Eleven Ps. Fifty Only). |  |  |  |

| 08. | Filling watering and ramming earth under floor with new excavated from outside lead upto one chain and lift upto 5ft (S.I.No.12 P.No.5). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 864.00 | 3630/- | %0Cft | Rs.3136/- |
|  |  |  | Rupees Three Thousand Six Hundred Thirty Only. |  |  |  |
| 09. | Extra lead for 3”mile. |  |  |  |  |  |
|  |  |  | 864.00 | 579/41 | %Cft | Rs.5006/- |
|  |  |  | Rupees Five Hundred Seventy Nine Ps. Forty One Only. |  |  |  |

| 10. | Supplying and filling sand under floor and plugging in walls.(S.I.No.29 P.No.31). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 346.00 | 1141/75 | %Cft | Rs.3949/- |
|  |  |  | Rupees One Thousand One Hundred Forty One Ps. Seventy Five Only. |  |  |  |

| SR# | ITEM OF WORKS |  |  |  | QTY: |  | RATE |  |  | UNIT |  | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 11. | Pacca brick work in Ground Floor and plinth in Ratio 1:6. (S.I.No.04 P.No.24). |  |  |  |  |  |  |  |  |  |  |  |
| (a) | Ground Floor |  |  | 968.00 |  |  | 12674/36 |  |  | %Cft |  | Rs.122688/- |
|  |  |  |  | Rupees Twelve Thousand Six Hundred Seventy Four Ps. Thirty Six Only. |  |  |  |  |  |  |  |  |
| 12. | Providing and Fixing Iron Girders (material Sch: item No: 140) |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30.714 |  | 3850/- |  |  | P.Cwt |  |  | Rs.118249/- |
|  |  |  |  | (Rs. Three Thousands Eight Hundred Fifty Only). |  |  |  |  |  |  |  |  |
| 13. | Supplying fixing T-Irons (material Sch: item No: 144) |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 15.741 |  |  | 3575/- |  |  | P.Cwt |  | Rs.56274/- |
|  |  |  |  | (Rs. Three Thousands Five Hundred Seventy Five Only). |  |  |  |  |  |  |  |  |
| 14. | Second class tile roofing consisting of 4" earth and 1" mud plaster with Gobri leeping over 1/2"thick cement plaster 1:6 with 34 Lbs. Of hot bitumen coating sand blinded, provided over one layer of 12" x 6"x2" tiles laid in 1:6 cement mortar including 1:2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 892.00 |  |  | 7607/25- |  |  | %Sft |  | Rs.67857/- |
|  |  |  |  | (Rs. Seventy Six Thousands Seven Ps. Twenty Five Only). |  |  |  |  |  |  |  |  |
| 15. | Providing and fixing G.I. frames chowkats of size 7x2xor 4-1/2x3 for door using 20 gauge G.I. sheet i/c welded hinge and fixing at with cement sand slurry of ration 1.6 despairing the jambs. The cost also i/c all carriage tools and plants used in making and fixing.(S.I.NO.18 P-98). |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 53.00 |  |  | 228/90 | P.Rft |  |  | Rs.12132/- |  |
|  |  |  |  | Rupees Two Hundred Twenty Eight Ps. Ninety Only. |  |  |  |  |  |  |  |  |
| 16. | Providing and fixing G.I. Frames chowkats of size 7x2xir 4-1/2x3 for windows using 20guage G.I. sheet i/c welded hinges and fixing at site with necessary hold fasts, filling with cement sand slurry of ration 1;6 and repainting the jamb. The cost also i/c all carriage tools and plants used in making and fixing. (S.I.No.29 P.No.98). |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 154.00 |  |  | 240/50 | P.Rft |  |  | Rs.37037/- |  |
|  |  |  |  | Rupees Two Hundred Forty Ps. Fifty Only. |  |  |  |  |  |  |  |  |
| 17. | Cement plaster 1:6 upto 20 height ½ thick. (S.I.No.13 P.No.58). |  |  |  |  |  |  |  |  |  |  |  |
| (a) | Ground Floor |  |  | 1890.00 |  |  | 2206/60 | %Sft |  |  | Rs.37380/- |  |
|  |  |  |  | Rupees Two Thousand Two Hundred Six Ps. Sixty Five Only. |  |  |  |  |  |  |  |  |
| 18. | Cement plaster 1:4 upto 20 height 3/8 thick. (S.I.No.11 P.No.58). |  |  |  |  |  |  |  |  |  |  |  |
| (a) | Ground Floor |  |  | 1890.00 |  |  | 2197/52 | %Sft |  |  | Rs.37226/- |  |
|  |  |  |  | Rupees Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only. |  |  |  |  |  |  |  |  |
| 19. | Cement concrete plain including placing compacting finishing and curing, complete (including screening and washing of stone aggregate without shuttering Ratio 1:2:4. (S.I.No.05 P.No.18). |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 122.00 |  | 14429/25- | %Cft |  |  | Rs.17604/- |  |
|  |  |  |  |  | Rupees Fourteen Thousand Four Hundred Twenty Nine Ps. Twenty Five Only. |  |  |  |  |  |  |  |
| 20. | Laying floor of approved colour glazed tiles 1/4" thick laid in white cement mortor 1:2 ( S.I.No. 25 P- 43 ) |  |  |  |  |  |  |  |  |  |  |  |
| (a) | Ground Floor |  |  |  | 720.00 |  | 27747/06 | %Sft |  |  | Rs.343779/- |  |
|  |  |  |  |  | Rupees Twenty Seven Thousand Seven Hundred Forty Seven Ps. Zero Six Only. |  |  |  |  |  |  |  |

| SR# | ITEM OF WORKS |  | QTY: | RATE |  | UNIT |  | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 21. | White glazed tile 1/4" thick dado jointed in white cement and laid over 1:2 cement sand mortar 3/4" thick including finishing. (SINO.37 P-45) |  |  |  |  |  |  |  |
|  |  |  | 348.00 | 28253/61 | %Sft |  | Rs.98322/- |  |
|  |  |  | Rupees Twenty Eight Thousand Two Hundred Fifty Three Ps. Sixty One Only. |  |  |  |  |  |
| 22. | Providing and fixing iron steel grill using solid square bars of size ½”x1/2” placed at 4” i/c and frame of flat iron patti of ¾”x3/4” i/c circle shape at 1-0 a part equivalent fitted with screws are pins i/c painting 3 coats with 1st coat of red oxide paint etc. (S.I.No.26 P.No.97). |  |  |  |  |  |  |  |
|  |  |  | 87.00 | 180/50 | P.Sft |  | Rs.15704/- |  |
|  |  |  | Rupees One Hundred Eighty Ps. Fifty Only. |  |  |  |  |  |

| 23. | First class deodar wood wrought, joinery in doors & windows etc. fixed in position i/c chowkhats holds fasts hinges, iron tower blts. Chocks cleats, handles and cords with hooks, etc. Deodar paneled or paneled and glazed or fully glazed (without chowkhats). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 161.00 | 902/93 | P.Sft | Rs.145372/- |
|  |  |  | Rupees Nine Hundred Two Ps. Ninety Three Only. |  |  |  |

| 24. | Cement Pointing i/c streaking of joints on wall ratio 1:3 (S.I.NO.16 P.No.21) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 854.00 | 1213/58 | %Sft | Rs.10364/- |
|  |  |  | Rupees One Thousand Two Hundred Thirteen Ps. Fifty Eight Only. |  |  |  |
| 25. | Colour wash two coats. |  |  |  |  |  |
|  |  |  | 854.00 | 859/90 | %Sft | Rs.7344/- |
|  |  |  | Rupees Eight Hundred Fifty Nine Ps. Ninety Only. |  |  |  |

| 26. | Primary coat of chalk under distemper. (S.I.No.23 P.No.59). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1694.00 | 442/75 | %Sft | Rs.7500/- |
|  |  |  | Rupees Four Hundred Forty Two Ps. Seventy Five Only. |  |  |  |
| 27. | Distemper in two coats. (S.I.No.24 P.No.60). |  |  |  |  |  |
|  |  |  | 1694.00 | 1079/65 | %Sft | Rs.18289/- |
|  |  |  | Rupees One Thousand Seventy Nine Ps. Sixty Five Only. |  |  |  |

| 28. | Painting New surface(a) Preparing surface painting corrugated surface, patent roofing etc. (SINO.5(a+b) P-69) for Two Coats |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 322.00 | 1489/68 | %Sft | Rs.4797/- |
|  |  |  | Rupees One Thousand Four Hundred Eighty Nine Ps. Sixty Eight Only. |  |  |  |

|  |  |  | TOTAL | Rs.1478762/- |
| --- | --- | --- | --- | --- |

% Above / Below (Amount to be added / deducted).		Rs.

Grand Total		Rs.

Total Amount in word:

Note:-	 Quantities and rates are provisional and may be changed as per /according to the Technical Sanction by the competent authority.

| CONTRACTOR | EXECUTIVE ENGINEER (BUILDINGS) WORKS & SERVICES DEPARTMENT KHAIRPUR |
| --- | --- |

(SCHEDULE B)

CONSTRUCTION OF IMAM BARGAH @ VILLAGE BALOCH ABAD U/C MAHEESAR WADA

TALUKA KINGRI DISTRICT KHAIRPUR.

(QUANTITY OF BOQ)

PART B COMPOUND WALL

| SR# | ITEM OF WORKS |  | QTY: | RATE | UNIT |  | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 01. | Excavation in foundation of buildings bridges and other structures including dag belling dressing refilling around structure with excavated earth watering and ramming lead upto 5ft. (b) in ordinary soil. (S.I.No.18 P.No.5). |  |  |  |  |  |  |
|  |  |  | 506.00 | 3176/25 | %0Cft |  | Rs.1607/- |
|  |  |  | (Rs. Three Thousand One Hundred Seventy Six Ps. Twenty Five Only). |  |  |  |  |
| 02. | Cement concrete brick or stone ballast 1-1/2 to 2 gauge. (A) ratio 1:5:10. (S.I.No.4 P.No.17). |  |  |  |  |  |  |
|  |  |  | 127.00 | 8694/95 | %Cft |  | Rs.11043/- |
|  |  |  | (Rs. Eight Thousand Six Hundred Ninety Four Ps. Ninety Five Only). |  |  |  |  |
| 03. | Un Coursed Rubble masonry in cement sand mortar ratio 1:6 (S.I.No.2 P.No.27). |  |  |  |  |  |  |
|  |  |  | 285.00 | 17723/- | %Cft |  | Rs.50510/- |
|  |  |  | Rupees Seventeen Thousand Seven Hundred Twenty Three Only. |  |  |  |  |
| 04. | Coursed Rubble masonry i/c hammer cement sand mortar ratio 1:6 (S.I.No.3 P.No.31). |  |  |  |  |  |  |
|  |  |  | 158.00 | 25321/- | %Cft |  | Rs.40007/ |
|  |  |  | (Rs. Twenty Five Thousand Three Hundred Twenty One Only). |  |  |  |  |
| 05. | R.C.C works in roof slab, beams columns rafts lintels and other structural members laid in situ recast laid in position complete in all respects ratio 1:2:4 90lbs cement 2cft sand 4cft shingle 1/8 to ¼ gauge (S.I.No.06 P.No.19). |  |  |  |  |  |  |
| (a) | Ground Floor |  | 99.00 | 337/- | %Cft |  | Rs.33363/- |
|  |  |  | Rupees Three Hundred Thirty Seven Only. |  |  |  |  |
| 06. | Fabrication of mild steel reinforcement for cement concrete including cutting bending, laying in position making joints and fastenings including cost of binding wire also including cost of binding wire also includes removal of rust from bars.(S.I.No.07 P.No.20). |  |  |  |  |  |  |
|  |  |  | 4.419 | 5001/70 | P.Cwt |  | Rs.22103/- |
|  |  |  | Rupees Five Thousand One Ps. Seventy Only. |  |  |  |  |
| 07. | Pacca brick work other than building in Ratio 1:6. (S.I.No.04 P.No.24). |  |  |  |  |  |  |
|  | C/wall |  | 344.00 | 12346/65 | %Cft |  | Rs.42472/- |
|  |  |  | Rupees Twelve Thousand Three Hundred Sixty Six Ps. Sixty Five Only. |  |  |  |  |
| 08. | Cement plaster 1:6 upto 20 height 1/2”thick. (S.I.No. 13 P.No. 51 ). |  |  |  |  |  |  |
|  |  |  | 458.00 | 2260/06 | %Sft | Rs.10106/- |  |
|  |  |  | (Rs. Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only). |  |  |  |  |
| 09. | Cement plaster 1:4 upto 20 height 3/8”thick. (S.I.No.11 P.No.58). |  |  |  |  |  |  |
|  |  |  | 458.00 | 2197/52 | %Sft | Rs.10065/- |  |
|  |  |  | (Rs. Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only). |  |  |  |  |

| SR# | ITEM OF WORKS | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- |

| 10. | White washing Three coats. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 458.00 | 829/50 | %Sft | Rs.3733/- |
|  |  |  | Rupees Eighty Hundred Fifty Nine Ps. Ninety Only. |  |  |  |
| 11. | Cement Pointing i/c streaking of joints on wall ratio 1:3 (S.I.NO.16 P.No.21) |  |  |  |  |  |
|  |  |  | 458.00 | 1213/58 | %Sft | Rs.5558/- |
|  |  |  | Rupees One Thousand Two Hundred Thirteen Ps. Fifty Eight Only. |  |  |  |
| 12. | Colour wash two coats. |  |  |  |  |  |
|  |  |  | 458.00 | 859/90 | %Sft | Rs.3938/- |
|  |  |  | Rupees Eight Hundred Fifty Nine Ps. Ninety Only. |  |  |  |
| 13. | M/F steel grated doors complete with locking arrangement angloe iron frame 2"x2"x3.8" and 3/4" sq. bars 4" centre to centre (SINO.23 P-92) |  |  |  |  |  |
|  |  |  | 48.00 | 726/72 | P.Sft | Rs.34882/- |
|  |  |  | Rupees Seven Hundred Twenty Six Ps. Seventy Five Only. |  |  |  |
| 14. | Painting New surface(a) Preparing surface painting corrugated surface, patent roofing etc. (SINO.5(a+b) P-69) for Two Coats |  |  |  |  |  |
|  |  |  | 96.00 | 1489/68 | %Sft | Rs.1430/- |
|  |  |  | Rupees One Thousand Four Hundred Eighty Nine Ps. Sixty Eight Only. |  |  |  |

|  |  |  | TOTAL | Rs.270817/- |
| --- | --- | --- | --- | --- |

% Above / Below (Amount to be added / deducted).		Rs.

Grand Total		Rs.

Total Amount in word:

Note:-	 Quantities and rates are provisional and may be changed as per /according to the Technical Sanction by the competent authority.

| CONTRACTOR | EXECUTIVE ENGINEER (BUILDINGS) WORKS & SERVICES DEPARTMENT KHAIRPUR |
| --- | --- |

(SCHEDULE B)

TALUKA KINGRI DISTRICT KHAIRPUR.

(QUANTITY OF BOQ)

COMPOUND WALL:

| SR# | ITEM OF WORKS |  | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- |
| 01. | Excavation in foundation of buildings bridges and other structures including dag belling dressing refilling around structure with excavated earth watering and ramming lead upto 5ft. (b) in ordinary soil. (S.I.No.18 P.No.5). |  |  |  |  |  |
|  |  |  | 1612.00 | 3176/25 | %0Cft | Rs.5120/- |
|  |  |  | (Rs. Three Thousand One Hundred Seventy Six Ps. Twenty Five Only). |  |  |  |
| 02. | Cement concrete brick or stone ballast 1-1/2 to 2 gauge. (A) ratio 1:5:10. (S.I.No.4 P.No.17). |  |  |  |  |  |
|  |  |  | 403.00 | 8694/95 | %Cft | Rs.35041/- |
|  |  |  | (Rs. Eight Thousand Six Hundred Ninety Four Ps. Ninety Five Only). |  |  |  |
| 03. | Pacca Brick work in foundation & plinth in cement & sand mortar Ratio 1:6 (S.I.No.4 (c) P.No.25). |  |  |  |  |  |
|  |  |  | 907.00 | 11948/36 | %Cft | Rs.108371/- |
|  |  |  | Rupees Eleven Thousand Nine Hundred Forty Eight Ps. Thirty Six Only. |  |  |  |
| 04. | Coursed Rubble masonry i/c hammer cement sand mortar ratio 1:6 (S.I.No.3 P.No.31). |  |  |  |  |  |
|  |  |  | 455.00 | 25321/- | %Cft | Rs.115211/ |
|  |  |  | (Rs. Twenty Five Thousand Three Hundred Twenty One Only). |  |  |  |
| 05. | R.C.C works in roof slab, beams columns rafts lintels and other structural members laid in situ recast laid in position complete in all respects ratio 1:2:4 90lbs cement 2cft sand 4cft shingle 1/8 to ¼ gauge (S.I.No.06 P.No.19). |  |  |  |  |  |
| (a) | Ground Floor |  | 245.00 | 337/- | %Cft | Rs.82565/- |
|  |  |  | Rupees Three Hundred Thirty Seven Only. |  |  |  |
| 06. | Fabrication of mild steel reinforcement for cement concrete including cutting bending, laying in position making joints and fastenings including cost of binding wire also including cost of binding wire also includes removal of rust from bars.(S.I.No.07 P.No.20). |  |  |  |  |  |
|  |  |  | 10.937 | 5001/70 | P.Cwt | Rs.54704/- |
|  |  |  | Rupees Five Thousand One Ps. Seventy Only. |  |  |  |
| 07. | Pacca brick work other than building in Ratio 1:6. (S.I.No.04 P.No.24). |  |  |  |  |  |
|  | C/wall |  | 1240.00 | 12346/65 | %Cft | Rs.153098/- |
|  |  |  | Rupees Twelve Thousand Three Hundred Sixty Six Ps. Sixty Five Only. |  |  |  |
| 08. | Cement plaster 1:6 upto 20 height 1/2”thick. (S.I.No. 13 P.No. 51 ). |  |  |  |  |  |
|  |  |  | 1682.00 | 2260/06 | %Sft | Rs.37115/- |
|  |  |  | (Rs. Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only). |  |  |  |
| 09. | Cement plaster 1:4 upto 20 height 3/8”thick. (S.I.No.11 P.No.58). |  |  |  |  |  |
|  |  |  | 1682.00 | 2197/52 | %Sft | Rs.36962/- |
|  |  |  | (Rs. Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only). |  |  |  |

| SR# | ITEM OF WORKS | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- |

| 10. | Cement Pointing i/c streaking of joints on wall ratio 1:3 (S.I.NO.16 P.No.21) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1564.00 | 1213/58 | %Sft |  | Rs.18980/- |  |
|  |  |  | Rupees One Thousand Two Hundred Thirteen Ps. Fifty Eight Only. |  |  |  |  |  |
| 11. | Colour wash two coats. |  |  |  |  |  |  |  |
|  |  |  | 1564.00 | 859/90 | %Sft |  | Rs.13449/- |  |
|  |  |  | Rupees Eight Hundred Fifty Nine Ps. Ninety Only. |  |  |  |  |  |
| 12. | Filling watering and ramming earth under floor with new excavated from outside lead upto one chain and lift upto 5feet. (S.I.No.12 P.No.5). |  |  |  |  |  |  |  |
|  |  |  | 15930.00 | 3630/- | %0Cft |  | Rs.57826/- |  |
|  |  |  | Rupees Three Thousand Six Hundred Thirty Only. |  |  |  |  |  |
| 13. | Extra lead for 3”mile. |  |  |  |  |  |  |  |
|  |  |  | 15930.00 | 579/41 | %Cft |  | Rs.92300/- |  |
|  |  |  | Rupees Five Hundred Seventy Nine Ps. Forty One Only. |  |  |  |  |  |
| 14. | Dressing and leveling of earth work to designed Section etc completed in ordinary soil. |  |  |  |  |  |  |  |
|  |  |  | 1593.00 | 187/55- |  | %0Cft |  | Rs.2988/- |
|  |  |  | Rupees One Hundred Eighty Seven Ps. Fifty Five Only. |  |  |  |  |  |
| 15. | M/F steel grated doors complete with locking arrangement angloe iron frame 2"x2"x3.8" and 3/4" sq. bars 4" centre to centre (SINO.23 P-92) |  |  |  |  |  |  |  |
|  |  |  | 40.00 | 726/72 | P.Sft |  | Rs.29068/- |  |
|  |  |  | Rupees Seven Hundred Twenty Six Ps. Seventy Five Only. |  |  |  |  |  |
| 16. | Painting New surface(a) Preparing surface painting corrugated surface, patent roofing etc. (SINO.5(a+b) P-69) for Two Coats |  |  |  |  |  |  |  |
|  |  |  | 80.00 | 674/60 | %Sft |  | Rs.540/- |  |
|  |  |  | Rupees Six Hundred Seventy Four Ps. Sixty Only. |  |  |  |  |  |

|  |  |  | TOTAL | Rs.843338/- |
| --- | --- | --- | --- | --- |

% Above / Below (Amount to be added / deducted).		Rs.

Grand Total		Rs.

Total Amount in word:

Note:-	 Quantities and rates are provisional and may be changed as per /according to the Technical Sanction by the competent authority.

| CONTRACTOR | EXECUTIVE ENGINEER (BUILDINGS) WORKS & SERVICES DEPARTMENT KHAIRPUR |
| --- | --- |

(SCHEDULE B)

CONSTRUCTION OF TOMB WITH C.C BLOCK DARGAH SYED KORAL SHAH

TALUKA KINGRI DISTRICT KHAIRPUR.

PART A MAIN BUILDING

| SR# | ITEM OF WORKS |  | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- |
| 01. | Excavation in foundation of buildings bridges and other structures including dag belling dressing refilling around structure with excavated earth watering and ramming lead upto 5ft. (b) in ordinary soil. (S.I.No.18 P.No.5). |  |  |  |  |  |
|  |  |  | 500.00 | 3176/25 | %0Cft | Rs.1588/- |
|  |  |  | (Rs. Three Thousand One Hundred Seventy Six Ps. Twenty Five Only). |  |  |  |
| 02. | Cement concrete brick or stone ballast 1-1/2 to 2 gauge. (A) ratio 1:5:10. (S.I.No.4 P.No.17). |  |  |  |  |  |
|  |  |  | 153.00 | 8694/95 | %Cft | Rs.13303/- |
|  |  |  | (Rs. Eight Thousand Six Hundred Ninety Four Ps. Ninety Five Only). |  |  |  |

| 03. | Un Coursed Rubble masonry in cement sand mortar ratio 1:6 (S.I.No.2 P.No.27). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 144.00 | 17723/- | %Cft | Rs.25521/- |
|  |  |  | Rupees Seventeen Thousand Seven Hundred Twenty Three Only. |  |  |  |

| 04. | Coursed Rubble masonry i/c hammer cement sand mortar ratio 1:6 (S.I.No.3 P.No.31). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 217.00 | 25321/- | %Cft | Rs.54947/- |
|  |  |  | (Rs. Twenty Five Thousand Three Hundred Twenty One Only). |  |  |  |
| 05. | Pacca Brick work in foundation & plinth in cement & sand mortar Ratio 1:6 (S.I.No.4 (c) P.No.25). |  |  |  |  |  |
|  |  |  | 18.00 | 11948/36 | %Cft | Rs.2150/- |
|  |  |  | Rupees Eleven Thousand Nine Hundred Forty Eight Ps. Thirty Six Only. |  |  |  |
| 06. | R.C.C works in roof slab, beams columns rafts lintels and other structural members laid in situ recast laid in position complete in all respects ratio 1:2:4 90lbs cement 2cft sand 4cft shingle 1/8 to ¼ gauge (S.I.No.06 P.No.19). |  |  |  |  |  |
| (a) | Ground Floor |  | 111.00 | 337/- | %Cft | Rs.37407/- |
|  |  |  | Rupees Three Hundred Thirty Seven Only. |  |  |  |
| 07. | Fabrication of mild steel reinforcement for cement concrete including cutting bending, laying in position making joints and fastenings including cost of binding wire also including cost of binding wire also includes removal of rust from bars.(S.I.No.07 P.No.20). |  |  |  |  |  |
|  |  |  | 4.955 | 5001/70 | P.Cwt | Rs.24783/- |
|  |  |  | Rupees Five Thousand One Ps. Seventy Only. |  |  |  |
| 08. | Pacca brick work in Ground Floor and plinth in Ratio 1:6. (S.I.No.04 P.No.24). |  |  |  |  |  |
| (a) | Ground Floor |  | 643.00 | 12674/36 | %Cft | Rs.81496/- |
|  |  |  | Rupees Twelve Thousand Six Hundred Seventy Four Ps. Thirty Six Only. |  |  |  |
| 09. | Providing and Fixing Iron Girders (material Sch: item No: 140) |  |  |  |  |  |
|  |  |  | 13.142 | 3850/- | P.Cwt | Rs.50597/- |
|  |  |  | (Rs. Three Thousands Eight Hundred Fifty Only). |  |  |  |

| SR# | ITEM OF WORKS |  | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- |
| 10. | Supplying fixing T-Irons (material Sch: item No: 144) |  |  |  |  |  |
|  |  |  | 6.910 | 3575/- | P.Cwt | Rs.24703/- |
|  |  |  | (Rs. Three Thousands Five Hundred Seventy Five Only). |  |  |  |
| 11. | Second class tile roofing consisting of 4" earth and 1" mud plaster with Gobri leeping over 1/2"thick cement plaster 1:6 with 34 Lbs. Of hot bitumen coating sand blinded, provided over one layer of 12" x 6"x2" tiles laid in 1:6 cement mortar including 1:2 |  |  |  |  |  |
|  |  |  | 398.00 | 7607/25- | %Sft | Rs.30277/- |
|  |  |  | (Rs. Seventy Six Thousands Seven Ps. Twenty Five Only). |  |  |  |
| 12. | Filling watering and ramming earth in floor with surplus earth from foundation lead upto one chain and lift upto 5ft.(S.I.NO:11 P-5) |  |  |  |  |  |
|  |  |  | 333.00 | 1512/50 | %Cft | Rs.504/- |
|  |  |  | (Rs. One Thousand Five Hundred Eleven Ps. Fifty Only). |  |  |  |

| 13. | Filling watering and ramming earth under floor with new excavated from outside lead upto one chain and lift upto 5ft (S.I.No.12 P.No.5). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 402.00 | 3630/- | %0Cft | Rs.1459/- |
|  |  |  | Rupees Three Thousand Six Hundred Thirty Only. |  |  |  |
| 14. | Extra lead for 3”mile. |  |  |  |  |  |
|  |  |  | 402.00 | 579/41 | %Cft | Rs.2329/- |
|  |  |  | Rupees Five Hundred Seventy Nine Ps. Forty One Only. |  |  |  |

| 15. | Supplying and filling sand under floor and plugging in walls.(S.I.No.29 P.No.31). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 161.00 | 1141/75 | %Cft | Rs.1837/- |
|  |  |  | Rupees One Thousand One Hundred Forty One Ps. Seventy Five Only. |  |  |  |
| 16. | Cement concrete brick or stone ballast 1-1/2 to 2 gauge. (A) ratio 1:6:12. (S.I.No. P.No. ). |  |  |  |  |  |
|  |  |  | 106.00 | 8122/95 | %Cft | Rs.8610/- |
|  |  |  | (Rs. Eight Thousand One Hundred Ninety Five Ps. Ninety Five Only). |  |  |  |
| 17. | Cement plaster 1:6 upto 20 height ½ thick. (S.I.No.13 P.No.58). |  |  |  |  |  |
| (a) | Ground Floor |  | 821.00 | 2206/60 | %Sft | Rs.18116/- |
|  |  |  | Rupees Two Thousand Two Hundred Six Ps. Sixty Five Only. |  |  |  |
| 18. | Cement plaster 1:4 upto 20 height 3/8 thick. (S.I.No.11 P.No.58). |  |  |  |  |  |
| (a) | Ground Floor |  | 821.00 | 2197/52 | %Sft | Rs.18042/- |
|  |  |  | Rupees Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only. |  |  |  |
| 19. | Providing and fixing G.I. frames chowkats of size 7x2xor 4-1/2x3 for door using 20 gauge G.I. sheet i/c welded hinge and fixing at with cement sand slurry of ration 1.6 despairing the jambs. The cost also i/c all carriage tools and plants used in making and fixing.(S.I.NO.18 P-98). |  |  |  |  |  |
|  |  |  | 19.00 | 228/90 | P.Rft | Rs.43491/- |
|  |  |  | Rupees Two Hundred Twenty Eight Ps. Ninety Only. |  |  |  |

| SR# | ITEM OF WORKS |  | QTY: | RATE | UNIT | AMOUNT |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 20. | Providing and fixing G.I. Frames chowkats of size 7x2xir 4-1/2x3 for windows using 20guage G.I. sheet i/c welded hinges and fixing at site with necessary hold fasts, filling with cement sand slurry of ration 1;6 and repainting the jamb. The cost also i/c all carriage tools and plants used in making and fixing. (S.I.No.29 P.No.98). |  |  |  |  |  |  |
|  |  |  | 44.00 | 240/50 | P.Rft |  | Rs.10582/- |
|  |  |  | Rupees Two Hundred Forty Ps. Fifty Only. |  |  |  |  |
| 21. | Cement concrete plain including placing compacting finishing and curing, complete (including screening and washing of stone aggregate without shuttering Ratio 1:2:4. (S.I.No.05 P.No.18). |  |  |  |  |  |  |
|  |  |  | 58.00 | 14429/- | %Cft | Rs.8369/- |  |
|  |  |  | Rupees Fourteen Thousand Four Hundred Twenty Nine Only. |  |  |  |  |
| 22. | Laying floor of approved colour glazed tiles 1/4" thick laid in white cement and pigement on a bned of 3/4" thick cement mortor 1:2 ( S.I.No. 25 P- 43 ) |  |  |  |  |  |  |
| (a) | Ground Floor |  | 340.00 | 27747/06 | %Sft |  | Rs.94340/- |
|  |  |  | Rupees Twenty Seven Thousand Seven Hundred Forty Seven Ps. Zero Six Only. |  |  |  |  |
| 23. | White glazed tile 1/4" thick dado jointed in white cement and laid over 1:2 cement sand mortar 3/4" thick including finishing. (SINO.37 P-45) |  |  |  |  |  |  |
|  |  |  | 121.00 | 28253/61 | %Sft |  | Rs.34242/- |
|  |  |  | Rupees Twenty Eight Thousand Two Hundred Fifty Three Ps. Sixty One Only. |  |  |  |  |

| 24. | First class deodar wood wrought, joinery in doors & windows etc. fixed in position i/c chowkhats holds fasts hinges, iron tower blts. Chocks cleats, handles and cords with hooks, etc. Deodar paneled or paneled and glazed or fully glazed (without chowkhats). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 63.00 | 902/93 | P.Sft | Rs.56885/- |
|  |  |  | Rupees Nine Hundred Two Ps. Ninety Three Only. |  |  |  |

| 25. | Cement Pointing i/c streaking of joints on wall ratio 1:3 (S.I.NO.16 P.No.21) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 647.00 | 1213/58 | %Sft | Rs.7852/- |
|  |  |  | Rupees One Thousand Two Hundred Thirteen Ps. Fifty Eight Only. |  |  |  |
| 26. | Colour wash two coats. |  |  |  |  |  |
|  |  |  | 647.00 | 859/90 | %Sft | Rs.5564/- |
|  |  |  | Rupees Eight Hundred Fifty Nine Ps. Ninety Only. |  |  |  |

| 27. | Primary coat of chalk under distemper. (S.I.No.23 P.No.59). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 821.00 | 442/75 | %Sft | Rs.3635/- |
|  |  |  | Rupees Four Hundred Forty Two Ps. Seventy Five Only. |  |  |  |
| 28. | Distemper in two coats. (S.I.No.24 P.No.60). |  |  |  |  |  |
|  |  |  | 821.00 | 1079/65 | %Sft | Rs.8864/- |
|  |  |  | Rupees One Thousand Seventy Nine Ps. Sixty Five Only. |  |  |  |

| SR# | ITEM OF WORKS |  | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- |
| 29. | Providing and fixing iron steel grill using solid square bars of size ½”x1/2” placed at 4” i/c and frame of flat iron patti of ¾”x3/4” i/c circle shape at 1-0 a part equivalent fitted with screws are pins i/c painting 3 coats with 1st coat of red oxide paint etc. (S.I.No.26 P.No.97). |  |  |  |  |  |
|  |  |  | 28.00 | 180/50 | P.Sft | Rs.5054/- |
|  |  |  | Rupees One Hundred Eighty Ps. Fifty Only. |  |  |  |
| 30. | Painting New surface(a) Preparing surface painting corrugated surface, patent roofing etc. (SINO.5(a+b) P-69) for Two Coats |  |  |  |  |  |
|  |  |  | 118.00 | 1489/68 | %Sft | Rs.1758/- |
|  |  |  | Rupees One Thousand Four Hundred Eighty Nine Ps. Sixty Eight Only. |  |  |  |

|  |  |  | TOTAL | Rs.678305/- |
| --- | --- | --- | --- | --- |

% Above / Below (Amount to be added / deducted).		Rs.

Grand Total		Rs.

Total Amount in word:

Note:-	 Quantities and rates are provisional and may be changed as per /according to the Technical Sanction by the competent authority.

| CONTRACTOR | EXECUTIVE ENGINEER (BUILDINGS) WORKS & SERVICES DEPARTMENT KHAIRPUR |
| --- | --- |

(SCHEDULE B)

CONSTRUCTION OF TOMB WITH C.C BLOCK DARGAH SYED KORAL SHAH

TALUKA KINGRI DISTRICT KHAIRPUR.

PART B PAVED COURTYARD

| SR# | ITEM OF WORKS |  | QTY: | RATE | UNIT |  | AMOUNT |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01. | Excavation in foundation of buildings bridges and other structures including dag belling dressing refilling around structure with excavated earth watering and ramming lead upto 5ft. (b) in ordinary soil. (S.I.No.18 P.No.5). |  |  |  |  |  |  |  |
|  |  |  | 126.00 | 3176/25 | %0Cft |  | Rs.400/- |  |
|  |  |  | (Rs. Three Thousand One Hundred Seventy Six Ps. Twenty Five Only). |  |  |  |  |  |
| 02. | Cement concrete brick or stone ballast 1-1/2 to 2 gauge. (A) ratio 1:5:10. (S.I.No.4 P.No.17). |  |  |  |  |  |  |  |
|  |  |  | 340.00 | 8694/95 | %Cft |  | Rs.29563/- |  |
|  |  |  | (Rs. Eight Thousand Six Hundred Ninety Four Ps. Ninety Five Only). |  |  |  |  |  |
| 03. | Pacca Brick work in foundation & plinth in cement & sand mortar Ratio 1:6 (S.I.No.4 (c) P.No.25). |  |  |  |  |  |  |  |
|  |  |  | 223.00 | 11948/36 | %Cft |  | Rs.26645/- |  |
|  |  |  | Rupees Eleven Thousand Nine Hundred Forty Eight Ps. Thirty Six Only. |  |  |  |  |  |
| 04. | Supplying and filling sand under floor and plugging in walls.(S.I.No.29 P.No.31). |  |  |  |  |  |  |  |
|  |  |  | 1260.00 | 1141/25 | %Cft |  |  | Rs.14379/- |
|  |  |  | Rupees One Thousand One Hundred Forty One Ps. Seventy Five Only. |  |  |  |  |  |
| 05. | P/L 3" thick topping c.c. 1:2:4 including surface finishing and dividing into panels.(SINO.16(c) P-41) |  |  |  |  |  |  |  |
|  |  |  | 840.00 | 4411/82 |  | %Sft |  | Rs.37059/- |
|  |  |  | Rupees One Thousand Two Hundred Thirteen Ps. Fifty Eight Only. |  |  |  |  |  |
| 06. | Cement plaster 1:6 upto 20 height 1/2”thick. (S.I.No. 13 P.No. 51 ). |  |  |  |  |  |  |  |
|  |  |  | 526.00 | 2260/06 | %Sft |  |  | Rs.11607/- |
|  |  |  | (Rs. Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only). |  |  |  |  |  |
| 07. | Cement plaster 1:4 upto 20 height 3/8”thick. (S.I.No.11 P.No.58). |  |  |  |  |  |  |  |
|  |  |  | 526.00 | 2197/52 | %Sft |  |  | Rs.11559/- |
|  |  |  | (Rs. Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only). |  |  |  |  |  |
| 08. | Primary coat of chalk under distemper. (S.I.No.23 P.No.59). |  |  |  |  |  |  |  |
|  |  |  | 526.00 | 442/72 |  | %Sft |  | Rs.2329/- |
|  |  |  | Rupees Four Hundred Forty Two Ps. Seventy Two Only. |  |  |  |  |  |
| 09. | Distemper in Three coats. (S.I.No.24 P.No.59). |  |  |  |  |  |  |  |
|  |  |  | 529.00 | 1079/65 |  | %Sft |  | Rs.5679/- |
|  |  |  | Rupees One Thousand Seventy Nine Ps. Sixty Five Only. |  |  |  |  |  |

| SR# | ITEM OF WORKS |  | QTY: | RATE | UNIT |  | AMOUNT |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10. | Providing and fixing iron steel grill using solid square bars of size ½”x1/2” placed at 4” i/c and frame of flat iron patti of ¾”x3/4” i/c circle shape at 1-0 a part equivalent fitted with screws are pins i/c painting 3 coats with 1st coat of red oxide paint etc. (S.I.No.26 P.No.97). |  |  |  |  |  |  |  |
|  |  |  | 186.00 | 180/50 |  | P.Sft |  | Rs.33573/- |
|  |  |  | Rupees One Hundred Eighty Ps. Fifty Only. |  |  |  |  |  |

|  |  |  | TOTAL | Rs.172793/- |
| --- | --- | --- | --- | --- |

% Above / Below (Amount to be added / deducted).		Rs.

Grand Total		Rs.

Total Amount in word:

Note:-	 Quantities and rates are provisional and may be changed as per /according to the Technical Sanction by the competent authority.

| CONTRACTOR | EXECUTIVE ENGINEER (BUILDINGS) WORKS & SERVICES DEPARTMENT KHAIRPUR |
| --- | --- |

(SCHEDULE B)

REHIBLITATION/CONSTRUCTION OF REVENUE COMPLEX / HOUSES TALUKA KINGRI

DISTRICT KHAIRPUR.

PART A MAIN BUILDING REVENUE

| SR# | ITEM OF WORKS | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- |

| 01. | Removing of Door with chowkhats etc completed. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 25Nos. | 142/18 | Each | Rs.4123/- |
|  |  |  | Rupees One Hundred Forty Two Ps. Eighteen Only. |  |  |  |
| 02. | Removing of Windwo with chowkhats etc completed. |  |  |  |  |  |
|  |  |  | 25Nos. | 102/85 | Each | Rs.2571/- |
|  |  |  | Rupees One Hundred Two Ps. Eighty Five Only. |  |  |  |
| 03. | Removing of Ventilator with chowkhats etc completed. |  |  |  |  |  |
|  |  |  | 8Nos. | 51/ | Each | Rs.408/- |
|  |  |  | Rupees Fifty One Only. |  |  |  |

| 04. | Dismantling brick work in lime or cement mortar.(S.I.No: 13 P-10) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 516.00 | 1285/63- | %Cft | Rs.6634/- |
|  |  |  | (Rs. One Thousand Two Hundred Eighty Five Ps.Sixty Three Only). |  |  |  |
| 05. | Dismantling R.C.C 1:2:4 cement concrete reinforcement (S.I.No.20 P.No.10). |  |  |  |  |  |
|  |  |  | 6258.00 | 5445/- | %0Cft | Rs.340748/- |
|  |  |  | (Rs. Five Thousand Four Hundred Forty Five Only). |  |  |  |
| 06. | Removing cement or lime plaster from walls (S.I.No.53 P.No.13). |  |  |  |  |  |
|  |  |  | 25844.00 | 121/- | %Sft | Rs.31271/- |
|  |  |  | (Rupees . One Hundred Twenty One Only) . |  |  |  |

| 07. | Dismantling cement concrete plain 1:2:4.(S.I.NO:19 P-11). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1486.00 | 3327/60 | %Sft | Rs.49447/- |
|  |  |  | Rupees Three Thousand Three Hundred Twenty Seven Ps. Sixty Only. |  |  |  |

| 08. | R.C.C works in roof slab, beams columns rafts lintels and other structural members laid in situ recast laid in position complete in all respects ratio 1:2:4 90lbs cement 2cft sand 4cft shingle 1/8 to ¼ gauge (S.I.No.06 P.No.19). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| (a) | Ground Floor |  | 7020.00 | 337/- | %Cft | Rs.2365740/- |
|  |  |  | Rupees Three Hundred Thirty Seven Only. |  |  |  |
| 09. | Fabrication of mild steel reinforcement for cement concrete including cutting bending, laying in position making joints and fastenings including cost of binding wire also including cost of binding wire also includes removal of rust from bars.(S.I.No.07 P.No.20). |  |  |  |  |  |
|  |  |  | 370.481 | 5001/70 | P.Cwt | Rs.1853035/- |
|  |  |  | Rupees Five Thousand One Ps. Seventy Only. |  |  |  |
| 10. | Pacca brick work in Ground Floor and plinth in Ratio 1:6. (S.I.No.04 P.No.24). |  |  |  |  |  |
| (a) | Ground Floor |  | 1128.00 | 12674/36 | %Cft | Rs.142967/- |
|  |  |  | Rupees Twelve Thousand Six Hundred Seventy Four Ps. Thirty Six Only. |  |  |  |

| 11. | Fixing of Old Door with chowkhats etc completed. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 12 Nos. | 279/51 | Each | Rs.3354/- |
|  |  |  | Rupees Two Hundred Seventy Nine Ps. Fifty one Only. |  |  |  |
| 12. | Fixing of Old Window with chowkhats etc completed. |  |  |  |  |  |
|  |  |  | 7 Nos. | 139/79 | Each | Rs.979/- |
|  |  |  | Rupees One Hundred Thirty Nine Ps. Seventy Nine Only. |  |  |  |

| SR# | ITEM OF WORKS |  | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- |
| 13. | Providing and fixing G.I. frames chowkats of size 7x2xor 4-1/2x3 for door using 20 gauge G.I. sheet i/c welded hinge and fixing at with cement sand slurry of ration 1.6 despairing the jambs. The cost also i/c all carriage tools and plants used in making and fixing.(S.I.NO.18 P-98). |  |  |  |  |  |
|  |  |  | 357.00 | 228/90 | P.Rft | Rs.81717/- |
|  |  |  | Rupees Two Hundred Twenty Eight Ps. Ninety Only. |  |  |  |
| 14. | Providing and fixing G.I. frames chowkats of size 7x2xor 4-1/2x3 for door using 20 gauge G.I. sheet i/c welded hinge and fixing at with cement sand slurry of ration 1.6 despairing the jambs. The cost also i/c all carriage tools and plants used in making and fixing.(S.I.NO.18 P-98). |  |  |  |  |  |
|  |  |  | 536.00 | 240/50 | P.Rft | Rs.128908/- |
|  |  |  | Rupees Two Hundred Forty Ps. Fifty Only. |  |  |  |
| 15. | Supplying and filling sand under floor and plugging in walls.(S.I.No.29 P.No.31). |  |  |  |  |  |
|  |  |  | 4337.00 | 1141/75 | %Cft | Rs.49496/- |
|  |  |  | Rupees One Thousand One Hundred Forty One Ps. Seventy Five Only. |  |  |  |
| 16. | Filling watering and ramming earth under floor with new excavated from outside lead upto one chain and lift upto 5feet. (S.I.No.12 P.No.5). |  |  |  |  |  |
|  |  |  | 4337.00 | 3630/- | %0Cft | Rs.15743/- |
|  |  |  | Rupees Three Thousand Six Hundred Thirty Only. |  |  |  |
| 17. | Extra lead for 3”mile. |  |  |  |  |  |
|  |  |  | 4337.00 | 579/41 | %Cft | Rs.15129/- |
|  |  |  | Rupees Five Hundred Seventy Nine Ps. Forty One Only. |  |  |  |
| 18. | Cement concrete brick or stone ballast 1-1/2 to 2 gauge. (A) ratio 1:4:8. (S.I.No. P.No. ). |  |  |  |  |  |
|  |  |  | 2862.00 | 9416/95 | %Cft | Rs.269513/- |
|  |  |  | (Rs. Nine Thousand Four Hundred Sixteen Ps. Ninety Five Only). |  |  |  |
| 19. | Cement concrete plain 1:2:4 including placing compacting finishing and curing, complete (including screening and washing of stone aggregate without shuttering Ratio 1:2:4. (S.I.No.05 P.No.18). |  |  |  |  |  |
|  |  |  | 1431.00 | 14429/- | %Cft | Rs.206483/- |
|  |  |  | Rupees Fourteen Thousand Four Hundred Twenty Nine Only. |  |  |  |
| 20. | Cement plaster 1:6 upto 20 height ½ thick. (S.I.No.13 P.No.58). |  |  |  |  |  |
| (a) | Ground Floor |  | 25844.00 | 2206/60 | %Sft | Rs.570274/- |
|  |  |  | Rupees Two Thousand Two Hundred Six Ps. Sixty Five Only. |  |  |  |
| 20. | Cement plaster 1:4 upto 20 height 3/8 thick. (S.I.No.11 P.No.58). |  |  |  |  |  |
| (a) | Ground Floor |  | 25844.00 | 2197/52 | %Sft | Rs.567927/- |
|  |  |  | Rupees Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only. |  |  |  |
| 22. | Cement Pointing i/c streaking of joints on wall ratio 1:3 (S.I.NO.16 P.No.21) |  |  |  |  |  |
|  |  |  | 1197.00 | 1213/58 | %Sft | Rs.15411/- |
|  |  |  | Rupees One Thousand Two Hundred Thirteen Ps. Fifty Eight Only. |  |  |  |

| 23. | Providing and laying 1”thick topping cement concrete 1:2:4 i/c surface finishing and dividing into panels. (S.I.No.16 (d) P.No.47). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 17093.00 | 3275/50 | %Sft | Rs.559881/- |
|  |  |  | Rupees Three Thousand Two Hundred Seventy Five Ps. Fifty Only. |  |  |  |

| SR# | ITEM OF WORKS |  | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- |
| 24. | First class deodar wood wrought, joinery in doors & windows etc. fixed in position i/c chowkhats holds fasts hinges, iron tower blts. Chocks cleats, handles and cords with hooks, etc. Deodar paneled or paneled and glazed or fully glazed (without chowkhats). |  |  |  |  |  |
|  |  |  | 686.00 | 902/93 | P.Sft | Rs.619410/- |
|  |  |  | Rupees Nine Hundred Two Ps. Ninety Three Only. |  |  |  |

| 25. | Laying white marble flooring fine dressed in the surface without winging set in lime mortar 1:2 including rubbing and polishing of the joints. (a) ¾”thick flooring. (S.I.No.28 P.No.49). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1596.00 | 567/48 | %Sft | Rs.905698/- |
|  |  |  | Rupees Five Hundred Sixty Seven Ps. Forty Eighty Only. |  |  |  |
| 26. | P/F 3/8 thick marble tiles of approved quality and colour shade size 8x4/6x4 in dado skirting and facing removal/tucking of existing plaster surface etc over 1/2thick base of cement mortar 1:3 setting mortar base including filling the joins and washing the tiles with white cement slurry. Current, finishing cleaning and polishing etc complete for new works.(S.I.NO.68 P-55). |  |  |  |  |  |
|  |  |  | 133.00 | 186/04 | %Sft | Rs.24743/- |
|  |  |  | Rupees One Hundred Eighty Six Ps. Four Only. |  |  |  |

| 27. | Laying floor of approved colour glazed tiles 1/4" thick laid in white cement and pigement on a bned of 3/4" thick cement mortor 1:2 ( S.I.No. 25 P- 43 ) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| (a) | Ground Floor |  | 2347.00 | 27747/06 | %Sft | Rs.651223/- |
|  |  |  | Rupees Twenty Seven Thousand Seven Hundred Forty Seven Ps. Zero Six Only. |  |  |  |
| 28. | White glazed tile 1/4" thick dado jointed in white cement and laid over 1:2 cement sand mortar 3/4" thick including finishing. (SINO.37 P-45) |  |  |  |  |  |
|  |  |  | 1046.00 | 28253/61 | %Sft | Rs.296011/- |
|  |  |  | Rupees Twenty Eight Thousand Two Hundred Fifty Three Ps. Sixty One Only. |  |  |  |

| 29. | Supplying and fixing imposition aluminum channels, framing for hinged doors or Alco made with 5mm thick tinted glass glazing (Belgium)and alpha (Japan)locks i/c handles stoppers etc deluxe mode(bronze) (S.I.NO.83 P-114). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 152.00 | 1507/66 | P.Sft | Rs.229169/- |
|  |  |  | Rupees One Thousand Five Hundred Seven Ps. Sixty Six Only. |  |  |  |

| 30. | Colour wash two coats. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 3891.00 | 859/90 | %Sft | Rs.33458/- |
|  |  |  | Rupees Eight Hundred Fifty Nine Ps. Ninety Only. |  |  |  |

| 31. | Primary coat of chalk under distemper. (S.I.No.23 P.No.59). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 25844.00 | 442/75 | %Sft | Rs.114424/- |
|  |  |  | Rupees Four Hundred Forty Two Ps. Seventy Five Only. |  |  |  |
| 32. | Distemper in two coats. (S.I.No.24 P.No.60). |  |  |  |  |  |
|  |  |  | 25844.00 | 1079/65 | %Sft | Rs.279025/- |
|  |  |  | Rupees One Thousand Seventy Nine Ps. Sixty Five Only. |  |  |  |
| 33. | Painting New surface(a) Preparing surface painting corrugated surface, patent roofing etc. (SINO.5(a+b) P-69) for Two Coats |  |  |  |  |  |
|  |  |  | 952.00 | 674/60 | %Sft | Rs.6422/- |
|  |  |  | Rupees One Six Hundred Seventy Four Ps. Sixty Six Only. |  |  |  |
| 34. | Painting New surface(a) Preparing surface painting corrugated surface, patent roofing etc. (SINO.5(a+b) P-69) for Two Coats |  |  |  |  |  |
|  |  |  | 1373.00 | 1489/68 | %Sft | Rs.20453/- |
|  |  |  | Rupees One Six Hundred Seventy Four Ps. Sixty Six Only. |  |  |  |

| SR# | ITEM OF WORKS |  | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- |
| 35. | P/L tiles HALA or pattern tiles glazed 6"x6"x1/2" on floor or wall facing in required floor and pattern of STILE specification jointed in white cement and pigment over a base of 1:2 (SINO.61 P-48) |  |  |  |  |  |
|  |  |  | 260.00 | 47651/56 | %Sft | Rs.123894/- |
|  |  |  | Rupees Forty Seven Thousand Six Hundred Fifty One Ps. Fifty Six Only. |  |  |  |
|  |  |  | TOTAL |  |  | Rs.10585689/- |

% Above / Below (Amount to be added / deducted).		Rs.

Grand Total		Rs.

Total Amount in word:

Note:-	 Quantities and rates are provisional and may be changed as per /according to the Technical Sanction by the competent authority.

| CONTRACTOR | EXECUTIVE ENGINEER (BUILDINGS) WORKS & SERVICES DEPARTMENT KHAIRPUR |
| --- | --- |

(SCHEDULE B)

REHIBLITATION/CONSTRUCTION OF REVENUE COMPLEX / HOUSES TALUKA KINGRI

DISTRICT KHAIRPUR.

PART A MAIN BUILDING VEHICLE SHADE

| SR# | ITEM OF WORKS |  | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- |
| 01. | Excavation in foundation of buildings bridges and other structures including dag belling dressing refilling around structure with excavated earth watering and ramming lead upto 5ft. (b) in ordinary soil. (S.I.No.18 P.No.5). |  |  |  |  |  |
|  |  |  | 626.00 | 3176/25 | %0Cft | Rs.1988/- |
|  |  |  | (Rs. Three Thousand One Hundred Seventy Six Ps. Twenty Five Only). |  |  |  |
| 02. | Cement concrete brick or stone ballast 1-1/2 to 2 gauge. (A) ratio 1:5:10. (S.I.No.4 P.No.17). |  |  |  |  |  |
|  |  |  | 193.00 | 8694/95 | %Cft | Rs.16781/- |
|  |  |  | (Rs. Eight Thousand Six Hundred Ninety Four Ps. Ninety Five Only). |  |  |  |

| 03. | Pacca Brick work in foundation & plinth in cement & sand mortar Ratio 1:6 (S.I.No.4 (c) P.No.25). |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 151.00 |  | 11948/36 |  | %Cft |  | Rs.18042/- |
|  |  |  |  | Rupees Eleven Thousand Nine Hundred Forty Eight Ps. Thirty Six Only. |  |  |  |  |  |  |
| 04. | R.C.C works in roof slab, beams columns rafts lintels and other structural members laid in situ recast laid in position complete in all respects ratio 1:2:4 90lbs cement 2cft sand 4cft shingle 1/8 to ¼ gauge (S.I.No.06 P.No.19). |  |  |  |  |  |  |  |  |  |
| (a) | Ground Floor |  |  | 875.00 |  | 337/- |  | %Cft |  | Rs.294875/- |
|  |  |  |  | Rupees Three Hundred Thirty Seven Only. |  |  |  |  |  |  |
| 05. | Fabrication of mild steel reinforcement for cement concrete including cutting bending, laying in position making joints and fastenings including cost of binding wire also including cost of binding wire also includes removal of rust from bars.(S.I.No.07 P.No.20). |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 46.875 |  | 5001/70 |  | P.Cwt |  | Rs.234455/- |
|  |  |  |  | Rupees Five Thousand One Ps. Seventy Only. |  |  |  |  |  |  |
| 06. | Cement plaster 1:6 upto 20 height ½ thick. (S.I.No.13 P.No.58). |  |  |  |  |  |  |  |  |  |
| (a) | Ground Floor |  |  |  | 523.00 |  | 2206/60 |  | %Sft | Rs.11541/- |
|  |  |  |  |  | Rupees Two Thousand Two Hundred Six Ps. Sixty Five Only. |  |  |  |  |  |
| 07. | Cement plaster 1:4 upto 20 height 3/8 thick. (S.I.No.11 P.No.58). |  |  |  |  |  |  |  |  |  |
| (a) | Ground Floor |  |  |  | 523.00 |  | 2197/52 |  | %Sft | Rs.11493/- |
|  |  |  |  |  | Rupees Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only. |  |  |  |  |  |

| 08. | Filling watering and ramming earth under floor with new excavated from outside lead upto one chain and lift upto 5ft (S.I.No.12 P.No.5). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1408.00 | 3630/- | %0Cft | Rs.5333/- |
|  |  |  | Rupees Three Thousand Six Hundred Thirty Only. |  |  |  |
| 09. | Extra lead for 3”mile. |  |  |  |  |  |
|  |  |  | 1408.00 | 579/41 | %Cft | Rs.8158/- |
|  |  |  | Rupees Five Hundred Seventy Nine Ps. Forty One Only. |  |  |  |

| SR# | ITEM OF WORKS |  | QTY: | RATE | UNIT |  | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 10. | Cement concrete brick or stone ballast 1-1/2 to 2 gauge. (A) ratio 1:4:8. (S.I.No. P.No. ). |  |  |  |  |  |  |
|  |  |  | 352.00 | 9416/95 |  | %Cft | Rs.33145/- |
|  |  |  | (Rs. Nine Thousand Four Hundred Sixteen Ps. Ninety Five Only). |  |  |  |  |
| 11. | Cement concrete plain including placing compacting finishing and curing, complete (including screening and washing of stone aggregate without shuttering Ratio 1:2:4. (S.I.No.05 P.No.18). |  |  |  |  |  |  |
|  |  |  | 704.00 | 4411/82 | %Cft |  | Rs.31059/- |
|  |  |  | Rupees Four Thousand Four Hundred Eleven Ps Eighty Two Only. |  |  |  |  |
| 12. | White washing Three coats. |  |  |  |  |  |  |
|  |  |  | 1397.00 | 829/50 | %Sft |  | Rs.11594/- |
|  |  |  | Rupees Eighty Hundred Fifty Nine Ps. Ninety Only. |  |  |  |  |
| 13. | P/F colour create to wall surface to provide, durable crust and aesthetics having thickness upto 3/4" with specified colour having water, fire and temite resistance ( upto 20'-0 height) (SINO.44 P-56 ) |  |  |  |  |  |  |
|  |  |  | 819.00 | 6319/50 | %Sft |  | Rs.51756/- |
|  |  |  | Rupees Six Thousand Three Hundred Ninteen Ps. Fifty Only. |  |  |  |  |

|  |  |  | TOTAL | Rs.729998/- |
| --- | --- | --- | --- | --- |

% Above / Below (Amount to be added / deducted).		Rs.

Grand Total		Rs.

Total Amount in word:

Note:-	 Quantities and rates are provisional and may be changed as per /according to the Technical Sanction by the competent authority.

| CONTRACTOR | EXECUTIVE ENGINEER (BUILDINGS) WORKS & SERVICES DEPARTMENT KHAIRPUR |
| --- | --- |

(SCHEDULE B)

REHIBLITATION/CONSTRUCTION OF REVENUE COMPLEX / HOUSES TALUKA KINGRI

DISTRICT KHAIRPUR.

COMPOUND WALL

| SR# | ITEM OF WORKS |  |  |  | QTY: |  | RATE | UNIT |  | AMOUNT |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01. | Excavation in foundation of buildings bridges and other structures including dag belling dressing refilling around structure with excavated earth watering and ramming lead upto 5ft. (b) in ordinary soil. (S.I.No.18 P.No.5). |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 1154.00 |  | 3176/25 | %0Cft |  | Rs.3665/- |  |
|  |  |  |  |  | (Rs. Three Thousand One Hundred Seventy Six Ps. Twenty Five Only). |  |  |  |  |  |  |
| 02. | Cement concrete brick or stone ballast 1-1/2 to 2 gauge. (A) ratio 1:5:10. (S.I.No.4 P.No.17). |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 289.00 |  | 8694/95 | %Cft |  | Rs.25128/- |  |
|  |  |  |  |  | (Rs. Eight Thousand Six Hundred Ninety Four Ps. Ninety Five Only). |  |  |  |  |  |  |
| 03. | Un Coursed Rubble masonry in cement sand mortar ratio 1:6 (S.I.No.2 P.No.27). |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 413.00 |  | 17723/- |  | %Cft |  | Rs.73196/- |
|  |  |  |  |  | Rupees Seventeen Thousand Seven Hundred Twenty Three Only. |  |  |  |  |  |  |
| 04. | Coursed Rubble masonry i/c hammer cement sand mortar ratio 1:6 (S.I.No.3 P.No.31). |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 466.00 |  | 25321/- |  | %Cft |  | Rs.117996/- |
|  |  |  |  |  | (Rs. Twenty Five Thousand Three Hundred Twenty One Only). |  |  |  |  |  |  |
| 04. | R.C.C works in roof slab, beams columns rafts lintels and other structural members laid in situ recast laid in position complete in all respects ratio 1:2:4 90lbs cement 2cft sand 4cft shingle 1/8 to ¼ gauge (S.I.No.06 P.No.19). |  |  |  |  |  |  |  |  |  |  |
| (a) | Ground Floor |  |  | 389.00 |  | 337/- |  |  | %Cft |  | Rs.131093/- |
|  |  |  |  | Rupees Three Hundred Thirty Seven Only. |  |  |  |  |  |  |  |
| 05. | Fabrication of mild steel reinforcement for cement concrete including cutting bending, laying in position making joints and fastenings including cost of binding wire also including cost of binding wire also includes removal of rust from bars.(S.I.No.07 P.No.20). |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 17.366 |  | 5001/70 |  |  | P.Cwt |  | Rs.86860/- |
|  |  |  |  | Rupees Five Thousand One Ps. Seventy Only. |  |  |  |  |  |  |  |
| 07. | Pacca brick work other than building in Ratio 1:6. (S.I.No.04 P.No.24). |  |  |  |  |  |  |  |  |  |  |
|  | C/wall |  |  |  | 722.00 |  | 12346/65 |  | %Cft |  | Rs.89143/- |
|  |  |  |  |  | Rupees Twelve Thousand Three Hundred Sixty Six Ps. Sixty Five Only. |  |  |  |  |  |  |
| 08. | Cement plaster 1:6 upto 20 height 1/2”thick. (S.I.No. 13 P.No. 51 ). |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 963.00 |  | 2260/06 |  | %Sft |  | Rs.21250/- |
|  |  |  |  |  | (Rs. Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only). |  |  |  |  |  |  |

| SR# | ITEM OF WORKS |  | QTY: | RATE |  | UNIT | AMOUNT |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 09. | Cement plaster 1:4 upto 20 height 3/8”thick. (S.I.No.11 P.No.58). |  |  |  |  |  |  |  |
|  |  |  | 963.00 | 2197/52 |  | %Sft |  | Rs.21162/- |
|  |  |  | (Rs. Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only). |  |  |  |  |  |
| 10. | Cement Pointing i/c streaking of joints on wall ratio 1:3 (S.I.NO.16 P.No.21) |  |  |  |  |  |  |  |
|  |  |  | 963.00 | 1213/58 |  | %Sft |  | Rs.11687/- |
|  |  |  | Rupees One Thousand Two Hundred Thirteen Ps. Fifty Eight Only. |  |  |  |  |  |
| 11. | Colour wash two coats. |  |  |  |  |  |  |  |
|  |  |  | 963.00 | 859/90 |  | %Sft |  | Rs.8281/- |
|  |  |  | Rupees Eight Hundred Fifty Nine Ps. Ninety Only. |  |  |  |  |  |
| 10. | Providing and fixing iron steel grill using solid square bars of size ½”x1/2” placed at 4” i/c and frame of flat iron patti of ¾”x3/4” i/c circle shape at 1-0 a part equivalent fitted with screws are pins i/c painting 3 coats with 1st coat of red oxide paint etc. (S.I.No.26 P.No.97). |  |  |  |  |  |  |  |
|  |  |  | 550.00 | 180/50 |  | P.Sft |  | Rs.99275/- |
| 13. | M/F steel grated doors complete with locking arrangement angloe iron frame 2"x2"x3.8" and 3/4" sq. bars 4" centre to centre (SINO.23 P-92) |  |  |  |  |  |  |  |
|  |  |  | 144.00 | 726/72 | P.Sft |  |  | Rs.104648/- |
|  |  |  | Rupees Seven Hundred Twenty Six Ps. Seventy Five Only. |  |  |  |  |  |
| 14. | Painting New surface(a) Preparing surface painting corrugated surface, patent roofing etc. (SINO.5(a+b) P-69) for Two Coats |  |  |  |  |  |  |  |
|  |  |  | 288.00 | 1489/68 | P.Sft |  |  | Rs.1942/- |
|  |  |  | Rupees One Thousand Four Hundred Eighty Nine Ps. Sixty Eight Only. |  |  |  |  |  |
|  |  |  | Rupees One Hundred Eighty Ps. Fifty Only. |  |  |  |  |  |

|  |  |  | TOTAL | Rs.795326/- |
| --- | --- | --- | --- | --- |

% Above / Below (Amount to be added / deducted).		Rs.

Grand Total		Rs.

Total Amount in word:

Note:-	 Quantities and rates are provisional and may be changed as per /according to the Technical Sanction by the competent authority.

| CONTRACTOR | EXECUTIVE ENGINEER (BUILDINGS) WORKS & SERVICES DEPARTMENT KHAIRPUR |
| --- | --- |

(SCHEDULE B)

REHIBLITATION/CONSTRUCTION OF REVENUE COMPLEX / HOUSES TALUKA KINGRI

DISTRICT KHAIRPUR.

PART  EXTERNAL DEVELOPMENT/PAVED COURTYARED

| SR# | ITEM OF WORKS |  | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- |
| 01. | Excavation in foundation of buildings bridges and other structures including dag belling dressing refilling around structure with excavated earth watering and ramming lead upto 5ft. (b) in ordinary soil. (S.I.No.18 P.No.5). |  |  |  |  |  |
|  |  |  | 1620.00 | 3176/25 | %0Cft | Rs.5146/- |
|  |  |  | (Rs. Three Thousand One Hundred Seventy Six Ps. Twenty Five Only). |  |  |  |
| 02. | Cement concrete brick or stone ballast 1-1/2 to 2 gauge. (A) ratio 1:5:10. (S.I.No.4 P.No.17). |  |  |  |  |  |
|  |  |  | 540.00 | 8694/95 | %Cft | Rs.46952/- |
|  |  |  | (Rs. Eight Thousand Six Hundred Ninety Four Ps. Ninety Five Only). |  |  |  |
| 03. | Pacca Brick work in foundation & plinth in cement & sand mortar Ratio 1:6 (S.I.No.4 (c) P.No.25). |  |  |  |  |  |
|  |  |  | 2230.00 | 11948/36 | %Cft | Rs.266448/- |
|  |  |  | Rupees Eleven Thousand Nine Hundred Forty Eight Ps. Thirty Six Only. |  |  |  |
| 04. | Cement plaster 1:6 upto 20 height 1/2”thick. (S.I.No. 13 P.No. 51 ). |  |  |  |  |  |
|  |  |  | 2970.00 | 2260/06 | %Sft | Rs.65536/- |
|  |  |  | (Rs. Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only). |  |  |  |
| 05. | Cement plaster 1:4 upto 20 height 3/8”thick. (S.I.No.11 P.No.58). |  |  |  |  |  |
|  |  |  | 2970.00 | 2197/52 | %Sft | Rs65266/- |
|  |  |  | (Rs. Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only). |  |  |  |

| 09. | Filling watering and ramming earth under floor with new excavated from outside lead upto one chain and lift upto 5feet. (S.I.No.12 P.No.5). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1500.00 | 3630/- | %0Cft | Rs.5445/- |
|  |  |  | Rupees Three Thousand Six Hundred Thirty Only. |  |  |  |
| 10. | Extra lead for 3”mile. |  |  |  |  |  |
|  |  |  | 1500.00 | 579/41 | %Cft | Rs.8691/- |
|  |  |  | Rupees Five Hundred Seventy Nine Ps. Forty One Only. |  |  |  |
| 11. | Cement concrete brick or stone ballast 1-1/2 to 2 guage ratio 1:6:12.(S.I.NO.4 P-14) |  |  |  |  |  |
|  |  |  | 500.00 | 8122/95 | %Cft | Rs.40615/- |
|  |  |  | Rupees.Eight thousand One Hundred Twenty Two Ps.Ninety Five |  |  |  |
| 12. | Cement concrete plain including placing compacting finishing and curing, complete (including screening and washing of stone aggregate without shuttering Ratio 1:2:4. (S.I.No.05 P.No.18). |  |  |  |  |  |
|  |  |  | 170.00 | 14429/- | %Cft | Rs.24529/- |
|  |  |  | Rupees Fourteen Thousand Four Hundred Twenty Nine Only. |  |  |  |

| SR# | ITEM OF WORKS |  | QTY: | RATE | UNIT |  | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 13. | P/F cement paving blocks flooring having size of 197x97x60(mm) of city/Quddra / cobble shape with natural colours, having strength b/w 5000 psi to 8500 psi i/c filling the joints with hill sand and laying specified manner / pattern and design etc complete (SN:72 P-49) |  |  |  |  |  |  |
|  |  |  | 1000.00 | 223/97- | 1Sft |  | Rs.223970/- |
|  |  |  | Rupees Three Hundred Thirty Seven Only. |  |  |  |  |
| 14. | Stone dust courser for paving block .(S.I.No P.No. ). |  |  |  |  |  |  |
|  |  |  | 100.00 | 2500/- | %Sft | Rs.2500/- |  |
|  |  |  | Rupees Two Thousand Five Hundred Only |  |  |  |  |

|  |  |  | TOTAL | Rs.755098/- |
| --- | --- | --- | --- | --- |

% Above / Below (Amount to be added / deducted).		Rs.

Grand Total		Rs.

Total Amount in word:

Note:-	 Quantities and rates are provisional and may be changed as per /according to the Technical Sanction by the competent authority.

| CONTRACTOR | EXECUTIVE ENGINEER (BUILDINGS) WORKS & SERVICES DEPARTMENT KHAIRPUR |
| --- | --- |

(SCHEDULE B)

REHIBLITATION/CONSTRUCTION OF REVENUE COMPLEX / HOUSES TALUKA KINGRI

DISTRICT KHAIRPUR.

PART     MAIN BUILDING iiid TYPE BUNGLOW MUKHTIARKAR

| SR# | ITEM OF WORKS |  | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- |
| 01. | Removing cement or lime plaster from walls (S.I.No.53 P.No.13). |  |  |  |  |  |
|  |  |  | 1432.00 | 121/- | %Sft | Rs.1733/- |
|  |  |  | (Rupees . One Hundred Twenty One Only) . |  |  |  |
| 02. | Dismantling R.C.C 1:2:4 cement concrete reinforcement (S.I.No.20 P.No.10). |  |  |  |  |  |
|  |  |  | 09.00 | 5445/- | %0Cft | Rs.490/- |
|  |  |  | (Rs. Five Thousand Four Hundred Forty Five Only). |  |  |  |

| 03. | Dismantling cement concrete plain 1:2:4.(S.I.NO:19 P-11). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 534.00 | 3327/60 | %Sft | Rs.17766/- |
|  |  |  | Rupees Three Thousand Three Hundred Twenty Seven Ps. Sixty Only. |  |  |  |

| . | Pacca brick work in Ground Floor and plinth in Ratio 1:6. (S.I.No.04 P.No.24). |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (a) | Ground Floor |  |  |  | 05.00 |  | 12674/36 | %Cft | Rs.634/- |
|  |  |  |  |  | Rupees Twelve Thousand Six Hundred Seventy Four Ps. Thirty Six Only. |  |  |  |  |
| 04. | Providing and fixing G.I. frames chowkats of size 7x2xor 4-1/2x3 for door using 20 gauge G.I. sheet i/c welded hinge and fixing at with cement sand slurry of ration 1.6 despairing the jambs. The cost also i/c all carriage tools and plants used in making and fixing.(S.I.NO.18 P-98). |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 54.00 |  | 228/90 | P.Rft | Rs.12361/- |
|  |  |  |  |  | Rupees Two Hundred Twenty Eight Ps. Ninety Only. |  |  |  |  |
| 05. | P/F G.I. Frame / Chowkats of size 7"x2" or 4-1/2"x3" for window using 20 gague G.I.sheet including welded hinges and fixing at site with necessary holds fasts filling with cement sand slurry or ratio 1:6 and repairing the jambs the cost also inclucing all carriage tolls and plants used in making and fixing . (S.I.No. 28 P-93 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 44.00 |  | 240/50 | P.Rft | Rs.10582/- |
|  |  |  |  |  | Rupees Two Hundred Forty Ps. Fifty Only. |  |  |  |  |
| 06. | Applying floating coat of cement (S.N 14 P.No 22) |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 1432.00 |  | 660/- | %Sft | Rs.9451/- |
|  |  |  |  |  | (Rupees. Six Hundred Sixty Only). |  |  |  |  |
| 07. | Cement plaster 1:6 upto 20 height ½ thick. (S.I.No.13 P.No.58). |  |  |  |  |  |  |  |  |
| (a) | Ground Floor |  |  |  | 1432.00 |  | 2206/60 | %Sft | Rs.32260/- |
|  |  |  |  |  | Rupees Two Thousand Two Hundred Six Ps. Sixty Five Only. |  |  |  |  |
| 08. | Cement plaster 1:4 upto 20 height 3/8 thick. (S.I.No.11 P.No.58). |  |  |  |  |  |  |  |  |
| (a) | Ground Floor |  |  |  | 1432.00 |  | 2197/52 | %Sft | Rs.32128/- |
|  |  |  |  |  | Rupees Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only. |  |  |  |  |
| 09. | R.C.C works in roof slab, beams columns rafts lintels and other structural members laid in situ recast laid in position complete in all respects ratio 1:2:4 90lbs cement 2cft sand 4cft shingle 1/8 to ¼ gauge (S.I.No.06 P.No.19). |  |  |  |  |  |  |  |  |
| (a) | Ground Floor |  |  | 05 |  | 337/- |  | %Cft | Rs.1685/- |
|  |  |  |  | Rupees Three Hundred Thirty Seven Only. |  |  |  |  |  |

| SR# | ITEM OF WORKS |  |  |  | QTY: |  |  | RATE |  |  |  |  | UNIT |  | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10. | Fabrication of mild steel reinforcement for cement concrete including cutting bending, laying in position making joints and fastenings including cost of binding wire also including cost of binding wire also includes removal of rust from bars.(S.I.No.07 P.No.20). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 0.223 |  |  | 5001/70 |  |  |  |  |  | P.Cwt |  | Rs.1115/- |
|  |  |  |  | Rupees Five Thousand One Ps. Seventy Only. |  |  |  |  |  |  |  |  |  |  |  |
| 11. | Cement concrete plain 1:2:4 including placing compacting finishing and curing, complete (including screening and washing of stone aggregate without shuttering Ratio 1:2:4. (S.I.No.05 P.No.18). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 189.00 |  |  | 14429/- |  | %Cft |  |  |  |  | Rs.27272/- |
|  |  |  |  |  | Rupees Fourteen Thousand Four Hundred Twenty Nine Only. |  |  |  |  |  |  |  |  |  |  |
| 12. | Laying floor of approved colour glazed tiles 1/4" thick laid in white cement and pigement on a bned of 3/4" thick cement mortor 1:2 ( S.I.No. 25 P- 43 ) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (a) | Ground Floor |  |  |  |  | 1145.00 |  |  | 27747/06 |  | %Sft |  |  | Rs.317704/- |  |
|  |  |  |  |  |  | Rupees Twenty Seven Thousand Seven Hundred Forty Seven Ps. Zero Six Only. |  |  |  |  |  |  |  |  |  |
| 13. | White glazed tile 1/4" thick dado jointed in white cement and laid over 1:2 cement sand mortar 3/4" thick including finishing. (SINO.37 P-45) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 778.00 |  |  | 28253/61 |  | %Sft |  |  | Rs.219813/- |  |
|  |  |  |  |  |  | Rupees Twenty Eight Thousand Two Hundred Fifty Three Ps. Sixty One Only. |  |  |  |  |  |  |  |  |  |
| 14. | First class deodar wood wrought, joinery in doors & windows etc. fixed in position i/c chowkhats holds fasts hinges, iron tower blts. Chocks cleats, handles and cords with hooks, etc. Deodar paneled or paneled and glazed or fully glazed (without chowkhats). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60.00 |  |  | 902/93 |  | P.Sft |  |  | Rs.54176/- |  |
|  |  |  |  |  |  | Rupees Nine Hundred Two Ps. Ninety Three Only. |  |  |  |  |  |  |  |  |  |
| 15. | Cement Pointing i/c streaking of joints on wall ratio 1:3 (S.I.NO.16 P.No.21) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 252.00 |  |  | 1213/58 |  |  | %Sft |  | Rs.3070/- |  |
|  |  |  |  |  |  | Rupees One Thousand Two Hundred Thirteen Ps. Fifty Eight Only. |  |  |  |  |  |  |  |  |  |
| 16. | Colour wash two coats. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 253.00 |  |  | 859/90 |  |  | %Sft |  | Rs.2176/- |  |
|  |  |  |  |  |  | Rupees Eight Hundred Fifty Nine Ps. Ninety Only. |  |  |  |  |  |  |  |  |  |

| 17. | White washing Three coats. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1130.00 | 829/50 | %Sft | Rs.9378/- |
|  |  |  | Rupees Eighty Hundred Fifty Nine Ps. Ninety Only. |  |  |  |
| 18. | Primary coat of chalk under distemper. (S.I.No.23 P.No.59). |  |  |  |  |  |
|  |  |  | 1462.00 | 442/75 | %Sft | Rs.6473/- |
|  |  |  | Rupees Four Hundred Forty Two Ps. Seventy Five Only. |  |  |  |
| 19. | Distemper in two coats. (S.I.No.24 P.No.60). |  |  |  |  |  |
|  |  |  | 4113.00 | 1043/90 | %Sft | Rs.42935/- |
|  |  |  | Rupees One Thousand Fourty Three Ps. Ninety Only. |  |  |  |
| 20. | Painting New surface(a) Preparing surface painting corrugated surface, patent roofing etc. (SINO.5(a+b) P-69) for Two Coats |  |  |  |  |  |
|  |  |  | 433.00 | 657/91 | %Sft | Rs.2859/- |
|  |  |  | Rupees Six Hundred Fifty Seven Ps. Ninety One Only. |  |  |  |
|  |  |  | TOTAL |  |  | Rs.806061/- |

% Above / Below (Amount to be added / deducted).		Rs.

Grand Total		Rs.

Total Amount in word:

Note:-	 Quantities and rates are provisional and may be changed as per /according to the Technical Sanction by the competent authority.

| CONTRACTOR | EXECUTIVE ENGINEER (BUILDINGS) WORKS & SERVICES DEPARTMENT KHAIRPUR |
| --- | --- |

(SCHEDULE B)

REHIBLITATION/CONSTRUCTION OF REVENUE COMPLEX / HOUSES TALUKA KINGRI

DISTRICT KHAIRPUR.

PART  PAVED COURTYARD MUKHTIARKAR BUNGLOW:

| SR# | ITEM OF WORKS | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- |

| 01. | Filling watering and ramming earth under floor with new excavated from outside lead upto one chain and lift upto 5feet. (S.I.No.12 P.No.5). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 400.00 | 3630/- | %0Cft | Rs.1452/- |
|  |  |  | Rupees Three Thousand Six Hundred Thirty Only. |  |  |  |
| 02. | Extra lead for 3”mile. |  |  |  |  |  |
|  |  |  | 400.00 | 579/41 | %Cft | Rs.2318/- |
|  |  |  | Rupees Five Hundred Seventy Nine Ps. Forty One Only. |  |  |  |
| 03. | Cement concrete plain including placing compacting finishing and curing, complete (including screening and washing of stone aggregate without shuttering Ratio 1:2:4. (S.I.No.05 P.No.18). |  |  |  |  |  |
|  |  |  | 170.00 | 14429/- | %Cft | Rs.24529/- |
|  |  |  | Rupees Fourteen Thousand Four Hundred Twenty Nine Only. |  |  |  |

| 04. | Providing and laying 1”thick topping cement concrete 1:2:4 i/c surface finishing and dividing into panels. (S.I.No.16 (d) P.No.47). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 500.00 | 3275/50 | %Sft | Rs.16377/- |
|  |  |  | Rupees Three Thousand Two Hundred Seventy Five Ps. Fifty Only. |  |  |  |

|  |  |  | TOTAL | Rs.20147/- |
| --- | --- | --- | --- | --- |

% Above / Below (Amount to be added / deducted).		Rs.

Grand Total		Rs.

Total Amount in word:

Note:-	 Quantities and rates are provisional and may be changed as per /according to the Technical Sanction by the competent authority.

| CONTRACTOR | EXECUTIVE ENGINEER (BUILDINGS) WORKS & SERVICES DEPARTMENT KHAIRPUR |
| --- | --- |

(SCHEDULE B)

REHIBLITATION/CONSTRUCTION OF REVENUE COMPLEX / HOUSES TALUKA KINGRI

DISTRICT KHAIRPUR.

COURYARD WALL MUKHTIAR BUNGLOW:

| SR# | ITEM OF WORKS |  | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- |
| 01. | Dismantling brick work in lime or cement mortar.(S.I.No: 13 P-10) |  |  |  |  |  |
|  |  |  | 180.00 | 1285/63- | %Cft | Rs.2314/- |
|  |  |  | (Rs. One Thousand Two Hundred Eighty Five Ps.Sixty Three Only). |  |  |  |
| 02. | Damp proof course with cement sand and shingle concrete 1:2:4 including 2 coats of asphaltic mixture ( c ) 3" thick .(S.I.No.27 ( c ) P-19) |  |  |  |  |  |
|  |  |  | 169.00 | 4982/18 | %Cft | Rs.8420/ |
|  |  |  | (Rs. EIGHT Thousand Nine Hundred Eighty Two Ps Eighteen Only). |  |  |  |
| 03. | Pacca brick work other than building in Ratio 1:6. (S.I.No.04 P.No.24). |  |  |  |  |  |
|  | C/wall |  | 849.00 | 12346/65 | %Cft | Rs.10482/- |
|  |  |  | Rupees Twelve Thousand Three Hundred Sixty Six Ps. Sixty Five Only. |  |  |  |
| 04. | R.C.C works in roof slab, beams columns rafts lintels and other structural members laid in situ recast laid in position complete in all respects ratio 1:2:4 90lbs cement 2cft sand 4cft shingle 1/8 to ¼ gauge (S.I.No.06 P.No.19). |  |  |  |  |  |
| (a) | Ground Floor |  | 32.00 | 337/- | %Cft | Rs.10784/- |
|  |  |  | Rupees Three Hundred Thirty Seven Only. |  |  |  |
| 05. | Fabrication of mild steel reinforcement for cement concrete including cutting bending, laying in position making joints and fastenings including cost of binding wire also including cost of binding wire also includes removal of rust from bars.(S.I.No.07 P.No.20). |  |  |  |  |  |
|  |  |  | 1.428 | 5001/70 | P.Cwt | Rs.7142/- |
|  |  |  | Rupees Five Thousand One Ps. Seventy Only. |  |  |  |
| 06. | Cement plaster 1:6 upto 20 height 1/2”thick. (S.I.No. 13 P.No. 51 ). |  |  |  |  |  |
|  |  |  | 967.00 | 2260/06 | %Sft | Rs.21338/- |
|  |  |  | (Rs. Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only). |  |  |  |
| 07. | Cement plaster 1:4 upto 20 height 3/8”thick. (S.I.No.11 P.No.58). |  |  |  |  |  |
|  |  |  | 967.00 | 2197/52 | %Sft | Rs.21250/- |
|  |  |  | (Rs. Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only). |  |  |  |

| 08. | Cement Pointing i/c streaking of joints on wall ratio 1:3 (S.I.NO.16 P.No.21) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 900.00 | 1213/58 | %Sft | Rs.10922/- |
|  |  |  | Rupees One Thousand Two Hundred Thirteen Ps. Fifty Eight Only. |  |  |  |
| 09. | Colour wash two coats. |  |  |  |  |  |
|  |  |  | 1867.00 | 859/90 | %Sft | Rs.16054/- |
|  |  |  | Rupees Eight Hundred Fifty Nine Ps. Ninety Only. |  |  |  |

| SR# | ITEM OF WORKS |  |  | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 10. | M/F steel grated doors complete with locking arrangement angloe iron frame 2"x2"x3.8" and 3/4" sq. bars 4" centre to centre (SINO.23 P-92) |  |  |  |  |  |  |
|  |  |  | 48.00 |  | 726/72 | P.Sft | Rs.34882/- |
|  |  |  | Rupees Seven Hundred Twenty Six Ps. Seventy Five Only. |  |  |  |  |
| 11. | Painting New surface(a) Preparing surface painting corrugated surface, patent roofing etc. (SINO.5(a+b) P-69) for Two Coats |  |  |  |  |  |  |
|  |  |  | 96.00 |  | 1489/68 | %Sft | Rs.1430/- |
|  |  |  | Rupees One Thousand Eight Hundred Ps. Sixty Eight Only. |  |  |  |  |

|  |  |  | TOTAL | Rs.145018/- |
| --- | --- | --- | --- | --- |

% Above / Below (Amount to be added / deducted).		Rs.

Grand Total		Rs.

Total Amount in word:

Note:-	 Quantities and rates are provisional and may be changed as per /according to the Technical Sanction by the competent authority.

| CONTRACTOR | EXECUTIVE ENGINEER (BUILDINGS) WORKS & SERVICES DEPARTMENT KHAIRPUR |
| --- | --- |

(SCHEDULE B)

REHIBLITATION/CONSTRUCTION OF REVENUE COMPLEX / HOUSES TALUKA KINGRI

DISTRICT KHAIRPUR.

PART     MAIN BUILDING iiid TYPE BUNGLOW ASSISTANT COMMINSOR

| SR# | ITEM OF WORKS |  | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- |
| 01. | Removing cement or lime plaster from walls (S.I.No.53 P.No.13). |  |  |  |  |  |
|  |  |  | 1432.00 | 121/- | %Sft | Rs.1733/- |
|  |  |  | (Rupees . One Hundred Twenty One Only) . |  |  |  |
| 02. | Dismantling R.C.C 1:2:4 cement concrete reinforcement (S.I.No.20 P.No.10). |  |  |  |  |  |
|  |  |  | 777.00 | 5445/- | %0Cft | Rs.42308/- |
|  |  |  | (Rs. Five Thousand Four Hundred Forty Five Only). |  |  |  |

| 03. | Dismantling cement concrete plain 1:2:4.(S.I.NO:19 P-11). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 534.00 | 3327/60 | %Sft | Rs.17766/- |
|  |  |  | Rupees Three Thousand Three Hundred Twenty Seven Ps. Sixty Only. |  |  |  |

| . | Pacca brick work in Ground Floor and plinth in Ratio 1:6. (S.I.No.04 P.No.24). |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (a) | Ground Floor |  |  |  | 05.00 |  | 12674/36 | %Cft | Rs.634/- |
|  |  |  |  |  | Rupees Twelve Thousand Six Hundred Seventy Four Ps. Thirty Six Only. |  |  |  |  |
| 04. | Providing and fixing G.I. frames chowkats of size 7x2xor 4-1/2x3 for door using 20 gauge G.I. sheet i/c welded hinge and fixing at with cement sand slurry of ration 1.6 despairing the jambs. The cost also i/c all carriage tools and plants used in making and fixing.(S.I.NO.18 P-98). |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 54.00 |  | 228/90 | P.Rft | Rs.12361/- |
|  |  |  |  |  | Rupees Two Hundred Twenty Eight Ps. Ninety Only. |  |  |  |  |
| 05. | P/F G.I. Frame / Chowkats of size 7"x2" or 4-1/2"x3" for window using 20 gague G.I.sheet including welded hinges and fixing at site with necessary holds fasts filling with cement sand slurry or ratio 1:6 and repairing the jambs the cost also inclucing all carriage tolls and plants used in making and fixing . (S.I.No. 28 P-93 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 44.00 |  | 240/50 | P.Rft | Rs.10582/- |
|  |  |  |  |  | Rupees Two Hundred Forty Ps. Fifty Only. |  |  |  |  |
| 06. | Applying floating coat of cement (S.N 14 P.No 22) |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 1432.00 |  | 660/- | %Sft | Rs.9451/- |
|  |  |  |  |  | (Rupees. Six Hundred Sixty Only). |  |  |  |  |
| 07. | Cement plaster 1:6 upto 20 height ½ thick. (S.I.No.13 P.No.58). |  |  |  |  |  |  |  |  |
| (a) | Ground Floor |  |  |  | 1432.00 |  | 2206/60 | %Sft | Rs.32260/- |
|  |  |  |  |  | Rupees Two Thousand Two Hundred Six Ps. Sixty Five Only. |  |  |  |  |
| 08. | Cement plaster 1:4 upto 20 height 3/8 thick. (S.I.No.11 P.No.58). |  |  |  |  |  |  |  |  |
| (a) | Ground Floor |  |  |  | 1432.00 |  | 2197/52 | %Sft | Rs.32128/- |
|  |  |  |  |  | Rupees Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only. |  |  |  |  |
| 09. | R.C.C works in roof slab, beams columns rafts lintels and other structural members laid in situ recast laid in position complete in all respects ratio 1:2:4 90lbs cement 2cft sand 4cft shingle 1/8 to ¼ gauge (S.I.No.06 P.No.19). |  |  |  |  |  |  |  |  |
| (a) | Ground Floor |  |  | 773 |  | 337/- |  | %Cft | Rs.260501/- |
|  |  |  |  | Rupees Three Hundred Thirty Seven Only. |  |  |  |  |  |

| SR# | ITEM OF WORKS |  |  |  | QTY: |  |  | RATE |  |  |  | UNIT |  | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10. | Fabrication of mild steel reinforcement for cement concrete including cutting bending, laying in position making joints and fastenings including cost of binding wire also including cost of binding wire also includes removal of rust from bars.(S.I.No.07 P.No.20). |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 34.509 |  |  | 5001/70 |  |  |  |  | P.Cwt |  | Rs.172604/- |
|  |  |  |  | Rupees Five Thousand One Ps. Seventy Only. |  |  |  |  |  |  |  |  |  |  |
| 11. | Cement concrete plain 1:2:4 including placing compacting finishing and curing, complete (including screening and washing of stone aggregate without shuttering Ratio 1:2:4. (S.I.No.05 P.No.18). |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 189.00 |  |  | 14429/- |  |  |  | %Cft |  | Rs.27272/- |
|  |  |  |  |  | Rupees Fourteen Thousand Four Hundred Twenty Nine Only. |  |  |  |  |  |  |  |  |  |
| 12. | Laying floor of approved colour glazed tiles 1/4" thick laid in white cement and pigement on a bned of 3/4" thick cement mortor 1:2 ( S.I.No. 25 P- 43 ) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (a) | Ground Floor |  |  |  |  | 1145.00 |  |  | 27747/06 | %Sft |  |  | Rs.317704/- |  |
|  |  |  |  |  |  | Rupees Twenty Seven Thousand Seven Hundred Forty Seven Ps. Zero Six Only. |  |  |  |  |  |  |  |  |
| 13. | White glazed tile 1/4" thick dado jointed in white cement and laid over 1:2 cement sand mortar 3/4" thick including finishing. (SINO.37 P-45) |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 778.00 |  |  | 28253/61 | %Sft |  |  | Rs.219813/- |  |
|  |  |  |  |  |  | Rupees Twenty Eight Thousand Two Hundred Fifty Three Ps. Sixty One Only. |  |  |  |  |  |  |  |  |
| 14. | First class deodar wood wrought, joinery in doors & windows etc. fixed in position i/c chowkhats holds fasts hinges, iron tower blts. Chocks cleats, handles and cords with hooks, etc. Deodar paneled or paneled and glazed or fully glazed (without chowkhats). |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60.00 |  |  | 902/93 | P.Sft |  |  | Rs.54176/- |  |
|  |  |  |  |  |  | Rupees Nine Hundred Two Ps. Ninety Three Only. |  |  |  |  |  |  |  |  |
| 15. | Cement Pointing i/c streaking of joints on wall ratio 1:3 (S.I.NO.16 P.No.21) |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 252.00 |  |  | 1213/58 |  | %Sft |  | Rs.3070/- |  |
|  |  |  |  |  |  | Rupees One Thousand Two Hundred Thirteen Ps. Fifty Eight Only. |  |  |  |  |  |  |  |  |
| 16. | Colour wash two coats. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 253.00 |  |  | 859/90 |  | %Sft |  | Rs.2176/- |  |
|  |  |  |  |  |  | Rupees Eight Hundred Fifty Nine Ps. Ninety Only. |  |  |  |  |  |  |  |  |

| 17. | White washing Three coats. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1130.00 | 829/50 | %Sft | Rs.9378/- |
|  |  |  | Rupees Eighty Hundred Fifty Nine Ps. Ninety Only. |  |  |  |
| 18. | Primary coat of chalk under distemper. (S.I.No.23 P.No.59). |  |  |  |  |  |
|  |  |  | 1462.00 | 442/75 | %Sft | Rs.6473/- |
|  |  |  | Rupees Four Hundred Forty Two Ps. Seventy Five Only. |  |  |  |
| 19. | Distemper in two coats. (S.I.No.24 P.No.60). |  |  |  |  |  |
|  |  |  | 4113.00 | 1043/90 | %Sft | Rs.42935/- |
|  |  |  | Rupees One Thousand Fourty Three Ps. Ninety Only. |  |  |  |
| 20. | Painting New surface(a) Preparing surface painting corrugated surface, patent roofing etc. (SINO.5(a+b) P-69) for Two Coats |  |  |  |  |  |
|  |  |  | 433.00 | 657/91 | %Sft | Rs.2859/- |
|  |  |  | Rupees Six Hundred Fifty Seven Ps. Ninety One Only. |  |  |  |
|  |  |  | TOTAL |  |  | Rs.1278183/- |

% Above / Below (Amount to be added / deducted).		Rs.

Grand Total		Rs.

Total Amount in word:

Note:-	 Quantities and rates are provisional and may be changed as per /according to the Technical Sanction by the competent authority.

| CONTRACTOR | EXECUTIVE ENGINEER (BUILDINGS) WORKS & SERVICES DEPARTMENT KHAIRPUR |
| --- | --- |

(SCHEDULE B)

REHIBLITATION/CONSTRUCTION OF REVENUE COMPLEX / HOUSES TALUKA KINGRI

DISTRICT KHAIRPUR.

PART  PAVED COURTYARD ASSISTANT COMMISSIONER BUNGLO:

| SR# | ITEM OF WORKS | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- |

| 01. | Filling watering and ramming earth under floor with new excavated from outside lead upto one chain and lift upto 5feet. (S.I.No.12 P.No.5). |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 400.00 | 3630/- |  | %0Cft |  | Rs.1452/- |
|  |  |  | Rupees Three Thousand Six Hundred Thirty Only. |  |  |  |  |  |
| 02. | Extra lead for 3”mile. |  |  |  |  |  |  |  |
|  |  |  | 400.00 | 579/41 | %Cft |  | Rs.2318/- |  |
|  |  |  | Rupees Five Hundred Seventy Nine Ps. Forty One Only. |  |  |  |  |  |
| 03. | Cement concrete plain including placing compacting finishing and curing, complete (including screening and washing of stone aggregate without shuttering Ratio 1:2:4. (S.I.No.05 P.No.18). |  |  |  |  |  |  |  |
|  |  |  | 170.00 | 14429/- |  | %Cft | Rs.24529/- |  |
|  |  |  | Rupees Fourteen Thousand Four Hundred Twenty Nine Only. |  |  |  |  |  |

| 04. | Providing and laying 1”thick topping cement concrete 1:2:4 i/c surface finishing and dividing into panels. (S.I.No.16 (d) P.No.47). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 500.00 | 3275/50 | %Sft | Rs.16377/- |
|  |  |  | Rupees Three Thousand Two Hundred Seventy Five Ps. Fifty Only. |  |  |  |

|  |  |  | TOTAL | Rs.20147/- |
| --- | --- | --- | --- | --- |

% Above / Below (Amount to be added / deducted).		Rs.

Grand Total		Rs.

Total Amount in word:

Note:-	 Quantities and rates are provisional and may be changed as per /according to the Technical Sanction by the competent authority.

| CONTRACTOR | EXECUTIVE ENGINEER (BUILDINGS) WORKS & SERVICES DEPARTMENT KHAIRPUR |
| --- | --- |

(SCHEDULE B)

REHIBLITATION/CONSTRUCTION OF REVENUE COMPLEX / HOUSES TALUKA KINGRI

DISTRICT KHAIRPUR.

COURYARD WALL ASSISTANT COMM: BUNGL:

| SR# | ITEM OF WORKS |  | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- |
| 01. | Dismantling brick work in lime or cement mortar.(S.I.No: 13 P-10) |  |  |  |  |  |
|  |  |  | 180.00 | 1285/63- | %Cft | Rs.2314/- |
|  |  |  | (Rs. One Thousand Two Hundred Eighty Five Ps.Sixty Three Only). |  |  |  |
| 02. | Damp proof course with cement sand and shingle concrete 1:2:4 including 2 coats of asphaltic mixture ( c ) 3" thick .(S.I.No.27 ( c ) P-19) |  |  |  |  |  |
|  |  |  | 169.00 | 4982/18 | %Cft | Rs.8420/ |
|  |  |  | (Rs. EIGHT Thousand Nine Hundred Eighty Two Ps Eighteen Only). |  |  |  |
| 03. | Pacca brick work other than building in Ratio 1:6. (S.I.No.04 P.No.24). |  |  |  |  |  |
|  | C/wall |  | 849.00 | 12346/65 | %Cft | Rs.10482/- |
|  |  |  | Rupees Twelve Thousand Three Hundred Sixty Six Ps. Sixty Five Only. |  |  |  |
| 04. | R.C.C works in roof slab, beams columns rafts lintels and other structural members laid in situ recast laid in position complete in all respects ratio 1:2:4 90lbs cement 2cft sand 4cft shingle 1/8 to ¼ gauge (S.I.No.06 P.No.19). |  |  |  |  |  |
| (a) | Ground Floor |  | 32.00 | 337/- | %Cft | Rs.10784/- |
|  |  |  | Rupees Three Hundred Thirty Seven Only. |  |  |  |
| 05. | Fabrication of mild steel reinforcement for cement concrete including cutting bending, laying in position making joints and fastenings including cost of binding wire also including cost of binding wire also includes removal of rust from bars.(S.I.No.07 P.No.20). |  |  |  |  |  |
|  |  |  | 1.428 | 5001/70 | P.Cwt | Rs.7142/- |
|  |  |  | Rupees Five Thousand One Ps. Seventy Only. |  |  |  |
| 06. | Cement plaster 1:6 upto 20 height 1/2”thick. (S.I.No. 13 P.No. 51 ). |  |  |  |  |  |
|  |  |  | 967.00 | 2260/06 | %Sft | Rs.21338/- |
|  |  |  | (Rs. Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only). |  |  |  |
| 07. | Cement plaster 1:4 upto 20 height 3/8”thick. (S.I.No.11 P.No.58). |  |  |  |  |  |
|  |  |  | 967.00 | 2197/52 | %Sft | Rs.21250/- |
|  |  |  | (Rs. Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only). |  |  |  |

| 08. | Cement Pointing i/c streaking of joints on wall ratio 1:3 (S.I.NO.16 P.No.21) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 900.00 | 1213/58 | %Sft | Rs.10922/- |
|  |  |  | Rupees One Thousand Two Hundred Thirteen Ps. Fifty Eight Only. |  |  |  |
| 09. | Colour wash two coats. |  |  |  |  |  |
|  |  |  | 1867.00 | 859/90 | %Sft | Rs.16054/- |
|  |  |  | Rupees Eight Hundred Fifty Nine Ps. Ninety Only. |  |  |  |

| SR# | ITEM OF WORKS |  |  | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 10. | M/F steel grated doors complete with locking arrangement angloe iron frame 2"x2"x3.8" and 3/4" sq. bars 4" centre to centre (SINO.23 P-92) |  |  |  |  |  |  |
|  |  |  | 48.00 |  | 726/72 | P.Sft | Rs.34882/- |
|  |  |  | Rupees Seven Hundred Twenty Six Ps. Seventy Five Only. |  |  |  |  |
| 11. | Painting New surface(a) Preparing surface painting corrugated surface, patent roofing etc. (SINO.5(a+b) P-69) for Two Coats |  |  |  |  |  |  |
|  |  |  | 96.00 |  | 1489/68 | %Sft | Rs.1430/- |
|  |  |  | Rupees One Thousand Eight Hundred Ps. Sixty Eight Only. |  |  |  |  |

|  |  |  | TOTAL | Rs.145018/- |
| --- | --- | --- | --- | --- |

% Above / Below (Amount to be added / deducted).		Rs.

Grand Total		Rs.

Total Amount in word:

Note:-	 Quantities and rates are provisional and may be changed as per /according to the Technical Sanction by the competent authority.

| CONTRACTOR | EXECUTIVE ENGINEER (BUILDINGS) WORKS & SERVICES DEPARTMENT KHAIRPUR |
| --- | --- |

(SCHEDULE B)

REHIBLITATION/CONSTRUCTION OF REVENUE COMPLEX / HOUSES TALUKA KINGRI

DISTRICT KHAIRPUR.

PART-		WATER SUPPLY & SANITARY FITTINGS

| SR# | ITEM OF WORKS | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 01. | Providing and fixing seething type white glazed earthen ware w.c. pan with front inlet and complete with including the cost of flushing cistern with internal fitting anode flush pipe with bend and making requisite number of holes in walls plinth and floor for pipe connection and making good in cement concrete 1:2:4(foreign exultant)(S.I.NO.2 P-1). | 14 Nos. | 4846/60 | Each | Rs.67852/- |
| 02. | Providing and fixing 24x18 lavatory basin in white glazed earthen ware complete with and i/c the cost of W.I.or C.I. cantilever brackets 6 inches built into painted white in two coats after a primary coat of red lead paint a pair of ½ dial and chrome plaster traps1-1/2 dial rubber plug and chrome plated brass chain 1-1/4 dial malleable iron or C.P. brass traps malleable iron or brass union and making requisite number of holes in wall plinth and floor for pipe connection and making good in cement concrete 1.2.4 (stand pattern.(S.I.NO.8 P-3). | 8 Nos. | 4928/- | Each | Rs.39424/- |
| 03. | Providing G.I. pipe specials clamps etc including fixing cutting and fitting complete with and i/c the cost of breaking through walls and roof making good etc painting two coats after cleaning the pipe etc with white Zink paint with pigment to match the color of the buildings .1/2 dial G.I. pipe.(S.I.NO.1 P-11). |  |  |  |  |
|  | 1 ¼”Dia. | 100 Rft | 188/97 | P.Rft | Rs.18897/- |
|  | 1”Dia. | 100 Rft | 128/55 | P.Rft | Rs.12855/- |
|  | ¾”Dia. | 130 Rft | 95/79 | P.Rft | Rs.12453/- |
|  | ½”Dia. | 120 Rft | 73/21 | P.Rft | Rs.8785/- |
| 04. | Add extra cancelled G.I pipe and filling etc complete (S.I.No.2 P.No.12). | 250 Rft | 7/82 | P.Rft | Rs.1955/- |
| 05. | P/F 6”x2”x or 6”x3” C.I. Floor trap of the approved self-cleaning design with a vent C.I. Skewed down gritting with or without a vent arm complete with and i/c the making requisite number of holes in walls plinth and floor for pipe connections and making good in c.,c.1.2.4. (S.I.NO.20 P-5). | 16 Nos. | 2024/43 | Each | Rs.32391/- |
| 06. | Supplying and fixing cancelled tee stop of superior quality with Crystal head ½” (S.I.No.12 P.No.18). | 16 Nos. | 889/46 | Each | Rs.14231/- |
| 07. | P/F chrome plated brass towel sail complete with brick nuts fixing on wooden cleats with 1 long C.P brass screws ¾”Dia bound C-square (Standard pattern) (S.I.No.1 (a) P.No.7). | 8 No. | 1269/95 | Each | Rs.10160/- |

| SR# | ITEM OF WORKS | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 08. | P/F 15”x12” beveled edge mirror of Belgium glass complete with 1/8”thick hard board and C.P screws fixed to wooden plate (Standard Pattern) (S.I.No.4 P.No.7). | 8 Nos. | 1161/60 | Each | Rs.9293/- |
| 09. | P/F in position nylon connections complete with ½”dia brass stop cock with pair of brass nuts and lining joints to nylon connection (S.I.No.23 P.No.6). | 16 Nos. | 447/15 | Each | Rs.7154/- |
| 10. | S/F Bib cock of superior quality with C.P head ½”dia. (S.I.No.13 P.No.15). | 10 Nos. | 1109/46 | Each | Rs.11095/- |
| 11. | S/F canceled stop cock of superior quality head ½”dia. (S.I.NO.11 (b) P-15). | 8 Nos. | 509/74 | Each | Rs.4077/- |
| 12. | S/F handle valve china ¾”dia (S.I.No.7 P.No.14). | 4 Nos. | 271/92 | Each | Rs.1088/- |
| 13. | S/F fiber glass tank of approved quality and design and wall thickness as specified i/c the cost of nuts bolts and fixing in platform of cement concrete 1.3.6.and making connection for inlet outlet and over flow inlet outlet flow pipe etc complete 250 gallon wall thickness 3.5mm. (S.I.No.3 (a) P.No.21). | 02 Nos. | 21989/61 | Each | Rs.43979/- |
| 14. | Providing RCC pipe with collars “Band digging the troches to required depth and fixing in position including cutting fitting and jointing with asphalt composition etc cement mortar 1:1 and testing with water pressure head of 4ft above the top of the height pipes and refilling with excavated staff (S.I.No.2 (c) P.No.23). | 300 Rft. | 199/25 | P.Rft | Rs.69775/- |
| 15. | Providing asbestos pipe with collar deed or equitant i/c digging the trenches to required depth and fixing to position and jointing with rubber rings including testing to water pressure head of 200feet (S.I.No.3 (a) P.No.71). | 120 Rft | 141/11 | P.Rft | Rs.16933/- |
| 16. | Providing and fixing 6x4 C.C Galley trap with 4 outlets complete with 4”thick 1:2:4 C.C for head and ½”thick cement plaster 1:3 to the karb grating 6x6 Cole cover and fasts 12x12 inside etc complete with C.I cover and frames (S.I.No.1 (c) P.No.23). | 14 Nos. | 1193/17 | Each | Rs.16704/- |
|  |  |  | TOTAL |  | Rs.389101/- |

% Above/Below (Amount to be added/deducted).Rs.

Grand Total	   Rs.

Total Amount in word:

Note:-	Quantities and rates may be changed as per/according to the Technical Sanction by the competent authority.

| CONTRACTOR | EXECUTIVE ENGINEER (BUILDINGS) WORKS & SERVICES DEPARTMENT KHAIRPUR |
| --- | --- |

(SCHEDULE B)

REHIBLITATION/CONSTRUCTION OF REVENUE COMPLEX / HOUSES TALUKA KINGRI

DISTRICT KHAIRPUR.

PART-D-II	P.H SCHEDULE ITEM

| SR# | ITEM OF WORKS | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 01. | Boring tube well in all water bearing foil from ground level up to 100 ft or 30.5 meter west i/c thinking & drawing the casing pipe Hospital & Quarter 4”Dia. | 15 Rft | 242/- | P.Rft | Rs.3630/- |
| 02. | Constructing man hole or inspection chamber for the required dial of circular screw and 3-6(1067mm) depth with walls of Bobbin cement motor 1..32 cement plastered 1.3 ½ thick inside of walls and 1`(25mm) thick over benching and channel i/c fixing Caiman hole cover with frames of clear opening 1-1/2x1:1-1/2 (457x457mm) of 1.75CWT (88.9kg) embedded in plain c.c.1.2.4 and fixing 1(25mm)dial M.S. Steps 6 (150mm) wide projecting 4 (102mm)from the face of wall at 12 (305mm) c/c duly painted etc complete as per specification and drawing NO;DP [/1of public heath circle southern 4 to 12 dial 2x2x3x6. | 02 Nos. | 14748/- | Each | Rs.29496/- |
| 03. | Providing and fixing PVC pipe class “D” class etc completed. | 100 Rft | 34/- | P.Rft | Rs.3400/- |
|  |  |  | TOTAL |  | Rs.36526/- |

% Above/Below (Amount to be added/deducted).Rs.

Grand Total	   Rs.

Total Amount in word:

Note:-	Quantities and rates may be changed as per/according to the Technical Sanction by the competent authority.

| CONTRACTOR | EXECUTIVE ENGINEER (BUILDINGS) WORKS & SERVICES DEPARTMENT KHAIRPUR |
| --- | --- |

(SCHEDULE B)

REHIBLITATION/CONSTRUCTION OF REVENUE COMPLEX / HOUSES TALUKA KINGRI

DISTRICT KHAIRPUR.

PART-D-III	E.I  ITEM

| SR# | ITEM OF WORKS | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 01. | Supplying and Fixing Water pumping Set ½” H.P Complete | 02 Nos | 8500/- | Each | Rs.17000/- |
|  |  |  | TOTAL |  | Rs.17000/- |

% Above/Below (Amount to be added/deducted).Rs.

Grand Total	   Rs.

Total Amount in word:

Note:-	Quantities and rates may be changed as per/according to the Technical Sanction by the competent authority.

| CONTRACTOR | EXECUTIVE ENGINEER (BUILDINGS) WORKS & SERVICES DEPARTMENT KHAIRPUR |
| --- | --- |

(SCHEDULE B)

REHIBLITATION OF GOVT: DISPENSARY RAFIQUE  MAHEASAR  TAL: KINGRI  DISTT: KHAIRPUR.

PART  MAIN BUILDING:

| SR# | ITEM OF WORKS |  |  |  | QTY: |  | RATE |  | UNIT |  |  | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01. | Removing cement or lime plaster from walls (S.I.No.53 P.No.13). |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 1433.00 |  | 121/- |  | %Sft |  |  | Rs.1734/- |
|  |  |  |  |  | (Rupees . One Hundred Twenty One Only) . |  |  |  |  |  |  |  |
| 02. | Applying floating coat of cement (S.N 14 P.No 22) |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1433.00 |  |  | 660/- |  | %Sft |  |  | Rs.9458/- |
|  |  |  |  | (Rupees. Six Hundred Sixty Only). |  |  |  |  |  |  |  |  |
| 03. | Cement plaster 1:6 upto 20 height ½ thick. (S.I.No.13 P.No.58). |  |  |  |  |  |  |  |  |  |  |  |
| (a) | Ground Floor |  |  | 1433.00 |  |  | 2206/60 |  | %Sft |  |  | Rs.31621/- |
|  |  |  |  | Rupees Two Thousand Two Hundred Six Ps. Sixty Five Only. |  |  |  |  |  |  |  |  |
| 04. | Cement plaster 1:4 upto 20 height 3/8 thick. (S.I.No.11 P.No.58). |  |  |  |  |  |  |  |  |  |  |  |
| (a) | Ground Floor |  |  | 1433.00 |  |  | 2197/52 |  | %Sft |  |  | Rs.31490/- |
|  |  |  |  | Rupees Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only. |  |  |  |  |  |  |  |  |
| 05. | Laying floor of approved colour glazed tiles 1/4" thick laid in white cement and pigement on a bned of 3/4" thick cement mortor 1:2 ( S.I.No. 25 P- 43 ) |  |  |  |  |  |  |  |  |  |  |  |
| (a) | Ground Floor |  |  | 200.00 |  |  | 27747/06 |  |  | %Sft |  | Rs.55494/- |
|  |  |  |  | Rupees Twenty Seven Thousand Seven Hundred Forty Seven Ps. Zero Six Only. |  |  |  |  |  |  |  |  |
| 06. | White glazed tile ¼ thick dado in white cement and laid over cement sand mortar ¾ thick including finishing. (S.I.NO. P-50). |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 120.00 |  | 28253/61 |  | %Sft |  | Rs.33959/- |
|  |  |  |  |  |  | Rupees Twenty Eight Thousand Two Hundred Fifty Three Ps. Sixty One Only. |  |  |  |  |  |  |
| 07. | Cement Pointing i/c streaking of joints on wall ratio 1:3 (S.I.NO.16 P.No.21) |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 180.00 |  |  | 1213/58 |  |  |  | %Sft | Rs.2184/- |
|  |  |  |  | Rupees One Thousand Two Hundred Thirteen Ps. Fifty Eight Only. |  |  |  |  |  |  |  |  |

| 08. | Primary coat of chalk under distemper. (S.I.No.23 P.No.59). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1433.00 | 442/75 | %Sft | Rs.6345/- |
|  |  |  | Rupees Four Hundred Forty Two Ps. Seventy Five Only. |  |  |  |
| 09. | Distemper in two coats. (S.I.No.24 P.No.60). |  |  |  |  |  |
|  |  |  | 8598.00 | 1043/90 | %Sft | Rs.89754/- |
|  |  |  | Rupees One Thousand Fourty Three Ps. Ninety Only. |  |  |  |
|  |  |  | TOTAL |  |  | Rs.262039/- |

% Above / Below (Amount to be added / deducted).		Rs.

Grand Total		Rs.

Total Amount in word:

Note:-	 Quantities and rates are provisional and may be changed as per /according to the Technical Sanction by the competent authority.

| CONTRACTOR | EXECUTIVE ENGINEER (BUILDINGS) WORKS & SERVICES DEPARTMENT KHAIRPUR |
| --- | --- |

(SCHEDULE B)

REHIBLITATION OF GOVT: DISPENSARY RAFIQUE  MAHEASAR

TALUKA KINGRI    DISTRICT KHAIRPUR.

PART  PAVED COURTYARD

| SR# | ITEM OF WORKS | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- |

| 01. |  | Filling watering and ramming earth under floor with new excavated from outside lead upto one chain and lift upto 5feet. (S.I.No.12 P.No.5). |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 360.00 | 3630/- | %0Cft |  | Rs.1306/- |
|  |  |  |  |  | Rupees Three Thousand Six Hundred Thirty Only. |  |  |  |  |
| 02. |  | Extra lead for 3”mile. |  |  |  |  |  |  |  |
|  |  |  |  |  | 2068.00 | 579/41 | %Cft |  | Rs.2068/- |
|  |  |  |  |  | Rupees Five Hundred Seventy Nine Ps. Forty One Only. |  |  |  |  |
| 03. |  | Dressing and leveling of earth work to designed Section etc completed in ordinary soil. |  |  |  |  |  |  |  |
|  |  |  |  |  | 360.00 | 187/55- | %0Cft |  | Rs.68/- |
|  |  |  |  |  | Rupees One Hundred Eighty Seven Ps. Fifty Five Only. |  |  |  |  |
| 04. | Cement concrete brick or stone ballast 1-1/2 to 2 guage ratio 1:6:12.(S.I.NO.4 P-14) |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 99.00 | 8122/95 | %Cft |  | Rs.8042/- |
|  |  |  |  |  | Rupees.Eight thousand One Hundred Twenty Two Ps.Ninety Five |  |  |  |  |
| 05. | Cement concrete plain 1:2:4 including placing compacting finishing and curing, complete (including screening and washing of stone aggregate without shuttering Ratio 1:2:4. (S.I.No.05 P.No.18). |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 300.00 | 14429/- | %Cft |  | Rs.9827/- |
|  |  |  |  |  | Rupees Fourteen Thousand Four Hundred Twenty Nine Only. |  |  |  |  |
|  |  |  |  |  | TOTAL |  |  | Rs.21311/- |  |

% Above / Below (Amount to be added / deducted).		Rs.

Grand Total		Rs.

Total Amount in word:

Note:-	 Quantities and rates are provisional and may be changed as per /according to the Technical Sanction by the competent authority.

| CONTRACTOR | EXECUTIVE ENGINEER (BUILDINGS) WORKS & SERVICES DEPARTMENT KHAIRPUR |
| --- | --- |

(SCHEDULE B)

CONSTRUCTION OF HUJRA BIBI PAK DAMAN AND OPEN YARD U/C DRIB MEHAR SHAH TALUKA KINGRI DISTRICT KHAIRPUR

PART A MAIN BUILDING

| SR# |  | ITEM OF WORKS |  |  |  | QTY: |  | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01. |  | Excavation in foundation of buildings bridges and other structures including dag belling dressing refilling around structure with excavated earth watering and ramming lead upto 5ft. (b) in ordinary soil. (S.I.No.18 P.No.5). |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 1320.00 |  | 3176/25 | %0Cft | Rs.4193/- |
|  |  |  |  |  |  | (Rs. Three Thousand One Hundred Seventy Six Ps. Twenty Five Only). |  |  |  |  |
| 02. | Cement concrete brick or stone ballast 1-1/2 to 2 gauge. (A) ratio 1:4:8. (S.I.No. P.No. ). |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 384.00 |  | 9416/95 |  | %Cft | Rs.36159/- |
|  |  |  |  |  | (Rs. Nine Thousand Four Hundred Sixteen Ps. Ninety Five Only). |  |  |  |  |  |
| 03. |  | Pacca Brick work in foundation & plinth in cement & sand mortar Ratio 1:6 (S.I.No.4 (c) P.No.25). |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 715.00 |  | 11948/36 | %Cft | Rs.85431/- |
|  |  |  |  |  |  | Rupees Eleven Thousand Nine Hundred Forty Eight Ps. Thirty Six Only. |  |  |  |  |
| 04. |  | R.C.C works in roof slab, beams columns rafts lintels and other structural members laid in situ recast laid in position complete in all respects ratio 1:2:4 90lbs cement 2cft sand 4cft shingle 1/8 to ¼ gauge (S.I.No.06 P.No.19). |  |  |  |  |  |  |  |  |
| (a) |  | Ground Floor |  |  |  | 566.00 |  | 337/- | %Cft | Rs.527742/- |
|  |  |  |  |  |  | Rupees Three Hundred Thirty Seven Only. |  |  |  |  |
| 05. |  | Fabrication of mild steel reinforcement for cement concrete including cutting bending, laying in position making joints and fastenings including cost of binding wire also including cost of binding wire also includes removal of rust from bars.(S.I.No.07 P.No.20). |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 79.910 |  | 5001/70 | P.Cwt | Rs.349669/- |
|  |  |  |  |  |  | Rupees Five Thousand One Ps. Seventy Only. |  |  |  |  |
| 06. |  | Pacca brick work in Ground Floor and plinth in Ratio 1:6. (S.I.No.04 P.No.24). |  |  |  |  |  |  |  |  |
| (a) |  | Ground Floor |  |  |  | 1242.00 |  | 12674/36 | %Cft | Rs.157416/- |
|  |  |  |  |  |  | Rupees Twelve Thousand Six Hundred Seventy Four Ps. Thirty Six Only. |  |  |  |  |

| 07. | Filling watering and ramming earth in floor with surplus earth from foundation lead upto one chain and lift upto 5ft.(S.I.NO:11 P-5) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 880.00 | 1512/50 | %0Cft | Rs.1331/- |
|  |  |  | Rupees One Thousand Five Hundred Twelve Ps. Fifty Only. |  |  |  |
| 08. | Filling watering and ramming earth under floor with new excavated from outside lead upto one chain and lift upto 5feet. (S.I.No.12 P.No.5). |  |  |  |  |  |
|  |  |  | 2803.00 | 3630/- | %0Cft | Rs.10175/- |
|  |  |  | Rupees Three Thousand Six Hundred Thirty Only. |  |  |  |

| SR# | ITEM OF WORKS |  | QTY: | RATE |  | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 09. | Extra lead for 3”mile. |  |  |  |  |  |  |
|  |  |  | 2803.00 | 579/41 | %Cft |  | Rs.16241/- |
|  |  |  | Rupees Five Hundred Seventy Nine Ps. Forty One Only. |  |  |  |  |
| 10. | Supplying and filling sand under floor and plugging in walls.(S.I.No.29 P.No.31). |  |  |  |  |  |  |
|  |  |  | 561.00 | 1141/25 |  | %Cft | Rs.6402/- |
|  |  |  | Rupees One Thousand One Hundred Forty One Ps. Seventy Five Only. |  |  |  |  |
| 11. | Cement concrete brick or stone ballast 1-1/2 to 2 guage ratio 1:6:12.(S.I.NO.4 P-14) |  |  |  |  |  |  |
|  |  |  | 370.00 | 8122/95 | %Cft |  | Rs.30055/- |
|  |  |  | Rupees.Eight thousand One Hundred Twenty Two Ps.Ninety Five |  |  |  |  |
| 12. | Cement concrete plain including placing compacting finishing and curing, complete (including screening and washing of stone aggregate without shuttering Ratio 1:2:4. (S.I.No.05 P.No.18). |  |  |  |  |  |  |
|  |  |  | 196.00 | 14429/- |  | %Cft | Rs.28281/- |
|  |  |  | Rupees Fourteen Thousand Four Hundred Twenty Nine Only. |  |  |  |  |
| 13. | Providing and fixing G.I. frames chowkats of size 7x2xor 4-1/2x3 for door using 20 gauge G.I. sheet i/c welded hinge and fixing at with cement sand slurry of ration 1.6 despairing the jambs. The cost also i/c all carriage tools and plants used in making and fixing.(S.I.NO.18 P-98). |  |  |  |  |  |  |
|  |  |  | 28.00 | 228/90 | P.Rft |  | Rs.6409/- |
|  |  |  | Rupees Two Hundred Twenty Eight Ps. Ninety Only. |  |  |  |  |
| 14. | Providing and fixing G.I. Frames chowkats of size 7x2xir 4-1/2x3 for windows using 20guage G.I. sheet i/c welded hinges and fixing at site with necessary hold fasts, filling with cement sand slurry of ration 1;6 and repainting the jamb. The cost also i/c all carriage tools and plants used in making and fixing. (S.I.No.29 P.No.98). |  |  |  |  |  |  |
|  |  |  | 118.00 | 240/50 | P.Rft |  | Rs.28379/- |
|  |  |  | Rupees Two Hundred Forty Ps. Fifty Only. |  |  |  |  |
| 15. | Cement plaster 1:6 upto 20 height 1/2”thick. (S.I.No. 13 P.No. 51 ). |  |  |  |  |  |  |
|  |  |  | 1480.00 | 2260/06 |  | %Sft | Rs.32678/- |
|  |  |  | (Rs. Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only). |  |  |  |  |
| 16. | Cement plaster 1:4 upto 20 height 3/8”thick. (S.I.No.11 P.No.58). |  |  |  |  |  |  |
|  |  |  | 1480.00 | 2197/52 |  | %Sft | Rs.32523/- |
|  |  |  | (Rs. Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only). |  |  |  |  |
| 17. | Laying white marble flooring fine dressed in the surface without winging set in lime mortar 1:2 including rubbing and polishing of the joints. (a) ¾”thick flooring. (S.I.No.28 P.No.49). |  |  |  |  |  |  |

|  |  |  | 1155.00 | 567/48 | %Sft | Rs.655439/- |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Rupees Five Hundred Sixty Seven Ps. Forty Eighty Only. |  |  |  |

| SR# | ITEM OF WORKS |  | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- |
| 18. | P/F 3/8 thick marble tiles of approved quality and colour shade size 8x4/6x4 in dado skirting and facing removal/tucking of existing plaster surface etc over 1/2thick base of cement mortar 1:3 setting mortar base including filling the joins and washing the tiles with white cement slurry. Current, finishing cleaning and polishing etc complete for new works.(S.I.NO.68 P-55). |  |  |  |  |  |
|  |  |  | 448.00 | 186/04 | %Sft | Rs.83346/- |
|  |  |  | Rupees One Hundred Eighty Six Ps. Four Only. |  |  |  |

| 19. | First class deodar wood wrought, joinery in doors & windows etc. fixed in position i/c chowkhats holds fasts hinges, iron tower blts. Chocks cleats, handles and cords with hooks, etc. Deodar paneled or paneled and glazed or fully glazed (without chowkhats). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 130.00 | 902/93 | P.Sft |  | Rs.117381/- |
|  |  |  | Rupees Nine Hundred Two Ps. Ninety Three Only. |  |  |  |  |
| 20. | Providing and fixing iron steel grill using solid square bars of size ½”x1/2” placed at 4” i/c and frame of flat iron patti of ¾”x3/4” i/c circle shape at 1-0 a part equivalent fitted with screws are pins i/c painting 3 coats with 1st coat of red oxide paint etc. (S.I.No.26 P.No.97). |  |  |  |  |  |  |
|  |  |  | 88.00 | 180/50 |  | P.Sft | Rs.15884/- |
|  |  |  | Rupees One Hundred Eighty Ps. Fifty Only. |  |  |  |  |
| 21. | Providing and laying 1”thick topping cement concrete 1:2:4 i/c surface finishing and dividing into panels. (S.I.No.16 (d) P.No.47). |  |  |  |  |  |  |
|  |  |  | 1559.00 | 3275/50 |  | %Sft | Rs.51065/- |
|  |  |  | Rupees Three Thousand Two Hundred Seventy Five Ps. Fifty Only. |  |  |  |  |
| 22. | Cement Pointing i/c streaking of joints on wall ratio 1:3 (S.I.NO.16 P.No.21) |  |  |  |  |  |  |
|  |  |  | 1276.00 | 1213/58 |  | %Sft | Rs.28156/- |
|  |  |  | Rupees One Thousand Two Hundred Thirteen Ps. Fifty Eight Only. |  |  |  |  |
| 23. | Colour wash two coats. |  |  |  |  |  |  |
|  |  |  | 1276.00 | 859/90 |  | %Sft | Rs.10972/- |
|  |  |  | Rupees Eighty Hundred Fifty Nine Ps. Ninety Only. |  |  |  |  |
| 24. | White washing Three coats. |  |  |  |  |  |  |
|  |  |  | 1155.00 | 829/50 |  | %Sft | Rs.9586/- |
|  |  |  | Rupees Eighty Hundred Fifty Nine Ps. Ninety Only. |  |  |  |  |

| 25. | Primary coat of chalk under distemper. (S.I.No.23 P.No.59). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1480.00 | 442/72 | %Sft | Rs.6553/- |
|  |  |  | Rupees Four Hundred Forty Two Ps. Seventy Two Only. |  |  |  |
| 26. | Distemper in Three coats. (S.I.No.24 P.No.59). |  |  |  |  |  |
|  |  |  | 1480.00 | 1079/65 | %Sft | Rs.15979/- |
|  |  |  | Rupees One Thousand Seventy Nine Ps. Sixty Five Only. |  |  |  |
| 27. | Painting New surface(a) Preparing surface painting corrugated surface, patent roofing etc. (SINO.5(a+b) P-69) for Two Coats |  |  |  |  |  |
|  |  |  | 260.00 | 1489/68 | P.Sft | Rs.3873/- |
|  |  |  | Rupees One Thousand Four Hundred Eighty Nine Ps. Sixty Eight Only. |  |  |  |
| 28. | P/L tiles HALA or pattern tiles glazed 6"x6"x1/2" on floor or wall facing in required floor and pattern of STILE specification jointed in white cement and pigment over a base of 1:2 (SINO.61 P-48) |  |  |  |  |  |
|  |  |  | 145.00 | 47651/56 | %Sft | Rs.69095/- |
|  |  |  | Rupees Forty Seven Thousand Six Hundred Fifty One Ps. Fifty Six Only. |  |  |  |

|  |  |  | TOTAL | Rs.2420413/- |
| --- | --- | --- | --- | --- |

% Above / Below (Amount to be added / deducted).		Rs.

Grand Total		Rs.

Total Amount in word:

Note:-	 Quantities and rates are provisional and may be changed as per /according to the Technical Sanction by the competent authority.

| CONTRACTOR | EXECUTIVE ENGINEER (BUILDINGS) WORKS & SERVICES DEPARTMENT KHAIRPUR |
| --- | --- |

(SCHEDULE B)

CONSTRUCTION OF HUJRA BIBI PAK DAMAN AND OPEN YARD U/C DRIB MEHAR SHAH TALUKA KINGRI DISTRICT KHAIRPUR

PART B WAZOO KHANA

| SR# |  | ITEM OF WORKS |  |  |  | QTY: | RATE |  | UNIT | AMOUNT |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01. |  | Excavation in foundation of buildings bridges and other structures including dag belling dressing refilling around structure with excavated earth watering and ramming lead upto 5ft. (b) in ordinary soil. (S.I.No.18 P.No.5). |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 266.00 | 3176/25 |  | %0Cft | Rs.845/- |  |
|  |  |  |  |  |  | (Rs. Three Thousand One Hundred Seventy Six Ps. Twenty Five Only). |  |  |  |  |  |
| 02. | Cement concrete brick or stone ballast 1-1/2 to 2 gauge. (A) ratio 1:4:8. (S.I.No. P.No. ). |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 67.00 |  |  | 9416/95 | %Cft | Rs.2826/- |  |
|  |  |  |  |  | (Rs. Nine Thousand Four Hundred Sixteen Ps. Ninety Five Only). |  |  |  |  |  |  |
| 03. |  | Pacca Brick work in foundation & plinth in cement & sand mortar Ratio 1:6 (S.I.No.4 (c) P.No.25). |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 242.00 | 11948/36 |  | %Cft | Rs.28915/- |  |
|  |  |  |  |  |  | Rupees Eleven Thousand Nine Hundred Forty Eight Ps. Thirty Six Only. |  |  |  |  |  |
| 04. |  | R.C.C works in roof slab, beams columns rafts lintels and other structural members laid in situ recast laid in position complete in all respects ratio 1:2:4 90lbs cement 2cft sand 4cft shingle 1/8 to ¼ gauge (S.I.No.06 P.No.19). |  |  |  |  |  |  |  |  |  |
| (a) |  | Ground Floor |  |  |  | 218.00 | 337/- |  | %Cft | Rs.73466/- |  |
|  |  |  |  |  |  | Rupees Three Hundred Thirty Seven Only. |  |  |  |  |  |
| 05. |  | Fabrication of mild steel reinforcement for cement concrete including cutting bending, laying in position making joints and fastenings including cost of binding wire also including cost of binding wire also includes removal of rust from bars.(S.I.No.07 P.No.20). |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 9.732 | 5001/70 |  | P.Cwt | Rs.48677/- |  |
|  |  |  |  |  |  | Rupees Five Thousand One Ps. Seventy Only. |  |  |  |  |  |
| 06. |  | Filling watering and ramming earth in floor with surplus earth from foundation lead upto one chain and lift upto 5ft.(S.I.NO:11 P-5) |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 177.00 | 1512/50 |  | %0Cft | Rs.268/- |  |
|  |  |  |  |  |  | Rupees One Thousand Five Hundred Twelve Ps. Fifty Only. |  |  |  |  |  |
| 07. |  | Supplying and filling sand under floor and plugging in walls.(S.I.No.29 P.No.31). |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 276.00 | 1141/25 |  | %Cft |  | Rs.3150/- |
|  |  |  |  |  |  | Rupees One Thousand One Hundred Forty One Ps. Seventy Five Only. |  |  |  |  |  |
| 08. |  | Pacca brick work in Ground Floor and plinth in Ratio 1:6. (S.I.No.04 P.No.24). |  |  |  |  |  |  |  |  |  |
| (a) |  | Ground Floor |  |  |  | 238.00 | 12674/36 |  | %Cft | Rs.29385/- |  |
|  |  |  |  |  |  | Rupees Twelve Thousand Six Hundred Seventy Four Ps. Thirty Six Only. |  |  |  |  |  |

| 09. | Cement concrete brick or stone ballast 1-1/2 to 2 guage ratio 1:6:12.(S.I.NO.4 P-14) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 45.00 | 8122/95 | %Cft |  | Rs.3655/- |
|  |  |  | Rupees.Eight thousand One Hundred Twenty Two Ps.Ninety Five |  |  |  |  |
| 10. | Cement concrete plain including placing compacting finishing and curing, complete (including screening and washing of stone aggregate without shuttering Ratio 1:2:4. (S.I.No.05 P.No.18). |  |  |  |  |  |  |
|  |  |  | 26.00 | 14429/- |  | %Cft | Rs.3752/- |
|  |  |  | Rupees Fourteen Thousand Four Hundred Twenty Nine Only. |  |  |  |  |

| SR# | ITEM OF WORKS |  | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- |
| 11. | Cement plaster 1:6 upto 20 height 1/2”thick. (S.I.No. 13 P.No. 51 ). |  |  |  |  |  |
|  |  |  | 466.00 | 2260/06 | %Sft | Rs.10283/- |
|  |  |  | (Rs. Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only). |  |  |  |
| 12. | Cement plaster 1:4 upto 20 height 3/8”thick. (S.I.No.11 P.No.58). |  |  |  |  |  |
|  |  |  | 466.00 | 2197/52 | %Sft | Rs.10240/- |
|  |  |  | (Rs. Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only). |  |  |  |
| 13. | Laying floor of approved coloured glazed tiles ¼” thick laid in white cement & 1:2 pigment on a bed of ¾” thick cement mortar 1:2 (S.I No. 25/P-42) |  |  |  |  |  |
|  |  |  | 150.00 | 27747/06 | %Sft | Rs.41621/- |
|  |  |  | (Rs. Two Thousand Seven Hundred Forty Seven Ps. Zero Six Only). |  |  |  |
| 14. | White glazed tile ¼ thick dado in white cement and laid over cement sand mortar ¾ thick including finishing. (S.I.NO. P-50). |  |  |  |  |  |
|  |  |  | 166.00 | 28253/61 | %Sft | Rs.46977/- |
|  |  |  | Rupees Twenty Eight Thousand Two Hundred Fifty Three Ps. Sixty One Only. |  |  |  |

| 15. | Providing and laying 1”thick topping cement concrete 1:2:4 i/c surface finishing and dividing into panels. (S.I.No.16 (d) P.No.47). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 283.00 | 3275/50 | %Sft | Rs.9270/- |
|  |  |  | Rupees Three Thousand Two Hundred Seventy Five Ps. Fifty Only. |  |  |  |
| 16. | Cement Pointing i/c streaking of joints on wall ratio 1:3 (S.I.NO.16 P.No.21) |  |  |  |  |  |
|  |  |  | 435.00 | 1213/58 | %Sft | Rs.5279/- |
|  |  |  | Rupees One Thousand Two Hundred Thirteen Ps. Fifty Eight Only. |  |  |  |
| 17. | Colour wash two coats. |  |  |  |  |  |
|  |  |  | 435.00 | 859/90 | %Sft | Rs.3741/- |
|  |  |  | Rupees Eighty Hundred Fifty Nine Ps. Ninety Only. |  |  |  |

| 18. | Primary coat of chalk under distemper. (S.I.No.23 P.No.59). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 466.00 | 442/72 | %Sft | Rs.2063/- |
|  |  |  | Rupees Four Hundred Forty Two Ps. Seventy Two Only. |  |  |  |
| 19. | Distemper in Three coats. (S.I.No.24 P.No.59). |  |  |  |  |  |
|  |  |  | 466.00 | 1079/65 | %Sft | Rs.503/- |
|  |  |  | Rupees One Thousand Seventy Nine Ps. Sixty Five Only. |  |  |  |

|  |  |  | TOTAL | Rs.332444/- |
| --- | --- | --- | --- | --- |

% Above / Below (Amount to be added / deducted).		Rs.

Grand Total		Rs.

Total Amount in word:

Note:-	 Quantities and rates are provisional and may be changed as per /according to the Technical Sanction by the competent authority.

| CONTRACTOR | EXECUTIVE ENGINEER (BUILDINGS) WORKS & SERVICES DEPARTMENT KHAIRPUR |
| --- | --- |

(SCHEDULE B)

CONSTRUCTION OF HUJRA BIBI PAK DAMAN AND OPEN YARD U/C DRIB MEHAR SHAH TALUKA KINGRI DISTRICT KHAIRPUR

PART C COMPOUND WALL

| SR# | ITEM OF WORKS |  | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- |
| 01. | Excavation in foundation of buildings bridges and other structures including dag belling dressing refilling around structure with excavated earth watering and ramming lead upto 5ft. (b) in ordinary soil. (S.I.No.18 P.No.5). |  |  |  |  |  |
|  |  |  | 565.00 | 3176/25 | %0Cft | Rs.1795 |
|  |  |  | (Rs. Three Thousand One Hundred Seventy Six Ps. Twenty Five Only). |  |  |  |
| 02. | Cement concrete brick or stone ballast 1-1/2 to 2 gauge. (A) ratio 1:5:10. (S.I.No.4 P.No.17). |  |  |  |  |  |
|  |  |  | 141.00 | 8694/95 | %Cft | Rs.13277 |
|  |  |  | (Rs. Eight Thousand Six Hundred Ninety Four Ps. Ninety Five Only). |  |  |  |
| 03. | Pacca Brick work in foundation & plinth in cement & sand mortar Ratio 1:6 (S.I.No.4 (c) P.No.25). |  |  |  |  |  |
|  |  |  | 304.00 | 11948/36 | %Cft | Rs.36323/- |
|  |  |  | Rupees Eleven Thousand Nine Hundred Forty Eight Ps. Thirty Six Only. |  |  |  |
| 04. | Coursed Rubble masonry i/c hammer cement sand mortar ratio 1:6 (S.I.No.3 P.No.31). |  |  |  |  |  |
|  |  |  | 253.00 | 25321/- | %Cft | Rs.64062/- |
|  |  |  | (Rs. Twenty Five Thousand Three Hundred Twenty One Only). |  |  |  |
| 05. | R.C.C works in roof slab, beams columns rafts lintels and other structural members laid in situ recast laid in position complete in all respects ratio 1:2:4 90lbs cement 2cft sand 4cft shingle 1/8 to ¼ gauge (S.I.No.06 P.No.19). |  |  |  |  |  |
| (a) | Ground Floor |  | 138.00 | 337/- | %Cft | Rs.46506/- |
|  |  |  | Rupees Three Hundred Thirty Seven Only. |  |  |  |
| 06. | Fabrication of mild steel reinforcement for cement concrete including cutting bending, laying in position making joints and fastenings including cost of binding wire also including cost of binding wire also includes removal of rust from bars.(S.I.No.07 P.No.20). |  |  |  |  |  |
|  |  |  | 6.160 | 5001/70 | P.Cwt | Rs.30810/- |
|  |  |  | Rupees Five Thousand One Ps. Seventy Only. |  |  |  |
| 07. | Pacca brick work other than building in Ratio 1:6. (S.I.No.04 P.No.24). |  |  |  |  |  |
|  | C/wall |  | 496.00 | 12346/65 | %Cft | Rs.61239/- |
|  |  |  | Rupees Twelve Thousand Three Hundred Sixty Six Ps. Sixty Five Only. |  |  |  |
| 08. | Cement plaster 1:6 upto 20 height 1/2”thick. (S.I.No. 13 P.No. 51 ). |  |  |  |  |  |
|  |  |  | 676.00 | 2260/06 | %Sft | Rs.14917 |
|  |  |  | (Rs. Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only). |  |  |  |
| 09. | Cement plaster 1:4 upto 20 height 3/8”thick. (S.I.No.11 P.No.58). |  |  |  |  |  |
|  |  |  | 676.00 | 2197/52 | %Sft | Rs.14855/- |
|  |  |  | (Rs. Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only). |  |  |  |

| 11. | Cement Pointing i/c streaking of joints on wall ratio 1:3 (S.I.NO.16 P.No.21) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 620.00 | 1213/58 | %Sft | Rs.7524 |
|  |  |  | Rupees One Thousand Two Hundred Thirteen Ps. Fifty Eight Only. |  |  |  |

| SR# | ITEM OF WORKS |  | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- |
| 12. | Colour wash two coats. |  |  |  |  |  |
|  |  |  | 1296.00 | 859/90 | %Sft | Rs.11144/- |
|  |  |  | Rupees Eighty Hundred Fifty Nine Ps. Ninety Only. |  |  |  |
| 13. | M/F steel grated doors complete with locking arrangement angloe iron frame 2"x2"x3.8" and 3/4" sq. bars 4" centre to centre (SINO.23 P-92) |  |  |  |  |  |
|  |  |  | 40.00 | 594/57 | P.Sft | Rs.29069/- |
|  |  |  | Rupees Five Hundred Ninety Four Ps. Fifty Seven Only. |  |  |  |
| 14. | Painting New surface(a) Preparing surface painting corrugated surface, patent roofing etc. (SINO.5(a+b) P-69) for Two Coats |  |  |  |  |  |
|  |  |  | 80.00 | 1489/68 | P.Sft | Rs.1192/- |
|  |  |  | Rupees One Thousand Four Hundred Eighty Nine Ps. Sixty Eight Only. |  |  |  |

|  |  |  | TOTAL | Rs.332713/- |
| --- | --- | --- | --- | --- |

% Above / Below (Amount to be added / deducted).		Rs.

Grand Total		Rs.

Total Amount in word:

Note:-	 Quantities and rates are provisional and may be changed as per /according to the Technical Sanction by the competent authority.

| CONTRACTOR | EXECUTIVE ENGINEER (BUILDINGS) WORKS & SERVICES DEPARTMENT KHAIRPUR |
| --- | --- |

(SCHEDULE B)

CONSTRUCTION OF HUJRA BIBI PAK DAMAN AND OPEN YARD U/C DRIB MEHAR SHAH TALUKA KINGRI DISTRICT KHAIRPUR

PART  PAVED COURTYARD

| SR# | ITEM OF WORKS | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- |

| 01. | Filling watering and ramming earth under floor with new excavated from outside lead upto one chain and lift upto 5feet. (S.I.No.12 P.No.5). |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 1250.00 |  | 3630/- |  | %0Cft | Rs.4538/- |
|  |  |  |  |  | Rupees Three Thousand Six Hundred Thirty Only. |  |  |  |  |  |
| 02. | Extra lead for 3”mile. |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 1250.00 |  | 579/41 |  | %Cft | Rs.7243/- |
|  |  |  |  |  | Rupees Five Hundred Seventy Nine Ps. Forty One Only. |  |  |  |  |  |
| 03. |  | Supplying and filling sand under floor and plugging in walls.(S.I.No.29 P.No.31). |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 625.00 |  | 1141/25 | %Cft | Rs.7133/- |
|  |  |  |  |  |  | Rupees One Thousand One Hundred Forty One Ps. Seventy Five Only. |  |  |  |  |
| 04. |  | Cement concrete brick or stone ballast 1-1/2 to 2 gauge. (A) ratio 1:5:10. (S.I.No.4 P.No.17). |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 413.00 |  | 8694/95 | %Cft | Rs.35910/- |
|  |  |  |  |  |  | (Rs. Eight Thousand Six Hundred Ninety Four Ps. Ninety Five Only). |  |  |  |  |
| 05. |  | Cement concrete plain 1:2:4 including placing compacting finishing and curing, complete (including screening and washing of stone aggregate without shuttering Ratio 1:2:4. (S.I.No.05 P.No.18). |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 1250.00 | 14429/- |  | %Cft | Rs.41009/- |
|  |  |  |  |  |  | Rupees Fourteen Thousand Four Hundred Twenty Nine Only. |  |  |  |  |
|  |  |  |  |  |  | TOTAL |  |  |  | Rs.95833/- |

% Above / Below (Amount to be added / deducted).		Rs.

Grand Total		Rs.

Total Amount in word:

Note:-	 Quantities and rates are provisional and may be changed as per /according to the Technical Sanction by the competent authority.

| CONTRACTOR | EXECUTIVE ENGINEER (BUILDINGS) WORKS & SERVICES DEPARTMENT KHAIRPUR |
| --- | --- |

(SCHEDULE B)

CONSTRUCTION OF HUJRA BIBI PAK DAMAN AND OPEN YARD U/C DRIB MEHAR SHAH TALUKA KINGRI DISTRICT KHAIRPUR

PART-		WATER SUPPLY & SANITARY FITTINGS

| SR# | ITEM OF WORKS | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 01. | Providing G.I. pipe specials clamps etc including fixing cutting and fitting complete with and i/c the cost of breaking through walls and roof making good etc.(S.I.NO.1 P-11). |  |  |  |  |
|  | 4”Dia. | 14 Rft | 896/17 | P.Rft | Rs.12546/- |
|  | 1”Dia. | 50 Rft | 128/55 | P.Rft | Rs.6428/- |
|  | 1 ¼”Dia. | 40 Rft | 153/19 | P.Rft | Rs.6128/- |
|  | 1½”Dia. | 60 Rft | 188/97 | P.Rft | Rs.11338/- |
| 02. | P/F 6”x2”x or 6”x3” C.I. Floor trap of the approved self-cleaning design with a vent C.I. Skewed down gritting with or without a vent arm complete with and i/c the making requisite number of holes in walls plinth and floor for pipe connections and making good in c.,c.1.2.4. (S.I.NO.20 P-5). | 04 Nos. | 2024/43 | Each | Rs.8098/- |
| 03. | S/F Bib cock of superior quality with C.P head ½”dia. (S.I.No.13 P.No.15). | 20 Nos. | 1109/46 | Each | Rs.22189/- |
| 04. | S/F canceled stop cock of superior quality head ½”dia. (S.I.NO.11 (b) P-15). | 01 Nos. | 509/74 | Each | Rs.510/- |
| 05. | S/F handle valve china ¾”dia (S.I.No.7 P.No.14). | 3 Nos. | 271/92 | Each | Rs.816/- |
| 06. | Providing and fixing 6x4 C.C Galley trap with 4 outlets complete with 4”thick 1:2:4 C.C for head and ½”thick cement plaster 1:3 to the karb grating 6x6 Cole cover and fasts 12x12 inside etc complete with C.I cover and frames (S.I.No.1 (c) P.No.23). | 02 Nos. | 1193/17 | Each | Rs.2386/- |

PART-D-II	P.H SCHEDULE ITEM

| 07. | Boring tube well in all water bearing foil from ground level up to 100 ft or 30.5 meter west i/c thinking & drawing the casing pipe Hospital & Quarter 4”Dia. | 15 Rft | 242/- | P.Rft | Rs.3630/- |
| --- | --- | --- | --- | --- | --- |
| 08. | Constructing man hole or inspection chamber for the required dial of circular screw and 3-6(1067mm) depth with walls of Bobbin cement motor 1..32 cement plastered 1.3 ½ thick inside of walls and 1`(25mm) thick over benching and channel i/c fixing Caiman hole cover with frames of clear opening etc. | 02 Nos. | 14748/- | Each | Rs.29496/- |
| 09. | Providing and fixing PVC pipe class “D” class etc completed. | 60 Rft | 34/- | P.Rft | Rs.2040/- |

PART-D-III	E.I  ITEM

| 10. | Supplying and Fixing Water pumping Set ½” H.P Complete | 01 Nos | 8500/- | Each | Rs.8500/- |
| --- | --- | --- | --- | --- | --- |
|  |  |  | TOTAL |  | Rs.114165/- |

% Above/Below (Amount to be added/deducted).Rs.

Grand Total	   Rs.

Total Amount in word:

Note:-	Quantities and rates may be changed as per/according to the Technical Sanction by the competent authority.

| CONTRACTOR | EXECUTIVE ENGINEER (BUILDINGS) WORKS & SERVICES DEPARTMENT KHAIRPUR |
| --- | --- |

(SCHEDULE B)

REMAINING WORK OF DARGAH SACHAL SHAH JELLANI U/C DRIB MEHAR SHAH  TALUKA KINGRI    DISTRICT KHAIRPUR.

PART A MAIN BUILDING

| SR# |  | ITEM OF WORKS |  |  |  | QTY: |  | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01. |  | Excavation in foundation of buildings bridges and other structures including dag belling dressing refilling around structure with excavated earth watering and ramming lead upto 5ft. (b) in ordinary soil. (S.I.No.18 P.No.5). |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 368.00 |  | 3176/25 | %0Cft | Rs.1168/- |
|  |  |  |  |  |  | (Rs. Three Thousand One Hundred Seventy Six Ps. Twenty Five Only). |  |  |  |  |
| 02. | Cement concrete brick or stone ballast 1-1/2 to 2 gauge. (A) ratio 1:4:8. (S.I.No. P.No. ). |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 510.00 |  | 9416/95 |  | %Cft | Rs.48023/- |
|  |  |  |  |  | (Rs. Nine Thousand Four Hundred Sixteen Ps. Ninety Five Only). |  |  |  |  |  |
| 03. |  | R.C.C works in roof slab, beams columns rafts lintels and other structural members laid in situ recast laid in position complete in all respects ratio 1:2:4 90lbs cement 2cft sand 4cft shingle 1/8 to ¼ gauge (S.I.No.06 P.No.19). |  |  |  |  |  |  |  |  |
| (a) |  | Ground Floor |  |  |  | 210.00 |  | 337/- | %Cft | Rs.70770/- |
|  |  |  |  |  |  | Rupees Three Hundred Thirty Seven Only. |  |  |  |  |
| 04. |  | Fabrication of mild steel reinforcement for cement concrete including cutting bending, laying in position making joints and fastenings including cost of binding wire also including cost of binding wire also includes removal of rust from bars.(S.I.No.07 P.No.20). |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 11.25 |  | 5001/70 | P.Cwt | Rs.56268/- |
|  |  |  |  |  |  | Rupees Five Thousand One Ps. Seventy Only. |  |  |  |  |

| 05. | Pacca Brick work in foundation & plinth in cement & sand mortar Ratio 1:6 (S.I.No.4 (c) P.No.25). |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 356.00 |  | 11948/36 |  |  | %Cft |  | Rs.45121/- |
|  |  |  |  | Rupees Eleven Thousand Nine Hundred Forty Eight Ps. Thirty Six Only. |  |  |  |  |  |  |  |
| 06. | Providing and laying 1”thick topping cement concrete 1:2:4 i/c surface finishing and dividing into panels. (S.I.No.16 (d) P.No.47). |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 1302.00 |  | 3275/50 |  |  | %Sft |  | Rs.57442/- |  |
|  |  |  | Rupees Three Thousand Two Hundred Seventy Five Ps. Fifty Only. |  |  |  |  |  |  |  |  |
| 07. | Providing and fixing G.I. frames chowkats of size 7x2xor 4-1/2x3 for door using 20 gauge G.I. sheet i/c welded hinge and fixing at with cement sand slurry of ration 1.6 despairing the jambs. The cost also i/c all carriage tools and plants used in making and fixing.(S.I.NO.18 P-98). |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 53.00 |  | 228/90 |  | P.Rft |  |  | Rs.12131/- |  |
|  |  |  | Rupees Two Hundred Twenty Eight Ps. Ninety Only. |  |  |  |  |  |  |  |  |
| 08. | First class deodar wood wrought, joinery in doors & windows etc. fixed in position i/c chowkhats holds fasts hinges, iron tower blts. Chocks cleats, handles and cords with hooks, etc. Deodar paneled or paneled and glazed or fully glazed (without chowkhats). |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 74.00 |  | 902/93 |  | P.Sft |  | Rs.66817/- |  |
|  |  |  |  | Rupees Nine Hundred Two Ps. Ninety Three Only. |  |  |  |  |  |  |  |

| SR# | ITEM OF WORKS |  | QTY: |  |  | RATE |  | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 09. | Cement plaster 1:6 upto 20 height 1/2”thick. (S.I.No. 13 P.No. 51 ). |  |  |  |  |  |  |  |  |
|  |  |  |  | 651.00 |  | 2260/06 |  | %Sft | Rs.14365/- |
|  |  |  |  | (Rs. Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only). |  |  |  |  |  |
| 10. | Cement plaster 1:4 upto 20 height 3/8”thick. (S.I.No.11 P.No.58). |  |  |  |  |  |  |  |  |
|  |  |  |  | 651.00 | 2197/52 |  | %Sft |  | Rs.14306/- |
|  |  |  |  | (Rs. Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only). |  |  |  |  |  |
| 11. | Providing and fixing iron steel grill using solid square bars of size ½”x1/2” placed at 4” i/c and frame of flat iron patti of ¾”x3/4” i/c circle shape at 1-0 a part equivalent fitted with screws are pins i/c painting 3 coats with 1st coat of red oxide paint etc. (S.I.No.26 P.No.97). |  |  |  |  |  |  |  |  |
|  |  |  |  | 540.00 | 180/50 |  | P.Sft |  | Rs.97502/- |
|  |  |  |  | Rupees One Hundred Eighty Ps. Fifty Only. |  |  |  |  |  |
| 12. | M/F steel grated doors complete with locking arrangement angloe iron frame 2"x2"x3.8" and 3/4" sq. bars 4" centre to centre (SINO.23 P-92) |  |  |  |  |  |  |  |  |
|  |  |  |  | 100.00 | 726/72 |  | P.Sft |  | Rs.72672/- |
|  |  |  |  | Rupees Seven Hundred Twenty Six Ps. Seventy Five Only. |  |  |  |  |  |
| 13. | Painting New surface(a) Preparing surface painting corrugated surface, patent roofing etc. (SINO.5(a+b) P-69) for Two Coats |  |  |  |  |  |  |  |  |
|  |  |  |  | 1448.00 | 1489/68 |  | P.Sft |  | Rs.21571/- |
|  |  |  |  | Rupees One Thousand Four Hundred Eighty Nine Ps. Sixty Eight Only. |  |  |  |  |  |
| 14. | Preparing the surface & painting with matt finishing i/c rubbing the surface with Bathy (silicon carbide rubbing brick) filling the voids with zink / chalk/plaster or pairs mixture, applying first coat premix making the surface smooth and then painting 3 coats with matt finish of approved make etc complete (new surface) (SINO.36 P-55) for three coats |  |  |  |  |  |  |  |  |
|  |  |  |  | 2080.00 | 3444/38 |  | %Sft |  | Rs.71635/- |
|  |  |  |  | Rupees Three Thousand Four Hundred Forty Four One Ps. Thirty Eight Only. |  |  |  |  |  |
| 15. | Preparing the surface and painting with weather coat i/c rubbing the surface with rubbing brick / sand paper filling the voids with chalk/plaster of pairs and then painting with weather of approved make 2nd & subsq: coat.(SINO.38(b) P-56) (a+b) |  |  |  |  |  |  |  |  |
|  |  |  |  | 3549.00 | 1948/10 |  | %Sft |  | Rs.69138/- |
|  |  |  |  | Rupees One Thousand Nine Hundred Forty Eight Ps. Ten Only. |  |  |  |  |  |
| 16. | P/L tiles HALA or pattern tiles glazed 6"x6"x1/2" on floor or wall facing in required floor and pattern of STILE specification jointed in white cement and pigment over a base of 1:2 (SINO.61 P-48) |  |  |  |  |  |  |  |  |
|  |  |  |  | 657.00 | 47651/56 |  | %Sft |  | Rs.313071/- |
|  |  |  |  | Rupees Forty Seven Thousand Six Hundred Fifty One Ps. Fifty Six Only. |  |  |  |  |  |

|  |  |  | TOTAL | Rs.1001001/- |
| --- | --- | --- | --- | --- |

% Above / Below (Amount to be added / deducted).		Rs.

Grand Total		Rs.

Total Amount in word:

Note:-	 Quantities and rates are provisional and may be changed as per /according to the Technical Sanction by the competent authority.

| CONTRACTOR | EXECUTIVE ENGINEER (BUILDINGS) WORKS & SERVICES DEPARTMENT KHAIRPUR |
| --- | --- |

(SCHEDULE B)

REMAINING WORK OF DARGAH SACHAL SHAH JELLANI U/C DRIB MEHAR SHAH  TALUKA KINGRI    DISTRICT KHAIRPUR.

PART A MAIN BUILDING  NON SCHEDULE ITEMS

| SR# |  | ITEM OF WORKS |  |  |  | QTY: |  | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01. |  | Golden Colour Tile |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 1232.00 |  | 563/- | Sft | Rs.693616/- |
|  |  |  |  |  |  | (Rs. Five Hundred sixty three Only). |  |  |  |  |
| 02. | Golden Type Mirror |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 508.00 |  | 563/- |  | Sft | Rs.286004/- |
|  |  |  |  |  | (Rs. Five Hundred Sixty three Only). |  |  |  |  |  |
| 03. |  | Golden colourNeal |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 01 Nos |  | 6700/- | %Cft | Rs.6700/- |
|  |  |  |  |  |  | Rupees Six Thousands seven Hundred Only. |  |  |  |  |
| 04. |  | Fiber Glass Shade |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 100.00 |  | 983/- | P,sft | Rs.98300/- |
|  |  |  |  |  |  | Rupees Nine Hundred Eighty three Only. |  |  |  |  |

| 05. | Wooden work Railing window |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 357.00 | 100/- | P.sft |  | Rs.35700/- |  |
|  |  |  | Rupees One Hundred Only. |  |  |  |  |  |
|  |  |  | TOTAL |  |  | Rs.1120320/- |  |  |

% Above / Below (Amount to be added / deducted).		Rs.

Grand Total		Rs.

Total Amount in word:

Note:-	 Quantities and rates are provisional and may be changed as per /according to the Technical Sanction by the competent authority.

| CONTRACTOR | EXECUTIVE ENGINEER (BUILDINGS) WORKS & SERVICES DEPARTMENT KHAIRPUR |
| --- | --- |

(SCHEDULE B)

REMAINING WORK OF DARGAH SACHAL SHAH JELLANI U/C DRIB MEHAR SHAH  TALUKA KINGRI    DISTRICT KHAIRPUR.

PART B PAVED COURTYARD

| SR# |  | ITEM OF WORKS |  |  |  | QTY: |  | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01. | Cement concrete brick or stone ballast 1-1/2 to 2 gauge. (A) ratio 1:4:8. (S.I.No. P.No. ). |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 2051.00 |  | 9416/95 |  | %Cft | Rs.193128/- |
|  |  |  |  |  | (Rs. Nine Thousand Four Hundred Sixteen Ps. Ninety Five Only). |  |  |  |  |  |
| 02. |  | Cement concrete plain including placing compacting finishing and curing, complete (including screening and washing of stone aggregate without shuttering Ratio 1:2:4. (S.I.No.05 P.No.18). |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 1025.00 |  | 14429/25 | %Cft | Rs.147900/- |
|  |  |  |  |  |  | Rupees Fourteen Thousand Four Hundred Twenty Nine Ps. Twenty Five Only. |  |  |  |  |
|  |  |  |  |  |  | 2788.00 |  | 8694/95 | %Cft | Rs.242415/- |
| 03. |  | P/F cement paving blocks flooring having size of 197x97x60(mm) of city/Quddra / cobble shape with natural colours, having strength b/w 5000 psi to 8500 psi i/c filling the joints with hill sand and laying specified manner / pattern and design etc complete (SN:72 P-49) |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 4101.00 |  | 223/97- | 1Sft | Rs.918500/- |
|  |  |  |  |  |  | Rupees Two Hundred Twenty Three Ps.Ninety Seven Only. |  |  |  |  |

|  |  |  | TOTAL | Rs.1259528/- |
| --- | --- | --- | --- | --- |

% Above / Below (Amount to be added / deducted).		Rs.

Grand Total		Rs.

Total Amount in word:

Note:-	 Quantities and rates are provisional and may be changed as per /according to the Technical Sanction by the competent authority.

| CONTRACTOR | EXECUTIVE ENGINEER (BUILDINGS) WORKS & SERVICES DEPARTMENT KHAIRPUR |
| --- | --- |

(SCHEDULE B)

CONSTRUCTION OF HYDERI IMAM BARGAH AT OLD JOGI MUHALLA  TALUKA KINGRI

DISTRICT KHAIRPUR.

PART A IMAM BARGAH

| SR# | ITEM OF WORKS |  | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- |
| 01. | Excavation in foundation of buildings bridges and other structures including dag belling dressing refilling around structure with excavated earth watering and ramming lead upto 5ft. (b) in ordinary soil. (S.I.No.18 P.No.5). |  |  |  |  |  |
|  |  |  | 316.00 | 3176/25 | %0Cft | Rs.1004/- |
|  |  |  | (Rs. Three Thousand One Hundred Seventy Six Ps. Twenty Five Only). |  |  |  |
| 02. | Cement concrete brick or stone ballast 1-1/2 to 2 gauge. (A) ratio 1:5:10. (S.I.No.4 P.No.17). |  |  |  |  |  |
|  |  |  | 145.00 | 8694/95 | %Cft | Rs.12608/- |
|  |  |  | (Rs. Eight Thousand Six Hundred Ninety Four Ps. Ninety Five Only). |  |  |  |

| 03. | Un Coursed Rubble masonry in cement sand mortar ratio 1:6 (S.I.No.2 P.No.27). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 165.00 | 17723/- | %Cft | Rs.29243/- |
|  |  |  | Rupees Seventeen Thousand Seven Hundred Twenty Three Only. |  |  |  |

| 04. | Coursed Rubble masonry i/c hammer cement sand mortar ratio 1:6 (S.I.No.3 P.No.31). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 178.00 | 25321/- | %Cft | Rs.45071/- |
|  |  |  | (Rs. Twenty Five Thousand Three Hundred Twenty One Only). |  |  |  |
| 05. | R.C.C works in roof slab, beams columns rafts lintels and other structural members laid in situ recast laid in position complete in all respects ratio 1:2:4 90lbs cement 2cft sand 4cft shingle 1/8 to ¼ gauge (S.I.No.06 P.No.19). |  |  |  |  |  |
|  |  |  | 66.00 | 337/- | %Cft | Rs.22242/- |
|  |  |  | Rupees Three Hundred Thirty Seven Only. |  |  |  |
| 06. | Fabrication of mild steel reinforcement for cement concrete including cutting bending, laying in position making joints and fastenings including cost of binding wire also including cost of binding wire also includes removal of rust from bars.(S.I.No.07 P.No.20). |  |  |  |  |  |
|  |  |  | 02.946 | 5001/70 | P.Cwt | Rs.14735/- |
|  |  |  | Rupees Five Thousand One Ps. Seventy Only. |  |  |  |
| 07. | Filling watering and ramming earth in floor with surplus earth from foundation lead upto one chain and lift upto 5ft.(S.I.NO:11 P-5) |  |  |  |  |  |
|  |  |  | 316.00 | 1512/50 | %Cft | Rs.319/- |
|  |  |  | (Rs. One Thousand Five Hundred Eleven Ps. Fifty Only). |  |  |  |

| 08. | Filling watering and ramming earth under floor with new excavated from outside lead upto one chain and lift upto 5ft (S.I.No.12 P.No.5). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 252.00 | 3630/- | %0Cft | Rs.915/- |
|  |  |  | Rupees Three Thousand Six Hundred Thirty Only. |  |  |  |
| 09. | Extra lead for 3”mile. |  |  |  |  |  |
|  |  |  | 252.00 | 579/41 | %Cft | Rs.1460/- |
|  |  |  | Rupees Five Hundred Seventy Nine Ps. Forty One Only. |  |  |  |

| SR# | ITEM OF WORKS | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- |

| 10. | Supplying and filling sand under floor and plugging in walls.(S.I.No.29 P.No.31). |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 101.00 |  | 1141/75 | %Cft |  |  | Rs.1153/- |  |
|  |  |  |  |  | Rupees One Thousand One Hundred Forty One Ps. Seventy Five Only. |  |  |  |  |  |  |  |
| 11. | Pacca brick work in Ground Floor and plinth in Ratio 1:6. (S.I.No.04 P.No.24). |  |  |  |  |  |  |  |  |  |  |  |
| (a) | Ground Floor |  |  |  | 462.00 |  | 12674/36 | %Cft |  |  | Rs.58556/- |  |
|  |  |  |  |  | Rupees Twelve Thousand Six Hundred Seventy Four Ps. Thirty Six Only. |  |  |  |  |  |  |  |
| 12. | Providing and fixing G.I. frames chowkats of size 7x2xor 4-1/2x3 for door using 20 gauge G.I. sheet i/c welded hinge and fixing at with cement sand slurry of ration 1.6 despairing the jambs. The cost also i/c all carriage tools and plants used in making and fixing.(S.I.NO.18 P-98). |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 54.00 |  | 228/90 | P.Rft |  |  | Rs.12361/- |  |
|  |  |  |  |  | Rupees Two Hundred Twenty Eight Ps. Ninety Only. |  |  |  |  |  |  |  |
| 13. | Providing and fixing G.I. Frames chowkats of size 7x2xir 4-1/2x3 for windows using 20guage G.I. sheet i/c welded hinges and fixing at site with necessary hold fasts, filling with cement sand slurry of ration 1;6 and repainting the jamb. The cost also i/c all carriage tools and plants used in making and fixing. (S.I.No.29 P.No.98). |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 27.00 |  | 240/50 | P.Rft |  |  | Rs.6494/- |  |
|  |  |  |  |  | Rupees Two Hundred Forty Ps. Fifty Only. |  |  |  |  |  |  |  |
| 14. | Providing and Fixing Iron Girders (material Sch: item No: 140) |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 11.071 |  | 3850/- |  |  | P.Cwt |  |  | Rs.42623/- |
|  |  |  |  | (Rs. Three Thousands Eight Hundred Fifty Only). |  |  |  |  |  |  |  |  |
| 15. | Supplying fixing T-Irons (material Sch: item No: 144) |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 5.089 |  |  | 3575/- |  |  | P.Cwt |  | Rs.18193/- |
|  |  |  |  | (Rs. Three Thousands Five Hundred Seventy Five Only). |  |  |  |  |  |  |  |  |
| 16. | Second class tile roofing consisting of 4" earth and 1" mud plaster with Gobri leeping over 1/2"thick cement plaster 1:6 with 34 Lbs. Of hot bitumen coating sand blinded, provided over one layer of 12" x 6"x2" tiles laid in 1:6 cement mortar including 1:2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 302.00 |  |  | 7607/25- |  |  | %Sft |  | Rs.22974/- |
|  |  |  |  | (Rs. Seventy Six Thousands Seven Ps. Twenty Five Only). |  |  |  |  |  |  |  |  |
| 17. | Cement plaster 1:6 upto 20 height ½ thick. (S.I.No.13 P.No.58). |  |  |  |  |  |  |  |  |  |  |  |
| (a) | Ground Floor |  |  |  | 846.00 |  | 2206/60 | %Sft |  |  | Rs.18668/- |  |
|  |  |  |  |  | Rupees Two Thousand Two Hundred Six Ps. Sixty Five Only. |  |  |  |  |  |  |  |
| 18. | Cement plaster 1:4 upto 20 height 3/8 thick. (S.I.No.11 P.No.58). |  |  |  |  |  |  |  |  |  |  |  |
| (a) | Ground Floor |  |  |  | 846.00 |  | 2197/52 | %Sft |  |  | Rs.18591/- |  |
|  |  |  |  |  | Rupees Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only. |  |  |  |  |  |  |  |

| SR# | ITEM OF WORKS |  | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- |
| 19. | Providing and fixing iron steel grill using solid square bars of size ½”x1/2” placed at 4” i/c and frame of flat iron patti of ¾”x3/4” i/c circle shape at 1-0 a part equivalent fitted with screws are pins i/c painting 3 coats with 1st coat of red oxide paint etc. (S.I.No.26 P.No.97). |  |  |  |  |  |
|  |  |  | 18.00 | 180/50 | P.Sft | Rs.3249/- |
|  |  |  | Rupees One Hundred Eighty Ps. Fifty Only. |  |  |  |
| 20. | Providing and laying 1”thick topping cement concrete 1:2:4 i/c surface finishing and dividing into panels. (S.I.No.16 (d) P.No.47). |  |  |  |  |  |
|  |  |  | 216.00 | 3275/50 | %Sft | Rs.7075/- |
|  |  |  | Rupees Three Thousand Two Hundred Seventy Five Ps. Fifty Only. |  |  |  |
| 21. | First class deodar wood wrought, joinery in doors & windows etc. fixed in position i/c chowkhats holds fasts hinges, iron tower blts. Chocks cleats, handles and cords with hooks, etc. Deodar paneled or paneled and glazed or fully glazed (without chowkhats). |  |  |  |  |  |
|  |  |  | 89.00 | 902/93 | P.Sft | Rs.80361/- |
|  |  |  | Rupees Nine Hundred Two Ps. Ninety Three Only. |  |  |  |

| 22. | Cement Pointing i/c streaking of joints on wall ratio 1:3 (S.I.NO.16 P.No.21) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 558.00 | 1213/58 | %Sft | Rs.6772/- |
|  |  |  | Rupees One Thousand Two Hundred Thirteen Ps. Fifty Eight Only. |  |  |  |
| 23. | Colour wash two coats. |  |  |  |  |  |
|  |  |  | 558.00 | 859/90 | %Sft | Rs.4798/- |
|  |  |  | Rupees Eight Hundred Fifty Nine Ps. Ninety Only. |  |  |  |

| 24. | Primary coat of chalk under distemper. (S.I.No.23 P.No.59). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 846.00 | 442/75 | %Sft | Rs.3746/- |
|  |  |  | Rupees Four Hundred Forty Two Ps. Seventy Five Only. |  |  |  |
| 25. | Distemper in two coats. (S.I.No.24 P.No.60). |  |  |  |  |  |
|  |  |  | 846.00 | 1079/65 | %Sft | Rs.9134/- |
|  |  |  | Rupees One Thousand Seventy Nine Ps. Sixty Five Only. |  |  |  |

| 26. | Painting New surface(a) Preparing surface painting corrugated surface, patent roofing etc. (SINO.5(a+b) P-69) for Two Coats |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 178.00 | 1489/68 | %Sft | Rs.2652/- |
|  |  |  | Rupees One Thousand Four Hundred Eighty Nine Ps. Sixty Eight Only. |  |  |  |

|  |  |  | TOTAL | Rs.444997/- |
| --- | --- | --- | --- | --- |

% Above / Below (Amount to be added / deducted).		Rs.

Grand Total		Rs.

Total Amount in word:

Note:-	 Quantities and rates are provisional and may be changed as per /according to the Technical Sanction by the competent authority.

| CONTRACTOR | EXECUTIVE ENGINEER (BUILDINGS) WORKS & SERVICES DEPARTMENT KHAIRPUR |
| --- | --- |

(SCHEDULE B)

CONSTRUCTION OF HYDERI IMAM BARGAH AT OLD JOGI MUHALLA  TALUKA KINGRI

DISTRICT KHAIRPUR.

PART B COMPOUND WALL

| SR# | ITEM OF WORKS |  | QTY: | RATE | UNIT |  | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 01. | Excavation in foundation of buildings bridges and other structures including dag belling dressing refilling around structure with excavated earth watering and ramming lead upto 5ft. (b) in ordinary soil. (S.I.No.18 P.No.5). |  |  |  |  |  |  |
|  |  |  | 800.00 | 3176/25 | %0Cft |  | Rs.2541/- |
|  |  |  | (Rs. Three Thousand One Hundred Seventy Six Ps. Twenty Five Only). |  |  |  |  |
| 02. | Cement concrete brick or stone ballast 1-1/2 to 2 gauge. (A) ratio 1:5:10. (S.I.No.4 P.No.17). |  |  |  |  |  |  |
|  |  |  | 200.00 | 8694/95 | %Cft |  | Rs.17390/- |
|  |  |  | (Rs. Eight Thousand Six Hundred Ninety Four Ps. Ninety Five Only). |  |  |  |  |
| 03. | Pacca Brick work in foundation & plinth in cement & sand mortar Ratio 1:6 (S.I.No.4 (c) P.No.25). |  |  |  |  |  |  |
|  |  |  | 525.00 | 11948/36 | %Cft |  | Rs.62729/- |
|  |  |  | Rupees Eleven Thousand Nine Hundred Forty Eight Ps. Thirty Six Only. |  |  |  |  |
| 04. | Coursed Rubble masonry i/c hammer cement sand mortar ratio 1:6 (S.I.No.3 P.No.31). |  |  |  |  |  |  |
|  |  |  | 251.00 | 25321/- | %Cft |  | Rs.63556/ |
|  |  |  | (Rs. Twenty Five Thousand Three Hundred Twenty One Only). |  |  |  |  |
| 05. | R.C.C works in roof slab, beams columns rafts lintels and other structural members laid in situ recast laid in position complete in all respects ratio 1:2:4 90lbs cement 2cft sand 4cft shingle 1/8 to ¼ gauge (S.I.No.06 P.No.19). |  |  |  |  |  |  |
| (a) | Ground Floor |  | 145.00 | 337/- | %Cft |  | Rs.48865/- |
|  |  |  | Rupees Three Hundred Thirty Seven Only. |  |  |  |  |
| 06. | Fabrication of mild steel reinforcement for cement concrete including cutting bending, laying in position making joints and fastenings including cost of binding wire also including cost of binding wire also includes removal of rust from bars.(S.I.No.07 P.No.20). |  |  |  |  |  |  |
|  |  |  | 6.473 | 5001/70 | P.Cwt |  | Rs.32376/- |
|  |  |  | Rupees Five Thousand One Ps. Seventy Only. |  |  |  |  |
| 07. | Pacca brick work other than building in Ratio 1:6. (S.I.No.04 P.No.24). |  |  |  |  |  |  |
|  | C/wall |  | 523.00 | 12346/65 | %Cft |  | Rs.64573/- |
|  |  |  | Rupees Twelve Thousand Three Hundred Sixty Six Ps. Sixty Five Only. |  |  |  |  |
| 08. | Cement plaster 1:6 upto 20 height 1/2”thick. (S.I.No. 13 P.No. 51 ). |  |  |  |  |  |  |
|  |  |  | 773.00 | 2260/06 | %Sft | Rs.17057/- |  |
|  |  |  | (Rs. Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only). |  |  |  |  |
| 09. | Cement plaster 1:4 upto 20 height 3/8”thick. (S.I.No.11 P.No.58). |  |  |  |  |  |  |
|  |  |  | 773.00 | 2197/52 | %Sft | Rs.16987/- |  |
|  |  |  | (Rs. Two Thousand One Hundred Ninety Seven Ps. Fifty Two Only). |  |  |  |  |

| SR# | ITEM OF WORKS | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- |

| 10. | Cement Pointing i/c streaking of joints on wall ratio 1:3 (S.I.NO.16 P.No.21) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 596.00 | 1213/58 | %Sft | Rs.7233/- |
|  |  |  | Rupees One Thousand Two Hundred Thirteen Ps. Fifty Eight Only. |  |  |  |
| 11. | Colour wash two coats. |  |  |  |  |  |
|  |  |  | 596.00 | 859/90 | %Sft | Rs.5125/- |
|  |  |  | Rupees Eight Hundred Fifty Nine Ps. Ninety Only. |  |  |  |
| 12. | M/F steel grated doors complete with locking arrangement angloe iron frame 2"x2"x3.8" and 3/4" sq. bars 4" centre to centre (SINO.23 P-92) |  |  |  |  |  |
|  |  |  | 35.00 | 726/72 | P.Sft | Rs.25435/- |
|  |  |  | Rupees Seven Hundred Twenty Six Ps. Seventy Five Only. |  |  |  |
| 13. | Painting New surface(a) Preparing surface painting corrugated surface, patent roofing etc. (SINO.5(a+b) P-69) for Two Coats |  |  |  |  |  |
|  |  |  | 70.00 | 908.50 | %Sft | Rs.636/- |
|  |  |  | Rupees One Thousand Four Hundred Eighty Nine Ps. Sixty Eight Only. |  |  |  |

|  |  |  | TOTAL | Rs.364503/- |
| --- | --- | --- | --- | --- |

% Above / Below (Amount to be added / deducted).		Rs.

Grand Total		Rs.

Total Amount in word:

Note:-	 Quantities and rates are provisional and may be changed as per /according to the Technical Sanction by the competent authority.

| CONTRACTOR | EXECUTIVE ENGINEER (BUILDINGS) WORKS & SERVICES DEPARTMENT KHAIRPUR |
| --- | --- |