NAME OF WORK:-	CONSTRUCTION   OF GOVERNMENT VETERINARY DISPENSARY AT VILLAGE KHUSH KHAIR MUHAMMAD TALUKA FAIZ GUNJ DISTRICT KHAIRPUR.

(SCHEDULE -B)

| Sr | Item of work |  | QTY | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- | --- |

1/- 	Excavation in foundation of building bridges and other structures i/c dag balling dressing refilling around structure with excavated earth watering and ramming lead up-to 5ft. (b) in ordinary soil. (S.I.No.18 (b) P.No.5)

Qty: 3141 Cft	@RS:3176.25		% Cft	RS:9977/-

Thirty one hundred seventy six rups:twenty five ps:

02/-       	Cement concrete stone of brick ballast 1½” to 2” gauge. Ratio (1:5:10). (S.I.No.4 (b) P.No.14)

Qty: 786 Cft	@RS: 8694.95		% Cft	RS:68342/-

(Eight six hundreds ninty four rupees,ninty five ps:)

03/-	Pacca Brick work in foundation / Plint  in cement

cement  & sand  mortar  ratio (1:6)(S.I No:05 (c)P.No:20)

Qty: 1571 Cft	@RS: 11948.36  	% Cft	RS:187709/-

(Eleven thousands nine hundered fourty eight rs: thirty six ps.)

04/-	Coursed Rubble Masnory 1:6 including hammer dressing (S.I No: 2 d P.26).

Qty: 1047 Cft	@RS: 25321/-		% Cft	RS: 265111/-

(twenty five thousands three hundreds te=wenty one rups:)

5/-	R.C.C. work in roof slab beams columns rafts lintels and other structural members laid in situ or precast laid in position complete in all respects: (i) Ratio (1:2:4) .(S,I No:06 P-19).

Qty:  1387cft	@RS: 337/-		P.Cft	RS: 467419/-

(Three hudred thirt seven ps.)

6/-	Fabrication of mild steel reinforcement for 	cement Concrete i/c cutting bending laying in position making joints and fastenings i/c cost of binding wire (also includes removal of rust from bars) (a) Using tor bars. (S.I.No.7 (a)  P.No.20)

Qty:  61.919cwt  @RS:5001.70	P.cwt	RS:309700/-

(Five thousands & one ripes, seventy ps:)

7/-	Pacca  Brick  work in Ground Floor   in cement sand mortarRatio (1:6) (S.I No:5 P-25).

Qty: 3399 Cft	@RS: 12674.65	 % Cft	RS: 430801/-

(Tweleve thousands six hundred seventy four rupees, sixty five ps.)

| Sr | Item of work |  | QTY | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- | --- |

10/-	Filling watering & ramming earth from foundation lead up to one chain lift up to 5 feet (S.I NO:   P.NO:     ).

Qty: 3141 cft		@RS: 1512.60		%0cft	RS: 4751/-

(Fifteen hundred tweleve rupess, sixty ps:)

11/-	Supplying Sand under floor plugging into wall (S.I NO: 29 P.NO: 26)

Qty: 2876 cft		@RS: 1141.25		% Cft	RS: 32822/-

(Eleven hundred fourty one rupees, twenty five ps.)

12/-	Cement Plaster 1:6 up to 20’-0 height ½” thick. (S.I NO: 13(b)    P: NO:52   )

Qty: 3102 Sft		@RS: 2206.60		% Sft	RS: 68449/-

(Twenty two hundreds six rupees, sixty ps.)

13/-	Cement Plaster 1:4 up to 20”-0 heights 3/8” thick. . ( S.I NO: 11(a) P:NO:52   )

Qty: 3102 Sft		@RS: 2197.52		% Sft	RS: 68167/-

(Twenty one hundred ninty seven rupees, fifty two ps.)

14/-	Cement concrete stone of brick ballast 1½” to 2” gauge. Ratio (1:6:12). (S.I.No.4 (b) P.No.14)

Qty: 987 cft		@RS: 8122.95		% Cft	RS: 77087/-

( eight thousands one hundreds twenty two rupees ninty five ps:)

15/-	Cement Concrete plain including placing Compacting

Finishing & curing, complete 	i/c screening & washing

Of stone aggregate Ratio (1:2:4) (S.I No:5 P-18).

[

Qty: 489 cft		@RS: 14429.25	% Cft	RS:70559/-

( fourteen thousands four hundreds twenty nine rupes twenty five ps:)

16/-	Laying white marble fine dressed in the surface without winding set in lime mortar 1:2 i/c rubbing and polishing pf the joints (a) ¾” thick flooring. (S.I.NO.28 (a) Page No. 43)

Qty: 469 Sft		@RS: 567.48		P.Sft	RS: 266148/-

17/-	Providing & Laying tiles 6”x6”x1/4” on floor on wall facing in required color and pattern of side specification jointed in white cement and pigment over a base 1:2 grey cement mortar ¾” i/c washing and filling of joints slurry  of white cement and pigment in desired  shape with finishing cleaning and coats polishing etc complete (S.I No:   P.NO:    )

Qty: 70 sft		@RS: 30509.77	% Sft	RS: 21357/-

( THIRTY  thousands FIVE hundred  nine rups: seventy seven ps.)

| Sr | Item of work |  | QTY | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- | --- |

18/-	Providing and fixing G.I frames/chowkhats of size 7”x2” or 4½”x3” for door using 20 gauge G.I sheet i/c welded hinges and fixing at site with necessary hold fasts, filling with cement sand slurry of ratio 1:6 and repairing the jambs. The cost also i/c all carriage, tools and plants used in making and fixing.   (S.I.No. 28 Page No. 93)

Qty: 86 Rft		@RS: 228/90		P.Rft	RS: 19685/-

(Two hundred twenty eight rupees, ninty ps.)

19/-	Providing and fixing G.I frames/chowkhats of size 7”x2” or 4½”x3” for window using 20 gauge G.I sheet i/c welded hinges and fixing at site with necessary hold fasts, filling with cement sand slurry of ratio 1:6 and repairing the jambs. The cost also i/c all carriage, tools and plants used in making and fixing.   (S.I.No. 28 Page No. 93)

Qty: 12 Rft		@RS: 240.50		P.Rft	RS: 2886/-

(Two hundred fourty rupees, fifty ps.)

20/-	First class deodar wood  wrought joinery  in doors & windows  etc fixed in position  including  hold fasts hinges  iron  tower bolts chock, cleats  handles  and  cords  with hooks etc complete.

(S.I No: 54 P.NO:  65).

Qty: 87 Sft		@RS:902.92		P.Sft	RS: 78555/-

(Nine hundred two rupees, ninty two ps.)

21/-	Providing & Fixing Wire gauze fixed to chowakats etc complete.

Qty: 90 Sft		@RS: 190.72		P.Sft	RS: 17165/-

(One hundred ninty rupees seventy two ps:)

22/-	Supplying and fixing in position iron/steel grill of ¾” x ¼” size flat iron of approved design including painting 3 coats etc complete (Weight or not less than 3.7 Lbs/sq feet of finished grill

(S.I No:       P.NO:       ).

Qty: 73 Sft		@RS: 180.50		P.Sft	RS: 13177/-

(One hundred eighty rupees, fifty ps.)

23/-	Reinofercement cement concrete spout including fixing in position 2 1/2x6”x5” (S.I NO: 14 P-14).

Qty: 06	No’s		@RS: 261.25		each	RS: 1568/-

(Two hundred sixty one rupees, twenty five ps.)

24/-	Providing & Fixing ornamental cement concrete jalli  24” thick (S.I NO:    P.NO:   )

Qty: 96 sft		@RS: 276.02		P.Sft	RS: 26498/-

| Sr | Item of work |  | QTY | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- | --- |

25/-	Providing and lying 1” thick C/C topping cement concrete (1:2:4) i/c surface finishing and dividing into panels.  (S.I.No.16 (d) P.No.42).(2” thick).

Qty: 1825 sft		@RS: 3275.50	% Sft	RS: 59778/-

(fourty four hundreds eleven rupees eighty two ps:)

26/-	Two coats of Bitumen hot using 34 Lbs (S.I NO:     P.NO:    ).

Qty: 1644 Sft		@RS: 1887.04		% Sft	RS: 31973/-

(eighteen hundreds eighty seven rupees four ps:)

27/-	Priming coat of Chalk under distemper (S.I NO:23 P-53).

Qty: 3102 Sft		@RS: 442.75		P.Sft	RS: 13734/-

(Four hundred fourty two rupees, seventy five ps.)

28/-	Cement Pointing struck joints on wall raio 1:6 (P.NO: 19 P-59).

Qty: 1998 sft		@RS: 1287.44		% Sft	RS: 25723/-

29/-	Mattfinish two coats (S.I NO: 24 P-60).

Qty: 3102 sft		@RS: 1478.58		% Sft	RS: 61718/-

30/-	Colour washing two 02 coats etc

Qty: 1998 Sft		@RS: 859.90		% Sft	RS: 17181/-

(Eight hundred fifty nine rupees, ninty ps.)

31/-	Painting doors & Windows any type including edges of three coats (S.I NO:40 ii P 67).

Qty: 416 Sft		@RS: 1489.68		% Sft	RS: 6197/-

(fourteen hundreds eght nine rupees sixty eight ps.)

32/-	Khapril of cement concrete 12”x8”x1” of approved design/Shape laidflot in 1:2 grey cement over a bed of ¾” thick grey cement mortar 1:2 (S.I No: 39 P.NO:380.

Qty: 358 sft		@RS: 3645.65		% Sft	RS: 13051/-

==================================

Total						RS: 2737288/-

______% Above / Below (Amount to be added as per/deducted)RS:___________

G.Total					RS:

Total Amount in word:________________________________________________

__________________________________________________________________

Note:- 	Quantities ans rates may be changed as per/according to the Techinical Sanction vy the competent authority.

EXECUTIVE  ENGINEER

(BUILDINGS) WORKS & SERVICES

DEPARTMENT  KHAIRPUR

| Sr | Item of work |  | QTY | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- | --- |

PART-    WATER SUPPLY

1/-	P/F squatting type white glazed of flushing cistern with internal fitting and flush pipe with band and making requisite Nos. of holes in wall plinth & floor for pipe connections and making good in

Cement concrete 1:2:4. (A) W.C pan of not less than 23” clear opening between flashing rims and  3 gallons flushing tank. With 4” dia C.I. tape. (S.I.NO.1(A) P.No.1)

Qty: 02 No	@RS: 4846.60	 each	RS: 9693/-

(fourty eight hundreds fourty six rupes sixity ps:)

2/-	P/F 22”x16” lavatory basin in white glazed earthen ware complete 	with & i/c the cost of W.I or C.I 	cantilever brackets 6” built into 	wall 	painted white in two coats after a primary coat of red lead 	paint a pair of ½” dia chrome plate pillar traps 1-1/2” dia rubber plug 	& chrome plated brass 	chain 	1-1/4” dia 	malleable iron or 	c.p brass traps malleable iron or brass union & making 	requisite 	number of holes in walls plinth & floor for pipe connection and 	making good in 	cement concrete 	1:2:4 (standard pattern). 	Add extra for labour for providing & fixing of 	earthenware 	pedestal white 	or coloured glazed (Standard pattern) .  	(S.I.NO. 8 P. No.3)

Qty: 02 No	@RS: 4928/-	each	RS: 9856/-

(fourty nine hundreds twenty eight rupes)

3/-	Providing G.I pipes special & clamps etc i/c fixing cutting and fitting complete with and i/c the cost of breaking through walls and roof making good etc painting two coats after cleaning the pipe etc with white zink paint with pigment to match the colour of the building and testing with water to a pressure head of 200 feet and handling. (Internal) (i) ½ dia G.I pipe. (S.I.NO.1 (i) P. No.11)

“	“	1 ¼” dia			Qty: 30 Rft	@RS: 188.97	P.Rft	RS: 5669/-

“	“	1” Dia				Qty: 20 Rft	@RS: 128.55	P.Rft	RS: 2571/-

“	“	¾” dia				Qty: 125 Rft	@RS: 95.79	P.Rft	RS: 11974/-

“	“	½” Dia				Qty: 75 Rft	@RS: 73.21	P.Rft	RS: 5491/-

4/-	P/F 6”x2” or 6”x3” C.I floor trap of the approved self cleaning design with a C.I screwed down gritting with or without a vent arm complete with and i/c making requisite number or holes in walls plinth and floor for pipe connections & making good in CC 1:2:4. (S.I. No. 20 P. No.5)

Qty: 02 NOs	@RS: 2024.43	each	RS: 4049/-

5/-	Supplying and fixing concealed stopcock of superior quality with c.p head ½” dia.(S.I.NO.13 (a) P. No.15)

Qty: 01 No	@RS: 889.46	each	RS: 889/-

| Sr | Item of work |  | QTY | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- | --- |

6/-	P/F Chrome plated brass towel rail complete with brakes fixing on wooden etc (S.I NO: 1 P-7)

Qty: 01 No	@RS: 1269.95	each	RS: 1270/-

7/-	Providing & fixing 15" x 12" bavelled edge mirror of belgium glass complete with 1/8" thick hard board and c.p screws fixed to wooden pleat.

Qty: 01 No	@RS: 1161.60	each	RS: 1162/-

8/-	Providing & fixing in position nyloon connections  complete with 1/2" dia, brass stop cock with pair of brass nuts and lining joints to nyloon connection.

Qty: 05 Nos	@RS: 447.15	each	RS: 2236/-

9/-	S/F Bib cock superior quality with C.P head  ½” dia (S.I NO: 13 P-19).

Qty: 04 Nos	@RS: 1109.46	each	RS: 4438/-

10/-	S/F Cancelead stop cock of superioir quality (S.I NO: 3 b P-15).

Qty: 02 NOs	@RS: 507.47	each	RS: 1019/-

11/-	S/F handle valve china ¾” Dia (S.I NO: 4 P-17).

Qty: 02 NOs	@RS: 271.92	each	RS: 544/-

12/-	Supplying & fixing Fiber Glass Tank of approved quality and design and wall thickness as specified i/c cost of nuts bolts and fixing in plate form of cement concrete 1:3:6 and making connection for in-let outlet & over flow in-let outlet flow pipes etc. complete. (S.I.No.3 P.No.18) (Rate per gallon Rs: 33/60 for tanks upto capacity of 250 gallons) 200 Gallons

Qty: 01 NO		@RS: 21989.16 each	 RS: 21990/-

(twenty one thousands nine hundreds eight nine rupees sixteen ps:)

13/-	Providing & Fixing R.C.C Pipe Collar glass Band digging the trenches to required depth and fixing in position including cutting , bending & fitting etc complete

6” Dia					Qty: 84  rft		RS: 199.25	P.Rft	RS:16737/-

(one hundred ninty nine rupees twenty five ps:)

14/-	P/F asbestive pipe with collar deed or equivalent i/c digging trencheshes etc (S.I NO: 1 c P23).

Qty: 58 Rft		@RS: 141.11	P.Rft	RS: 8184/-

(One hundred fourty one rupees eleven ps:)

15/-	Providing & fixing 6" x 4" C.C gully trap with  4" outlet complete with 4" thick 1: 2: 4 C.C. for bed  & 1/2" thick cement plaster (1:3) to the karb, C.I grating  6" x 6" &  C.I cover  and frame 12" x 12" (inside) etc. complete.

Qty: 03 NOs		@RS: 1193.17	each	RS: 3580/-

(eleven hundreds ninty three rupees seventeen ps:)

=========================================

Total						RS: 111665/-

| Sr | Item of work | No: L:B:D | QTY | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- | --- |

P.H.S . ITEMS

01/-	Boring for tube well in all water bearing soils from ground level up-to 100 ft or 30.5 meter depth i/c sinking and with drawing of casing pipe. (100 mm 4”) (S.I.No. 01 (b) Page No.36).

Qty: 60 Rft	@RS: 242/-	P.Rft	RS: 14520/-

02/-	Construction of manhole  or inspection chamber  for the required dia

of circular sewer   and 3’-6’ 	(1067 mm ) depth with walls  of 	B.B  in cement mortar  1:3 cement plaster  1:3 ½”  thick in side 1”  	(25 	mm ) thick over  beching  and channel i/c  fixing C.I main 	hole cover with frame of clear 	opening  1 ½”x 1 ½”” 457x457 	mm) of 1.75 cwt . (88.9 kg) embedded in plain c.c 1:2:4) & fixing 	(25 mm) dia M.S steps 6” 150 mm)   white projecting 4” (102 mm) 	from the face of walls at 12” 	(305 mm) c/c duly painted etc 	complete as per standard specification & drawing.(S.I No- P-46).

Qty:01 Nos	@Rs:14748/-			each	RS: 14748/-

(fourtheen thousands seven hundreds fourty eight)

03/-	Providing & Fixing PVC  Pipe  Class “D” etc complete

Qty: 40 rft	@RS: 34.			P.Rft	RS: 1360/-

(two hundreds fifty rupees sixty ps:)

=========================================

Total						RS: 30628/-

E-I ITEm

01/-	Supplying & Fixing water pumping set ½” H.P complete

Qty: 01 No	RS: 3646.59			each	RS: 3647/-

=========================================

Total						RS: 3647

______% Above / Below (Amount to be added as per/deducted)RS:___________

G.Total					RS:

Total Amount in word:________________________________________________

__________________________________________________________________

Note:- 	Quantities ans rates may be changed as per/according to the Techinical Sanction vy the competent authority.

EXECUTIVE  ENGINEER

(BUILDINGS) WORKS & SERVICES

DEPARTMENT  KHAIRPUR