NAME OF WORK:- 	CONSTRUCTION OF NEW ANNEXE BUILDING I & II IN THE PREMISES OF 				SINDH HIGH COURT KARACHI (WALL PANNELLING   &  FALSE  CEILING

IN THE CHAMBER OF ADVOCATE GENERAL & PROSECUTOR GENERAL

SINDH).

SCHEDULE”B”

| S/No | Description | Qty | Rate |  | Unit | Amount |
| --- | --- | --- | --- | --- | --- | --- |
|  | PART-A(SCHEDULE ITEM). |  |  |  |  |  |
| 1. | Preparing the surface and painting with matt finish in/c rubbing the surface with bathy (silicon carbide rubbing brick) filling the voids with zink / chalk / plaster of paris mixture , applying first coat premix, making the surface smooth and then painting 3 coats with matt finish of approved make etc complete (b) 2nd & subsequent coat. (36-a+b+b/55). | 5526.0 Sft. | 3444/38 |  | %Sft. | 190336/- |
|  |  |  | Total (A)Rs. |  |  | 190336/- |
|  | PART (B) (N.S.I) |  |  |  |  |  |
| 1. | P/F partition with fixed glass (frosted) 5mm thick using 4” lucky section in champion color as frame on floor or block masonary fixed with necessary fixture rubber packing etc the cost of in/c tools and plants used in making and carriage from shop to site as directed by the Engineer Incharge. | 4874.25 Sft. |  |  | P.Sft. |  |
| 2. | Providing and fixing wall paneling with lassani sheet ¾” thick and teak ply in/c frame wrk of partal wood 2”x1” & fixing lassani sheet & then teak ply in approved design and texture including all cost of making and cartage etc complete. | 3081.63 Sft. |  |  | P.Sft. |  |
| 3. | Making and fixing wooden pelmet type false ceiling of approved design including making wooden frame size( 2 ½”x1 ½” ) to required space, & double step over hanging ceiling with 1/8” thick ply sheet, including making/ cutting holes for lighting arrangement and coloring work to required shed etc complete in all respect. | 2189.34 Sft. |  |  | P/Sft. |  |
| 4. | Providing and fixing false ceiling gypsum fibre board in/c panel of size (24”x24) in/c frame work of aluminum double channel section hanged with nails, hooks, wire to ceiling etc as directed by the consultants. | 6300.0 Sft. |  |  | P.Sft. |  |
|  |  | Total (B) Rs. |  |  |  |  |
|  |  | G.Total(A+B) |  |  |  |  |

TERMS AND CONDITION

1. 	Any typographical  errors in the schedule ‘B’ are subjected to any correction with the 	reference to the schedule of Rates General item 2012 in force from 12-07-2012 as 	approved by the standing rates committee Sindh Karachi.

2.	100% well graded crushed bajri shall be used in item of RCC 1:2:4.

3.	No Premium shall be paid on Non schedule items.

4.	The work will be carried out as per PWD Specifications.

| Contractor. | Sd/- ASSISTANT ENGINEER Provincial Building Sub Divisionb-XI, Karachi | Sd/- EXECUTIVE ENGINEER Provincial Building Division-No.III Karachi |
| --- | --- | --- |